| International Telecommunication Union Telecommunication Development Bureau |  |  |  |
| --- | --- | --- | --- |
|  | Regional Telecommunication Development Conference for the Arab States (AR-RTDC-96) Beirut (Lebanon), 11-15 November 1996 | Document 37-E 24 October 1996 Original: English For Information/Support |  |

Agenda item: 3.1	PLENARY MEETING

SOURCE:	SAUDI ARABIA

TITLE:	CURRENT CHALLENGES FACING SAUDI ARABIAN MOPTT IN OPERATING A NEW SDH NETWORK

________

Abstract

The Saudi Arabian MOPTT is in the midst of its sixth major telecommunications expansion due for completion in the year 2001. This impressive $US 5 billion project will expand the Kingdom's telephone network by 1.5 million lines over a seven-year period and 500 000 GSM subscribers in the next three years. The existing, predominantly analogue transport network, will be transformed into a modern digital infrastructure with some 10 000 km of fibre optic cables in SDH rings configuration and it will be managed by new Operations Support Systems. The undeniable magnitude of the rapid changes and growth is bringing new challenges to the MOPTT. The Operations and Maintenance managers and engineers are now concentrating their efforts on ensuring a smooth transition from the interim to the future operating environments. The introduction of new technologies will also necessitate significant re-training of the operations and maintenance resources to equip them with new technical competencies.

## Introduction

The Ministry of Post, Telephone and Telegraph (MOPTT) of the Kingdom of Saudi Arabia is currently undertaking a major telecommunications expansion designed to provide the Kingdom's population with a 21st century network and services. This is the MOPTT's sixth major telephone expansion. The contract to implement this project was awarded in August 1994 to a joint partnership of AT&T and A.S. Bugshan & Brothers. AT&T was recently renamed to Lucent Technologies Inc.

The expansion programme, referred to as TEP6/GSM, will by its completion in the year 2001 double the number of subscriber lines to a new total of 3 million local exchange subscriber lines. In order to provide the infrastructure that will support this network, the MOPTT will expand the following key elements:

•	local switching network;

•	national and international switching network;

•	junction transmission network;

•	outside plant;

•	Subscriber Radio Systems (SRS) network for rural and remote areas;

•	satellite earth stations.

The TEP6/GSM also includes a digitization of the long-distance transmission network, as well as other switching and transmission enhancement projects, e.g., ISDN and network synchronization. Our customers in rural and medium-sized communities across the Kingdom are going to have access to the same level of modern telephone service enjoyed by customers in the largest cities.

In order to meet the public demand for wireless services in the Kingdom, the TEP6/GSM programme also includes an implementation of the GSM cellular network operating in the 900 MHz band. This network has been in operation since January 1966 and by now is already serving some 170 000 subscribers in four major cities in the Kingdom, namely Riyadh, Jeddah, Dammam and Makkah. This network will be expanded over the next three years to serve a total of 500 000 customers in 45 major cities and 3 500 km of highways across Saudi Arabia.

The TEP6/GSM project is a matter of great pride for all of use who are associated with it in the MOPTT. While we are confident in our and our contractor's abilities to complete this enormous undertaking, there are tremendous challenges ahead of us due to the scope of this project. In my presentation, I will be sharing with you some of the engineering and management challenges we are facing in implementing this project and integrating it with our existing network.

## Existing terrestrial networks

Prior to the TEP6/GSM expansion, the backbone of the Saudi long-distance network was formed by an extensive, mostly analogue, microwave radio. Some of the initial 4 and 6 GHz microwave systems are now 19 years old. The first Saudi Arabian Intra Kingdom microwave network was established between 1977 and 1979 and expanded in the early 1980's. The existing network also includes approximately 4 000 route-km of analogue coaxial cables between Riyadh and Damman and between Riyadh and Jeddah, approximately 1 200 route-km of digital coaxial cable, and approximately 2 000 route-km of optical fibre systems including a multimode fibre system between Makkah and Taif.

The existing junction (inter-exchange) transport network consists of short-haul digital microwave links, 2 Mbit/s PCM cables, and some PDH optical fibre systems. Multimode cables were used for 850 nm and 1 300 nm systems installed some 10 years ago. All later installations used single-mode cables and 1 300 nm wavelengths.

## New SDH junction and access networks

With the expansion project the existing network is being significantly expanded. The programme includes an installation of new digital transmission systems for new sites as well as over-building on existing analogue routes. The new transport systems will be optical fibres and digital radios conforming to the Synchronous Digital Hierarchy standard.

Extensions of the junction network are built to connect new exchanges to the network and handle the expanded traffic from the new exchanges and from the extensions of the existing exchanges. Close to 100 000 junction trunks are being added to the existing network. More than half of these

new trunks are going to be transported over new optical fibre systems. New digital microwave systems are planned to add new capacity for some 10 000 junction trunks.

Based on the current seven-year plan, the Expansion Project is providing 170 junction and access network rings in the whole Kingdom. This installation includes 50 junction network rings with a bandwidth of STM-4 (622 Mbit/s) and 120 access network rings with an STM-1 bandwidth (155 Mbit/s).

The main network element of both the junction and the access network rings is ISM-2000 Intelligent Synchronous Multiplexer which is based on the worldwide standard Synchronous Digital Hierarchy (SDH) technology. The ISM-2000 is designed to multiplex plesiochronous G.703 tributary signals, i.e. 2 Mbit/s, 34 Mbit/s or 140 Mbit/s signals into the STM-1 or STM-4 signals. It is equipped with two STM-N ports, where N is 1 or 4, and designed as a drop and insert multiplexer with built-in cross-connect facilities and flexible tributary arrangements with optical or electrical line terminations. The ISM-2000 can be used as a terminal multiplexer or regenerator and can be configured as a 1+1 line protection or as a ring add/drop system.

The junction networks are utilizing ISM-2000-4 multiplexers (intelligent multiplexer for 4 STM-1 signals) while the access networks are employing ISM-2000-1 (intelligent multiplexer for 1 STM-1 signal). Both networks work on optical single mode fibre cable operating with 1 310 nm and 1 550 nm wavelengths. The microwave junction networks employ SDH microwave systems with STM-1 capacity.

## New SDH long-distance networks

The long-distance network is being built using a backbone of five 2.5 Gbit/s fibre optic rings augmented by high capacity digital microwave radio hops.

When completed, the long-distance Fibre Optic Network (FON) will cover the geographical area of Saudi Arabia (2 240 000 sq. km) in its entirety by replacing eventually all of the existing analogue microwave and coaxial systems.

The new long-distance fibre optic network is designed to accommodate traffic generated in 22 new national service nodes and two international gateway service nodes. The service nodes will be exchanges equipped with five ESS switches.

The ring configuration of the network will allow:

•	clockwise or counter-clockwise traffic flow within each ring;

•	transfer of traffic between rings;

•	entrance, exit or through movement of digital messages at each ADM;

•	self-healing structure, offering seamless traffic restoration.

The heart of the long-distance fibre optic network is based on SLM-16 synchronous line multiplexers connected in a 2-fibre add/drop terminal configuration. Each fibre ring system has a capacity of 16 STM-1s with a total optical line rate of 2.5 Gbit/s. To benefit from the self-healing properties of this technology, only half (8 STM-1s) of the total capacity is used for traffic in each direction of transmission.

Digital distribution frames, normally subject to manual maintenance and prone to service interruptions, are being replaced with software driven digital access and cross-connect systems such as the DACS VI. Service nodes are going to access the transport network via DACS at STM-1 speeds. DACS V are being used to interface rings at locations where the rings meet each other.

STM-16 fibre optic rings are going to have adjacent STM-4 linear routes with the purpose of transporting low volume traffic to sites between service nodes. This low volume traffic includes telephony, roadside emergency telephones and 34 Mbit/s digital television signals.

Part of the benefits of using the SDH technology is the compatibility of different systems and the opportunity of deploying these in the same network without additional expenses for interfaces. In our case, microwave radio systems continue to play an important role in the Saudi MOPTT's transport strategy. Digital microwave equipment forms a part of the long-distance network either in a linear configuration or as part of a fibre/microwave ring.

Some 40 high capacity SDH radio hops are being installed or planned to be installed in various parts of the long-distance network. These microwave links are going to operate in the 6 and 11 GHz frequency bands. They are designed to transport an SDH capacity of 155.52 Mbit/s. They are also compatible with PDH systems working at speeds of 139.264 Mbit/s.

## Network management systems

The operations and maintenance philosophy of the Kingdom's network is based on the concept of centralized data acquisition, functional control of switch and transmission facilities, performance analysis and dispatch for corrective and preventive maintenance. This concept is applied throughout all exchange and transmission operations and maintenance functions, including traffic, exchange and outside plant organizations.

The O&M of our switched and transmission networks is organized in three levels: national, regional and district.

At the national level, the National Network Control Center (NNCC) is the highest level in the network operation and management hierarchy with an overall responsibility for the MOPTT network. The Regional Maintenance Center (RMC) constitutes an intermediate level in the network management hierarchy with a responsibility to monitor exchanges and local junction networks. The District Dispatch Center (DDC) is the base level of the network management hierarchy. The DDC responds to directives from the RMC and serves as a focal point for all work activity associated with exchanges and junction networks within its geographical jurisdiction.

The long-distance microwave is supervised by the Telecommunications Alarm and Surveillance Center (TSAC) while the coaxial cable systems are monitored by the TIC alarm surveillance systems. Another OSS system CAMODS (Centralized Alarm and Monitoring of Digital Systems) is being completed to provide centralized alarm and performance monitoring of existing digital transmission systems.

With the introduction of the TEP6/GSM project and the expansion of the telephone network, the operations and maintenance systems are being upgraded as well. Provision of centralized operations and maintenance systems to cover the whole network is part of the programme for the proper management and maintenance of the existing and new networks. A new Telecommunications Management Network (TMN) is being implemented to control all of the new networks and to integrate all existing network management systems, including TASC, TIC, CAMDOS, switching management systems, billing, network records, etc.

The switch to SDH will allow us to get a greater control over a more robust and reliable network. All centralized telecommunications management network facilities are planned to be collocated with the National Network Control Center in Riyadh. The management of all SDH network

elements is going to be provided by I-2000. This centralized network management system has been developed for the operation, control and monitoring of SDH networks.

## Manpower resources and technical skills

All over the world PTTs and private Telcos are searching for more cost-effective ways of running the business. Even without the competitive environment forces, the increasingly high expectations of the customers in Saudi Arabia is forcing the MOPTT to strive for more efficient processes and organizational set up. The introduction of new technologies, such as SDH, fibre optic network, network management systems and the state-of-the-art switching, assists in reaching the more cost-effective service solutions and places the MOPTT on a path of continuous improvements to the network and service management processes.

These new technologies, however, also require new methods of operating and maintaining the network and new technical skills in supervising and controlling the network elements. These tremendous changes are challenging the PTT's management ability to effectively utilize its internal resources including its staff as well as its network. The initial TEP6/GSM project phase included an overall review of the Present Method of Operation (PMO) in order to identify a series of Intermediate Methods of Operation (IMO). The IMOs are required to evolve the principal processes based on phased deployment of new technologies, the associated OSSs and the resulting organizational changes.

The introduction of new technologies is not only necessitating changes to the network management processes, but also requires changes to the customer services processes, such as service provisioning, trouble management and billing. The Network Operations Department is analysing and forecasting the impact of these changes on the staffing requirements. This includes forecasting of the numbers of persons required to perform key jobs as well as their locations and time-frames. The most fundamental component of these studies is determining the skills required to perform these jobs. The MOPTT Training Department is assisting with identifying the competencies required to perform and manage the jobs and the training programmes that must be provided to the employees.

The Saudi Arabian MOPTT puts a great emphasis on the training of its employees. The staff go through extensive short- and medium-length training in operating and maintaining the new equipment. There are also numerous courses being taught in management and control of the new switching and transport technologies and courses in operating all support systems being introduced under the TEP6/GSM programme. The PTT's management is certainly aware that a complete success of the TEP6/GSM implementation and the subsequent operation and maintenance can only be achieved through a well-trained workforce.

## Looking ahead

The use of the SDH technology and rings will have a dramatic impact on all aspects of service provided to our customers. Ring networks recover traffic interruptions in milliseconds in the event of fibre cable discontinuity. Because of the synchronous mode, add-drop multiplex equipment allows the entrance and exit of traffic in the fibre ring with a minimum combination of electronic circuitry and software application. The reduction in translating processes and reduction in the quantities of in-service equipment reduces the initial capital expenditures, increases service reliability, reduces maintenance requirements and saves building space and the associated environmental control costs.

The introduction of the new SDH transport technologies managed by new operations and maintenance support systems will, from a customer's perspective, not only provide increased network reliability and service availability but also a wider product selection as well as reduced servicing intervals. The customer satisfaction will be increased with increased efficiency realized by new technologies, new operations support systems and more effective organization.

figure 1

TEP6/GSM expansion of the long-distance network

figure 2

SDH junction ring

figure 3

SDH long-distance ring

Figure 4

SDH telecommunications management networks

_____________