| International Telecommunication Union Telecommunication Development Bureau |  |  |  |
| --- | --- | --- | --- |
|  | Regional Telecommunication Development Conference for the Arab States (AR-RTDC-96) Beirut (Lebanon), 11-15 November 1996 | Document 43-E 4 November 1996 Original: English For Action |  |

Agenda item: 3.1	PLENARY MEETING

SOURCE:	RAPPORTEUR FOR Q.5/1 - INDUSTRIALIZATION AND TRANSFER OF TECHNOLOGY

TITLE:	TELEBRÁS INDUCTIVE CARD PAYPHONE SYSTEM:
A SUCCESSFUL CASE OF TELECOMMUNICATIONS IMPROVEMENT THROUGH TECHNOLOGICAL DEVELOPMENT, TRANSFERENCE OF TECHNOLOGY AND INDUSTRIALIZATION IN BRAZIL

________

Introduction

Before 1992, all efforts of TELEBRÁS system to expand the payphone plant were frustrated by a set of factors:

–	the high maintenance costs;

–	the volume of operational costs mainly due to the token collection device malfunctioning;

–	the breaks due to vandalism for stealing the tokens;

–	the lack of an efficient telesupervision system; and

–	the logistics necessary to collect, clean, package and distribute the tokens.

During the Earth Summit, held in Rio de Janeiro in 1992, a new payphone concept was introduced in Brazil, namely the TELEBRÁS inductive card payphone system.

The results have been so good that TELEBRÁS decided to install only card payphones since then. In comparison to token payphones, the operating and maintenance costs have decreased in some areas by a factor of four. The acquisition investments for long-distance payphones have been cut by half. Besides that, usage rates per payphone have increased by factors above ten times.

The modern services options, e.g. virtual telephone and information services, together with new revenue sources, e.g. media associated with cards, combined with the much lower implementation, operating and maintenance costs, all related to the inductive card payphone system, are making it possible to establish goals never before imagined to be achieved.

Regarding expansion, TELEBRÁS established the goal to implement 600 000 inductive card payphones by the year 2000. At the end of 1996, it is expected that more than 200 000 of these payphones will be in operation.

The main reason for TELEBRÁS to assume such a challenge is the success of its Research and Development Centre's inductive card development project. It is a very good example of a successful telecommunications quality improvement programme involving technological development, transference of technology and industrialization.

Reference situation

The situation of the payphone system in Brazil at the end of June 1991 has been taken as a reference for future comparisons.

At this point of time, the users' opinion gathered through OPUS market research data processing ranked the payphone services as the worst provided by TELEBRÁS system in Brazil. Backing the users' opinion, the physical operational data indicated that out of a plant of 175 000 token payphones about one third used to be out of order every month. The major causes for these problems, were related to the token collection devices and the lack of telesupervision, in addition to vandalism due to both, breaking the safes for stealing tokens and because the payphones were not working well.

The financial data showed that considering the tokens collected, the annual token revenue of a token payphone was $US 546, against an annual cost of $US 570. The picture then was complete: besides being the worst service provider, the payphone system was also a money loser.

Taking the above into account, a task force was established to propose a technical solution for this telecommunications quality problem. The work of this task force, coordinated by TELEBRÁS Services Planning Department, was performed by professionals from the engineering and operations areas, and especially from TELEBRÁS Research and Development Centre - CPqD.

The technical configuration proposed

The task force divided the work into three steps. Initially, considering that cards are more and more becoming key components of the telecommunications services, it was a consensus that cards should replace the tokens. Moreover, since the card payphones would not require safes, one of the major causes of vandalism would be eliminated. The second step was an extensive technological assessment covering all systems components. The third and last job of the group was a technical configuration proposal, which should be jointly developed by TELBRÁS CPqD and the industrial sector.

Technological assessment

The technological assessment emphasized the points listed below, in terms of four components: payphones, cards, telesupervision and switching.

Payphones

1)	Only one type of payphone for all kinds of calls (local, long distance and international).

2)	Minimization of the movable parts in the payphone (these parts were identified as the major causes of problems).

3)	Working in very different climates (from very humid and hot in the north, to very dry in the north-east, to mild and sometimes cold in the south).

4)	Working in a variety of environmental conditions (besides pollution of the big cities, many of the Brazilian capitals are located at the coast).

5)	No need for commercial energy (energy provided by the telephone line).

6)	Possibility of development of a wireless payphone for isolated areas (energy provided by solar panels).

7)	Possibility of the development of a payphone for internal use (less robust and consequently requiring less investment).

8)	Brazilian industrial capacity to manufacture payphones (two existing suppliers of token payphones, possibility of three additional ones).

Cards

1)	Expected card technology evolution (long-term future vision indicates a trend towards a contactless chip card energized through magnetic induction).

2)	Low production costs for cards (to make it feasible to produce cards with a small number of call units for both local calls and low-income people).

3)	Brazilian industrial capacity to produce cards (non-existing capacity, possibility of establishing from three to five suppliers out of the conventional card manufacturers).

Telesupervision

1)	Possibility of development of a user friendly telesupervision system (preferably in an environment extensively used, e.g. Windows).

2)	Messages oriented to both operational and commercial uses.

3)	Alarms to point specific trouble situations.

Switching

1)	Ability to work in an environment of many switching vendors (different technologies, multiplicity of facilities available, polarity inversion and 12/16 kHz pulse signalling).

2)	Possibility of transferring functions from the switching equipment to the payphone (essentially tariffing functions, which means portability in different plant situations).

3)	Possibility to develop the concept of a platform of small size digital switching centres fully dedicated to payphones.

Technical configuration

The technical solution proposed by the task force was a card payphone system completely integrated into the multi-vendor existing network, able to work within the concept of a fully dedicated platform of small size digital switching centres, and having user friendly telesupervision facilities.

The development of such a system was established to be done according to the following schedule: initially inductive contactless solution for prepaid cards, followed by a solution encompassing the inductive contactless and some type of metallic contact chip card. The ultimate solution will be a family of inductive contactless cards, including chip cards, for many kinds of applications.

Even knowing that especially in Europe the credit cards with a magnetic strip are in the process of being substituted by the chip cards, a high volume of magnetic cards are still in use today. Addressing this share of the market, a simple magnetic card reader that could be attached to the payphones located in specific locations is in the prototype stage.

Regarding the fully dedicated platform of small size digital switching centres for payphones, this concept is being implemented within the card payphones expansion plans of the TELEBRÁS system operating companies. It is expected that by the end of 1996 there will be a statewide platform implemented in the state of Rio de Janeiro by TELERJ, the local TELEBRÁS system operating company.

Transference of technology

The transference of the technology process followed by TELEBRÁS for the payphones and the associated telesupervision system started at the initial stage of development. At that point, the two existing suppliers of token payphones were invited to participate in the development of the card payphone.

The reason for the invitation was the expected similarity between the manufacturing processes of the card payphone and the token payphone. The same reasoning was used for the telesupervision system, because the two token payphone suppliers had experience in developing software.

In order to guarantee additional suppliers, a mandatory condition of non-exclusivity after the development was established. As a consequence, today there is a total of five suppliers, which means more competition into the TELEBRÁS system bidding processes for acquiring payphones.

Besides being the first to enter the market, the only permanent tangible advantage for the two inductive card payphone suppliers that participate in the development effort refers to the royalties paid, which are a bit lower than those to be paid by the latter licensees.

Starting in 1992, the TELEBRÁS system operating companies are acquiring the inductive card payphones through decentralized bidding processes.

Regarding the cards, the manufacturing and handling processes involve three distinct groups of activities:

–	chemical processing for creating the card basis on plastic boards;

–	printing the plastic boards, and individual card cutting and testing; and

–	handling of products having face value ("quasi money") with security.

Due to the fact that there were no similar in the world for the inductive cards production, TELEBRÁS considered the experience in handling products with face values as the important characteristic to be emphasized.

According to this view, three manufacturers of products with face value (two real money producers) were invited to participate in the card development process. Out of the three, one had experience in chemical processing (gold melting), and the other two were manufacturers of conventional cards.

The major challenge for producing cards came a few months before the Earth Summit. The facilities available at CPqD were able to manually produce a few thousand cards per month, against requirements for the event of more than three hundred thousand cards. For coping with this

problem, one existing chemical line at CPqD was then transformed into a semi-automatic card production line working 24 hours a day, with people and material from the three invited manufacturers.

At the end of this effort, that can be considered as the turning point of the project, more than 300 000 valid inductive cards had been produced.

Regarding additional suppliers, a fourth manufacturer was licensed in 1995, and another one is starting the process of receiving the technology. It is worthwhile to say that these two new entrants were chosen through public selection processes.

Since the security of the payphone system against fraud relies heavily on the delicate chemical production processing, TELEBRÁS is playing a centralization role in the acquisition of cards.

The development of the concept of fully dedicated platforms for payphones is not expected to generate relevant transference of technology.

Industrialization achieved

The implementation of the TELEBRÁS inductive card payphone system has resulted in real industrialization as described below:

•	Growth of the payphone manufacturing sector: from a duopoly of token payphones, to a situation of five suppliers, with a total installed capacity close to 20 000 payphones per month.

•	Creation of new capacity in the card sector: from a null capacity, to the existence of four suppliers (one more in the process of licensing), with a total installed capacity of more than 80 million cards per month expected to be achieved during the first semester of 1996.

•	Growth of the switching manufacturing sector: from a situation where the payphones were considered marginal within the conventional telephone expansion, to a new vision of fully dedicated platforms to payphones.

Taking into account the goals established for the TELEBRÁS system during 1996 (implementation of more than 100 000 payphones with the associated switching facilities and usage of more than 600 million cards), the TELEBRÁS inductive card payphone system alone will be responsible for more than 3 000 jobs in the industrial sector.

Besides the industrial jobs, in the new service providers business levels to be established in the near future, the creation of additional jobs related to messaging, virtual telephone, information services and media is expected .

As a final remark, the transference of a technology model adopted by TELEBRÁS is allowing competitive advantages abroad for the Brazilian industrial sector.

According to this model, the industrial sector is responsible for manufacturing activities of the telecommunications production chain that lies in between development and operations, both conducted by TELEBRÁS. This means that the acquisition of products with TELEBRÁS technology brings together the perspective that future versions will be constantly updated, since such improvements are mandatory for keeping the TELEBRÁS system competitive.

In addition, as the technology holder, TELEBRÁS permits the transference of technology according to a schedule encompassing SKD/CKD activities, before full local production.

_________________