| International Telecommunication Union Telecommunication Development Bureau |  |  |  |
| --- | --- | --- | --- |
|  | Regional Telecommunication Development Conference for the Arab States (AR-RTDC-96) Beirut (Lebanon), 11-15 November 1996 | Addendum 1 to Document 48-E 13 November 1996 Original: English only For Information/Support |  |

Agenda item: 3.1	COMMITTEES 1 AND 2

SOURCE:	LEBANON

TITLE:	THE LEBANESE EXPERIENCE IN THE PROVISION OF 
TELEPHONE SERVICE

________

| Acronyms: | CSP: | Customer service point |
| --- | --- | --- |
|  | POS: | Provision of service |
|  | PSTN: | Public switched telephone network |
|  | MPT: | Ministry of Post and Telecommunications |

Abstract

The aim of this paper is to show the methodology that the Lebanese Ministry of Telecommunications used in providing the telephone service over the newly introduced public switched telephone network (PSTN). Lebanon has now (on the date of 11 November 1996) 115 000 working telephone lines on its new PSTN which comprises on the same date some 350 000 lines available for provision of service (POS). This has been accomplished during a period of 10 months only.

Introduction

During the last twenty years, the Lebanese population had at its disposal an old PSTN with the following major problems:

–	a very poor quality of service offered to the customers even for the basic telephone service (congestion problems with very high call blocking (or rejection) probabilities, a very degraded voice quality, etc.);

–	a small number of lines available and very complicated administrative procedures which means that big difficulties were faced by customers in need of telephone lines;

–	the customer plant records were not computerized and the information related to them when available is not reliable which resulted in an extremely poor maintenance.

Besides all of this the MPT (Ministry of Post and Telecommunications) staff were corrupted in their majority because of small salaries and therefore the Lebanese telephone customers were the helpless victims of an uncontrolled piracy.

Almost two years ago a decision had been taken to sign a contract with three well-known telecommunications companies (Ericsson, Siemens and Alcatel) in order to build a completely modern PSTN with approximately 1 million telephone lines that should be available for provision of service. This meant a lot of investment and therefore a successful provision of telephone service was needed for bringing back some revenues. The short-term goals of the Lebanese Administration were therefore:

–	to offer telephone service to the customers as soon as available in their areas;

–	to computerize the customer plant records and purify the billing system;

–	to measure the quality of work of the PSTN contractors for forcing them to improve their performance where needed, although a consultant had been appointed.

There was a strong will among the Lebanese telecommunications experts to have this work completely performed by Lebanese staff and they finally convinced their administration that the work would be better performed in this way.

How did we proceed?

ii)	The first thing to do was to address the following set of issues:

–	The delivery of lines available for POS is taking place in several parts of the country starting mainly in urban areas. In order to facilitate the citizen's access to the telephone service, customer service points (CSPs) were to be progressively created all around the country as dictated by the order of delivery of the lines.

–	Having a certain number of lines available for POS in a certain area does not imply the whole area is covered. We chose therefore to address the citizens in a selective manner based on the availability of lines for POS in their locations.

–	Asking the old customers to address their CSP before being transferred to the new network will saturate the CSPs since the delivery of lines started in regions covered by the old PSTN implying a lot of transfers at the beginning. We therefore chose to collect as much information as possible about the real situation of old customers when we address them in the serviced areas so that this information can be compared to that provided by our billing system and old customers can be immediately transferred to the new network only when the collected information corresponds to that provided by the billing system.

–	A new numbering plan needs to be defined since the number of customers will increase and therefore a strategy for allocating telephone numbers has to be introduced taking into account the exchanges that will exist and the long-term demand forecast for these exchanges.

–	The coexistence of two networks (the old and new ones) during a certain period of time with two authorities (one following the old administrative procedures and the other planning and implementing new procedures for the new network) necessitated to define procedures for having a reasonable cooperation between those two authorities so that the customers do not face problems when having to deal with both of them.

–	The old network should not be used in a certain area whenever service can be offered there on the new network. Therefore the old network should be discarded as soon as possible.

–	A feedback should be given to the administration concerning the quality of work of the PSTN contractors, therefore formats for reports and procedures should be introduced in this prospect.

–	The maintenance of the new network should receive great attention. This represents a very big and important issue for which training of young Lebanese staff on the new technology is fundamental and extremely urgent, otherwise the Lebanese customers will have to experience again a poor quality of telephone service.

–	The issue of third-party damages caused to the new network should be resolved by defining procedures for dealing with it especially during the period where the cable entities are still under the contractors warranty.

–	For maintenance and billing purposes a sophisticated data network linking all CSPs and exchanges should be built and its implementation should start as soon as possible.

–	The customers should be kept satisfied therefore efficient procedures should be implemented for dealing with all the problems they might face and a fault management system should be introduced in the CSPs with the double goal of solving the customer's problems and quantifying our performance in the provision of service.

–	In Lebanon the internal wiring inside the buildings does not follow international standards therefore a study should be made on the subject and the results should be implemented as soon as possible.

ii)	To develop a software that takes into account the solutions that have been adopted for the set of problems discussed above. This software is to be used in each CSP and run over a platform that can support all the needed evolutions.

iii)	To define several types of training programmes:

–	training for the CSP staff;

–	training for the "canvassing" teams;

–	training for the installation teams;

–	training for an operations and supervision team.

iv)	To start recruiting staff for the several teams.

Our achievements during 10 months

Annex A gives the flowcharts of our customer services department. It comprises 460 persons.

Annex B gives some figures related to the installation activity.

In short we have (over a period of 10 months):

–	15 CSPs all around the country;

–	125 000 lines attributed to customers on the new network;

–	115 000 thousand working lines on the new network.

–	18 million US dollars earned only from subscription fees.

The maximum delay needed for installing one customer's line is one week assuming no faulty lines and assuming the customers have done what they were requested to do as far as the building's internal wiring is concerned.

annex a

Current structure of the Customer Services Department

|  |  |  |  |  |  |  | Director |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Assistants |  |  |  |  |  |  |  |  |  |  |  |
| Computerization of faults section |  |  | Operations team |  |  | CSPs |  |  | Installation teams |  |  | Canvassing teams |  |  | Software team |  |
|  |  |  |  |  |  | 15 sites |  |  |  |  |  |  |  |  |  |  |

annex b

Weekly progress for the last 21 weeks

11/11/96

annex b (cont.)

Monthly new connections

annex b (cont.)

Monthly transfers