| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| GROUP OF TECHNICAL EXPERTS GENEVA, 6-9 APRIL 1998 | Document GTE98-1/14-E 6 April 1998 Original: English |

Saudi Arabia, Benin, Burkina Faso, 
Cameroon, Ghana, Morocco, Syria

contribution

On the basis of the information available on the application of the tasks decided by the first meeting of the IRG as described in Attachment C of the report of the first meeting, it becomes clear that approaches A and B will not lead to any practical result. Consequently, the only remaining possibility is approach C and therefore the group should consider the details of the application of that approach. The following is proposed in this respect:

1	The planning exercise has to consider two matters which may be difficult to undertake simultaneously. These are:

a)	the provision to each administration/territory of an amount of spectrum of around ten analogue channels without taking account of the existing systems; and

b)	the inclusion in the plan of existing systems.

2	The extent to which step a) is possible depends on:

–	the technical criteria to be used, it is assumed in this respect that these criteria would be those appearing in Resolution 531 as they may be improved as a result of the studies requested in Resolution 532;

–	the modulation technique, analogue or digital, however at this stage the tests should be limited to a full analogue plan;

–	the free selection of orbital location by each Member State or the designation by the Bureau of the most appropriate location for those administrations which have not indicated their preferences;

–	in order to facilitate this step some limitations need to be attached to the free choice of the orbital locations, assume that in Region 1 the 800 MHz band is split in four sub-bands of 400 MHz each, the allotment of this sub-band to each of the adjacent countries having selected the same orbital location is possible only if the number of these countries is equal to or lower than four;

–	if, in Region 1, more that four adjacent countries request the same orbital location, these countries will be considered as a group with a single territory to which the whole 800 MHz will be allotted, leaving to them the ways they will assign sub-bands to the different constituents of the group,

should planning be limited to ten channels per beam.

3	It is proposed to carry out the first action a) as follows:

a)	start by identifying the means by which allotting ten channels per administration/territory may be achieved, using the requested orbital locations or notified ones;

b)	if it is possible, include gradually those existing systems which are considered by the group as the first priority;

c)	if it is possible, include gradually the remaining existing systems;

d)	if the steps a) and/or b) are not possible, make the same attempt a), b) and c) with a smaller sub-band.

4	When considering requirements, existing or planned, that covers more than one territory, such requirements should apply to only subregional systems to be defined by the group.

___________________

The term "existing system" is used to simplify reference to those systems that will be included in the plan following consideration of different cases in conformity with the Radio Regulations.

The same difficulties would exist when more than four neighbouring countries request the same orbital location or orbital locations within a small arc, so that this statement, if accepted will need to cover such cases.

The term allotment is used intentionally in the contribution because it may be a way to resolve some of the difficulties that could be encountered in this exercise.