| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| GROUP OF TECHNICAL EXPERTS GENEVA, 15-16 and 18-19 FEBRUARY 1999 | Document GTE99-2/13-E 12 February 1999 Original: English only |

France

# Questions and comments on document IRG-2/8

# (Approach 2)

1	The planning exercise has to consider two matters which may be difficult to undertake simultaneously. These are:

a)	the provision to each administration/territory of an amount of spectrum of around ten analogue channels 400 MHz without taking account of the existing systems; and

- the inclusion in the plan of existing systems as defined in principle 3.

- It is to be noted that the terms “existing systems” is now defined differently from what it was in the GTE document concerning Approach D.

- The proposed approach starts from the principle that the two tasks a) and b) should be undertaken separately, whilst it recognises that this may be difficult. Section 3 is understood as describing the way in which this inconsistency may be reconciled by adopting a iterative process associating both tasks. Is it the case ?

- The concept of an allotment Plan with 400 MHz allotted to each country in one polarisation (as confirmed verbally during IRG-2, but not explicitly stated in document 8) is understood as the provision to each country of 10 contiguous non-overlapping channels in the same polarisation (e.g. all the odd channels from channel 1 to channel 19), which is equivalent to a contiguous bandwidth of 400 MHz, if also including the current guard bands.

- If this interpretation is correct, is this approach limited to Region 1 or does it also involve Region 3 ? If it does, are Region 3 countries given 250 MHz in one polarization (in which case there is a problem of equitability) or 400 MHz in one polarisation (in which case there is a systematic waste of 100 MHz at each orbital location) ? What is proposed to solve this dilemma ?

2	The extent to which step a) is possible depends on:

- the technical criteria to be used, it is assumed in this respect that these criteria would be those appearing in Resolution 531 as they may be improved as a result of the studies requested in Resolution 532;

- 5.			The criteria in resolution 531 now appear in Appendix S30/S30A. It is understood that, under the proposed approach, where channelization will disappear, the use of this criteria should be applied under the worst case assumptions, i.e. co-channel interference. This will lead to decrease the capacity of the Plan in respect of the current assignment plan approach. This will make more difficult, if not impossible, to satisfy several of the principles of Resolution 532 : principles 1, 3, 5, 7 and 8.

–	the modulation technique,  should be digital, however at this stage the planning exercises should take account of existing analog systems;

- Comment : This protection has to be ensured in the framework of principle 3 of Resolution 532 in any case.

–	the free selection of orbital location by each Member State or the designation by the Bureau of the most appropriate location for those administrations which have not indicated their preferences;

- The concept of “free selection” of the orbital positions by the administrations is mostly inappropriate in this context : the planning exercises have to be performed in a heavily constrained environment, in addition to the need to avoid mutual interference between the transmissions operated within each location and at various orbital locations. Proposing this concept as one of the main features of this approach (as repeatedly mentioned by Algeria at the IRG-2) is extremely misleading since it can only give many administrations the expectation that their requirements can be satisfied, whereas even a minimum understanding of the issue proves that it cannot.

- in order to facilitate this step some limitations need to be attached to the free choice of the orbital locations, assume that in Region 1 the 800 MHz band is split in four sub-bands of 400 MHz each, the allotment of this sub-band to each of the adjacent countries having selected the same orbital location is possible only if the number of these countries is equal to or lower than four;

- if, in Region 1, more that four adjacent countries request the same orbital location, these countries will be considered as a group with a single territory to which the whole 800 MHz will be allotted, leaving to them the ways they will assign sub-bands to the different constituents of the group,

- This point implicitly recognises the impossibility of the previous one, and attempts to solve it by creating an even more difficult problem, i.e. allocating capacity to a group of administrations, rather than to administrations, and doing that over a regional coverage (“a single territory” for four adjacent countries has to be understood as a sub-regional coverage), none of these two issues being in conformity with principle 2 of Resolution 532.

–	should planning be limited to ten channels per beam.

3	It is proposed to carry out the first action a) as follows:

a)	start by identifying the means by which allotting ten channels per administration/territory may be achieved, using the requested orbital locations or notified ones;

- if it is possible, include gradually the remaining systems;

- if the steps a) and/or b) are not possible, make the same attempt a), b) and c) with a smaller sub-band.

- Section 3 is understood, after the clarification given verbally at IRG-2 on the GTE document on Approach D, as dealing with the protection of the existing systems, as defined by Principle 3 of Resolution 532. However, in document 8, the word “existing” is stricked out in c), which reopens the issue of the conformity with Annex 1 of Resolution 532.

- Section 3 in document 8 is not self-consistent, since it includes references to a paragraph b) which has disappeared, whereas the logic of the whole section was revolving around this paragraph . Should this Section 3 be understood as meaning that, if is not possible to protect all the existing systems with an allotment of 400 MHz per country in one polarisation, another attempt should be made with a smaller bandwidth per country, such as 300 MHz ?

- 4	Consideration of requirements, existing or planned, that covers more than one territory,  may involve a series of non technical constraints to be examined by the appropriate body.

- This is not an issue for the GTE and IRG, which are concerned only with laying the basis of a future planning.

___________________

The same difficulties would exist when more than four neighbouring countries request the same orbital location or orbital locations within a small arc, so that this statement, if accepted will need to cover such cases.

The term allotment is used intentionally in the contribution because it may be a way to resolve some of the difficulties that could be encountered in this exercise.