| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| GROUP OF TECHNICAL EXPERTS GENEVA, 28-30 JUNE 1999 | Document GTE99-3/1-E 25 May 1999 Original: English only |

Working Party 8A

LIAISON STATEMENT TO the group of technical experts (GTE)

Frequency sharing between the mobile service and the
broadcasting-satellite service in the 12 GHz band
(Annex 7 to RR Appendix S30)

Working Party 8A has received with thanks the liaison statement from GTE in Document 8A/53 (source: Document GTE98-1/TEMP/8) and considered your request "to determine whether operational and planned systems in the terrestrial mobile services in the planned BSS bands would also be protected by the Annex 1 criteria absent the 8 dB e.i.r.p. reduction."

WP 8A has been informed that Joint Working Party 10-11S has reached a preliminary conclusion that the 8 dB e.i.r.p. reduction clause in Annex 7 to RR Appendix S30 may not be necessary to protect terrestrial services (see Document IRG 98-1/4, 10-11S/TEMP/15(Rev.1)). WP 8A also noted that SG 9 confirmed the conclusion reached by Working Party 9D regarding the protection of fixed services, such that operational and planned systems in the fixed service are adequately protected by the sharing criteria of Annex 1 and do not require the 8 dB e.i.r.p. reduction.

Moreover, it was noted that the Regions 1 and 3 plan revised by the WRC-97 adopted BSS satellite e.i.r.p. which is 5 dB smaller than that of the WARC-77, and it was felt that the provisions of Annex 1 to RR Appendix S30 adequately protect operational and planned MS systems.

In view of the above factors, Working Party 8A concludes that the 8 dB reduction in Annex 7 to RR Appendix S30 is no longer necessary, as far as the mobile service is concerned.

It should also be noted that at the February 1999 meeting WP 8A approved a new Question relating to the study of 4th generation mobile services. Thus, the implication of the proposed relaxation of BSS e.i.r.p. limits on mobile service spectrum allocations for future technologies is unknown.

______________