| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| GROUP OF TECHNICAL EXPERTS GENEVA, 27 SEPTEMBER - 1 OCTOBER 1999 | Document GTE99-4/8-E 22 September 1999 Original: English |

Radiocommunication Bureau

STUDY TO DETERMINE WHETHER IT IS POSSIBLE TO ADD TWO ADDITIONAL CHANNELS FOR EACH REGION 3 COUNTRY

- 1.	Introduction

During IRG-4 it was concluded that at the end of Step 4, the GTE would attempt to add two additional channels for each Region 3 country at the orbital positions determined at the end of Step 4” and also “perform studies to identify any undue constraints on Region 1 BSS resulting in the border area between Regions 1 and 3 in determining the feasibility of these additional two channels.

In order to facilitate the discussion during GTE-4 on this subject, the Radiocommunication Bureau conducted a study on the effect of two additional channels added to the beams of Region 3 countries at the orbital positions determined at the end of Step 4.  Due to lack of time only the effect in the downlink was studied.

- 2.	Impact of two additional channels

Two additional channels which give the same channel arrangement (even or odd ) and same polarisation as those in “the draft Step 4 Plan” were selected from channels 21, 22, 23 and 24 except the beams of AUS and CHN at orbital position 92°E.  In case of AUS, channel 9 was added to the composite beam at orbital position 152°E which covers entire Australia and channel 12 was added to the composite beam at orbital position 164°E which also covers entire Australia in order to provide 12 channels at any test point in Australia.  In case of Chinese beams at orbital position 92°, as channels 21, 22, 23, 24 were already used for CHN18000, no attempt was made to add additional channels to those Chinese beams at orbital position 92°E.

The following incompatibility cases between BSS-BSS downlink were identified.

Table 1  Incompatibility cases

| Case | Region 1 beam/channel | Region 3 beam/channel | The worst EPM |
| --- | --- | --- | --- |
| 1 | UAE (CR)/21,23 at 50°E TKM(CR)/22, 24 at 50°E | AFG (CR)/21,23 at 50°E | -22 dB |
| 2 | RUS (CL)/25 at 56°E | IND (CL)/24 at 56°E | -2.4dB |
| 3 | TJK (CL)/21, 23 at 62°E | CHN (CL)/21, 23 at 62°E | -24dB |

- 3.	Possible solutions to the identified incompatibility

In order to solve the incompatibilities identified above the following solutions were applied and negative EPMs were removed.

Table 2  Possible solutions

| Case | Proposed solution |
| --- | --- |
| 1 | Relocation of TKM beam to 74°E with channels 22, 24, 26, 28, 30, 32, 34, 36, 38, 40 (CL). Change of polarisation of the UAE beam from CR to CL. |
| 2 | Change of polarisation of IND composite beam at 56°E covering southern part of India form CR to CL Change of polarisation of IND composite beam at 56°E covering northern part of India form CL to CR |
| 3 | Relocation of TJK beam to 80°E with channels 22, 24, 26, 28, 30, 32, 34, 36, 38, 40 (CL). |

If Raster D is used for the TJK beam, it is not necessary to change its orbital position.

It would be possible to provide the Chinese beams at orbital position 92°E with 12 channels if Beams CHN180 is combined with CHN161, CHN175 and CHN179 to formulate a composite beam.

- 4.	Conclusion

It is possible to add two additional channels for each Region 3 country for the downlink if the orbital positions and channel blocks of some beams determined at the end of Step 4 are properly adjusted.  It should be noted that 10 countries in Europe are still not accommodated in “the draft Step 4 Plan”.

The possibility of adding two additional channels to the uplink is yet to be studied.