| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| RADIOCOMMUNICATION STUDY GROUPS | Document SC97-1/14-E 27 January 1997 Original: English only |

Chairman, ITU-R Study Group 9

NEW TEXT FOR CHAPTER 7.5 OF DRAFT CPM-97 REPORT

The attached text was prepared by Working Party 9B and adopted by last meeting of ITU-R SG 9 (Geneva, 21-23 January 1997).

It is intended as a replacement of the existing text in draft CPM-97 Report (version November 1996).

Attachment: Document 9/102.

Source:	Document 9B/TEMP/52(Rev.1)

Working Party 9B

Revision of Chapter 7.5 for inclusion as an Addendum to 
Chapter 7.5 of the DRAFT CPM Report to WRC-97 
(VERSION, NOVEMBER 1996)

### 7.5.1	WRC-97 agenda item 1.9.6

The identification of suitable frequency bands above 30 GHz for use by the fixed service for high-density applications.

For clarity, the term "High Density Fixed Service" or "HDFS" will be used throughout the text to refer to high density fixed service system applications.

### 7.5.2	Summary of technical and operational studies

ITU-R chose to draw a technical demarcation between two broad categories of radio-relay systems at about 17 GHz (Recommendations ITU-R F. 1101 and F.1102):

•	bands below about 17 GHz, where multipath fading is the controlling propagation impairment mechanism and hop lengths of up to about 80 kilometres are typical;

•	above 17 GHz, where the usable hop lengths are determined by precipitation fading and atmospheric absorption; they are on the order of 10 kilometres and shorter.

TABLE 1

Status of radio-relay services in bands below 30 GHz

| Bands | Usage | Room for growth |
| --- | --- | --- |
| 1 to 17 GHz | Primarily high-capacity long distance and regional trunking | Most available bands at or near saturation in many administrations. Recent MSS and BSS allocations between 1 and 3 GHz and MSS feeder links at 6 GHz limit FS spectrum availability |
| 17 to 30 GHz | Mobile network backhaul and other trunking; medium/high density local access | Some proposed new satellite systems already constrain further FS deployment and/or operation in most available bands, progressively increasing constraints arising from sharing with space and satellite services. HDFS deployment would be impeded under envisioned co-primary sharing constraints and due to higher self-interference than in bands above 30 GHz |

Table 2

Ranges and status of existing FS allocations above 30 GHz

| Band range (GHz) | FS allocation | Other primary services |
| --- | --- | --- |
| 31.0 - 31.3 | Primary | MS |
| 31.5 - 31.8 | Secondary | EESS, RAS, SRS, in Regions 1 and 3 |
| 4 allocations in 33.4 - 36.0 | Primary in 41 administrations by footnote (S5.549) | Radiolocation, SRS (deep space, E-to-S), SRS (deep space, S-to-E), meteorological aids, MS primary in 41 administrations by footnote (S5.549) |
| 6 allocations in 36.0 - 40.5 | Primary | EESS (passive), MS, SRS (passive), SRS (S-to-E), FSS (S-to-E), MSS (S-to-E), EESS (E-to-S), SRS (E-to-S) |
| 40.5 - 42.5 | Secondary | BS, BSS |
| 42.5 - 43.5 | Primary | FSS (E-to-S), MS, radio astronomy |
| 3 allocations in 47.2 - 51.4 | Primary | FSS (E-to-S), MS, EESS (passive), SRS (passive), radio astronomy by footnote S5.556 |

| 54.25 - 58.2 | Primary | EESS (passive), ISS, MS, SRS (passive), aeronautical mobile service by footnote S5.558, radiolocation in two administrations by footnote S5.557 |
| --- | --- | --- |
| 59.0 - 64.0 | Primary | ISS, MS, radiolocation, aeronautical mobile service by footnote S5.558, airborne radar in the radiolocation service by footnote S5.559 |
| 92.0 - 95.0 | Primary | FSS (E-to-S), MS, radiolocation |

Recommendation ITU-R F.746 provides summary information on the ITU recommended radio-frequency channel arrangements for radio-relay systems including the frequency bands 
17.7 - 19.7 GHz ("18 GHz"), 21.2 - 23.6 GHz ("23 GHz"), 24.25 - 29.5 GHz ("27 GHz"); 
31.0 - 31.3 GHz ("31 GHz"), 36.0 - 40.5 GHz ("38 GHz") and 54.25 - 58.2 GHz ("55 GHz").

The types of radio-relay services that may be provided in FS frequency bands is determined largely by maximum usable hop lengths (given required performance and availability objectives) and available bandwidth. As hop lengths become shorter and frequency reuse (i.e., potential deployment density) becomes correspondingly higher moving through the bands above 17 GHz, higher and higher density FS services become more technically practical.

#### 7.5.2.1	Impact of an increasingly competitive environment

Local access and other high density radio-relay service planning and system deployments have rapidly accelerated in the last several years in many administrations. This acceleration is due in large part to the worldwide trend towards deregulation and increased competition in the provision of local telecommunications and video distribution services. Because of cost and speed of deployment considerations, these developments are placing a major new focus on the provision of services directly to end-users via fixed wireless systems. As a result, the number of fixed service subscribers in the bands above 17 GHz worldwide is currently close to 200 thousand and growing at a rapid rate. The demand for local wireless broadband services is experiencing unprecedented growth, leading in turn to a new and pressing emphasis on the national, regional, and international designation of spectrum for the implementation of HDFS systems. Thus is formed the basis for WRC-97 agenda item 1.9.6.

#### 7.5.2.2		HDFS characteristics

Recommendation ITU-R F.1102 covers characteristics of radio-relay systems operating in frequency bands above about 17 GHz. It includes a variety of applications, hop length considerations, and digital radio implementations. The companion Recommendation ITU-R F.1101 supplements it in several important aspects, including a comparison of current and advanced modulation and coding methods. Additional relevant aspects are covered in Recommendations ITU-R F.756 on point-to-multipoint systems, ITU-R F.1104 on requirements for point-to-multipoint radio systems in the local grade portion of an ISDN connection, and ITU-R F.750 on SDH-based networks. The relevant band specific radio frequency channel arrangements are provided in Recommendations ITU-R F.749 (38 GHz), and F.1100 (55 GHz). Recommendation ITU-R F.758 provides specific system parameters and other information relating to the development of sharing

criteria for fixed service systems. This group of ITU-R Recommendations and related references provide a framework for analysing HDFS system deployment, particularly in the bands above 30 GHz.

Current HDFS data rates over individual circuit paths range from about 1.5 MB/s to 45 MB/s, and are expected to reach at least 310 MB/s within the next several years, as radios utilizing higher order modulation schemes become available (see Document 9D/TEMP/49). To further increase spectral efficiency and service flexibility, advanced HDFS systems are likely to employ various techniques that may include dynamic allocation of bit rate, modulation, and antenna beamwidth. Finally, HDFS links may operate at elevation angles of up to about 45 degrees in dense urban environments.

#### 7.5.2.3		HDFS deployment

The vast majority of current worldwide HDFS deployments are in the 38 GHz band, with a primary focus on urban and suburban business and industrial areas. Future HDFS deployment is expected to extend to residential areas, spearheaded by local distribution of television programmes in competition to cable TV and other new broadband service offerings to the home. The variety of possible current HDFS network configurations includes: conventional point-to-point (P-P), conventional point-to-multipoint (P-MP), and combinations thereof, e.g. P-P systems deployed in multisectored P-MP configurations. High density deployment of independent P-P links similarly results in clusters that assume the essential characteristics of P-MP deployment. The densest HDFS deployment cases have reached the range of 1 to 10 stations per square kilometre, and are expected to increase several fold within a few years.

Newly developed technology enables stationary lighter than air repeaters to be located at fixed points in the stratosphere above commercial airflight altitudes and over metropolitan areas. Such radio-relay systems using stratospheric repeaters can provide both domestic and international broadband wireless service to user terminals (see NOTE 1).

TABLE 3

Status of high density FS in bands above 30 GHz

| Bands | Usage | Room for growth |
| --- | --- | --- |
| 30 to 36 GHz | Light to moderate use due to limited available allocations | Contingent on future availability of sufficient spectrum for HDFS |
| 36 to about 50 GHz | Local access, inter-cell links for mobile and wireless local loop networks, fiber backup, local TV distribution, broadband GII access, intelligent transport, SDH access, RLANs, ATM compatible transport | Substantial 38 GHz deployments in several administrations; expanded and new licensing commencing in these and additional countries. Bands below about 40 GHz are currently preferred to satisfy HDFS hop length, reuse, and antenna sizing objectives. Uses proposed in other bands. HDFS in these bands could be subject to constraints resulting from future sharing with co-primary space and satellite service systems. HDFS systems using stratospheric repeaters proposed to operate in 2 x 300 MHz in the range 47 - 49 GHz |
| Above about 50 GHz | Technically similar to 30 - 50 GHz, but over shorter hop lengths, thus limiting service offerings as compared to lower HDFS bands | HDFS deployment in progress in 55 - 58 GHz. Expanding HDFS uses in this and other bands planned. Limited availability of equipment and co-primary sharing considerations, such as with EESS, are principal restrictive factors. High absorption losses result in short hop lengths but very high frequency reuse |

### 7.5.3	Analysis of the results of studies

#### 7.5.3.1	Preferred frequency ranges

The following factors determine the preferred frequency range for HDFS applications:

•	Service requirements: The achievable hop length requirements are derived from the network performance objectives of ITU-T Recommendation G.826 and market requirements which can translate into a minimum availability objective of 99.999% (as used in [Document 4-9S/TEMP/29]).

•	Propagation conditions: The propagation conditions are such that usable hop length decreases with increase of frequency as illustrated in Table 4 below. The frequency bands above 50 GHz have unique propagation characteristics which are suitable for certain high density fixed service applications. The high absorption losses in this range tend to favour short hop length applications that can benefit from high frequency reuse.

•	Availability of equipment: Commercial radio equipment in the 30 - 50 GHz range is now available for mass production. In bands above 50 GHz, the more compact size of the equipment, including antennas, is conducive to particular applications such as in-building local area networks.

TABLE 4

Representative propagation distances(1)

| Frequency band | Climatic zone B(2) | Climatic zone G(2) | Climatic zone K(2) |
| --- | --- | --- | --- |
|  | U = 0.001% | U = 0.001% | U = 0.001% |
| 38GHz(3) | 8.2 km | 3.7 km | 2.7 km |
| 55GHz(3) | 3.5 km | 2.2 km | 1.9 km |
| U% Unavailability (1) Demonstrating achievable propagation distance based on Recommendation ITU-R F.1102. (2) Climatic zones B, G, K are defined in Recommendations ITU-R P.620 and P.837. (3) Antenna gain Gi: 44 dB, Transmitter output: 15 dBm, BER 10-3 threshold: -84.5 dBm. |  |  |  |

The frequency range between 30 and 40 GHz is preferred to satisfy current and near term future market requirements, including hop length and availability objectives. This explains the popularity of the existing 38 GHz band, which is already becoming congested in a number of countries and may be subject to constraints arising from co-primary sharing with the FSS and other space services.

HDFS systems using stratospheric repeaters can also make beneficial use of millimeter wave frequencies up to the sharp increase in atmospheric absorption at about 50 GHz owing to the very high elevation angles (up to 90 degrees) obtainable from a stratospheric location.

#### 7.5.3.2	HDFS sharing considerations between 30 GHz and up to about 55 GHz

HDFS intraservice station distances are substantially smaller than interservice separation distances. Therefore, the integrity of HDFS service areas can be severely affected by the exclusion zones required for sharing. Accordingly coordination with and by other services should be carried out for HDFS service areas instead of individual HDFS stations. Similar findings are contained in [Document 4-9S/TEMP/16], [Document 4-9S/TEMP/19], Document 9D/TEMP/44 and Document 9D/TEMP/45 Attachment 9.

Similarly, HDFS systems using stratospheric repeaters use a full range of elevation angles. Thus, sharing with services other than broadcasting-satellite service feeder links located outside primary coverage areas may present difficulties in the bands proposed for such systems. See, also [Document 4-9S/TEMP/30] and Document 9B/TEMP/38.

All the frequency bands above 30 GHz allocated to the fixed service are also allocated to one or more other services on a shared basis. Sharing with another service in the same deployment area would severely impair:

•	achievable coverage density;

•	service quality;

•	cost effectiveness;

•	spectral efficiency;

•	rapid installation of large quantities of links.

### 7.5.4	Methods to satisfy WRC-97 agenda item 1.9.6

Geographic and/or frequency separation using one or more of the following three conventional methods:

1)	identify potential frequency bands for additional primary FS allocations, taking due account of the requirements of other allocated services;

2)	within the existing co-primary allocations, designate frequency bands for the fixed service and the other allocated services to allow each service to independently optimize the trade-offs between coverage density, service quality, cost effectiveness and spectral efficiency;

3)	further investigate the feasibility of mutually advantageous co-frequency sharing between high density fixed service and the other allocated services with the aim of increasing the overall spectral efficiency of the shared bands.

The following framework should be used for the designation of any suitable HDFS band.

Frequency range:

1)	within the 30 to 50 GHz range, preferably below 40 GHz for current HDFS applications; within the 47.2 - 50.2 GHz range for HDFS systems using stratospheric repeaters;

2)	above 50 GHz for applications requiring high atmospheric absorption characteristics.

Band structure:

1)	about 2 GHz contiguous or, about 2 GHz made up of two or four equally split, non-adjacent subbands;

2)	for HDFS systems using stratospheric repeaters, one paired band of 2 x 300 MHz.

Minimum bandwidth of each subband: 400 MHz.

Generally, the achievable transmit/receive spacing from HDFS bands should be in the range of 500 MHz to 1.5 GHz, preferably around 1 GHz; for HDFS systems using stratospheric repeaters, a minimum transmit/receive spacing of 400 MHz is preferred.

### 7.5.5	Advantages and disadvantages of identified methods

Method (1):

Advantage: identify more suitable spectrum for HDFS expansion.

disadvantage: may require more substantial regulatory actions.

Method (2):

Advantage: each service can independently optimize its deployment.

Disadvantage: the amount of spectrum available to each service is smaller than the full existing shared allocation.

Method (3):

Advantage: all sharing services can potentially utilize their entire allocation.

Disadvantage: the necessary sharing restrictions may severely reduce service viability and thereby offset the potential gain of utilizing the entire allocation.

### 7.5.6	Regulatory and procedural considerations

The methods to satisfy WRC-97 agenda item 1.9.6 could entail changes to the Radio Regulations affecting allocations in bands above 30 GHz.

WRC-97 may need to establish an agenda item for WRC-99 to address continuing study issues and related regulatory matters that may arise from the treatment of HDFS issues at WRC-97.

NOTE 1 - It is recognized that radio-relay systems using stratospheric repeaters could potentially be utilized to provide various other services, such as the mobile and broadcasting services. This CPM chapter focuses on HDFS applications. The technical and regulatory aspects of possible requirements for systems using stratospheric repeaters in other services should be treated as required in other appropriate venues.

______________