�INCLUDEPICTURE R:\\APP\\WW6\\ITULOGO.WMF����INTERNATIONAL TELECOMMUNICATION UNION

RADIOCOMMUNICATION�STUDY GROUPS�Document SC97-1/3-E

27 November 1996

Original: English��





Rapporteur of Rapporteur Group SC-3

report to the sc

regulatory/procedural aspects of the revision of�appendices 30/30a for regions 1 and 3

(Resolution No. 524 (WARC-92), Resolution 531 (WRC-95))

This is the report from the SC-3 Rapporteur to the SCRPM regarding the regulatory/procedural aspects of the review of Appendices 30 and 30A, in support of the preparation of WRC-97 Agenda Item 1.10, which relates to the Plans for the broadcasting-satellite service in Regions 1 and 3. It is submitted to the SCRPM for its consideration. It is based essentially on the inputs received from WP 10-11S and provided separately to the SCRPM.

Given the very complex relationships between planned and unplanned services allocated in the three Regions in the frequency bands covered by Appendices 30 and 30A, it has been generally recognized that the improvements required in the regulatory/procedural provisions applicable to Regions 1 and 3 would be very difficult to implement if harmonization of the procedures was not achieved over all three Regions, in particular in relation to other than planned services and to the possible means to facilitate the introduction of subregional systems.

In this context, one of the most significant concerns at this stage is the strong views which have been expressed by several Region 2 administrations, concerning the changes which could affect Region 2 and the consequential need that such changes be clearly identified. Region 2 administrations are therefore encouraged to actively participate in the preparation of this WRC-97 agenda item with the view to achieve worldwide harmonization in the regulatory/procedural aspects of Appendices 30 and 30A, to the benefit of the development of the broadcasting-satellite service. 

Introduction

The regulatory/procedural aspects of the revision of Appendices 30 and 30A in Regions 1 and 3 include the following provisions:

a)	the procedures for modifying the Regions 1 and 3 Plans (Article 4 of Appendices 30/30A);

b)	the procedures for notification, examination and recording of assignments to stations in the BSS and associated feeder-link Plans (Article 5 of Appendices 30/30A);

c)	the procedures for the coordination, notification and recording of assignments to stations in other services to which the planned bands are allocated, so far as the relationship of these services with the planned services is concerned (Articles 6 and 7 of Appendices 30/30A);

�d)	miscellaneous procedural/regulatory provisions applying to the broadcasting-satellite service, and to the other services sharing the same bands:

•	in Appendices 30 and 30A (Articles 2, 3, 8, 9, 12, 13 and 14 of Appendix 30, Articles 2, 3, 8, 10 and 11 of Appendix 30A, and various Annexes to these Appendices containing technical parameters and criteria);

•	in Article S5, relating to the allocation of the frequency bands from 11.7 to 12.7 GHz in Regions 1, 2 and 3 to the broadcasting-satellite service and to several other services: S5.485 (RR 836), S5.487 (RR 838), S5.488 (RR 839), S5.490 (RR 844), S5.491 (RR 845), S5.492 (RR 846), S5.493 (RR 847);

•	in Article S5, relating to the allocation of the frequency bands from 14.5 GHz to 14.8 GHz and from 17.3 GHz to 18.1 GHz in Regions 1, 2 and 3 to the fixed-satellite service, for feeder links to the broadcasting-satellite service and to several other services: S5.510 (RR 863), S5.515 (RR 868A), S5.516 (RR 869), S5.517 (RR 869A) and S5.518 (RR 869B);

•	in Articles 11, 13, 14, 15 and 15A and in Articles S9 and S11;

•	in Article S23, a general provision applying to the broadcasting-satellite service: S23.23 (RR 2674);

•	in Resolutions 32, 33, 34, 42, 106, 109, 506, 507, 518, 519, 524 and 531;

e)	the Rules of Procedure adopted by the Radio Regulations Board concerning all these provisions.

The present experience in the application of these provisions and in the operation of broadcasting-satellite systems has highlighted the conservativeness of the technical parameters and criteria used in the Plan to determine the need for seeking agreement (Appendix 30, Annex 5 parameters, including reference antenna diagram and protection ratios, Annex 1 pfd and C/I degradation thresholds, Annex 7 limitations on the orbital locations, and similar limitations in Appendix 30A), or to determine conformity with the Plans.

Taking into account the improvements in technology, it appears that these technical parameters and criteria could be reviewed and suitably modified, in order to increase the flexibility of the Plan to accommodate new systems (in particular digital systems) and requirements, as well as eliminating unnecessary coordinations.

With the objective of improving the flexibility of the Plans and increasing their potential to meet the expected requirements of administrations, an in-depth review has been undertaken of all these provisions. On the basis of this review, proposals are made to consider the regulatory provisions on the basis of the recent technical and technological improvements.

1	Structure of the current and simplified procedures relating to Appendices 30 and 30A

1.1	Current structure of the regulatory procedures applicable in the frequency bands of Appendices 30/30A 

The frequency bands associated to the Appendix 30 and 30A Plans are different in Regions 1, 2 and 3. Several terrestrial as well as space services, which are not subject to these Plans, are sharing the same bands with an equal primary status, on a regional and/or interregional basis, either through allocations within the Table of Article S5 itself, or through footnotes to this Table. 

�This results in a number of different sharing situations for both the Appendices 30 and 30A bands. These situations are addressed through regulatory procedures which may appear either in Appendix 30/30A or in other parts of the Radio Regulations. 

The following table provides a general outline of the sharing situations and associated procedures in the bands covered by Appendix 30 and 30A.



�Coordination or agreement to be sought by (��Coordination of agreement sought from (�Planned services�Others��Planned services�Articles 4 and 5 of Appendix 30

Articles 4 and 5 of Appendix 30A�Article 6 and 7 of Appendices 30, 30A

Articles 11, 13 and 14, Resolution 33, Resolution 34 (for interregional inter-service sharing and for sharing between unplanned and planned services)��Others�Articles 4 and 5 of Appendix 30

Articles 4 and 5 of Appendix 30A�Articles 11, 12, 13, 14

Resolution 33, Resolution 34��1.2	Relationship with the simplified procedures adopted by WRC-95

Simplified procedures (Articles S7 through S14) have been adopted by WRC-95, subject to satisfactory resolution at WRC-97 of possible inconsistencies within these procedures. Work to identify and correct these inconsistencies is being carried out within SCRPM SC-1. One of the main difficulties encountered in carrying out this task is the relationship between the simplified procedures and the procedures of Appendices S30 and S30A.

Three possible approaches have been identified in order to resolve these difficulties:

–	Approach 1 (Status Quo ante WRC-95): In this approach, the entire set of procedures under Appendices 30/30A would be kept within these Appendices, as was the case before WRC-95. This could lead to difficulties since there are a number of provisions in the current Appendix 30/30A procedures which either refer to or are similar or identical to other provisions in the RR which have been modified substantially by WRC-95. Keeping those unchanged within Appendices 30 and 30A would therefore not take full advantage the work carried out by both the VGE and WRC-95.

–	Approach 2 (VGE approach): This approach would be along the lines proposed by the VGE and WRC-95, in that, on the basis of Recommendation 35, the modification procedures of the Plans would be simplified, and the procedures applicable to the other services would continue to appear under Article S9, which was the approach followed by WRC-95. The simplified procedures for modifying the Appendix 30 and 30A Plans and notifying assignments in these Plans could remain within these plans, or alternatively, appear in a new Article using the available slot in the sequence of the new Articles of the Radio Regulations (i.e. Article S10). This new Article S10, and its related technical coordination criteria (Appendix S6) would be applicable only to Appendices 30 and 30A. The procedures �applicable to the unplanned and planned services would therefore appear in only three Articles (S9, S10 and S11), with the parameters and sharing criteria appear in the related Appendices S4, S5, S6, S7 and S8.

–	Approach 3 (Further simplification): In this approach, further simplification would be made by locating all the procedures of Appendices 30 and 30A in Articles S9 and S11. All the procedures for all services would then be found in these two Articles, with Appendix S4, Appendix S5, S7 and S8 used for the network parameters and the sharing criteria and methods. Although this approach would provide the most efficient procedures in terms of volume of text, it would entail significant risk in the future that modifications in the procedures for an unplanned service lead to unintended modifications to the procedures of a planned service, which would not be satisfactory.

In view of this, the SC-3 Rapporteur suggests that Approach 2, which appears to be the least controversial, and does not involve per se any change of substance in the current procedures, be recommended to CPM-97 and WRC-97 as the basis for the revision of the procedures relating to Appendices 30 and 30A.

2	Provisions for the modifications of the Plans

Existing regulatory/procedural provisions in the Radio Regulations, which are presently applicable to the Broadcasting-Satellite Service (BSS) or to the Fixed-Satellite Service (FSS), have been reviewed and compared, together with the experience gained in applying these provisions. As a result of the work of the VGE and the decisions of WRC-95, the simplified procedures that may apply to the frequency bands of Appendices 30/30A have also been reviewed.

The Bureau’s experience in the application of Appendices 30 and 30A has shown that, in most cases, the problems encountered had been resolved by the BR, through appropriate modifications to the existing M-space software (implementation of shaped beams), consultation with WP 10�11S (different polarization type, and, provisionally, non-standard parameters) or by the RRB, through the establishment of Rules of Procedure. 

Based on the experience of the Bureau and the administrations in the application of Appendices 30 and 30A procedures, the studies undertaken to improve the current procedures for modifications of the Plans (Articles 4 of Appendices 30 and 30A and associated technical parameters and criteria in Annexes 1, 2, 5 and 7 of Appendix 30 and in Annexes 1, 2, 3 and 4 of Appendix 30A) have focused on four main objectives: simplification, in line with the recommendations of the VGE and consistent with the decisions taken by WRC-95, completeness, flexibility and efficient use of the orbit/spectrum resource.

Satisfaction of these objectives could be facilitated by the following measures. An example of implementation of some of these measures into procedural text is provided in Annex 1, based on Recommendation 35 (WRC-95), which provide a basis for developing such a procedure (Article T10). For clarity, this example text is called Article T4 in the following.

a)	Simplification

•	Avoid duplications between the provisions applicable to the various Regions within the same Appendices;

•	avoid duplications between the provisions applicable to the BSS and the Feeder-Link Plans;

�•	avoid redundant or superfluous provisions;

•	facilitate the understanding and the application of the provisions.

These objectives could be satisfied by the following approach:

•	Merge Article 4 of Appendix 30 and Article 4 of Appendix 30A into a single Article T4 used to modify both Plans;

•	merge Annex 1 of Appendix 30 and Annex 1 of Appendix 30A into a single set of provisions (Appendix S6) used to identify administrations affected by a modification to a Plan;

•	merge Annex 2 of Appendix 30 and Annex 2 of Appendix 30A into Appendix S4, as already implemented by WRC-95.

It should be noted that Article T4 and Appendix S6 could be located either within Appendices 30 and 30A, or alternatively, in a specific Article in the series of those of the simplified Radio Regulations adopted by WRC-95, which would operate only to Appendices 30 and 30A.

b)	Completeness

•	Address the sharing situations not presently covered by Appendices 30 and 30A.

From Section 2, it can be seen that one such situation would need to be covered:

•	Protection of the unplanned feeder links to the BSS in Region 2 from modifications in the Regions 1 and 3 feeder-link Plan (2.4 of Appendix S6).

c)	Flexibility and efficient use of orbit/spectrum resources

•	Introduce a time limit in the case when an administration having made a comment to a modification of the Plan does not answer to a request for coordination (See T4.21 and T4.22).

•	Discourage administrations to request additional resources in the Plan without using the resources allocated to them by the Plan (See T4.10). It was noted that the proposed text, taken from the existing text in Article 4 of Appendix 30A (4.1.1), may not be sufficient to achieve this objective. Care should be exercised however, in attempting to strengthen this provision, to avoid undue constraints on the development of subregional systems. 

•	Ensure a maximum time interval of [4-5 years] between the date receipt of the information concerning a proposed addition to the Plan, with a maximum extension of [2-3 years] to be approved by the RRB under specific, exceptional circumstances (T4.8).

•	Introduce the cluster concept, as appearing in Region 2 Plans and introduced in Regions 1 and 3 Plans by the Rules of Procedures (text not provided).

•	Introduce the group concept, as appearing in Article 10 of Appendix 30 and the Rules of Procedures, and endorsed by WRC-95 (text not provided).

•	Introduce the possibility for an administration to modify the Plans on behalf of a group of administrations (T4.7 bis).

•	Introduce a section in the modification procedures (Appendix S6, par. 1.6 and 2.7) containing the cases of modification to a Plan where agreement is not required. This section would contain, in particular, the conditions in which an analogue assignment in conformity with the Plan can be converted to the digital mode without the need for seeking agreement.

•	Limit the use of steerable beams to the identification, under Article T4, of suitable fixed pointing directions for which inclusion in the Plan is possible (text not provided).

�Annex 7 of Appendix 30 and Section 3.15 of Annex 3 of Appendix 30A contain provisions on the orbital positions to be used when modifying the Plans. Annex 5 of Appendix 30 and Annex 3 of Appendix 30A contain technical assumptions on which the Plans were established. All these provisions have an impact on the flexibility of the Plans. 

However, additional studies are required before a recommendation can be made on the possibilities to modify these annexes and, in the second case, on the possibility, or desirability to use them as reflecting standard requirements that would be applied on a mandatory basis for modifying the Plans.

It should be noted that there are conflicting requirements concerning the approach to be taken on the issue of non-standard antennas:

–	equal treatment to all new comers may warrant the introduction of standard antenna patterns for all transmit and receive space and earth station antennas, with protection afforded only at the level of the standard pattern;

–	antennas performing better than the standard pattern in all directions should be allowed;

–	any such measures would have the potential to seriously inhibit the implementation of new technologies, at the potential detriment of the use of the Plan resources.

Many administrations are of the opinion that the rigidity of procedures is the main reason for a lack of flexibility of the Plan. Some administrations consider that a complete review of WARC�BS�77 procedures would give the wanted flexibility to the Plan. 

In this instance, and in view of the somewhat different status currently given to additions to the Plan and to modifications of existing assignments in the Plan (the former being subject to a time limit of 5 + 3 years), consideration might be given to associate different procedures to each of these two types of assignments.

For example, both types could be subject to the procedures mentioned above, except that additions would be from a distinct part of the Plan, which would be protected by somewhat more flexible procedures, such as Articles 11 and 13. The original assignments in the Plan, and their subsequent modifications, would continue to be protected by the above procedures (Article T4 and Appendix S6).

Further studies are required, however, to investigate the feasibility of such an approach.

3	Procedures for the notification, examination and recording of assignments in the Plans

The existing procedures for the notification, examination and recording of assignments in the Plans appear in Article 5 of Appendix 30 and in Article 5 of Appendix 30A.

In reviewing these provisions, WP 10-11S noted that there were only minimum differences between the three Regions and between the provisions applying under Appendix 30 and Appendix 30A, and that these differences could in general be avoided by the elimination of superfluous provisions. Annex 2 provides the results of this review and shows the feasibility of having a unified procedure (Article T5) that could replace both Articles 5 of Appendices 30 and 30A without any change of substance.

Apart from Resolution 42, the main difference concerns the examination of notices by the BR and in particular the examination with respect to the conformity with the Plan taking into consideration that the existing provisions provide for parameters to be less than those of the Plan (par. 5.2.1.c of App. 30). 

�It is considered that with the RRB’s existing Rules, the provisions under T.5.10.b does provide for this examination and recording when the parameters are within the envelope of the Plan assignment, with the understanding that such usage is recorded in the MIFR and not the Plan, therefore this usage is not protected from subsequent modifications to the Plan.

In reviewing the existing procedures for notification, examination and recording, the following difficulty was identified, which may warrant further consideration by the Special Committee.

In the existing Regulations, there is a problem in that if the agreement with an administration is still lacking, the notice will continue to be returned by the Bureau to the notifying administration until all of the agreements are obtained. This results in one administration having the ability to block a new entry into the Plan, even in the absence of valid technical reasons. One of the important conclusions of the VGE is that there should be no procedure leading to a dead end and this situation should therefore be addressed.

The VGE in its proposals recognized this difficulty and suggested a provision similar to the technical examination under No. 1506, however, WRC-95 had some difficulty in accepting such technical examinations. It should be recognized however that under Article 7 of Appendix 30, such technical examination of the probability of harmful interference into planned assignments is conducted by the Board for FSS assignments.

Alternatively, under Art. 14 (and associated Rules of Procedure) and under Art. 11 for unplanned space systems, there are provisions which permit the operation of such systems that have not concluded all of the necessary agreements to be operated and recorded in the MIFR with the obligation of protecting those assignments which led to this situation. 

These two options need to be studied.

4	Procedures applicable to unplanned services in respect of the Plans

Existing procedures in the Radio Regulations currently applicable to other than the planned services, in relation to the Plans, have been reviewed and compared, together with the experience gained in their application. These procedures encompass Articles 6 and 7 of Appendices 30 and 30A, as well as Articles 11, 13 and 14. As a result of the work of the VGE and the decisions of WRC-95, the simplified procedures intended to apply in the frequency bands of Appendices 30 and 30A have also been reviewed. On this basis, the following conclusions have been reached.

In studying the provisions that would be applied in the case of Approach 2, as described in section 1, it was concluded that the provisions appearing in WRC-95 Final Acts under Article S9, Article S11 and Appendix S5, adequately cover, in general, the scope of the existing procedures. The need for some adjustments, however, has been identified, in order to solve some inconsistencies related to the relationship between Article S9 and Appendices 30 and 30A. It should be noted that similar inconsistencies would be have to be resolved under Approach 1, but have not been investigated.

Annex 3 attempts to identify, in the case of Approach 2, the areas within Article S9 and Appendix S5 for which potential inconsistencies could exist in relation to Appendices S30 and S30A, and to propose modifications to correct these inconsistencies, with the objective that the procedures and associated technical criteria currently applicable to unplanned services in relation to planned services in the frequency bands covered by Appendices 30 and 30A be properly reflected in �Article S9 and Appendix S5. The technical criteria appearing under Appendix S5 to determine the need for coordination with assignments in the Plans are identical to those currently applicable. Proposals for modifications of these criteria are addressed in section 10 of this report.

The modifications under Annex 3 are also intended to avoid duplications in the procedures and to cover sharing situations which were not identified at the time Appendices 30 and 30A have been developed or revised, or which have emerged as a result of subsequent conferences. Those situations are as follows:

–	coordination of unplanned BSS in Region 3 (12.5 - 12.7 GHz) in respect of Region 2 Appendix 30 Plan;

–	coordination of unplanned BSS feeder links and terrestrial services in Region 2 (17.7 � 18.1 GHz) in respect of the Regions 1 and 3 feeder link Appendix 30A Plan;

–	coordination of unplanned BSS in Region 2 (17.3 - 17.8 GHz) in respect of Regions 1 and 3 feeder link Appendix 30A Plan;

–	coordination of FSS (Earth-to-space) in Region 1 (12.5 - 12.75 GHz) in respect of Region 2 Appendix 30 Plan.

More fundamental difficulties have been found in reviewing the current procedures, which relate to differences in the principles applied in Region 1 and 3 and in Region 2 concerning the relationship between the Plans and the other services, and the problems that the application of these procedures, either in their current form or in the simplified form provided in Annex 3, would raise in the case of subregional systems. These difficulties are reported in the following paragraphs and require joint consideration of all the applicable procedures.

4.1	Treatment of feeder-link antennas under Appendix 30A

Coordination of other services sharing the band 17.7 - 18.1 GHz with feeder link stations in Appendix 30A Plans leads to a different treatment in Region 2, where only specific feeder link stations can be considered in modifying the Plan, and in Regions 1 and 3, where typical stations can be considered, but where in practice, agreement to modifications of the Plan is only granted after a detailed coordination, i.e. on a case-by-case basis. In both cases, it seems that in practice, the end result of Appendix 30A procedures in sharing these bands with other services is equivalent to the application of Article 11, which was the approach followed for Appendix 30B under similar circumstances.

For these reasons, the SC-3 Rapporteur suggests that the same approach as followed under Appendix 30B be taken, which equates to the suppression of Article 6 of Appendix 30A and suppression of Section 7.2 of Article 7 of Appendix 30A, which would be covered under S9.17bis, with no specific change required to it for this purpose. 

4.2	Equitable access to orbit/spectrum resources for BSS and FSS

Article 7 of Appendix 30 contains provisions for the coordination of new FSS assignments in the bands shared with Appendix 30 Plan with only the assignments in conformity with this Plan. Article 4 of Appendix 30 (par. 4.3.1.5) provides for protection of an FSS network against BSS modifications as soon as Appendix S4 information is received, whereas protection of a BSS modification under Article 7 of Appendix 30 is only granted vis-à-vis FSS once the modification has been included in the Plan, which could be years after Appendix S4 information has been received by the Bureau.

�Establishing a more equitable access to the orbit/spectrum resources in all regions would be facilitated by protecting modified assignments in the Plans against other space services from the date of receipt of Appendix S4 information. This could be done by inserting the following sentence in section 1 of Appendix S5, which identifies the assignments with which coordination is required:

ebis)	for which the procedure of Article [T4] has been initiated from the date of receipt by the Bureau, in accordance with [T4.8], of the basic characteristics as specified in Appendix S4; or

Such an approach would also avoid the occurrence of the following situation as a result of the application of the current procedures:

–	Administration A in Region 1 successfully modifies the Plan by applying Article 4. Administration B in Region 3, which has plans for an FSS Network, is not identified as affected because it has not sent Appendix 3 information at the time where Administration A Annex 2 information is received by the Bureau.

–	Before the Article 4 Procedure is finalized by Administration A, Administration B initiates Article 7 procedure. The administrations identified as potentially affected are those with assignments in conformity with the Plan, hence Administration A is not identified.

–	Since none of the two administrations is aware of the other's plans, and no subsequent examination is done by the Bureau at the notification stage which could identify this new situation, the assignments of both administrations are recorded in the MIFR. In case of harmful interference, there is no way to decide which administration should cease the interference.

It should also be noted that, in a similar sharing situation between FSS in Region 2 and BSS in Regions 1 and 3, application of Article 14 in the 11.7 - 12.2 GHz band in Region 2 FSS is putting FSS and BSS on a more equitable procedural footing. Recognizing that both services may need different protection criteria, this situation confirms that the RoP extension of the provisions of Annex 1 of Appendix 30 from the case of FSS in Region 2 (where Article 14 applies) to the case of FSS in Region 3 (where Article 14 does not apply), needs to be reviewed (see section 6.1.3).

4.3	Interaction between the various provisions applicable to the services sharing the bands of Appendix 30 and Appendix 30A and consequences of this situation on subregional systems

4.3.1	Interaction between the provisions applicable to the services sharing Appendix 30 bands

Article 4 of Appendix 30 foresees the need for coordination of a modification to the Plan with the services of other administrations only to the extent that interference could be caused to these services by the proposed modification. In that sense, the possibility for a modification to the Plan to be protected from other services sharing the same band does not result from the application of the procedure.

The reason for this situation can be found in Footnote RR 838 for Regions 1 and 3, as well as in RR 844 for Region 2. These footnotes provide a super-primary status to the Broadcasting-Satellite Service over the other primary services sharing the same band (FSS, BS, FS), in that these services shall not cause harmful interference to broadcasting-satellite stations operating in accordance with the provisions of Appendix 30 (Orb-85). This footnote is also the subject of the following Rule of �Procedure, which concludes that, if, despite the application of the procedures of Appendix 30, harmful interference is actually caused to a broadcasting-satellite station, the station in the other service shall cease this interference.

This conclusion means that, although Articles 6 and 7 of Appendix 30 ensure that the unplanned services can be implemented without causing harmful interference into the assignments of the Plans, successful application of these two procedures does bring any rights. 

One of the achievements of WARC-77 was to allow for sharing of the band between BSS and terrestrial services by generally allocating five channels to each country in a way that left a band of 400 MHz available for terrestrial services. With the capability of current technology to operate satellite systems over the full 800 MHz band on the same service area from the same orbital location, application of Article 4 without the need for coordination with terrestrial services (because Annex 1 pfd levels are not exceeded) now appears generally feasible. This could lead to situations where terrestrial stations coordinated with one administration and notified under Article 6 of Appendix 30 need to stop transmission because that administration has successfully applied Article 4 for a higher number of channels without the need to coordinate with terrestrial services. A similar situation could arise in the case of the FSS, where a genuinely coordinated and notified space station would have to cease transmission. This gives concerns as to the respect of the rights of administrations to operate other than planned services within their own territory, even in the absence of interference to stations located in the territory of other administrations.

A similar situation could occur as a result of one administration successfully applying Article 4 to extend its service area without exceeding Annex 1 pfd limits. A terrestrial station or an FSS space station might in that case have to cease transmission, even after having objected to the publication on the basis of RR 2674.

Another anomaly in the application of the current procedures arises from the use of Article 6 and 7 of Appendix 30 for a regional service area. It should be recognized that the current Article 6 and 7 of Appendices 30 and 30A have been drafted by considering only situations where a BSS space station and the corresponding BSS earth stations are the responsibility of the same administration. With the increased use of subregional networks, this situation will no longer be prevailing and could cause difficulties when an administration has coordinated the use of the FS, BS or FSS with the national BSS assignments of a neighbouring country, but this country wants to use other BSS assignments associated with a supra-national system.

Under the current drafting of Article 6 of Appendix 30, if the administration responsible for the BSS space station (Administration A) is assumed to have successfully carried out Article 4 procedure with a supra-national service area, it will be contacted by Administration C (responsible for a terrestrial transmit station) to coordinate the protection of a receive BSS station on the territory of a third Administration (B). Under normal circumstances, such coordination between a terrestrial station and a receive earth station should be left to the two administrations on the territory of which the two stations are located.

In this instance, while it is recognized that RR 838 aims to protect BSS reception from harmful interference caused by other administration's radio services, particularly in border areas, extending its scope to afford protection of one administration's BSS transmissions when received in the territory of another administration might be arguable.

�4.3.2	Interaction between the provisions applicable to the services sharing Appendix 30A bands

An anomaly in the application of the current procedures arises from the use of Article 6 and 7 of Appendix 30A for a regional service area. It should be recognized that the current Article 6 and 7 of Appendices 30 and 30A have been drafted by considering only situations where a BSS feeder link space station and the corresponding feeder link transmit earth stations are the responsibility of the same administration. With the increased use of subregional networks, this situation will no longer be prevailing and could cause difficulties.

Under the current drafting of Article 6 of Appendix 30A, if the administration responsible for the BSS feeder link space station (Administration A) is assumed to have successfully carried out Article 4 procedure with a supra-national service area, it will contact Administration C (responsible for a terrestrial receive station) to coordinate the operation of a transmit feeder link station on the territory of a third Administration (B). Under normal circumstances, such coordination between a terrestrial station and a transmit earth station should be left to the two administrations on the territory of which the two stations are located.

4.3.3	Possible solution

A possible solution to these difficulties would be:

–	for Appendix 30, limitation of the scope of RR 838 and 844 to those assignments which are part of the original Plan, provided that the establishment of the "original" Plan has adequately taken into account the coordination with other services;

–	concerning Appendix 30A, the suggestion made in section 4.1 above would resolve the difficulties.

5	Studies concerning the sharing criteria 

5.1	Sharing within the BSS in Regions 1 and 3

The adoption of the Overall Equivalent Protection Margin (OEPM) method for the establishment of the Plan has been generally supported and adopted by WRC-95 in Resolution 531 to carry out planning exercises, on the assumption that it would offer more flexibility in establishing the revised Plans, by balancing the up and downlink margins.

This decision leaves open the question of using, or not, the same approach in the criteria selected to protect the assignments of the Plan in the modification procedures.

This issue needs to be considered in the light of the following elements:

–	There is currently no firm linkage between the 17.3 - 18.1 GHz band, the 14.5 - 14.8 GHz band on one side and the 11.7 - 12.5 GHz band on the other side. If such a linkage was introduced, it would lead to less flexibility in using these bands independently, especially in Region 3, where 1 100 MHz of uplink bands would be rigidly associated with only 500 MHz of downlink bands, therefore also leading to the waste very large amounts of valuable spectrum.

–	This waste would be further increased by the fact that the 17.3 - 17.7 GHz is a worldwide primary allocation reserved exclusively to BSS feeder links.

�–	Combining the up and downlink EPM into OEPM to serve as a basis for determination of affected administrations under Article 4 is generally considered as providing increased flexibility to accept interference on the overall link. 

–	In some cases, a given uplink may be used for distribution to several downlinks, either from one or several space stations, possibly through the use of inter-satellite links.

In view of the above, it is concluded that the use of the OEPM method offers significant advantages in terms of facilitating the establishment of the Plan and could be envisaged as the basis to protect the Plan from subsequent modifications once it has been established. This method however, should be reviewed in order to ensure that it does not introduce a firm linkage between Appendices 30 and 30A Plans, which could be detrimental from the point of view of both the preservation of the orbit/spectrum resources and the flexibility of these Plans. It is noted in this instance that the only linkage existing at present in Regions 1 and 3 Appendices 30 and 30A Plans appears in the title of Appendix 30A, which would therefore need to be reviewed.

5.2	Sharing with other services

The sharing criteria to protect the other services are a critical element in ensuring the flexibility of the Plans. Concerning the sharing criteria between the BSS and the FSS, WP 10-11S sent a liaison statement to WP 4A in order to adequately and equitably protect both services sharing the same orbit/spectrum resources.

6	Other regulatory/procedural issues 

Under Resolution 531, WRC-95 requested the ITU-R to conduct further studies in relation to the regulatory/procedural aspects of WRC-97 agenda item 1.10. Some of these aspects, as well as other regulatory/procedural provisions have not been covered in the previous sections and are reported hereafter.

6.1	Miscellaneous provisions in the Radio Regulations

Significant difficulties have been identified and are reported in section 5 on the exact interpretation to be given to RR 838 in the framework of modifications to the Plan including supra-national coverage or additional capacity. These difficulties should be resolved.

RR 846/S5.492 could be extended to all three Regions.

The reference to the application of Article 7 of Appendix 30 within RR 845/S5.491 could be suppressed.

The reference to the application of Resolution 34 within RR 847/S5.493 could be suppressed, since such a provision could be included in Article S9 (as proposed in Annex 9.1).

A similar treatment could be given to RR 868A/S5.515 which indicates that, in the band 17.3 � 17.8 GHz, sharing between the FSS (Earth-to-space) and the BSS shall also be in accordance with the provisions of section 1 of Annex 4 of Appendix 30A. 

Under RR 869/S5.516, the procedures applicable to BSS feeder links in the corresponding bands should be clarified.

�Article S23 (S23.23/RR 2674)

S23.23/RR 2674 is a general guidance to use available technical means in order to limit radiations (which is different from the concept of service area). WRC-95 provided guidelines to develop a Rule of Procedure on this provision. The scope of this Rule, which was adopted in 1996 by the RRB, may need to be clarified in relation to the other services sharing the same bands.

Resolutions

Provided satisfactory conclusion of WRC-97 on Agenda Item 1.10, as well as other agenda items involving BSS, consideration could be given to the suppression of Resolutions 507, 519, 524 and 531, as well as Resolutions 32, 33 and 34. 

Resolution 506, which limits the use of the bands covered by the Plans of Appendices 30 and 30A (from 11.7 to 12.7 GHz) to space stations operated in the geostationary-satellite orbit, could also be reviewed, in the light of the emergence of NGSO systems capable of effectively sharing the spectrum/orbit resources with GSO systems ("Quasi-Geo", systems operating under RR 2613).

6.2	Suppression of superfluous provisions in Appendices 30 and 30A

Appendices 30 and 30A contain several Articles for which deletion could be considered:

–	Article 2 of Appendices 30 and 30A restate the scope of the procedures covered by these Appendices. Depending on which approach is selected in the relation to the simplified procedures, these Articles may be modified or suppressed.

–	Article 8 of Appendix 30 and Article 8 of Appendix 30A contain miscellaneous provisions which can also be found in the simplified procedures (Article S13).

–	Article 9 of Appendix 30 mention absolute pfd limits applicable to protect terrestrial services in Regions 1 and 3 against modifications of Region 2 Plan. These provisions are in contradiction with those of Article 4 and Annex 1, which foresees coordination when these levels are exceeded.

–	Article 12 of Appendix 30 indicates that the Plan are in accordance with Resolution 507. There have been proposals under the review of Resolution 18 (Kyoto-94) to suppress this resolution. If WRC-97 was to take such an approach, Article 12 would need to be suppressed.

–	Article 13 of Appendix 30 and Article 10 of Appendix 30A contain provision on interference that also appear in S7.8.

6.3	Procedures for subregional/interim systems

WP 10-11S studies have shown that the procedures of Resolution 42 or the procedures for the introduction of a subregional system under Appendix 30B do not offer the same advantages as the combined application of existing provisions under Appendix 30/30A procedures (group concept, time limited agreements) and the adoption of a new provision allowing an administration to act on behalf of a group of administrations. In addition, such an approach would provide protection for interim systems, whereas Resolution 42 does not provide such protection against subsequent modifications of the Plans.

It is therefore proposed to suppress Resolution 519, which calls for the development of specific provisions for interim systems for Regions 1 and 3.

�6.4	Alignment of Appendices 30 and 30A

There are inclusions in the Articles of Appendix 30A adopted at WARC-88 which differ from those in Appendix 30. WRC-95 stated that it would be desirable for these to be brought into alignment as far as possible. Annexes 1 and 2 provide possible examples of such alignments.

6.5	Relationship with Appendix 30B

Although WRC-95 concluded that the planning exercises for the revision of Appendices 30/30A Plans would be complicated by attempting to align the orbital positions with those of Appendix 30B, the procedures should have enough flexibility to enable this alignment when it is needed.

Removing some of the provisions of Annex 7 of Appendix 30 and Section 3.15 of Annex 3 of Appendix 30A would facilitate such alignment.

6.6	Use of assignments in the BSS Plan for the fixed-satellite service

In Region 2, S5.492 (RR 846) provides that "assignments to stations of the broadcasting-satellite service in the Plan for Region 2 contained in Appendix S30 may also be used for transmissions in the fixed-satellite service (space-to-Earth), provided that such transmissions do not cause more interference or require more protection from interference than the broadcasting-satellite service transmissions operating in conformity with the Region 2 Plan."

Extension of this provision to Regions 1 and 3 would be in line with the principle set out by WRC�95 in Resolution 531 (section 2.2.3). This extension should provide for interregional plan protection, which is not the case in S5.492.

Since no protection from or interference into the Plan is foreseen, the use of BSS assignments under this provision would be through the notification process. WP 10-11S has developed a draft new Recommendation [Document 10-11S/TEMP/61] to enable the Bureau to determine conformity with S5.492 in such an instance, hence the regulatory implementation of this concept has been completed.

In the procedures, this implementation is currently done at the notification stage, by examination of the conformity with the provisions of RR 846, under par. 5.2.1 c) of Article 5 of Appendix 30.

A parallel provision may be necessary to allow the non-feeder link usage of the band by suitably modifying S5.510 (RR 863) and S5.516 (RR 869). However, since the use of the BSS downlink under RR 846 is recognized by a Rule of Procedure as still being under a BSS allocation, the corresponding uplink use in Appendix 30A bands would in principle still be considered as a feeder link to BSS, hence be regarded as a transmission in conformity with the feeder-link Plan as long as it does not produce more interference that the feeder link assignment on which it is based, rather than as a transmission not in conformity with the Table of Frequency Allocations. If such an interpretation was correct, there would be no need for an additional footnote under Article S5 for this purpose.

6.7	Coexistence of analogue and digital systems

The protection ratios selected for the planning exercises and endorsed by WRC-95 have taken into account the possibility of digital assignments in the Plan and the possible coexistence of such assignments with analogue ones. It is expected that, if these protection ratios are kept in the process �of modifying the Plans, together with those provisionally adopted by WP 10-11S for the feeder-link Plan, coexistence of such systems and adequate flexibility of the Plan with regard to accommodating digital systems will be ensured, with no need for specific procedural developments. 

In particular, the conditions for the conversion of an analogue assignment in conformity with the Plan to a digital assignment should explicitly appear under the modification procedures (see Appendix S6).

6.8	Very low EPM in the Plan

The sensitivity to interference, in terms of being identified as affected, by networks submitted to the Bureau decreases when these networks have a very low equivalent protection margins (EPM). In these cases, if, due to the above-mentioned phenomenon they are not identified as affected, the administrations concerned should be aware of this situation and must take necessary action, as appropriate. This conclusion has been included in the Rules of Procedure.

WP 10-11S considered that occurrence of such situations is inherent to the EPM (or OEPM) method, but could not identify a protection criterion which would offer advantages over this criterion (see section 2.3). WP 10-11S also noted that, when such situations arise, it would generally be in the interest of both parties involved to resolve the situation, since the assignment causing this very low EPM to occur would generally also receive very high interference from the affected assignment.

7.	Transitional Aspects Relating to the Implementation of the Decisions of WRC-97

WP 10-11/S recognized that there would be a need for certain transitional actions required after the WRC-97. On the assumption that modifications to the Regions 1 and 3 Plans are adopted by WRC 97, involving new criteria and procedures and that many pending proposed modifications (Step 3) to these Plans will not be taken into account by WRC-97, this conference may wish to consider and include in its decisions the following: 

a)	in order to provide for the early implementation of the WRC-97 decisions, that the new procedures and technical criteria of the Appendices be used by BR and administrations as of the end of the conference;

b)	that the procedure to be followed by the Bureau and administrations to deal with the outstanding modifications be as follows:

•	BR should use the updated Plans and the new technical criteria to calculate the new reference situation for the Regions 1/3 Plans as of the end of the conference;

•	BR should treat all pending modifications to the Plans of App. S30 and App. S30A in the same date order that they were received by BR and using the new technical criteria and reference situation, identify for each pending modification the list of administrations for which the agreement is required and to publish this list of affected administrations;

•	within 4 months from the date of the above publication possibly affected administrations should provide comments to BR and the notifying administration shall indicate any agreements which have been obtained previously and any new agreements;

•	in those cases where the degradation of the margins (EPM or OEPM) caused by the proposed modification is no worse under the new situation arising from the revision of the Plan than what it was under the original situation, any agreements previously obtained under the Art. 4 procedures of App. 30 or App. 30A should continue to be valid as long as the orbital positions remain the same.

�In addition, taking into consideration that there may be some changes to the regulatory procedures for the unplanned bands resulting from the review under Resolution 18 (Kyoto-94) that could also be applied to modifications to the Plans of Appendices 30/30A, it might be appropriate for some of these changes to be implemented during the transitional period such as:

•	the time period to bring the modifications into use of [4-5] years plus a possible extension of [2-3] years should continue to be counted as from the date of the receipt by the Bureau of the complete information on the request for modification, except that the minimum period from the end of the conference to bring the assignments into use should be no less than [one] year;

•	any modifications involving new frequencies and/or orbit positions which have not been brought into service within this [4-5 + 2-3] year period should be cancelled by the Bureau BR after informing the notifying administration.

It is also expected that WRC-97 will add new assignments in the Plans or modify existing assignments. This process will require verification that these do not affect and are not affected by existing or planned services sharing the same bands.

Specific provisions may therefore be required to establish the rights of these new or modified assignments in the Plan in respect of these services, such as those appearing in section 5.1.4 of Appendix 30A.

It has also been noted that in the current process of Article 4 of Appendix 30, there is no provision for proposed modifications to the Plan to seek protection from other services.

8	Impact on Region 2 Plan 

Of the three approaches identified under Section 1, Approach 2 appeared to the be the least controversial. Under this approach, the Plan modification procedures could be located either within Appendices 30 and 30A, or in a self-contained Article addressing specifically the procedures for modification of these Plans (T4), and for notification, examination and recording (T5) of assignments in these Plans, together with their associated criteria. 

This approach would not per se require any change of substance. However, WP 10-11S studies have identified certain problems in the general process of interaction between the various regulatory/procedural provisions relating to Appendix 30 and 30A, which are common to all three Regions. The solutions that could be identified to resolve these problems may also be beneficial to Region 2, and Region 2 administrations are encourage to participate in the discussions under this WRC-97 agenda item in order to find ways of worldwide harmonization in this area.

Given that these solutions are not fully identified at this stage, the impact, if any, on the Region 2 Planned and unplanned services due to Regions 1 and 3 procedural modifications can not be determined at this time This impact will need to be determined once suitable solutions have been fully identified. 

�9	Conclusions

Since 1993, WP 10-11S conducted extensive studies in the area of the regulatory/procedural aspects associated with the revision of Appendices 30 and 30A in Regions 1 and 3. Although a number of solutions have been identified that would have the potential to significantly simplify the current procedures and increase the flexibility of Appendices 30 and 30A Plans in Regions 1 and 3 to accommodate future requirements, these studies also identified significant difficulties relating to differences in the principles applied in Region 1 and 3 and in Region 2 concerning the relationship between the Plans and the other services, and to the problems that the application of these procedures, either in their current form or in a simplified form, would raise in the case of subregional systems. 

As a consequence of the close interaction between the frequency allocations to planned and unplanned services in the three Regions in the bands covered by Appendices 30 and 30A, resolution of these difficulties will require significant efforts by administrations in all three Regions prior to WRC-97.

�ANNEX 1

Example of a possible simplification of Article 4 of Appendix 30 and Article 4 of Appendix 30A into a single provision for modifying Appendices S30 and S30A Plans

Article T4 and Appendix S6

This annex provides a possible text for a modification procedure (Article T4) and its associated sharing criteria (Appendix S6), based on the current modification procedures of Appendices 30 and 30A Plans and, in line with Recommendation 35 on the consideration of Article T10

This text of Article T4 is presented in the form of amendments to the text of Article T10, in parallel with the current text of Articles 4 of Appendices 30 and 30A, to allow for a direct comparison between the current procedures and their possible simplification. In order to facilitate this comparison, the order of the current Appendices 30/30A procedures has been modified in some instances, which would not modify the scope of these procedures. When the text of Article T10 is modified, for reasons other than the need for alignment with the current text of Articles 4 of Appendices 30/30A, the justifications for the modification are provided in the same box.

The text of Appendix S6 is presented in the form of amendments to the current text of Articles 4 of Appendices 30 and 30A, with the corresponding justifications. This Appendix is used to identify the administrations whose services might be affected by the proposed modification.

The sharing criteria currently applicable in Appendices 30 and 30A bands are consolidated into a single Annex to Appendix S6.

Article T4 and Appendix S6 could be located either within Appendices 30 and 30A or in a separate Article of the Radio Regulations (e.g. Article S10) applying only to Appendices 30 and 30A.



 �ARTICLE T4 T10 

(ANNEX to Recommendation 35)

Possible Procedure for the Modification of the a Frequency Allotment or Assignment Plans contained in 

Appendices S30 and S30A

T10.1

	For the frequency allotment or assignment Plans contained in Appendices to these Regulations, the Bureau shall maintain the master copies of the Plans, incorporating any agreed modifications, and shall provide such copies in an appropriate form for publication by the Secretary-General when justified by circumstances.



( Reason: Covered under S13.17 bis, which could be modified as follows to cover the specific requirement to contain the margins of the plans, for the appropriate plans :



S13.17bis

		The Bureau shall maintain master copies of all world frequency allotment or assignment plans contained in appendices to these Regulations, or adopted by world conferences convened by the Union, including, where appropriate, the margins associated with each assignment or allotment, and incorporating any agreed modifications resulting from the successful application of the relevant modification procedure, and shall provide such copies in an appropriate form for publication by the Secretary-General when justified by circumstances.

�



ARTICLE 4 OF APPENDIX 30



Procedure for Modifications to the Plans











4.5	Master copy of the Plans



4.5.1	a)	The Board shall maintain an up-to-date master copy of the Regions 1 and 3 Plan taking account of the application of the procedure specified in this Article. The Board shall prepare a document listing the amendments to be made to the Plan as a result of modifications made in accordance with the procedure in this Article.



	b)	The Board shall maintain an up-to-date master copy of the Region 2 Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure specified in this Article. This master copy shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference and those derived from all modifications to the Plan as a result of the successful completion of the modification procedure described in this Article. The Board shall prepare a document listing the amendments to be made to the Plan as a result of modifications made in accordance with the procedure described in this Article.



4.5.2	The Secretary-General shall be informed by the Board of any modifications made to the Regional Plans and shall publish an up-to-date version of those Plans in an appropriate form when justified by the circumstances.�



ARTICLE 4 OF APPENDIX 30A



Procedure for Modifications to the Plans





4.4	Master copies of the Plans



4.4.1	The Board shall maintain up-to-date master copies of the Plans as well as master copies of the margin reports, including for each assignment the overall equivalent protection margins in respect of Region 2 and the feeder-link equivalent protection margins and the overall equivalent protection margins in respect of Regions 1 and 3, taking account of the application of the procedure specified in this Article. Each master copy of the margin reports shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference in the case of Region 2 and the feeder-link equivalent protection margins and the overall equivalent protection margins for the 1988 Conference in the case of Regions 1 and 3 and those derived from all modifications to the Plans as a result of the successful completion of the modification procedure of this Article.









4.4.2	The Secretary-General shall be informed by the Board of any modifications made to the Regional Plans and shall publish up-to-date versions of the Plans in an appropriate form when justified by the circumstances.

���

T4T10.2

	Before notifying any assignment which is subject to the Plans of Appendix S30 or Appendix S30A, a plan the an administration shall ensure that it is in conformity with the corresponding Plan.1, 2  If the assignment is not in conformity, the administration shall apply the procedure2 of this Article to effect an appropriate modification to the Plan by seeking the agreement of those e administrations whose services , which are identified in accordance with Appendix S6, as having planned allotments or assignments which may might be affected by the proposed modification.�4.1	When an administration intends to make a modification2 to one of the Regional Plans,...., the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix);







�4.1	When an administration intends to make a modification to one of the Regional Plans, ....,the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix and Resolution 42 (Rev.Orb-88)).



��T4T10.2.1

	1		An assignment is subject to a plan when it is for a station in a radiocommunication service and in a frequency band and in a geographical area covered by for which the procedures for modification of this Plan or for notification, examination and recording of assignments in this Pplan apply a plan. An assignment is in conformity with a the Plan, if it appears in this the Plan, or corresponds to an allotment in the Plan, or if the procedure for modification of the this Plan has been successfully applied to it.

( Reason: see Article 2 of Appendices 30 and 30A�





4.2	The term “frequency assignment in conformity with the Plan” used in this and the following Articles is defined in Article 1.

[Article 1, paragraph 1.6	Frequency assignment in conformity with the Plan: Any frequency assignment which appears in the Regions 1 and 3 Plan or the Region 2 Plan or for which the procedure of Article 4 of this Appendix has been successfully applied.]����T10.2.2

	2	Where an existing Plan contains a supplementary or alternative procedure, that procedure shall continue to be applied.

Reason: the assumption is that Articles 4, 5, 6 and 7 of Appendices 30 and 30A will be replaced by Articles S9, S10 (i.e. T4 and T5), S11 and related Appendices. Hence, no supplementary or alternative procedure would appear in the Plan.

T4.2.2

See also Article T5 and Resolution 42.�����

T4T10.3

	A proposed modification to a plan may consist of:

T4T10.4

	a)	a change in the characteristics of an frequency assignment entry in the Plan; or

T4T10.5

	b)	the inclusion of a new entry frequency assignment in the Plan; or

T4T10.6

	c)	the cancellation of an entry a frequency assignment in the Plan.�4.1	When an administration intends to make a modification2 to one of the Regional Plans, i.e. either:

	a)	to modify the characteristics of any of its frequency assignments to a space station3 in the broadcasting-satellite service which are shown in the appropriate Regional Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

	b)	to include in the appropriate Regional Plan a new frequency assignment to a space station in the broadcasting-satellite service; or

	c)	to cancel a frequency assignment to a space station in the broadcasting-satellite service;

the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix);





2 The intention not to employ energy dispersal in accordance with paragraph 3.18 of Annex 5 shall be treated as a modification and thus subject to the appropriate provisions of this Article.





3  The expression “frequency assignment to a space station”, wherever it appears in this Article, shall be understood to refer to a frequency assignment associated with a given orbital position. See also Annex 7 and Resolution 43 (Orb-85) for the orbital limitations�4.1	When an administration intends to make a modification to one of the Regional Plans, i.e. either:

	a)	to modify the characteristics of any of its frequency assignments in the fixed-satellite service which are shown in the appropriate Regional Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

	b)	to include in the Plan a new frequency assignment in the fixed-satellite service; or

	c)	to cancel a frequency assignment in the fixed-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix and Resolution 42 (Rev.Orb-88)).







���

T4T10.7	Before an administration proposes to include in the Plan under the provisions of T4T10.5, a new frequency assignment to a space station or to include in the Plan new frequency assignments to a space station whose orbital position is not designated in the Plan for this administration, all the assignments to the service area involved should normally have been brought into service or have been notified to the Bureau in accordance with the relevant provisions of the Plan. Should this not be the case, the administration concerned shall inform the Bureau of the reasons therefor.



T4.7.bis

In proposing a modification to a plan, an administration may act on behalf of a group of named administrations, in which case all members of the group retain the right to respond in respect of their own services and assignments which could be affected by the proposed modification.



Reason: Facilitate the establishment of sub-regional systems (text taken from S9.6.1)�4.1.1	Before an administration proposes to include in the Region 2 Plan under the provisions of paragraph 4.1 b), a new frequency assignment to a space station or to include in the Plan new frequency assignments to a space station whose orbital position is not designated in the Plan for this administration, all of the assignments to the service area involved should normally have been brought into service or have been notified to the Board in accordance with Article 5 of this Appendix. Should this not be the case, the administration concerned shall inform the Board of the reasons therefor.

�4.1.1	Before an administration proposes to include in the Plan under the provisions of paragraph 4.1 b) a new frequency assignment for reception at a space station1 or to include in the Plan a new frequency assignment for reception at a space station whose orbital position is not designated in the Plan to that administration, all of the assignments to the service areas involved should normally have been brought into service or have been notified to the Board in accordance with Article 5 of this Appendix. Should this not be the case, the administration concerned shall inform the Board of the reasons thereof.



1 The expression "frequency assignment for reception to a space station", wherever it appears in this Article, shall be understood to refer to a frequency assignment associated with a given orbital position.

���4.3	Proposed modifications to a frequency assignment in conformity with one of the Regional Plans or inclusion in that Plan of a new frequency assignment

�4.2	Proposed modifications to a frequency assignment in conformity with one of the Regional Plans or proposed inclusion in that Plan of a new frequency assignment.��Note: Existing provisions until par. 4.3.5 of Appendix 30/Par. 4.2.5 of Appendix 30A should appear under Appendix S6.�	For Regions 1 and 3:

4.3.1	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Regions 1 and 3 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

�	For Regions 1 and 3

4.2.1	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Regions 1 and 3 Plan or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations:

���

�4.3.1.1	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with the Regions 1 and 3 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

�4.2.1.1	of Regions 1 and 3 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, in the same orbital position or an adjacent orbital position in the range ± 12.5° which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; or

���4.3.1.2	of Region 2 having a frequency assignment to a space station in the broadcasting-satellite service with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Region 2 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

�4.2.1.2	having a frequency assignment in the band 17.7 - 18.1 GHz to an earth station in the fixed-satellite service (space-to-Earth), which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

����4.3.1.3	identified in accordance with resolves 2 of Resolution 43 (Orb-85); or

�4.2.1.3	having a frequency assignment in the bands 14.5 - 14.8 GHz or 17.7 - 18.1 GHz to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

���

�4.3.1.4	having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

�4.2.1.4	having an assignment for feeder links in the fixed-satellite service (Earth-to-space) with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Region 2 feeder-link Plan, or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article;

���4.3.1.5	having a frequency assignment in the band 11.7 - 12.2 GHz in Region 2 or 12.2 - 12.5 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations, or those of paragraph 7.2.1 of this Appendix;

����4.3.1.6	whose services are considered to be affected.

4.3.2	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

�4.2.1.5	which are considered affected.

4.2.1.6	The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

���

��4.2.2	The agreement referred to in paragraph 4.2.1 is not required when an administration proposes to bring into use, with characteristics1 appearing in the Plan, a fixed feeder-link earth station or a transportable feeder-link earth station in the bands 14.5 - 14.8 GHz or 17.3 - 18.1 GHz.

		1 The power to be taken into account is obtained by adding the values specified in columns 8 and 9 of the Plan.

���	For Region 2:

4.3.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

�	For Region 2

4.2.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations:

���4.3.3.1	of Region 2 having a frequency assignment in the Region 2 Plan to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with that Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

�4.2.3.1	of Region 2 having a feeder-link frequency assignment in the fixed- satellite service (Earth-to-space) in the same channel or an adjacent channel, which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; or

���

�4.3.3.2	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Regions 1 and 3 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

�4.2.3.2	having a frequency assignment in the band 17.7 - 17.8 GHz to an earth station in the fixed-satellite service (space-to-Earth), which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or���4.3.3.3	identified in accordance with resolves 1 of Resolution 43 (Orb-85); or

����4.3.3.4	having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

�	4.2.3.3	having a frequency assignment in the band 17.7 - 17.8 GHz to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

���

�4.3.3.5	having a frequency assignment in the band 12.5 - 12.7 GHz in Region 1 or 12.2 - 12.7 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations or those of paragraph 7.2.1 of this Appendix; or

�	4.2.3.4	having an assignment for feeder links in the fixed-satellite service (Earth-to-space) with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Regions 1 and 3 feeder-link Plan, or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article;

���4.3.3.6	having a frequency assignment to a space station in the broadcasting-satellite service in the band 12.5 - 12.7 GHz in Region 3 with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment and which

	a)	is recorded in the Master Register, or

	b)	has been coordinated or is being coordinated under the provisions of Resolution 33, or

	c)	appears in a Region 3 Plan to be adopted at a future administrative radio conference, taking account of modifications to that Plan which may be introduced in accordance with the Final Acts of the Conference;

����

4.3.3.7	whose services are considered to be affected.

4.3.4	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.�

	4.2.3.5	which are considered affected.

	4.2.3.6	The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

���

��	4.2.4	The agreement referred to in paragraph 4.2.3 is not required when an administration proposes to bring into use, with characteristics appearing in the Plan, a fixed feeder-link earth station in the band 17.3 - 17.8 GHz or a transportable feeder-link earth station in the band 17.3 - 17.7 GHz. Administrations may communicate to the Board the characteristics of such earth stations for inclusion in the Plan.

��

T4T10.8 	For the purpose of effecting a modification to a plan, the administration concerned shall, having regard to the relevant provisions associated with the Plan, not earlier than eight [4-5] years but preferably not later than eighteen months before the date on which the assignment is to be brought into use, send to the Bureau the relevant information listed in Appendix S4 of these regulations.. This action shall be taken within the time limits specified in the relevant appendix. An extension of [2-3] years may be approved by the Board under the same conditions as contained in [S11.xx].

Modifications to a Plan involving additions under paragraph T410.5 shall lapse if the assignment is not brought into use by that date, including any approved extension.

�	For all Regions:

4.3.5	An administration intending to modify characteristics in one of the Regional Plans shall send to the Board, not earlier than five years but preferably not later than eighteen months before the date on which the assignment is to be brought into use, the relevant information listed in Annex 2. Modifications to that Plan involving additions under paragraph 4.1 b) shall lapse if the assignment is not brought into use by that date.

�		For all Regions

	4.2.5	An administration intending to modify characteristics in one of the Regional Plans shall send to the Board, not earlier than eight years but preferably not later than eighteen months before the date on which the assignment is to be brought into use, the relevant information listed in Annex 2 to this Appendix.





����4.2.6	If an administration wishes to modify its assignments in the Plans contained in Appendix 30 (Orb-85) and in Appendix 30A (Orb-88), the eight-year period of paragraph 4.2.5 will be applicable in lieu of the five-year period specified in paragraph 4.3.5 of Appendix 30 (Orb-85).

���

�4.3.5.1	Where as a result of the intended modification the limits defined in Annex 1 are not exceeded, this fact shall be indicated when submitting to the Board the information required by paragraph 4.3.5. The Board shall then publish this information in a special section of its weekly circular.

�4.2.6.1	Where as a result of the intended modification the limits defined in Annex 1 to this Appendix are not exceeded, this fact shall be indicated when submitting to the Board the information required by paragraph 4.2.5. The Board shall then publish this information in a special section of its weekly circular.

���4.3.5.2	In all other cases the administration shall notify the Board of the names of the administrations whose agreement it considers should be sought in order to arrive at the agreement referred to in paragraph 4.3.1 or 4.3.3, as well as of those with which agreement has already been reached.

�4.2.6.2	In all other cases the administration shall notify the Board of the names of the administrations whose agreement it considers should be sought in order to arrive at the agreement referred to in paragraphs 4.2.1 and 4.2.3 as well as of those with which agreement has already been reached.

��T4T10.9	The Bureau, upon receiving the information under No. T4T10.8:

T4T10.10

	a)	determine in accordance with Appendix S6 the administrations whose allotments or assignments are considered to be affected;

T4T10.11

	b)	include their names in the publication information received under No. T4T10.812;

T4T10.12

	c)	publish the complete information in its Weekly Circular;



T4T10.13

	d)	promptly inform all administrations affected of its actions and the results of its calculations, drawing their attention to the relevant Weekly Circular.�4.3.6	The Board shall determine on the basis of Annex 1 the administrations whose frequency assignments are considered to be affected within the meaning of paragraph 4.3.1 or 4.3.3. The Board shall include the names of those administrations with the information received under paragraph 4.3.5.2 and shall publish the complete information in a special section of its weekly circular. The Board shall immediately send the results of its calculations to the administration proposing the modification to the appropriate Regional Plan.





.

�4.2.7	The Board shall determine on the basis of Annex 1 to this Appendix the administrations whose frequency assignments are considered to be affected within the meaning of paragraphs 4.2.1 and 4.2.3. The Board shall include the names of those administrations with the information received under paragraph 4.2.6.2 and shall publish the complete information in a special section of its weekly circular. The Board shall immediately send the results of its calculations to the administration proposing the modification to the Plan.

���

T4T10.13 bis



One month before the expiry of the period referred to in paragraph T4T10.17, the Bureau shall send a telegram to the administrations listed in the special section of the weekly circular drawing their attention to the matter.



( Reason: reflect current Rule of Procedure and current Article 4 text.�4.3.7	The Board shall send a telegram to the administrations listed in the special section of the weekly circular drawing their attention to the information it contains and shall send them the results of its calculations�4.2.8	The Board shall send a telegram to the administrations listed in the special section of the weekly circular drawing their attention to the information it contains and shall send them the results of its calculations.

��T4T10.14	Following receipt of the Weekly Circular, an administration believing that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Bureau to include its name. The Bureau shall study this request on the basis of Appendix S6 and the relevant rules of Procedure. In the event that the request to be included in the list of affected administrations is accepted by the Bureau, an addendum to the publication mentioned in T4T10.12 shall be published by the Bureau. Should the Bureau reach a negative conclusion, it shall inform the administrations concerned.�4.3.8	An administration which feels that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Board to include its name. The Board shall study this request on the basis of Annex 1 and shall send a copy of the request with an appropriate recommendation to the administration proposing the modification to the appropriate Regional Plan.

�4.2.9	An administration which feels that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Board to include its name. The Board shall study this request on the basis of Annex 1 to this Appendix and shall send a copy of the request with an appropriate recommendation to the administration proposing the modification to the Plan.

��

T4.14bis	Any modification to a frequency assignment which is in conformity with the appropriate Regional Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Appendix S6 shall be subject to the agreement of all administrations whose services are considered to be affected.

�

4.3.9	Any modification to a frequency assignment which is in conformity with the appropriate Regional Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.

�4.2.10	Any modification to a frequency assignment which is in conformity with the Plan or any inclusion in the Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 to this Appendix shall be subject to the agreement of all affected administrations.

	��T4T10.15	The administration seeking agreement and those with which it is sought, or the Bureau, may request any additional information they consider necessary. The Bureau shall be sent copies of any such requests and the replies.�4.3.10	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Board of such requests.

�4.2.11	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Board of such requests.

���



T4T10.16	Comments from administrations on the information published pursuant to T4T10.12 should be sent either directly to the administration proposing the modification or through the Bureau. In any event the Bureau shall be informed that comments have been made. The Bureau shall inform the administration proposing the modification of the comments that have been made.

T4T10.17 	An administration which has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the weekly circular referred to in T4T10.12 shall be understood to have agreed to the proposed modification. This time-limit may be extended by up to three months for an administration that has requested additional information under T4T10.15 or for an administration that has requested the assistance of the Bureau under T4T10.18. In the latter case the Bureau shall inform the administrations concerned of this request.�

4.3.11	Comments from administrations on the information published pursuant to paragraph 4.3.6 should be sent either directly to the administration proposing the modification or through the Board. In any event the Board shall be informed that comments have been made.





4.3.12	An administration that has not notified its comments either to the administration seeking agreement or to the Board within a period of four months following the date of the weekly circular referred to in paragraph 4.3.5.1 or 4.3.6 shall be understood to have agreed to the proposed assignment. This time limit may be extended by up to three months for an administration that has requested additional information under paragraph 4.3.10 or for an administration that has requested the assistance of the Board under paragraph 4.3.20. In the latter case the Board shall inform the administrations concerned of this request.

�4.2.12	Comments from administrations on the information published pursuant to paragraph 4.2.7 should be sent either directly to the administration proposing the modification or through the Board. In any event the Board shall be informed that comments have been made.



4.2.13	An administration which has not notified its comments either to the administration seeking agreement or to the Board, within a period of four months following the date of the weekly circular referred to in paragraph 4.2.6.1 or paragraph 4.2.7 shall be understood to have agreed to the proposed modification. This time-limit may be extended by up to three months for an administration which has requested additional information under paragraph 4.2.11 or for an administration which has requested the assistance of the Board under paragraph 4.2.21. In the latter case the Board shall inform the administrations concerned of this request.

���T4T10.18	Any administration involved in this procedure may request the assistance of the Bureau in accordance with S7.5.in seeking agreement:

T4T10.19

	a)	in applying any step of this procedure;

T4T10.20

	b)	in carrying out any technical study necessary for the application of this procedure.�4.3.20	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Board, particularly in seeking the agreement of another administration.

�4.2.21	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Board, particularly in seeking the agreement of another administration.

���

T4T10.21	If, following action by the Bureau in response to a request for assistance under No. T4T10.18 the Bureau receives no reply or decision within three months of its request for a decision in the matter from an administration whose agreement has been sought, the administration which requested the agreement shall be deemed to have fulfilled its obligations under this procedure. It shall also be deemed that the administration which did not give its decision has undertaken:

T4T10.22

	That no complaint will be made in respect of harmful interference affecting the services rendered by its stations which may be caused by the use of the assignment in conformity with the proposed modification to the Plan, and.



Reason: Avoid current rigidity of the Plans in cases where, after having made a comment to the proposed modification, an administration does not respond to the Bureau's request for specifying its decision. The text proposed under T4.21 and T4.22 is the same as in similar cases under Article 6 and 7 of Appendices 30 and 30A (e.g. 6.1.10 of Appendix 30).�����T4.22 bis 	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of T4.8 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal. If however, these modifications to the initial proposal do not lead to an increase in the interference level which would be caused to the services of another administration compared with that caused by the initial modification, further agreement from this administration is not required.

�4.3.13	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of paragraph 4.3.5 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

�4.2.14	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of paragraph 4.2.5 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

	

���

T4T10.23	If no comments have been received on the expiry of the periods specified in T4T10.17, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, or if the provisions of T4T10.21 have been applied, the administration proposing the modification shall inform the Bureau, indicating the final characteristics of the frequency assignment, together with the names of the administrations with which agreement has been reached.�4.3.14	If no comments have been received on the expiry of the periods specified in paragraph 4.3.12, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5 and shall inform the Board, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

�4.2.15	If no comments have been received on the expiry of the periods specified in paragraph 4.2.13, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5 of this Appendix and shall inform the Board, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

��T4T10.24	The Bureau shall publish in a special section of its weekly circular the information received under T4T10.23 together with the names of any administrations with which the provisions of this Article have been successfully applied. The Bureau shall then up-date the master copy of the Plan. The new or modified entry in the Plan shall then have the same status as others appearing in the Plan and shall be considered as being in conformity with the Plan.�4.3.17	The Board shall publish in a special section of its weekly circular the information received under paragraph 4.3.14 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the appropriate Regional Plan and will be considered as a frequency assignment in conformity with the Plan.

�4.2.18	The Board shall publish in a special section of its weekly circular the information received under paragraph 4.2.15 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Plan and will be considered as a frequency assignment in conformity with the Plan.����4.3.18	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement from an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

�4.2.19	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement from an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

	���

T10.25	The relevant provisions of the Plan shall be applied when frequency assignments are notified to the Bureau. �4.3.21	The relevant provisions of Article 5 of this Appendix shall be applied when frequency assignments are notified to the Board.

�4.2.22	The relevant provisions of Article 5 of this Appendix shall be applied when frequency assignments are notified to the Board.

��T4T10.25 bis



The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.�4.3.15	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

�4.2.16	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period.

��T10.26	If no agreement is reached between the administrations concerned the Bureau shall carry out any study that may be requested by those administrations. The Bureau shall inform them of the results and of any recommendations it may be able to offer for a solution of the problem.



( Reason: Already covered under T10.18.�4.3.19	If no agreement is reached between the administrations concerned, the Board shall carry out any study that may be requested by these administrations; the Board shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

�4.2.20	If no agreement is reached between the administrations concerned, the Board shall carry out any study that may be requested by these administrations; the Board shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

	���T4.26	Cancellation of frequency assignments

	When a frequency assignment in conformity with one of the Regional Plans is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a special section of its weekly circular and delete the assignment from the appropriate Regional Plan.

�4.4	Cancellation of frequency assignments

	When a frequency assignment in conformity with one of the Regional Plans is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Board. The Board shall publish this information in a special section of its weekly circular and delete the assignment from the appropriate Regional Plan.

�	4.3	Cancellation of frequency assignments

		When a frequency assignment in conformity with one of the Regional Plans is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Board. The Board shall publish this information in a special section of its weekly circular and delete the assignment from the Plan.

	��T4T10.27	When a proposed modification to a plan involves developing countries, administrations shall seek all practicable solutions conducive to the economic development of the radiocommunications broadcasting-satellite systems of those countries.�4.3.16	When the proposed modification to the appropriate Regional Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

�	4.2.17	When the proposed modification to the Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

���appendix s6

Identification of Administrations From Which Agreement Is To Be Sought�Under the Provisions of Article T4

1	Modification to Appendix S30 Plans

	For Regions 1 and 3:

4.3.1	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Regions 1 and 3 the appropriate regional Plan under Appendix S30Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

4.3.1.11.1	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with the Regions 1 and 3 the appropriate regional Plan, or in respect of which proposed modifications to that Plan have already been published by the Bureau Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 T4.12 of this Article; or

4.3.1.2	of Region 2 having a frequency assignment to a space station in the broadcasting-satellite service with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Region 2 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

• Reason: duplication with previous paragraph.

4.3.1.3	identified in accordance with resolves 2 of Resolution 43 (Orb-85); or

• Reason: Resolution 43 has been suppressed.

4.3.1.41.2	having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

4.3.1.51.3	having a frequency assignment in the band 11.7 - 12.2 GHz in Region 2 or 12.2 - 12.5 GHz in Region 3 to a space station in the fixed-satellite service or in the broadcasting satellite service where it is not subject to a Plan, with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, and which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. S9.71060, or S9.8 of the Radio Regulations, or those of paragraph 7.2.1 of this Appendix;

• NOTE - 1.3 replaces 4.3.1.5 and 4.3.3.5., and merges these with 4.3.3.6.

�Both S9.7 and S9.8 need to be referred to since an FSS network could have started coordination under S9.7 and not under S9.8 if the threshold condition to trigger coordination with BSS planned services was not exceeded. Also, coordination under S9.7 is the only one applicable in the case of an FSS receive space station (e.g. at 12.5 - 12.75 GHz in Region 1).

• NOTE - a) and b) under 4.3.3.6 are covered under 1.3, and S9.7 is in relation to the provisions of Resolution 34, which states that Resolution 33 and Article 11 procedures have to be followed in the case of an unplanned BSS assignment in the planned bands (12.5 - 12.75 GHz in region 3). The scope of S9.7 needs to be modified to cover all unplanned space services with respect to all unplanned space services, in all bands. c) under 4.3.3.6 is superfluous: the future conference mentioned, if and when it takes place, could carry out the task of modifying this provision, if required. The scope of S9.8 also needs to be extended to cover all unplanned space services in respect of the planned BSS.

4.3.1.6	whose services are considered to be affected.

4.3.2	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

	For Region 2:

4.3.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

4.3.3.1	of Region 2 having a frequency assignment in the Region 2 Plan to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with that Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

4.3.3.2	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Regions 1 and 3 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or

4.3.3.3	identified in accordance with resolves 1 of Resolution 43 (Orb-85); or

4.3.3.4	having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

4.3.3.5	having a frequency assignment in the band 12.5 - 12.7 GHz in Region 1 or 12.2 - 12.7 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations or those of paragraph 7.2.1 of this Appendix; or

4.3.3.6	having a frequency assignment to a space station in the broadcasting-satellite service in the band 12.5 - 12.7 GHz in Region 3 with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment and which

�	a)	is recorded in the Master Register, or

	b)	has been coordinated or is being coordinated under the provisions of Resolution 33, or

	c)	appears in a Region 3 Plan to be adopted at a future administrative radio conference, taking account of modifications to that Plan which may be introduced in accordance with the Final Acts of the Conference;

4.3.3.71.4	whose services are considered to be affected.

4.3.4	1.5	The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

1.6	No agreement is required when the proposed modification to the characteristics of a frequency assignment in conformity with the appropriate regional plan relates to the same orbital position and would not result in an increase above that of the original assignment, in the level of potential interference into the services of other administrations sharing the same band with equality of rights.

2	Modification to Appendix S30A Plans

	For Regions 1 and 3

4.2.1	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Regions 1 and 3 appropriate regional Plan under Appendix S30A or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations:

4.2.1.12.1	of Regions 1 and 3 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, in the same orbital position or an adjacent orbital position in the range ± 12. 5° which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Board Bureau in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; T4.12 of the Radio Regulations, or

4.2.1.22.2	having a frequency assignment in the band 17.7 - 18.1 GHz to an earth station in the fixed-satellite service (space-to-Earth) or the broadcasting satellite service where this service is not subject to a plan, which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. S9.7, S9.9 1060 or S9.17 bis of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

• NOTE - The first modification is intended to include the case of the Region 2 BSS allocation in the 17.3 - 17.8 GHz band. The second modification is the consequence of the possibility for the affected administration to have started coordination in three possible different situations: S9.7 if there was no need to coordinate with the feeder link Plan at that time, S9.9 is there was such a need as a result of the space station interfering into the feeder link Plan, S9.17 bis if the procedure between earth stations operating in opposite directions of transmission was started.

4.2.1.32.3	having a frequency assignment in the bands 14.5 - 14.8 GHz or 17.7 - 18.1 GHz to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

�4.2.1.4	having an assignment for feeder links in the fixed-satellite service (Earth-to-space) with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Region 2 feeder-link Plan, or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article;

4.2.1.5	which are considered affected.

4.2.1.6	The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

	For Region 2

4.2.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations:

4.2.3.1	of Region 2 having a feeder-link frequency assignment in the fixed- satellite service (Earth-to-space) in the same channel or an adjacent channel, which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; or

4.2.3.2	having a frequency assignment in the band 17.7 - 17.8 GHz to an earth station in the fixed-satellite service (space-to-Earth), which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

4.2.3.3	having a frequency assignment in the band 17.7 - 17.8 GHz to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

4.2.3.4	having an assignment for feeder links in the fixed-satellite service (Earth-to-space) with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Regions 1 and 3 feeder-link Plan, or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article;

2.4	having a feeder link frequency assignment to a space station in the fixed-satellite service (Earth-to-space), with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. S9.7 or S9.9 of the Radio Regulations;

• NOTE - The intent of this new provision is to correct the lack of provision under the current Radio Regulations to protect a receiving unplanned feeder link space station in Region 2 from a transmit feeder link earth station in Region 1 and 3 planned band. S9.9 cannot be applied in this case since it refers to a transmit space station. The scope of S9.9 should therefore be expanded to cover this case.

4.2.3.52.5	which are considered affected.

4.2.3.62.6	The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

4.2.4	The agreement referred to in paragraph 4.2.3 is not required 

�4.2.22.7		No The agreement referred to in paragraph 4.2.1 is not required:

	a) when an administration in Regions 1 and 3 proposes to bring into use, with characteristics1 appearing in the Plan, a fixed feeder-link earth station or a transportable feeder-link earth station in the bands 14.5 - 14.8 GHz or 17.3 - 18.1 GHz.

		1The power to be taken into account is obtained by adding the values specified in columns 8 and 9 of the Plan.

	b) when an administration in Region 2 proposes to bring into use, with characteristics appearing in the Plan, a fixed feeder-link earth station in the band 17.3 - 17.8 GHz or a transportable feeder-link earth station in the band 17.3 - 17.7 GHz. Administrations may communicate to the Board the characteristics of such earth stations for inclusion in the Plan.

	c) when, the proposed modification to the characteristics of a frequency assignment in conformity with the appropriate regional plan would not result in an increase above that of the original assignment, in the level of potential interference into the services of other administrations sharing the same band with equality of rights.



�ANNEX 1 to Appendix S6

Limits for Determining Whether a Service of an�Administration is Affected by a Proposed Modification to�the Plans1

Tables 1 and 2 respectively, provide the various possible cases for which a modification to one of the Plans in Appendix S30 and Appendix S30A, respectively, may affect the relevant services of another administration, in the same or another Region.

For each of these cases, Table 1 and 2 provide:

–	the applicable frequency band,

–	the applicable paragraph of Appendix S6 in respect of which an administration shall be considered as being affected if the applicable criteria are exceeded, and 

–	the type of applicable criteria, and the reference to the applicable paragraph under section 1 or 2, as applicable, in which the applicable limit is provided.

1	Limits applicable to modifications of Appendix S30 Plans

An administration shall be considered affected by a proposed modification to one of the Regional Plans contained in Appendix S30 if the limits indicated below and referred to in Table 1 are exceeded.



�Table 1 

 Sharing criteria associated to Appendix S6 for the modification of Appendix 30 Plans

�Intended Modification of Appendix 30 Plan in Region��Seeking agreement from �Region 1�Region 2�Region 3��BSS in Region 1

11.7 - 12.5 GHz�11.7-12.5 GHz

Par. 1.1

EPM degradation (1)�12.2 - 12.5 GHz

Par. 1.1

pfd limits (3)�11.7-12.2 GHz

Par. 1.1

EPM degradation (1)��BSS in Region 2

12.2 - 12.7 GHz�12.2 - 12.5 GHz

Par. 1.1

pfd limits (3)�12.2-12.7 GHz

Par 1.1

OEPM degradation (2)�12.5 - 12.7 GHz

Par. 1.1

pfd limits (3)��BSS in Region 3

11.7 - 12.2 GHz

�11.7-12.2 GHz

Par. 1.1

EPM degradation (1)�12.5 - 12.7 GHz

Par. 1.1

pfd limits (3)�11.7-12.2 GHz

Par. 1.1

EPM degradation (1)��Unplanned BSS in Region 3

12.5 - 12.75 GHz�

N/A�12.5-12.7 GHz

Par. 1.4

pfd limits (3)�

N/A��FSS (Earth-to-space)

in Region 1

12.5-12.7 GHz�

N/A�12.5-12.7 GHz

Par. 1.3

(T/T limit s(7)�

N/A��FSS (space-to-Earth)

in Region 1

12.5 - 12.7 GHz�

N/A�12.5-12.7 GHz

Par. 1.3

pfd limit s(6)�

N/A��FSS (space-to-Earth)

in Region 2

11.7 - 12.2 GHz�11.7-12.2 GHz

Par. 1.3

pfd limits (6)�

N/A�11.7-12.2 GHz

Par. 1.3

pfd limits (6)��FSS (space-to-Earth)

in Region 3

12.2 - 12.5 GHz

12.5 - 12.75 GHz�12.2 - 12.5 GHz



Par. 1.3

pfd limits (6)�12.2 - 12.7 GHz



Par. 1.3

pfd limits (6)�



N/A��Terrestrial services in Region 1

11.7 - 12.5 GHz

12.5 - 12.7 GHz�11.7 - 12.5 GHz

Par. 1.2

pfd limits (8)�12.2 - 12.7 GHz

Par. 1.2

pfd limits (5)�11.7 - 12.2 GHz

Par. 1.2

pfd limits (8) ��Terrestrial services in Region 2

11.7 - 12.1 GHz

12.1 - 12.2 GHz

12.2 - 12.7 GHz�11.7 - 12.5 GHz

Par. 1.2

pfd limits (4)Annex 1 Sec. 4�12.2 - 12.7 GHz

Par. 1.2

pfd limits (8)

�11.7 - 12.2 GHz

Par. 1.2

pfd limits (4)

��Terrestrial services in Region 3

11.7 - 12.75 GHz

�11.7 - 12.5 GHz

Par. 1.2

pfd limits (8)�12.2 - 12.7 GHz

Par. 1.2

pfd limits (5)�11.7 - 12.2 GHz

12.5 - 12.7 GHz

Par. 1.2

pfd limits (8)���(1)	To protect the Regions 1 and 3 Plan from its own modifications

The Equivalent Protection Margin (EPM) of any assignment at any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(2)	To protect the Region 2 Plan from its own modifications

The Overall Equivalent Protection Margin (OEPM) of any assignment at any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(3)	To protect a regional Plan from modifications to another regional Plan

The proposed modification to another regional Plan results in exceeding, for any angle of arrival, and at any point of the service area of the affected assignment, the following power flux-density values:

	-147			dB(W/m2/27 MHz)	for 0° ( ( ( 0.44°,

	-138 + 25 log (	dB(W/m2/27 MHz)	for 0.44° ( ( ( 19.1°,

	-106			dB(W/m2/27 MHz)	for 19.1° ( ( ,

where ( is the difference in degrees between the longitudes of the broadcasting-satellite space stations.

(4)	To protect terrestrial services in Region 2 from modifications to the Regions 1 and 3 Plan

The proposed modification to Region 1 and 3 Plan results in exceeding a power flux-density, for any angle of arrival, and at any point of the territory, of:

	-125			dB(W/m2/4 kHz)	for BSS station using circular polarization,

	-128			dB(W/m2/4 kHz)	for BSS station using linear polarization.

(5)	To protect terrestrial services in Regions 1 or 3 from modifications to the Region 2 Plan

The proposed modification to Region 2 Plan results in exceeding a power flux-density, for any angle of arrival, and at any point of the territory, of:

�a)	In the frequency band 12.2 - 12.7 GHz, for all the territories of administrations in Regions 11 and 3, for any angle of arrival:

	-125			dB(W/m2/4 kHz)	for BSS station using circular polarization,

	-128				dB(W/m2/4 kHz)	for BSS station using linear polarization;

b)	In the frequency band 12.2 - 12.5 GHz, for territories of administrations in Region 3 and those in the western part of Region 1, west of longitude 30°E:

	-132				dB(W/m2/5 MHz)	for 0° ( ( ( 10°,

	-132 + 4.2 (( - 10)	dB(W/m2/5 MHz)	for 10° ( ( ( 15°,

	-111				dB(W/m2/5 MHz)	for 15° ( ( ( 90°;

where ( is the angle of arrival.

c)	In the frequency band 12.2 - 12.7 GHz, for territories of administrations in Region 12 east of longitude 30°E3:

	-134				dB(W/m2/5 MHz)	for ( = 0°,

	-134 + 4.6975 (2	dB(W/m2/5 MHz)	for 0° ( ( ( 0.8,

	-128 + 25 log (	dB(W/m2/5 MHz)	for ( ( 0.8;

where ( is the angle of arrival.

d)	In the frequency band 12.5 - 12.7 GHz, for all the territories of administrations in Regions 12 and 3,

	-148				dB(W/m2/4 kHz)	for ( = 0°,

	-148 + 4.6975 (2	dB(W/m2/4 kHz)	for 0° ( ( ( 0.8,

	-142.5 + 25 log (	dB(W/m2/4 kHz)	for ( ( 0.8;

where ( is the angle of arrival.

(6)	To protect the fixed-satellite service (space-to-Earth) in one Region from modifications to the Plan of another Region

The proposed modification results in an increase in the power flux-density at any point of the affected territory of 0.25 dB or more above that resulting from the frequency assignments in the corresponding regional Plan at the time of its entry into force.

�However, where an assignment the Regions 1 and 3 Plan or its subsequent modification gives a power flux-density of less than -138 dB(W/m2/27 MHz) anywhere in the territory of an administration of Region 2, that administration shall be considered as not being affected; where an assignment in the Region 2 Plan or its subsequent modification gives a power flux-density of less than -160 dB(W/m2/4 kHz) anywhere in the territory of an administration of Region 1 or 3, that administration shall be considered as not being affected.

(7)	To protect the fixed satellite service (Earth-to-space) in Region 1 from modifications to the Region 2 Plan

The proposed modification results in:

–	a DT/T greater than the DT/T resulting from the assignment in the Region 2 Plan as of the date of entry into force of the Final Acts; and

–	a DT/T greater than 4%,

using the method of Appendix 29 (Case II).

(8)	To protect terrestrial services in one Region from modifications to the Plan of another region

a)	The proposed modification to an existing assignment in a regional Plan results in an increase in the power flux-density at any point of the affected territory of 0.25 dB or more above that resulting from the frequency assignments in this Plan at the time of its entry into force, or

b)	The proposed modification to the Regions 1 and 3 Plan results in a power flux-density at any point of the territory of an administration in Region 1 or 3 greater than the values expressed in section (5) a) or (5) b) above, or

c)	The proposed modification to the Region 2 Plan results in a power flux-density greater than �115 dB(W/m2) at any point of the territory of an administration in Region 2.

2	Limits applicable to modifications of Appendix S30A Plans

An administration shall be considered affected by a modification to one of the Regional Feeder Link Plans contained in Appendix S30A if the limits indicated below and referred to in Table 2 are exceeded.

(1)	To protect the Regions 1 and 3 Feeder-Link Plan from its own modifications

The Equivalent Protection Margin4 (EPM) of any assignment at any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

�(2)	To protect the Region 2 Feeder-Link Plan from its own modifications

The Overall Equivalent Protection Margin5 (OEPM) of any assignment at any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, except in cases covered by Resolution 42 (Rev.Orb-88), falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(3)	To protect a regional Feeder-Link Plan from modifications to another regional Feeder Link Plan

The proposed modification to another regional Feeder Link Plan causes an increase in the noise temperature of the feeder-link space station which exceeds the threshold value of DT/T corresponding to 3%, where DT/T is calculated in accordance with the method given in Appendix 29, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the total RF bandwidth of the feeder-link carriers (24 MHz for Region 2 and 27 MHz for Regions 1 and 3).

Interim systems of Region 2 in accordance with Resolution 42 (Rev.Orb-88) shall not be taken into consideration when applying this provision to proposed modifications to the Regions 1 and 3 Plan. However, this provision shall be applied to Region 2 interim systems with respect to the Regions 1 and 3 Plan.

(4)	To protect the fixed-satellite service (space-to-Earth)

Upon the application of procedures of Section 3 of Annex 4 of Appendix S30A [these procedures are in fact a method, which is identical to that of Appendix S7 for earth station operating in opposite directions of transmission], any part of the territory of the affected administration is included in the coordination area of the frequency assignment to a transmitting feeder-link earth station.

For the purpose of this calculation, the feeder-link transmitting earth station parameters notified by the administration, which may differ from those given in Annex 3 to this Appendix, are used.

(5)	To protect a terrestrial station 

Upon application of the procedures of Appendix S7 [28] to the Radio Regulations, any part of the territory of the affected administration is included in the coordination area of the frequency assignment to a transmitting feeder-link earth station6 .

For the purpose of this calculation, the feeder-link transmitting earth station parameters notified by the administration, which may differ from those given in Annex 3 to this Appendix, are used.

�(6)	To protect the fixed-satellite service (Earth-to-space) which is not subject to a Plan from modifications to the Feeder Link Plan of another Region

The proposed modification to the Feeder Link Plan of another Region causes an increase in the noise temperature of the feeder-link space station which exceeds the threshold value of DT/T corresponding to 6%, where DT/T is calculated in accordance with the method given in Appendix 29, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per Hertz averaged over the total RF bandwidth of the feeder-link carriers.



�Table 2

Sharing criteria associated to Appendix S6 for the modification of Appendix 30A Plans



�Intended Modification of Appendix 30A Plan in Region (��







Seeking agreement from (�

Regions 1 and 3



14.5 - 14.8 GHz (outside Europe)

17.3 - 18.1 GHz�

Region 2



17.3 - 17.8 GHz��FSS (Earth-to-space) 

(i.e. feeder links to BSS)

Regions 1 and 3



14.5 - 14.8 GHz (outside Europe)

17.3 - 18.1 GHz�

14.5 - 14.8 GHz (outside Europe)

17.3 - 18.1 GHz

Par. 2.1

EPM degradation (1)

�

17.3 - 17.8 GHz

Par. 2.1

DT/T (3)��FSS (Earth-to-space) 

(i.e. feeder links to BSS)

Region 2 planned



17.3 - 17.8 GHz�

17.3 - 17.8 GHz

Par. 2.1

DT/T (3)

�

17.3 - 17.8 GHz

Par. 1.2

OEPM degradation (2)��FSS (Earth-to-space) 

(i.e. feeder links to BSS)

Region 2 unplanned



17.8 - 18.1 GHz�

17.8 - 18.1 GHz

Par. 2.4

DT/T (6)

�



N/A��FSS (space-to-Earth)

Regions 1 and 3 :

17.7 - 18.1 GHz

Region 2 :

17.7 - 18.1 GHz until 31 March 2007

17.8 - 18.1 GHz after 1 April 2007�

17.7 - 18.1 GHz

Par. 2.2

coordination area (4)

�

17.7 - 18.1 GHz

Par. 2.2

coordination area (4)

��BSS

Region 2



17.3 - 17.8 GHz

(after 1 April 2007)����FS and MS

Region 1, 2, 3



14.5 - 14.8 GHz

17.7 - 18.1 GHz�

14.5 - 14.8 GHz (Outside Europe)

17.7 - 18.1 GHz

Par. 2.3

coordination area (5)�

17.7 - 18.1 GHz

Par. 2.3

coordination area (5)��



�Annex 2

Example of a possible simplification of Article 5 of Appendix 30 and Article 5 of Appendix 30A into a single provision for notification, examination and recording of assignments in Appendices S30 and S30A Plans

Article T5

This Annex provides a possible text for a unified procedure, Article T5, for notification, examination and recording of assignments in Appendices 30 and 30A, for Regions 1, 2 and 3.

This text of Article T5 is presented in parallel with the current text of Articles 5 of Appendices 30 and 30A, to allow for a direct comparison between the current procedures and their possible simplification. In order to facilitate this comparison, the order of the current Appendices 30/30A procedures has been modified in some instances, which would not modify the scope of these procedures. 



�

ARTICLE T5�Appendix 30 ARTICLE 5�Appendix 30A ARTICLE 5��Procedures for the Notification of Frequency Assignments

 included in the Plans of Appendices S30 and S30A

�Notification, Examination and Recording in the�Master Register of Frequency Assignments�to Space Stations in the�Broadcasting-Satellite Service�Coordination, Notification, Examination and Recording in the Master International Frequency Register of Frequency Assignments to Feeder-Link Transmitting Earth Stations and Receiving Space Stations in the Fixed Satellite-Service���5.1	Notification�5.1	Coordination and notification��Reason: To be dealt with under Article 4 to establish conformity with the Plan.��5.1.1	When an administration wishes to determine whether it is possible to use, at a given location, an amount of power control which is in excess of that contained in column 9 of the Regions 1 and 3 feeder-link Plan, it shall request the Board to determine the amount of permissible power control (not to exceed 10 dB) from that given location using the procedure contained in Section 3.11 of Annex 3 to this Appendix.��T.5.1	Whenever an administration intends to bring into use a frequency assignment, it shall notify this frequency assignment to the Bureau. For this purpose, the notifying administration shall apply the following provisions.�5.1.1	Whenever an administration intends to bring into use a frequency assignment to a space station in the broadcasting-satellite service, it shall notify this frequency assignment to the Board. For this purpose, the notifying administration shall apply the following provisions.�5.1.2	Whenever an administration intends to bring into use a frequency assignment to a transmitting earth station or receiving space station in the fixed-satellite service in the bands between 14.5 GHz and 14.8 GHz and between 17.3 GHz and 18.1 GHz in Regions 1 and 3, and between 17.3 GHz and 17.8 GHz in Region 2, it shall notify this frequency assignment to the Board. For this purpose, the notifying administration shall apply the following provisions.���Reason: To be dealt with under Article 4 to establish conformity with the Plan.��5.1.3	Before an administration in Region 1 or 3 notifies to the Board or brings into use any frequency assignment to a transmitting feeder-link earth station in the bands 14.5 - 14.8 GHz and 17.7 - 18.1 GHz with an e.i.r.p. greater than the sum of the values specified in columns 8 and 9 of the Plan, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station using the method detailed in Appendix 28.��Reason: Could be dealt with under a footnote to Article S5 and examined under T5.10 a)��5.1.4	Before an administration in Region 1 or 3 notifies to the Board or brings into use any frequency assignment to a transmitting feeder-link earth station in the bands 14.5 - 14.8 GHz and 17.7 - 18.1 GHz, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station, using the method detailed in Appendix 28, in respect of notices concerning stations of the mobile and fixed services in the bands 14.5 - 14.8 GHz and 17.7 - 18.1 GHz and of the fixed-satellite service (space-to-Earth) in the band 17.7 - 18.1 GHz received by the Board prior to 29 August 1988 for recording in the Master Register.���Reason: Related to 5.1.4��5.1.5	If an administration with which coordination is sought under paragraph 5.1.4 does not respond within three months, the administration intending to bring into use a frequency assignment to a feeder-link earth station shall notify this frequency assignment in accordance with paragraph 5.1.2 above1.���

T.5.2	For any notification under paragraph T.5.1, a notice for each frequency assignment shall be drawn up as prescribed in [App. S4], the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful.�5.1.2	For any notification under paragraph 5.1.1, an individual notice for each frequency assignment shall be drawn up as prescribed in Annex 2, the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful.�5.1.6	For any notification under 5.1.2, an individual notice for each frequency assignment shall be drawn up as prescribed in Annex 2 to this Appendix, the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful.��T.5.3	Each notice must reach the Bureau not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Bureau not later than three months before that date4.�5.1.3	Each notice must reach the Board not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Board not later than three months before that date4.�5.1.7	Each notice must reach the Board not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Board not later than three months before that date1.���		4 Where appropriate, the notifying administration shall initiate the procedure for modifying the Plan concerned in sufficient time to ensure that this limit is observed. For Region 2, see also Resolution 42 (Orb-88)/. 



Reasons:

Resolution 709 has been suppressed.

Conformity with paragraph B of Annex 7 to Appendix 30 dealt with conformity with the Plan. �		4 Where appropriate, the notifying administration shall initiate the procedure for modifying the Plan concerned in sufficient time to ensure that this limit is observed. For Region 2, see also Resolution 42 (Orb-85) and paragraph B of Annex 7.�		1 In order to facilitate the coordination process, attention is drawn to Resolution 709 (Orb-88).���5.1.4	Any frequency assignment the notice of which reaches the Board after the applicable period specified in paragraph 5.1.3 shall, where it is to be recorded, bear a remark in the Master Register to indicate that it is not in conformity with paragraph 5.1.3.�5.1.8	Any frequency assignment the notice of which reaches the Board after the applicable period specified in paragraph 5.1.7 shall, where it is to be recorded, bear a remark in the Master Register to indicate that it is not in conformity with paragraph 5.1.7.��T.5.5	Any notice made under paragraph T.5.1 which does not contain the characteristics specified in [App. S4/Annex 2] shall be returned by the Bureau immediately by airmail to the notifying administration with the relevant reasons.�5.1.5	Any notice made under paragraph 5.1.1 which does not contain the characteristics specified in Annex 2 shall be returned by the Board immediately by airmail to the notifying administration with the relevant reasons.�5.1.9	Any notice made under paragraph 5.1.2 which does not contain the characteristics specified in Annex 2 to this Appendix shall be returned by the Board immediately by airmail to the notifying administration with the relevant reasons.��T.5.6.	Upon receipt of a complete notice, the Bureau shall include its particulars, with the date of receipt, in its weekly circular, which shall contain the particulars of all such notices received since the publication of the previous circular.�5.1.6	Upon receipt of a complete notice, the Board shall include its particulars, with the date of receipt, in its weekly circular, which shall contain the particulars of all such notices received since the publication of the previous circular.�5.1.10	Upon receipt of a complete notice, the Board shall include its particulars, with the date of receipt, in its weekly circular which shall contain the particulars of all such notices received since the publication of the previous circular.��T.5.7.	The circular shall constitute the acknowledgment to the notifying administration of the receipt of a complete notice.�5.1.7	The circular shall constitute the acknowledgment to the notifying administration of the receipt of a complete notice.�5.1.11	The circular shall constitute the acknowledgements to the notifying administration of the receipt of a complete notice��T.5.8.	Complete notices shall be considered by the Bureau in order of receipt. The Bureau shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Bureau shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Bureau until it has reached a finding with respect to such earlier notice.�5.1.8	Complete notices shall be considered by the Board in order of receipt. The Board shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Board shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Board until it has reached a finding with respect to such earlier notice.�5.1.12	Complete notices shall be considered by the Board in order of receipt. The Board shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Board shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Board until it has reached a finding with respect to such earlier notice��T.5.8.bis In notifying a frequency assignment, an administration may act on behalf of a group of named administrations, in which case any further notice (modification or deletion) relating to such an assignment shall, in the absence of information to the contrary, be regarded as having be submitted on behalf of the entire group.

Reason: To facilitate he establishment of sub-regional systems����T.5.9.	Examination and recording�5.2	Examination and recording�5.2	Examination and recording��T.5.10.	The Bureau shall examine each notice:�5.2.1	The Board shall examine each notice:�5.2.1	The Board shall examine each notice:��	a)	with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to b below);�	a)	with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to b), c) and d) below);�	a)	with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to b), c), d) and e) below); and���	b)	with respect to its conformity with the appropriate Regional Plan.�	b)	with respect to its conformity with the appropriate Regional Plan; or�	b)	with respect to its conformity with the appropriate Regional Plan; or��Reason: To be dealt with under T5.10 a).�	c)	with respect to its conformity with the appropriate Regional Plan, however, having characteristics differing from those in the appropriate Regional Plan in one or more of the following aspects:�	-	use of a reduced e.i.r.p.,

	-	use of a reduced coverage area entirely situated within the coverage area appearing in the appropriate Regional Plan,�	-	use of other modulating signals in accordance with the provisions of paragraph 3.1.3 of Annex 5,

	-	use of the assignment for transmission in the fixed-satellite service in accordance with No. 846 of the Radio Regulations

	-	use of an orbital position under the conditions specified in paragraph B of Annex 7; or�	c)	with respect to its conformity with the appropriate Regional Plan, however, having characteristics differing from those in the Plan in one or more of the following aspects

		-	use of a reduced e.i.r.p.,:

		-	use of a reduced coverage area entirely situated within the coverage area appearing in the Plan

		-	use of other modulating signals in accordance with the provisions of Section 3.1.3 to Annex 5 of Appendix 30 (Orb-85)

		-	in the case of Region 2, use of an orbital position under the conditions specified in paragraph B of Annex 7 to Appendix 30 (Orb�85),

		-	in the case of Regions 1 and 3, use of an orbital position under the conditions specified in Section 3.15 of Annex 3 to Appendix 30A (Orb�88)1

		-	use of an antenna diameter greater than 5 metres for the 17.3 - 18.1 GHz band and 6 metres for the 14.5 - 14.8 GHz band without increasing the on-axis e.i.r.p.,

		-	in the case of Region 2, use of an antenna diameter greater than 5 metres resulting in a greater on-axis e.i.r.p. if the orbital separation with any other space station is greater than 0.5( or�����			1 The Board shall also apply this provision to paragraph 5.2.1 c) of Appendix 30 (Orb-85) for Regions 1 and 3.��c)     with respect to its conformity with the provisions of Resolution 42 (Orb-88).�	d)	with respect to its conformity with the provisions of Resolution 42 (Orb-85).�		d)	for Region 2, with respect to its conformity with the provisions of Resolution 42 (Rev.Orb-88);��Reason: Consequential of deletion of 5.1.4 and 5.1.5��		e)	for Regions 1 and 3, with respect to its conformity with the provisions of paragraph 5.1.3 and also its conformity with paragraph 5.1.4 or 5.1.5 relating to coordination.��T.5.11.	Where the Bureau reaches a favorable finding with respect to paragraphs T.5.10 a) and T.5.10 b), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Bureau shall be recorded. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates recorded for such frequency assignments.�5.2.2	Where the Board reaches a favourable finding with respect to paragraphs 5.2.1 a) and 5.2.1 b), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments.�5.2.2	When the Board reaches a favourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 e), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use in conformity with the Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments.���Reason: Consequential of deletion of 5.2.1 c).�5.2.2.1	Where the Board reaches a favourable finding with respect to paragraphs 5.2.1 a) and 5.2.1 c), the frequency assignment shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. When recording these assignments, the Board shall indicate by an appropriate symbol the characteristics having a value different from that appearing in the appropriate Regional Plan.�5.2.2.1	When the Board reaches a favourable finding with respect to paragraphs 5.2.1 a), 5.2.1 c) and 5.2.1 e), the frequency assignment shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations, all frequency assignments brought into use in conformity with the Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. When recording these assignments, the Board shall indicate by an appropriate symbol the characteristics having a value different from that appearing in the Plan���T.5.12	Where the Bureau reaches a favorable finding with respect to paragraph T.5.10a) but an unfavorable finding with respect to paragraphs T.5.10.b), the notice, except those covered by T.5.13, shall be returned to the notifying administration and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem

T.5.13	In the case of notices from Region 2 administrations which have requested the application of Res. 42, the Bureau shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Orb- 88). A frequency assignment for which the provisions of Resolution 42 (Orb- 88) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. If the Bureau finds that the provisions of Res. 42 have not been applied successfully, the notice shall be returned to the notifying administration.�5.2.2.2	Where the Board reaches a favourable finding with respect to paragraph 5.2.1 a) but an unfavourable finding with respect to paragraphs 5.2.1 b) and 5.2.1 c), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Orb-85). A frequency assignment for which the provisions of Resolution 42 (Orb-85) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Orb-85) and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments�5.2.2.2	In the case of Region 2, when the Board reaches a favourable finding with respect to paragraph 5.2.1 a) but an unfavourable finding with respect to paragraphs 5.2.1 b) and 5.2.1 c), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Rev.Orb-88). A frequency assignment for which the provisions of Resolution 42 (Rev.Orb-88) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Rev.Orb-88) and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. If the finding with respect to paragraph 5.2.1d) is unfavourable, the notice shall be returned immediately by airmail to the notifying administration.���Reason: Consequential of previous deletions��5.2.2.3	In the case of Regions 1 and 3, when the Board reaches a favourable finding with respect to paragraph 5.2.1 a) but an unfavourable finding with respect to paragraphs 5.2.1 b) and 5.2.1 c), the notice shall be returned immediately by airmail to the notifying administration with the Board's reasons for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem.��Reason: Consequential of previous deletions��5.2.2.4	In the case of Regions 1 and 3, when the Board reaches a favourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 c) but an unfavourable finding with respect to paragraph 5.2.1 e), the notice shall be returned immediately by airmail to the notifying administration with the Board's reasons for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem. If the unfavourable finding under paragraph 5.2.1 e) is due to the coordination under paragraph 5.1.3 only not being effected, the administration shall undertake only to bring this assignment into use with an e.i.r.p. level not greater than the sum of the values specified in columns 8 and 9 of the Regions 1 and 3 Plan.��Reason: Consequential of previous deletions��5.2.2.5	When an assignment is recorded as a result of a favourable finding with respect to paragraph 5.2.1 e), a remark shall be included indicating that coordination has been effected.���T.5.14.	Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Bureau shall be indicated by a symbol.�5.2.3	Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Board shall be indicated by a symbol in Column 13a.�5.2.3	Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Board shall be indicated by a symbol in Column 13a.��Reason: Consequential of previous deletions�5.2.4	Where the Board reaches an unfavourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 c), the notice shall be returned immediately by airmail to the notifying administration with the reasons of the Board for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem.�5.2.4	When the Board reaches an unfavourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 c), the notice shall be returned immediately by airmail to the notifying administration with the Board's reasons for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem.��T.5.15	Where the notifying administration resubmits the notice and the finding of the Bureau becomes favorable with respect to the appropriate parts of paragraph T.5.10, the notice shall be treated as in T.5.11 or T.5.13, as appropriate.�5.2.5	Where the notifying administration resubmits the notice and the finding of the Board becomes favourable with respect to the appropriate parts of paragraph 5.2.1, the notice shall be treated as in paragraph 5.2.2, 5.2.2.1 or 5.2.2.2, as appropriate.�5.2.5	When the notifying administration resubmits the notice and the finding of the Board becomes favourable with respect to the appropriate parts of paragraph 5.2.1, the notice shall be treated as in paragraph 5.2.2, 5.2.2.1 or 5.2.2.2 as appropriate.���T.5.16	If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Bureau’s finding with respect to paragraph T.5.10.a) is favorable and the finding with respect to paragraph T.5.10.b) remains unfavorable, the notice is returned to the notifying administration. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in T5.15 is fulfilled.



In the event that the Bureau has been informed of agreement to modification of the Plan for a specified period of time in accordance withT4.25 bis, the frequency assignment shall be recorded in the Master Register with a note indicating that the frequency assignment is valid only for the period specified. The notifying administration using the frequency assignment over a specified period shall not subsequently invoke this fact to justify the continued use of the frequency beyond the period specified unless it obtains the agreement of the administration(s) concerned.�5.2.6	If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Board's finding with respect to paragraph 5.2.1 remains unfavourable, the notice is returned to the notifying administration in accordance with paragraph 5.2.4. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in paragraph 5.2.5 is fulfilled. For Regions 1, 2 and 3, in the event that the Board has been informed of agreement to modification of the Plan for a specified period of time in accordance with Article 4, the frequency assignment shall be recorded in the Master Register with a note indicating that the frequency assignment is valid only for the period specified. The notifying administration using the frequency assignment over a specified period shall not subsequently invoke this fact to justify the continued use of the frequency beyond the period specified unless it obtains the agreement of the administration(s) concerned.�5.2.6	If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Board's finding with respect to 5.2.1 remains unfavourable, the notice is returned to the notifying administration in accordance with 5.2.4. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in 5.2.5 is fulfilled.���T.5.17	If a frequency assignment notified in advance of bringing into use in conformity with paragraph T.5.3 has received a favorable finding by the Bureau with respect to the provisions of paragraph T.5.10, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry.�5.2.7	If a frequency assignment notified in advance of bringing into use in conformity with paragraph 5.1.3 has received a favourable finding by the Board with respect to the provisions of paragraph 5.2.1, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry.�5.2.7	If a frequency assignment notified in advance of bringing into use in conformity with 5.1.3 has received a favourable finding by the Board with respect to the provisions of 5.2.1, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry.��T.5.18	For notices recorded under paragraph T.5.17, when the Bureau has received confirmation that the frequency assignment has been brought into use, the Bureau shall remove the symbol in the Master Register.�5.2.8	When the Board has received confirmation that the frequency assignment has been brought into use, the Board shall remove the symbol in the Master Register�5.2.8	When the Board has received confirmation that the frequency assignment has been brought into use, the Board shall remove the symbol in the Master Register��Reason: Simplification: "Information only" means no regulatory bearing�5.2.9	The date in Column 2c shall be the date of bringing into use notified by the administration concerned. It is given for information only.�5.2.9	The date in Column 2c shall be the date of bringing into use notified by the administration concerned. It is given for information only.���T.5.19	Cancellation of entries in the Master Register�5.3	Cancellation of entries in the Master Register�5.3	Cancellation of entries in the Master Register��T.5.20	If an administration has not confirmed the bringing into use of a frequency assignment under paragraph T.5.17, the Bureau will make inquiries of the administration not earlier than six months after the expiry of the period specified in paragraph T.5.3. On receipt of the relevant information, the Bureau will either modify the date of coming into use or cancel the entry.�5.3.1	If an administration has not confirmed the bringing into use of a frequency assignment under paragraph 5.2.8, the Board will make inquiries of the administration not earlier than six months after the expiry of the period specified in paragraph 5.1.3. On receipt of the relevant information, the Board will either modify the date of coming into use or cancel the entry.�5.3.1	If an administration has not confirmed the bringing into use of a frequency assignment under 5.2.8, the Board will make inquiries of the administration not earlier than six months after the expiry of the period specified in 5.1.3. On receipt of the relevant information, the Board will either modify the date of coming into use or cancel the entry.���T.5.21	If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Bureau within three months, whereupon the entry shall be removed from the Master Register.�5.3.2	If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Board within three months, whereupon the entry shall be removed from the Master Register.�5.3.2	If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Board within three months, whereupon the entry shall be removed from the Master Register.��T.5.22	In the case of modifications to the Plan under paragraph T.4.5, if the assignment has not been brought into use within the period specified under T.4.8, including any approved extension , the assignment shall be cancelled after the notifying administration has been informed by the Bureau.



Reason: To close the loop opened by T4.8����

�Annex 3

Possible modifications of Article S9 and Appendix S5�in relation to Appendices S30 and S30A

This Annex is intended to identify the areas within Article S9 and Appendix S5 for which potential inconsistencies could exist in relation to Appendices S30 and S30A, and to propose modifications to correct these inconsistencies, with the objective that the procedures and associated technical criteria currently applicable to unplanned services in relation to planned services in the frequency bands covered by Appendices 30 and 30A be properly reflected in Article S9 and Appendix S5.

The technical criteria appearing under Appendix S5 to determine the need for coordination with assignments in the Plans are identical to those currently applicable. Proposals for modifications of these criteria are addressed in a separate part of this report.





�

Text of Article S9

�Comments��Article  s9

Procedure for Effecting Coordination with or �Obtaining Agreement of Other Administrations1, 2, 3, 4

���A.S9.1	1	For the application of the provisions of this Article with respect to stations in a space radiocommunication service using frequency bands covered by the fixed-satellite service allotment plan, see also Appendix S30B and Resolution 107 (Orb-88).

A.S9.2	2	These procedures may be applicable to stations on board satellite launching vehicles.���A.S9.4	3	See Appendices S30 and S30A for the coordination of frequency assignments of other services in relation to stations of the broadcasting-satellite service and to stations of feeder links for this service in the bands covered by those Appendices�This provision A.S9.4 implies that such coordination is not dealt with under this Article, which is not the case. IT also implies that the provisions Section I of Article S9 (advance publication), which do not relate to coordination, would be applicable to Appendices S30, S30A, which is not the case either. To remove these inconsistencies, three approaches are available, as identified by WP 10-11S:��(Approach 1)

A.S9.4	For the coordination of frequency assignments to stations in the broadcasting-satellite service and other services in the frequency bands 11.7 - 12.2 GHz (in Region 3), 11.7 - 12.5 GHz (in Region 1) and 12.2 - 12.7 GHz (in Region 2) as well as the coordination of frequency assignments to feeder-link stations utilizing the fixed-satellite service (Earth-to-space) in the frequency band 17.3 - 17.8 GHz (in Region 2) and in the frequency bands 14.5 - 14.8 GHz and 17.3 - 18.1 GHz in Regions 1 and 3 and the other services in these bands, see also Appendices S30 and S30A respectively. �Approach 1) (Status quo ante WRC-95): Appendices S30 and S30A contain all the procedures applicable to all services in the bands covered by Appendices S30 and S30A, as far as their relationship with the planned services (BSS and BSS feeder links) are concerned.

���

(Approach 2)

A.S9.4	For the coordination of frequency assignments to stations in the broadcasting-satellite service in the frequency bands 11.7 - 12.2 GHz (in Region 3), 11.7 - 12.5 GHz (in Region 1) and 12.2 - 12.7 GHz (in Region 2) with the other services in these bands, as well as the coordination of frequency assignments to feeder-link stations utilizing the fixed-satellite service (Earth-to-space) in the frequency band 17.3 - 17.8 GHz (in Region 2) and in the frequency bands 14.5 - 14.8 GHz and 17.3 - 18.1 GHz in Regions 1 and 3 with the other services in these bands, see Article T4 /Appendices S30 and S30A respectively.

�Approach 2) (VGE approach): Appendices S30 and S30A modification procedures are contained in Article T4 and Art. S9 contains the procedures applicable to other services, in particular in their relationship with the planned BSS and BSS feeder links in the Appendices 30 and 30A bands.

��(Approach 3)



�Approach 3)  (Further simplification) : All the procedures applicable to all services would appear in Articles S9 and S11.���Section I.  Advance Publication of Information �on Planned Satellite Networks or Satellite Systems

���S9.1		Before initiating any action under this Article in respect of frequency assignments for a satellite network or a satellite system, an administration, or one4 acting on behalf of a group of named administrations, shall, prior to the coordination procedure described in Section II of Article S9 below, where applicable, send to the Bureau a general description of the network or system for advance publication in the Weekly Circular not earlier than six years and preferably not later than two years before the planned date of bringing into use of the network or system (see also No. S11.44). The characteristics to be provided for this purpose are listed in Appendix S4. The coordination or notification information may also be communicated to the Bureau at the same time; it shall be considered as having been received by the Bureau not earlier than six months after the date of receipt of the information for advance publication where coordination is required by Section II of Article S9. Where coordination is not required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the advance publication information.����

S9.1.1	Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.



S9.2		Amendments to the information sent in accordance with the provisions of No. S9.1 shall also be sent to the Bureau as soon as they become available. For geostationary-satellite networks and non�geostationary-satellite networks which are subject to Section II of Article S9, the use of an additional frequency band will require the application of the advance publication procedure for this band. For non�geostationary-satellite networks which are not subject to Section II of Article S9, the use of an additional frequency band or an extension of the service area will require the application or recommencing, respectively, of the advance publication procedures for these modifications (see Resolution COM4-6).



S9.2bis		If the information is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.



S9.2ter		On receipt of the complete information sent under Nos. S9.1 and S9.2, the Bureau shall publish it in a Special Section of its Weekly Circular within three months. When the Bureau is not in a position to comply, it shall periodically so inform the administrations, giving the reasons therefor.



S9.3		If, upon receipt of the Weekly Circular containing information published under No. S9.2ter, an administration believes that interference which may be unacceptable may be caused to its existing or planned satellite networks or systems or terrestrial stations1, it shall within four months of the date of the Weekly Circular communicate to the publishing administration its comments on the particulars of the anticipated interference to its existing or planned systems. A copy of these comments shall also be sent to the Bureau. Thereafter, both administrations shall endeavour to cooperate in joint efforts to resolve any difficulties, with the assistance of the Bureau, if so requested by either of the parties, and shall exchange any additional relevant information that may be available. If no such comments are received from an administration within the aforementioned period, it may be assumed that the administration concerned has no basic objections to the planned satellite network(s) of the system on which details have been published.

S9.3.1bis	1	The only terrestrial stations to be taken into account are those for which the requirement to coordinate is under Nos. S9.11, S9.11bis and S9.21.����

�S9.4		In the case of difficulties, the administration responsible for the planned satellite network shall explore all possible means to resolve the difficulties without considering the possibility of adjustment to networks of other administrations. If no such means can be found, it may request the other administrations to explore all possible means to meet its requirements. The administrations concerned shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks. An administration on behalf of which details of planned satellite networks have been published in accordance with the provisions of No. S9.2ter shall, after the period of four months, inform the Bureau of the progress made in resolving any difficulties. If necessary, a further report shall be provided prior to the commencement of coordination or the submission of notices to the Bureau.



S9.5		The Bureau shall inform all administrations of the list of administrations which have sent comments under No. S9.3 and provide a summary of the comments received.���Section II.  Coordination Procedure2, 3

A.S9.II.1	2	These procedures are also applicable for earth stations of the Earth exploration-satellite, space research, space operation and radiodetermination-satellite services intended to be used while in motion or during halts at unspecified points.

A.S9.II.2	3	The word "coordination" as used throughout this Article refers also to the process of seeking an agreement of other administrations when required under No. S9.21.

�







���

Sub-Section IIA.  Requirement and Request for Coordination

S9.6		Before an administration1 notifies to the Bureau or brings into use a frequency assignment in any of the cases listed below, it shall effect coordination, as required, with other administrations identified under No. S9.27: 

S9.6.1	1	In the case of coordination of an assignment in a satellite network in relation to another satellite network, an administration may act on behalf of a group of named administrations. Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of the group retain the right to respond in respect of their own services which could affect or be affected by the proposed assignment networks or systems.

�





�The administrations participating in the group should also retain this right in relation to their terrestrial services.��S9.7	a)	for a station in a satellite network using the geostationary-satellite orbit, in respect of any other satellite network using that orbit, for any space radiocommunication services and frequency bands except those covered by the Plans of Appendices S30, S30A and S30B;

MOD

S9.7	a)	for a station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in a frequency band and in a region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a region where this service is not subject to a Plan, with the exception of the coordination under No S9.17bis.�S9.7 is intended to replace Section II of Article 11 (RR 1060) and Section B of Resolution 33 (coordination between BSS space stations and space systems of other administrations), as well as to allow for the protection of receive earth stations in unplanned BSS bands against interference from space stations of other networks.

In the current Radio Regulations, the scope of the various coordination procedures applicable in relation to Appendices 30/30A is addressed in the following provisions :

- Article 11 title, which specifically excludes BSS.

- footnote A.11.1 in the title of Article 11 states that :

"For the coordination of frequency assignments to stations in the broadcasting-satellite service and other services in the frequency bands 11.7 - 12.2 GHz (in Region 3), 11.7 - 12.5 GHz (in Region 1) and 12.2 - 12.7 GHz (in Region 2) as well as the coordination of frequency assignments to feeder-link stations utilizing the fixed-satellite service (Earth-to-space) in the frequency band 17.3 - 17.8 GHz (in Region 2) and in the frequency bands 14.5 - 14.8 GHz and 17.3 - 18.1 GHz in Regions 1 and 3 and the other services in these bands, see also Article 15 and 15A respectively."

- Article 15 (respectively 15A) : "The provisions and associated plans ... as contained in Appendix 30 (respectively 30A) shall apply to the assignment and use of frequencies by stations in the BSS (respectively FSS Earth to space) and to the stations of other services to which these bands are allocated so far as their relationship to the BSS (respectively FSS Earth to space) in these bands is concerned."

	- Article 2 of Appendix 30 (respectively 30A), which reproduces the text of Article 15 (Respectively 15A).���

�

�- However, there are several cases, in particular in Appendix 30A, in which the application of these procedures is not foreseen in this respect, which means that Articles 11 and 13 continue to apply in some cases where "their relationship to the BSS (or FSS Earth-to-space) is concerned".

		S9.7 may attempt to clarify this relationship by gathering in Appendices S30 and S30A all the procedures applicable in these bands, for all services. However, A.S9.4 and the remaining part of S9 is not consistent with this principle : 

	- AS9.4 indicates that Appendices S30 And S30A are "applicable to for the coordination of frequency assignments of other services in relation to stations of the broadcasting-satellite service and to stations of feeder links for this service in the bands covered by those Appendices."����













�- S9.8, S9.9, S9.17bis and S9.19 apply to other services "in relation with the BSS and BSS feeder links. 



The three options identified by WP 10-11S and referred in relation to A.S9.4 above are intended to remove these inconsistencies. The proposed alternative text could cover all three options.



Another inconsistency, unrelated to Appendices 30/30A, is the possible duplication of this provision with S9.17bis. Since S9.7 potentially covers also the case of earth station to earth station coordination when operated in opposite directions of transmission. To avoid this duplication, the scope of S9.7 has to explicitly exclude the case of S9.17bis. ���S9.8		b)* 	for a transmitting space station of the fixed-satellite service using the geostationary-satellite orbit in a frequency band shared on an equal primary basis with the broadcasting-satellite service, in respect of stations of the latter service which are subject to the Appendix S30 Plan;



S9.8		b)	for a transmitting space station using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30 Plan for the broadcasting-satellite service, in respect of this Plan;

�S9.8 is intended to reflect Section 7.2 of Article 7 of Appendix 30, as highlighted by the threshold conditions associated with S9.8 in Appendix S5, which are those of Annex 4 of Appendix 30.

However, the scope of S9.8 has to be extended to cover the case of unplanned BSS in respect of the Plan. This reflects the application of Resolution 34 in the band 12.5-12.7 GHz in Region 3 in respect of the Region 2 BSS Plan.

���S9.9	c)* 	for a transmitting space station of the fixed-satellite service using the geostationary-satellite orbit in a frequency band shared on an equal primary basis with the feeder links of the broadcasting-satellite service which are subject to the Appendix S30A Plan;

S9.9  c) 	for a station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30A Feeder Link Plan for the broadcasting-satellite service, in respect of this Plan, with the exception of the coordination under No S9.17bis.;

�S9.9 is intended to reflect Section 7.1 of Appendix 30A, which is confirmed by the threshold conditions associated in Appendix S5.



However, footnote S5.515 extends the validity of these conditions to the case of BSS (instead of FSS space-to-Earth) in the band 17.3-17.8 GHz. Section 7.1 of Article 7 of Appendix 30A was modified by WARC-92 to include this extension and S9.9 should therefore take this into account.



In addition, in order to protect receive BSS feeder link space stations in Regions 1 and 3, from FSS transmit space stations in Region 2, the frequency band specified under Appendix S5 for Region 2 should be 17.7-18.1 GHz.



Also, the scope of S9.9 has to be extended to transmit earth stations and receive space stations as well, to cover the case of an unplanned BSS feeder link in Region 2 in the band 17.8-18.1 GHz in respect of Regions 1 and 3 feeder link plan. This case has been omitted under Appendix 30A. The words "transmit space station" should therefore be replaced by "station", which in fact is already the case in the associated part of Appendix S5.



Finally, the scope of S9.17bis also need to be excluded, as under S9.7, for the same reasons.��S9.11	d)	for a space station in the broadcasting-satellite service in any band shared on an equal primary basis with terrestrial services and where in which there is no plan for the broadcasting-satellite service is not subject to a plan, in respect of terrestrial services;

�S9.11 is intended to reflect Section A of Resolution 33, as confirmed by the conditions and frequencies associated in Appendix S5.

Some editorial changes are required to align the text with the terminology used in the previous paragraphs.

In the bands associated under Appendix S5, the band 17.3-17.8 GHz is allocated to BSS in Region 2 from 1 April 2005, but not shared with terrestrial services on a primary basis. This band should therefore be restricted to 17.7-17.8 GHz. ���

�S9.11bis	e)	for a station for which the requirement to coordinate is included in a footnote of the Table of Frequency Allocations referring to this provision:

S9.12	i)	in a satellite network using a non-geostationary-satellite orbit, in respect of any other satellite network using a non-geostationary-satellite orbit, and in respect of any other satellite network using the geostationary-satellite orbit, with the exception of the coordination under No. S9.17bis;

S9.13	ii)	in a satellite network using the geostationary-satellite orbit, in respect of any other satellite network using a non-geostationary-satellite orbit, with the exception of the coordination under No S9.17bis;

S9.14	iii)	which is a space station of a satellite network, in respect of stations of terrestrial services where the threshold value is exceeded

S9.15	iv)	which is either a specific earth station or typical earth station of a non�geostationary satellite network, in respect of terrestrial stations in frequency bands allocated with equal rights to space and terrestrial services and where the coordination area of the earth station includes the territory of another country;

S9.16	v)	which is a transmitting station of a terrestrial service located within the coordination area of an earth station in a non�geostationary-satellite network;

�S9.11 bis and the following four provisions are intended to reflect Resolution 46. Coordination between earth stations operating in opposite directions of transmissions is not mentioned, but treated under S9.17bis.











For the same reasons as given under S9.7, and for consistency with S9.12, the scope of S9.17bis has to be specifically excluded in this case.���

�S9.17		f)** 	for any specific earth station or typical mobile earth station in frequency bands above 1 GHz allocated with equal rights to space and terrestrial services, in respect of terrestrial stations, where the coordination area of the earth station includes the territory of another country, with the exception of the coordination under No. S9.15;

�This reflects Section III of Article 11 and could also cover S9.15 in the same manner as S9.17 bis, but the 1 GHz limit needs careful consideration.

This is also intended to cover section 5.1.4 of Article 5 of Appendix 30A in Regions 1 and 3. The situation in Region 2 is similar but covered within Article 4 of Appendix 30A. Therefore, S9.17 does not need to be changed

Exclusion of the Appendix S30 bands in the corresponding part of Appendix S5 is valid, since this case is dealt with under the procedure for modification of Appendix S30 plans. This is redundant however, since there is no specific earth station in the BSS.

Since Appendix S30A bands are covered in the corresponding part of Appendix S5, there is duplication between S9.17 and that part of the procedure for modification of Appendix S30A Plans dealing with the protection of terrestrial services, in the cases where specific earth stations are considered. Two approaches can be used to resolve this inconsistency :

- delete that part of the procedure for modification of Appendix S30A Plan and replace it by S9.17.

- exclude Appendix S30A bands from Appendix S5 table on S9.17.

���S9.17bis	g)	for any specific earth station, in respect of other earth stations operating in the opposite direction of transmission, in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission and where the coordination area of the earth station includes the territory of another country;

�This is the extension of Section III of Resolution 46 to all cases of earth stations operating in opposite directions of transmissions.

This could reflect Section 7.2 of Article 7 of Appendix 30A as well, but the coordination threshold in Appendix 30A is different. A new line may therefore be introduced under Appendix S5 table to reflect the specific case of Appendix S30A bands, unless Appendix S7 can be modified by WRC-97 to cover this case as well, which would be preferable.

The case of a transmit FSS earth station (at 12.5-12.75 GHz in Region 1) interfering into a receive BSS earth station also needs to be covered, which is currently not the case. Appendix S7 should therefore also cover this case with the aim to protect the BSS service area, in a way similar to the method use in the case of S9.19.

The reference to the frequency bands of S9.11bis, made in the corresponding part of Appendix S5, in the calculation method column, is irrelevant, since it refers to threshold conditions to protect the terrestrial services.



The specific conditions to trigger coordination between earth stations should appear in Appendix S5 rather than in Article S9.

Since Appendix S30A bands are covered in the corresponding part of Appendix S5, there is duplication between S9.17 and that part of the procedure for modification of Appendix S30A Plans dealing with the protection of other space services operating in the opposite direction of transmission, in the cases where specific earth stations are considered. Two approaches can be used to resolve this inconsistency :

- delete that part of the procedure for modification of Appendix S30A Plan and replace it by S9.17bis.

- exclude Appendix S30A bands from Appendix S5 table on S9.17bis.

A similar problem arises in relation to Appendix S30 in the band 12.5-12.7 GHz.

 ���S9.18	h)	for any transmitting station of a terrestrial service in the bands referred to in No. S9.17 within the coordination area of an earth station, with the exception of the coordination under No. S9.16 and S9.19 ;�This is intended to reflect Section IV of Article 11. Could also cover S9.16 in the same manner as S9.17 bis, which would avoid the exception at the end of the paragraph..

A new provision S9.18bis might be introduced (see below) in order to reflect Article 6 of Appendix 30A (coordination of receive terrestrial stations) and the associated conditions introduced in Appendix S5. Is this warranted, in view of the fact that receive terrestrial stations do not get this treatment with other space services ?

The exclusion of Appendices 30 and 30A bands in the corresponding part of Appendix S5 is not valid, since both bands may have unplanned space services sharing the band with terrestrial services. In order to be consistent with S9.19, S9.18 should exclude the scope of S9.19.��S9.18bis

	h)	for any receiving station of a terrestrial service allocated with equal rights to the fixed-satellite service for feeder finks to the broadcasting-satellite service, in the frequency bands and regions where this latter service is subject to Appendix 30A Plan;

�This is intended to reflect Article 6 of Appendix 30A if such a type of coordination was to be maintained.

In Regions 1 and 3, receiving terrestrial stations may coordinate with specific feeder link earth stations (6.1). The earth station administration has 3 months to provide the details of such earth stations (6.3).

In Region 2, when the Plan contains information on specific earth stations, this shall be used to calculate the interference. Otherwise, the earth station administration has three months to provide the details of such earth stations, on request.���S9.19 	i)** 	for any transmitting station of a terrestrial service in a frequency band shared on an equal primary basis with the broadcasting-satellite service , in the frequency bands and regions where this latter service is subject to Appendix S30 Plan;

�This is intended to reflect Article 6 of Appendix 30.

As highlighted by the frequency bands mentioned in the associated part of Appendix S5, it is also intended to provide for the protection of the BSS against terrestrial service in unplanned BSS bands. However, this extension to unplanned BSS is not foreseen under Resolution 33 and might raise sovereignty issues or require provisions to notify BSS earth stations. It may therefore be more consistent to avoid such extension explicitly and delete the reference to S9.11 frequency bands in the corresponding part of Appendix S5.

��S9.21 	j)	for any station of a service for which the requirement to seek the agreement of other administrations is included in a footnote of the Table of Frequency Allocations referring to this provision.

�

This is intended to reflect Article 14.

���TABLE S5.1

Technical conditions for coordination (See Article S9)

Reference of Article S9�Case�Frequency bands (and Region) of the service for which coordination is sought�Threshold/Condition�Calculation method�Remarks��No. S9.7�GSO/GSO�A station in a satellite network using the geostationary-satellite orbit in respect of any other satellite network using that orbit

A station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in a frequency band and in a region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a region where this service is not subject to a Plan, with the exception of the coordination under No S9.17bis�Any frequency band allocated to a space service, where this service is not subject to a Plan, except those mentioned in the plans of Appendices S30, S30A and S30B�Value of �SYMBOL 68 \f "Symbol"�T/T exceeds 6%�Appendix S8����

No. S9.8�GSO/GSO��A transmitting space station of the fixed-satellite service (FSS) using the geostationary-satellite orbit in a frequency band shared with the BSS on an equal primary basis, in respect of space stations of the latter service which are subject to the plan in Appendix S30

A transmitting space station using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30 Plan for the broadcasting-satellite service, in respect of this Plan;

��11.7 - 12.2 GHz (R2)�12.2 - 12.7 GHz (R3)�12.5 - 12.7 GHz (R1)��i) There is an overlap in the necessary bandwidths of the space stations of FSS and BSS; and 

ii) the powerflux-density (pfd) of the transmitting space station of the FSS, assumed under free space propagation conditions, exceeds, on the territory of another administration located in another region, the following values: given in Annex 4 of Appendix S30 on the territory of another administration located in another region.

-147 dB(W/m2/27 MHz) for 0° ( ( ( 0.44°,

-138 + 25 log ( dB(W/ m2/27 MHz) for 0.44° ( ( ( 19.1,

-106 dB(W/m2/27 MHz) for 19.1° ( (,



where ( is the difference in degrees between the longitudes of the BSS and FSS space stations.



��Check by using the�assigned�frequencies and�bandwidths;

��See also Article 7 of Appendix S30.

The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendices 30 and 30A is suspended pending the decision of WRC�97 on revision of Appendices 30 and 30A.���

�No. S9.9�GSO/GSO��A station of the FSS in a frequency band shared on an equal primary basis with the feeder links of the BSS, which are subject to the plan in Appendix S30A



A station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30A Feeder Link Plan for the broadcasting-satellite service, in respect of this Plan, with the exception of the coordination under No S9.17bis.;

�

17.7 - 18.1 GHz (R1)�17.7 - 18.1 GHz (R3)�17.7 - 17.8 18.1 GHz (R2)��i) Value of �SYMBOL 68 \f "Symbol"�Ts/Ts exceeds 4% (see Section 1 of Annex 4 of Appendix S30A); and

�ii) geocentric inter-satellite angular separation is less than�3�SYMBOL 176 \f "Symbol"� or greater than 150�SYMBOL 176 \f "Symbol"���i) Case II of Appendix S8�





ii) Annex I of Appendix S8��The threshold/ conditions do not apply when the geocentric angular separation, between the a transmitting space station in the fixed-satellite service and the a receiving feeder link space station in the feeder-link Plan, exceeds 150° of arc and the free-space power flux-density of the transmitting space station in the fixed-satellite service does not exceed a value of �137 dB(W/m2/MHz) on the surface of the Earth at the equatorial limb.

�The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendices 30 and 30A is suspended pending the decison of WRC-97 on revision of Appendices 30 and 30A.����17.8-18.1 GHz (R2)�Value of �SYMBOL 68 \f "Symbol"�Ts/Ts exceeds 6%�i) Case I of Appendix S8����

��No. S9.11�GSO/terrestrial��A space station in the BSS, in any band shared on an equal primary basis with terrestrial services and where the broadcasting-satellite service is not subject to a plan, in which there is no plan for the BSS, in respect of terrestrial services

��620 - 790 MHz�1 452 - 1 492 MHz�2 310 - 2 360 MHz �2 520 - 2 655 MHz�2 655 - 2 670 MHz�12.5 - 12.75 GHz (R3) 17.37- 17.8 GHz (R2) 21.4 - 22 GHz (R1, R3)�40.5 - 42.5 GHz�84 - 86 GHz��Condition:�Bandwidths overlap���Check by using the assigned �frequencies and bandwidths

����No. S9.16�terrestrial/�non�GSO��A transmitting station of a terrestrial service within the coordination area of an earth station in a non-geostationary-satellite network in frequency bands for which a footnote refers to S9.11bis��See Table S5-1A��Transmitting terrestrial station is situated within the coordination area of a receiving earth station which has already been coordinated���The coordination�area of the affected earth station has already been determined using the calculation method of No. S9.15���

�No. S9.17�GSO, non-GSO/�terrestrial��A specific earth station or a typical mobile earth station in frequency bands above 1 GHz allocated with equal rights to space and terrestrial services in relation to respect of terrestrial stations., where the coordination area of the earth station includes the territory of another country��Any frequency band allocated to a space service, except those mentioned in the plans of Appendix S30:

11.7 - 12.5 GHz (R1)�112.7-12.2 GHz (R3)�12.2 - 12.7 GHz (R2)��The coordination area of the earth station covers the territory of another administration��Appendix S7�(For earth stations in the radiodetermination-satellite service (RDSS) in the bands �1 610 - 1 626.5,�2 483.5 - 2 500 and�2 500 � 2 516.5 MHz,�see Remarks column)��1) The coordination area of aircraft earth stations is determined by increasing the service area by 1 000 km with respect to the aeronautical mobile service (terrestrial) or 500 km with respect to terrestrial services other than the aeronautical mobile service



2) For receiving earth stations in the meteorological-satellite service in frequency bands shared with the meteorological aids service, the coordination distance is considered to be the visibility distance as a function of the earth station horizon elevation angle for a radiosonde at an altitude of 20 km above mean sea level, assuming 4/3 Earth radius

��Note: For RDSS earth stations, a uniform coordination distance of 400 km corresponding to an airborne earth station shall be used. In cases where the earth stations are all ground-based, a coordination distance of 100 km shall be used. �





The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendix 30A is suspended pending the decision of WRC-97 on revision of Appendix 30A.���

�No. S9.17bis�GSO, non-GSO/ GSO, non-GSO��A specific earth station in respect of other earth stations operating in the opposite direction of transmission in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission, where the coordination area of the earth station includes the territory of another country or the earth station is located within the coordination area of a coordinated earth station��Any frequency band allocated to a space service��The coordination area of the earth station covers the territory of another administration or the earth station is located within the coordination area of an coordinated earth station��i) for bands in Table S5-1A, see Table S5-2

ii) see Recommendations ITU-R IS.847, IS.848 and IS.849



Appendix S7 (WRC-97)

(a special section needs to be included in Appendix S7 to protect the service area in the case of Appendices 30 and 30A)��The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendices 30 and 30A is suspended pending the decision of WRC-97 on revision of Appendices 30 and 30A.���No. S9.18�terrestrial/GSO, non-GSO��Any transmitting station of a terrestrial service in the bands mentioned in No. S9.17 within the coordination area of an earth station in a frequency band allocated with equal rights to terrestrial and space radiocommunication services, with the exception of the coordination under S9.19.

��Any frequency band allocated to a space service, except those mentioned in Appendices S30 and S30A with the exception of the frequency bands covered under S9.19.

��Transmitting terrestrial station is situated within the coordination area of an already coordinated receiving earth station��See remarks��The coordination�area of the affected earth station has already been determined using the calculation method of No. S9.17���

S9.18bis

terrestrial/GSO, NGSO�	h)	for any receiving station of a terrestrial service shared allocated with equal rights to Feeder Links for the broadcasting-satellite service, in the frequency bands and regions where this latter service is subject to Appendix 30A Plan;

�17.7 - 18.1 GHz�(R1 and 3)�17.7 - 17.8 GHz (R2)�14.5 - 14.8 GHz�(R1 outside Europe, R2, R3)�Receiving terrestrial station is located within the coordination area of the closest feeder link earth which could be located on the boarder of the territory of another administration, and within the service area of the feeder link assignment in conformity with the appropriate regional Plan in Appendix 30A.��Appendix S7���No. S9.19�terrestrial/GSO, non-GSO��A transmitting station of a terrestrial service in a frequency band shared on an equal primary basis with allocated with equal rights to the broadcasting satellite serviceBSS, in the frequency bands and regions where this latter service is subject to Appendix S30 Plan;

��11.7 - 12.72 GHz (R3) 11.7 - 12.5 GHz (R1) 12.2 - 12.7 GHz (R2)�(R1, 2 and 3)and the bands listed in No. S9.11





Reason: Inter-regional sharing not taken into account.��i) Necessary bandwidths overlap; and 

ii) [the pfd of the terrestrial station at the edge of the BSS service area exceeds the permissible level]��i) Check by using the assigned frequencies and bandwidths

ii) Appendix S7 (WRC-97)

[a special section of Appendix S7 needs to be developed replacing  Annex 3 to Appendix S30]

for the bands covered by that Appendix 11.7-12.7 GHz.��See also Article 6 of Appendix S30 

The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendices 30 and 30A is suspended pending the decision of WRC-97 on revision of Appendices 30 and 30A.����No. S9.21�terrestrial, GSO, non-GSO/�terrestrial, GSO, non-GSO��A station of a service for which the requirement to obtain the agreement of other administrations is included in a footnote to the Table of Frequency Allocations, referring to No. S9.21��Band(s) indicated in the relevant footnotes��Condition: Incompatibility established by the use of Appendices S7, S8, technical annexes of Appendices S30, S30A and S30B, pfd values specified in some of the footnotes, other technical provisions of the Radio Regulations or ITU�R Recommendations as appropriate��Methods specified in, or adapted from, Appendices S7, S8, S30, S30A, S30B, other technical provisions of the Radio Regulations or ITU-R Recommendations���

                        

1	With respect to this Annex, except for sub-section 8 c), the power flux-density limits relate to the values which would be obtained assuming free-space propagation conditions.

	With respect to subsection 8 c) of this Annex, the limits relate to the power flux-density which would be obtained assuming clear-sky propagation conditions using the method contained in Annex 5 of Appendix S30.

1 	In the frequency band 12.5 - 12.7 GHz, in Region 1, these limits are applicable only to the territories of administrations mentioned in S5.494 [RR 848] and S5.496 [RR 850] of the Radio Regulations.

2 	In the frequency band 12.5 - 12.7 GHz, in Region 1, these limits are applicable only to the territories of administrations mentioned in S5.494 [RR 848] and S5.496 [RR 850] of the Radio Regulations.

3 	See Resolution 34.

4 	For the definition of the equivalent protection margin, see Section 1.7 of Annex 3 of this Appendix.

5 	For the definition of the overall equivalent protection margin, see Section 1.14 of Annex 5 to Appendix 30 (Orb-85).

6 	In Regions 1 and 3, for the application of the procedures of Appendix S7, the e.i.r.p. for the feeder-link earth station is the sum of the values specified in columns 8 and 9 of the Plan.





* 	Application of these provisions is suspended pending the decision of the 1997 World Radiocommunication Conference on revision of Appendices 30 and 30A with respect to Articles 6 and 7 of those two Appendices.

** The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendices 30 and 30A is suspended pending the decision of the 1997 World Radiocommunication Conference on revision of Appendices 30 and 30A.



** The application of these provisions with respect to the bands and services of Articles 6 and 7 of Appendices 30 and 30A is suspended pending the decision of the 1997 World Radiocommunication Conference on revision of Appendices 30 and 30A.



____________________



- � PAGE �47� -

SC97-1/3-E



n:\BRDIR\DMU\SC\sc97-1\003E.doc	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)



- 61 -

SC97-1/3-E





n:\BRDIR\dmu\sc\sc97-1\003e.doc	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)



n:\BRDIR\dmu\sc\sc97-1\003E.doc	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)



n:\brsga\text96\sg11\10-11s\000\063e.doc 



� FILENAME \p \* MERGEFORMAT �N:\BRDIR\DMU\SC\SC97-1\003E.DOC�	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)



N:\BRSGA\TEXT96\SG11\10-11S\000\064E.DOC





�PAGE  �31�





� FILENAME \p \* MERGEFORMAT �N:\BRDIR\DMU\SC\SC97-1\003E.DOC�	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)



� FILENAME \p \* MERGEFORMAT �N:\BRDIR\DMU\SC\SC97-1\003E.DOC�	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)



- � PAGE �59� -

SC97-1/3-E



- � PAGE �79� -

SC97-1/3-E



� FILENAME \p \* MERGEFORMAT �N:\BRDIR\DMU\SC\SC97-1\003E.DOC�	� savedate \@ dd.MM.yy �02.12.96�	� printdate \@ dd.MM.yy �02.12.96�

(38809)








