Doc SC-1/27 Rev1

Doc SC-3/6 Rev1

France

Possible modifications of Article S9 and Appendix S5

in relation to Appendix S30

The attached text is intended to propose possible modifications to Article S9 and Appendix S5 that could solve the difficulties identified by WP 10-11S with respect of Appendix 30 Plan protection against new assignments to terrestrial stations.

The proposed modifications are based on the assumption that the scope of RR 838 would be limited to the protection of the assignments appearing in the original plan, i.e. as it would appear in Article 10 of Appendix S30 (WRC-97). Any modification to this plan would be protected against terrestrial services on the same ground as any unplanned space service in respect of terrestrial services allocated the same bands with equal rights.

The modifications identified hereafter are in respect of the text currently proposed in the SC-1 report.

Article  s9

Procedure for Effecting Coordination with or 
Obtaining Agreement of Other Administrations1, 2, 3, 4

NOC	S9.18	h)	for any transmitting station of a terrestrial service in the bands referred to in No. S9.17 within the coordination area of an earth station, with the exception of the coordination under No. S9.16 and S9.19 ;

MOD 	S9.19 	i) 	for any transmitting station of a terrestrial service in a frequency band shared on an equal primary basis with the broadcasting-satellite service , in the frequency bands and regions where this latter service is subject to Appendix S30 Plan, in respect of assignments appearing in this Plan as of [21 November 1997];

| NOC No. S9.18 terrestrial/GSO, non-GSO | Any transmitting station of a terrestrial service in a frequency band allocated with equal rights to terrestrial and space radiocommunication services, with the exception of the coordination under S9.19.. | Any frequency band allocated to a space service, with the exception of the frequency bands covered under S9.19. | Transmitting terrestrial station is situated within the coordination area of areceiving earth station | See remarks | The coordination area of the affected earth station has already been determined using the calculation method of No. S9.17 |
| --- | --- | --- | --- | --- | --- |
| MOD No. S9.19 terrestrial/GSO, non-GSO | A transmitting station of a terrestrial service in a frequency band allocated with equal rights to the broadcasting satellite service, in the frequency bands and regions where this latter service is subject to Appendix S30 Plan in respect of assignments appearing in this Plan as of [21 November 1997]; | 11.7 - 12.7 GHz (R1, 2 and 3) | i) Necessary bandwidths overlap; and ii) [the pfd of the terrestrial station at the edge of the BSS service area exceeds the permissible level] | i) Check by using the assigned frequencies and bandwidths ii) Appendix S7 (WRC-97) [a special section of Appendix S7 needs to be developed replacing Annex 3 to Appendix S30] for the band 11.7-12.7 GHz. |  |