ANNEX 1

Example of a possible simplification of Article 4 of Appendix 30 and Article 4 of Appendix 30A

into a single provision for modifying Appendices S30 and S30A Plans :

Article T4 and Appendix S6

This Annex provides a possible text for a modification procedure (Article T4) and its associated sharing criteria (Appendix S6), based on the current modification procedures of Appendices 30 and 30A Plans and, in line with Recommendation 35 on the consideration of Article T10

This text of Article T4 is presented in the form of amendments to the text of Article T10, in parallel with the current text of Articles 4 of Appendices 30 and 30A, to allow for a direct comparison between the current procedures and their possible simplification. In order to facilitate this comparison, the order of the current Appendices 30/30A procedures has been modified in some instances, which would not modify the scope of these procedures. When the text of Article T10 is modified, for reasons other than the need for alignment with the current text of Articles 4 of  Appendices 30/30A, the justifications for the modification are provided in the same box.

The text of Appendix S6 is presented in the form of amendments to the current text of Articles 4 of Appendices 30 and 30A, with the corresponding justifications. This Appendix is used to identify the administrations whose services might be affected by the proposed modification.

The sharing criteria currently applicable in Appendices 30 and 30A bands are consolidated into a single Annex to Appendix S6.

Article T4 and Appendix S6 could be located either within Appendices 30 and 30A or in a separate Article of the Radio Regulations (e.g. Article S10) applying only to Appendices 30 and 30A.

| ARTICLE T4 Procedure for the Modification of the Frequency Assignment Plans contained in Appendices S30 and S30A · Reason : Covered under S13.17 bis, which could be modified as follows to cover the specific requirement to contain the margins of the plans, for the appropriate plans : S13.17bis The Bureau shall maintain master copies of all world frequency allotment or assignment plans contained in appendices to these Regulations, or adopted by world conferences convened by the Union, including, where appropriate, the margins associated with each assignment or allotment, and incorporating any modifications resulting from the successful application of the relevant modification procedure, and shall provide such copies in an appropriate form for publication by the Secretary-General when justified by circumstances. | ARTICLE 4 OF APPENDIX 30 Procedure for Modifications to the Plans 4.5 Master copy of the Plans 4.5.1 a) The Board shall maintain an up-to-date master copy of the Regions 1 and 3 Plan taking account of the application of the procedure specified in this Article. The Board shall prepare a document listing the amendments to be made to the Plan as a result of modifications made in accordance with the procedure in this Article. b) The Board shall maintain an up-to-date master copy of the Region 2 Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure specified in this Article. This master copy shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference and those derived from all modifications to the Plan as a result of the successful completion of the modification procedure described in this Article. The Board shall prepare a document listing the amendments to be made to the Plan as a result of modifications made in accordance with the procedure described in this Article. 4.5.2 The Secretary-General shall be informed by the Board of any modifications made to the Regional Plans and shall publish an up-to-date version of those Plans in an appropriate form when justified by the circumstances. | ARTICLE 4 OF APPENDIX 30A Procedure for Modifications to the Plans 4.4 Master copies of the Plans 4.4.1 The Board shall maintain up-to-date master copies of the Plans as well as master copies of the margin reports, including for each assignment the overall equivalent protection margins in respect of Region 2 and the feeder-link equivalent protection margins and the overall equivalent protection margins in respect of Regions 1 and 3, taking account of the application of the procedure specified in this Article. Each master copy of the margin reports shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference in the case of Region 2 and the feeder-link equivalent protection margins and the overall equivalent protection margins for the 1988 Conference in the case of Regions 1 and 3 and those derived from all modifications to the Plans as a result of the successful completion of the modification procedure of this Article. 4.4.2 The Secretary-General shall be informed by the Board of any modifications made to the Regional Plans and shall publish up-to-date versions of the Plans in an appropriate form when justified by the circumstances. |
| --- | --- | --- |
| T4.2 Before notifying any assignment which is subject to the Plans of Appendix S30 or Appendix S30A, an administration shall ensure that it is in conformity with the corresponding Plan.1, 2 If the assignment is not in conformity, the administration shall apply the procedure of this Article to effect an appropriate modification to the Plan by seeking the agreement of those administrations whose services might be affected by the proposed modification. | 4.1 When an administration intends to make a modification2 to one of the Regional Plans,...., the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix); | 4.1 When an administration intends to make a modification to one of the Regional Plans, ....,the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix and Resolution 42 (Rev.Orb-88)). |
| T4.2.1 1 An assignment is subject to a plan when it is for a station in a radiocommunication service and in a frequency band and in a geographical area for which the procedures for modification of this Plan or for notification, examination and recording of assignments in this Plan apply . An assignment is in conformity with a Plan, if it appears in this Plan, , or if the procedure for modification of this Plan has been successfully applied to it. · Reason : see Article 2 of Appendices 30 and 30A Reason : the assumption is that Articles 4, 5, 6 and 7 of Appendices 30 and 30A will be replaced by Articles S9, S10 (i.e. T4 and T5), S11 and related Appendices. Hence, no supplementary or alternative procedure would appear in the Plan. T4.2.2 See also Article T5 and Resolution 42. | 4.2 The term “frequency assignment in conformity with the Plan” used in this and the following Articles is defined in Article 1. [Article 1, paragraph 1.6 Frequency assignment in conformity with the Plan: Any frequency assignment which appears in the Regions 1 and 3 Plan or the Region 2 Plan or for which the procedure of Article 4 of this Appendix has been successfully applied.] |  |
| T4.3 A modification to a plan may consist of: T4.4 a) a change in the characteristics of a frequency assignment in the Plan; or T4.5 b) the inclusion of a new frequency assignment in the Plan; or T4.6 c) the cancellation of a frequency assignment in the Plan. | 4.1 When an administration intends to make a modification2 to one of the Regional Plans, i.e. either: a) to modify the characteristics of any of its frequency assignments to a space station3 in the broadcasting-satellite service which are shown in the appropriate Regional Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or b) to include in the appropriate Regional Plan a new frequency assignment to a space station in the broadcasting-satellite service; or c) to cancel a frequency assignment to a space station in the broadcasting-satellite service; the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix); 2 The intention not to employ energy dispersal in accordance with paragraph 3.18 of Annex 5 shall be treated as a modification and thus subject to the appropriate provisions of this Article. 3 The expression “frequency assignment to a space station”, wherever it appears in this Article, shall be understood to refer to a frequency assignment associated with a given orbital position. See also Annex 7 and Resolution 43 (Orb-85) for the orbital limitations | 4.1 When an administration intends to make a modification to one of the Regional Plans, i.e. either: a) to modify the characteristics of any of its frequency assignments in the fixed-satellite service which are shown in the appropriate Regional Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or b) to include in the Plan a new frequency assignment in the fixed-satellite service; or c) to cancel a frequency assignment in the fixed-satellite service, the following procedure shall be applied before any notification of the frequency assignment is made to the International Frequency Registration Board (see Article 5 of this Appendix and Resolution 42 (Rev.Orb-88)). |
| T4.7 Before an administration proposes to include in the Plan under the provisions of T4.5, a new frequency assignment to a space station or to include in the Plan new frequency assignments to a space station whose orbital position is not designated in the Plan for this administration, all the assignments to the service area involved should normally have been brought into service or have been notified to the Bureau in accordance with the relevant provisions of the Plan. Should this not be the case, the administration concerned shall inform the Bureau of the reasons therefor. T4.7.bis In proposing a modification to a plan, an administration may act on behalf of a group of named administrations, in which case all members of the group retain the right to respond in respect of their own services and assignments which could be affected by the proposed modification. Reason : Facilitate the establishment of sub-regional systems (text taken from S9.6.1) | 4.1.1 Before an administration proposes to include in the Region 2 Plan under the provisions of paragraph 4.1 b), a new frequency assignment to a space station or to include in the Plan new frequency assignments to a space station whose orbital position is not designated in the Plan for this administration, all of the assignments to the service area involved should normally have been brought into service or have been notified to the Board in accordance with Article 5 of this Appendix. Should this not be the case, the administration concerned shall inform the Board of the reasons therefor. | 4.1.1 Before an administration proposes to include in the Plan under the provisions of paragraph 4.1 b) a new frequency assignment for reception at a space station1 or to include in the Plan a new frequency assignment for reception at a space station whose orbital position is not designated in the Plan to that administration, all of the assignments to the service areas involved should normally have been brought into service or have been notified to the Board in accordance with Article 5 of this Appendix. Should this not be the case, the administration concerned shall inform the Board of the reasons thereof. 1 The expression "frequency assignment for reception to a space station", wherever it appears in this Article, shall be understood to refer to a frequency assignment associated with a given orbital position. |
|  | 4.3 Proposed modifications to a frequency assignment in conformity with one of the Regional Plans or inclusion in that Plan of a new frequency assignment | 4.2 Proposed modifications to a frequency assignment in conformity with one of the Regional Plans or proposed inclusion in that Plan of a new frequency assignment. |
| - Note : Existing provisions until par. 4.3.5 of Appendix 30/Par. 4.2.5 of Appendix 30A should appear under Appendix S6. | For Regions 1 and 3: 4.3.1 An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Regions 1 and 3 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations: | For Regions 1 and 3 4.2.1 An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Regions 1 and 3 Plan or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations: |
|  | 4.3.1.1 of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with the Regions 1 and 3 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or | 4.2.1.1 of Regions 1 and 3 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, in the same orbital position or an adjacent orbital position in the range ± 12.5° which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; or |
|  | 4.3.1.2 of Region 2 having a frequency assignment to a space station in the broadcasting-satellite service with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Region 2 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or | 4.2.1.2 having a frequency assignment in the band 17.7 - 18.1 GHz to an earth station in the fixed-satellite service (space-to-Earth), which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or |
|  | 4.3.1.3 identified in accordance with resolves 2 of Resolution 43 (Orb-85); or | 4.2.1.3 having a frequency assignment in the bands 14.5 - 14.8 GHz or 17.7 - 18.1 GHz to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or |
|  | 4.3.1.4 having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or | 4.2.1.4 having an assignment for feeder links in the fixed-satellite service (Earth-to-space) with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Region 2 feeder-link Plan, or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; |
|  | 4.3.1.5 having a frequency assignment in the band 11.7 - 12.2 GHz in Region 2 or 12.2 - 12.5 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations, or those of paragraph 7.2.1 of this Appendix; |  |
|  | 4.3.1.6 whose services are considered to be affected. 4.3.2 The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded. | 4.2.1.5 which are considered affected. 4.2.1.6 The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded. |
|  |  | 4.2.2 The agreement referred to in paragraph 4.2.1 is not required when an administration proposes to bring into use, with characteristics1 appearing in the Plan, a fixed feeder-link earth station or a transportable feeder-link earth station in the bands 14.5 - 14.8 GHz or 17.3 - 18.1 GHz. 1 The power to be taken into account is obtained by adding the values specified in columns 8 and 9 of the Plan. |
|  | For Region 2: 4.3.3 An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations: | For Region 2 4.2.3 An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations: |
|  | 4.3.3.1 of Region 2 having a frequency assignment in the Region 2 Plan to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with that Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or | 4.2.3.1 of Region 2 having a feeder-link frequency assignment in the fixed- satellite service (Earth-to-space) in the same channel or an adjacent channel, which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; or |
|  | 4.3.3.2 of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Regions 1 and 3 Plan, or in respect of which proposed modifications to that Plan have already been published by the Board in accordance with the provisions of paragraph 4.3.5.1 or 4.3.6 of this Article; or | 4.2.3.2 having a frequency assignment in the band 17.7 - 17.8 GHz to an earth station in the fixed-satellite service (space-to-Earth), which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or |
|  | 4.3.3.3 identified in accordance with resolves 1 of Resolution 43 (Orb-85); or |  |
|  | 4.3.3.4 having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or | 4.2.3.3 having a frequency assignment in the band 17.7 - 17.8 GHz to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or |
|  | 4.3.3.5 having a frequency assignment in the band 12.5 - 12.7 GHz in Region 1 or 12.2 - 12.7 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 1060 of the Radio Regulations or those of paragraph 7.2.1 of this Appendix; or | 4.2.3.4 having an assignment for feeder links in the fixed-satellite service (Earth-to-space) with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, which is in conformity with the Regions 1 and 3 feeder-link Plan, or in respect of which proposed modifications to the Plan have already been published by the Board in accordance with the provisions of paragraphs 4.2.6.1 and 4.2.7 of this Article; |
|  | 4.3.3.6 having a frequency assignment to a space station in the broadcasting-satellite service in the band 12.5 - 12.7 GHz in Region 3 with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment and which a) is recorded in the Master Register, or b) has been coordinated or is being coordinated under the provisions of Resolution 33, or c) appears in a Region 3 Plan to be adopted at a future administrative radio conference, taking account of modifications to that Plan which may be introduced in accordance with the Final Acts of the Conference; |  |
|  | 4.3.3.7 whose services are considered to be affected. 4.3.4 The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded. | 4.2.3.5 which are considered affected. 4.2.3.6 The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded. |
|  |  | 4.2.4 The agreement referred to in paragraph 4.2.3 is not required when an administration proposes to bring into use, with characteristics appearing in the Plan, a fixed feeder-link earth station in the band 17.3 - 17.8 GHz or a transportable feeder-link earth station in the band 17.3 - 17.7 GHz. Administrations may communicate to the Board the characteristics of such earth stations for inclusion in the Plan. |
| T4.8 For the purpose of effecting a modification to a plan, the administration concerned shall, not earlier than [4-5] years but preferably not later than eighteen months before the date on which the assignment is to be brought into use, send to the Bureau the relevant information listed in Appendix S4 of these regulations. An extension of [2-3] years may be approved by the Board under the same conditions as contained in [S11.xx]. Modifications to a Plan involving additions under paragraph T4.5 shall lapse if the assignment is not brought into use by that date, including any approved extension. | For all Regions: 4.3.5 An administration intending to modify characteristics in one of the Regional Plans shall send to the Board, not earlier than five years but preferably not later than eighteen months before the date on which the assignment is to be brought into use, the relevant information listed in Annex 2. Modifications to that Plan involving additions under paragraph 4.1 b) shall lapse if the assignment is not brought into use by that date. | For all Regions 4.2.5 An administration intending to modify characteristics in one of the Regional Plans shall send to the Board, not earlier than eight years but preferably not later than eighteen months before the date on which the assignment is to be brought into use, the relevant information listed in Annex 2 to this Appendix. |
|  |  | 4.2.6 If an administration wishes to modify its assignments in the Plans contained in Appendix 30 (Orb-85) and in Appendix 30A (Orb-88), the eight-year period of paragraph 4.2.5 will be applicable in lieu of the five-year period specified in paragraph 4.3.5 of Appendix 30 (Orb-85). |
|  | 4.3.5.1 Where as a result of the intended modification the limits defined in Annex 1 are not exceeded, this fact shall be indicated when submitting to the Board the information required by paragraph 4.3.5. The Board shall then publish this information in a special section of its weekly circular. | 4.2.6.1 Where as a result of the intended modification the limits defined in Annex 1 to this Appendix are not exceeded, this fact shall be indicated when submitting to the Board the information required by paragraph 4.2.5. The Board shall then publish this information in a special section of its weekly circular. |
|  | 4.3.5.2 In all other cases the administration shall notify the Board of the names of the administrations whose agreement it considers should be sought in order to arrive at the agreement referred to in paragraph 4.3.1 or 4.3.3, as well as of those with which agreement has already been reached. | 4.2.6.2 In all other cases the administration shall notify the Board of the names of the administrations whose agreement it considers should be sought in order to arrive at the agreement referred to in paragraphs 4.2.1 and 4.2.3 as well as of those with which agreement has already been reached. |
| T4.9 The Bureau, upon receiving the information under No. T4.8: T4.10 a) determine in accordance with Appendix S6 the administrations whose assignments are considered to be affected; T4.11 b) include their names in the publication under No. T4.12; T4.12 c) publish the complete information in its Weekly Circular; T4.13 d) promptly inform all administrations affected of its actions and the results of its calculations, drawing their attention to the relevant Weekly Circular. | 4.3.6 The Board shall determine on the basis of Annex 1 the administrations whose frequency assignments are considered to be affected within the meaning of paragraph 4.3.1 or 4.3.3. The Board shall include the names of those administrations with the information received under paragraph 4.3.5.2 and shall publish the complete information in a special section of its weekly circular. The Board shall immediately send the results of its calculations to the administration proposing the modification to the appropriate Regional Plan. . | 4.2.7 The Board shall determine on the basis of Annex 1 to this Appendix the administrations whose frequency assignments are considered to be affected within the meaning of paragraphs 4.2.1 and 4.2.3. The Board shall include the names of those administrations with the information received under paragraph 4.2.6.2 and shall publish the complete information in a special section of its weekly circular. The Board shall immediately send the results of its calculations to the administration proposing the modification to the Plan. |
| T4.13 bis One month before the expiry of the period referred to in paragraph T4.17, the Bureau shall send a telegram to the administrations listed in the special section of the weekly circular drawing their attention to the matter. · Reason : reflect current Rule of Procedure and current Article 4 text. | 4.3.7 The Board shall send a telegram to the administrations listed in the special section of the weekly circular drawing their attention to the information it contains and shall send them the results of its calculations | 4.2.8 The Board shall send a telegram to the administrations listed in the special section of the weekly circular drawing their attention to the information it contains and shall send them the results of its calculations. |
| T4.14 Following receipt of the Weekly Circular, an administration believing that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Bureau to include its name. The Bureau shall study this request on the basis of Appendix S6 and the relevant rules of Procedure. In the event that the request to be included in the list of affected administrations is accepted by the Bureau, an addendum to the publication mentioned in T4.12 shall be published by the Bureau. Should the Bureau reach a negative conclusion, it shall inform the administrations concerned. | 4.3.8 An administration which feels that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Board to include its name. The Board shall study this request on the basis of Annex 1 and shall send a copy of the request with an appropriate recommendation to the administration proposing the modification to the appropriate Regional Plan. | 4.2.9 An administration which feels that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Board to include its name. The Board shall study this request on the basis of Annex 1 to this Appendix and shall send a copy of the request with an appropriate recommendation to the administration proposing the modification to the Plan. |
| T4.14bis Any modification to a frequency assignment which is in conformity with the appropriate Regional Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Appendix S6 shall be subject to the agreement of all administrations whose services are considered to be affected. | 4.3.9 Any modification to a frequency assignment which is in conformity with the appropriate Regional Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected. | 4.2.10 Any modification to a frequency assignment which is in conformity with the Plan or any inclusion in the Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 to this Appendix shall be subject to the agreement of all affected administrations. |
| T4.15 The administration seeking agreement and those with which it is sought, or the Bureau, may request any additional information they consider necessary. The Bureau shall be sent copies of any such requests and the replies. | 4.3.10 The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Board of such requests. | 4.2.11 The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Board of such requests. |
| T4.16 Comments from administrations on the information published pursuant to T4.12 should be sent either directly to the administration proposing the modification or through the Bureau. In any event the Bureau shall be informed that comments have been made. The Bureau shall inform the administration proposing the modification of the comments that have been made. T4.17 An administration which has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the weekly circular referred to in T4.12 shall be understood to have agreed to the proposed modification. This time-limit may be extended by up to three months for an administration that has requested additional information under T4.15 or for an administration that has requested the assistance of the Bureau under T4.18. In the latter case the Bureau shall inform the administrations concerned of this request. | 4.3.11 Comments from administrations on the information published pursuant to paragraph 4.3.6 should be sent either directly to the administration proposing the modification or through the Board. In any event the Board shall be informed that comments have been made. 4.3.12 An administration that has not notified its comments either to the administration seeking agreement or to the Board within a period of four months following the date of the weekly circular referred to in paragraph 4.3.5.1 or 4.3.6 shall be understood to have agreed to the proposed assignment. This time limit may be extended by up to three months for an administration that has requested additional information under paragraph 4.3.10 or for an administration that has requested the assistance of the Board under paragraph 4.3.20. In the latter case the Board shall inform the administrations concerned of this request. | 4.2.12 Comments from administrations on the information published pursuant to paragraph 4.2.7 should be sent either directly to the administration proposing the modification or through the Board. In any event the Board shall be informed that comments have been made. 4.2.13 An administration which has not notified its comments either to the administration seeking agreement or to the Board, within a period of four months following the date of the weekly circular referred to in paragraph 4.2.6.1 or paragraph 4.2.7 shall be understood to have agreed to the proposed modification. This time-limit may be extended by up to three months for an administration which has requested additional information under paragraph 4.2.11 or for an administration which has requested the assistance of the Board under paragraph 4.2.21. In the latter case the Board shall inform the administrations concerned of this request. |
| T4.18 Any administration involved in this procedure may request the assistance of the Bureau in accordance with S7.5.in seeking agreement T4 T4 | 4.3.20 An administration may at any stage in the procedure described, or before applying it, request the assistance of the Board, particularly in seeking the agreement of another administration. | 4.2.21 An administration may at any stage in the procedure described, or before applying it, request the assistance of the Board, particularly in seeking the agreement of another administration. |
| T4.21 If, following action by the Bureau in response to a request for assistance under. T4.18 the Bureau receives no reply or decision within three months of its request for a decision in the matter from an administration whose agreement has been sought, the administration which requested the agreement shall be deemed to have fulfilled its obligations under this procedure. It shall also be deemed that the administration which did not give its decision has undertaken: T4.22 That no complaint will be made in respect of harmful interference affecting the services rendered by its stations which may be caused by the use of the assignment in conformity with the proposed modification to the Plan. - Reason : Avoid current rigidity of the Plans in cases where, after having made a comment to the proposed modification, an administration does not respond to the Bureau's request for specifying its decision. The text proposed under T4.21 and T4.22 is the same as in similar cases under Article 6 and 7 of Appendices 30 and 30A (e.g. 6.1.10 of Appendix 30). |  |  |
| T4.22 bis If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of T4.8 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal. If however, these modifications to the initial proposal do not lead to an increase in the interference level which would be caused to the services of another administration compared with that caused by the initial modification, further agreement from this administration is not required. | 4.3.13 If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of paragraph 4.3.5 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal. | 4.2.14 If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of paragraph 4.2.5 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal. |
| T4.23 If no comments have been received on the expiry of the periods specified in T4.17, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, or if the provisions of T4.21 have been applied, the administration proposing the modification shall inform the Bureau, indicating the final characteristics of the frequency assignment, together with the names of the administrations with which agreement has been reached. | 4.3.14 If no comments have been received on the expiry of the periods specified in paragraph 4.3.12, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5 and shall inform the Board, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached. | 4.2.15 If no comments have been received on the expiry of the periods specified in paragraph 4.2.13, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5 of this Appendix and shall inform the Board, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached. |
| T4.24 The Bureau shall publish in a special section of its weekly circular the information received under T4.23 together with the names of any administrations with which the provisions of this Article have been successfully applied. The Bureau shall then up-date the master copy of the Plan. The new or modified entry in the Plan shall then have the same status as others appearing in the Plan and shall be considered as being in conformity with the Plan. | 4.3.17 The Board shall publish in a special section of its weekly circular the information received under paragraph 4.3.14 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the appropriate Regional Plan and will be considered as a frequency assignment in conformity with the Plan. | 4.2.18 The Board shall publish in a special section of its weekly circular the information received under paragraph 4.2.15 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Plan and will be considered as a frequency assignment in conformity with the Plan. |
|  | 4.3.18 When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement from an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so. | 4.2.19 When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement from an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so. |
|  | 4.3.21 The relevant provisions of Article 5 of this Appendix shall be applied when frequency assignments are notified to the Board. | 4.2.22 The relevant provisions of Article 5 of this Appendix shall be applied when frequency assignments are notified to the Board. |
| T4.25 bis The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. | 4.3.15 The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. | 4.2.16 The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. |
| · Reason : Already covered under T10.18. | 4.3.19 If no agreement is reached between the administrations concerned, the Board shall carry out any study that may be requested by these administrations; the Board shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem. | 4.2.20 If no agreement is reached between the administrations concerned, the Board shall carry out any study that may be requested by these administrations; the Board shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem. |
| T4.26 Cancellation of frequency assignments When a frequency assignment in conformity with one of the Regional Plans is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a special section of its weekly circular and delete the assignment from the appropriate Regional Plan. | 4.4 Cancellation of frequency assignments When a frequency assignment in conformity with one of the Regional Plans is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Board. The Board shall publish this information in a special section of its weekly circular and delete the assignment from the appropriate Regional Plan. | 4.3 Cancellation of frequency assignments When a frequency assignment in conformity with one of the Regional Plans is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Board. The Board shall publish this information in a special section of its weekly circular and delete the assignment from the Plan. |
| T4.27 When a proposed modification to a plan involves developing countries, administrations shall seek all practicable solutions conducive to the economic development of the broadcasting-satellite systems of those countries. | 4.3.16 When the proposed modification to the appropriate Regional Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting- satellite systems of these countries. | 4.2.17 When the proposed modification to the Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries. |

appendix s6

Identification of Administrations From Which Agreement Is To Be Sought

Under the Provisions of Article T4

1.	MODIFICATION TO APPENDIX S30 PLANS

An administration proposing a modification to the characteristics of a frequency assignment in conformity with the appropriate regional Plan under Appendix S30, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

1.1	having a frequency assignment to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with the appropriate regional Plan, or in respect of which proposed modifications to that Plan have already been published by the Bureau in accordance with the provisions of paragraph T4.12 ; or

· Reason : duplication with previous paragraph.

· Reason : Resolution 43 has been suppressed.

1.2	having no frequency assignment in the broadcasting-satellite service in the channel concerned but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

1.3	having a frequency assignment  to a space station in the fixed-satellite service or in the broadcasting satellite service where it is not subject to a Plan, with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, and which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. S9.7, or S9.8 of the Radio Regulations;

· Note : 1.3 replaces 4.3.1.5 and 4.3.3.5., and merges these with 4.3.3.6

Both S9.7 and S9.8 need to be referred to since an FSS network could have started coordination under S9.7 and not under S9.8 if the threshold condition to trigger coordination with BSS planned services was not exceeded.Also, coordination under S9.7 is the only one applicable in the case of an FSS receive space station (e.g. at 12.5-12..75 GHz in Region 1).

· Note : a) and b) under 4.3.3.6 are covered under 1.3, and S9.7 is in relation to the provisions of Resolution 34, which states that Resolution 33 and Article 11 procedures have to be followed in the case of an unplanned BSS assignment in the planned bands (12.5-12.75 GHz in region 3). The scope of S9.7 needs to be modified to cover all unplanned space services with respect to all unplanned space services, in all bands. c) under 4.3.3.6 is superfluous : the future conference mentioned, if and when it takes place, could carry out the task of modifying this provision, if required.  The scope of S9.8 also needs to be extended to cover all unplanned space services in respect of the planned BSS.

1.4	whose services are considered to be affected.

1.5		The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

1.6 	No agreement is required when the proposed modification to the characteristics of a frequency assignment in conformity with the appropriate regional plan relates to the same orbital position and would not result in an increase above that of the original assignment, in the level of potential interference into the services of other administrations sharing the same band with equality of rights.

2.	MODIFICATION TO APPENDIX S30A PLANS

An administration proposing a modification to the characteristics of a frequency assignment in conformity with the appropriate regional Plan under Appendix S30A or the inclusion of a new frequency assignment in that Plan shall seek the agreement of those administrations:

2.1	having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, in the same orbital position or an adjacent orbital position  ° which appears in the Plan or in respect of which proposed modifications to the Plan have already been published by the Bureau in accordance with the provisions of paragraphT4.12 of the Radio regulations, or

2.2	having a frequency assignment  to an earth station in the fixed-satellite service (space-to-Earth) or the broadcasting satellite service where this service is not subject to a plan, which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. S9.7, S9.9 or S9.17 bis of the Radio Regulations and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

· Note : the first modification is intended to include the case of the Region 2 BSS allocation in the 17.3-17.8 GHz band. The second modification is the consequence of the possibility for the affected administration to have started coordination in three possible different situations : S9.7 if there was no need to coordinate with the feeder link Plan at that time, S9.9 is there was such a need as a result of the space station interfering into the feeder link Plan, S9.17 bis if the procedure between earth stations operating in opposite directions of transmission was started.

2.3	having a frequency assignment  to a terrestrial station in use or intended to be brought into use within three years of the projected date of bringing the feeder-link modification into use, and which is located within the coordination area of the feeder-link fixed-satellite earth station; or

2.4	having a feeder link frequency assignment  to a space station in the fixed-satellite service (Earth-to-space), with the necessary bandwidth , any portion of which falls within the necessary bandwidth of the proposed assignment, which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. S9.7 or S9.9 of the Radio Regulations;

· Note : The intent of this new provision is to correct the lack of provision under the current Radio Regulations to protect a receiving unplanned feeder link space station in Region 2 from a transmit feeder link earth station in Region 1 and 3 planned band. S9.9 cannot be applied in this case since it refers to a transmit space station. The scope of S9.9  should therefore be expanded to cover this case.

2.5	which are considered affected.

2.6	The services of an administration are considered to be affected when the limits shown in Annex 1 to this Appendix are exceeded.

2.7	No agreement  is required

a) when an administration in Regions 1 and 3 proposes to bring into use, with characteristics1 appearing in the Plan, a fixed feeder-link earth station or a transportable feeder-link earth station in the bands 14.5 - 14.8 GHz or 17.3 - 18.1 GHz.

1 The power to be taken into account is obtained by adding the values specified in columns 8 and 9 of the Plan.

b) when an administration in Region 2 proposes to bring into use, with characteristics appearing in the Plan, a fixed feeder-link earth station in the band 17.3 - 17.8 GHz or a transportable feeder-link earth station in the band 17.3 - 17.7 GHz. Administrations may communicate to the Board the characteristics of such earth stations for inclusion in the Plan.

c) when, the proposed modification to the characteristics of a frequency assignment in conformity with the appropriate regional plan would not result in an increase above that of the original assignment, in the level of potential interference into the services of other administrations sharing the same band with equality of rights.

ANNEX 1 to Appendix S6

Limits for Determining Whether a Service of an
Administration is Affected by a Proposed Modification to
the Plans 1

Tables 1 and 2 respectively, provide the various possible cases for which a modification to one of the Plans in Appendix S30 and Appendix S30A, respectively, may affect the relevant services of another administration, in the same or another Region.

For each of these cases, Table 1 and 2 provide :

- the applicable frequency band,

- the applicable paragraph of Appendix S6 in respect of which an administration shall be considered as being affected if the applicable criteria are exceeded, and

- the type of applicable criteria, and the reference to the applicable paragraph under section 1 or 2, as applicable, in which the applicable limit is provided.

1. LIMITS APPLICABLE TO MODIFICATIONS OF APPENDIX S30 PLANS

An administration shall be considered affected by a proposed modification to one of the Regional Plans contained in Appendix S30 if the limits indicated below and referred to in Table 1 are exceeded.

Table 1

Sharing criteria associated to Appendix S6 for the modification of Appendix 30 Plans

|  | Intended Modification of Appendix 30 Plan in Region |  |  |
| --- | --- | --- | --- |
| Seeking agreement from | Region 1 | Region 2 | Region 3 |
| BSS in Region 1 11.7 - 12.5 GHz | 11.7-12.5 GHz Par. 1.1 EPM degradation (1) | 12.2 - 12.5 GHz Par. 1.1 PFD limits (3) | 11.7-12.2 GHz Par. 1.1 EPM degradation (1) |
| BSS in Region 2 12.2 - 12.7 GHz | 12.2 - 12.5 GHz Par. 1.1 PFD limits (3) | 12.2-12.7 GHz Par 1.1 OEPM degradation (2) | 12.5 - 12.7 GHz Par. 1.1 PFD limits (3) |
| BSS in Region 3 11.7 - 12.2 GHz | 11.7-12.2 GHz Par. 1.1 EPM degradation (1) | 12.5 - 12.7 GHz Par. 1.1 PFD limits (3) | 11.7-12.2 GHz Par. 1.1 EPM degradation (1) |
| Unplanned BSS in Region 3 12.5 - 12.75 GHz | N/A | 12.5-12.7 GHz Par. 1.4 PFD limits (3) | N/A |
| FSS (Earth-to-Space) in Region 1 12.5-12.7 GHz | N/A | 12.5-12.7 GHz Par. 1.3 DT/T limit s(7) | N/A |
| FSS (Space-to-Earth) in Region 1 12.5 - 12.7 GHz | N/A | 12.5-12.7 GHz Par. 1.3 PFD limit s(6) | N/A |
| FSS (space-to-Earth) in Region 2 11.7 - 12.2 GHz | 11.7-12.2 GHz Par. 1.3 PFD limits (6) | N/A | 11.7-12.2 GHz Par. 1.3 PFD limits (6) |
| FSS (space-to-Earth) in Region 3 12.2 - 12.5 GHz 12.5 - 12.75 GHz | 12.2 - 12.5 GHz Par. 1.3 PFD limits (6) | 12.2 - 12.7 GHz Par. 1.3 PFD limits (6) | N/A |
| Terrestrial services in Region 1 11.7 - 12.5 GHz 12.5 - 12.7 GHz | 11.7 - 12.5 GHz Par. 1.2 PFD limits (8) | 12.2 - 12.7 GHz Par. 1.2 PFD limits (5) | 11.7 - 12.2 GHz Par. 1.2 PFD limits (8) |
| Terrestrial services in Region 2 11.7 - 12.1 GHz 12.1 - 12.2 GHz 12.2 - 12.7 GHz | 11.7 - 12.5 GHz Par. 1.2 PFD limits (4)Annex 1 Sec. 4 | 12.2 - 12.7 GHz Par. 1.2 PFD limits (8) | 11.7 - 12.2 GHz Par. 1.2 PFD limits (4) |
| Terrestrial services in Region 3 11.7 - 12.75 GHz | 11.7 - 12.5 GHz Par. 1.2 PFD limits (8) | 12.2 - 12.7 GHz Par. 1.2 PFD limits (5) | 11.7 - 12.2 GHz 12.5 - 12.7 GHz Par. 1.2 PFD limits (8) |

(1)	To protect the Regions 1 and 3 Plan from its own modifications  :

The Equivalent Protection Margin (EPM) of any assignment at  any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(2)	To protect the Region 2 Plan from its own modifications  :

The Overall Equivalent Protection Margin (OEPM) of any assignment at  any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(3)	To protect a regional Plan from modifications to another regional Plan :

The proposed modification to another regional Plan results in exceeding, for any angle of arrival, and at any point of the service area of the affected assignment, the following power flux-density values :

-147	dB(W / m2 / 27 MHz)	for 0° £ q < 0.44°  ,

-138 + 25 log q	dB(W / m2 / 27 MHz)	for 0.44° £ q < 19.1°  ,

-106	dB(W / m2 / 27 MHz)	for 19.1° £ q ,

where q is the difference in degrees between the longitudes of the broadcasting-satellite space stations.

(4)	To protect terrestrial services in Region 2 from modifications to the Regions 1 and 3 Plan

The proposed modification to Region 1 and 3 Plan results in exceeding a power flux-density, for any angle of arrival, and at any point of the territory, of :

-125	dB(W / m2 / 4 kHz)	for BSS station using circular polarisation,

-128	dB(W / m2 / 4 kHz)	for BSS station using linear polarisation.

(5)	To protect terrestrial services in Regions 1 or 3  from modifications to the Region 2 Plan :

The proposed modification to Region 2 Plan results in exceeding a power flux-density, for any angle of arrival, and at any point of the territory, of :

a)	In the frequency ban d 12.2 - 12.7 GHz, for all the territories of administrations in Regions 1 and 3, for any angle of arrival :

-125	dB(W / m2 / 4 kHz)	for BSS station using circular polarisation,

-128	dB(W / m2 / 4 kHz)	for BSS station using linear polarisation;

b)	In the frequency band 12.2 - 12.5 GHz, for territories of administrations in Region 3 and those in the western part of Region 1,  west of longitude 30°E :

-132	dB(W / m2 / 5 MHz)	for 0° £ g < 10°  ,

-132 + 4.2 (g - 10)	dB(W / m2 / 5 MHz)	for 10° £ g < 15°  ,

-111	dB(W / m2 / 5 MHz)	for 15° £ g < 90°  ;

where g is the angle of arrival.

c)	In the frequency band 12.2 - 12.7 GHz, for territories of administrations in Region 1  east of longitude 30°E :

-134	dB(W / m2 / 5 MHz)	for g = 0°  ,

-134 + 4.6975 g2	dB(W / m2 / 5 MHz)	for 0° £ g < 0.8°  ,

-128 + 25 log g	dB(W / m2 / 5 MHz)	for g > 0.8°  ;

where g is the angle of arrival.

d)	In the frequency band 12.5 - 12.7 GHz, for all the territories of administrations in Regions 12 and 3,

-148	dB(W / m2 / 4 kHz)	for g = 0°  ,

-148 + 4.6975 g2	dB(W / m2 / 4 kHz)	for 0° £ g < 0.8°  ,

-142.5 + 25 log g	dB(W / m2 / 4 kHz)	for g > 0.8°  ;

where g is the angle of arrival.

(6)	To protect the fixed-satellite service (space-to-earth) in one region

from modifications to the Plan of another Region :

The proposed modification results in an increase in the power flux-density at any point of the affected territory of 0.25 dB or more above that resulting from the frequency assignments in the corresponding regional Plan at the time of its entry into force.

However, where an assignment the Regions 1 and 3 Plan or its subsequent modification gives a power flux-density of less than -138 dB(W/m2/27 MHz) anywhere in the territory of an administration of Region 2, that administration shall be considered as not being affected; where an assignment in the Region 2 Plan or its subsequent modification gives a power flux-density of less than -160 dB(W/m2/4 kHz) anywhere in the territory of an administration of Region 1 or 3, that administration shall be considered as not being affected.

(7)	To protect the fixed satellite service (Earth-to-space) in Region 1

from modifications to the Region 2 Plan

The proposed modification results in :

-	a T/T greater than the T/T resulting from the assignment in the Region 2 Plan as of 	the date of entry into force of the Final Acts; and

-	a T/T greater than 4%,

using the method of Appendix 29 (Case II).

(8)	To protect terrestrial services in one Region

from modifications to the Plan of another region

a)	The proposed modification to an existing assignment in a regional Plan results in an increase in the power flux-density at any point of the affected territory of  0.25 dB or more above that resulting from the frequency assignments in this Plan at the time of its entry into force, or

b)	The proposed modification to the Regions 1 and 3 Plan results in a power flux-density at any point of the territory of an administration in Region 1 or 3 greater than the values expressed in section (5) a) or (5) b) above, or

c)	The proposed modification to the Region 2 Plan results in a power flux-density greater than              -115 dB(W/m²) at any point of the territory of an administration in Region 2.

# 2. LIMITS APPLICABLE TO MODIFICATIONS OF APPENDIX S30A PLANS

An administration shall be considered affected by a modification to one of the Regional Feeder Link Plans contained in Appendix S30A if the limits indicated below  and referred to in Table 2 are exceeded.

(1) 	To protect the Regions 1 and 3 Feeder-Link Plan  from its own modifications

The Equivalent Protection Margin (EPM) of any assignment at  any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(2)	To protect the Region 2 Feeder-Link Plan  from its own modifications

The Overall Equivalent Protection Margin (OEPM) of any assignment at  any test point within the service area, including the cumulative effect of any previously received request for modification to this Plan, or any previous agreement, except in cases covered by Resolution 42 (Rev.Orb-88), falls more than 0.25 dB below 0 dB or, if already negative, decreases by more than 0.25 dB.

(3)	To protect a regional Feeder-Link Plan

from modifications to another regional Feeder Link Plan

The proposed modification to another regional Feeder Link Plan causes an increase in the noise temperature of the feeder-link space station which exceeds the threshold value of T/T corresponding to 3%, where T/T is calculated in accordance with the method given in Appendix 29, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the total RF bandwidth of the feeder-link carriers (24 MHz for Region 2 and 27 MHz for Regions 1 and 3).

Interim systems of Region 2 in accordance with Resolution 42 (Rev.Orb-88) shall not be taken into consideration when applying this provision to proposed modifications to the Regions 1 and 3 Plan. However, this provision shall be applied to Region 2 interim systems with respect to the Regions 1 and 3 Plan.

(4)	To protect the  fixed-satellite service (space-to-Earth)

Upon the application of procedures of Section 3 of Annex 4 of Appendix S30A [these procedures are in fact a method, which is identical to that of Appendix S7 for earth station operating in opposite directions of transmission], any part of the territory of the affected administration is included in the coordination area of the frequency assignment to a transmitting feeder-link earth station.

For the purpose of this calculation, the feeder-link transmitting earth station parameters notified by the administration, which may differ from those given in Annex 3 to this Appendix, are used.

(5)	To protect a terrestrial station

Upon application of the procedures of Appendix S7 [28] to the Radio Regulations, any part of the territory of the affected administration is included in the coordination area of the frequency assignment to a transmitting feeder-link earth station.

For the purpose of this calculation, the feeder-link transmitting earth station parameters notified by the administration, which may differ from those given in Annex 3 to this Appendix, are used.

(6)	To protect the fixed-satellite service (Earth-to-space) which is not subject to a Plan from 	modifications to the Feeder Link Plan of another Region

The proposed modification to the Feeder Link Plan of another Region causes an increase in the noise temperature of the feeder-link space station which exceeds the threshold value of T/T corresponding to 6%, where T/T is calculated in accordance with the method given in Appendix 29, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per Hertz averaged over the total RF bandwidth of the feeder-link carriers.

Table 2

Sharing criteria associated to Appendix S6 for the modification of Appendix 30A Plans

|  | Intended Modification of Appendix 30A Plan in Region ¯ |  |
| --- | --- | --- |
| Seeking agreement from ¯ | Regions 1 and 3 14.5-14.8 GHz (outside Europe) 17.3-18.1 GHz | Region 2 17.3-17.8 GHz |
| FSS (Earth-to-space) (i.e. feeder links to BSS) Regions 1 and 3 14.5-14.8 GHz (outside Europe) 17.3-18.1 GHz | 14.5-14.8 GHz (outside Europe) 17.3-18.1 GHz Par. 2.1 EPM degradation (1) | 17.3-17.8 GHz Par. 2.1 T/T (3) |
| FSS (Earth-to-space) (i.e. feeder links to BSS) Region 2 planned 17.3-17.8 GHz | 17.3-17.8 GHz Par. 2.1 T/T (3) | 17.3-17.8 GHz Par. 1.2 OEPM degradation (2) |
| FSS (Earth-to-space) (i.e. feeder links to BSS) Region 2 unplanned 17.8-18.1 GHz | 17.8-18.1 GHz Par. 2.4 T/T (6) | N/A |
| FSS (space-to-Earth) Regions 1 and 3 : 17.7-18.1 GHz Region 2 : 17.7-18.1 GHz until 31 March 2007 17.8-18.1 GHz after 1 April 2007 | 17.7-18.1 GHz Par. 2.2 coordination area (4) | 17.7-18.1 GHz Par. 2.2 coordination area (4) |
| BSS Region 2 17.3-17.8 GHz (after 1 April 2007) |  |  |
| FS and MS Region 1, 2, 3 14.5-14.8 GHz 17.7-18.1 GHz | 14.5-14.8 GHz (Outside Europe) 17.7-18.1 GHz Par. 2.3 coordination area (5) | 17.7-18.1 GHz Par. 2.3 coordination area (5) |

Annex 2

Example of a possible simplification of Article 5 of Appendix 30 and Article 5 of Appendix 30A

into a single provision for notification, examination and recording of assignments in

Appendices S30 and S30A Plans :

Article T5

This Annex provides a possible text for a unified procedure, Article T5,  for notification, examination and recording of assignments in Appendices 30 and 30A, for Regions 1, 2 and 3.

This text of Article T5 is presented in parallel with the current text of Articles 5 of Appendices 30 and 30A, to allow for a direct comparison between the current procedures and their possible simplification. In order to facilitate this comparison, the order of the current Appendices 30/30A procedures has been modified in some instances, which would not modify the scope of these procedures.

| ARTICLE T5 | Appendix 30 ARTICLE 5 | Appendix 30A ARTICLE 5 |
| --- | --- | --- |
| Procedures for the Notification of Frequency Assignments included in the Plans of Appendices S30 and S30A | Notification, Examination and Recording in the Master Register of Frequency Assignments to Space Stations in the Broadcasting-Satellite Service | Coordination, Notification, Examination and Recording in the Master International Frequency Register of Frequency Assignments to Feeder-Link Transmitting Earth Stations and Receiving Space Stations in the Fixed Satellite-Service |
|  | 5.1 Notification | 5.1 Coordination and notification |
| - Reason: To be dealt with under Article 4 to establish conformity with the Plan. |  | 5.1.1 When an administration wishes to determine whether it is possible to use, at a given location, an amount of power control which is in excess of that contained in column 9 of the Regions 1 and 3 feeder-link Plan, it shall request the Board to determine the amount of permissible power control (not to exceed 10 dB) from that given location using the procedure contained in Section 3.11 of Annex 3 to this Appendix. |
| T.5.1 Whenever an administration intends to bring into use a frequency assignment, it shall notify this frequency assignment to the Bureau. For this purpose, the notifying administration shall apply the following provisions. | 5.1.1 Whenever an administration intends to bring into use a frequency assignment to a space station in the broadcasting-satellite service, it shall notify this frequency assignment to the Board. For this purpose, the notifying administration shall apply the following provisions. | 5.1.2 Whenever an administration intends to bring into use a frequency assignment to a transmitting earth station or receiving space station in the fixed-satellite service in the bands between 14.5 GHz and 14.8 GHz and between 17.3 GHz and 18.1 GHz in Regions 1 and 3, and between 17.3 GHz and 17.8 GHz in Region 2, it shall notify this frequency assignment to the Board. For this purpose, the notifying administration shall apply the following provisions. |
| - Reason: To be dealt with under Article 4 to establish conformity with the Plan. |  | 5.1.3 Before an administration in Region 1 or 3 notifies to the Board or brings into use any frequency assignment to a transmitting feeder-link earth station in the bands 14.5 - 14.8 GHz and 17.7 - 18.1 GHz with an e.i.r.p. greater than the sum of the values specified in columns 8 and 9 of the Plan, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station using the method detailed in Appendix 28. |
| - Reason: Could be dealt with under a footnote to Article S5 and examined under T5.10 a) |  | 5.1.4 Before an administration in Region 1 or 3 notifies to the Board or brings into use any frequency assignment to a transmitting feeder-link earth station in the bands 14.5 - 14.8 GHz and 17.7 - 18.1 GHz, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station, using the method detailed in Appendix 28, in respect of notices concerning stations of the mobile and fixed services in the bands 14.5 - 14.8 GHz and 17.7 - 18.1 GHz and of the fixed-satellite service (space-to-Earth) in the band 17.7 - 18.1 GHz received by the Board prior to 29 August 1988 for recording in the Master Register. |
| - Reason: Related to 5.1.4 |  | 5.1.5 If an administration with which coordination is sought under paragraph 5.1.4 does not respond within three months, the administration intending to bring into use a frequency assignment to a feeder-link earth station shall notify this frequency assignment in accordance with paragraph 5.1.2 above1. |
| T.5.2 For any notification under paragraph T.5.1, a notice for each frequency assignment shall be drawn up as prescribed in [App. S4], the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful. | 5.1.2 For any notification under paragraph 5.1.1, an individual notice for each frequency assignment shall be drawn up as prescribed in Annex 2, the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful. | 5.1.6 For any notification under 5.1.2, an individual notice for each frequency assignment shall be drawn up as prescribed in Annex 2 to this Appendix, the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful. |
| T.5.3 Each notice must reach the Bureau not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Bureau not later than three months before that date4. | 5.1.3 Each notice must reach the Board not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Board not later than three months before that date4. | 5.1.7 Each notice must reach the Board not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Board not later than three months before that date1. |
| 4 Where appropriate, the notifying administration shall initiate the procedure for modifying the Plan concerned in sufficient time to ensure that this limit is observed. For Region 2, see also Resolution 42 (Orb-88)/. - Reasons: Resolution 709 has been suppressed. Conformity with paragraph B of Annex 7 to Appendix 30 delt with conformity with the Plan. | 4 Where appropriate, the notifying administration shall initiate the procedure for modifying the Plan concerned in sufficient time to ensure that this limit is observed. For Region 2, see also Resolution 42 (Orb-85) and paragraph B of Annex 7. | 1 In order to facilitate the coordination process, attention is drawn to Resolution 709 (Orb-88). |
|  | 5.1.4 Any frequency assignment the notice of which reaches the Board after the applicable period specified in paragraph 5.1.3 shall, where it is to be recorded, bear a remark in the Master Register to indicate that it is not in conformity with paragraph 5.1.3. | 5.1.8 Any frequency assignment the notice of which reaches the Board after the applicable period specified in paragraph 5.1.7 shall, where it is to be recorded, bear a remark in the Master Register to indicate that it is not in conformity with paragraph 5.1.7. |
| T.5.5 Any notice made under paragraph T.5.1 which does not contain the characteristics specified in [App. S4/Annex 2] shall be returned by the Bureau immediately by airmail to the notifying administration with the relevant reasons. | 5.1.5 Any notice made under paragraph 5.1.1 which does not contain the characteristics specified in Annex 2 shall be returned by the Board immediately by airmail to the notifying administration with the relevant reasons. | 5.1.9 Any notice made under paragraph 5.1.2 which does not contain the characteristics specified in Annex 2 to this Appendix shall be returned by the Board immediately by airmail to the notifying administration with the relevant reasons. |
| T.5.6. Upon receipt of a complete notice, the Bureau shall include its particulars, with the date of receipt, in its weekly circular, which shall contain the particulars of all such notices received since the publication of the previous circular. | 5.1.6 Upon receipt of a complete notice, the Board shall include its particulars, with the date of receipt, in its weekly circular, which shall contain the particulars of all such notices received since the publication of the previous circular. | 5.1.10 Upon receipt of a complete notice, the Board shall include its particulars, with the date of receipt, in its weekly circular which shall contain the particulars of all such notices received since the publication of the previous circular. |
| T.5.7. The circular shall constitute the acknowledgment to the notifying administration of the receipt of a complete notice. | 5.1.7 The circular shall constitute the acknowledgment to the notifying administration of the receipt of a complete notice. | 5.1.11 The circular shall constitute the acknowledgments to the notifying administration of the receipt of a complete notice |
| T.5.8. Complete notices shall be considered by the Bureau in order of receipt. The Bureau shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Bureau shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Bureau until it has reached a finding with respect to such earlier notice. | 5.1.8 Complete notices shall be considered by the Board in order of receipt. The Board shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Board shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Board until it has reached a finding with respect to such earlier notice. | 5.1.12 Complete notices shall be considered by the Board in order of receipt. The Board shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Board shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Board until it has reached a finding with respect to such earlier notice |
| T.5.8.bis In notifying a frequency assignment, an administration may act on behalf of a group of named administrations, in which case any further notice (modification or deletion) relating to such an assignment shall, in the absence of information to the contrary, be regarded as having be submitted on behalf of the entire group. - Reason: To facilitate he establishment of sub-regional systems |  |  |
| T.5.9. Examination and recording | 5.2 Examination and recording | 5.2 Examination and recording |
| T.5.10. The Bureau shall examine each notice: | 5.2.1 The Board shall examine each notice: | 5.2.1 The Board shall examine each notice: |
| a) with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to b below); | a) with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to b), c) and d) below); | a) with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to b), c), d) and e) below); and |
| b) with respect to its conformity with the appropriate Regional Plan. | b) with respect to its conformity with the appropriate Regional Plan; or | b) with respect to its conformity with the appropriate Regional Plan; or |
| - Reason: To be dealt with under T5.10 a). | c) with respect to its conformity with the appropriate Regional Plan, however, having characteristics differing from those in the appropriate Regional Plan in one or more of the following aspects: - use of a reduced e.i.r.p., - use of a reduced coverage area entirely situated within the coverage area appearing in the appropriate Regional Plan, - use of other modulating signals in accordance with the provisions of paragraph 3.1.3 of Annex 5, - use of the assignment for transmission in the fixed-satellite service in accordance with No. 846 of the Radio Regulations - use of an orbital position under the conditions specified in paragraph B of Annex 7; or | c) with respect to its conformity with the appropriate Regional Plan, however, having characteristics differing from those in the Plan in one or more of the following aspects - use of a reduced e.i.r.p.,: - use of a reduced coverage area entirely situated within the coverage area appearing in the Plan - use of other modulating signals in accordance with the provisions of Section 3.1.3 to Annex 5 of Appendix 30 (Orb-85 - in the case of Region 2, use of an orbital position under the conditions specified in paragraph B of Annex 7 to Appendix 30 (Orb-85), - in the case of Regions 1 and 3, use of an orbital position under the conditions specified in Section 3.15 of Annex 3 to Appendix 30A (Orb-88)1 - use of an antenna diameter greater than 5 metres for the 17.3 - 18.1 GHz band and 6 metres for the 14.5 - 14.8 GHz band without increasing the on-axis e.i.r.p., - in the case of Region 2, use of an antenna diameter greater than 5 metres resulting in a greater on-axis e.i.r.p. if the orbital separation with any other space station is greater than 0.5° or |
|  |  | 1 The Board shall also apply this provision to paragraph 5.2.1 c) of Appendix 30 (Orb-85) for Regions 1 and 3. |
| c) with respect to its conformity with the provisions of Resolution 42 (Orb-88). | d) with respect to its conformity with the provisions of Resolution 42 (Orb-85). | d) for Region 2, with respect to its conformity with the provisions of Resolution 42 (Rev.Orb-88); |
| - Reason: Consequential of deletion of 5.1.4 and 5.1.5 |  | e) for Regions 1 and 3, with respect to its conformity with the provisions of paragraph 5.1.3 and also its conformity with paragraph 5.1.4 or 5.1.5 relating to coordination. |
| T.5.11. Where the Bureau reaches a favorable finding with respect to paragraphs T.5.10 a) and T.5.10 b), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Bureau shall be recorded. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates recorded for such frequency assignments. | 5.2.2 Where the Board reaches a favourable finding with respect to paragraphs 5.2.1 a) and 5.2.1 b), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. | 5.2.2 When the Board reaches a favourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 e), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use in conformity with the Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. |
| - Reason: Consequential of deletion of 5.2.1 c). | 5.2.2.1 Where the Board reaches a favourable finding with respect to paragraphs 5.2.1 a) and 5.2.1 c), the frequency assignment shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. When recording these assignments, the Board shall indicate by an appropriate symbol the characteristics having a value different from that appearing in the appropriate Regional Plan. | 5.2.2.1 When the Board reaches a favourable finding with respect to paragraphs 5.2.1 a), 5.2.1 c) and 5.2.1 e), the frequency assignment shall be recorded in the Master Register. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations, all frequency assignments brought into use in conformity with the Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. When recording these assignments, the Board shall indicate by an appropriate symbol the characteristics having a value different from that appearing in the Plan |
| T.5.12 Where the Bureau reaches a favorable finding with respect to paragraph T.5.10a) but an unfavorable finding with respect to paragraphs T.5.10.b), the notice, except those covered by T.5.13, shall be returned to the notifying administration and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem T.5.13 In the case of notices from Region 2 administrations which have requested the application of Res. 42, the Bureau shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Orb- 88). A frequency assignment for which the provisions of Resolution 42 (Orb- 88) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. If the Bureau finds that the provisions of Res. 42 have not been applied successfully, the notice shall be returned to the notifying administration. | 5.2.2.2 Where the Board reaches a favourable finding with respect to paragraph 5.2.1 a) but an unfavourable finding with respect to paragraphs 5.2.1 b) and 5.2.1 c), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Orb-85). A frequency assignment for which the provisions of Resolution 42 (Orb-85) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Orb-85) and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments | 5.2.2.2 In the case of Region 2, when the Board reaches a favourable finding with respect to paragraph 5.2.1 a) but an unfavourable finding with respect to paragraphs 5.2.1 b) and 5.2.1 c), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Rev.Orb-88). A frequency assignment for which the provisions of Resolution 42 (Rev.Orb-88) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Board shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Rev.Orb-88) and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. If the finding with respect to paragraph 5.2.1d) is unfavourable, the notice shall be returned immediately by airmail to the notifying administration. |
| - Reason: Consequential of previous deletions |  | 5.2.2.3 In the case of Regions 1 and 3, when the Board reaches a favourable finding with respect to paragraph 5.2.1 a) but an unfavourable finding with respect to paragraphs 5.2.1 b) and 5.2.1 c), the notice shall be returned immediately by airmail to the notifying administration with the Board's reasons for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem. |
| - Reason: Consequential of previous deletions |  | 5.2.2.4 In the case of Regions 1 and 3, when the Board reaches a favourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 c) but an unfavourable finding with respect to paragraph 5.2.1 e), the notice shall be returned immediately by airmail to the notifying administration with the Board's reasons for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem. If the unfavourable finding under paragraph 5.2.1 e) is due to the coordination under paragraph 5.1.3 only not being effected, the administration shall undertake only to bring this assignment into use with an e.i.r.p. level not greater than the sum of the values specified in columns 8 and 9 of the Regions 1 and 3 Plan. |
| - Reason: Consequential of previous deletions |  | 5.2.2.5 When an assignment is recorded as a result of a favourable finding with respect to paragraph 5.2.1 e), a remark shall be included indicating that coordination has been effected. |
| T.5.14. Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Bureau shall be indicated by a symbol. | 5.2.3 Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Board shall be indicated by a symbol in Column 13a. | 5.2.3 Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Board shall be indicated by a symbol in Column 13a. |
| - Reason: Consequential of previous deletions | 5.2.4 Where the Board reaches an unfavourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 c), the notice shall be returned immediately by airmail to the notifying administration with the reasons of the Board for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem. | 5.2.4 When the Board reaches an unfavourable finding with respect to paragraphs 5.2.1 a), 5.2.1 b) and 5.2.1 c), the notice shall be returned immediately by airmail to the notifying administration with the Board's reasons for this finding and with such suggestions as the Board may be able to offer with a view to a satisfactory solution of the problem. |
| T.5.15 Where the notifying administration resubmits the notice and the finding of the Bureau becomes favorable with respect to the appropriate parts of paragraph T.5.10, the notice shall be treated as in T.5.11 or T.5.13, as appropriate. | 5.2.5 Where the notifying administration resubmits the notice and the finding of the Board becomes favourable with respect to the appropriate parts of paragraph 5.2.1, the notice shall be treated as in paragraph 5.2.2, 5.2.2.1 or 5.2.2.2, as appropriate. | 5.2.5 When the notifying administration resubmits the notice and the finding of the Board becomes favourable with respect to the appropriate parts of paragraph 5.2.1, the notice shall be treated as in paragraph 5.2.2, 5.2.2.1 or 5.2.2.2 as appropriate. |
| T.5.16 If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Bureau’s finding with respect to paragraph T.5.10.a) is favorable and the finding with respect to paragraph T.5.10.b) remains unfavorable, the notice is returned to the notifying administration. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in T5.15 is fulfilled. In the event that the Bureau has been informed of agreement to modification of the Plan for a specified period of time in accordance withT4.25 bis, the frequency assignment shall be recorded in the Master Register with a note indicating that the frequency assignment is valid only for the period specified. The notifying administration using the frequency assignment over a specified period shall not subsequently invoke this fact to justify the continued use of the frequency beyond the period specified unless it obtains the agreement of the administration(s) concerned. | 5.2.6 If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Board's finding with respect to paragraph 5.2.1 remains unfavourable, the notice is returned to the notifying administration in accordance with paragraph 5.2.4. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in paragraph 5.2.5 is fulfilled. For Regions 1, 2 and 3, in the event that the Board has been informed of agreement to modification of the Plan for a specified period of time in accordance with Article 4, the frequency assignment shall be recorded in the Master Register with a note indicating that the frequency assignment is valid only for the period specified. The notifying administration using the frequency assignment over a specified period shall not subsequently invoke this fact to justify the continued use of the frequency beyond the period specified unless it obtains the agreement of the administration(s) concerned. | 5.2.6 If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Board's finding with respect to 5.2.1 remains unfavourable, the notice is returned to the notifying administration in accordance with 5.2.4. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in 5.2.5 is fulfilled. |
| T.5.17 If a frequency assignment notified in advance of bringing into use in conformity with paragraph T.5.3 has received a favorable finding by the Bureau with respect to the provisions of paragraph T.5.10, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry. | 5.2.7 If a frequency assignment notified in advance of bringing into use in conformity with paragraph 5.1.3 has received a favourable finding by the Board with respect to the provisions of paragraph 5.2.1, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry. | 5.2.7 If a frequency assignment notified in advance of bringing into use in conformity with 5.1.3 has received a favourable finding by the Board with respect to the provisions of 5.2.1, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry. |
| T.5.18 For notices recorded under paragraph T.5.17, when the Bureau has received confirmation that the frequency assignment has been brought into use, the Bureau shall remove the symbol in the Master Register. | 5.2.8 When the Board has received confirmation that the frequency assignment has been brought into use, the Board shall remove the symbol in the Master Register | 5.2.8 When the Board has received confirmation that the frequency assignment has been brought into use, the Board shall remove the symbol in the Master Register |
| - Reason: Simplification: "Information only" means no regulatory bearing | 5.2.9 The date in Column 2c shall be the date of bringing into use notified by the administration concerned. It is given for information only. | 5.2.9 The date in Column 2c shall be the date of bringing into use notified by the administration concerned. It is given for information only. |

| T.5.19 Cancellation of entries in the Master Register | 5.3 Cancellation of entries in the Master Register | 5.3 Cancellation of entries in the Master Register |
| --- | --- | --- |
| T.5.20 If an administration has not confirmed the bringing into use of a frequency assignment under paragraph T.5.17, the Bureau will make inquiries of the administration not earlier than six months after the expiry of the period specified in paragraph T.5.3. On receipt of the relevant information, the Bureau will either modify the date of coming into use or cancel the entry. | 5.3.1 If an administration has not confirmed the bringing into use of a frequency assignment under paragraph 5.2.8, the Board will make inquiries of the administration not earlier than six months after the expiry of the period specified in paragraph 5.1.3. On receipt of the relevant information, the Board will either modify the date of coming into use or cancel the entry. | 5.3.1 If an administration has not confirmed the bringing into use of a frequency assignment under 5.2.8, the Board will make inquiries of the administration not earlier than six months after the expiry of the period specified in 5.1.3. On receipt of the relevant information, the Board will either modify the date of coming into use or cancel the entry. |
| T.5.21 If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Bureau within three months, whereupon the entry shall be removed from the Master Register. | 5.3.2 If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Board within three months, whereupon the entry shall be removed from the Master Register. | 5.3.2 If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Board within three months, whereupon the entry shall be removed from the Master Register. |
| T.5.22 In the case of modifications to the Plan under paragraph T.4.5, if the assignment has not been brought into use within the period specified under T.4.8, including any approved extension , the assignment shall be canceled after the notifying administration has been informed by the Bureau. - Reason: To close the loop opened by T4.8 |  |  |

Annex 3

Possible modifications of Article S9 and Appendix S5

in relation to Appendices S30 and S30A

This Annex is intended to identify the areas within Article S9 and Appendix S5 for which potential inconsistencies could exist in relation to Appendices S30 and S30A, and to propose modifications to correct these inconsistencies, with the objective that the procedures and associated technical criteria currently applicable to unplanned services in relation to planned services in the frequency bands covered by Appendices 30 and 30A be properly reflected in Article S9 and Appendix S5.

The technical criteria appearing under Appendix S5 to determine the need for coordination with assignments in the Plans are identical to those currently applicable. Proposals for modifications of these criteria are addressed in a separate part of this report.

| Text of Article S9 |  |  | Comments |  |
| --- | --- | --- | --- | --- |
| Article s9 Procedure for Effecting Coordination with or Obtaining Agreement of Other Administrations1, 2, 3, 4 |  |  |  |  |
| A.S9.1 1 For the application of the provisions of this Article with respect to stations in a space radiocommunication service using frequency bands covered by the fixed-satellite service allotment plan, see also Appendix S30B and Resolution 107 (Orb-88). A.S9.2 2 These procedures may be applicable to stations on board satellite launching vehicles. |  |  |  |  |
|  |  |  | This provision A.S9.4 implies that such coordination is not dealt with under this Article, which is not the case. IT also implies that the provisions Section I of Article S9 (advance publication), which do not relate to coordination, would be applicable to Appendices S30, S30A, which is not the case either. To remove these inconsistencies, three approaches are available, as identified by WP 10-11S: |  |
| (Approach 1) A.S9.4 For the coordination of frequency assignments to stations in the broadcasting-satellite service and other services in the frequency bands 11.7 - 12.2 GHz (in Region 3), 11.7 - 12.5 GHz (in Region 1) and 12.2 - 12.7 GHz (in Region 2) as well as the coordination of frequency assignments to feeder-link stations utilizing the fixed-satellite service (Earth-to-space) in the frequency band 17.3 - 17.8 GHz (in Region 2) and in the frequency bands 14.5 - 14.8 GHz and 17.3 - 18.1 GHz in Regions 1 and 3 and the other services in these bands, see also Appendices S30 and S30A respectively. |  |  | Approach 1) (Status quo ante WRC-95) : Appendices S30 and S30A contain all the procedures applicable to all services in the bands covered by Appendices S30 and S30A, as far as their relationship with the planned services (BSS and BSS feeder links) are concerned. |  |
| (Approach 2) A.S9.4 For the coordination of frequency assignments to stations in the broadcasting-satellite service in the frequency bands 11.7 - 12.2 GHz (in Region 3), 11.7 - 12.5 GHz (in Region 1) and 12.2 - 12.7 GHz (in Region 2) with the other services in these bands, as well as the coordination of frequency assignments to feeder-link stations utilizing the fixed-satellite service (Earth-to-space) in the frequency band 17.3 - 17.8 GHz (in Region 2) and in the frequency bands 14.5 - 14.8 GHz and 17.3 - 18.1 GHz in Regions 1 and 3 with the other services in these bands, see Article T4 /Appendices S30 and S30A respectively. |  |  | Approach 2) (VGE approach) : Appendices S30 and S30A modification procedures are contained in Article T4 and Art. S9 contains the procedures applicable to other services, in particular in their relationship with the planned BSS and BSS feeder links in the Appendices 30 and 30A bands. |  |
| (Approach 3) |  |  | Approach 3) (Further simplification) : All the procedures applicable to all services would appear in Articles S9 and S11. |  |
| Section I. Advance Publication of Information on Planned Satellite Networks or Satellite Systems |  |  |  |  |
| S9.1 Before initiating any action under this Article in respect of frequency assignments for a satellite network or a satellite system, an administration, or one4 acting on behalf of a group of named administrations, shall, prior to the coordination procedure described in Section II of Article S9 below, where applicable, send to the Bureau a general description of the network or system for advance publication in the Weekly Circular not earlier than six years and preferably not later than two years before the planned date of bringing into use of the network or system (see also No. S11.44). The characteristics to be provided for this purpose are listed in Appendix S4. The coordination or notification information may also be communicated to the Bureau at the same time; it shall be considered as having been received by the Bureau not earlier than six months after the date of receipt of the information for advance publication where coordination is required by Section II of Article S9. Where coordination is not required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the advance publication information. S9.1.1 Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems. S9.2 Amendments to the information sent in accordance with the provisions of No. S9.1 shall also be sent to the Bureau as soon as they become available. For geostationary-satellite networks and non-geostationary-satellite networks which are subject to Section II of Article S9, the use of an additional frequency band will require the application of the advance publication procedure for this band. For non-geostationary-satellite networks which are not subject to Section II of Article S9, the use of an additional frequency band or an extension of the service area will require the application or recommencing, respectively, of the advance publication procedures for these modifications (see Resolution COM4-6). S9.2bis If the information is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided. S9.2ter On receipt of the complete information sent under Nos. S9.1 and S9.2, the Bureau shall publish it in a Special Section of its Weekly Circular within three months. When the Bureau is not in a position to comply, it shall periodically so inform the administrations, giving the reasons therefor. S9.3 If, upon receipt of the Weekly Circular containing information published under No. S9.2ter, an administration believes that interference which may be unacceptable may be caused to its existing or planned satellite networks or systems or terrestrial stations1, it shall within four months of the date of the Weekly Circular communicate to the publishing administration its comments on the particulars of the anticipated interference to its existing or planned systems. A copy of these comments shall also be sent to the Bureau. Thereafter, both administrations shall endeavour to cooperate in joint efforts to resolve any difficulties, with the assistance of the Bureau, if so requested by either of the parties, and shall exchange any additional relevant information that may be available. If no such comments are received from an administration within the aforementioned period, it may be assumed that the administration concerned has no basic objections to the planned satellite network(s) of the system on which details have been published. S9.3.1bis 1 The only terrestrial stations to be taken into account are those for which the requirement to coordinate is under Nos. S9.11, S9.11bis and S9.21. S9.4 In the case of difficulties, the administration responsible for the planned satellite network shall explore all possible means to resolve the difficulties without considering the possibility of adjustment to networks of other administrations. If no such means can be found, it may request the other administrations to explore all possible means to meet its requirements. The administrations concerned shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks. An administration on behalf of which details of planned satellite networks have been published in accordance with the provisions of No. S9.2ter shall, after the period of four months, inform the Bureau of the progress made in resolving any difficulties. If necessary, a further report shall be provided prior to the commencement of coordination or the submission of notices to the Bureau. S9.5 The Bureau shall inform all administrations of the list of administrations which have sent comments under No. S9.3 and provide a summary of the comments received. S9.5bis The procedure of Section I shall be considered solely for the purposes of informing all administrations of developments in the use of space radiocommunications and minimizing any difficulties that might otherwise arise during the coordination stage. |  |  |  |  |
| Section II. Coordination Procedure2, 3 A.S9.II.1 2 These procedures are also applicable for earth stations of the Earth exploration-satellite, space research, space operation and radiodetermination-satellite services intended to be used while in motion or during halts at unspecified points. A.S9.II.2 3 The word "coordination" as used throughout this Article refers also to the process of seeking an agreement of other administrations when required under No. S9.21. |  |  |  |  |
| Sub-Section IIA. Requirement and Request for Coordination S9.6 Before an administration1 notifies to the Bureau or brings into use a frequency assignment in any of the cases listed below, it shall effect coordination, as required, with other administrations identified under No. S9.27: S9.6.1 1 In the case of coordination of an assignment in a satellite network , an administration may act on behalf of a group of named administrations. Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of the group retain the right to respond in respect of their own services which could affect or be affected by the proposed assignment |  |  |  | The administrations participating in the group should also retain this right in relation to their terrestrial services. |
| MOD S9.7 a) for a station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in a frequency band and in a region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a region where this service is not subject to a Plan, with the exception of the coordination under No S9.17bis. | S9.7 is intended to replace Section II of Article 11 (RR 1060) and Section B of Resolution 33 (coordination between BSS space stations and space systems of other administrations), as well as to allow for the protection of receive earth stations in unplanned BSS bands against interference from space stations of other networks. In the current Radio Regulations, the scope of the various coordination procedures applicable in relation to Appendices 30/30A is addressed in the following provisions : - Article 11 title, which specifically excludes BSS. - footnote A.11.1 in the title of Article 11 states that : "For the coordination of frequency assignments to stations in the broadcasting-satellite service and other services in the frequency bands 11.7 - 12.2 GHz (in Region 3), 11.7 - 12.5 GHz (in Region 1) and 12.2 - 12.7 GHz (in Region 2) as well as the coordination of frequency assignments to feeder-link stations utilizing the fixed-satellite service (Earth-to-space) in the frequency band 17.3 - 17.8 GHz (in Region 2) and in the frequency bands 14.5 - 14.8 GHz and 17.3 - 18.1 GHz in Regions 1 and 3 and the other services in these bands, see also Article 15 and 15A respectively." - Article 15 (respectively 15A) : "The provisions and associated plans ... as contained in Appendix 30 (respectively 30A) shall apply to the assignment and use of frequencies by stations in the BSS (respectively FSS Earth to space) and to the stations of other services to which these bands are allocated so far as their relationship to the BSS (respectively FSS Earth to space) in these bands is concerned." - Article 2 of Appendix 30 (respectively 30A), which reproduces the text of Article 15 (Respectively 15A). - However, there are several cases, in particular in Appendix 30A, in which the application of these procedures is not foreseen in this respect, which means that Articles 11 and 13 continue to apply in some cases where "their relationship to the BSS (or FSS Earth-to-space) is concerned". S9.7 may attempt to clarify this relationship by gathering in Appendices S30 and S30A all the procedures applicable in these bands, for all services. However, A.S9.4 and the remaining part of S9 is not consistent with this principle : - AS9.4 indicates that Appendices S30 And S30A are "applicable to for the coordination of frequency assignments of other services in relation to stations of the broadcasting-satellite service and to stations of feeder links for this service in the bands covered by those Appendices." - S9.8, S9.9, S9.17bis and S9.19 apply to other services "in relation with the BSS and BSS feeder links. The three options identified by WP 10-11S and referred in relation to A.S9.4 above are intended to remove these inconsistencies. The proposed alternative text could cover all three options. Another inconsistency, unrelated to Appendices 30/30A, is the possible duplication of this provision with S9.17bis. Since S9.7 potentially covers also the case of earth station to earth station coordination when operated in opposite directions of transmission. To avoid this duplication, the scope of S9.7 has to explicitly exclude the case of S9.17bis. |  |  |  |
| S9.8 b) for a transmitting space station using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30 Plan for the broadcasting-satellite service, in respect of this Plan; |  | S9.8 is intended to reflect Section 7.2 of Article 7 of Appendix 30, as highlighted by the threshold conditions associated with S9.8 in Appendix S5, which are those of Annex 4 of Appendix 30. However, the scope of S9.8 has to be extended to cover the case of unplanned BSS in respect of the Plan. This reflects the application of Resolution 34 in the band 12.5-12.7 GHz in Region 3 in respect of the Region 2 BSS Plan. |  |  |
| S9.9 c) for a station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30A Feeder Link Plan for the broadcasting-satellite service, in respect of this Plan, with the exception of the coordination under No S9.17bis.; |  | S9.9 is intended to reflect Section 7.1 of Appendix 30A, which is confirmed by the threshold conditions associated in Appendix S5. However, footnote S5.515 extends the validity of these conditions to the case of BSS (instead of FSS space-to-Earth) in the band 17.3-17.8 GHz. Section 7.1 of Article 7 of Appendix 30A was modified by WARC-92 to include this extension and S9.9 should therefore take this into account. In addition, in order to protect receive BSS feeder link space stations in Regions 1 and 3, from FSS transmit space stations in Region 2, the frequency band specified under Appendix S5 for Region 2 should be 17.7-18.1 GHz. Also, the scope of S9.9 has to be extended to transmit earth stations and receive space stations as well, to cover the case of an unplanned BSS feeder link in Region 2 in the band 17.8-18.1 GHz in respect of Regions 1 and 3 feeder link plan. This case has been omitted under Appendix 30A. The words "transmit space station" should therefore be replaced by "station", which in fact is already the case in the associated part of Appendix S5. Finally, the scope of S9.17bis also need to be excluded, as under S9.7, for the same reasons. |  |  |
| S9.11 d) for a space station in the broadcasting-satellite service in any band shared on an equal primary basis with terrestrial services and where the broadcasting-satellite service is not subject to a plan, in respect of terrestrial services; |  | S9.11 is intended to reflect Section A of Resolution 33, as confirmed by the conditions and frequencies associated in Appendix S5. Some editorial changes are required to align the text with the terminology used in the previous paragraphs. In the bands associated under Appendix S5, the band 17.3-17.8 GHz is allocated to BSS in Region 2 from 1 April 2005, but not shared with terrestrial services on a primary basis. This band should therefore be restricted to 17.7-17.8 GHz. |  |  |

| S9.11bis e) for a station for which the requirement to coordinate is included in a footnote of the Table of Frequency Allocations referring to this provision: S9.12 i) in a satellite network using a non-geostationary-satellite orbit, in respect of any other satellite network using a non-geostationary-satellite orbit, and in respect of any other satellite network using the geostationary-satellite orbit, with the exception of the coordination under No. S9.17bis; S9.13 ii) in a satellite network using the geostationary-satellite orbit, in respect of any other satellite network using a non-geostationary-satellite orbit, with the exception of the coordination under No S9.17bis; S9.14 iii) which is a space station of a satellite network, in respect of stations of terrestrial services where the threshold value is exceeded S9.15 iv) which is either a specific earth station or typical earth station of a non-geostationary satellite network, in respect of terrestrial stations in frequency bands allocated with equal rights to space and terrestrial services and where the coordination area of the earth station includes the territory of another country; S9.16 v) which is a transmitting station of a terrestrial service located within the coordination area of an earth station in a non-geostationary-satellite network; |  | S9.11 bis and the following four provisions are intended to reflect Resolution 46. Coordination between earth stations operating in opposite directions of transmissions is not mentioned, but treated under S9.17bis. For the same reasons as given under S9.7, and for consistency with S9.12, the scope of S9.17bis has to be specifically excluded in this case. |
| --- | --- | --- |
| S9.17 f) for any specific earth station or typical mobile earth station in frequency bands above 1 GHz allocated with equal rights to space and terrestrial services, in respect of terrestrial stations, where the coordination area of the earth station includes the territory of another country, with the exception of the coordination under No. S9.15; | This reflects Section III of Article 11 and could also cover S9.15 in the same manner as S9.17 bis, but the 1 GHz limit needs careful consideration. This is also intended to cover section 5.1.4 of Article 5 of Appendix 30A in Regions 1 and 3. The situation in Region 2 is similar but covered within Article 4 of Appendix 30A. Therefore, S9.17 does not need to be changed Exclusion of the Appendix S30 bands in the corresponding part of Appendix S5 is valid, since this case is dealt with under the procedure for modification of Appendix S30 plans. This is redundant however, since there is no specific earth station in the BSS. Since Appendix S30A bands are covered in the corresponding part of Appendix S5, there is duplication between S9.17 and that part of the procedure for modification of Appendix S30A Plans dealing with the protection of terrestrial services, in the cases where specific earth stations are considered. Two approaches can be used to resolve this inconsistency : - delete that part of the procedure for modification of Appendix S30A Plan and replace it by S9.17. - exclude Appendix S30A bands from Appendix S5 table on S9.17. |  |
| S9.17bis g) for any specific earth station, in respect of other earth stations operating in the opposite direction of transmission, in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission | This is the extension of Section III of Resolution 46 to all cases of earth stations operating in opposite directions of transmissions. This could reflect Section 7.2 of Article 7 of Appendix 30A as well, but the coordination threshold in Appendix 30A is different. A new line may therefore be introduced under Appendix S5 table to reflect the specific case of Appendix S30A bands, unless Appendix S7 can be modified by WRC-97 to cover this case as well, which would be preferable. The case of a transmit FSS earth station (at 12.5-12.75 GHz in Region 1) interfering into a receive BSS earth station also needs to be covered, which is currently not the case. Appendix S7 should therefore also cover this case with the aim to protect the BSS service area, in a way similar to the method use in the case of S9.19. The reference to the frequency bands of S9.11bis, made in the corresponding part of Appendix S5, in the calculation method column, is irrelevant, since it refers to threshold conditions to protect the terrestrial services. The specific conditions to trigger coordination between earth stations should appear in Appendix S5 rather than in Article S9. Since Appendix S30A bands are covered in the corresponding part of Appendix S5, there is duplication between S9.17 and that part of the procedure for modification of Appendix S30A Plans dealing with the protection of other space services operating in the opposite direction of transmission, in the cases where specific earth stations are considered. Two approaches can be used to resolve this inconsistency : - delete that part of the procedure for modification of Appendix S30A Plan and replace it by S9.17bis. - exclude Appendix S30A bands from Appendix S5 table on S9.17bis. A similar problem arises in relation to Appendix S30 in the band 12.5-12.7 GHz. |  |
| S9.18 h) for any transmitting station of a terrestrial service in the bands referred to in No. S9.17 within the coordination area of an earth station, with the exception of the coordination under No. S9.16 and S9.19 ; | This is intended to reflect Section IV of Article 11. Could also cover S9.16 in the same manner as S9.17 bis, which would avoid the exception at the end of the paragraph.. A new provision S9.18bis might be introduced (see below) in order to reflect Article 6 of Appendix 30A (coordination of receive terrestrial stations) and the associated conditions introduced in Appendix S5. Is this warranted, in view of the fact that receive terrestrial stations do not get this treatment with other space services ? The exclusion of Appendices 30 and 30A bands in the corresponding part of Appendix S5 is not valid, since both bands may have unplanned space services sharing the band with terrestrial services. In order to be consistent with S9.19, S9.18 should exclude the scope of S9.19. |  |
| S9.18bis h) for any receiving station of a terrestrial service allocated with equal rights to the fixed-satellite service for feeder finks to the broadcasting-satellite service, in the frequency bands and regions where this latter service is subject to Appendix 30A Plan; | This is intended to reflect Article 6 of Appendix 30A if such a type of coordination was to be maintained. In Regions 1 and 3, receiving terrestrial stations may coordinate with specific feeder link earth stations (6.1). The earth station administration has 3 months to provide the details of such earth stations (6.3). In Region 2, when the Plan contains information on specific earth stations, this shall be used to calculate the interference. Otherwise, the earth station administration has three months to provide the details of such earth stations, on request. |  |
| S9.19 i) for any transmitting station of a terrestrial service in a frequency band shared on an equal primary basis with the broadcasting-satellite service , in the frequency bands and regions where this latter service is subject to Appendix S30 Plan; | This is intended to reflect Article 6 of Appendix 30. As highlighted by the frequency bands mentioned in the associated part of Appendix S5, it is also intended to provide for the protection of the BSS against terrestrial service in unplanned BSS bands. However, this extension to unplanned BSS is not foreseen under Resolution 33 and might raise sovereignty issues or require provisions to notify BSS earth stations. It may therefore be more consistent to avoid such extension explicitly and delete the reference to S9.11 frequency bands in the corresponding part of Appendix S5.' |  |
| S9.21 j) for any station of a service for which the requirement to seek the agreement of other administrations is included in a footnote of the Table of Frequency Allocations referring to this provision. | This is intended to reflect Article 14. |  |

TABLE S5.1

Technical conditions for coordination (See Article S9)

| Reference of Article S9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/Condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. S9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in a frequency band and in a region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a region where this service is not subject to a Plan, with the exception of the coordination under No S9.17bis | Any frequency band allocated to a space service, where this service is not subject to a Plan, | Value of DT/T exceeds 6% | Appendix S8 |  |
| No. S9.8 GSO/GSO | A transmitting space station using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30 Plan for the broadcasting-satellite service, in respect of this Plan; | 11.7 - 12.2 GHz (R2) 12.2 - 12.7 GHz (R3) 12.5 - 12.7 GHz (R1) | i) There is an overlap in the necessary bandwidths of the space stations ; and ii) the powerflux-density (pfd)of the transmitting space station , assumed under free space propagation conditions, exceeds, on the territory of another administration located in another region, the following values : . -147 dB(W / m2 / 27 MHz) for 0° £ q < 0.44° , -138 + 25 log q dB(W / m2 / 27 MHz) for 0.44° £ q < 19.1° , -106 dB(W / m2 / 27 MHz) for 19.1° £ q , where q is the difference in degrees between the longitudes of the space stations. | Check by using the assigned frequencies and bandwidths; | - |

| No. S9.9 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in any region and frequency band where this service is not subject to a Plan, in a frequency band shared on an equal primary basis with the Appendix S30A Feeder Link Plan for the broadcasting-satellite service, in respect of this Plan, with the exception of the coordination under No S9.17bis.; | 17.7 - 18.1 GHz (R1) 17.7 - 18.1 GHz (R3) 17.7 -18.1 GHz (R2) | i) Value of DTs/Ts exceeds 4%and ii) geocentric inter-satellite angular separation is less than 3° or greater than 150° | i) Case II of Appendix S8 ii) Annex I of Appendix S8 | The threshold/ conditions do not apply when the geocentric angular separation, between the transmitting space station and the receiving feeder link space station , exceeds 150° of arc and the free-space power flux-density of the transmitting space station does not exceed a value of -137 dB(W/m2/MHz) on the surface of the Earth at the equatorial limb. . |
| --- | --- | --- | --- | --- | --- |
|  |  | 17.8-18.1 GHz (R2) | Value of DTs/Ts exceeds 6% | i) Case I of Appendix S8 |  |

| No. S9.11 GSO/terrestrial | A space station in the BSS, in any band shared on an equal primary basis with terrestrial services and where the broadcasting-satellite service is not subject to a plan, , in respect of terrestrial services | 620 - 790 MHz 1 452 - 1 492 MHz 2 310 - 2 360 MHz 2 520 - 2 655 MHz 2 655 - 2 670 MHz 12.5 - 12.75 GHz (R3) 177- 17.8 GHz (R2) 21.4 - 22 GHz (R1, R3) 40.5 - 42.5 GHz 84 - 86 GHz | Condition: Bandwidths overlap | Check by using the assigned frequencies and bandwidths |  |
| --- | --- | --- | --- | --- | --- |
| No. S9.16 terrestrial/ non-GSO | A transmitting station of a terrestrial service non-geostationary-satellite network in frequency bands for which a footnote refers to S9.11bis | See Table S5-1A | Transmitting terrestrial station is situated within the coordination area of a receiving earth station which has already been coordinated |  | The coordination area of the affected earth station has already been determined using the calculation method of No. S9.15 |
| No. S9.17 GSO, non-GSO/ terrestrial | A specific earth station or a typical mobile earth station in frequency bands above 1 GHz allocated with equal rights to space and terrestrial services in respect of terrestrial stations. | Any frequency band allocated to a space service, except : 11.7 - 12.5 GHz (R1) 11.7-12.2 GHz (R3) 12.2 - 12.7 GHz (R2) | The coordination area of the earth station covers the territory of another administration | Appendix S7 (For earth stations in the radiodetermination-satellite service (RDSS) in the bands 1 610 - 1 626.5, 2 483.5 - 2 500 and 2 500 - 2 516.5 MHz, see Remarks column) 1) The coordination area of aircraft earth stations is determined by increasing the service area by 1 000 km with respect to the aeronautical mobile service (terrestrial) or 500 km with respect to terrestrial services other than the aeronautical mobile service 2) For receiving earth stations in the meteorological-satellite service in frequency bands shared with the meteorological aids service, the coordination distance is considered to be the visibility distance as a function of the earth station horizon elevation angle for a radiosonde at an altitude of 20 km above mean sea level, assuming 4/3 Earth radius | Note: For RDSS earth stations, a uniform coordination distance of 400 km corresponding to an airborne earth station shall be used. In cases where the earth stations are all ground-based, a coordination distance of 100 km shall be used. . |

| No. S9.17bis GSO, non-GSO/ GSO, non-GSO | A specific earth station in respect of other earth stations operating in the opposite direction of transmission in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission, | Any frequency band allocated to a space service | The coordination area of the earth station covers the territory of another administration or the earth station is located within the coordination area of an earth station | i Appendix S7 (WRC-97) (a special section needs to be included in Appendix S7 to protect the service area in the case of Appendices 30 and 30A) |  |
| --- | --- | --- | --- | --- | --- |
| No. S9.18 terrestrial/GSO, non-GSO | Any transmitting station of a terrestrial service in a frequency band allocated with equal rights to terrestrial and space radiocommunication services, with the exception of the coordination under S9.19.. | Any frequency band allocated to a space service, with the exception of the frequency bands covered under S9.19. | Transmitting terrestrial station is situated within the coordination area of areceiving earth station | See remarks | The coordination area of the affected earth station has already been determined using the calculation method of No. S9.17 |
| S9.18bis terrestrial/GSO, NGSO | h) for any receiving station of a terrestrial service shared allocated with equal rights to Feeder Links for the broadcasting-satellite service, in the frequency bands and regions where this latter service is subject to Appendix 30A Plan; | 17.7-18.1 GHz (R1 and 3) 17.7-17.8 GHz (R2) 14.5-14.8 GHz (R1 outside Europe, R2, R3) | Receiving terrestrial station is located within the coordination area of the closest feeder link earth which could be located on the boarder of the territory of another administration, and within the service area of the feeder link assignment in conformity with the appropriate regional Plan in Appendix 30A. |  | Appendix S7 |
| No. S9.19 terrestrial/GSO, non-GSO | A transmitting station of a terrestrial service in a frequency band allocated with equal rights to the broadcasting satellite service, in the frequency bands and regions where this latter service is subject to Appendix S30 Plan; . | 11.7 - 12.7 GHz (R1, 2 and 3) - reason : Inter-regional sharing not taken into account. | i) Necessary bandwidths overlap; and ii) [the pfd of the terrestrial station at the edge of the BSS service area exceeds the permissible level] | i) Check by using the assigned frequencies and bandwidths ii) Appendix S7 (WRC-97) [a special section of Appendix S7 needs to be developed replacing Annex 3 to Appendix S30] for the band 11.7-12.7 GHz. |  |
| No. S9.21 terrestrial, GSO, non-GSO/ terrestrial, GSO, non-GSO | A station of a service for which the requirement to obtain the agreement of other administrations is included in a footnote to the Table of Frequency Allocations, referring to No. S9.21 | Band(s) indicated in the relevant footnotes | Condition: Incompatibility established by the use of Appendices S7, S8, technical annexes of Appendices S30, S30A and S30B, pfd values specified in some of the footnotes, other technical provisions of the Radio Regulations or ITU-R Recommendations as appropriate | Methods specified in, or adapted from, Appendices S7, S8, S30, S30A, S30B, other technical provisions of the Radio Regulations or ITU-R Recommendations |  |

1 	The power to be taken into account is obtained by adding the values specified in columns 8 and 9 of the Plan.

1 	With respect to this Annex, except for sub-section 8 c), the power flux-density limits relate to the values which would be obtained assuming free-space propagation conditions.

With respect to sub-section 8 c) of this Annex, the limits relate to the power flux-density which would be obtained assuming clear-sky propagation conditions using the method contained in Annex 5 of Appendix S30.

In the frequency band 12.5-12.7 GHz, in Region 1, these limits are applicable only to the territories of administrations mentioned in S5.494 [RR 848] and S5.496 [RR 850] of the Radio regulations.

In the frequency band 12.5-12.7 GHz, in Region 1, these limits are applicable only to the territories of administrations mentioned in S5.494 [RR 848] and S5.496 [RR 850] of the Radio regulations.

See Resolution 34.

2

For the definition of the equivalent protection margin, see Section 1.7 of Annex 3 to this Appendix.

For the definition of the overall equivalent protection margin, see Section 1.14 of Annex 5 to Appendix 30 (Orb-85).

In Regions 1 and 3, for the application of the procedures of Appendix S7, the e.i.r.p. for the feeder-link earth station is the sum of the values specified in columns 8 and 9 of the Plan.

4

*

*	*

*

**