Doc SC-3/5

20 December 1996

France

Possible modifications of Article T4 and Appendix S6

in relation to Appendix S30 and S30A

Introduction of the concept of group

The following text is intended to propose possible additions to Article T4 to introduce the concept of group.

ADD	T4.7 ter

In proposing a modification to a plan, an administration or a group of administrations may indicate that several assignments, either already included in the plan or proposed to be included in the plan, will be part of a group. In this case, the aggregate interference produced by all emissions from that group into any channel shall not exceed the worst case interference produced in the same channel by any of the assignments which are part of the group.

ADD	Appendix S6

Note to be called in all occurrences where the terms EPM or OEPM are used :

a)	When an assignment appearing in the plan or proposed as a modification to the plan is part of a group, the equivalent protection margin or the overall equivalent protection margin to be used for the application of Article [T4] and Resolution 42 (Orb-85) shall be calculated on the following basis:

-	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included; and

-	for the calculation of interference from assignments belonging to a group to assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.

b)	If an administration notifies the same frequency in more than one beam of a group for use at the same time, the aggregated C/I produced by all emissions from that group shall not exceed the C/I calculated on the basis of a) above.