| µ § INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| RADIOCOMMUNICATION STUDY GROUPS | Document 10/1-E 30 November 1995 Original: English |

Radiocommunication Study Group 10

QuEstions assigned to Radiocommunication STUdy Group 10

According to the revised version of Resolution ITU-R 5 of the Radiocommunication Assembly, (Geneva, 16-20 October 1995) (RA-95), the texts of the Questions are to be found in Document 1 of the series of documents for the next study period.

Attached please find the Questions assigned to Radiocommunication Study Group 10 for 1996-1997.

The following extract from Resolution ITU-R 5 gives the definition of categories of Questions:

C: 	Conference oriented Questions associated with work related to specific preparations for, and decisions of, world and regional radiocommunication conferences:

C1:	Very urgent and priority studies, required for the World Radiocommunication Conference to be held within the next two-year period;

C2:	urgent studies, expected to be required for other Radiocommunication Conferences.

S:	Questions which are intended to respond to:

–	matters referred to the Radiocommunication Assembly by the Plenipotentiary Conference, any other conference, the Council, the Radio Regulations Board (see Note);

–	advances in radiocommunication technology or spectrum management;

–	changes in radio usage or operation:

S1:	urgent studies which are intended to be completed within two years;

S2:	important studies, necessary for the development of radiocommunications;

S3:	required studies, expected to facilitate the development of radiocommunications.

Note - Where appropriate, Questions maintained (but unmodified) by the Radiocommunication Assembly have been editorially updated to reflect the working methods given in Resolution ITU-R 1 and the current status of texts within the Study Group.  In such cases, the version number and date of the Question have remained unchanged.

Questions assigned by the Radiocommunication Assembly to Study Group 10

Broadcasting service  –  sound

| Question ITU-R No. | Title | Category | Page number |
| --- | --- | --- | --- |
| 44-2/10 | LF, MF and HF sound broadcasting | S3 | 5 |
| 49-2/10 | Receivers for sound broadcasting below 30 MHz | S3 | 7 |
| 55-1/10 | Protection ratios in LF, MF and HF broadcasting | C2 | 8 |
| 56/10 | Minimum usable field strength in LF, MF and HF broadcasting | C2 | 10 |
| 57/10 | Sky-wave reception in LF, MF and HF broadcasting | C2 | 11 |
| 58-1/10 | Coverage in LF, MF and HF broadcasting | C2 | 12 |
| 60-1/10 | Transmission of supplementary information in amplitude modulation sound broadcasting | S2 | 13 |
| 61-1/10 | Single-sideband (SSB) system for broadcasting (HF) | C2 | 14 |
| 62-1/10 | Synchronized transmitters in HF sound broadcasting | C2 | 16 |
| 64/10 | System design for HF broadcasting | S1 | 17 |
| 65-1/10 | Short-distance broadcasting in band 7 (HF) in the Tropical Zone | S2 | 18 |
| 67-1/10 | Interference from other services to sound broadcasting in the Tropical Zone | S2 | 19 |
| 68/10 | Modulation level control in frequency-modulation sound broadcasting | S1 | 21 |
| 69/10 | Polarization of emissions in FM sound broadcasting | S2 | 22 |
| 70/10 | Transmission of several programmes with a single transmitter in FM sound broadcasting | S2 | 23 |
| 71/10 | Transmission of supplementary information with a single transmitter in frequency-modulation sound broadcasting | S1 | 25 |

| Question ITU-R No. | Title | Category | Page number |
| --- | --- | --- | --- |
| 73-1/10 | Sound broadcasting in band 8 (VHF) in the Tropical Zone | S2 | 26 |
| 75/10 | Immunity of FM broadcast receivers against interference | S2 | 27 |
| 76-3/10 | Transmitting and receiving antenna at VHF and UHF | S1 | 29 |
| 78-1/10 | Standards for the transmission of several sound signals in one television channel in terrestrial or satellite broadcasting including high-definition and enhanced definition television systems | S1 | 31 |
| 83/10 | Determination of the electro-acoustical properties of studio monitor headphones | S3 | 32 |
| 84-1/10 | Sound systems for the hearing impaired | S2 | 33 |
| 85-2/10 | Subjective assessment of sound quality in broadcasting using digital techniques | S2 | 34 |
| 91-1/10 | Digital recording of sound programmes on magnetic tape for international exchange | S3 | 35 |
| 92-1/10 | Optical recording of sound-broadcasting programmes for international exchange | S3 | 36 |
| 93-1/10 | Characteristics of systems in the broadcasting-satellite service (sound) for individual reception by means of portable and vehicular receivers | C2 | 37 |
| 96-1/10 | Determination of the effects of atmospheric noise on the grade of reception in the Tropical Zone | S2 | 39 |
| 102/10 | Transmission of data information as an alternative to the main programme in frequency-modulation sound broadcasting | S2 | 40 |
| 105-1/10 | Multi-lingual services in multi-channel sound systems | S2 | 41 |
| 106-1/10 | Subjective assessment of sound quality | S2 | 42 |
| 107/10 | Characteristics of terrestrial digital sound-broadcasting systems for reception by vehicular, portable and fixed receivers | S1 | 44 |
| 201-1/10 | Transmitting and receiving antennas at LF and MF | C2 | 46 |

| Question ITU-R No. | Title | Category | Page number |
| --- | --- | --- | --- |
| 202/10 | Receivers for terrestrial FM sound broadcasting | S2 | 47 |
| 203/10 | Antenna characteristics | S3 | 48 |
| 204/10 | Frequency sharing issues related to the introduction of the broadcasting-satellite service (sound) in the frequency range 1-3 GHz | C2 | 49 |
| 205-1/10 | Evaluating fields from broadcasting transmitting systems operating at frequencies below 30 MHz for assessing exposure to non-ionizing radiation | S2 | 51 |
| 207/10 | Standards for digital audio techniques | S2 | 53 |
| 208-1/10 | Low bit-rate audio coding standards | S1 | 54 |
| 209/10 | Parameters and tolerance limits for the international exchange of sound programmes | S3 | 56 |
| 210/10 | Objective perceptual quality assessment methods | S1 | 57 |
| 211/10 | System parameters for multichannel sound systems | S3 | 59 |
| 212/10 | Planning procedures for HF broadcasting | C2 | 60 |
| 213/10 | Synchronized transmitters in LF and MF sound broadcasting | S2 | 62 |
| 214/10 | Unified identification label for international exchange of sound recordings | S2 | 63 |
| 215/10 | Recording of sound programmes for international exchange | S3 | 65 |
| 216/10 | Archival of sound programmes in broadcasting | S1 | 66 |
| 217/10 | Digital broadcasting in AM bands | S2 | 68 |
| 218/10 | Broadcasting of film programmes with multichannel sound | S2 | 69 |

µQUESTION  ITU-R  44-2/10

LF,  MF  AND  HF  SOUND  BROADCASTING

(1982-1990-1994)

Q. ITU-R 44-2/10

The ITU Radiocommunication Assembly,

considering

a)	that any sound broadcasting system should specify the essential emission and reception characteristics taking into account the effects of propagation;

b)	that the system, or systems, chosen should permit efficient use of the frequency bands available, whilst maintaining reasonable receiver costs;

c)	that apart from the double-sideband amplitude-modulation system, other systems may be used;

d)	that sky-wave and/or ground-wave reception may be required,

e)	that the WARC-79, in its Resolutions and Recommendations, invited the ex-CCIR to study this problem, with specific reference to:

–	Resolution No. 508: Convening of a World Administrative Radio Conference (HF broadcasting planning);

–	Recommendations:

No. 8:	Automatic identification of stations,

No. 64:	Protection ratios and minimum field strengths required in the various services,

No. 69:	Frequency tolerances of transmitters,

No. 70:	Studies of the technical characteristics of equipment, bandwidths and channel spacing,

No. 501:	Introduction of single-sideband techniques in the HF broadcasting bands in preparation for the WARC (HF broadcasting),

No. 503:	Studies of “out-of-band spectrum” in the HF bands;

f)	that the WARC HFBC-87 in its Recommendations invited the ex-CCIR to continue its studies on the following items:

–	Recommendations:

No. 503(Rev.) (HFBC-87)	HF broadcasting,

No. 510 (HFBC-87)	Planning parameters for DSB system,

No. 515 (HFBC-87)	Introduction of transmitters and receivers capable of DSB and SSB,

No. 516 (HFBC-87)	Use of synchronized transmitters in HFBC,

No. 517 (HFBC-87)	Relative RF-protection ratios for SSB;

g)	that in the broadcasting service it is necessary to provide coverage of an intended service area, taking into account the levels of the wanted signal and of any interfering signals,

decides  that the following Question should be studied

1.	What LF, MF and HF sound broadcasting systems should be standardized?

2.	What should be the characteristics of these systems with respect to:

–	emission (type of modulation, audio-frequency bandwidth and necessary bandwidth of emission, dynamic compression, pre-emphasis, limiting, channel spacing etc.);

–	reception (principle of demodulation, audio-frequency response, dynamic expansion, de-emphasis, intermediate frequency, etc.);

–	the transmission of supplementary information?

3.	What factors should be taken into account in the selection of:

–	transmitter power;

–	type of transmitting antenna;

–	operating frequency band,

in order to achieve satisfactory coverage of an intended service area, noting that the three items listed above are interdependent and affect the efficiency of spectrum utilization?

4.	Should a system differing from the present system be chosen, what are the consequences resulting from that choice during any transition period on:

–	existing receivers;

–	receivers constructed in conformity with the new system?

5.	What are the criteria for sharing between sound broadcasting and other services?

Note 1 – See Reports ITU-R BS.458, ITU-R BS.1059, ITU-R BS.1061 and Recommendations ITU-R BS.80, ITU-R BS.411, ITU-R BS.498, ITU-R BS.597, ITU-R BS.598, ITU-R BS.638, ITU-R BS.639, ITU-R BS.640, ITU-R BS.702, ITU-R BS.703, ITU-R BS.705 and ITU-R BS.706.

further decides

1.	that the results of the above studies should be addressed to:

–	update existing Recommendation(s);

–	prepare new Recommendation(s);

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  49-2/10

RECEIVERS  FOR  SOUND  BROADCASTING  BELOW  30  MHz

(1982-1990-1994)

Q. ITU-R 49-2/10

The ITU Radiocommunication Assembly,

considering

a)	that efficient planning must take into account the main parameters of the receiving installations;

b)	the information contained in Recommendation ITU-R BS.703 may need to be complemented and/or updated;

c)	that Question ITU-R 59/10 and Question ITU-R 201/10 are addressing, inter alia, the studies on receiving antennas respectively for LF, MF and HF sound broadcasting,

decides  that the following Question should be studied

1.	What are the main characteristics of receiving installations which may be useful for planning?

2.	What are the minimum performance specifications for receiving installations permitting a sound quality compatible with ITU-R specifications, for the various types of sound-broadcasting systems:

–	double-sideband amplitude modulation;

–	single-sideband amplitude modulation (including the technical and economic problems concerning receivers that would arise with the introduction of this type of modulation in the HF broadcasting bands);

–	digital modulation?

These technical characteristics should be defined for the three cases of:

–	individual reception;

–	community reception;

–	community listening.

Note 1 – The minimum performance specifications should take account of the latest International Electrotechnical Commission (IEC) work.

Note 2 – This Question deals only with terrestrial sound broadcasting until such time when the system characteristics for satellite sound broadcasting are determined.

further decides

1.	that the results of the above studies should be addressed to:

–	update and complement Recommendation ITU-R BS.703;

–	prepare new Recommendation(s);

2.	that the above studies should be completed by 1998.

§

§§§§§§

µQUESTION  ITU-R  61-1/10

SINGLE-SIDEBAND  (SSB)  SYSTEM  FOR  BROADCASTING  (HF)

(1990-1993)

Q. ITU-R 61-1/10

The ITU Radiocommunication Assembly,

considering

a)	that the application of single-sideband technique allows for more efficient utilization of the radio spectrum;

b)	Recommendation No. 501 of the World Administrative Radio Conference (Geneva, 1979) (WARC-79);

c)	that the Second Session of the World Administrative Radio Conference for the Planning of the HF Bands Allocated to the Broadcasting Service (Geneva, 1987) (WARC HFBC-87) recommends to administrations that new transmitters which are installed after 31 December 1990 should, as far as possible, be capable of working either in both modes, SSB and DSB, or in the SSB mode alone (SSB with the possibility of both a 6 dB and a 12 dB carrier reduction relative to peak envelope power);

d)	that Recommendation No. 515 of the WARC HFBC-87 invites the ITU-R to complete its studies into receivers for SSB;

e)	the content of Recommendation ITU-R BS.640 “Single-sideband (SSB) system for HF broadcasting”,

decides  that the following Question should be studied

1.	What technical characteristics of a single sideband HF sound-broadcasting system can be recommended?

2.	What economies in spectrum utilization should be expected from the use of SSB only in HF sound broadcasting?

3.	What technical and economic problems concerning transmitters and receivers can arise with the introduction of single-sideband technique in the HF bands allocated to broadcasting?

Note 1 – Some of the technical studies required for the transition period when both SSB and DSB systems will be in operation include the following:

–	channel spacing;

–	co-channel and relative radio-frequency protection ratios;

–	equivalent sideband power;

–	SSB carrier reduction;

–	receiver compatibility;

–	statistical characteristics of the SSB signal, in particular the spectral distribution of energy,

further decides

1.	that the results of the above studies should be addressed to:

–	complement Recommendation ITU-R BS.640 with the specification of the technical characteristics of an SSB HF sound-broadcasting system;

–	prepare an Annex to Recommendation ITU-R BS.640 including the technical information related to decides 2 and 3;

2.	that the above studies should be completed by 1999.

§§§§§§§§§§

µQUESTION  ITU-R  75/10*

IMMUNITY  OF  FM  BROADCAST  RECEIVERS  AGAINST  INTERFERENCE

(1990)

Q. ITU-R 75/10

The ITU Radiocommunication Assembly,

considering

(a)	that both the number of broadcast FM transmitters and the usable field strength are increasing;

(b)	that in areas of high population density, coexistence of FM broadcasting services, carried out by various stations situated in different locations and with great differences in the radiated powers, frequently occurs;

(c)	that the situation involves difficulties particularly in receiving the weaker signals in the presence of one or more high intensity signals;

(d)	that the above difficulties, due to the saturation effect in the RF stages of receivers, are greater in the case of particular combinations of signals;

(e)	that transmission standards defined in Recommendation 450, especially concerning maximum frequency deviation, may not be followed by some stations;

(f)	that the use of compression systems may have an impact on protection ratios;

(g)	that excessive radiated power levels may adversely affect desired reception;

(h)	that supplementary sub-carrier signals may be transmitted with the main channel by a single transmitter,

unanimously decides  that the following question should be studied

1.	what are the receiving conditions of FM broadcasting signals, mono and stereo, in the presence of various signals of different intensities and frequencies, with particular attention to the process by which interference occurs, in particular, in the following cases:

1.1	interfering transmitter exceeding the maximum permitted deviation (75 kHz or 50 kHz as appropriate), due to imperfect limiter or clipping equipment (short peaks) or deliberately;

1.2	average deviation increasing without exceeding the maximum deviation (e.g. compression);

1.3	critical carrier frequency differences (e.g. 450-500 kHz; 5.35 MHz; 10.7 MHz; 21.4 MHz) existing either for wanted and unwanted, or for two unwanted transmitters;

1.4	frequency differences greater than 400 kHz;

1.5	use of the newly-proposed United States companding system, as described in Report 300, § 3.2;

1.6	non-compliance with the RF protection ratio for various frequency differences;

1.7	when two or more interfering transmitters cause intermodulation products within the receiver, including intermodulation in the case of strong wanted and/or strong interfering signals;

2.	what are the protection ratios values to be expected:

–	when levels of signals strength higher than the minimum usable field strengths, given in Recommendation 412, for wanted or unwanted signals are used;

–	when the interfering transmitter is carrying the same programme as the desired transmitter;

–	when the desired transmitter is carrying RDS signals (see Recommendation  643) or supplementary information signals (see Report 463)?

further decides  that

1.	the result of the above studies should be included in a Recommendation(s);

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

§Question ITU-R 76-3/10*

Transmitting and Receiving Antennas at VHF and UHF

(1990-1993-1994-1995)

The ITU Radiocommunication Assembly,

considering

a)	that transmitting and receiving antenna characteristics are required for frequency planning;

b)	that planning the most efficient spectrum use often requires establishing the antenna radiation patterns to a high degree of accuracy;

c)	that planning the most efficient spectrum use may imply using the same antenna for several emissions;

d)	that in establishing transmitting radiation patterns, account must be taken of the influence of the supporting structure, significant nearby structures and the electrical stability of the antenna assembly;

e)	that it is essential to verify the antenna radiation pattern by measurements;

f)	that the RF power density in the vicinity of transmitting antennas is significant;

g)	that the Radiocommunication Study Groups have already made significant studies on the radiation characteristics of LF, MF, HF and satellite antennas;

h)	that comprehensive information on the characteristics of transmitting and receiving antenna systems at VHF and UHF (including those for Direct Satellite Broadcasting use at frequencies above 1 GHz) is not readily available in the ITU Radiocommunication Volumes,

decides that the following Question should be studied

1	What are the radiation patterns of VHF and UHF transmitting and receiving antenna systems (including those for Direct Satellite Broadcasting use at frequencies above 1 GHz)?

2	What is the influence of supporting structures and significant nearby structures on the technical characteristics of antennas in the VHF and UHF bands?

3	What differences are to be expected between theoretical and practical performance due to practical aspects of VHF and UHF antennas?

4	What are the methods of calculation of the electric and magnetic field strengths in the vicinity of transmitting antennas?

5	What computer-based procedures can be recommended to give, in a standardized form, the gain and directivity patterns of transmitting and receiving antennas (including those for Direct Satellite Broadcasting use at frequencies above 1 GHz) in common use at VHF and UHF?

further decides

1	that the results of the above studies should be addressed to prepare a new Recommendation on:

-	receiving antennas at VHF and UHF (including those for Direct Satellite Broadcasting use at frequencies above 1 GHz);

-	reference receiving antenna patterns for use in planning;

-	transmitting antennas at VHF and UHF (including those for Direct Satellite Broadcasting use at frequencies above 1 GHz);

2	that the above studies should be completed by 1998;

3	that administrations be invited to provide appropriate measurements of VHF and UHF antenna radiation patterns (including those for Direct Satellite Broadcasting use at frequencies above 1 GHz).

µQUESTION  ITU-R  78-1/10

STANDARDS  FOR  THE  TRANSMISSION  OF  SEVERAL  SOUND  SIGNALS  IN  ONE
TELEVISION  CHANNEL  IN  TERRESTRIAL  OR  SATELLITE  BROADCASTING
INCLUDING  HIGH-DEFINITION  AND  ENHANCED  DEFINITION  TELEVISION  SYSTEMS

(1990-1993)

Q. ITU-R 78-1/10

The ITU Radiocommunication Assembly,

considering

a)	that there is a requirement to transmit more than one sound signal including stereophony with a television picture;

b)	that within a television channel as used in present systems several sound channels could be accommodated;

c)	that the use of some of these additional possibilities could necessitate changes to the present television standards;

d)	that changes affecting existing transmissions can only be introduced if compatibility with existing receivers, at least during a sufficiently long transition period, is retained;

e)	that a prerequisite of a new system is that the sound signals should serve at least the same coverage area as the vision signal;

f)	that a new system could also provide for the transmission of sound programmes, independent from the picture, or in place of a television picture;

g)	that the technical quality of the sound transmission can be improved;

h)	that the improvements in picture quality associated with high-definition and enhanced definition television may warrant examination of the sound system needed to keep in step with the higher level of realism available in the picture,

decides  that the following Question should be studied

1.	What are the unified* standards for the transmission of several sound signals to be incorporated in one television channel in terrestrial or satellite broadcasting including the use of digital modulation and the case where some of those sound signals are to be integrated into the video waveform?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1999.

§§§§

µQUESTION  ITU-R  91-1/10

DIGITAL  RECORDING  OF  SOUND  PROGRAMMES  ON  MAGNETIC
TAPE  FOR  INTERNATIONAL  EXCHANGE

(1990-1993)

Q. ITU-R 91-1/10

The ITU Radiocommunication Assembly,

considering

a)	that digital recording techniques are suitable for high quality programme recording;

b)	that present magnetic recording devices and materials have adequate characteristics for high density recording on magnetic tape,

decides  that the following Question should be studied

1.	What digital magnetic tape recording format(s) can be recommended for the international exchange of sound programmes?

2.	What operating practices can be recommended for such exchanges?

Note 1 – See Report ITU-R BR.950 and Recommendation ITU-R BR.648.

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1997.

§§§§§§§§§

µQUESTION  ITU-R  202/10

RECEIVERS  FOR  TERRESTRIAL  FM  SOUND  BROADCASTING

(1993)

Q. ITU-R 202/10

The ITU Radiocommunication Assembly,

considering

a)	that efficient planning should take into account the main parameters of the receiving installations;

b)	that a receiving installation consists of a (or several) receiver(s) in connection with an associated antenna;

c)	that Question ITU-R 76/10 addresses the studies on receiving antennas;

d)	that the information contained in Recommendation ITU-R BS.704 should be complemented and/or updated,

decides  that the following Question should be studied

1.	What are the main characteristics of FM receivers together with the associated antenna which are useful in a planning context?

2.	What are the minimum performance specifications for FM receiving installations permitting a sound quality compatible with ITU Radiocommunication specifications?

These technical characteristics should be defined for the two cases of:

–	individual reception;

–	community reception.

Note 1 – To measure these specifications, the appropriate ITU-R Recommendations and latest International Electrotechnical Commission (IEC) Publications should be taken into account.

further decides

1.	that the results of the above studies should be addressed to:

–	update and complement Recommendation ITU-R BS.704;

–	prepare (a) new Recommendation(s);

2.	that the above studies should be completed by 1999.

§§§§§QUESTION ITU-R 208-1/10

LOW BIT-RATE AUDIO CODING STANDARDS

(1993-1995)

The ITU Radiocommunication Assembly,

considering

a)	that a number of different low bit-rate coding systems are currently being proposed for digital audio applications;

b)	that recent progress in digital audio coding techniques allows for a very considerable degree of bit-rate reduction to be achieved while maintaining high quality;

c)	that applications exist for low bit-rate audio coding systems for digital audio broadcasting, television sound (especially accompanying HDTV and EDTV) and for storage of signals;

d)	that the requirements for the various broadcasting applications mentioned in § c) might be very different and should be carefully defined in view of the planned services;

e)	that the performance of the whole broadcasting chain should be considered, and multiple transcoding between low bit-rate standards used for production, transmission links and broadcasting should be avoided as far as possible,

decides that the following Question should be studied

1	What are the sound quality and other requirements needed for production including recording, for transmission links and for various broadcasting applications from terrestrial transmitters and for satellite, including multichannel audio formats as contained in Recommendation ITU-R BS.775?

2	What are the bit-rate reduction techniques that would satisfy the quality level and other requirements for each of the applications listed in § 1 while making efficient use of storage or transmission media?

3	What are the techniques which would maximize interoperability between the various parts of the broadcasting chain?

4	What is the nature of signal impairments due to low bit-rate coding techniques, in particular after a number of cascaded codecs?

5	What are the lossless bit-rate reduction techniques which could be applied to audio coding, especially for studio application?

6	What methods could be used to minimize the incompatibilities between the various low bit-rate coding techniques and what interface for low bit-rate audio can be recommended to avoid transcoding digital signals in linear format?

7	What methods of transcoding between low bit-rate coding techniques adopted by ITU-R can be recommended, if unavoidable?

Note 1 - Telecommunication Standardization Study Group 9 is studying transmission line aspects of Question ITU-T 25/9. There should be close cooperation between both Study Groups.

further decides

1	that the results of the above studies should be included in (a) Recommendation(s);

2	that further studies on low bit-rate coding systems at bit rates below or equal to 192 kbit/s per stereo channel for emission application, and below or equal to 60 kbit/s per mono channel for commentary links should be completed by 1999;

3	that further studies on low bit-rate coding for multichannel audio systems should be completed by 1998.

µQUESTION  ITU-R  209/10

PARAMETERS  AND  TOLERANCE  LIMITS  FOR  THE  INTERNATIONAL
EXCHANGE  OF  SOUND  PROGRAMMES

(1993)

Q. ITU-R 209/10

The ITU Radiocommunication Assembly,

considering

a)	that Recommendation ITU-R BS.644 specifies the audio quality parameters of a high quality sound-programme transmission chain;

b)	that measurement methods continue to evolve;

c)	that perceptual objective quality assessment methods may eventually lead to measurements which relate consistently to subjectively assessed quality,

decides  that the following Question should be studied

1.	What are the parameters and tolerances which should characterize the international sound-programme exchange?

2.	What are the most appropriate measuring methods for broadcasting use?

Note 1 – See Recommendations ITU-R BS.468, ITU-R BS.644 and ITU-R BS.645.

further decides

1.	that the results of the above studies should be addressed to:

–	update and complement Recommendation ITU-R BS.644;

–	prepare (a) new Recommendation(s);

2.	that evolution of new test methods is likely to continue for some years, necessitating occasional revisions of Recommendation ITU-R BS.644;

3.	that it would be appropriate to consider updating Recommendation ITU-R BS.644 by 1999.

Note 1 – Telecommunication Standardization Study Group 9 is studying transmission line aspects of Question ITU-T 23/9. There should be close cooperation between both Study Groups.

§§§§§

µQUESTION  ITU-R  213/10

SYNCHRONIZED  TRANSMITTERS  IN  LF  AND  MF  SOUND  BROADCASTING

(1994)

Q. ITU-R 213/10

The ITU Radiocommunication Assembly,

considering

a)	that the use of synchronized-frequency transmitter operation may lead to improved efficiency in the use of the spectrum,

decides  that the following Question should be studied

1.	Under what technical conditions can synchronized transmitters (i.e. on the same frequency and modulated by the same programme) be used for sound broadcasting in band 5 (LF) and band 6 (MF)?

2.	To what extent can the coverage areas of synchronized transmitters overlap in practice?

3.	What combination(s) of types of antennas may be used for synchronized operations?

further decides

1.	that the results of the above studies should be addressed to prepare a Recommendation on synchronized operation of LF and MF sound broadcasting transmitters;

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  214/10

UNIFIED  IDENTIFICATION  LABEL  FOR  INTERNATIONAL
EXCHANGE  OF  SOUND  RECORDINGS

(1994)

Q. ITU-R 214/10

The ITU Radiocommunication Assembly,

considering

a)	that Recommendation ITU-R BR.408 specifies minimum programme information to be provided with recorded audio tapes; Recommendation ITU-R BR.469 specifies minimum information to be provided with analogue television tapes and gives an example of suitable labels; Recommendation ITU-R BR.779 specifies minimum information to be provided with digital television tapes; Recommendation ITU-R BR.265 specifies minimum information to be provided on the identification leader of films;

b)	that the minimum information specified for those different recordings is not always homogeneous, either in its content or in its presentation;

c)	that modern methods used in the production, broadcasting and storage of sound recordings increasingly rely on automatic equipment and machine-readable identification of recording content (e.g. bar-coded information);

d)	that at present most broadcasters operate with their own identification method for recording content, and most manufacturers of automatic broadcast equipment implement their own machine-readable identification method;

e)	that the specification of a unified identification method for recording content would ease international programme exchange, and, if the unified information were machine-readable, it could potentially be ported to existing in-house identification methods by means of simple computer software,

decides  that the following Question should be studied

1.	What is the essential information on programme content to be provided with recorded sound tapes, and films intended for international programme exchange?

2.	How should that information be provided (e.g. by means of:

–	a programme sheet placed in the recording box;

–	a label fixed to the recording box;

–	a label fixed to the tape reel or cassette;

–	a combination of the above)?

3.	How could that information be formatted to be provided in machine-readable form?

4.	Which machine-readable method should be recommended for this application?

5.	How could the essential programme information provided in machine-readable form on internationally exchanged tapes and films, be correlated to similar information currently used by broadcasters for in-house automated operation and archives?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that Recommendations adopted in response to this Question should preferably be aligned with those adopted in response to Question ITU-R 233/11;

3.	that the above studies should be completed by 1998.

§

QUESTION ITU-R 215/10

RECORDING OF SOUND PROGRAMMES FOR INTERNATIONAL EXCHANGE

(1995)

The ITU Radiocommunication Assembly,

considering

a)	that the exchange of sound programmes is very important and extensive and it should be taken into account;

b)	that international standardization of the media and of the recording formats used for programme exchange offers both economical and operational advantages;

c)	that the alignment of operational practices used in connection with the recording and 
play-back of sound programmes is highly desirable,

decides that the following Question should be studied

1	Which recording media are suited for the international exchange of sound programmes?

2	Which recording formats offer the greatest advantages for use in the international exchange of programmes?

3	Which operational practices should be recommended to ease the international exchange of programmes?

further decides

1	that the results of the above studies should be included in (a) Recommendation(s);

2	that the above studies should be completed by 1997.

QUESTION ITU-R 216/10

ARCHIVAL OF SOUND PROGRAMMES IN BROADCASTING

(1995)

The ITU Radiocommunication Assembly,

considering

a)	that preservation of important sound programme recordings is essential and it forms an integral part of a broadcast organization;

b)	that instant access to the huge archive material is not possible and the process is time consuming;

c)	that the digitization of an audio signal leads to the improvement of quality of the signal;

d)	that the storage medium plays an important role in determining the life span of archive material;

e)	that impairment to audio signals could be minimized and the cost of transcoding equipment could be reduced if there is a simple relationship between the coding standards used for production, transmission, emission and archiving;

f)	that Recommendations on digital audio coding standards are already available both for studio and broadcast environments;

g)	that archival equipment may need coding parameters different from those required for the studio and the emission of high quality broadcast signals;

h)	that the quality of old recordings could be improved by using restoration techniques;

j)	that with the advancement in technologies, the restoration technique may itself undergo many improvements and this may require the preservation of original recordings;

k)	that the large channel capacity required for the digital recording of audio signals can be minimized by using bit-rate reduction techniques;

l)	that a number of digital bit-rate reduction techniques are available and ISO/IEC have already set standards for this technique (ISO/IEC 11172-3),

decides that the following Question should be studied

1	In what signal format (digital or analogue) should the archival material be stored for broadcasters purposes?

2	What recording methods are best suited for the storage of programme material in broadcast archives?

3	What digital coding standards should be used if digital storage is chosen, and could  
bit-rate reduction be used?

4	What method could be used for easy and fast access to the archived programmes?

further decides

1	that the results of the above studies should be included in (a) Recommendation(s);

2	that the above studies should be completed by 1998.

QUESTION ITU-R 217/10

DIGITAL BROADCASTING IN AM BANDS

(1995)

The ITU Radiocommunication Assembly,

considering

a)	that the reception quality of AM broadcasting in the LF, MF and HF bands (150 kHz to 26 MHz) suffers from:

-	limited AF bandwidth, partly due to planning constraints;

-	the disturbing effects of sky-wave propagation;

-	man-made noise;

b)	that AM reception quality is relatively poor compared with FM broadcast or CD quality;

c)	that new digital techniques have enabled significant improvements in reception quality to be obtained in other broadcasting bands;

d)	that preliminary consideration indicates possibilities to introduce digital modulation systems in the AM broadcasting bands which can take advantage of low bit rate coding,

decides that the following Question should be studied

1	What new digital modulation systems can be used in the LF, MF and HF bands, to improve the reception quality for national and international coverage?

2	What type of digital coding and modulation system can be used with new transmissions while maintaining compatibility with existing planning of the LF, MF and HF broadcasting bands?

3	What service enhancement is provided by using a digital modulation system compared with existing AM broadcasting?

4	How can a digital modulation system be implemented with existing transmitting equipment?

5	What kind of new services, if any, can be introduced in the AM broadcasting bands using digital modulation?

6	What is the complexity of the dual standard receivers able to receive transmissions using digital modulation and AM systems?

further decides

1	that the results of the above studies should lead to the formulation of an appropriate Recommendation by 1998;

2	that administrations be invited to provide relevant contributions.

QUESTION ITU-R 218/10

BROADCASTING OF FILM PROGRAMMES WITH MULTICHANNEL SOUND

(1995)

The ITU Radiocommunication Assembly,

considering

a)	that television organizations will receive, replay and transmit programmes on film with multichannel sound;

b)	that there are several multichannel sound systems in operation for programmes on film;

c)	that television organizations will receive picture and multichannel sound on film or video tape;

d)	that Study Group 10 is working on multichannel sound systems with or without accompanying picture,

decides that the following Question should be studied

1	What multichannel sound systems exist for cinema presentation?

2	What is the distribution support for multichannel sound for programmes on film?

3	What is the replay equipment necessary for multichannel sound on film?

4	What are the operational practices and experience with the television transmission of multichannel sound from film?

5	How will multichannel sound for programmes on film be exploited for future multichannel television sound systems?

further decides

1	that the results of the above studies should be included in (a) Recommendation(s);

2	that the above studies should be completed by 1998.

µQUESTION  ITU-R  212/10*

PLANNING  PROCEDURES  FOR  HF  BROADCASTING

(1993)

Q. ITU-R 212/10

The ITU Radiocommunication Assembly,

considering

a)	that the Additional Plenipotentiary Conference (Geneva, 1992) (APP-92) in Resolution No. 9 and the Council during its 47th Session requested the Radiocommunication Assembly to establish the work programme and the Study Groups for the new Radiocommunication Sector, including any future work on HF broadcasting;

b)	that the ex-IFRB, in its report to the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92), stressed the difficulties experienced by administrations and the ex-IFRB in implementing the improved HFBC Planning System which is described in Resolution No. 515 Second Session of the World Administrative Radio Conference for the Planning of HF Bands Allocated to the Broadcasting Service (Geneva, 1987) (WARC HFBC-87), and showed that broadcasters’ requirements far outnumbered the channels currently available;

c)	that, in the absence of planning procedures, the additional HF spectrum allocated by the World Administrative Radio Conference (Geneva, 1979) (WARC-79) and WARC-92 has not become available;

d)	that congestion in the HF broadcasting bands continues to increase;

e)	that the report on HF broadcasting by the Radiocommunication Bureau (RA93/PLEN/6)

–	indicates that possible HF broadcasting planning approaches might include an allotment plan approach or an improved procedure under Article 17 of the Radio Regulations and should be studied in the Radiocommunication Sector;

–	emphasizes that under both approaches the keyword for successful planning is “coordination”;

–	concludes that a study of the structure and work procedures of existing HF broadcasting coordination between broadcasters is required;

–	completes the tasks requested of the ex-IFRB through Resolution No. 523 (WARC-92),

decides  that the following Question should be studied

1.	What alternative planning procedures can be used to alleviate HF congestion and optimize the use of the bands allocated to HF broadcasting?

2.	What technical parameters and procedures should be considered in decides 1?

3.	What are the structure, work procedures and effectiveness of existing HF broadcasting coordination mechanisms and could these procedures be used in the development of alternative planning procedures?

4.	Which automated tools would be necessary to apply these alternative planning procedures?

5.	What information is required to enable administrations to evaluate the effectiveness of these alternative planning procedures?

further decides

1.	that the above studies should be completed by 1997.

µQUESTION  ITU-R  211/10

SYSTEM  PARAMETERS  FOR  MULTICHANNEL  SOUND  SYSTEMS

(1993)

Q. ITU-R 211/10

The ITU Radiocommunication Assembly,

considering

a)	that the improvements in picture quality associated with high-definition and enhanced definition television may warrant examination of the sound system needed to keep in step with the higher level of realism available in the picture;

b)	that conventional two-channel stereophonic representation conveys substantial acoustic information by phantom sources, which cannot adequately provide for coincidence of the visual and aural images independent of viewer's location;

c)	the transmission systems with bit-rate reduced coding for multichannel sound transmission are under development in several countries;

d)	that a liaison status exists with the International Organization for Standardization (ISO)/International Electrotechnical Commission (IEC) working on the standardization of a generic multichannel sound system;

e)	that a first Recommendation (ITU-R BS.775) exists, which needs to be extended,

decides  that the following Question should be studied

1.	What are the optimum arrangements for monitoring multichannel sound during production and recording, such as:

–	loudspeakers/room responses;

–	suitable arrangements and characteristics for loudspeaker handling low frequency signals;

–	suitable methods for aligning the reproduction levels of the monitor loudspeakers;

–	suitable methods for visual monitoring of multichannel sound signal parameters such as level, phase, delay, etc.?

2.	What requirements do the multichannel operations place on such things as track allocation on storage media, channel allocation on channel interfaces, etc.?

3.	What are the optimum methods to ensure appropriate system compatibility, such as:

–	backward compatibility of multichannel sound systems with two-channel sound systems;

–	forward compatibility of two-channel sound systems with multichannel sound systems;

–	conversion between film sound formats and broadcasting sound formats?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1997.

µQUESTION  ITU-R  210/10

OBJECTIVE  PERCEPTUAL  QUALITY  ASSESSMENT  METHODS

(1993)

Q. ITU-R 210/10

The ITU Radiocommunication Assembly,

considering

a)	that conventional objective methods (e.g. for measuring signal-to-noise ratio and distortion) may no longer be adequate in assessing systems which use low bit-rate coding schemes or which employ analogue or digital signal processing;

b)	that low bit-rate coding schemes are rapidly being deployed;

c)	that not all implementations conforming to a specification or standard guarantee the best quality achievable with that specification or standard;

d)	that formal subjective assessment methods are not suitable for the monitoring of audio quality under operational conditions;

e)	that objective perceptual test methods may eventually complement or supersede conventional objective test methods in all areas of measurements;

f)	that objective perceptual test methods may usefully complement subjective assessment methods;

g)	that, for some applications, a method which can be implemented in real time can be advantageous,

decides  that the following Question should be studied

1.	What are the characteristics of perceptual models which correlate well with perceived audio quality?

2.	How may these models be used in the assessment of audio quality?

3.	What are the parameters of quality which can be measured by objective perceptual methods?

4.	What are the possibilities for defining a set of test signals to be used in the objective perceptual measurement of audio quality?

5.	What are the possibilities for real-time implementation of objective perceptual methods?

6.	What are the possibilities for making measurements when the input signal is available, when some suitable stored reference* signal is available, or when no reference* signal is available at the input of the model?

7.	By what criteria should the measurement methods be assessed?

further decides

1.	that the initial results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1997.

µQUESTION  ITU-R  207/10

STANDARDS  FOR  DIGITAL  AUDIO  TECHNIQUES

(1993)

Q. ITU-R 207/10

The ITU Radiocommunication Assembly,

considering

a)	that although the use of digital sound signals in studios should lead to an improvement in quality, digital techniques entail their own specific impairments;

b)	that impairments to broadcast sound signals will be minimized, and the cost of transcoding equipment may be reduced, if there is a simple relationship between the coding standards used for production, transmission links and broadcasting;

c)	that sound studio equipment may need coding parameters different from those required for the emission of high quality broadcast signals, for example, they may need higher numbers of bits/sample to allow processing “headroom”;

d)	that there may be other factors affecting the choice of coding parameters in certain equipment used in broadcasting studios. For example, there might be constraints due to video-recording on the same recorder;

e)	that Radiocommunication Study Group 11 is currently studying the coding parameters needed to meet different production requirements for video signals in television studios;

f)	that interconnection of equipment in the digital domain is necessary to avoid impairments occurring when converting a sound signal from analogue to digital,

decides  that the following Question should be studied

1.	What is the nature of signal impairments due to digital processing in studios and how should the appropriate methods of objective measurement be defined?

2.	What are the objective limits for signal impairments in the different links in the studio chain?

3.	What methods of transcoding can be recommended?

4.	What are the problems arising from the need in studios to make use of asynchronous signals and appropriate methods of synchronization?

5.	What are the problems arising as a result of delay introduced by digital coding and transcoding, and techniques for minimizing delay in applications for which delay is important?

6.	What digital audio interface can be recommended to interconnect digital audio equipment, taking into account the need for transmitting auxiliary data along with the programme?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1995.

µQUESTION  ITU-R  205-1/10*

EVALUATING  FIELDS  FROM  BROADCASTING  TRANSMITTING  SYSTEMS  OPERATING  AT FREQUENCIES  BELOW 30 MHz  FOR ASSESSING EXPOSUSRE  TO NON-IONIZING  RADIATION

(1993-1995)

Q. ITU-R 205/10

The ITU Radiocommunication Assembly,

considering

a)	that radio-frequency energy is known to have harmful effects on the human body when absorbed in a certain quantity;

b)	that radio-frequency energy may induce harmful electric potentials in conducting material;

c)	that radio-frequency energy is known to have harmful effects on apparatus (such as radiocommunication apparatus, navigation instruments, cardiac pacemakers, scientific or medical equipment, etc.);

d)	that radio-frequency energy may lead to inadvertent ignition of inflammable or explosive material;

e)	that determinations of hazardous radiation levels and electric potentials are being made by competent authorities;

f)	that persons not associated with such systems may be exposed inadvertently to such radiation (including travellers by air) or to such electric potentials;

g)	that persons operating and maintaining radio transmitting systems may be required to work in close proximity to sources of such radiation,

decides  that the following Question should be studied

1.	What are the radio frequency power flux-densities and/or electric and magnetic field strengths to be expected from broadcast transmitting systems operating at frequencies below 30 MHz?

2.	What method should be used for calculating the power flux-densities and/or the electric and magnetic field strengths, especially in the near field zone?

3.	What methods are suitable for measuring the power flux-densities and/or the electric and magnetic field strengths, especially in the near field zone?

4.	What design precautions and technical operational procedures at transmitting stations and what precautions within areas in the vicinity of such stations, in which hazardous radio frequency may occur, are necessary to prevent the hazardous exposure of human beings, apparatus and inflammable or explosive materials to radio-frequency radiation?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1999.

Note 1– This subject is partially treated in Reports ITU-R SA.543, ITU-R SM.671 and ITU-R SA.682.

µQUESTION  ITU-R  204/10

FREQUENCY  SHARING  ISSUES  RELATED  TO  THE  INTRODUCTION  OF  THE
BROADCASTING-SATELLITE  SERVICE  (SOUND)  
IN  THE  FREQUENCY  RANGE  1-3  GHz

(1993)

Q. ITU-R 204/10

The ITU Radiocommunication Assembly,

considering

a)	that the World Administrative  Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) allocated frequencies to the broadcasting-satellite service (BSS) (sound) and complementary terrestrial broadcasting in bands near 1.5, 2.3 and 2.6 GHz for digital sound broadcasting to portable and vehicular receivers;

b)	that all three of the allocated bands contain allocations to certain terrestrial services and that the 2.6 GHz band also contains an allocation to the fixed-satellite service (space-to-Earth) in Regions 2 and 3 and mobile-satellite service (Earth-to-space);

c)	that it is necessary to ensure that the introduction of the BSS (sound) and complementary terrestrial broadcasting proceeds in a flexible and equitable manner;

d)	that this objective is addressed by Resolution No. 528 (WARC-92) which calls for the convening of a competent conference for the planning of the broadcasting-satellite sound in the allocated bands and the development of procedures for the coordinated use of complementary terrestrial broadcasting;

e)	that Resolution No. 528 (WARC-92) also specifies a coordination procedure to be used for the introduction of digital sound broadcasting-satellite systems in the interim period prior to the conference and that the calculation methods and interference criteria to be employed in the application of this procedure be based upon relevant ITU-R Recommendations;

f)	that Resolution No. 528 (WARC) calls upon the conference cited in § d) above to review the criteria for sharing with other services;

g)	that systems in the BSS (sound) could employ satellites in the geostationary-satellite orbit (GSO) or in non-geostationary-satellite orbits (non-GSO);

h)	that Resolution No. 522 (WARC-92) invited the CCIR (now ITU Radiocommunication Study Groups) to study the means to be employed for coordinating and avoiding mutual harmful interference between non-GSO systems, between GSO and non-GSO systems of the BSS (sound), and between BSS (sound) systems and the systems of other services, including the development of appropriate sharing criteria,

decides  that the following Question should be studied

1.	What are the preferred technical and operational characteristics of the BSS (sound) systems, both GSO and non-GSO, to be protected, including the number of satellites and their orbital parameters, the noise and interference performance requirements and the budgeting of interference between intrasystem, intraservice and interservice sources?

2.	What are the interference protection requirements of the various types of BSS (sound) systems expressed, for example, in terms of the maximum acceptable level of power flux-density incident from other systems?

3.	What constraints on the technical characteristics (e.g. e.i.r.p., pointing angle, pfd) of BSS (sound) systems and on the service they provide (e.g. coverage, availability) might be acceptable in the interest of reducing interference into the systems of other services to acceptable levels?

4.	What are the means for coordinating and avoiding mutual harmful interference within the BSS (sound) among GSO systems, among non-GSO systems, and between GSO and non-GSO systems, and how do these means compare in effectiveness?

5.	What are the means for coordinating BSS (sound) systems with the systems of other services and what sharing criteria should be used to trigger such coordination?

6.	What changes, if any, are needed in the ITU-R Recommendations that specify the calculation methods and interference criteria to be used in evaluating interference under the interim coordination procedures cited in § e)?

7.	What are the technical bases for resolving the issues to be addressed by the conference cited in § d)?

Note 1 – This Question should lead to a Recommendation by 1997.

µQUESTION  ITU-R  203/10*

(ex Q. 40/6)

ANTENNA  CHARACTERISTICS

(1993)

Q. ITU-R 203/10

The ITU Radiocommunication Assembly,

considering

a)	that the characteristics of antennas affect the practical performance of radio systems and should be included in predictions,

decides  that the following Question should be studied

1.	What are the theoretical performance characteristics of antennas used for systems involving ionospheric propagation, and to what extent are these consistent with practical data?

Note 1 – See Report ITU-R PI.891.

µQUESTION  ITU-R  201-1/10

TRANSMITTING  AND  RECEIVING  ANTENNAS  AT  LF  AND  MF

(1993-1994)

Q. ITU-R 201-1/10

The ITU Radiocommunication Assembly,

considering

a)	that any transmission system should permit efficient use of the available frequency bands and transmitter power, whilst maintaining reasonable receiver costs and an acceptable level of quality;

b)	that it is preferable to use high-efficiency antennas appropriate to the size and shape of the required coverage area especially when the size of the coverage area is limited by interference between the sky and ground waves;

c)	that the use of appropriate and well-designed directional antenna systems can lead to improved wanted coverage and reduced interference;

d)	that the radiation pattern from antennas can be seriously affected by re-radiation from nearby structures which are not part of the antenna system;

e)	that it is essential to verify the antenna radiation pattern by measurements;

f)	that the RF power density in the vicinity of transmitting antennas is significant;

g)	that efficient planning will need a definition of the characteristics of antennas,

decides  that the following Question should be studied

1.	What are the radiation patterns of types of transmitting and receiving antennas commonly used at LF and MF?

2.	What is the effect of imperfect and irregular ground on the radiation pattern of antennas?

3.	What are the most appropriate designations for different types of LF and MF antennas?

4.	What are the methods of calculation of the electric and magnetic field strengths in the vicinity of transmitting antennas?

5.	What computer-based procedures can be recommended to give in a standardized form, the gain and directivity patterns of transmitting and receiving antennas in common use at LF and MF?

further decides

1.	that the results of the above studies should be addressed to prepare a new Recommendation on:

–	receiving antennas at LF and MF;

–	reference receiving antenna patterns for use in planning;

–	transmitting antennas at LF and MF;

2.	that the above studies should be completed by 1998;

3.	that administrations be invited to provide appropriate measurements of antenna radiation patterns.

µQUESTION  ITU-R  107/10

CHARACTERISTICS  OF  TERRESTRIAL  DIGITAL  SOUND  BROADCASTING  SYSTEMS FOR  RECEPTION  BY  VEHICULAR,  PORTABLE  AND  FIXED  RECEIVERS

(1993)

Q. ITU-R 107/10

The ITU Radiocommunication Assembly,

considering

a)	that there is an increasing requirement by some countries for suitable means of broadcasting high quality stereo/multi-channel sound to vehicular, portable and fixed receivers;

b)	that significant progress has been made in technical studies on digital sound broadcasting systems and that some systems have been demonstrated with success;

c)	that it has been demonstrated that advanced digital sound broadcasting systems can lead to greater spectrum and power efficiency and immunity to multipath compared with conventional analogue sound broadcasting systems;

d)	that digital sound broadcasting systems can be designed to allow common signal processing in receivers for various broadcasting bands;

e)	that digital sound broadcasting systems can provide national, regional and local terrestrial services;

f)	that it would be advantageous for a digital sound broadcasting system if a common receiver, capable of receiving terrestrial and satellite services, could be designed;

g)	that digital sound broadcasting systems may be configured to transmit programmes with lower bit rates in order to trade quality against the number of sound channels;

h)	that new digital sound broadcasting systems will be able to provide additional facilities for programme-related and non programme-related data;

j)	that the World Administrative Radio Conference (Malaga-Torremolinos, 1992) (WARC-92) asked the CCIR to undertake as a matter of urgency the technical studies associated with terrestrial digital audio broadcasting,

decides  that the following Question should be studied

1.	What are the technical characteristics of digital sound broadcasting systems for reception by vehicular, portable and fixed receivers?

2.	What are the most suitable VHF/UHF bands, technically, economically and from a sharing and programme capacity point of view, for the implementation of a terrestrial digital sound broadcasting service?

3.	What are the system and service requirements for a digital sound broadcasting service?

4.	What are the most appropriate source coding, channel coding, multiplexing and modulation systems for a digital sound broadcasting service?

5.	What are the effects of normal, abnormal and very abnormal propagation, including multipath on digital sound broadcasting systems?

6.	What protection ratios are required to prevent mutual interference between different digital sound broadcasting services and other services using the same and adjacent frequency bands?

7.	What are the necessary planning criteria for national, regional and local area coverage for vehicular, portable and fixed reception?

8.	What advantages can be obtained by the combined use of satellite and terrestrial services operating in the same frequency band?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1996.

µQUESTION  ITU-R  106-1/10

SUBJECTIVE  ASSESSMENT  OF SOUND  QUALITY

(1993)

Q. ITU-R 106-1/10

The ITU Radiocommunication Assembly,

considering

a)	that subjective methods of testing are frequently necessary to assess the relative quality of programme sound and the effect of interference and other impairments upon it;

b)	that many different methods of subjective testing are possible;

c)	that in particular other Questions urgently call for guidelines for subjective assessment;

d)	that it is highly desirable to standardize the methods of subjective testing and the interpretation of the results, so that best possible comparisons may be made between results obtained at different times and in different places;

e)	that it is highly desirable that the grading scales which are used to describe the subjective quality of sound, should permit more consistent statistical processing methods, independent from the language expressing the opinions;

f)	that it is highly desirable that the grading scales should be identical to those used for vision;

g)	that the introduction of new sound-broadcasting systems may require advanced development of the existing methods of subjective assessments;

h)	that the geometric and acoustical properties of control rooms and listening rooms can have a considerable influence on audition, especially in two channel stereophony and multichannel systems;

j)	that this influence may cause studio operators to impair the response curve by means of the filters with which the rooms are equipped;

k)	that the unification of closely specified listening conditions at the studio end is an essential prerequisite for a well-defined subjective assessment and quality control, in the interest of the international programme exchange;

l)	that Recommendation ITU-R BS.562 has established some methods for assessing the subjective quality of sound;

m)	that there is a common core of assessment methodology for sound and vision;

n)	that the introduction of new kinds of sound signal processing such as digital coding and bit-rate reduction, new kinds of television signals using time-multiplexed components and new services such as enhanced television and HDTV, may require changes in or new methods of making subjective assessments of the sound quality,

decides  that the following Question should be studied

1.	Methodology

–	How should assessment problems be classified?

–	What test methods should be defined?

–	What guidelines should be established for linking the question to the most appropriate test method?

2.	Stimulus

–	What methods should be recommended to define, select and present stimuli (with or without accompanying picture, etc.)?

3.	Reproduction system

–	What reproduction device and methods to determine acoustical properties of listening rooms should be recommended?

4.	Subjects

–	What methods should be recommended to select listening panels to define training procedures and to instruct the listeners prior to the tests?

5.	Statistical analysis and presentation of the results

–	What methods should be recommended to determine statistical analysis techniques and the means of presenting the results?

further decides

1.	that the studies concerning the subjective assessment of the quality of the sound accompanying pictures need to be carried out;

2.	that the studies concerning the subjective assessment of digital source coding are urgent;

3.	that the studies concerning the determination of the acoustical properties of listening test rooms are urgent;

4.	that the results of the above studies should be included in (a) Recommendation(s);

5.	that the above studies should be completed by 1997.

µQUESTION  ITU-R  105-1/10

MULTILINGUAL  SERVICES  IN  MULTI-CHANNEL  SOUND  SYSTEMS

(1993-1994)

Q. ITU-R 105-1/10

The ITU Radiocommunication Assembly,

considering

a)	that, wherever possible, all languages in a multilingual service should be treated with equality;

b)	that recorded drama programmes, particularly films, are often available in different languages, with multichannel sound;

c)	that for sports programmes (and some types of recorded drama) a multichannel sound signal comprising “effects” may be provided together with one or more mono “commentary” or “dialogue” signals in different languages;

d)	that certain “live” programmes (for example game shows) will always favour the original language in which they are performed, and that it would be unrealistic to expect interpreted sound signals to provide a service equivalent to the service in the original language,

decides  that the following Question should be studied

1.	What reasonable number of sound services could usefully be employed in a television or in a sound-only situation. This should take into account the number of different language services to be provided, the range of programme types, and the desirability of providing special sound signals for listeners with impaired hearing and for television viewers with impaired sight?

2.	What minimum number of sound signals per service might be used in a flexible manner to enable broadcasters to serve their programme and audience requirements satisfactorily?

further decides

1.	that the results of the above studies should be included in Recommendations;

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  102/10

TRANSMISSION  OF  DATA  INFORMATION  AS  AN  ALTERNATIVE
TO  THE  MAIN  PROGRAMME  IN  FREQUENCY-MODULATION
SOUND  BROADCASTING

(1990)

Q. ITU-R 102/10

The ITU Radiocommunication Assembly,

considering

(a)	that new data broadcasting services conveyed by sound channels, addressed to wide audiences and/or special users, especially for educational and commercial applications, are attracting a growing interest in several countries;

(b)	the advantages of using the main sound channel for this purpose, permitting the use of domestic sound receivers to recover the data signal;

(c)	the possibility of using these data broadcasting systems for short time periods, during specific programmes, or for longer periods, e.g. in the night, when no sound programmes are broadcast;

(d)	that it is desirable to encourage development of compatible or possibly common systems,

unanimously decides  that the following question should be studied

1.	what applications can be envisaged;

2.	what are the technical characteristics of the data broadcasting systems, aiming at the best use of the sound channel capacity;

3.	what are the consequent compatibility problems with sound broadcasting services, particularly as regards the protection ratios?

µQUESTION  ITU-R  96-1/10

DETERMINATION  OF  THE  EFFECTS  OF  ATMOSPHERIC  NOISE  ON
THE  GRADE  OF  RECEPTION  IN  THE  TROPICAL  ZONE

(1990-1994)

Q. ITU-R 96-1/10

The ITU Radiocommunication Assembly,

considering

a)	that the determination of the transmitter power required depends in part upon the value of the signal-to-atmospheric noise ratio regarded as being the minimum for an acceptable broadcasting service in the Tropical Zone,

decides  that the following Question should be studied

1.	What characteristics of atmospheric noise are important in determining the response of a typical broadcast receiver and the extent to which they affect the grade of reception?

2.	What is the average value of signal-to-atmospheric noise ratio required to ensure satisfactory reception for at least 90% of the total time?

further decides

1.	that the results of the above studies should be presented in the form of a handbook;

2.	that the above studies and handbook should be completed by 1998.

µQUESTION  ITU-R  93-1/10

CHARACTERISTICS  OF  SYSTEMS  IN  THE  BROADCASTING-SATELLITE
SERVICE  (SOUND)  FOR  INDIVIDUAL  RECEPTION  BY  MEANS  OF
PORTABLE  AND  VEHICULAR  RECEIVERS

(1990-1993)

Q. ITU-R 93-1/10

The ITU Radiocommunication Assembly,

considering

a)	that there is an interest shown by some countries, in a sound-broadcasting service covering a fairly large area with the same programme, and giving adequate quality at all times when using portable and vehicular receivers;

b)	that such a service cannot always be provided economically by terrestrial networks but might be feasible by satellite systems;

c)	that because of the levels of power flux-density required for reception by portable and vehicular receivers sharing with terrestrial services could be difficult;

d)	that advanced digital modulation and coding systems with low power flux-density requirements and improved sharing capabilities have been demonstrated;

e)	that different source coding rates can be used for broadcasting-satellite service (BSS) (sound) systems depending on the sound quality needed;

f)	that the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) allocated frequencies to the BSS (sound) and to complementary terrestrial broadcasting in the bands 1 452-1 492 MHz, 2 310-2 360 MHz, and 2 535-2 655 MHz for digital sound broadcasting to portable and vehicular receivers;

g)	that Resolution No. 522 (WARC-92) calls upon the Radiocommunication Study Groups to carry out studies that focus on the characteristics of BSS (sound) systems which employ non-geostationary-satellite orbits (non-GSO) as well as those that employ satellites in the GSO;

h)	that Resolution No. 528 (WARC-92) resolved that a competent conference be convened by the ITU preferably not later than 1998 for the planning of BSS (sound) in the bands allocated for this service and calls upon the Radiocommunication Study Groups to conduct the necessary preparatory studies,

decides  that the following Question should be studied

1.	What are the service objectives and what are the different service concepts using satellites that can meet these objectives?

2.	What are the propagation impairments likely to be encountered in various receiving environments for the bands allocated to the service?

3.	What mitigation techniques are available for overcoming the anticipated propagation impairments?

4.	What are the types and characteristics of the most suitable source coding algorithms and channel coding modulation techniques?

5.	What are the characteristics to be specified for the receivers, and in particular for the receiving antennas of transportable, portable and/or mobile receivers?

6.	What are the characteristics to be specified for the satellites, in particular as regards the choice of the orbit, the transmission power and the coverage, taking into account the possibility of mixed satellite/terrestrial systems?

7.	What are the bandwidth requirements for the feeder links?

8.	What frequency allocations to the fixed-satellite service are most suitable for the feeder links?

9.	What are the estimated costs of the transmission delivery system and the portable and vehicular receivers?

10.	What are the optimal sharing arrangements with the complementary terrestrial digital sound-broadcasting service?

11.	What are the test procedures and measurement methods that should be used for objective and subjective performance evaluation of digital sound-broadcasting systems?

Note 1 – See Report ITU-R BO.955-1.

Note 2 – This Question will lead to new Recommendations by 1997 and a specific handbook by 1996.

Note 3 – The frequency sharing and service implementation issues are raised in Question ITU-R 204/10.

µQUESTION  ITU-R  92-1/10

OPTICAL  RECORDING  OF  SOUND-BROADCASTING  PROGRAMMES
FOR  INTERNATIONAL  EXCHANGE

(1990-1993)

Q. ITU-R 92-1/10

The ITU Radiocommunication Assembly,

considering

a)	that optical digital sound recording on erasable and non-erasable media is suitable for the production, archival storage, reproduction and exchange of disk-recorded programmes;

b)	that the introduction of such media in sound broadcasting would raise the technical quality of sound programmes;

c)	that the adoption of a single optical recording format consistent with the requirements of the Recommendations on digital sound signals in sound-broadcasting studios would help to improve conditions for sound-broadcasting programme exchange,

decides  that the following Question should be studied

1.	What digital optical recording format(s) can be recommended for the international exchange of sound programmes?

2.	What operating practices can be recommended for such exchanges?

further decides

1.	that the results of the above studies should be included in (a) Recommendation(s);

2.	that the above studies should be completed by 1997.

µQUESTION  ITU-R  85-2/10

SUBJECTIVE  ASSESSMENT  OF  SOUND  QUALITY  IN  
BROADCASTING  USING  DIGITAL  TECHNIQUES

(1990-1993-1994)

Q. ITU-R 85-2/10

The ITU Radiocommunication Assembly,

considering

a)	that the introduction of digital techniques in the studio, in transmission circuits and at the transmitter should allow an improvement in quality and more efficient operational use;

b)	that these techniques entail their own specific signal impairments;

c)	that it is desirable to ensure high quality performance throughout the broadcasting chain without unnecessary expense,

decides  that the following Question should be studied

1.	What is the relation between subjective assessment and objective measurements for digital systems?

2.	What are the tolerable values of subjective impairment in the whole broadcasting chain?

3.	What is the subjective effect of the various studio processes on the digital signal (level changes, frequency response corrections, etc.)?

4.	What is the maximum number of times that processing of any kind may be applied to the signal in the studio and the associated subjective effect?

5.	What are the acceptable values of impairment (expressed in objective terms):

–	in recording equipment;

–	in other studio equipment;

–	in broadcasting emissions?

Note 1 – Digital signals used in the studio may differ from those used for transmission.

Note 2 – See in particular Reports ITU-R CMTT.647 and ITU-R BS.799.

Note 3 – The studies should also include a comparison with the best analogue techniques.

Note 4 – Close cooperation between Radiocommunication Study Group 10 and Telecommunication Standardization Study Group 9 will be required in this study,

further decides

1.	that the results of the above studies should be included in a Recommendation(s);

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  84-1/10

SOUND  SYSTEMS  FOR  THE  HEARING  IMPAIRED

(1990-1994)

Q. ITU-R 84-1/10

The ITU Radiocommunication Assembly,

considering

a)	the increasing incidence and awareness in the community of hearing impairment;

b)	the fact that as hearing is impaired, the threshold of hearing rises, but this is not necessarily accompanied by a corresponding increase in the threshold of pain;

c)	that this requires automatic gain control to be used in hearing-aids to reduce the dynamic range of the sound;

d)	that the dynamic range and the balance of some sound programmes may therefore be inappropriate for the hearing-impaired,

decides  that the following Question should be studied

1.	What is the most suitable dynamic range of the sound programme needed for the hearing-impaired?

2.	What is the relationship between the level of dialogue, music and effects needed for the hearing-impaired?

3.	How the values of the technical parameters might be chosen to benefit the hearing-impaired?

4.	What efficient method of producing, distributing and transmitting sound programmes for the hearing-impaired can be recommended?

5.	What technical facilities might be provided at the receiver to improve the audibility of sound programmes for the hearing-impaired?

further decides

1.	that the results of the above studies should be included in a Recommendation(s);

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  83/10

DETERMINATION  OF  THE  ELECTRO-ACOUSTICAL  PROPERTIES
OF  STUDIO  MONITOR  HEADPHONES

(1990)

Q. ITU-R 83/10

The ITU Radiocommunication Assembly,

considering

(a)	that unified and closely specified reference listening conditions are an essential prerequisite for subjective asessment and quality control;

(b)	that there are great difficulties in harmonizing the acoustical characteristics of existing control rooms and listening rooms;

(c)	that some aspects of the audio signal are more clearly perceptible by using headphones than by using loudspeakers;

(d)	that for these reasons it is desirable to prepare a Recommendation on the electro-acoustical properties of studio monitor headphones,

unanimously decides  that the following question should be studied

1.	what are the quality parameters of headphones;

2.	what are the relevant measuring methods with regard to the relationship between objective and subjective evaluation;

3.	what are the attainable and desirable values and tolerances to ensure neutral headphone reproduction?

Note – See Recommendation 562.

further decides  that

1.	the results of the above studies should be included in a Recommendation;

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

µQUESTION  ITU-R  73-1/10

SOUND  BROADCASTING  IN  BAND  8  (VHF)  IN  THE  TROPICAL  ZONE

(1990-1994)

Q. ITU-R 73-1/10

The ITU Radiocommunication Assembly,

considering

a)	that sound broadcasting in band 8 (VHF) is providing a high-quality service in many parts of the world and that its use is steadily growing in the Tropical Zone;

b)	that the rate of failure of transmission systems in band 8 is extremely low;

c)	that domestic receivers with adequate sensitivity are readily available for operation in band 8;

d)	that band 6 (MF), used for good quality broadcast service is heavily congested in tropical countries and that further expansion of broadcast service in this band is limited, especially at night-time;

e)	that band 7 (HF) suffers from adverse effects of interference and fading and cannot provide a reliable broadcasting service of good quality in tropical countries;

f)	that particular propagation conditions exist in the Tropical Zone due to humid and hot climate, duct formation over sea, thick and dense forests and strong sporadic-E layers and irregularities of the spread-F layer;

g)	that Questions ITU-R 69/10 and ITU-R 72/10 consider some aspects related to this Question,

decides  that the following Question should be studied

1.	What is the minimum usable field strength and protection ratio for planning of frequency-modulation services at VHF in tropical countries?

2.	How could the propagation effects described in § f) above be taken into account in planning of frequency-modulation services of VHF in tropical countries and what are the values of propagation margins to be provided for these effects to ensure a specified reliability of the service?

further decides

1.	that the results of the above studies should be presented in the form of a handbook;

2.	that the above studies and handbook should be completed by 1998.

µQUESTION  ITU-R  71/10

TRANSMISSION  OF  SUPPLEMENTARY  INFORMATION  WITH  A  SINGLE
TRANSMITTER  IN  FREQUENCY-MODULATION  SOUND  BROADCASTING

(1990)

Q. ITU-R 71/10

The ITU Radiocommunication Assembly,

considering

(a)	that the broadcasting with a single transmitter of information supplementary to the sound programmes is technically feasible, particularly through the use of digital signals modulating a sub-carrier;

(b)	that the use of many types of supplementary information for various applications can be foreseen;

(c)	that the transmission of such information with no direct relationship to the main programme can be accomplished by such techniques;

(d)	that it is desirable to encourage development of compatible or possibly common systems;

(e)	that Recommendation 643 specifies a “system for automating tuning and other applications in FM radio receivers with the pilot tone system”,

unanimously decides  that the following question should be studied

1.	what are the applications that can be envisaged;

2.	what are the minimum demands on the transmission channel for these applications, e.g. information rate, access time, coverage compared with that of the monophonic service, acceptable loss- and error-ratios;

3.	what are the possibilities of using the same transmission channel for future applications;

4.	what transmission systems satisfy the preceding minimum demands, in particular, what is the frequency of sub-carrier, type of modulation, information rate, digital frame structure, error-detecting and/or correcting systems, sensitivity to the content of the main programme and to the reception conditions;

5.	what compatibility with existing broadcasting services can be achieved;

6.	what information and method of presentation of some of the undefined groups in an RDS system can be recommended with regard to applications such as text and graphic transmission?

further decides  that

1.	the results of the above studies should be addressed to:

–	update and complement Recommendation 643,

–	prepare new Recommendation(s);

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

µQUESTION  ITU-R  70/10

TRANSMISSION  OF  SEVERAL  PROGRAMMES  WITH  A  SINGLE  TRANSMITTER
IN  FM  SOUND  BROADCASTING

(1990)

Q. ITU-R 70/10

The ITU Radiocommunication Assembly,

considering

(a)	that, for certain applications, in frequency-modulation broadcasting, it might be desirable to transmit two or more sound programmes with the same transmitter;

(b)	that such transmissions might be particularly useful in areas where several languages are spoken;

(c)	that two or more programmes might be transmitted with a specially designed system;

(d)	that use of existing receivers (adapted or modified) for reception of the supplementary programmes is not to be considered a primary requirement;

(e)	that individual automatic receiver control including programme selection, volume balancing, and regulation of dynamic range, etc., might be required by listeners,

unanimously decides  that the following question should be studied

1.	what systems should be used with a single transmitter for the transmission of several programmes in frequency-modulation broadcasting and what technical parameters should be standardized, such as:

1.1	the frequency of each sub-carrier;

1.2	the type of modulation of sub-carriers;

1.3	the degree of modulation of the main carrier by each sub-channel signal;

1.4	the degree of modulation of each sub-carrier if frequency modulation is chosen;

1.5	the maximum audio modulating frequency;

1.6	the pre-emphasis characteristics of supplementary channels;

1.7	the time constants and input/output characteristic of any compressor/expander used in the system;

1.8	the dynamic range;

1.9	any other parameters which need to be considered;

2.	what are the influences of such standardized parameters upon:

2.1	the intermodulation between the different channels;

2.2	the signal-to-noise ratio of each channel;

2.3	the frequency response of each channel;

2.4	the harmonic distortion in each channel;

2.5	the radio-frequency protection ratios;

2.6	any other characteristics which need to be considered;

2.7	stereophonic broadcasting;

3.	what types of receiver should be used to allow the listener to select each programme;

4.	what system should be used for the transmission of control signals while still maintaining the possibility of simultaneous transmission of two or more sound programmes;

5.	what frequency economy can be obtained by the system when comparing, on an equal audio-quality basis, the coverage areas (limited by noise and/or interference) of each programme with those that could be obtained if the various programmes were broadcast by separate transmitters of adequate power;

6.	to what extent it is possible to increase the coverage area of the additional programme transmissions by using either a compressed signal at the input to the transmitter only or a complete compandor system, without increasing the level of interference to reception of other emissions and without decreasing the quality of reception?

further decides  that

1.	the results of the above studies should be presented in the form of (a) Recommendation(s);

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

µQUESTION  ITU-R  69/10

POLARIZATION  OF EMISSIONS  IN  FM  SOUND  BROADCASTING

(1990)

Q. ITU-R 69/10

The ITU Radiocommunication Assembly,

considering

that emissions from frequency modulation sound-broadcasting stations are usually transmitted with horizontal polarization,

unanimously decides  that the following question should be studied

1.	which is the preferred polarization in terms of coverage and interference which should be used for transmission from the same location and on the same frequency for monophonic and stereophonic sound broadcasting in band 8 (UHF), of the following:

1.1	horizontal polarization;

1.2	vertical polarization;

1.3	a polarization having both horizontal and vertical components?

Note – See Report 464.

further decides  that

1.	the results of the above studies should be presented in the form of a Recommendation;

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

µQUESTION  ITU-R  68/10

MODULATION  LEVEL  CONTROL  IN  FREQUENCY-MODULATION
SOUND  BROADCASTING

(1990)

Q. ITU-R 68/10

The ITU Radiocommunication Assembly,

considering

(a)	that maximum peak programme level and test signal level to be used for a programme distribution network are specified by the CCITT in its Recommendations J.14, J.15 and N.13;

(b)	that peak programme meters for programme level monitoring are specified in Report 292;

(c)	that exceeding the maximum frequency deviation of the FM system can increase adjacent channel interference and cause distortion in the receivers;

(d)	that the use of both pre-emphasis networks (50 ms or 75 ms) and compressors in FM transmitters causes a risk of excessive modulation if higher audio-frequency components are present, even if the programme level is correctly monitored;

(e)	that the high-frequency content of programme material appears to have increased over the years;

(f)	that it is desirable to take advantage of the total dynamic range of the transmitter,

unanimously decides  that the following question should be studied

1.	what is the relationship between the test level used and the frequency deviation of the transmitter, giving sufficient margin for programme peaks and pre-emphasis;

2.	what maximum permissible peak programme level readings should be allowed during a programme to minimize the risk of excessive modulation, taking the effect of pre-emphasis into account;

3.	what limiting techniques should be used to prevent excessive modulation in extreme cases without significantly affecting the programme quality, taking the effect of pre-emphasis into account;

4.	what techniques should be used to avoid overmodulation that may be caused, in the polar-modulation system, by a change in the levels of signals (A + B) and (A – B);

5.	what is the effect of excessive modulation on protection ratio requirements and on non-linear distortion in receivers?

further decides  that

1.	the results of the above studies should be presented in the form of a Recommendation;

2.	the above studies should be completed by1996.

µQUESTION  ITU-R  67-1/10*

INTERFERENCE  FROM  OTHER  SERVICES  TO  SOUND
BROADCASTING  IN  THE  TROPICAL  ZONE

(1990-1994)

Q. ITU-R 67-1/10

The ITU Radiocommunication Assembly,

considering

a)	Recommendation No. 64 of the World Administrative Radio Conference (Geneva, 1979) and the studies pursued at the Plenary Assemblies of the former CCIR;

b)	that Recommendation ITU-R BS.216 is based on incomplete information and that further studies are necessary to determine finally a value for the radio-frequency protection ratio for sound-broadcasting services operating in the Tropical Zone in the shared bands;

c)	that comprehensive data could be obtained by the use of objective methods as described in Recommendation ITU-R BS.559 as well as  the subjective methods used hitherto,

decides  that the following Question should be studied

1.	What are the values of the radio-frequency protection ratio to be provided for a sound-broadcasting service, operating in the shared bands in the Tropical Zone, against interference from telegraphy (A1A/A1B and A2A/A2B) and telephony (A3E) emissions and how should they be experimentally determined when:

–	the interference is caused by one of these three types of emissions;

–	the interference is caused by two or more types of emission at the same time?

Note 1 – This study should be carried out taking into account transmitter frequency variations (up to and including those equal to the sum of the permissible frequency tolerances) of the sound-broadcasting service in the Tropical Zone and other services sharing the bands as laid down in the Radio Regulations.

Note 2 – Measurements should be carried out at the output of a receiver fitted with a simple filter having an audio-frequency cut-off of 5 kHz (the characteristics of the filter employed should be given).

Note 3 – Measurements should be carried out for carrier frequencies separated by 0, 1, 2, ... 10 kHz.

Note 4 – The results should be expressed in terms of percentage of listeners satisfied, as well as of percentage of time during which the satisfaction is achieved.

2.	What is the minimum usable field strength to which a protection ratio (to be determined under different conditions as staged in § 1 and as stipulated in § 1 of Recommendation ITU-R BS.216), should relate (taking into account the nature, intensity and distribution of noise levels in different parts of the Tropical Zone) and how should it be experimentally determined?

further decides

1.	that the results of the above studies related to the values of the radio-frequency protection ratio in the presence of the mentioned interfering emissions and to the minimum usable field strength should be presented in the form of a Recommendation;

2.	that the procedure to experimentally determine the above parameter should be included in an Annex to the above Recommendation;

3.	that the above studies should be completed by 1998.

Note 1 – See Recommendations ITU-R BS.48, ITU-R BS.216 and Report ITU-R BS.302.

µQUESTION  ITU-R  65-1/10

SHORT-DISTANCE  BROADCASTING  IN  BAND  7  (HF)  IN  THE  TROPICAL  ZONE

(1990-1994)

Q. ITU-R 65-1/10

The ITU Radiocommunication Assembly,

considering

a)	that the evaluation of the power required for a given grade of short-distance broadcasting service in the Tropical Zone requires precise knowledge of transmission and reception characteristics including statistical variation of natural radio noise;

b)	that it would be helpful in the planning of new broadcasting services in the Tropical Zone to have more reliable data;

c)	that more reliable data using representative transmission and reception conditions in the Tropical Zone would be helpful in the organization of various services in the shared bands allocated in the Tropical Zone (see No. 2669 of the Radio Regulations),

decides  that the following Question should be studied

1.	What should be the value of the signal-to-noise ratio and the radio-frequency wanted-to-interfering signal ratio that should be adopted as representative of an acceptable broadcasting service in the Tropical Zone?

2.	Whether the power limits in Recommendation ITU-R BS.215 need to be changed to new values depending upon long period practical data to give an acceptable broadcasting service in the Tropical Zone?

3.	What is the interrelationship between:

–	location of the transmitter,

–	distance from the transmitter,

–	radiated carrier power,

–	transmitting and receiving antennas*,

–	radio frequency used

for achieving a required quality of service?

4.	How should the value of natural radio noise be taken into account in the planning of broadcasting services in the Tropical Zone for different hours of the day and if necessary for different geographical zones?

further decides

1.	that the results of the above studies should be presented in the form of a handbook;

2.	that the above studies and handbook should be completed by 1998.

µQUESTION  ITU-R  64/10

SYSTEM  DESIGN  FOR  HF  BROADCASTING

(1990)

Q. ITU-R 64/10

The ITU Radiocommunication Assembly,

considering

(a)	that there is a need to achieve at least a minimum signal/noise ratio throughout a desired HF broadcasting service area;

(b)	that a CCIR propagation prediction method made available by Study Group 6 is given in Report 894 which permits an estimate to be made of the signal level achieved over individual propagation paths;

(c)	that the choice of:

–	transmitter power,

–	type of transmitting antenna,

–	operating frequency band,

–	required reliability,

has a major impact on the coverage which can be achieved for an intended HF broadcasting service area (see Recommendations 80 and 705) and that these items are interrelated;

(d)	that it is necessary to take account of both wanted and interfering signal levels in order to optimize the efficiency of spectrum utilization,

unanimously decides  that the following question should be studied

1.	what is the type of transmitting antenna most suitable for use in band 7 (HF) and its influence on coverage and interference;

2.	what is the interrelationship between:

–	transmitter power,

–	type of transmitting antenna,

–	operating frequency band,

–	required reliability,

in achieving coverage of a specified service area;

3.	how many HF broadcasting services can satisfactorily share a single channel, due account being taken of adjacent channel interference?

further decides  that

1.	a Special Rapporteur should be appointed to coordinate the above studies;

2.	the results of the above studies should be presented in the form of a handbook;

3.	this handbook should be presented to the next Final Meeting of Study Group 10 for its approval.

µQUESTION  ITU-R  62-1/10

SYNCHRONIZED  TRANSMITTERS  IN  HF  SOUND  BROADCASTING

(1990-1994)

Q. ITU-R 62-1/10

The ITU Radiocommunication Assembly,

considering

a)	that the use of synchronized-frequency transmitter operation may lead to improved efficiency in the use of the spectrum;

b)	that the WARC HFBC-87, in its Recommendation No. 516, invites the former CCIR to accelerate its studies on the use of synchronized transmitters for broadcasting,

decides  that the following Question should be studied

1.	Under what technical conditions can synchronized transmitters (i.e. on the same frequency and modulated by the same programme) be used for sound broadcasting in band 7 (HF)?

2.	To what extent can the coverage areas of synchronized transmitters overlap in practice?

3.	What combination(s) of types of antennas may be used for synchronized operation?

further decides

1.	that the results of the above studies should be inserted in Recommendation ITU-R BS.702 on synchronized operation of HF sound broadcasting transmitters;

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  60-1/10

TRANSMISSION  OF  SUPPLEMENTARY  INFORMATION  IN  AMPLITUDE
MODULATION  SOUND  BROADCASTING

(1990-1993)

Q. ITU-R 60-1/10

The ITU Radiocommunication Assembly,

considering

a)	that, in addition to the normal sound programme, it is technically possible to transmit supplementary information in an amplitude-modulation sound-broadcasting channel;

b)	that such supplementary signals can have various applications, having no direct relationship to the normal sound programme;

c)	that the systems used must be compatible with existing services,

decides  that the following Question should be studied

1.	What applications for the supplementary signals in the various frequency bands, band 5 (LF), band 6 (MF) and band 7 (HF) can be recommended?

2.	What are the characteristics of the supplementary transmission channel, such as: information rate, access time, acceptable error ratio, coverage compared to that of the normal programme?

3.	What are the related transmission systems usable for:

–	double-sideband and stereophonic emissions in band 5 (LF) and 6 (MF);

–	double-sideband and single-sideband emissions in band 7 (HF)?

4.	What types of modulation, methods of demodulation and methods of error detection and/or correction are required?

5.	What degree of compatibility with existing broadcasting services can be achieved?

further decides

1.	that the results of the above studies should be addressed to:

–	prepare a Recommendation on a possible worldwide standard system;

2.	that the above studies should be completed by 1999.

Note 1 – See Recommendation ITU-R BS.706.

µQUESTION  ITU-R  58-1/10

COVERAGE  IN  LF,  MF  AND  HF  BROADCASTING

(1990-1994)

Q. ITU-R 58-1/10

The ITU Radiocommunication Assembly,

considering

a)	the need to ensure efficient spectrum utilization when planning LF, MF and HF broadcasting services,

decides  that the following Question should be studied

1.	What is the dependence of ground-wave and sky-wave sound broadcasting coverage on:

–	the characteristics of the system;

–	the radio-frequency protection ratios;

–	the channel spacing;

–	the minimum usable field strength;

–	the radiated power;

–	the geographical distribution of the transmitters;

–	the distribution of population density;

–	the use of directional transmitting antennas;

–	the use of groups of synchronized transmitters;

for bands 5 (LF), 6 (MF) and 7 (HF)?

2.	What methods are required (including the use of computers), for calculating the minimum number of channels for the coverage for broadcasting in bands 5 (LF) and 6 (MF), for each of the systems envisaged, taking into account the points listed under § 1?

Note 1 – See Recommendation ITU-R BS.598,

further decides

1.	that the results of the above studies should be addressed to:

–	update existing Recommendation(s);

–	prepare new Recommendation(s);

2.	that the above studies should be completed by 1998.

µQUESTION  ITU-R  57/10

SKY-WAVE  RECEPTION  IN  LF,  MF  AND  HF  BROADCASTING

(1990)

Q. ITU-R 57/10

The ITU Radiocommunication Assembly,

considering

(a)	that broadcasting services at LF, MF and HF are also provided by sky-wave propagation;

(b)	that to plan a satisfactory service via sky-wave propagation, fading allowances are required to be taken into account;

(c)	that the WARC HFBC-87, in its Recommendation No. 510, invites the CCIR to continue to study fading allowances,

unanimously decides  that the following question should be studied

1.	what is the nature of the distortion due to the effects of the ionosphere at distances beyond the ground-wave coverage area;

2.	what is the degradation of the subjective quality of the sky-wave signal when received beyond the limit of the ground-wave coverage area, according to the system envisaged and as a function of distance and other possible factors, such as the geographical location of the transmitter, the frequency, the season of the year and the characteristics of the transmitting antenna;

3.	what is the degree of distortion for a given percentage of time that would, after demodulation at the receiver, provide satisfactory sky-wave reception;

4.	what is the reduction in distortion, due to the effects of the ionosphere, that can be secured by improvements in methods of reception, for example, replacing envelope detectors by other detection devices?

Note 1 – See Report 619.

further decides  that

1.	the results of the above studies should be addressed to:

–	update existing Recommendation(s);

–	prepare new Recommendation(s);

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

µQUESTION  ITU-R  56/10

MINIMUM  USABLE  FIELD  STRENGTH  IN  LF,  MF  AND  HF  BROADCASTING

(1990)

Q. ITU-R 56/10

The ITU Radiocommunication Assembly,

considering

(a)	that efficient planning should be based on a precise knowledge of the minimum usable field strength;

(b)	that the WARC HFBC-87, in its Recommendation No. 510, invites the CCIR to continue to study the value of the minimum usable field strength in HF broadcasting,

unanimously decides  that the following question should be studied

what is the minimum usable field strength for broadcasting in bands 5 (LF), 6 (MF) and 7 (HF), taking account of:

–	the characteristics of the system;

–	whether ground-wave or sky-wave reception is envisaged;

–	atmospheric noise;

–	man-made noise?

Note – See Recommendation 638.

further decides  that

1.	the results of the above studies should be addressed to:

–	update existing Recommendation(s);

–	prepare new Recommendation(s);

2.	the above studies should be completed within the study period 1990-1994 or 1994-1998 at the latest.

µQUESTION  ITU-R  55-1/10

PROTECTION  RATIOS  IN  LF,  MF  AND  HF  BROADCASTING

(1990-1994)

Q. ITU-R 55-1/10

The ITU Radiocommunication Assembly,

considering

a)	that any sound broadcasting system should allow efficient use of the available frequency bands and transmitter power, whilst maintaining reasonable receiver costs,

decides  that the following Question should be studied

How should the radio-frequency protection ratios be determined under conditions which take account simultaneously of the following items, which may vary independently of each other?

# 1.	Fading (see Question ITU-R 57/10)

1.1	wanted signal and unwanted signal stable;*

1.2	wanted signal stable, and unwanted signal fluctuating;*

1.3	wanted signal and unwanted signal fluctuating;*

1.4	wanted signal fluctuating and unwanted signal stable.*

# 2.	Frequency separation from 0 to 30 kHz between the wanted and unwanted carrier.

# 3.	Broadcasting programmes

3.1	wanted and unwanted signals modulated with the same programme;*

3.2	wanted and unwanted signals modulated with different programmes.*

# 4.	Class of emission (the characteristics should be stated in each case)

4.1	double-sideband amplitude modulation:

4.1.1	with current receivers;

4.1.2	with receivers with characteristics adapted to the defined double-sideband transmission;

4.2	single-sideband or vestigial-sideband, including compatible single-sideband modulation;

4.2.1	with current-type receivers;

4.2.2	with receivers specially adapted to the system under consideration;

4.3	any other class of emission;

4.4	coexistence of two systems as follows:

µTABLE  1

§

| Wanted emission | Unwanted emission |
| --- | --- |
| Double-sideband amplitude modulation | Class of emission as under § 4.2 or 4.3 |
| Class of emission as under § 4.2 or 4.3 | Double-sideband amplitude modulation |

–	additionally for HF, the coexistence of the following systems*:

–	as well as the co-channel protection ratio between synchronized transmissions within the same network, and

–	the relative radio-frequency protection ratios.

5.	Frequency band (5 (LF), 6 (MF) or 7 (HF)).

6.	Minimum usable field strength.

7.	Modulation processing (compression and pre-emphasis).

8.	Necessary bandwidth.

Note 1 – See also Recommendation ITU-R BS.560, Annex II.

Note 2 – See Reports ITU-R BS.516, ITU-R BS.798 and Recommendations ITU-R BS.559, ITU-R BS.560, ITU-R BS.597, ITU-R BS.638 and ITU-R BS.639,

further decides

1.	that the results of the above studies should be addressed to:

–	update existing Recommendation(s);

–	prepare new Recommendation(s);

2.	that the above studies should be completed by 1998.

*	 	This Question should be brought to the attention of the International Electrotechnical Commission (IEC).

*	Studies related to this Question need to be co-ordinated with WP 10-11S for mixed satellite/terrestrial digital sound broadcasting.

*	In this context the term “unified” implies that the standards to be defined should as far as practicable apply both to terrestrial and to satellite broadcasting.

*	This Question should be brought to the attention of Radiocommunication Study Groups 1 and 3.

*	 	This is not meant to imply the normal 1 kHz reference tone.

*	 	This Question should be brought to the attention of Radiocommunication Study Groups 1, 8 and 9.

*	 	Radiocommunication Study Group 3 (former SG 6) agreed to transfer this Question to Radiocommunication Study Group 10.

*	 	This Question should be brought to the attention of Radiocommunication Study Group 9.

*	 	Relevant information in the texts of Radiocommunication Study Group 10 should be taken into account.

*	 	Tests should also be made with several unwanted signals and additional data broadcasting.

*	 	Recommendation Nos. 517 and 510 of WARC HFBC-87.