| µ § INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| RADIOCOMMUNICATION STUDY GROUPS | Addendum 1 to Document 7/1-E 26 July 1996 |

Radiocommunication Study Group 7

QUESTION ASSIGNED TO RADIOCOMMUNICATION STUDY GROUP 7

The attached revised Question approved by correspondence since the last Radiocommunication Assembly is to be added to the list of Questions assigned to ITU-R Study Group 7.

Attachment: Question ITU-R 118-1/7

QUESTION ITU-R 118-1/7

FACTORS WHICH AFFECT FREQUENCY SHARING BETWEEN DATA RELAY SATELLITE SYSTEMS AND SYSTEMS OF OTHER SERVICES

(1990 - 1996)

The ITU Radiocommunication Assembly,

considering

a) 	that data relay satellites are used to relay mission-gathered data, television and voice communications in manned missions, orbit-tracking data such as position and velocity of the spacecraft, and telecommands for guidance and control of the spacecraft;

b) 	that frequencies between 2 and 30 GHz used by both near-Earth satellite and data relay satellites are shared with other service systems;

c) 	that frequency bands between 20 and 30 GHz are becoming more heavily occupied by existing and planned systems;

d)	that frequency sharing between space stations of a data relay satellite network and terrestrial systems is feasible, provided;

- that appropriate power flux-density limits are applied to the  emissions of space stations 	  	of the data relay satellite networks;

- that appropriate e.i.r.p. density limits are applied to terrestrial service stations;

e)	that the use by data relay satellites of interference mitigation techniques such as frequency agility, adaptive interference cancellation and avoidance of mainbeam-to-mainbeam coupling may facilitate sharing with the terrestrial services,

decides that the following question should be studied:

1 	what are the services with which, and under what conditions, is the sharing of frequency bands practicable  for data relay satellite systems;

2 	what are the limits of power flux-density from data relay satellite emissions needed to protect other services sharing frequencies between 2 and 30 GHz;

3 	what are the e.i.r.p. density limitations to be applied to terrestrial service stations to facilitate sharing with data relay satellite networks;

4 	what are the preferred interference mitigation techniques for use by data relay satellite networks operating above 20 GHz ?

further decides

1	that the above studies should be completed by 1996.

____________