| INTERNATIONAL TELECOMMUNICATION UNION Radiocommunication Bureau (Direct Fax No. +41 22 730 57 85) |  |
| --- | --- |

Circular Letter* 	2 November 1998

8/LCCE/61

To Administrations of Member States of the ITU 
and other members of the Radiocommunication Sector 
participating in the work of Working Party 8D of Radiocommunication Study Group 8

Subject:	Special Rapporteurs Group on essential technical requirements for global and regional geostationary mobile-satellite service in the 1-3 GHz band

At its eighth meeting (Geneva, 19 October - 30 October 1998), Working Party 8D set up a Special Rapporteurs Group to assist in the preparation of a draft new Recommendation on the essential technical requirements for global and regional GSO MSS in the 1-3 GHz band.

Information about the terms of reference and the draft text approved by Working Party 8D is attached.

Robert W. Jones
	Director, Radiocommunication Bureau

Attachment:  1

Distribution:

-	Member States and Radiocommunication Sector Members participating in the work of Working Party 8D of Radiocommunication Study Group 8

-	Chairman and Vice-chairmen of Radiocommunication Study Group 8

-	Secretary General of the ITU, Director of the Telecommunication Standardization Bureau, Director of the Telecommunication Development Bureau

annex

(Source: Document 8D/TEMP/134)

terms of reference for THE special rapporteurs group 
dealing with essential Technical requirements for global AND REGIONAL GSO MSS in the 1 - 3 GHz band

A Special Rapporteurs Group (SRG) was established for Working Party 8D (WP 8D) to finalize the working document towards a preliminary draft new Recommendation on - Essential technical requirements of mobile earth stations for global and regional geostationary (GSO) land mobile-satellite service (MSS) in the bands 1 - 3 GHz. The terms of reference for this SRG shall be as follows:

1	To resolve all outstanding issues associated with the attached working document trying first to solve the problem associated with Notes 5, 5a and 8, taking into account the progress achieved at the 8th meeting of WP 8D.

2	The deadline for the work (a meeting report and an updated working document) of the SRG is one month before the start of the next WP 8D meeting, which is in April 1999.

3	The Chairman of the SRG is:

Dr. Cionaith Cullen,	tel:	+33 1 4529 6487

France Telecom / Cnet,	fax:	+33 1 4529 4534

38-40, rue du General-Leclerc,	e-mail:	cionaith.cullen@cnet.francetelecom.fr

92794 Issy Moulineaux Cedex 9, France.

4	There will be one meeting of the SRG which is currently proposed to take place, at the kind invitation of the UK Administration, in London, from 8-10 March 1999. Notification of the details for this meeting will be distributed by the ITU-BR.

5	The SRG will make every effort, by correspondence, to resolve the outstanding issues prior to the planned meeting in March 1999. If these efforts are successful, the Chairman of the SRG may cancel the meeting.

6	Participation in the SRG is open to all ITU-R members and interested parties are requested to inform the Chairman of the SRG as soon as possible and preferably not later than the 31 December 1998.

Attachment:	1

Attachment TO DOC. 8D/TEMP/34

(Source: Document 8D/83 Att. 11, 8D/112, 8D/141, 8D/163 and 8D/INFO/14)

WORKING DOCUMENT TOWARDS A DRAFT NEW RECOMMENDATION

ESSENTIAL TECHNICAL REQUIREMENTS OF LAND MOBILE EARTH STATIONS OF GEOSTATIONARY MOBILE-SATELLITE SERVICE
SYSTEMS IN THE BANDS 1 - 3 GHz

(Question ITU-R 218/8)

## Summary

The type approval of equipment forms a part of the authorization for the use of Land Mobile Earth Station (LMES) terminals. The mutual recognition by administrations of type approval of LMES terminals for geostationary mobile-satellite service systems is a key element in facilitating the circulation of LMES terminals. To achieve mutual recognition of the type approval of LMES terminals on a global basis it is necessary to have common global technical requirements.

This working document towards a draft new Recommendation addresses the matter of essential technical requirements for LMES terminals of GSO MSS systems as given in Question ITU-R 218/8.

The main objectives of the proposed Recommendation are to provide a common technical basis for facilitating the establishment of type approval requirements for LMES terminals, for the circulation of LMES terminals among administrations, and for the development of mutual recognition arrangements for type approval of LMES terminals among administrations.

Existing regional and global GSO MSS systems were designed to comply with unwanted emission limits specified by global MSS specifications (e.g. Inmarsat System Definition Manuals), and existing regional standards (e.g. ETSI in Europe). There is a significant infrastructure of GSO MSS systems and LMES terminals, which are built based on these specifications.

[In the report of the 7th WP 8D meeting, Doc. 8D/83 Attachment 11, the working document was made very complex because of problems with the time-phased approach. Also, it is agreed that it is difficult to discuss any time-phased approach proposals until the final limits are defined. Therefore to simplify the document to show the final values, many deletions have been made to editorial notes, notes to the tables, and Appendix 1. Once this working document on final values is stable, it will be clear how proposals to include time phased approaches relate] M

- M

- M

The ITU Radiocommunication Assembly,

considering

a)	that various technically different global and regional geostationary-satellite systems in the MSS (GSO MSS systems) are operating and others are expected to commence operating around the end of the 1990s;

b)	that these GSO MSS systems are providing and expecting to provide personal communications and/or broadband communications to either handheld, land vehicular, or transportable LMES terminals;

c)b	that some GMSS systems may have requirements (which are provided in Recommendation xyz) developed for pre-emption of an LMES channel when using the bands utilized for AMS(R)S or GMDSS (see S5.357A, S5.362A and S5.353A);a

d)	that the circulation of terminals among administrations is usually subject to a number of regulations including satisfactory type approval to an agreed technical standard;

e)	that there is a need for identifying the essential technical requirements for the type approval of Land Mobile Earth Station (LMES) terminals of global and regional GSO MSS systems;

f)	that the identification by the ITU-R of essential technical requirements for land mobile earth stations operating with global and regional GSO MSS systems would provide a common technical basis for facilitating type approval of LMES terminals by various national authorities, and mutual recognition of type approvals of LMES terminals, and mutual recognition arrangements for circulation of LMES terminals among administrations;

g)	that essential technical requirements should be measurable and/or testable;

h) 	that essential technical requirements should achieve [an acceptable balance between equipment design and production cost and the need for] effective use of the radio-frequency spectrum;

j)	that there is a need to protect safety services from harmful interference,

considering also

a)	that existing regional and global GSO MSS systems were designed to comply with unwanted emission limits specified by global MSS specifications and existing regional standards, leading to a significant infrastructure of GSO MSS systems and LMES terminals, which have been built based on these specifications;

[b)	that there is a need to accommodate the existing GSO MSS industry and current LMES terminal users for type approval arrangements and circulation of LMES terminals among administrations;]

c)	that the World Telecommunication Policy Forum (WTPF-96) which addressed Global Mobile Personal Communications by Satellite (GMPCS) issues, adopted Opinion No. 3, calling upon the three ITU Sectors, each within its competence, to initiate new studies or pursue current ones, and to reach conclusions as soon as practicable to facilitate the introduction of GMPCS on a global and regional basis;

d)	that the World Telecommunication Policy Forum (WTPF-96) which addressed Global Mobile Personal Communications by Satellite (GMPCS) issues, adopted Opinion No. 4, establishment of a Memorandum of Understanding (MoU) to facilitate the circulation of GMPCS user terminals (GMPCS-MoU) which, inter alia, requested the Secretary-General to take the necessary steps to prepare a GMPCS-MoU related to the circulation of GMPCS user terminals to facilitate the full implementation of GMPCS;

e)	that the development of relevant ITU Recommendations could facilitate the preparation and adoption of the GMPCS-MoU;

f)	that the identification by the ITU-R of essential technical requirements for LMES terminals operating with GSO MSS systems would provide a common technical basis for facilitating type approval of LMES terminals by various national authorities and the development of mutual recognition arrangements for type approvals of LMES terminals and mutual recognition arrangements for circulation of LMES terminals among administrations;

g)	that the identification by the ITU-R of essential technical requirements for LMES terminals operating with GSO MSS systems in the 1 to 3 GHz range [would ensure that interference] will not be caused to other radio services by GSO MSS LMES terminals;

h)	that Recommendation ITU-R [Document 1/24] contains a list of radio equipment parameters that are relevant to achieve spectrum efficiency and compatibility;

j)	that, for a particular GMPCS system, some LMES equipment parameters such as e.i.r.p. and necessary bandwidth which are listed in Recommendation ITU-R [Document 1/24] but are not contained in this Recommendation, are contained in the information provided as part of the Appendix S4 of the Radio Regulations,

recommends

that the essential technical requirements of LMES terminals for global and regional GSO MSS systems in the bands 1 - 3 GHz in Annex 1 and 2 should be used by administrations as a common technical basis for:

a)	establishing type approval requirements for LMES terminals;

b)	facilitating the licensing of LMES terminal operations

c)	facilitating the development of mutual recognition arrangements of type approval

of LMES terminals;

d)	facilitating the development of mutual recognition arrangements to facilitate the circulation and use of LMES terminals;

ANNEX 1 (to the Attachment)

Essential technical requirements of land mobile earth stations for global GSO MSS systems providing voice and/or data communications in the bands 1 - 3 GHz

# 1	Scope

Editorial Note - Clarification of the operational band to which the limits apply is required.

This annex specifies the technical requirements that apply to LMES with the following characteristics:

–	the LMES are operating in one or more frequency ranges of the MSS band;

–	the LMES could be either vehicle mounted or portable equipment;

–	the LMES operate through geostationary satellites as part of a network providing voice and/or data communications.

The requirements have been selected to ensure an adequate level of compatibility with other radio services. The levels, however, do not cover extreme cases which may occur in any location but with a low probability of occurrence.

# 2	Definitions

For the purposes of this annex, the following definitions apply:

carrier-off state: A LMES is in this state when either it is authorized by the NCF to transmit but when it does not transmit any signal, or when it is not authorized by the NCF to transmit.

carrier-on state: A LMES is in this state when it is authorized by the NCF to transmit and when it transmits a signal.

nominated bandwidth: The bandwidth of the LMES radio frequency transmission. The nominated bandwidth is wide enough to encompass all spectral elements of the transmission necessary for communication and which have a level greater than the specified unwanted emissions limits. The nominated bandwidth is wide enough to take account of the transmit carrier frequency stability. The nominated bandwidth is within the MSS transmit frequency band within which the LMES operates.

unwanted emissions: Unwanted emissions are those falling outside the nominated bandwidth.

# 3	Abbreviations

For the purposes of this annex, the following abbreviations apply:

e.i.r.p.	Equivalent Isotropically Radiated Power

MSS	Mobile-Satellite Service

LMES	Land Mobile Earth Station

MES	Mobile Earth Station

NCF	Network Control Facility

# 4	Requirements

## 4.1	Unwanted emissions outside the band 1 625.8 MHz to 1 661.2 MHz

Unwanted emissions from MESs outside the band 1 625.8 MHz to 1 661.2 MHz shall be below the following limits.

1)	The unwanted emissions over the frequency range 30 MHz to 1 000 MHz shall not exceed the limits in Table 1.

Table 1

Limits of unwanted emissions up to 1 000 MHz at a measuring distance of 10 m

| Frequency (MHz) | Quasi-peak limits (dB(µV/m)) |
| --- | --- |
| 30 to 230 | 30 |
| 230 to 1 000 | 37 |

The lower limit shall apply at the transition frequency.

2)	The unwanted emissions e.i.r.p. above 1 000 MHz in the measurement bandwidth and in all directions shall not exceed the limits of Table 2:

Table 2

Limits of unwanted emissions above 1 000 MHz 
and outside the band 1 625.8 MHz to 1 661.2 MHz (Editorial Note 1)

| Frequency range | Carrier-on |  | Carrier-off |  |
| --- | --- | --- | --- | --- |
| (MHz) | e.i.r.p. limit (dBW) | Measurement bandwidth (kHz) | e.i.r.p. limit (dBW) | Measurement bandwidth (kHz) |
| 1 000.0 - 1 525.0 | -61 | 1 000 | -72 | 100 |
| 1 525.0 - 1 559.0 | -61 | 1 000 | [-103] | 3 |
| 1 559.0 - 1 600.0 | -70 | 1 000 | [-70] | [1000] |
| 1 600.0 - 1 605.0 [(Note 8)] | -70 | 1 000 | [-70] | [1000] |
| [ 1 605.0 - 1 610.0 [(Note 8)] | (Note 5) [(Note 5a)] | 1 000 | [(Note 6)] | 100] |
| [ [1 610.0 - 1 625.8] | [-36] | 1 000 | [-72] | 100] |
| 1 625.8 - 1 661.2 | (Note 1) | (Note 1) | (Note 1) | (Note 1) |
| 1 661.2 - 1 690.0 | -36 | 1 000 | -72 | 100 |
| 1 690.0 - 3 400.0 | -61(Note 2) | 1 000 | -72 | 100 |

| 3 400 - 10 700 | -55 (notes 3 & 4) | 1000 | -72 | 100 |
| --- | --- | --- | --- | --- |
| 10 700 - 21 200 | -49 | 1000 | -76 | 100 |
| 21 200 - 40 000 | -43 | 1000 | -60 | 100 |
| NOTE 1: The unwanted emissions in that frequency range are limited by Subclause 4.2. NOTE 2: In the band 3 263.0 MHz to 3 321.0 MHz the maximum e.i.r.p. in one, and only one, 100 kHz measurement bandwidth shall not exceed -38 dBW. Elsewhere in this band the power limit in Table 2 shall be applied. NOTE 3: In each of the bands 4 894.5 MHz to 4 981.5 MHz, 6 526.0 MHz to 6 642.0 MHz and 8 175.5 MHz to 8 302.5 MHz the maximum e.i.r.p. in one, and only one, 100 kHz measurement bandwidth shall not exceed -48 dBW. Elsewhere in this band the power limit in Table 2 shall be applied. NOTE 4: In the band 9 789.0 MHz to 9 963.0 MHz the maximum e.i.r.p. in one, and only one, 100 kHz measurement bandwidth shall not exceed -59 dBW. Elsewhere in this band the power limit in Table 2 shall be applied. NOTE 5: Linearly interpolated from -70 dBW in 1 MHz at 1 605.0 MHz to -36 dBW in 1 MHz at 1 610.0 MHz. [NOTE 5a: The Russian Federation states that a level of -70 dBW/MHz in the band 1 605 to 1 607.5 MHz and a linear interpolation between -70 dBW/MHz at 1 607.5 and –36 dBW/MHz at 1 610 MHz shall be used to provide protection of GLONASS receiver operations.the level should be no more stringent B NOTE 6: Linearly interpolated from -70 dBW in 1 MHz at 1 605.0 MHz to -62 dBW in 1 MHz at 1 610.0 MHz. HE NOTE 8 [In the interim period prior to 1 June 2002, the carrier-on limits of -36 dBW/MHz and the carrier off limits of -62 dBW/MHz shall apply in the band 1600-1610 MHz. LMSS terminal manufacturers are encouraged to meet final limits as soon as possible but not later than 1 June 2002.] |  |  |  |  |

Editorial Note 1     All Administrations agreed that a global solution is a key objective of this Recommendation.

Mt

M

## 4.2	Maximum unwanted emissions within the 1 625.8 MHz to 1 661.2 MHz band

Specification 1: Carrier-on state

The unwanted emission e.i.r.p. in any 3 kHz band within the 1 625.8 MHz to 1 661.2 MHz band, but outside the nominated bandwidth, shall not exceed the limits of Table 3.

Table 3

Limits for unwanted emissions within the 1 625.8 MHz to 1 661.2 MHz band

| Offset from the edge of the band of the nominated bandwidth (kHz) | Maximum e.i.r.p. (dBW) |
| --- | --- |
| 0 to 10 | 5 |
| 10 to 20 | -10 |
| 20 to 100 | -15 |
| 100 to 200 | -25 |
| 200 to 700 | -35 |
| greater than 700 | -45 |

The limits in Table 3 may be exceeded provided that the sum in watts of the spectrum components exceeding the limits of Table 3 does not exceeds -16 dBW.

Specification 2: Carrier-off state

The e.i.r.p. of any emission in any 3 kHz band within the 1 625.8 MHz to 1 661.2 MHz band shall not exceed -63 dBW.

ANNEX 2 (to the Attachment)

Essential technical requirements of low data rate land mobile earth stations for global GSO MSS systems in the bands 1 - 3 GHz

# 1	Scope

This annex specifies the technical requirements that apply to satellite land mobile earth station equipment that is capable of operation in one or more of the following frequency ranges:

–	1 525.0 to 1 559.0 MHz (space-to-Earth);

–	1 626.5 to 1 660.5 MHz (Earth-to-space);

of the Mobile-Satellite Service (MSS).

This annex specifies the requirements for satellite earth station equipment that:

–	is capable of being used either for transmission only, or for transmission and reception (transmit-receive), or for reception only (receive-only), of radio-communications signals in any of the bands specified above;

–	is not purpose built satellite earth station equipment intended for use as part of the public telecommunications network.

# 2	Definitions

For the purposes of this annex the following definitions apply:

carrier-off state: A LMES is in this state when either it is authorised by the NCF to transmit but when it does not transmit any signal, or when it is not authorised by the NCF to transmit.

carrier-on state: A LMES is in this state when it is authorised by the NCF to transmit and when it transmits a signal.

nominated bandwidth: The bandwidth of the LMES radio frequency transmission. The nominated bandwidth is wide enough to encompass all close-in spectral elements of the transmission which have a level greater than the specified unwanted emissions limits. The nominated bandwidth is wide enough to take account of the transmit carrier frequency stability. The nominated bandwidth is within the MSS transmit frequency band within which the LMES operates.

unwanted emissions: Unwanted emissions are those falling outside the nominated bandwidth.

# 3	Abbreviations

e.i.r.p.	Equivalent Isotropically Radiated Power

LMES	Land Mobile Earth Station

MSS	Mobile-Satellite Service

MES	Mobile Earth Station

NCF	Network Control Facility

# 4	Requirements

## 4.1	Unwanted emissions outside the bands

The unwanted emissions in the measurement bandwidth and in all directions from the LMES outside the bands 1 626.5 to 1 660.5 MHz, within which the LMES is designed to operate, shall be below the following limits.

1)	The unwanted emissions over the frequency range 30 MHz to 1 000 MHz shall not exceed the limits in Table 1.

Table 1

Limits of unwanted emissions below 1 000 MHz at a measuring distance of 10 m

| Frequency (MHz) | Quasi-peak limits (dB(µV/m)) |
| --- | --- |
| 30 to 230 | 30 |
| 230 to 1 000 | 37 |

The lower limit shall apply at the transition frequency.

2)	The unwanted emissions e.i.r.p. above 1 000 MHz, in the measurement bandwidth and in all directions shall not exceed the limits of Table 2:

Table 2

Limits of unwanted emissions above 1 000 MHz 
and outside the bands 1 626.5 to 1 660.5 MHz (Editorial Note 1)

| Frequency range | Carrier-on |  | Carrier-off |  |
| --- | --- | --- | --- | --- |
| (MHz) | e.i.r.p. limit (dBW) | Measurement bandwidth (kHz) | e.i.r.p. limit (dBW) | Measurement bandwidth (kHz) |
| 1 000 - 1 525 | -71 | 100 | -72 | 100 |
| 1 525 - 1 559 | -71 | 100 | [-103] | 3 |
| 1 559.0 - 1 600.0 | -70 | 1 000 | -70 | 1 000 |
| 1 600.0 - 1 6050 [(Note 7)] | -70 | 1 000 | -70 | 1 000 |
| 1 605.0 - 1 610.0 [(Note 7)] ] | (note 4) [(Note 4a)] | 100 | [(note 5)] | 100 |
| [1 610.0 - 1 626.0] | [-46] | 100 | [-72] | 100 |
| 1 626 - 1 626.5 | -36 | 3 | -72 | 100 |

| 1 645.5 - 1 645.6 | -16 | 3 | -63 | 3 |
| --- | --- | --- | --- | --- |
| 1 645.6 - 1 646.1 | -36 | 3 | -63 | 3 |
| 1 646.1 - 1 655.9 | -46 | 3 | -63 | 3 |
| 1 655.9 - 1 656.4 | -36 | 3 | -63 | 3 |
| 1 656.4 - 1 656.5 | -16 | 3 | -63 | 3 |
| 1 660.5 - 1 661 | -36 | 3 | -72 | 100 |
| 1 661 - 1 663.5 | -46 | 100 | -72 | 100 |
| 1 663.5 - 1 690 | -46 | 100 | -72 | 100 |
| 1 690 - 3 400 | -71 (note 2) | 100 | -72 | 100 |
| 3 400 - 10 700 | -65 (note 3) | 100 | -72 | 100 |
| 10 700 - 21 200 | -59 | 100 | -66 | 100 |
| 21 200 - 40 000 | -53 | 100 | -60 | 100 |
| NOTE 1: The lower limits shall apply at the transition frequencies. NOTE 2: In the band 3 253.0 to 3 321.0 MHz the maximum e.i.r.p. in one, and only one, 100 kHz measurement bandwidth shall not exceed -38 dBW. Elsewhere in this band the power limit in this table shall be applied. NOTE 3: In each of the bands 4 879.5 to 4 981.5 MHz, 6 506.0 to 6 642.0 MHz and 8 132.5 to 8 302.5 MHz the maximum e.i.r.p. in one, and only one, 100 kHz measurement bandwidth shall not exceed -48 dBW. In the band 9 759.0 to 9 963.0 MHz the maximum power in one, and only one, 100 kHz measurement bandwidth shall not exceed -59 dBW. Elsewhere in these bands the power limit in this table shall be applied. NOTE 4: Linearly interpolated from -70 dBW in 1 MHz at 1 605.0 MHz to -36 dBW in 1 MHz at 1 610.0 MHz. NOTE 4a: [The Russian Federation states that a level of -70 dBW/MHz in the band 1 605 to 1 607.5 MHz and a linear interpolation between -70 dBW/MHz at 1 607.5 and –36 dBW/MHz at 1 610 MHz shall be used to provide protection of GLONASS receiver operations.] NOTE 5: Linearly interpolated from -70 dBW in 1 MHz at 1605.0 MHz to -62 dBW in 1 MHz at 1610.0 MHz. NOTE 7: [In the interim period prior to 1 June 2002, the carrier-on limits of -36 dBW/1 MHz and the carrier-off limits of -62 dBW/ 1 MHz shall apply in the band 1 600 - 1 610 MHz. MES terminal manufacturers are encouraged to meet the final limits as soon as possible but not later than 1 June 2002.] (Editorial Note 2) |  |  |  |  |

Editorial Note 1 - All Administrations agreed that a global solution is a key objective of this Recommendation.

## 4.2	Maximum unwanted emission within the bands

Specification 1: Carrier-on state

The e.i.r.p. of the unwanted emissions in any 3 kHz bandwidth within the bands 1 626.5 to 1 660.5 MHz shall not exceed the limits in Table 3.

Table 3

Limits of unwanted emission within the operating band with carrier-on

| Offset from the edge of the band of the nominated bandwidth (kHz) | Maximum e.i.r.p. (dBW) |
| --- | --- |
| 0 | -3 |
| 100 | -16 |
| 200 | -36 |
| greater than 700 | -46 |

## Specification 2: Carrier-off state

The e.i.r.p. spectral density of any emission within the bands 1 626.5 to 1 660.5 MHz shall not exceed -63 dBW in any 3 kHz band.

A comparison of two different views of required unwanted emission limits and the associated bands based on input documents 8D/200 add. 1 (Att. 2) and 8D/48

_______________

* English only.

English only.

To facilitate the work of the of the SRG by correspondence, an e-mail reflector was set-up on ITU-TIES with the following Internet address: wp8d-srg@itu.int.