ITU-T Recommendation G.964 –

V-Interfaces at the Digital Local Exchange (LE) -
V.5.1-Interface (based on 2048 kbit/s) for the support of Access Network (AN)

# SUMMARY

This Recommendation defines a V-interface (V5.1) for the connection of an Access Network (AN) to a Local Exchange (LE) for the support of the following access types:

–	analogue telephone access;

–	ISDN basic access with a NT1 separate from the AN or integrated in the AN, based on Recommen-dations G.960 and I.430;

–	other analogue or digital accesses for semi-permanent connections without associated outband signalling information,

with flexible (provisioned) information channel (bearer channel) allocation but without concentration capability within the AN.

The electrical and functional interface specification uses the 2048 kbit/s parts of Recommendations G.703, G.704 and G.706.

The signalling from the PSTN user port is converted into a stimulus protocol with a functional part for the signalling path using layer 3 multiplexing for the information from the different user ports.

The information from the ISDN D-channels is frame relayed in the Access Network using the mechanisms defined in Recommendation Q.922.

A control protocol defined in this Recommendation is used for the exchange of the individual port status and control functions required for the coordination with the call control procedures in the Local Exchange.

In order to coordinate the traffic demands in the various protocols, 1, 2 or 3 communication channels can be provisioned to transport the various protocols and frame relayed information. The data link layer for the protocols is defined on the basis of Recommendations Q.920 and Q.921.

Interface V5.1 is upgradable to V5.2 through re-provisioning and by implementation of the additional functionality.

Major differences between the first V5.1 recommendation issued (06/94) with this version areAdditional definitions in clause 3

- Permanent leased line service have been clarified in clause 6

- Subclause 8.7.4 adds flow control on LAPV5-DL

- Subclause 13.3 adds conditional information elements that’s affects the messages for PSTN and Control protocol.

- Subclause 13.4.7.11 to 13.4.7.13 adds three information elements to the V5 PSTN protocol (listed in table 7.

- Table 29 to 31 have been updated and completed. Last part of the LE table was missing in the previous version. A proposal is to renumber the tables and use table 29 (three part/pages) for the AN side and table 30 for the LE side (also three pages). This changes have not been done since it affects the Table references in clause 14 as well.

- Additional subclause to annex B (B.14)

- Annex C have highest number of changes. The most important are:

- System startup (subclause C.13)

- Startup of PSTN data link (subclause C.14)

- Data link failure and reset (subclause C.15)

- PSTN restart procedure (subclause C.16)

- States in the system management procedures (subclause C.17)

- Control protocol layer 3 protection mechanism errors (subclause C.18)

- Timers (table C.1)

- Failure on layer 1 (subclause C.20)

- Restarting of interface (subclause C.21)

- Handling of timer TV1 (related to variant and ID check) (subclause C.22)

- Alignment of blocking/unblocking (subclause C.23)

- Annex J added