Draft Corrigendum 1 to Recommendation G.992.5

SUMMARY

- This text contains the draft Corrigendum 1 to the Recommendation G.992.5. It deals with the Control Plan Procedures, PSD Mask Control parameters, the Loop Diagnostic Timing Diagram and Test Parameter Messages.

CORRIGENDUM 1 TO RECOMMENDATION G.992.5

1.	Control Plane Procedures (§ 7.8)

Add new subsection with following title and text:

§ 7.8.2.4.3   Overhead Message Segmentation (supplements G.992.3 § 7.8.2.4)

An Overhead message shall be segmented if the message length P is higher than the maximum of 1024 octets per HDLC frame. Otherwise, an overhead message may still be segmented at the transmitter’s discretion. To avoid starvation of the protocol, it may be desirable to reduce the transmit duration of a segment to a value significantly shorter than the timeout of the highest priority message, e.g., 200 ms.

If an overhead message of length P is semented into N segments, the n-th segment (1  n  N) contains Pn message octets. To allow for the message indicator and message type to be included in each message segment, the following relationship shall be satisfied:

n=1N(Pn−2) = P−2 , with ∀n:2<Pn<P

The last (P-2) message octets of the non-segmented message shall be mapped to the N message segments in the same order as they are contained in the non-segmented message. The third message octet of the non-segmented message shall map to the third message octet of the first message segment. The last octet of the non-segmented message shall map to the PN-th message octet of the N-th message segment. Each message segment shall be transmitted using the HDLC frame structure encapsulation defined in § 7.8.2.3, with Pn message octets contained in the HDLC frame encapsulating the n-th message segment. Each message segment may contain a different number of message octets. The maximum number of message segments is 8 (i.e., 2  N  8). Figure 7-1 shows the bit assignments for the the control field.

Figure 7-1/G.992.5: Bit assignment for control field.

The following shall apply to all encapsulated message segments:

- The message segment length is variable length with a maximal length of 1024 message octets per message segment;

- The address field shall be the same for all message segments (identical to the non-segmented case).

- Each of the three least significant bits of the control field shall be the same for all message segments (identical to the non-segmented case);

- The segment ID field of the control field shall contain the message segment ID n, with n in the range 0 to 7 (msb of message segment ID mapping to msb of the segment ID field);

- The segment ID shall count down from N-1 to 0 where n is the total number of segments in the message;

- The flag field shall be set to 102 for the first and last segment and shall be set to 002 for all other segments;

- A segmented message shall have at least two segments (i.e., N  2);

- The first octet of the message segment shall be the message designator (same for all segments, identical to the non-segmented case);

- The second octet of the message segment shall be the message type (same for all segments, identical to the non-segmented case).

An example sequence of control fields in subsequent message segments is shown in Figure 7-2.

Figure 7-2/G.992.5: Example showing the control field in a segmented message of length N.

###Editor’s Note: Change “n” to “N” in above Figure.

Each transmitted message segment shall be acknowledged by the far-end with a segment acknowledge message, except the last one. The last command message segment shall be acknowledged by the far-end with the appropriate response message (identical to the non-segmented case). The last response message segment shall not be acknowledged. The Table 7-17a defines the segment acknowledge message. The following shall apply to all encapsulated segment acknowledge messages:

- HDLC frame shall contain 5 message octets (P=5);

- The two least significant bits of the address field shall be identical to the corresponding bits in the address field of the acknowledged message segment. All other bits of the address field shall be set to 02.

- The second least significant bit of the control field shall either indicate a command message (request to continue the response, e.g., L2 grant) or a response message (request to continue the command, e.g., OLR);

- The least significant bit of the control field shall toggle for each acknowledgement message as it normally toggles for each command/response message (see § 7.8.2.4.2).

- All other bits of the control field shall be set to 02.

Table 7-17a/G.992.3 – Segment Acknowledge Message

| Message octet number | Message octet definition |
| --- | --- |
| Octet 1 | Message designator 1111 0000b for acknowledgement of high priority message segment 1111 0001b for acknowledgement of normal priority message segment 1111 0010b for acknowledgement of low priority message segment |
| Octet 2 | Message type of segment acknowledge message 0116 |
| Octet 3 | Acknowledged message segment ID (in range 1 to 7) |
| Octet 4 | Message Designator (first message octet of acknowledged message segment) |
| Octet 5 | Message Type (second message octet of acknowledged message segment) |

Every message segment must be acknowledged by the far-end before the next message segment is transmitted. The segment acknowledge message shall not be segmented.

Timeouts shall be defined as follows:

- For a non-segmented message, timeout shall apply between the last octet of the command message transmitted and the first octet of the response message received.

- For a segmented response message, the timeout shall apply between the last octet of the command message transmitted and the first octet of the first message segment received.

- A timeout corresponding to the command priority shall apply between the last octet of the message segment transmitted and the first octet of the segment acknowledge message received.

- Another timeout corresponding to the command priority shall apply between the last octet of the segment acknowledge message transmitted and the first octet of the next message segment received.

If a timeout expires, the transmitter may repeat the last transmitted message. This repeated message can be an unsegmented command message, a command/response message segment (except the last segment of a response message), or a segment acknowledge message. Alternately, the ATU may abandon the message after an implementation specific number of retransmissions.

If a segment of a segmented message is aborted by higher priority message, the transmitter shall retransmit only the aborted segment and continue to transmit the remaining segments if any. A segment acknowledge message shall not be aborted by a higher priority message.

Examples of overhead message segmentation and applicable timeouts are shown in Figure 7.10 and Figure 7.11.

Figure 7.10/G.992.3 - Example of segmentation of a command and response of priority 2 in two segments.

Figure 7.11/G.992.3 - Example of retransmission of segment and acknowledgment.

2.	PSD Mask Control Parameters

Definition of control parameters (§ 8.5.1)

Change the text of § 8.5.1 as follows:

2.	Low frequency end and High frequency end of MIB PSD mask (f)

- t1 = roundup(f_pb_start/f) or  (73 ≤ t1 ≤ 271)

- ….[rest of text unchanged]

3.	MIB PSD stopband in lower frequency part

if  (73 ≤ t1 ≤ 271) then :

- ….[rest of text unchanged]

4.	MIB PSD inband shaping

- if t1 = roundup(f_pb_startf) then for n = 1 to N-1:

- if  (73 ≤ t1 ≤ 271) then for n = 2 to N-1 :

- ….[rest of text unchanged]

- NOTE – 	If the first breakpoint has subcarrier index   73 ≤ t1 ≤ 271, then a stopband is created in the lower frequency   …[rest of text unchanged]

- Change the Figure 8.2 as follows:

Figure 8.2/G.992.5 – Illustration of a stopband in the first part of the frequency band.

3.	Timing Diagram of the Loop Diagnostics Procedures

Loop Diagnostics Timing Diagram (Part 1) (§ 8.15.6)

The R-MSG-PCB length is defined as 2560, which corresponds to 8*(NSCds+48+16) where NSCds=256. For NSCds=512, it should be 4608.

The C-QUIET4 length is defined as 2954, which corresponds to 64+64+10+256+LEN_R-MSG-PCB where LEN_R-MSG-PCB = 2560. For NSCds=512, it should be 5002, corresponding to LEN_R-MSG-PCB = 4608.

In Figure 8-14/G.992.5 - Loop Diagnostics Timing Diagram (part 1), change the following:

For R-MSG-PCB, replace “2560” by “8*(NSCds+64)”

For C-QUIET4, replace “2594” by “394+8*(NSCds+64)”

#### 4.	Test Parameter Messages (§ 9.4.1.10)

Correct the section text as follows:

The PMD test parameters read commands shall be used to access the value of certain PMD test parameters maintained by the far ATU in accordance with the description of the PMD function. The local parameter values shall be retrieved as described in this sub-clause. The PMD test parameter read command may be initiated by either ATU as shown in Table 9-28. The responses shall be using the command shown in Table 9-29. The PMD test parameter read command shall consist of two to six octets. The first octet shall be PMD test parameter command designator shown in Table 9-4. The remaining octets shall be as shown in Table 9-28. The PMD test parameter read response command shall be multiple octets. The first octet shall be PMD test parameter read command designator shown in Table 9-4. The second shall correspond to received management counter read command. The remaining octets shall be as shown in Table 9-29. The octets shall be sent using the format described in § 7.8.2.3 and using the protocol described in § 7.8.2.4.

Table 9-28/G.992.5 - PMD Test Parameter Read Commands Transmitted by the Initiator

| Message length (Octets) | ELEMENT NAME (Command) |
| --- | --- |
| 3 | 0116 Single Read followed by 1 octet describing the test parameter id |
| 2 | 0316 Next Multiple Read |
| 4 | 0416 Multiple Read Block followed by 2 octets describing the sub-carrier index |
| 6 | 0516 Block Read followed by 2 octets describing the start sub-carrier index 2 octets describing the stop sub-carrier index |
|  | All other octet values are reserved by the ITU-T. |

Table 9-29/G.992.5 - PMD Test Parameter Read Command Transmitted by the Responder

| Message length (Octets) | ELEMENT NAME (Command) |
| --- | --- |
| Variable (see Note) | 8116 followed by octets for the test parameter arranged for the single read format |
| 12 | 8216 followed by octets for the test parameters arranged for the multiple read format |
| 2 | 8016 NACK |
| Variable (see Note) | 8416 followed by octets for the test parameter arranged for the block read format |
|  | All other octet values are reserved by the ITU-T. |
| NOTE – Variable length equals 2 plus length shown in Table 9-30. |  |

Upon receipt of one of the PMD test parameter read commands, the receiving ATU shall transmit the corresponding response message. If an unrecognised test parameter is requested, the response shall be a PMD test parameter command for NACK. The function of the receiving or transmitting ATUs is not otherwise affected.

The PMD test parameters are all derived according to the procedures in the PMD function sub-clause of this Recommendation. Following initialization, the PMD shall maintain training test parameters until the overhead command for update test parameters is received.

The parameters are transferred in the order and format defined in Table 9-30. During a test parameter read command for single read, all information for the test parameter is transferred. If the test parameter is an aggregate parameter, only one value is transferred. If the test parameter has a value per sub-carrier, then all values are transferred from sub-carrier index #0 to sub-carrier index #NSC-1 in a single message. The format of the octets is as described in PMD sub-clause. Values that are formatted as multiple octets shall be inserted in the response message most significant to least significant octet order.

During a test parameter read command for multiple read or next, information for all test parameters associated with a particular sub-carrier is transferred. Aggregate test parameters are not transferred with the PMD test parameter read command for multiple read or next. The sub-carrier used for a PMD test parameter read command for multiple read shall be the sub-carrier contained within the command. This sub-carrier index shall be saved. Each subsequent PMD test parameter command for next shall increment and use the saved sub-carrier index. If the sub-carrier index reaches NSC, the response shall be a PMD test parameter command for NACK. The per sub-carrier values are inserted into the message according to the numeric order of the octets designators show in Table 9-30. The format of the octets is as described in PMD sub-clause of this Recommendation. Values that are formatted as multiple octets shall be inserted in the response message most significant to least significant octet order.

During a test parameter read command for a block read, information for the test parameter is transferred within the specified block of sub-carriers. Aggregate test parameters are not transferred with the PMD test parameter block read command. If the test parameter has a value per sub-carrier, then all values are transferred from sub-carrier index #start sub-carrier to sub-carrier index #stop sub-carrier in a single message. The format of the octets is as described in the PMD sub-clause. Values that are formatted as multiple octets shall be inserted in the response message most significant to least significant octet order.

Table 9-30/G.992.5 - PMD Test Parameter ID Values

| Test Parameter ID | Test Parameter Name | Length for Single Read | Length for Multiple Read | Length for Multiple Read |
| --- | --- | --- | --- | --- |
| 0116 | Channel Transfer Function Hlog(f) per sub-carrier | 2 + NSC * 2 octets | 4 octets | 2 + (stop sub-carrier – start sub_carrier + 1) * 2 octets |
| 0216 | Reserved by ITU-T |  |  |  |
| 0316 | Quiet Line Noise PSD QLN(f) per sub-carrier | 2 + NSC octets | 3 octet | 2 + (stop sub-carrier – start sub_carrier + 1) octets |
| 0416 | Signal to noise ratio SNR(f) per sub-carrier | 2 + NSC octets | 3 octet | 2 + (stop sub-carrier – start sub_carrier + 1) octets |
| 0516 | Reserved by ITU-T |  |  |  |
| 2116 | Line Attenuation LATN | 2 octets | N/a | N/a |
| 2216 | Signal Attenuation SATN | 2 octets | N/a | N/a |
| 2316 | Signal-to-Noise Margin SNRM | 2 octets | N/a | N/a |
| 2416 | Attainable Net Data Rate ATTNDR | 4 octets | N/a | N/a |
| 2516 | Near-end Actual Aggregate Transmit Power ACTATP | 2 octets | N/a | N/a |
| 2616 | Far-end Actual Aggregate Transmit Power ACTATP | 2 octets | N/a | N/a |

In transferring the value of the channel transfer function Hlog(f), the measurement time shall be inserted into the message, followed by the value m (see § 8.12.3.1). The measurement time is included only once in a PMD test parameter response for single read. The measurement time is included in each response for multiple read or next multiple read.

In transferring the value of the quiet line noise QLN(f), the measurement time shall be inserted into the message, followed by the n value (see § 8.12.3.2). The measurement time is incuded only once in a PMD test parameter response for single read. The measurement time is included in each response for multiple read or next multiple read.

In transferring the value of the signal-to-noise ration SNR(f), the measurement time shall be inserted into the message, followed by the snr value (see § 8.12.3.3). The measurement time is included only once in a PMD test parameter response for single read. The measurement time is included in each response for multiple read or next multiple read.

The values for test parameters defined with fewer bits than shown in Table 9-30, shall be inserted into the message using the least significant bits of the two octets. Unused more significant bits shall be set to 0 for unsigned quantities and to the value of the sign bit for signed quantities.

##### 9.4.1.10.2	Single Read Command

See ITU-T Recommendation G.992.3 § 9.4.1.10.1.

##### 9.4.1.10.2	Multiple Read Protocol with Next

See ITU-T Recommendation G.992.3 § 9.4.1.10.2.

##### 9.4.1.10.3	Block Read Command

See ITU-T Recommendation G.992.3 § 9.4.1.10.3.

______________________