Draft new Corrigendum 1 to ITU-T G.726

Correction of Annex A Extensions of Recommendation 
G.726 for use with uniform-quantized input and output

# AAP Summary

G.726 contains the specification for the adaptive differential PCM voice coding algorithm at 16, 24, 32, and 40 kbit/s. Its Annex A contains the specification for linear PCM input and output. This corrigendum to G.726 fixes an omission in Decoder Block LIMO of its Annex A, which limits the output linear two’s complement sample to a 14-bit value. The pseudo-code, as originally published, misses the case for reconstructed samples SR equal to 57344.

______________________