Recommendation H.248.22 "Shared Risk Group Package"

- Summary:

H.248.22 "Shared Risk Package" describes a package to enable the Media Gateway Controller (MGC) to indicate to the Media Gateway (MG) to use or to not use network resources associated with a shared risk group when setting up connections. When network connections are associated with ephemeral terminations in the MG certain resources are used. A failure in the MG may result in the loss of certain groups of resources, whilst other groups of resources remain unaffected. The resources that share a risk of failure are called a shared risk group. A shared risk group is simply a grouping of the network resources (e.g. IP interfaces) sharing the same risk of failure. Each shared risk group could for example contain a number of IP interfaces located on the same piece of hardware. If a resource failure occurs only terminations in the shared risk group corresponding to the failed resource are affected while other terminations are not.