Draft new ITU-T Recommendation H.248.32

Detailed Congestion Reporting Package

##### AAP Summary

H.248.32 “Detailed Congestion Reporting Package” defines a package that allows the MG to report the current state of its resource usage to the MGC. The MGC can, according to its own capabilities, customize the set of resources reported as well as when the reports are provided. The resources that can be reported may be in the form of general resources and/or individual resources, and can represent either hardware or software resources. Based on the resource usage report received from the MG, the MGC may take corrective action to improve the efficiency of the whole system (e.g. re-route calls, reduce possible message intensive audits, throttle calls, etc.).

In general, the precise definition of the following resources is left to implementation. The resources whose usage may be reported with this package include: General Resources of the MG, Digital Signal Processor (DSP) resources, Internet Protocol (IP) resources, Asynchronous Transfer Mode (ATM), other resources mutually recognized by the MG & MGC.

The MGC can customize the reporting of the MG’s resource usage. The MGC can enable reporting for only those resources upon which it is capable of taking meaningful action. The MGC can also determine at what threshold level(s) the MG sends reports. The threshold levels are specified as a percentage of the resource used. The number of levels reported is also selectable.

The reporting of different resource shortages will allow the MGC to take different preventive actions. The specific preventive action that the MGC takes in response to receiving a resource shortage report is beyond the scope of this package.

____________________