ITU-T Recommendation H.450.12 - Common Information Additional Network Feature for H.323

Summary:

This new Recommendation describes the procedures and the signaling protocol for the Common Information Additional Network Feature in H.323 (ANF-CMN).

ANF-CMN enables the exchange of Common Information between ANF-CMN endpoints. This Common Information is a collection of miscellaneous information that relates to the user or equipment at one end of a connection and includes one or more of the following: Feature Identifiers, Party Category. This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1.

This Recommendation requires H.323 version 2 (1998) or later. Version 2 products can be identified by H.225.0 messages containing a protocolIdentifier = {itu-t (0) recommendation (0) h (8) 2250 version (0) 2} and H.245 messages containing a protocolIdentifier = {itu-t (0) recommendation (0) h (8) 245 version (0) 3}.

The procedures and the signaling protocol of this Recommendation are derived from the Common Information Additional Network Feature specified in ISO/IEC 15771 and 15772.