Draft new ITU-T Recommendation H.460.20

Location number within H.323 systems

AAP Summary

Nowadays more and more telephone numbers do not provide any information on geographic localization of the calling party. Although there is a need to have an accurate geographic position, there is also a need to convey already existing information about localization coming from the PSTN.

In the PSTN, a parameter is defined to transport a location number coming from a Mobile Network. This parameter (Location Number in ISUP) is also used inside PSTN to give more accurate information on the calling party, when the calling party number can not be used to provide any geographic information. For example this information is used for IN services like premium services, (free) 800-numbers, emergency services, etc.

This Recommendation defines a parameter “LocationSourceAddress” to convey location numbers as a field that carries a sequence of an alias address that matches the Location Number from/to non-H.323 networks. This ensures better interwork with ISUP. Coming from non-H.323 networks this field can be inserted by Gateways. Coming from H.323 networks, this field can be inserted by originating Gatekeepers.

It should be noted that in this Recommendation, the geographic information provided is only a number conveyed. This number may be a zip code, an E.164 number, a zone number, or whatever the need may be. This parameter should not be used to convey accurate geographical information (such as earth coordinates).

__________________