Draft new Recommendation K.63

Maintaining the suitability of production telecommunications equipment to its intended electromagnetic environment

Summary

This Recommendation suggests some Electromagnetic Compatibility tests that may be used by both Telecommunications Operators and Equipment Manufacturers to determine the suitability of a batch of Telecommunications Equipment to its intended Electromagnetic Environment.

It is recognised that variation will occur between the Electromagnetic Compatibility performance of the Telecommunications Equipment item tested to demonstrate compliance with market entry requirements and the Electromagnetic Compatibility performance of Telecommunications Equipment items manufactured subsequently.  Hence variation in the Electromagnetic Compatibility performance will be seen within a batch of Telecommunications Equipment either received by the Telecommunications Operator or manufactured by the Equipment Manufacturer.

Methods to allow both Telecommunications Operator and Equipment Manufacturers to demonstrate the compliance of a batch of Telecommunications Equipment are therefore needed.  As it is not economically feasible to test the Electromagnetic Compatibility performance of every item within the batch, this Recommendation suggests that just three tests be performed on a small number of Equipment items selected at random from the batch.

The recommended tests are Radiated Emissions, Electrostatic Discharge and Electrical Fast Transient.  It is the past experience of Telecommunications Operators that the largest variation in performance occurs for these three tests.

Telecommunications Operators may therefore use this Recommendation to assure the suitability of a batch of equipment received from a manufacturer.  Equipment Manufacturers may use this Recommendation to monitor the variation in Electromagnetic Compatibility performance from their manufacture line.