| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 7-E April 1997 Original: English |

Question: 15/11

STUDY GROUP 11 – REPORT R 7

SOURCE*:	STUDY GROUP 11

TITLE:	REPORT OF THE MEETING HELD IN GENEVA FROM 13-31 JANUARY 1997: PART I - IMPLEMENTORS' GUIDE FOR Q.2110 (07/1994)

________

The reports of the meeting of Study Group 11 and its Working Parties are published in the documents listed in COM 11-R 1.

STATUS - This text was approved by Study Group 11, January 1997.

NOTE - This text is also available from the ITUDOC.

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.2110 (07/94): B-ISDN ATM Adaptation Layer - Service Specific Connection Oriented Protocol (SSCOP)

## 1.2	Background

This guide is a compilation of reported defects, their resolutions and minor upgrades to the 1994 edition of the ITU-T Recommendation Q.2110[1]. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following categories:

•	editorial errors;

•	technical errors, such as omissions, inconsistencies, etc.; and

•	ambiguities.

In addition, this guide records minor enhancements to the Recommendation in the following categories:

•	increased interoperability; and

•	deployment possibly in new environments.

### 1.4	Contacts

| Rapporteur (Q.15/11): | Dr. Pietro Schicker Swiss Telecom PTT/ Scientific Consulting | Tel: +41 1 938 1555 Fax: +41 1 938 1557 email: schicker@scicon.ch |
| --- | --- | --- |

| Former Rapporteur: | G. Ratta Bellcore | Tel: +1 908 758 5696 Fax: +1 908 758 4131 email: gratta@notes.cc.bellcore.com |
| --- | --- | --- |

### 2	Editorial errors

None.

### 3	Technical errors

None.

### 4	Ambiguities

### 4.1	Use of SSCOP in conjunction with SSCF

Mod. a)		Add the following sentence at the end of § 1:

In particular, an SSCOP entity can only be used in conjunction with the requirements specified by a particular SSCOP user, e.g., a particular SSCF Recommendation, and as a peer entity to the corresponding SSCOP entity, operating in conjunction with the requirements specified by the same SSCOP user, e.g., the same SSCOP Recommendation.

### 5	Increased interoperability

### 5.1	Increased robustness during power-up connection establishment

Synopsis

An option is introduced to allow the SSCOP entity to accept a BGN PDU in the state IDLE irrespective of the values of N(SQ) in the BGN PDU and the state variable VR(SQ).

Mod. a)		In the SDL diagram (Figure 20/Q.2110), replace the actions after receipt of a "BGN PDU" in the state "Idle" with the definition as shown in Figure 5.1 at the right-hand side.

figure 5.1

Mod. b)		In § B.5.3, add a further row to the table as follows

| Item # | Protocol feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| SP11 | Is power-up robustness implemented? | O | SDL | Yes:_No:_X:_ |

### 6	Deployment possibility in new environments

None.

_____________