| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 137-E April 1999 Original: English |

Question: 16/11

Texte disponible seulement enText available only in E

STUDY GROUP 11 – REPORT R 137

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (03/99) FOR RECOMMENDATION Q.704 (07/96)

________

# 1	Introduction

This document contains additions to the Service Indicator of values for the Signal Processing Network Equipment User Part (SPNEUP) of Q.55 and the AAL type 2 Signalling Transport Converter on Broadband MTP of Q.2150.1. It updates the User Part identities used in the User Part Unavailable message (UPU) in line with the Service Indicator values. It reserves in Q.704 the message codes for the extended changeover order and acknowledgement of Q.2210 broadband MTP. It also corrects some editorial errors in the SDL.

## 1.1	References

[1]	ITU-T Recommendation Q.704 (07/96) - Specifications of Signalling System No. 7, Message Transfer Part, Signalling network functions and messages.

## 1.2	Background

This guide is a compilation of four minor upgrades to the 1996 edition of the ITU-T Recommendation Q.704, and corrections of some editorial errors in the SDL.

The guide is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records minor enhancements to the Recommendation in the following categories:

Attention: This is not an ITU publication made available to the public, but an internal ITU Document intended only for use by the Member States of the ITU and by its Sector Members and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of the ITU.

•	deployment possibility in new environments;

•	corrections of editorial errors;

•	correction of publication error.

### 1.4	Contact

| Rapporteur Q.16/11: (MTP Part) | Mike McGrew Lucent Technologies 6200 East Broad Street Columbus, Ohio 43213 United States | Tel: +1 614 860 3585 Fax: +1 614 868 2806 Email: mmcgrew@lucent.com |
| --- | --- | --- |

### 1.5	Document history

| Version | Summary |
| --- | --- |
| (03/99) | New Implementors' Guide |

### 2	Deployment possibilities in new environments

### 2.1	Change to Q.704 service indicator and user part identification in UPU messages

Replace the definition of service indicator with the following:

"

### 14.2.1	Service indicator

The service indicator is used by signalling handling functions to perform message distribution (see 2.4) and, in some special applications, to perform message routing (see 2.3).

The service indicator codes for the international signalling network are allocated as follows:

| bits | D | C | B | A |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Signalling network management messages |
|  | 0 | 0 | 0 | 1 | Signalling network testing and maintenance messages |
|  | 0 | 0 | 1 | 0 | Reserved |
|  | 0 | 0 | 1 | 1 | SCCP |
|  | 0 | 1 | 0 | 0 | Telephone User Part |
|  | 0 | 1 | 0 | 1 | ISDN User Part |
|  | 0 | 1 | 1 | 0 | Data User Part (call and circuit-related messages) |
|  | 0 | 1 | 1 | 1 | Data User Part (facility registration and cancellation messages) |
|  | 1 | 0 | 0 | 0 | MTP Tester |
|  | 1 | 0 | 0 | 1 | Broadband ISDN User Part |
|  | 1 | 0 | 1 | 0 | Satellite ISDN User Part |
|  | 1 | 0 | 1 | 1 | Signal Processing Network Equipment User Part |
|  | 1 | 1 | 0 | 0 | Reserved for AAL type 2 Signalling Transport Converter |
|  | 1 | 1 | 0 | 1 | )Reserved |
|  | 1 | 1 | 1 | 0 | ) |
|  | 1 | 1 | 1 | 1 | ) |

The allocation of the service indicator codes for national signalling networks is a national matter. However, it is suggested to allocate the same service indicator code to a User Part which performs similar functions as in the international network."

Replace the definition of User Part identity in the User Part Unavailable (UPU) message with:

"

### 15.17.4	The user part identity is coded as follows:

|  | bit | D | C | B | A |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 0 | 0 | 0 | Reserved |
|  |  | 0 | 0 | 0 | 1 | Reserved |
|  |  | 0 | 0 | 1 | 0 | Reserved |
|  |  | 0 | 0 | 1 | 1 | SCCP |
|  |  | 0 | 1 | 0 | 0 | TUP |
|  |  | 0 | 1 | 0 | 1 | ISUP |
|  |  | 0 | 1 | 1 | 0 | DUP |
|  |  | 0 | 1 | 1 | 1 | Reserved |
|  |  | 1 | 0 | 0 | 0 | MTP Tester |
|  |  | 1 | 0 | 0 | 1 | Broadband ISDN User Part |
|  |  | 1 | 0 | 1 | 0 | Satellite ISDN User Part |
|  |  | 1 | 0 | 1 | 1 | Signal Processing Network Equipment User Part |
|  |  | 1 | 1 | 0 | 0 | Reserved for AAL type 2 Signalling Transport Converter |
|  |  | 1 | 1 | 0 | 1 | ) |
|  |  | 1 | 1 | 1 | 0 | ) Reserved |
|  |  | 1 | 1 | 1 | 1 | ) |

"

## 2.2	Reservation of extended changeover order and acknowledgement message codes

The extended changeover order and acknowledgement (XCO and XCA) message codes used in broadband MTP are reserved in narrow-band MTP.

Replace Table 1/Q.704 with:

Table 1/Q.704

Heading code allocation of signalling network management messages

| Message |  | H1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| group | H0 | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
|  | 0000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHM | 0001 |  | COO | COA | 1) | 2) | CBD | CBA |  |  |  |  |  |  |  |  |  |
| ECM | 0010 |  | ECO | ECA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FCM | 0011 |  | RCT | TFC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TFM | 0100 |  | TFP | * | TFR |  | TFA | * |  |  |  |  |  |  |  |  |  |
| RSM | 0101 |  | RST | RSR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MIM | 0110 |  | LIN | LUN | LIA | LUA | LID | LFU | LLT | LRT |  |  |  |  |  |  |  |
| TRM | 0111 |  | TRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DLM | 1000 |  | DLC | CSS | CNS | CNP |  |  |  |  |  |  |  |  |  |  |  |
|  | 1001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UFC | 1010 |  | UPU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1111 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1 - Values marked 1) and 2) are reserved (codes used in Q.2210 for extended changeover order (XCO) and extended changeover acknowledgement (XCA) respectively). NOTE 2 - Values marked * should not be used (codes used in the Yellow Book for TFP and TFA acknowledgement). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CBA Changeback-acknowledgement signal CBD Changeback-declaration signal CHM Changeover and changeback messages CNP Connection-not-possible signal CNS Connection-not-successful signal COA Changeover-acknowledgement signal COO Changeover-order signal CSS Connection-successful signal DLC Signalling-data-link-connection-order signal DLM Signalling-data-link-connection-order message ECA Emergency-changeover-acknowledgement signal ECM Emergency-changeover message ECO Emergency-changeover-order signal FCM Signalling-traffic-flow-control messages LFU Link forced uninhibit signal LIA Link inhibit acknowledgement signal LID Link inhibit denied signal LIN Link inhibit signal LLT Link local inhibit test signal LUA Link uninhibit acknowledgement signal LUN Link uninhibit signal LRT Link remote inhibit test signal MIM Management inhibit messages RCT Signalling-route-set-congestion-test signal RSM Signalling-route-set-test message RSR Signalling-route-set-test signal for restricted destination (national option) RST Signalling-route-set-test signal for prohibited destination TFA Transfer-allowed signal TFC Transfer-controlled signal TFM Transfer-prohibited-transfer-allowed-transfer-restricted messages TFP Transfer-prohibited signal TFR Transfer-restricted signal (national option) TRA Traffic-restart-allowed signal TRM Traffic-restart-allowed message UFC User part flow control messages UPU User part unavailable signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 3	Alignment of SDL with text

## 3.1	Editorial errors in Figure 29/Q.704

Figure 29 sheet 11 has a page reference error and sheet 16 has a connector reference error. These errors were also present in the report COM XI-R 191-E.

## 3.2	Correction of publication error

The published version of Q.704 (07/96) has an identical diagram for Figure 30/Q.704 (sheet 5 of 6) and (sheet 6 of 6), although the titles are different. The correct diagram for Figure 30/Q.704 (sheet 6 of 6) was included in the report COM XI-R 191-E, and is as follows:

______________