|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2001 - 2004 | COM 11-R 6-E January 2001 Original: English |

Question: 13/11

Texte disponible seulement enText available only inE

STUDY GROUP 11 - REPORT R 6

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (12/2000) FOR RECOMMENDATION Q.704 (07/96)

________

Summary

This is the latest Implementors' Guide to Recommendation Q.704 (07/96). This document contains the contents of the previous version (12/99, which itself contained the first (03/99) version) of the Implementors' Guide.

This document extends the 03/99 Implementors' Guide, which defined additions to the Service Indicator of values for the Signal Processing Network Equipment User Part (SPNEUP) of Q.55 and the AAL type 2 Signalling Transport Converter on Broad Band MTP of Q.2150.1. It updated the User Part identities used in the User Part Unavailable message (UPU) in line with the Service Indicator values. It reserved in Q.704 the message codes for the extended changeover order and acknowledgement of Q.2210 broad band MTP. It also corrected some editorial errors in the SDL.

The extensions in the 12/99 document consisted of the allocation of a Service Indicator (decimal 13) to the Bearer Independent Call Control MTP User (Q.1901), and another (decimal 14) to the Media Gateway Control Protocol MTP User (H.248).

This document includes all these updates, and in addition clarifies the linkset emergency restart procedures. It also contains the description of the Service Indicator (SI) extension mechanism agreed at the 12/99 meeting. Note that the ITU is considering the deletion of the Data User Part (DUP), of Recommendation X.61/Q.741, so the DUP SIs have been marked "reserved".

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.704 (07/96)  Specifications of Signalling System No. 7, Message Transfer Part, Signalling network functions and messages.

## 1.2	Background

This guide is a compilation of six minor upgrades to the 1996 edition of the ITU-T Recommendation Q.704, corrections of some editorial errors in the SDL and text, and includes the Service Indicator (SI) extension mechanism.

The guide is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records minor enhancements to the Recommendation in the following categories:

•	deployment possibility in new environments;

•	corrections of editorial errors;

•	clarifications of text;

•	correction of publication error.

## 1.4	Contacts

| Rapporteur Q.16/11 (MTP Part): | Michael McGrew Lucent Technologies 6200 East Broad Street Columbus, Ohio 43213 United States | Tel: +1 614 860 3585 Fax: +1 614 868 3998 E-mail: mmcgrew@lucent.com |
| --- | --- | --- |

## 1.5	Document History

| Version | Summary |
| --- | --- |
| 03/1999 | New Implementors' Guide |
| 11/99 | New SI and UP values 13 and 14 allocated. SI value 15 reserved for extension. |
| 12/00 | Clarification of linkset emergency restart, addition of SI extension mechanism, clarification of SLS values for SNM messages. DUP SIs marked reserved. |

# 2	Deployment possibilities in new environments

## 2.1	Change to Q.704 service indicator and User Part identification in UPU messages

Replace the definition of service indicator with the following:

### "14.2.1	Service indicator

The service indicator is used by signalling handling functions to perform message distribution (see 2.4) and, in some special applications, to perform message routing (see 2.3).

The service indicator codes for the international signalling network are allocated as follows:

| bits | D | C | B | A |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Signalling network management messages |
|  | 0 | 0 | 0 | 1 | Signalling network testing and maintenance messages |
|  | 0 | 0 | 1 | 0 | Reserved |
|  | 0 | 0 | 1 | 1 | SCCP |
|  | 0 | 1 | 0 | 0 | Telephone User Part |
|  | 0 | 1 | 0 | 1 | ISDN User Part |
|  | 0 | 1 | 1 | 0 | Reserved |
|  | 0 | 1 | 1 | 1 | Reserved |
|  | 1 | 0 | 0 | 0 | MTP Tester |
|  | 1 | 0 | 0 | 1 | Broadband ISDN User Part |
|  | 1 | 0 | 1 | 0 | Satellite ISDN User Part |
|  | 1 | 0 | 1 | 1 | Signal Processing Network Equipment User Part |
|  | 1 | 1 | 0 | 0 | Reserved for AAL type 2 Signalling Transport Converter |
|  | 1 | 1 | 0 | 1 | Bearer Independent Call Control (BICC, Q.1901) |
|  | 1 | 1 | 1 | 0 | Reserved For Gateway Control Protocol (GCP, H.248) |
|  | 1 | 1 | 1 | 1 | Reserved for extension |

The allocation of the service indicator codes for national signalling networks is a national matter. However, it is suggested to allocate the same service indicator code to a User Part which performs similar functions as in the international network. Note that the ITU is considering deleting the Data User Part."

Replace the definition of User Part identity in the User Part Unavailable (UPU) message with:

"15.17.4	The User Part identity is coded as follows:

| bit | D | C | B | A |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Reserved |
|  | 0 | 0 | 0 | 1 | Reserved |
|  | 0 | 0 | 1 | 0 | Reserved |
|  | 0 | 0 | 1 | 1 | SCCP |
|  | 0 | 1 | 0 | 0 | TUP |
|  | 0 | 1 | 0 | 1 | ISUP |
|  | 0 | 1 | 1 | 0 | Reserved |
|  | 0 | 1 | 1 | 1 | Reserved |
|  | 1 | 0 | 0 | 0 | MTP Tester |
|  | 1 | 0 | 0 | 1 | Broadband ISDN User Part |
|  | 1 | 0 | 1 | 0 | Satellite ISDN User Part |
|  | 1 | 0 | 1 | 1 | Signal Processing Network Equipment User Part |
|  | 1 | 1 | 0 | 0 | Reserved for AAL type 2 Signalling Transport Converter |
|  | 1 | 1 | 0 | 1 | Bearer Independent Call Control (BICC, Recommendation Q.1901) |
|  | 1 | 1 | 1 | 0 | Reserved for Gateway Control Protocol (GCP, Recommendation H.248) |
|  | 1 | 1 | 1 | 1 | Reserved |

"

## 2.2	Service Indicator extension

A Service Indicator extension mechanism has been defined to allow new SS No. 7 applications using the MTP to be deployed.

This mechanism is backwards (and forwards) compatible, and does not require changes to STPs in the SS No. 7 network.

### 2.2.1	Outline of the extension

Messages for applications using this mechanism have the general structure shown below, including the fields marked with *:

Figure 1/IG  General MSU format at MTP level 3

The User Data offset and Extended Service Indicator fields are placed after the routing label (see Figure 1/IG), this change of the MSU format is transparent to intermediate STPs.

In signalling end points, only the MTP distribution function and small parts of MTP management (signalling traffic flow control) have to be modified.

### 2.2.2	Backward Compatibility considerations

The MTP payload size is (minimally) reduced for new user parts, this does not cause a problem for existing user parts and thus creates no compatibility problems. New user parts should be made aware of this reduced size at the time of their specification.

The MTP network management procedure user part unavailability (within signalling traffic flow control (TSFC)) broadcast of STATUS indications in signalling end points involving the new user parts has to be extended.

If an MSU assigned to a "new" user part is sent to a node not supporting it, that node's MTP might also not yet support the new message/UPU format. In this case it replies with an old format UPU message identifying user 1111 as unavailable. In the node receiving the UPU no information would be available as to which of the new user parts was "unequipped". As, however, no new user part at all is available at the "old" node, informing all new user parts of this unavailability cannot cause harm. New user parts can be designed to ignore any "unequipped" STATUS for destinations with which no signalling relation exists.

New nodes would have to support the two possible formats of UPU message, the old and the new one (see Figure 3/IG), and choose among them according to which type of user part is concerned.

Thus backward compatibility with nodes not providing the new user parts is guaranteed.

### 2.2.3	Details of the new MSU format, and forward compatibility considerations

The User Data offset field is a mechanism to support forward compatibility, and is included in the MTP header when the SI field is "1111". This provides a facility for future format extensions such as a further SI extension, an SLS extension, or even an extension of the point codes.

| H | G | F | E | D | C | B | A | Bit Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSB Sub-service field LSB |  |  |  | MSB Service Indicator LSB 1 1 1 1 |  |  |  | 1 |
| LSB DPC |  |  |  |  |  |  |  | 2 |
| OPC LSB |  | MSB DPC |  |  |  |  |  | 3 |
| OPC |  |  |  |  |  |  |  | 4 |
| MSB LSB SLS |  |  |  | MSB OPC |  |  |  | 5 |
| MSB LSB user data offset 0 0 0 0 0 0 1 0 |  |  |  |  |  |  |  | 6 |
| MSB LSB SI Extension |  |  |  | MSB LSB Service Indicator Tail |  |  |  | 7 |
| MSB Extended Service Indicator LSB |  |  |  |  |  |  |  |  |
| User data |  |  |  |  |  |  |  | 8 |
| User data |  |  |  |  |  |  |  | .... |
| User data |  |  |  |  |  |  |  | n |

Figure 2/IG  Detailed MSU Format for Extension of Service Indicator

If the SI is coded '1111' then the "routing label" is extended by 2 octets. Octet 6 is currently coded '00000010' and contains the offset (from octet 6) of the actual user data, allowing a backward compatible extension of the routing label in the future.

The Extended Service Indicator is contained in octet 7 and consists of the SI Tail and the SI Extension. If the SI Extension is coded '0000' then the SI Tail contains an (old) 4 bit SI value. This allows the new MSU format to be used for current user parts. If the SI Extension is not coded '0000' then the Extended SI (XSI) represents a "new" user part. The XSI can, therefore, be regarded as a single field identifying a user part, whether new or old. Thus there is room for 240 new user parts.

MTP implementations supporting the new format must at least accept – and distribute correctly – the new MSU format (with the SI Extension containing '0000') for MSUs addressed to old user parts, including MTP management. While currently only MSUs for "new" user parts will use the new format, in the future if other "extensions" should be defined, the new format will also be used for existing user parts, including MTP management and testing.

### 2.2.4	UPU format for extended User Part identity

| H | G | F | E | D | C | B | A | Bit Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSB Sub-service field LSB |  |  |  | MSB Service Indicator LSB |  |  |  | 1 |
| DPC LSB |  |  |  |  |  |  |  | 2 |
| LSB OPC |  | MSB DPC |  |  |  |  |  | 3 |
| OPC |  |  |  |  |  |  |  | 4 |
| MSB SLS LSB |  |  |  | MSB OPC |  |  |  | 5 |
| H1 |  |  |  | H0 |  |  |  | 6 |
| Destination LSB |  |  |  |  |  |  |  | 7 |
| 0 | 0 | MSB Destination |  |  |  |  |  | 8 |
| MSB Unavailability Cause LSB |  |  |  | MSB User Part ID LSB 1 1 1 1 |  |  |  | 9 |
| MSB User Part ID Extension LSB |  |  |  | #### MSB User Part ID Tail LSB |  |  |  | 10 |
| MSB Extended User Part I D LSB |  |  |  |  |  |  |  |  |

Figure 3/IG  Format of User Part Unavailable Message for extended User Part ID

## 2.3	Reservation of extended changeover order and acknowledgement message codes

The extended changeover order and acknowledgement (XCO and XCA) message codes used in broad band MTP are reserved in narrow band MTP.

Replace Table 1/Q.704 with:

Table 1/Q.704 – Heading code allocation of signalling network management messages

| Message |  | H1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group | H0 | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
|  | 0000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHM | 0001 |  | COO | COA | 1) | 2) | CBD | CBA |  |  |  |  |  |  |  |  |  |
| ECM | 0010 |  | ECO | ECA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FCM | 0011 |  | RCT | TFC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TFM | 0100 |  | TFP | * | TFR |  | TFA | * |  |  |  |  |  |  |  |  |  |
| RSM | 0101 |  | RST | RSR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MIM | 0110 |  | LIN | LUN | LIA | LUA | LID | LFU | LLT | LRT |  |  |  |  |  |  |  |
| TRM | 0111 |  | TRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DLM | 1000 |  | DLC | CSS | CNS | CNP |  |  |  |  |  |  |  |  |  |  |  |
|  | 1001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UFC | 1010 |  | UPU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1111 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1 – Values marked 1) and 2) are reserved (codes used in Q.2210 for extended changeover order (XCO) and extended changeover acknowledgement (XCA) respectively). NOTE 2 – Values marked * should not be used (codes used in the Yellow Book for TFP and TFA acknowledgement). CBA Changeback-acknowledgement signal CBD Changeback-declaration signal CHM Changeover and changeback messages CNP Connection-not-possible signal CNS Connection-not-successful signal COA Changeover-acknowledgement signal COO Changeover-order signal CSS Connection-successful signal DLC Signalling-data-link-connection-order signal DLM Signalling-data-link-connection-order message ECA Emergency-changeover-acknowledgement signal ECM Emergency-changeover message ECO Emergency-changeover-order signal FCM Signalling-traffic-flow-control messages LFU Link forced uninhibit signal LIA Link inhibit acknowledgement signal LID Link inhibit denied signal LIN Link inhibit signal LLT Link local inhibit test signal LUA Link uninhibit acknowledgement signal LUN Link uninhibit signal LRT Link remote inhibit test signal MIM Management inhibit messages RCT Signalling-route-set-congestion-test signal RSM Signalling-route-set-test message RSR Signalling-route-set-test signal for restricted destination (national option) RST Signalling-route-set-test signal for prohibited destination TFA Transfer-allowed signal TFC Transfer-controlled signal TFM Transfer-prohibited-transfer-allowed-transfer-restricted messages TFP Transfer-prohibited signal TFR Transfer-restricted signal (national option) TRA Traffic-restart-allowed signal TRM Traffic-restart-allowed message UFC User part flow control messages UPU User part unavailable signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 3	Clarification of use of linkset emergency restart

Replace section 12.2.4.2 (changed portions are in italics) with:

#### 12.2.4.2	Link Set Emergency Restart

Link set emergency restart is applicable when an immediate re-establishment of the signalling capability of a link set is required, (i.e. in a situation when the link set normal restart procedure is not fast enough). The precise criteria for initiating link set emergency restart instead of normal restart may vary between different applications of the signalling system.

A signalling point should use emergency restart procedures when all the links in the restarting link set are unavailable (i.e. there are no other available links in the link set) and one or more of the following criteria are satisfied:

1)	The signalling point at the remote end is inaccessible (i.e. it is not possible to communicate with the signalling point at the remote end of the link set via any available route).

2)	The local signalling point is performing MTP restart.

A signalling point may use link set emergency restart procedures when all the links in the restarting link set are unavailable and/or one or more of the following criteria are satisfied provided that the signalling point is capable of determining these conditions at the time the link set restart is initiated:

1)	One or more non-adjacent signalling points normally accessible via that link set are not accessible via any other route (i.e. the restarted link set would allow communication with one or more inaccessible destination signalling points).

2)	The link set is a member of a combined link set and the other (available) link sets in the combined link set are experiencing severe local transmit link congestion.

When link set emergency restart is initiated, signalling link activation starts on as many signalling links as possible, in accordance with the principles specified for normal link set activation. In this case, the signalling terminals will have emergency status (see clause 7/Q.703) resulting in the sending of status indications of type "E" when applicable. Furthermore, the signalling terminals employ the emergency proving procedure and short time-out values in order to accelerate the procedure.

When the emergency situation ceases, a transition from emergency to normal signalling terminal status takes place resulting in the employment of the normal proving procedure and normal time-out values.

# 4	Correction of text inconsistency

Replace section 15.2 with:

## 15.2	Label

For signalling network management messages the label coincides with the routing label and indicates the destination and originating signalling points of the message; moreover, in the case of messages related to a particular signalling link, it also indicates the identity of the signalling link among those interconnecting the destination and originating points. The standard label structure of Message Transfer Part level 3 messages appears in Figure 14; the total length is 32 bits.

Figure 14/Q.704 – Standard label structure

The meaning and use of the Destination Point Code (DPC) and of the Originating Point Code (OPC) fields are described in clause 2. The Signalling Link Code (SLC) indicates the signalling link, connecting the destination and originating points, to which the message is related. Messages not related to a signalling link may be assigned any Signalling Link Code (SLC) to allow load sharing of the messages, or may be assigned a default SLC such as 0000.

# 5	Alignment of SDL with text

## 5.1	Editorial errors in Figure 29/Q.704

Figure 29 sheet 11 has a page reference error and sheet 16 has a connector reference error. These errors were also present in the report COM XI-R 191-E.

Figure 29/Q.704 (sheet 11 of 21)  Signalling traffic management: 
signalling routing control (TSRC)

Figure 29/Q.704 (sheet 16 of 21)  Signalling traffic management: 
signalling routing control (TSRC)

### 5.2	Correction of publication error

The published version of Q.704 (07/96) has an identical diagram for Figure 30/Q.704 (sheet 5 of 6) and (sheet 6 of 6), although the titles are different. The correct diagram for Figure 30/Q.704 (sheet 6 of 6) was included in the report COM XI-R 191-E, and is as follows:

Figure 30/Q.704 (sheet 6 of 6)  Signalling traffic management; 
changeover control (TCOC)

___________________

Only if such STPs were to screen MSUs on an SI basis – a possible but non-standard compliant feature – and wanted to screen for specific new SI values would a modification of the screening process be necessary. For routing purposes alone, however, no modification is required.