| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 109-E June 1998 Original: English |

Question: 2/11

E

STUDY GROUP 11 – REPORT R 109

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (05/98) FOR RECOMMENDATION Q.751.3 (07/97)

________

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.751.3 - (07/97) - Network Element Information Model for MTP Accounting

Change marks in this Implementors' Guide refer to this version of Q.751.3.

## 1.2	Background

This guide is a compilation of reported defects, their resolutions and minor upgrades to the 1997 edition of the ITU-T Recommendation Q.751.3. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following categories:

•	editorial errors;

•	technical errors, such as omissions, inconsistencies, etc.; and

•	ambiguities.

## 1.4	Contact

Rapporteur Q.2/11:	Clemens Suerbaum 	Tel.	+ 49 89 722 - 42418
	Siemens AG	Fax:	+ 49 89 722 - 24159
	OEN SN ESC 12 	Email:	clemens.suerbaum@oen.siemens.de
	Machtlfingerstr. 1
	D-81359 Munich
	Germany

## 1.5	Document history

| Version | Summary |
| --- | --- |
| 05/98 | New Implementors' Guide |

# 2	Editorial errors

## 2.1	Missing objectManagementNotificationsPackage in Table 4 and 5/Q.751.3

Synopsis: The objectManagementNotificationsPackage is defined to be part of the managed object classes mtpAccount and siGroup, but is missing in the corresponding tables in the informal section:

Mod. a): Add the following lines to Table 4/Q.751.3 (object class mtpAccount) and Table 5/Q.751.3 (object class siGroup):

| attributes | notifications | actions |
| --- | --- | --- |
| "ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage (M) |  |  |
|  | "ITU-T Rec. X.721 (1992)":attributeValueChange |  |
|  | "ITU-T Rec. X.721 (1992)":objectCreation |  |
|  | "ITU-T Rec. X.721 (1992)":objectDeletion |  |

## 2.2	Wrong object class referenced in Table 6/Q.751.3 for dpcGroup

Synopsis: In this table the specific errors for the different object classes are listed. In the lines relating to object class dpcGroup object class mtpAccount is referenced erroneously.

Mod. a): Change Table 6/Q.751.3 as follows:

| Operation | Error |  | Description |
| --- | --- | --- | --- |
|  | object class dpcGroup |  |  |
| REPLACE ADD-REMOVE pointCodeSet + CREATE | pointCodeNotExistingInSameMtp SignPointError |  | At least one of the point codes is not used by any signRouteSetNePart instance contained in the same mtpSignPoint as the dpcGroup. |
|  |  | pointCodeUsedByMtpSignPointError | At least one of the point codes is used as point code by the mtpSignPoint containing the dpcGroup. |

# 3	Technical errors

Although some of these errors have their origin in an editorial error, they were categorized as technical errors, if the formal definitions are affected.

## 3.1	Duplication of attribute controlPointer in object class mtpAccount

Synopsis: The attribute controlPointer was moved in a separate conditional package. It was missed to delete it in the mtpAccountPackage.

Mod. a): Delete the attribute controlPointer from the mtpAccountPackage. This results in the following:

mtpAccountPackage PACKAGE

BEHAVIOUR mtpAccountPackageBehaviour BEHAVIOUR DEFINED AS

".....";;

ATTRIBUTES

mtpAccountId  GET SET-BY-CREATE,

signLinkSetTpSet  GET SET-BY-CREATE ADD-REMOVE,

operatorName  GET SET-BY-CREATE,

selectionGroupSetForAccounting  GET REPLACE ADD-REMOVE;

selectionGroupSetForVerification  GET REPLACE ADD-REMOVE;

NOTIFICATIONS

mtpAccounting,

mtpAccountingVerification;

REGISTERED AS {mtpAccountPackage-POi};

## 3.2	Duplication of OBJECT IDENTIFIERS

Synopsis: Some of the OBJECT IDENTIFIERS defined in the ASN.1 module are used twice, but must be unique.

Mod. a): Change the OBJECT IDENTIFIERS as follows:

-- ASN.1 OBJECT IDENTIFIER definitions

dpcGroupId-AOi OBJECT IDENTIFIER ::= {accountingAttribute dpcGroupId(13)}

mtpAccountId-AOi OBJECT IDENTIFIER ::= {accountingAttribute mtpAccountId(14)}

_____________