| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 139-E April 1999 Original: English |

Question: 2/11

Texte disponible seulement enText available only in E

STUDY GROUP 11 – REPORT R 139

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (03/99) FOR RECOMMENDATION Q.751.1 (10/95)

________

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.751.1 - 1995 - Network Element Information Model for the Message Transfer Part (MTP).

Change marks in this document refer to this version of Q.751.1.

## 1.2	Background

This guide is a compilation of reported defects, their resolutions and minor upgrades to the 1995 edition of ITU-T Recommendation Q.751.1. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following categories:

•	editorial errors,

•	technical errors, such as omissions, inconsistencies, etc., and

•	ambiguities.

Attention: This is not an ITU publication made available to the public, but an internal ITU Document intended only for use by the Member States of the ITU and by its Sector Members and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of the ITU.

In addition, this guide records minor enhancements to the Recommendation in the following categories:

•	increased interoperability, and

•	deployment possibility in new environments.

## 1.4	Contact

| Rapporteur Q.2/11 | Clemens Suerbaum Siemens AG ICN WN CS ES C 12 Machtlfingerstr. 1 D-81359 Munich Germany | Tel: +49 89 722 42418 Fax: +49 89 722 24159 Email: clemens.suerbaum@icn.siemens.de |
| --- | --- | --- |

## 1.5	Document History

| Version | Summary |
| --- | --- |
| (05/98) | New Implementor's Guide |
| (03/99) | Addition regarding ASN.1 mode StpScreeningTableLineId (§ 3.18) |

# 2	Editorial Errors

## 2.1	initialLevelIfCongested

Synopsis

In Table 6.2.4 and also in the description there is an attribute named "initialLevelIfCongested", but in the formal description § 7.2 Package definition (multipleTransmissionCongestionStatesPackage) and also in § 7.4 Attribute definitions and the ASN.1 Module (§ 7.8) the name "initialLevelOfCongestion" is used.

Mod. a)		Use initialLevelIfCongested.

## 2.2	MTP Signalling Point: createDeleteNotificationPackage

Synopsis

In the formal description 37.1.6 the "createDeleteNotificationPackage" is defined as a conditional package. In the table in § 6.2.6 it is not marked as conditional.

Mod. a)		Mark it as conditional.

## 2.3	screeningByLinkSetOrByOpc

Synopsis

The attribute screeningByLinkSetOrByOpc is written in the object template "... ByDPCOpc". The attribute itself and its ASN.1 syntax is correct ("... ByOpc"). This way it should not be compilable.

Mod. a)		Use "screeningByLinkSetOrByOpc" (also in the description of stpScreeningTableLineId).

## 2.4	Unused action replaceSignTerm

Synopsis

Action replaceSignTerm is defined, but nowhere used in an object template or in the informal tables.

Mod. a)		Delete it.

## 2.5	Registration path

Synopsis

In the registration path of the ASN.1 Module the "(0)" after itu-t in the object identifier registration path is missing.

Mod. a)		Add the "(0)".

## 2.6	Wrong headlines and behaviour in Annex E

Synopsis

E.3.8, E.3.9, and E.10 headlines and behaviour say "obligated".

Mod. a)		"obligatory".

## 2.7	EXPORT

Synopsis

EXPORTS EVERYTHING is missing in the ASN.1 Module.

Mod. a)		Add it.

## 2.8	Wrong reference of X.722

Synopsis

The version of X.722 which is referenced by Q.751.1 does not contain the SET BY CREATE construct, which is widely used in Q.751.1. This construct is part of a later amendment to X.722.

Mod. a)		Add the reference to this addendum.

## 2.9	Wrong object mentioned in informal description of mtpAccessPoint

Synopsis

In the informal description of object class mtpAccessPoint a sentence starts with "A signalling point can have the MTP status ...". Correct is the corresponding text in the formal part: "A MTP Access Point ...".

Mod. a)		Replace "signalling point" by "MTP Access Point".

## 2.10	Missing mapping of MTP status to stati of mtpAccessPoint

Synopsis

The state mapping for the object class mtpAccessPoint is missing.

Mod. a)		Add the following:

"A mtpAccessPoint can have the MTP status

•	allowed, the mtp3bAccessPoint is accessible

•	congested, this means that the mtp3bAccessPoint is accessible but the path to it is heavily loaded

•	prohibited, this means that the mtp3bAccessPoint is not accessible.

The MTP status is mapped to the operationalState and availabilityStatus attributes as follows:

| MTP Status | operationalState | availabilityStatus |
| --- | --- | --- |
| allowed | enabled | { } |
| congested | enabled | {degraded} |
| prohibited | disabled | {off line} |

## 2.11	Capitalization in components of StpScreeningTableLineId

Synopsis

The CHOICE components of StpScreeningTableLineId should be written "designatedLinkSet" and "designatedOpc". Currently the changes from lower- to upper-case letters are missing.

There is also a typo in designatedOpc (doubled d).

Mod. a)		Change the typing.

## 2.12	Missing table for lsTimersProfile

Synopsis

In the version of Q.751.1 presented in SG 11 R 131, Table 6.2.1 is missing.

Mod. a)	 	Restore the table (see PL/11-24, Geneva, May 1995).

## 2.13	Wrong entry in table for managedSwitchingElement

Synopsis

In Table 6.2.2 a notification "processingAlarm" is listed. The correct name for this notification is "processingErrorAlarm".

Mod. a)	 	Correct the entry.

## 2.14	signLinkTpPointer superfluous

Synopsis

In section 7.4 Attribute Definitions an attribute template for signLinkTpPointer is listed which is not used neither by a package nor by an object class.

Mod. a) 	Delete the attribute template.

## 2.15	Missing mark for conditional remoteExchangeLabelPackage

Synopsis

In the table in Q.751.1 section 6.2.13 (signRouteSetNePart) the remoteExchangeLabelPackage is not marked as conditional as it is in the formal definitions.

Mod. a) 	Add the corresponding indication "(C)" after the entry in the table.

## 2.16	Attribute slUnavailable in Annex E

Synopsis

This attribute is not used anywhere in Q.751.1. Obviously this attribute is a result of a copy-paste-error (the attribute slsUnavailable has identical text).

Mod. a) 	Remove this attribute.

## 2.17	Behaviour of attribute discardedMsus

Synopsis

In the behaviour of the attribute only Measurement 3.10/Q.752 is mentioned, but it is used also for Measurement 5.5 (see also table in section E.1/Q.751.1, page 156).

Mod. a) 	Extend the behaviour as follows:

discardedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR discardedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurements Q.752/3.10 and Q.752/5.5";;

REGISTERED AS { mtpAttribute 138 };

## 2.18	stpScreeningTableLine

Synopsis

The conditional stpScreeningTableLineNamePackage (listed in the table of § 6.2.16/Q.751.1) is missing in the formal definitions of the object class; also the definition of the package itself is missing.

Mod. a) 	Change the formal definition of the stpScreeningTableLine as follows and add the stpScreeningTableLineNamePackage definition:

stpScreeningTableLine MANAGED OBJECT CLASS
	...

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

stpScreeningTableLineNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 16 };

stpScreeningTableLineNamePackage PACKAGE
	ATTRIBUTES
		stpScreeningTableLineName GET-REPLACE;
REGISTERED AS { mtpPackage 53 };

## 2.19	all5And30MinSignallingLinkDurationData

Synopsis

In the behaviour of all5And30MinSignallingLinkDurationData (Annex E.3.2) it reads: "... contains all obligated signalling link duration data. The preferred granularityPeriod is 5 and 30 Minutes." But the listed Measurements are not obligatory.

Mod. a) 	Replace the cited text by: "... contains all signalling link duration data with preferred granularityPeriod 5 and 30 Minutes".

## 2.20	Figure 2/Q.751.1

Synopsis

In Figure 2 in the MOC signLinkTp there is no line for separating attributes and notifications.

Mod. a) 	Add the line.

## 2.21	Figure 3/Q.751.1

Synopsis

Two object classes have the same name "mtpRouteVerifTest" in this figure.

Mod. a) 	Replace the name of the right one of both object classes by "mtpAccessPoint".

## 2.22	Too long sentence

Synopsis

The description in § 6.2.12 "In the signRouteNePart instances contained in the same signRouteSetNePart information is in existence via which signLinkSetTp instances - as possible first segment of the route in network view - messages are intended to be routed and actually can be routed to the respective destination signalling point." is to long and not very much understandable.

Mod. a) 	Change text to:
SignRouteNePart instances contained in the same signRouteSetNePart represent routing information for MSUs towards the DPC of the signRouteSetNePart. They reference the signLinkSetTp instances via which messages to this DOCs are intended to be routed. The information whether they actually can be routed to that DPC via these linksets - as possible first segment of the route in network view - is represented in the attributes operationalState and availabilityStatus of the signRouteNePart."

## 2.23	Unnecessarily complicated wording

Synopsis

In the description of § 6.2.13 "This information comprises among others ... (the signalling point of the network element B), its accessibility from the own mtpSignPoint of network element A via ..." to many "network elements" make the sentence quite unreadable.

Mod: a) 	Change it to "This information among others comprises ... (the signalling point of B), its accessibility from the own mtpSignPoint of A via ...".

## 2.24	Mixed-up CongestionLevels

Synopsis

In § 6.2.4, in the descriptions of attributes 19), 21) and 24) there are wrong congestionLevels mentioned. Correct would be:
attribute 19): level 1
attribute 21): level 2
attribute 24): level 3

Mod. a) 	Change the levels to the correct values.

## 2.25	Typo

Synopsis

In § 6.2.6 in footnote 1) it reads "SSCP":

Mod. a) 	Change it to "SCCP".

## 2.26	Missing part of a word

Synopsis

In § 6.2.6 it reads "In case the spTimersProfilePackage is used, a create or set request ...", in § 6.2.8 "In case the lsTimersProfilePackage is used, a create or set request ..." but the … ProfilePointerPackages ... should be referenced.
The same errors exist in the name binding definitions of the object classes mtpSignPoint and signLinkSetTp.

Mod. a) 	Replace "... ProfilePackage" by "... ProfilePointerPackage".

## 2.27	Typos reference wrong Recommendation

Synopsis

In the comments to attributes defined in § E.3.3 and E.3.9 Q.751 is referenced.

Mod. a) 	Replace "Q.751" by "Q.752".

## 2.28	Wrong entry in table of § 3.1

Synopsis

For the measurement 5.1 the column "In Managed Object Class" lists object class signallingPointData.

Mod. a) 	Replace it by "allSignallingPointUtilizationData".

# 3	Technical Errors

## 3.1	maxCapacitySL

Synopsis

Operation SET BY CREATE is missing.

Mod. a)		Add the operation.

## 3.2	linkCongestionLevel

Synopsis

Operation SET BY CREATE is wrong.

Mod. a)		Delete it.

## 3.3	inLsLoadShareAlgorithm

Synopsis

Operation SET BY CREATE is missing.

Mod. a)		Add the operation.

## 3.4	signDataLinkTpList/signTermList

Synopsis

Operations SET BY CREATE and ADD-REMOVE are missing.

Mod. a)		Add the operations.

## 3.5	Q704-t23

Synopsis

ASN.1 type TimerValue has a wrong upper limit 60 000. It should have the upper limit 360 000 (the 3 is missing). Q704-t23 range is from 3 to 6 minutes (=360 000 ms) and uses this ASN.1 type.

Mod. a)		Change upper limit to 360 000.

## 3.6	Modelling of Q.752 measurements 3.6, 3.9, 4.5, 4.6, 5.1, 5.4

Synopsis

Modelling results in alarms which are never cleared or in trying to clear an alarm which was never set (for a detailed description see contribution M8 of the interim meeting in Munich, 9-13 October 1995).

Mod. a)	 	Add statements at the appropriate places saying that the modelling of these measurements is for further study (remark: 3.9 was cancelled in draft new Q.752).

## 3.7	Modelling of Q.752 measurements 4.11/4.12 and 5.1

Synopsis

If the measurements 4.11 or 4.12 and measurement 5.1 would be active, there would be two notifications for the same event (adjacent point inaccessible).

Mod. a) 	Add statements to the corresponding behaviours of the notification representing 4.11 and 4.12 and to their MOCS, stating that these notifications should not be sent out, if the notification for 5.1 is supported.

## 3.8	Name binding for managedSwitchingElement

Synopsis

A name binding managedElement-managedSwitchingElement is missing. In the MRCS it is to be listed as optional.

Mod. a) 	Add the name binding.

## 3.9	Delete rejection reasons for signDataLinkTp

Synopsis

The following rejection reason is missing:
"If an attempt is made to delete a signDataLinkTp instance which is still referenced by a signLinkTp, the delete request is rejected."
This rejection reason is needed to ensure data consistency.

Mod. a)		Add this text to the name binding behaviour of signDataLinkTp.

## 3.10	Create rejection reason for signLinkTp

Synopsis

The following create rejection reason for signLinkTp is missing:
"If an attempt is made to create a signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod different to "unknown" or "ccmQ704International" and the signLinkTp instance would not support the linkCongestionLevelPackage, then the create request is rejected."
This rejection reason is needed to ensure support of the corresponding congestion control functions by the data.

Mod. a)		Add this text to the name binding behaviour and behaviour of object class signLinkTp.

## 3.11	Create and set rejection reasons for signLinkTp

Synopsis

The following rejection reasons are missing:

"A signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod "ccmQ704NationalWithPrio" must reference a mtpL2ProtocolProfile instance which is supporting either the multipleTransmissionCongestionStatesPackage or the multipleTransmissionCongestionLevelsPackage.

A signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod "ccmQ704NationalNoPrioBufferMechanism" must reference a mtpL2ProtocolProfile instance which is supporting the multipleTransmissionCongestionLevelsPackage.

A signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod "ccmQ704NationalNoPrioTimerMechanism" must reference a mtpL2ProtocolProfile instance which is supporting the multipleTransmissionCongestionStatesPackage.

If an attempt is made to reference an unsuitable mtpL2ProtocolProfile instance, then the corresponding request is rejected."

This rejection reason is needed to ensure support of the corresponding congestion control functions by the data.

Mod. a)		Add this text for create requests to the name binding behaviour and for set requests (concerning the mtpL2ProtocolProfilePointer of a signLinkTp instance) to the object class behaviour of signLinkTp.

## 3.12	Create rejection reasons

Synopsis

A common create rejection for system dependent rejection is missing.

Mod. a)	 	Add a general statement saying:
"Creation of new object class instances may be rejected due to lack of system resources, e.g. the system specific maximal number of instances of this object class per superior object instance have been exceeded."

## 3.13	oblSpUtilization and allSignallingPointUtilizationData

Synopsis

In Figure E-2/Q.751.1 the object classes oblSignalingPointUtilizationData and allSignallingPointUtilizationData are contained under mtpSignPoint (For oblSignalingPointUtilizationData there is also a corresponding name binding in the formal definitions).
This is an error, because the related measurements Q.752/5.2 resp. 5.1 refer to an adjacent signalling point, which is represented by a signRouteSetNePart, and have to be measured per routeset.

Mod. a)		Change Figure E-2/Q.751.1 accordingly and change/add the formal definition of these object classes as follows:

signRouteSetNePart -oblSignallingPointUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS oblSignallingPointUtilisationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS signRouteSetNePart;
	WITH ATTRIBUTE 		"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 19};

signRouteSetNePart-allSignallingPointUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS allSignallingPointUtilisationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS signRouteSetNePart;
	WITH ATTRIBUTE 		"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 40 };

## 3.14	name binding of ss7FirstAndIntervalThresholdData

Synopsis

The object class ss7FirstAndIntervalThresholdData is a kind of template with fixed threshold value 1 to model first and interval measurements of Q.752 according to Q.822. It therefore makes no sense to have individual templates for each mtpSignPoint with identical values as it is introduced by Figure E-2/Q.751.1. The formal definition for the name binding of this object class is missing in Q.751.1.

Mod. a)		Make the object class ss7FirstAndIntervalThresholdData contained in managedSwitchingElement, change Figure E-2/Q.751.1 accordingly and add the following formal definition to E.6/Q.751.1:

managedElement-ss7FirstAndIntervalThresholdData NAME BINDING
	SUBORDINATE OBJECT CLASS ss7FirstAndIntervalThresholdData;
	NAMED BY
	SUPERIOR OBJECT CLASS managedElement AND SUBCLASSES;
	WITH ATTRIBUTE "ITU-T Rec. Q.822 (1994)": thresholdDataId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 41};

## 3.15	ResultMtpRouteTest and MtpRouteVerifTestResultSntx

Synopsis

ResultMtpRouteTest is the ASN.1 definition for the reply syntax for the ACTION to start an MRVT. 
The definition of ResultMtpRouteTest contains the following elements which are not useful in the reply syntax: announcementOfMRVR -- this is a parameter which makes only sense in an MRVA 
-- the following data only would make sense, if the test by definition can only deliver one route (but this is not the case):
routeTrace
pointCodeInfo
pointCodeListInfo
routePriorityListInfo
copyData
All these parameters are optional, but it makes no sense to have even optional useless parameters in a data structure

Mod. a)		Change ResultMtpRouteTest as follows:

ResultMtpRouteTest ::= SEQUENCE {

testedDestination		PointCode,

routeTestActionResult	MRVTActionResult,

failureType				[1] 	MRVTFailureString OPTIONAL,

...}

Synopsis

Some of the superfluous parameters mentioned above are necessary in MtpRouteVerifTestResultSntx, which is the ASN.1 definition for an MRVR content to be notified. There are missing:
pointCodeInfo
pointCodeListInfo
routePriorityListInfo

Mod. a)		Extend the MtpRouteVerifTestResultSntx as follows:

MtpRouteVerifTestResultSntx ::= SEQUENCE {

testedDestination	 	PointCode,		-- 2.2.2.3/Q.753 (b)
	routeTestResult		ResultOfMRVT,		-- 2.2.2.3/Q.753 (c)
	failureType			[1] MRVTFailureString OPTIONAL, -- 2.1.1.3.1/Q.754 
	routeTrace			[2] RouteTrace OPTIONAL, 		    -- 2.2.2.3/Q.753 (d)
	pointCodeInfo		[3] PointCode OPTIONAL,
	pointCodeListInfo	[4] PointCodeList OPTIONAL,
	routePriorityListInfo	[5] RoutePriorityListInfo OPTIONAL,
	copyData			[6] OCTET STRING OPTIONAL                -- 2.2.2.3/Q.753 (e)
	}

## 3.16	Screening Option Outgoing Linkset/DPC deleted

Synopsis

It was agreed (see "Implementors" Guide for Q.705' approved by SG 11) to delete Q.705 § 8.2 ii), which contains the option to screen the combination of outgoing linkset and destination point code. This deletion needs to be reflected in Q.751.1.

Mod. a)		Delete the value "byOutgoingLinkset" in the Type ScreeningByLinkSetOrByOpc.

ScreeningByLinkSetOrByOpc ::= ENUMERATED {

byOpc	(0),

byIncomingLinkset	(1)

}

Mod. b)		Delete the reference to this option in § 6.2.15.

## 3.17	Wrong modes for localMgnt/Un-/Inhibit

Synopsis

In MOC all5and30MinSignallingLinkDurationData the units of the two attributes:localMgntInhibit and localMgntUninhibit in Annex E/Q.751 are "seconds", while in Tables 2.13, 2.14/Q.752 the units is EVENT/SL.

Mod. a) 	Change the modes of these attributes in Annex E to "Counter".

## 3.18	Wrong CHOICE sub-mode for stpScreeningTableLineId

Synopsis

The mode Objectinstance for the CHOICE designatedLinkSet would allow to reference a deliberately chosen object instance. This allows several error possibilities, e.g. by referencing an instance of a wrong object class (all but signLinkSetTp) or a signLinksetTp which is not contained in the same superior mtpSignPoint instance. Therefore this mode would force unnecessary plausibility checks to avoid these errors. This is avoided by using the mode SignLinkSetTpId (=NameType). This way the designatedLinkSet directly and unambiguously references the SignLinkSetTp instance contained in the same mtpSignPoint as the superior stpScreeningTable of the stpScreeningTableLine for which screening is done.

Mod. a) 	Change the mode of designatedLinkSet to SignLinkSetTpId

StpScreeningTableLineId ::= CHOICE {

designatedLinkSet	[0]	SignLinkSetTpId,

designatedOPC	[1]	PointCode}

Mod. b)		Add the definition for SignLinkSetTpId

SignLinkSetTpId ::= NameType

# 4	Ambiguities

## 4.1	Attributes of sort objectClassName

Synopsis

For all such attributes (example for object class signLinkSetTp: signLinkSetTpName) a description is given, that a create request is rejected if the name is already used by another instance of the same class. This description is missing for set requests to the name attributes.

Mod. a)		Add this description to the attribute behaviours of the object classes.

# 5	Increased Interoperability

## 5.1	Changes in signLinkTp to allow inheritance of it in Q.2751

The changes do not effect a change in the protocol via the Q3 interface.

### 5.1.1	Change name of protocolProfilePointer of signLinkTp

Problem

Currently there is an attribute mtpL2ProtocolProfilePointer in the object class signLinkTp. This attribute points for narrow-band links to a mtpL2ProtocolProfile. If a broadband link shall reuse (inherit) definitions of signLinkTp, via this attribute a profile for SAAL NNI parameter should be referenced. Therefore the label of the attribute should be independent of the bandwidth, e.g. protocolProfilePointer.

Mod. a)		Change the label "mtpL2ProtocolProfilePointer" and text references to it at all (to be exact: 5) places in Q.751.1 (two times in the informal part for signLinkTp, one appearance in the attribute list of the signLinkTp object template, and two times in the attribute definition) to "protocolProfilePointer".

No ASN.1 description is affected because the mode of the attribute remains unchanged (pointer).

### 5.1.2	Name binding of signLinkTp subclasses

Problem

Currently the signLinkSetTp-signLinkTp name binding refers to "signLinkTp AND SUBCLASSES". This is a remainder of a first draft, when there were subclasses of signLinkTp defined in Q.751.1, but now there are not any. Now for Q.2751.1 it is useful to really have a subclass of signLinkTp. For this subclass a different name binding is defined, because in the name binding behaviour additional create rejection reasons have to be described.

Mod. a)		In order to avoid ambiguity, which name binding is applicable, it is proposed to delete the unnecessary "AND SUBCLASSES" in Q.751.1.

# 6	Deployment Possibility in New Environments

## 6.1	Flexibility for pointCode

Problem

Currently the ASN.1 value for maxSpc is adjusted for 14-bit point codes and therefore equals 16383 (=214-1). This restricts Q.751.1 to be used for administration of signalling networks with 14-bit point codes.

Mod. a)		To allow the use of Q.751.1 without re-registration in networks using other point code bit lengths the mode of the pointCode should be changed to INTEGER or the range should be extended to a bigger value.

Annex A

(to Implementors' Guide for Q.751.1)

Corrected formal specifications of § 7 and Annexes E and F/Q.751.1

This annex collects all changes contained in the Implementors' Guide which affect the formal definitions of § 7/Q.751.1, of Annex E/Q.751.1 and of Annex F/Q.751.1. The changes are marked by double underlining (new), strike-outs () and change bars (any change).

References within the formal definitions refer to Q.751.1 itself.

## 7.1	Managed object classes definitions

### 7.1.1	Signalling Link Set Timers Profile

lsTimersProfile MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY lsTimersProfilePackage PACKAGE

BEHAVIOUR lsTimersProfileBehaviour BEHAVIOUR DEFINED AS

"A signalling link set timer profile is a set of Q.704 timers which are related to the characteristics of signalling links sets.

The managed object class lsTimersProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signLinkSetTp one of the lsTimersProfiles is referenced.";;

ATTRIBUTES

lsTimersProfileId	GET SET BY CREATE,

q704-t6		GET SET BY CREATE,

q704-t8		GET SET BY CREATE,

q704-t10		GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

lsTimersProfileNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 1 };

### 7.1.2	Managed Switching Element

managedSwitchingElement MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. M.3100 (1992)":managedElement;

CHARACTERIZED BY	managedSwitchingElementPackage PACKAGE

BEHAVIOUR	managedSwitchingElementBehaviour BEHAVIOUR DEFINED AS

"The Managed Switching Element represents, for management purposes, an exchange, i.e. the aggregate of traffic carrying devices, switching stages, controlling and signalling means at a network node that enables subscriber lines to be interconnected and packets to be forwarded as required by individual users.";;

ATTRIBUTES

"ITU-T Rec. M.3100 (1992)": vendorName	GET SET BY CREATE;;;

CONDITIONAL PACKAGES

managedSwitchingElementNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 2 };

### 7.1.3	MTP Access Point

mtpAccessPoint MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.283 (1993) | ISO/IEC 10733 : 1992":nSAP;

CHARACTERIZED BY mtpAccessPointPkg PACKAGE

BEHAVIOUR mtpAccessPointBehaviour BEHAVIOUR DEFINED AS

"The sap2Address attribute contains the address of the mtpAccessPoint. It contains the Signalling Point Code (SPC), Service Indicator (SI) and MTP Network Identity (NI). The sapId attribute is used in naming instances of the mtpAccessPoint managed object class.

The userEntityNames attribute contains the distinguished names of the managed objects that represent the user entities that are using the mtpAccessPoint, e.g. the SCCP instance. The providerEntityNames attribute contains the distinguished names of the managed objects that represent the provider entities that are supporting the mtpAccessPoint, i.e., the instance of the MTP.

A mtpAccessPoint can have the MTP status:

–	allowed, this means that the mtpAccessPoint is accessible and is functioning normally

–	congested, this means that the mtpAccessPoint is accessible but the path to it is heavily loaded

–	prohibited, this means that the mtpAccessPoint not accessible.

If the ss7OnOccEventPackage is present, event notification with the following probable causes can apply: Probable Cause = 001 (localUserPartUnavailable)";;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and ss7OnOccEventPackage is present",

mtpAccessPointNamePackage PRESENT IF "an instance supports it",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 3 };

### 7.1.4	MTP Level 2 Protocol Profile

mtpL2ProtocolProfile MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	mtpL2ProtocolProfilePkg PACKAGE

BEHAVIOUR	mtpL2ProtocolProfileBehaviour BEHAVIOUR DEFINED AS

"An MTP level 2 protocol profile is a set of Q.703 parameters which defines the layer 2 characteristics of Signalling Links.

The managed object class mtpL2ProtocolProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signLinkTp one of the mtpL2ProtocolProfiles is referenced.";;

ATTRIBUTES

mtpL2ProtocolProfileId	GET SET BY CREATE,

errorCorrectionMethod	GET SET BY CREATE,

transmissionRate		GET SET BY CREATE,

l2TimerT1		GET SET BY CREATE,

l2TimerT2		GET SET BY CREATE,

l2TimerT3		GET SET BY CREATE,

l2TimerT4N		GET SET BY CREATE,

l2TimerT4E		GET SET BY CREATE,

l2TimerT5		GET SET BY CREATE,

l2TimerT6		GET SET BY CREATE,

l2TimerT7		GET SET BY CREATE,

transmissionCongestionThresholdOnsetLevel1	GET SET BY CREATE,

transmissionCongestionThresholdAbatementLevel1	GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

bufferMechanismPackage PRESENT IF "an instance supports it",

congestionHandlingPackage PRESENT IF "an instance supports it",

loopDelayPackage PRESENT IF "an instance supports it",

mtpL2ProtocolProfileNamePackage PRESENT IF "an instance supports it",

multipleTransmissionCongestionLevelsPackage PRESENT IF "an instance supports it",

multipleTransmissionCongestionStatesPackage PRESENT IF "an instance supports it",

pCRPackage PRESENT IF "an instance supports it",

receiveCongestionLevelsPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 4 };

### 7.1.5	MTP Level 3

mtpLevel3 MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	mtpLevel3Package PACKAGE

BEHAVIOUR 	mtpLevel3Behaviour BEHAVIOUR DEFINED AS

"This managed object class models MTP Level 3 functionality.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992':communicationsAlarm notification is used, the following probable causes can apply:

Probable Cause = 501 (AdjacentPCInaccessible), with the inaccessible adjacent pointcode as a parameter, which represents measurement Q.752/5.1 (PerceivedSeverity=Maj/Min/War) and measurement Q.752/5.4 (PerceivedSeverity=Cleared).

-- Modelling of Measurements Q.752/5.1 and 5.4 is for further study

If the ss7OnOccEventPackage is included, event notifications with the following probable causes can apply:

Probable Cause = 405 (LinkSetFailure), with the inaccessible adjacent pointcode as a parameter, which represents measurement Q.752/4.5 (PerceivedSeverity=Maj/Min/War) and measurement Q.752/4.6 (PerceivedSeverity=Cleared).

-- Modelling of Measurements Q.752/4.5 and 4.6 is for further study

Probable Cause = 413 (LinkSetChange), with the involved link sets as parameter, which represents measurement Q.752/4.13.

";;

ATTRIBUTES

mtpLevel3Id		GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usageState		GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and the (communicationsAlarmPackage or ss7OnOccEventPackage) is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

mtpLevel3NamePackage PRESENT IF "an instance supports it",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 5 };

### 7.1.6	MTP Signalling Point

mtpSignPoint MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	mtpSignPointPackage PACKAGE

BEHAVIOUR 	mtpSignPointBehaviour BEHAVIOUR DEFINED AS

"A signalling point is a node in the signalling network. It is part of a managed switching element (exchange). The MTP permits three types of node:

–	SEP: the signalling end point, this type of SP contains MTP and ISUP (and or other MTP Users), but cannot act as an intermediate MTP transfer node in the SS No.7 network. (It can act as an SCCP Relay Node.)

–	STP: the MTP signalling transfer point, this type of SP acts as an intermediate node between SEPs to transfer messages through the SS No.7-network, it only contains MTP. (In case of an STP, OMAP, and hence also TC and SCCP may be present.)

–	STEP: MTP signalling transfer and end point, the type combines the previous mentioned types, thus acting both as a SEP and as a STP.

A mtpSignPoint can have the MTP status:

–	allowed, this means that the mtpSignPoint is accessible and is functioning normally (operational state = enabled, availability status = Æ)

–	congested, this means that the mtpSignPoint is accessible but is heavily loaded (operational state = enabled, availability status = { degraded })

–	prohibited, this means that the mtpSignPoint not accessible (operational state = disabled).

During MTP restart, the proceduralStatus is {initializing} until the restart is finished.";;

ATTRIBUTES

mtpSignPointId		GET SET BY CREATE,

pointCode		GET SET BY CREATE,

spType		GET SET BY CREATE,

-- In some implementations the spType attribute might additionally be writable.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET,

"ITU-T Rec. M.3100 (1992)":version	GET SET BY CREATE,

networkIndicator		GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange,

mtpRestarting;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

mtpSignPointNamePackage PRESENT IF "an instance supports it",

spTimersPackage PRESENT IF "the instance does not support the spTimersProfilePointerPackage",

spTimersProfilePointerPackage PRESENT IF "the instance does not support the spTimersPackage";

REGISTERED AS { mtpObjectClass 6 };

### 7.1.7	Signalling Data Link Termination Point

signDataLinkTp MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY signDataLinkTpPackage PACKAGE

BEHAVIOUR signDataLinkTpBehaviour BEHAVIOUR DEFINED AS

"This managed object represents the termination of the signalling data link within the SP as defined in Q.702.

The operational state is 'enabled' in normal operation. In case of failure of the part of the signalling data link that is within control of the managed switching element, this state will be 'disabled'.";;

-- Note that for a complete view of the status of a signalling data link a network view is required.

ATTRIBUTES

signDataLinkTpId	GET SET BY CREATE,

adjPc	GET SET BY CREATE,

transmissionRate	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

equipmentPointer 	GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

clCPackage PRESENT IF "an instance supports it",

signDataLinkTpNamePackage PRESENT IF "an instance supports it",

stmChannelPackage PRESENT IF "an instance supports it or the signalling point supports the automatic allocation of signalling datalinks";

REGISTERED AS { mtpObjectClass 7 };

### 7.1.8	Signalling Link Set Termination Point

-- An example of the OSI state mapping for this class can be found in Annex C.

signLinkSetTp MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY signLinkSetTpPackage PACKAGE

BEHAVIOUR signLinkSetTpBehaviour BEHAVIOUR DEFINED AS

"This managed object class represents the SS No. 7 signalling link set termination point. Link sets are defined in Recommendation Q.704.

The states are directly related to the set of link termination point's states, except for the optional administrativeState.

General principles on state information:

–	The states of the managed object linkset are only readable, except for the optional administrativeState.

–	Activation and deactivation of a linkset may be done via operations on the links (e.g. by using scoping functions) or the optional administrativeState can be used for this.

Based on these principles the states of a linkset are defined as follows:

–	operational state (read-only for the OS). The operational state is enabled whenever one link of the linkset is enabled, i.e. neither FAILED nor DEACTIVATED nor LOCAL BLOCKED nor REMOTE BLOCKED.

–	usage state (read-only for the OS). The usage state reflects the usage of the belonging links. It is ACTIVE if at least one of the belonging links is ACTIVE and no link is BUSY. It contains BUSY if one of the belonging links is BUSY. It is IDLE if all links are UNAVAILABLE.

–	availability status (read-only for the OS). The availability status indicates unavailability or reduced availability of the belonging links. It contains DEPENDENCY if all the belonging links are unavailable for user traffic. It contains DEGRADED if at least one belonging link is not available for user traffic.

For the different reasons of unavailability (failed, deactivated, local blocked, remote blocked, local inhibited and remote inhibited) see 3.2.1/Q.704.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992':communicationsAlarm notification is used, the following probable causes can apply:

Probable Cause=403 (LinkSetFailure), which represents measurements Q.752/4.3

(PerceivedSeverity=Maj/Min/War) and Q.752/4.4 (PerceivedSeverity=Cleared).

State change notifications are emitted for all state and status changes. ";;

ATTRIBUTES

signLinkSetTpId	GET SET BY CREATE,

adjPc		GET SET BY CREATE,

maxCapacityLS	GET SET BY CREATE,

-- In some implementations, the maxCapacityLS attribute might be additionally writable.

currentCapacityLS	GET,

congestionControlMethod	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usage State	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET,

periodicLinkTestFlag	DEFAULT VALUE MTPDefinedTypesModule.

periodicTestFlagDefault 	GET-REPLACE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

administrativeStatePackage PRESENT IF "an instance supports it",

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an 
instance supports it and the communicationsAlarmPackage is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

inLsLoadShareAlgorithmPackage PRESENT IF "an instance supports it",

lsTimersProfilePointerPackage PRESENT IF "the instance of superior class mtpSignPoint does not support the conditional spTimersPackage",

noBasicLinkAllocationPackage PRESENT IF "Basic link allocation is not done",

signLinkSetTpNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 8 };

### 7.1.9	Signalling Link Termination Point

-- An example of mapping of the MTP functional states to the incorporated OSI management states

-- can be found in Annex C.

signLinkTp MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY signLinkTpPackage PACKAGE

BEHAVIOUR signLinkTpBehaviour BEHAVIOUR DEFINED AS

"This managed object represents the termination of the signalling link within the signalling point as defined in Recommendation Q.703. The signLinkTp object class represents that part of a signalling link which is located in one NE.

The following state and status attributes are supported:

administrative state:

–	locked: it is administratively not permitted to transport traffic on the link termination point.

–	unlocked: it is administratively permitted to transport user part traffic or test traffic on the link termination point.

operational state:

–	enabled: the link termination point is operationally able to transport user part traffic or test traffic.

–	disabled: the link termination point is operationally unable to transport user part traffic or test traffic.

The operational state is disabled whenever the link termination point status contains Failed, Deactivated, Local Blocked, Remote Blocked, or the procedural status is Initializing.

usage state:

–	idle: the link termination point carries no user part traffic (test traffic may be present).

–	'active': the link termination point is not congested. Currently the link termination point carries user part traffic.

–	busy: the link termination point is congested due to user part traffic.

The usage state is idle whenever the link termination point status is not empty i.e. the usage state immediately reflects the availability of the signLinkTp for user traffic. For national MTP options applying several congestion levels an additional attribute indicating these levels may be added.

procedural status:

–	The procedural status initializing is valid during initial alignment procedure.

–	The procedural status not initalized applies when the link termination point is not aligned and the initial alignment procedure has not been started.

Link termination point status. This is a set-valued attribute. The following statusses can exist alone or in combinations with each other. The exact definition for each of the following SS7-functional states is to be gained from Q.704

–	local blocked

–	remote blocked

–	local inhibited

–	remote inhibited

–	failed

–	deactivated.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992':communicationsAlarm notification is used, the following probable causes can apply:

Probable Cause = 102 (slFailure) which represents measurement Q.752/1.2 – measurement Q.752/1.6 (SpecificProblems=003 abnormalFIBRorBSNR, 004 excessiveAckDelay, 005 excessiveErrorRate, 006 excessiveCongDuration) and Q.752/1.12.(PerceivedSeverity=cleared)

If the ss7OnOccEventPackage is present, the event notifications with the following probable causes can apply:

Probable Cause = 110 (localChangeOver) which represents measurement Q.752/1.10

(PerceivedSeverity=Maj/Min/War) and measurement Q.752/1.11 (PerceivedSeverity=Cleared).

Probable Cause = 210 (remoteProcOutage) which represents measurements Q.752/2.10

(PerceivedSeverity=Maj/Min/War) and Q.752/2.11 (PerceivedSeverity=Cleared).

Probable Cause = 216 (start of local inhibition) which represents measurement Q.752/2.16

(PerceivedSeverity=Maj/Min/War) and Q.752/2.17 (PerceivedSeverity=Cleared).

Probable Cause = 218 (start of remote inhibition) which represents measurement Q.752/2.18

(PerceivedSeverity=Maj/Min/War) and Q.752/2.19 (PerceivedSeverity=Cleared).

A create or set request is rejected, if

i)	a pointer would reference an instance, which does not belong to the appropriate object class

OR	ii)	a pointer would reference an instance which is not existing

OR	iii)	a signDataLinkTpPointer would reference a signDataLinkTp which is not in the same mtpSignPoint

OR	iv)	a signDataLinkTpPointer would reference a signDataLinkTp which has a different value of adjPc than the superior signLinkSetTp

OR	v)	the link would reference a datalink and a mtpL2ProtocolProfile whose transmissionRates is not the same

OR	vi)	the link would reference a mtpL2ProtocolProfile where the appropriate packages for the congestionControlMethod of the superior signLinkSetTp are not present (see B.1.2.9.2)

OR	vii)	the spTimersProfilePackage is present in the superior mtpSignPoint AND the link would reference a mtpL2ProtocolProfile whose l2TimerT2 is not bigger than the q704-t17 of the spTimersProfile referenced by the mtpSignPoint

OR	viii)	the spTimersPackage is present in the superior mtpSignPoint AND the link would reference a mtpL2ProtocolProfile whose l2TimerT2 is not bigger than the q704-t17 of the mtpSignPoint.

The rejection also takes place if only a single pointer within the signDataLinkList or the signTermList (if present) is wrong.

A delete request is rejected, if the linkTpStatus does not contain the value deactivated.

A create request or a set request for the attribute protocolProfilePointer is rejected, if

i)	a signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod ‘ccmQ704NationalWithPrio’ would not reference a mtpL2ProtocolProfile instance which is supporting either the multipleTransmissionCongestionStatesPackage or the multipleTransmissionCongestionLevelsPackage

OR 	ii)	a signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod ‘ccmQ704NationalNoPrioBufferMechanism’ would not reference a mtpL2ProtocolProfile instance which is supporting the multipleTransmissionCongestionLevelsPackage

OR 	iii)	a signLinkTp instance whose superior signLinkSetTp has a value of its attribute congestionControlMethod ‘ccmQ704NationalNoPrioTimerMechanism’ would not reference a mtpL2ProtocolProfile instance which is supporting the multipleTransmissionCongestionStatesPackage.";;

ATTRIBUTES

slCode		GET SET BY CREATE,

slsCodeCurrentList	GET,

maxCapacitySL	GET SET BY CREATE,

currentCapacitySL	GET,

linkTpStatus		GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usageState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET,

protocolProfilePointer	GET-REPLACE,

signTermPointer	GET SET BY CREATE,

signDataLinkTpPointer	GET-REPLACE;

ACTIONS

localInhibit

localUninhibit

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and the (communicationsAlarmPackage or ss7OnOccEventPackage) is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

linkCongestionLevelPackage PRESENT IF "the exchange support the operation, management and administration of multiple transmission congestion levels (national option)",

relatedLinkGroupNumberPackage PRESENT IF "an instance supports it",

signDataLinkTpListPackage PRESENT IF "an instance supports it",

signLinkTestPackage PRESENT IF "the on-demand signalling link test as described in Q.707 is supported",

signLinkTpNamePackage PRESENT IF "an instance supports it",

signTermListPackage PRESENT IF "an instance supports it or an instance supports the signDataLinkTpListPackage",

slsCodeNormalListPackage PRESENT IF "an instance supports it",

slTimersProfilePointerPackage PRESENT IF "the instance of superior class mtpSignPoint does not support the conditional spTimersPackage",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS [ mtpObjectClass 9 };

### 7.1.10	Signalling Link Timers Profile

slTimersProfile MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

slTimersProfilePackage PACKAGE

BEHAVIOUR slTimersProfileBehaviour BEHAVIOUR DEFINED AS

"A signalling link timer profile is a set of Q.704 timers which are related to the characteristics of Signalling Links.

The managed object class slTimersProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signLinkTp one of the slTimersProfiles may be referenced..";;

ATTRIBUTES

slTimersProfileId	GET SET BY CREATE,

q704-t1		GET SET BY CREATE,

q704-t3		GET SET BY CREATE,

q704-t17		GET SET BY CREATE,

q704-t24		GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

slTimersProfileNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 10 };

### 7.1.11	Signalling Point Timers Profile

spTimersProfile MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

spTimersProfilePackage PACKAGE

BEHAVIOUR spTimersProfileBehaviour BEHAVIOUR DEFINED AS

"A signalling link set timer profile is set of Q.704 and Q.707 timers which are related to the characteristics of signalling points.

The managed object class spTimersProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signPoint one of the spTimersProfiles is referenced.";;

ATTRIBUTES

spTimersProfileId	GET SET BY CREATE,

q704-t2	GET SET BY CREATE,

q704-t4	GET SET BY CREATE,

q704-t5	GET SET BY CREATE,

q704-t7	GET SET BY CREATE,

q704-t11	GET SET BY CREATE,

q704-t12	GET SET BY CREATE,

q704-t13	GET SET BY CREATE,

q704-t14	GET SET BY CREATE,

q704-t15	GET SET BY CREATE,

q704-t16 	GET SET BY CREATE,

q704-t18	GET SET BY CREATE,

q704-t19	GET SET BY CREATE,

q704-t20	GET SET BY CREATE,

q704-t21	GET SET BY CREATE,

q704-t22	GET SET BY CREATE,

q704-t23	GET SET BY CREATE,

q707-t1	GET SET BY CREATE,

q707-t2	GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

spTimersProfileNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 11 };

### 7.1.12	Signalling Route Network Element Part

NOTE - The optional package ClsLoadsharingInformationPackage shows an example of how the load sharing mechanism may be implemented. However, this option might complicate the operation of SS No. 7 networks. Therefore, other implementation dependent solutions might be used in which the load sharing within the individual link sets of a combined link set is made independent of the destination.

signRouteNePart MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;
	CHARACTERIZED BY	signRouteNePartPackage PACKAGE

BEHAVIOUR	signRouteNePartBehaviour BEHAVIOUR DEFINED AS

"A signalling route (A,C,B)i is defined as an ordered sequence of the near end signalling point A (i.e. the network element), an adjacent SP C and the destination SP B. The adjacent SP C may be identical with the destination B, or it may or may not be directly interconnected to the destination SP.

The adjacent SP C must be directly connected to the near end SP by at least one linkset which is intended to convey MSUs sent from A to B (hence the use of the term 'adjacent').

Two signalling routes (A,C,B)i and (A,C',B)j are distinct if the adjacent SP is different.

The signRouteNePart Managed Object class defines the management capabilities of the resources which define a specific route segment (Link Set to be traversed) and its priority within the routeset as seen from the own SP.

A particular signRouteNePart is related to one or more Signalling Routes on the network level.

The priority in which the network Signalling Routes are used, is defined by means of assigning priorities to all involved route segments. If from a particular SP two or more route segments are used with the same priority, loadsharing between Signalling Routes may occur. A Combined Link Set is a set of Link Sets used with the same priority by message routing.

The signLinkSetTp used for routing is allocated at creation time and cannot be changed during the existence of a particular signRouteNePart.

The administrativeState attribute describes whether it is administratively permitted to route messages towards the respective destination point via the referenced signLinkSetTp.

The operationalState attribute is 'enabled' if both the referenced signLinkSetTp is in the operational state 'enabled' and no TFP message concerning the respective destinations has been received from the adjPC of the respective signLinkSetTp.

The optional usageState attribute is used to cover restriction of a route. Its value is set to 'busy', if the route is restricted ("transfer restricted' is a national option).

The availabilityStatus attribute gives additional information about the availability of the signRouteNePart instance. It is furthermore needed to map some of the MTP functional states (prohibited: availabilityStatus = {dependency}, restricted: availabilityStatus={degraded}) and is used for the case that the route is locked not by the OS, but e.g. by a local administrator (administrativeState=unlocked and availabilityStatus={off line}).";;

ATTRIBUTES

signRouteNePartId		GET SET BY CREATE,

signLinkSetTPPointer	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState 	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState 	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

clsLoadsharingInformationPackage PRESENT IF "an instance supports it",

fixedRouteNePartPriorityPackage PRESENT IF "the instance does not use the flexibleRouteNePartPriorityPackage",

flexibleRouteNePartPriorityPackage PRESENT IF "the instance does not use the fixedRouteNePartPriorityPackage",

loadsharingInformationRouteNePartPackage PRESENT IF "the instance supports it",

mtpLoadsharingObjectForRouteNePartPointerPackage PRESENT IF "the instance supports it",

signRouteNePartNamePackage PRESENT IF "an instance supports it",

slsListPackage PRESENT IF "the instance supports it",

usageStatePackage PRESENT IF "the instance supports 'transfer restricted'";

REGISTERED AS { mtpObjectClass 12 }

### 7.1.13	Signalling Route Set Network Element Part

signRouteSetNePart MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	signRouteSetNePartPackage PACKAGE

BEHAVIOUR signRouteSetNePartBehaviour BEHAVIOUR DEFINED AS

"A Signalling Route Set is a complete set of routes that can be used to carry traffic toward a specific destination signalling point.

The signRouteSetNePart Managed Object class defines the management capabilities of the resources which represent a specific destination SP and its accessibility as seen from the own SP. This SP may be the originating SP of the MTP messages or an STP.

A particular signRouteSetNePart may be related to one or more Signalling Route Sets on network level.

The administrativeState attribute describes whether it is administratively permitted to route SS No. 7 MSUs towards the respective destination point. Before the administrative state of a signRouteSetNePart can be set to 'unlocked', there must be at least one signRouteNePart contained within the signRouteSetNePart.

The operationalState attribute describes whether the respective destination signalling point is accessible - ('enabled') or not ('disabled'). If the operational states of all signRouteNePart instances contained in the signRouteSetNePart are 'disabled', then the operational state of the signRouteSetNePart is 'disabled', in any other case it is 'enabled'.

The usageState attribute is used to reflect route set congestion. The value is 'busy' in case of route set congestion.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2: 1992':communicationsAlarm notification is present, it indicates at least the following probable causes:

Probable Cause = 411(RouteSetUnavailable), which represents measurements Q.752/4.11

(PerceivedSeverity=Maj/Min/War) and Q.752/4.12 (PerceivedSeverity=Cleared).

If the notification for Q.752/5.1 is supported, no notifications for Q.752/4.11 and 4.12 should be sent out.

If the ss7OnOccEventPackage is present, event notifications with the following probable cause can apply:

Probable Cause = 002 (remoteUserPartUnavailable), with the involved user part as parameter. Possible values for SpecificProblems are: 007 'unknown', 008 'unequipped' and 009 'inaccessibleRemoteUser'.

An activation or deactivation of a signRouteSetNePart must lead to the activation respectively deactivation of all contained signRouteNeParts.";;

ATTRIBUTES

signRouteSetNePartId	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and the (communicationsAlarmPackage or ss7OnOccEventPackage) is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

congestedStatePackage PRESENT IF "the instance does not use the congestionLevelPackage",

congestionLevelPackage PRESENT IF "the instance does not use the congestedStatePackage",

loadsharingInformationRouteSetNePartPackage PRESENT IF "the instance supports it",

mtpLoadsharingObjectForRouteSetNePartPointerPackage PRESENT IF "the instance supports it",

remoteExchangeLabelPackage PRESENT IF "the instance supports it",

signRouteSetNePartNamePackage PRESENT IF "the instance supports it",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 13 };

### 7.1.14	Signalling Terminal

signTerm MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	signTermPackage PACKAGE

BEHAVIOUR	signTermBehaviour BEHAVIOUR DEFINED AS

"The signTerm MOC defines the management capabilities for the signalling terminal. The signalling terminal is defined in ITU-T Rec. Q.703.";;

ATTRIBUTES

signTermId		GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

-- the application of the following state is f.f.s.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usageState		GET,

-- the application of the following state is f.f.s.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET,

-- the application of the following state is f.f.s.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET,

equipmentPointer		GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":stateChange;;;
	CONDITIONAL PACKAGES

ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

signTermNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 14 };

### 7.1.15	STP Screening Table

stpScreeningTable MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	stpScreeningTablePkg PACKAGE
		BEHAVIOUR stpScreeningTableBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTable object class is the base for STP screening. Together with its subordinate stpScreeningTableLine's it contains all the information which are necessary for an STP to identify and to handle unauthorized SS No. 7 messages as defined in 8.2/Q.705 and 8.3/Q.705.

";;

ATTRIBUTES

stpScreeningTableId	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState

INITIAL VALUE MTPDefinedTypesModule.

administrativeStateLocked	GET-REPLACE,

treatmentOfOutsideRanges	GET-REPLACE,

stpScreeningTableListMode	GET-REPLACE,

screeningByLinkSetOrByOpc	GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

getScreenedOpcsOrLinkSetsByDpcActionPackage PRESENT IF "an instance supports it",

stpScreeningTableNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 15 };

### 7.1.16	STP Screening Table Line

stpScreeningTableLine MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top,

CHARACTERIZED BY stpScreeningTableLinePkg PACKAGE

BEHAVIOUR stpScreeningTableLineBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableLine together with its superior stpScreeningTable contains all the information which are necessary for an STP to identify and to handle unauthorized SS No. 7

messages as defined in 8.2/Q.705 and 8.3/Q.705.

The total number of screened DPC's across all stpScreeningTableLine instances contained by a stpScreeningTable must be less or equal to maxNoOfScreenedDpcs.";;

ATTRIBUTES

stpScreeningTableLineId	GET SET BY CREATE.

stpScreeningTableLineContent	GET-REPLACE ADD-REMOVE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class";

stpScreeningTableLineNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 16 };

## 7.2	Package definitions

administrativeStatePackage PACKAGE

BEHAVIOUR administrativeStatePackageBehaviour BEHAVIOUR DEFINED AS

"This package can be used for activiated and deactivated link sets. Possible values of the administrativeState attribute are: locked and unlocked.

If transition takes place from locked to unlocked, this will result in activiation of all links in the set that are unlocked.

If transition takes place from unlocked to locked, this will result in deactiviation of all links in the set. This has no effect on the administrative states of the links themselves";;

ATTRIBUTES

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE;

REGISTERED AS { mtpPackage 1 };

bufferMechanismPackage PACKAGE

ATTRIBUTES

numberOfThresholdLevels	GET SET BY CREATE;

REGISTERED AS { mtpPackage 2 };

clCPackage PACKAGE

ATTRIBUTES

clC	GET SET BY CREATE;

REGISTERED AS { mtpPackage 3 };

clsLoadsharingInformationPackage PACKAGE

ATTRIBUTES

inClsLoadsharingAlgorithm	GET-REPLACE;

REGISTERED AS { mtpPackage 4 };

communicationsAlarmPackage PACKAGE

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":communicationsAlarm

inaccessibleSp;

REGISTERED AS { mtpPackage 5 };

congestedStatePackage PACKAGE

BEHAVIOUR congestedStatePackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies an attribute for the simple congestion method used e.g. in the international network.";;

ATTRIBUTES

congestedState	GET;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":attributeValueChange;

REGISTERED AS { mtpPackage 6 };

congestionHandlingPackage PACKAGE

ATTRIBUTES

congestionCounting	GET SET BY CREATE,

congestionReportingBaseObject	GET SET BY CREATE;

REGISTERED AS { mtpPackage 7 };

congestionLevelPackage PACKAGE

BEHAVIOUR congestionLevelPackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies an attribute to allow for several congestion levels for the signRouteSetNePart.";;

ATTRIBUTES

congestionLevel	GET;

REGISTERED AS { mtpPackage 8 };

getScreenedOpcsOrLinkSetsByDpcActionPackage PACKAGE

ACTIONS

getScreenedOpcsOrLinkSetsByDpc

REGISTERED AS { mtpPackage 9 };

fixedRouteNePartPriorityPackage PACKAGE

ATTRIBUTES

fixedPriority	GET SET BY CREATE;

REGISTERED AS { mtpPackage 10 };

flexibleRouteNePartPriorityPackage PACKAGE

ATTRIBUTES

flexiblePriority	GET-REPLACE,

priorityMode	GET-REPLACE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":attributeValueChange;

REGISTERED AS { mtpPackage 11 };

inLsLoadShareAlgorithmPackage PACKAGE

ATTRIBUTES

inLsLoadShareAlgorithm	GET SET BY CREATE;

-- in some implementations the inLsLoadShareAlgorithm attribute might be additionally writable

REGISTERED AS { mtpPackage 12 };

linkCongestionLevelPackage PACKAGE

ATTRIBUTES

linkCongestionLevel	GET;

REGISTERED AS { mtpPackage 13 };

loadsharingInformationRouteNePartPackage PACKAGE

ATTRIBUTES

loadsharingInformationRouteNePart	GET-REPLACE;

REGISTERED AS { mtpPackage 14 };

loadsharingInformationRouteSetNePartPackage PACKAGE

ATTRIBUTES

loadsharingInformationRouteSetNePart	GET-REPLACE;

REGISTERED AS { mtpPackage 15 };

loopDelayPackage PACKAGE

ATTRIBUTES

loopDelay	GET SET BY CREATE;

REGISTERED AS { mtpPackage 16 };

lsTimersProfileNamePackage PACKAGE

ATTRIBUTES

lsTimersProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 17 };

lsTimersProfilePointerPackage PACKAGE

ATTRIBUTES

lsTimersProfilePointer	GET-REPLACE;

REGISTERED AS { mtpPackage 18 };

managedSwitchingElementNamePackage PACKAGE

ATTRIBUTES

managedSwitchingElementName	GET-REPLACE;

REGISTERED AS { mtpPackage 19 };

mtpAccessPointNamePackage PACKAGE

ATTRIBUTES

mtpAccessPointName	GET-REPLACE;

REGISTERED AS { mtpPackage 20 };

mtpL2ProtocolProfileNamePackage PACKAGE

ATTRIBUTES

mtpL2ProtocolProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 21 };

mtpLevel3NamePackage PACKAGE

ATTRIBUTES

mtpLevel3Name	GET-REPLACE;

REGISTERED AS { mtpPackage 22 };

mtpLoadsharingObjectForRouteNePartPointerPackage PACKAGE

ATTRIBUTES

mtpLoadsharingObjectForRouteNePartPointer		GET-REPLACE;

REGISTERED AS { mtpPackage 23 };

mtpLoadsharingObjectForRouteSetNePartPointerPackage PACKAGE

ATTRIBUTES

mtpLoadsharingObjectForRouteSetNePartPointer		GET-REPLACE;

REGISTERED AS { mtpPackage 24 };

mtpSignPointNamePackage PACKAGE

ATTRIBUTES

mtpSignPointName	GET-REPLACE;

REGISTERED AS { mtpPackage 25 };

multipleTransmissionCongestionLevelsPackage PACKAGE

BEHAVIOUR multipleTransmissionCongestionLevelsPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the second and third levels of transmission congestion. The values of the thresholds belonging to congestion level 2 have to be bigger than the respective ones of congestion level 1. The values of the thresholds belonging to congestion level 3 have to be bigger than the respective ones of congestion level 2.";;

ATTRIBUTES

transmissionCongestionThresholdDiscardLevel1	GET SET BY CREATE,

transmissionCongestionThresholdOnsetLevel2	GET SET BY CREATE,

transmissionCongestionThresholdAbatementLevel2	GET SET BY CREATE,

transmissionCongestionThresholdDiscardLevel2	GET SET BY CREATE,

transmissionCongestionThresholdOnsetLevel3	GET SET BY CREATE,

transmissionCongestionThresholdAbatementLevel3	GET SET BY CREATE,

transmissionCongestionThresholdDiscardLevel3	GET SET BY CREATE,

REGISTERED AS { mtpPackage 26};

multipleTransmissionCongestionStatesPackage PACKAGE

BEHAVIOUR multipleTransmissionCongestionStatesPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the attributes related to the management of 'multiple link congestion states without congestion priority' method. This method is described in 3.8.2.3/Q.704. See also B.1.2.9.2. The range of initialLevelIfCongested is [1,numberOfCongestionStates].";;

ATTRIBUTES

timerTx	GET SET BY CREATE,

timerTy	GET SET BY CREATE,

numberOfCongestionStates	GET SET BY CREATE,

initialLevelIfCongested	GET SET BY CREATE;

REGISTERED AS { mtpPackage 27 };

noBasicLinkAllocationPackage PACKAGE

ATTRIBUTES

numberOfNormallyActiveSignLinksTps	GET-REPLACE;

REGISTERED AS { mtpPackage 28 };

pCRPackage PACKAGE

BEHAVIOUR pCRPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains signLinkTp attributes for the PCR method";;

ATTRIBUTES

maxMSUsRetransN1	GET SET BY CREATE,

maxOctRetransN2	GET SET BY CREATE;

REGISTERED AS { mtpPackage 29 };

receiveCongestionLevelsPackage PACKAGE

BEHAVIOUR receiveCongestionLevelsPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the levels of receive congestions";;

ATTRIBUTES

receiveCongestionThresholdOnset	GET SET BY CREATE,

receiveCongestionThresholdAbatement	GET SET BY CREATE;

REGISTERED AS { mtpPackage 30 };

relatedLinkGroupNumberPackage PACKAGE

ATTRIBUTES

relatedLinkGroupNumber	GET-REPLACE;

REGISTERED AS { mtpPackage 31 };

remoteExchangeLabelPackage PACKAGE

BEHAVIOUR remoteExchangeLabelPackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies the possibility to have textual information about the remote exchange that contains the destination signalling point.";;

ATTRIBUTES

remoteExchangeLabel	GET-REPLACE;

REGISTERED AS { mtpPackage 32 };

signDataLinkTpListPackage PACKAGE

ATTRIBUTES

signDataLinkTpList	GET SET BY CREATE ADD-REMOVE;

REGISTERED AS { mtpPackage 33 };

signDataLinkTpNamePackage PACKAGE

ATTRIBUTES

signDataLinkTpName	GET-REPLACE;

REGISTERED AS { mtpPackage 34 };

signLinkSetTpNamePackage PACKAGE

ATTRIBUTES

signLinkSetTpName	GET-REPLACE;

REGISTERED AS { mtpPackage 35 };

signLinkTestPackage PACKAGE

ACTIONS

signLinkTest;

REGISTERED AS { mtpPackage 36 };

signLinkTpNamePackage PACKAGE

ATTRIBUTES

signLinkTpName	GET-REPLACE;

REGISTERED AS { mtpPackage 37 };

signRouteNePartNamePackage PACKAGE

ATTRIBUTES

signRouteNePartName	GET-REPLACE;

REGISTERED AS { mtpPackage 38 };

signRouteSetNePartNamePackage PACKAGE

ATTRIBUTES

signRouteSetNePartName	GET-REPLACE;

REGISTERED AS { mtpPackage 39 };

signTermListPackage PACKAGE

ATTRIBUTES

signTermList	GET SET BY CREATE ADD-REMOVE;

REGISTERED AS { mtpPackage 40 };

signTermNamePackage PACKAGE

ATTRIBUTES

signTermName	GET-REPLACE;

REGISTERED AS { mtpPackage 41 };

slsCodeNormalListPackage PACKAGE

ATTRIBUTES

slsCodeNormalList	GET SET BY CREATE;

-- in some implementations this attribute might additionally be writable

REGISTERED AS { mtpPackage 42 };

slsListPackage PACKAGE

ATTRIBUTES

slsList	GET SET BY CREATE;

REGISTERED AS { mtpPackage 43 };

slTimersProfileNamePackage PACKAGE

ATTRIBUTES

slTimersProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 44 };

slTimersProfilePointerPackage PACKAGE

ATTRIBUTES

slTimersProfilePointer	GET-REPLACE;

REGISTERED AS { mtpPackage 45 };

spTimersPackage PACKAGE

BEHAVIOUR spTimersPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the values of the Q.704 and Q.707 timers.";;

ATTRIBUTES

q704-t1	GET SET BY CREATE,

q704-t2	GET SET BY CREATE,

q704-t3	GET SET BY CREATE,

q704-t4	GET SET BY CREATE,

q704-t5	GET SET BY CREATE,

q704-t6	GET SET BY CREATE,

q704-t7	GET SET BY CREATE,

q704-t8	GET SET BY CREATE,

q704-t10	GET SET BY CREATE,

q704-t11	GET SET BY CREATE,

q704-t12	GET SET BY CREATE,

q704-t13	GET SET BY CREATE,

q704-t14	GET SET BY CREATE,

q704-t15	GET SET BY CREATE,

q704-t16	GET SET BY CREATE,

q704-t17	GET SET BY CREATE,

q704-t18	GET SET BY CREATE,

q704-t19	GET SET BY CREATE,

q704-t20	GET SET BY CREATE,

q704-t21	GET SET BY CREATE,

q704-t22	GET SET BY CREATE,

q704-t23	GET SET BY CREATE,

q704-t24	GET SET BY CREATE,

q707-t1	GET SET BY CREATE,

q707-t2	GET SET BY CREATE;

REGISTERED AS { mtpPackage 46 };

spTimersProfileNamePackage PACKAGE

ATTRIBUTES

spTimersProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 47 };

spTimersProfilePointerPackage PACKAGE

ATTRIBUTES

spTimersProfilePointer	GET-REPLACE;

REGISTERED AS { mtpPackage 48 };

ss7OnOccEventPackage PACKAGE

NOTIFICATIONS

ss7OnOccEvent

changeInLsToAdjSp

inaccessibleSp

remoteUserPartUnavailable;

REGISTERED AS { mtpPackage 49 };

stmChannelPackage PACKAGE

ATTRIBUTES

stmChannel	GET;

REGISTERED AS { mtpPackage 50 };

stpScreeningTableLineNamePackage PACKAGE
	ATTRIBUTES
		stpScreeningTableLineName GET-REPLACE;
REGISTERED AS { mtpPackage 53 };

stpScreeningTableNamePackage PACKAGE

ATTRIBUTES

stpScreeningTableName	GET-REPLACE;

REGISTERED AS { mtpPackage 51 };

usageStatePackage PACKAGE

BEHAVIOUR usageStatePackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies a possibility to cover 'transfer restricted' for a route.";;

ATTRIBUTES

ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":usageState	GET;

REGISTERED AS { mtpPackage 52 };

## 7.3	Parameter definitions

changeInLsToAdjSp		PARAMETER

CONTEXT	EVENT-INFO;

WITH SYNTAX		MTPDefinedTypesModule.ChangeInLsToAdjSpInfo;

REGISTERED AS { mtpParameter 1 };

inaccessibleSp		PARAMETER

CONTEXT	EVENT-INFO;

WITH SYNTAX		MTPDefinedTypesModule.InaccessibleSpInfo;

REGISTERED AS { mtpParameter 2 };

remoteUserPartUnavailable		PARAMETER

CONTEXT	EVENT-INFO;

WITH SYNTAX	MTPDefinedTypesModule.UserPart;

REGISTERED AS { mtpParameter 3 };

## 7.4	Attribute definitions

adjPc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR adjPcBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the pointcode of an adjacent SP.";;

REGISTERED AS { mtpAttribute 1 };

cIC ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Cic;

MATCHES FOR EQUALITY;

BEHAVIOUR cicBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference the SS No. 7 trunk used by the datalink. Its value has to be unique within the SP's connected by the trunk.";;

REGISTERED AS { mtpAttribute 2 };

congestedState ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestedState

MATCHES FOR EQUALITY;

BEHAVIOUR congestedStateBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reflect route set congestion, if no multiple congestion levels are used. Its value is 'congested' in case of route set congestion. Its value can change from system inside.";;

REGISTERED AS { mtpAttribute 3 };

congestionControlMethod ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionControlMethod;

BEHAVIOUR congestionControlMethodBehaviour BEHAVIOUR DEFINED AS

"This attribute reflects the congestion control method used within a signalling point.";;

REGISTERED AS { mtpAttribute 4 };

congestionCounting ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionCounting;

BEHAVIOUR congestionCountingBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates whether congestion counting is message based or octet based.";;

REGISTERED AS { mtpAttribute 5 };

congestionLevel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionLevel;

MATCHES FOR EQUALITY;

BEHAVIOUR congestionLevelBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reflect route set congestion, if multiple congestion levels are used. Its value in case of no congestion is 'none'. Its value can change from system inside.";;

REGISTERED AS { mtpAttribute 6 };

congestionReportingBaseObject ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionReportingBaseObject;

BEHAVIOUR congestionReportingBaseObjectBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates on what object congestion counting is based.";;

REGISTERED AS { mtpAttribute 7 };

currentCapacityLS ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLS;

BEHAVIOUR currentCapacityLSBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the current capacity of a signalling link set.";;

REGISTERED AS { mtpAttribute 8 };

currentCapacitySL ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLink;

BEHAVIOUR currentCapacitySLBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the current capacity of a signalling link.";;

REGISTERED AS { mtpAttribute 9 };

dpc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR dpcBehaviour BEHAVIOUR DEFINED AS

"This attribute represents a destination pointcode.";;

REGISTERED AS { mtpAttribute 10 };

equipmentPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR equipmentPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference physical equipment.";;

REGISTERED AS { mtpAttribute 11 };

errorCorrectionMethod ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.ErrorCorrectionMethod;

MATCHES FOR EQUALITY;

BEHAVIOUR errorCorrectionMethodBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to specify the type of error-correction the signalling link is using.";;

REGISTERED AS { mtpAttribute 12 };

fixedPriority ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.FixedPriority;

BEHAVIOUR fixedPriorityBehaviour BEHAVIOUR DEFINED AS

"This attribute determines, if the signRouteNePart is used as current route. The signRouteNePart instances with operational state = 'enabled' contained in the same signRouteSetNePart are chosen in ascending order as current routes (The lower the value, the higher the priority).

There are different behaviours for the handling of 'priority gaps' possible:

i) priorityGapAllowedBEHAVIOUR ii) priorityGapNotAllowedBehaviour:

In the latter case it is not possible, to have a priority value, if there is a lower value, which is not used in one of the routes of the route set. E.g. in case of deleting a signRouteNePart instance whose priority value is not equal to the priority of at least one of the remaining signRouteNeParts contained in the same signRouteSetNePart, the priority values of the remaining instances have to be decremented by one (see also example in table below).

What kind of priority gap behaviour is performed by the switch is to be derived from the combination of the values of the attributes vendorName and version of the managedSwitchingElement.";;

REGISTERED AS { mtpAttribute 13 };

flexiblePriority ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.FlexiblePriority;

BEHAVIOUR flexiblePriorityBehaviour BEHAVIOUR DEFINED AS

"This attribute determines, if the signRouteNePart is used as current route. The signRouteNePart instances with operational state = 'enabled' contained in the same signRouteSetNePart are chosen in ascending order as current routes (The lower the value, the higher the priority).

There are different behaviours for the handling of 'priority gaps' possible:

i) priorityGapAllowedBEHAVIOUR ii) priorityGapNotAllowedBehaviour:

In the latter case it is not possible, to have a priority value. If there is a lower value, which is not used in one of the routes of the route set. E.g. in case of deleting a signRouteNePart instance whose priority value is not equal to the priority of at least one of the remaining signRouteNeParts contained in the same signRouteSetNePart, the priority values of the remaining instances have to be decremented by one (see also example in table below).

What kind of priority gap behaviour is performed by the switch is to be derived from the combination of the values of the attributes vendorName and version of the managedSwitchingElement.

This attribute is writable only together with the attribute priorityMode.";;

REGISTERED AS { mtpAttribute 14 };

inClsLoadsharingAlgorithm ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.InClsLoadsharingAlgorithm;

BEHAVIOUR inClsLoadsharingAlgorithmBehaviour BEHAVIOUR

DEFINED AS

"This attribute represents the over- and in-link set loadsharing algorithm within a combined link set.";;

REGISTERED AS { mtpAttribute 15 };

initialLevelIfCongested ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.InitialLevelIfCongested;

BEHAVIOUR initialLevelIfCongestedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the initial level of congestion.";;

REGISTERED AS { mtpAttribute 16 };

inLsLoadShareAlgorithm ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.InLsLoadShareAlgorithm;

BEHAVIOUR inLsLoadShareAlgorithmBehaviour BEHAVIOUR DEFINED AS

"The inLsLoadShareAlgorithm specifies a registered, vendor-specific within-linkset loadsharing algorithm.";;

REGISTERED AS { mtpAttribute 17 };

l2TimerT1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT1;

BEHAVIOUR	l2TimerT1Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Alignment ready'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 18 };

l2TimerT2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT2;

BEHAVIOUR	l2TimerT2Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Not aligned'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 19 };

l2TimerT3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT3;

BEHAVIOUR	l2TimerT3Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Aligned'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 20 };

l2TimerT4E ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT4E;

BEHAVIOUR	l2TimerT4eBehaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Proving period timer emergency'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 21 };

l2TimerT4N ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT4N;

BEHAVIOUR	l2TimerT4nBehaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Proving period timer normal'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 22 };

l2TimerT5 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT5;

BEHAVIOUR	l2TimerT5Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Sending SIB'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 23 };

l2TimerT6 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT6;

BEHAVIOUR	l2TimerT6Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Remote congestion'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 24 };

l2TimerT7 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT7;

BEHAVIOUR	l2TimerT7Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Delay to avoid message mis-sequence on changeover. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 25 };

linkCongestionLevel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionLevel;

MATCHES FOR EQUALITY;

BEHAVIOUR linkCongestionLevelBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reflect link congestion, if multiple congestion levels are used. Its value in case of no congestion is 'none'. Its value can change from system inside.";;

REGISTERED AS { mtpAttribute 26 };

linkTPStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LinkTPStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR linkTpStatusBehaviour BEHAVIOUR DEFINED AS

"The linkTpStatus contains the SS No. 7 functional statuses as described in Recommendation Q.704. Possible states indicated are local blocked, remote blocked, local inhibited, remote inhibited, failed and deactivated.";;

REGISTERED AS { mtpAttribute 27 };

loadsharingInformationRouteNePart ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LoadsharingInformationRouteNePart;

BEHAVIOUR loadsharingInformationRouteNePartBehaviour BEHAVIOUR DEFINED AS

"This attribute contains specific information for target specific loadsharing via the current routes working on a route basis. This information can be used by e.g. the referenced mtpLoadsharingObjectFor-Route to execute the target specific loadsharing.";;

REGISTERED AS { mtpAttribute 28 };

loadsharingInformationRouteSetNePart ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LoadsharingInformationRouteSetNePart;

BEHAVIOUR loadsharingInformationRouteSetNePartBehaviour BEHAVIOUR DEFINED AS

"This attribute contains specific information for target specific loadsharing via the current routes working on a routeset basis via the current routes. This information can be used by e.g. the referenced mtpLoadsharingObjectFor-RouteSet to execute the target specific loadsharing.";;

REGISTERED AS { mtpAttribute 29 };

loopDelay ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LoopDelay;

MATCHES FOR EQUALITY;

BEHAVIOUR loopDelayBehaviour BEHAVIOUR DEFINED AS

"This attribute models loop delay.";;

REGISTERED AS { mtpAttribute 30 };

lsTimersProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR lsTimersProfileIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 31 };

lsTimersProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR lsTimersProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the lsTimersProfile managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 32 };

lsTimersProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR lsTimersProfilePointerBehaviour BEHAVIOUR DEFINED AS

"This attribute to the instance of the class lsTimersProfile, containing the Q.704 timers for the signalling link set.";;

REGISTERED AS { mtpAttribute 33 };

managedSwitchingElementName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR managedSwitchingElementNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the managedSwitchingElement managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 34 };

maxCapacityLS ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLS;

REGISTERED AS { mtpAttribute 35 };

maxCapacitySL ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLink;

REGISTERED AS { mtpAttribute 36 };

maxMSUsRetransN1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.MaxMSUsRetransN1;

BEHAVIOUR maxMSUsRetransN1Behaviour BEHAVIOUR

DEFINED AS

"This attribute specifies the maximum number of MSUs which are available for retransmission (name in Q.703:N1).";;

REGISTERED AS { mtpAttribute 37 };

maxOctRetransN2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.MaxOctRetransN2;

BEHAVIOUR maxOctRetransN2Behaviour BEHAVIOUR DEFINED AS

"This attribute specifies the maximum number of octets, which are available for retransmission (Q.703: N2).";;

REGISTERED AS { mtpAttribute 38 };

mtpAccessPointName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpAccessPointNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpAccessPoint managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 39 };

mtpL2ProtocolProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpL2ProtocolProfileIdBehaviour BEHAVIOUR

DEFINED AS

"Identifies a mtpL2ProtocolProfile instance within the managedSwitching-element; this is an attribute type whose distinguished value can be used as an RDN when naming an instance of the mtpL2ProtocolProfile object class.";;

REGISTERED AS { mtpAttribute 40 };

mtpL2ProtocolProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpL2ProtocolProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpL2ProtocolProfile managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 41 };

protocolProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR protocolProfilePointerBehaviour BEHAVIOUR

DEFINED AS

"Identifies an protocolProfile instance.";;

REGISTERED AS { mtpAttribute 42 };

mtpLevel3Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLevel3IdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 43 };

mtpLevel3Name ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLevel3NameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpLevel3 managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 44 };

mtpLoadsharingObjectForRouteNePartPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLoadsharingObjectForRouteNePartPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference implementation dependent loadsharing object class instances needed to model implementation dependent mechanisms for loadsharing over linksets working on a route basis.";;

REGISTERED AS { mtpAttribute 45 };

mtpLoadsharingObjectForRouteSetNePartPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLoadsharingObjectForRouteSetNePartPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference implementation dependent loadsharing object class instances needed to model implementation dependent mechanisms for loadsharing over linksets working on a routeset basis.";;

REGISTERED AS { mtpAttribute 46 };

mtpSignPointId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpSignPointIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 47 };

mtpSignPointName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpSignPointNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpSignPoint managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 48 };

networkIndicator ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NetworkIndicator;

MATCHES FOR EQUALITY;

BEHAVIOUR networkIndicatorBehaviour BEHAVIOUR DEFINED AS

"This attribute models the network indicator of the network the signalling point belongs to.";;

REGISTERED AS { mtpAttribute 49 };

numberOfCongestionStates ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NumCongStates;

MATCHES FOR EQUALITY;

BEHAVIOUR numberOfCongestionStatesBehaviour BEHAVIOUR DEFINED AS

"This attribute specifies the number of congestion states used in the 'multiple link congestion states without congestion priority' mechanism as described in 3.8.2.3/Q.704.";;

REGISTERED AS { mtpAttribute 50 };

numberOfNormallyActiveSignLinksTps ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NumberOfNormallyActiveSignLinksTps;

MATCHES FOR EQUALITY;

BEHAVIOUR numberOfNormallyActiveSignLinksTpsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the number of normally active signalling links in a link set, as defined in clause 12/Q.704.";;

REGISTERED AS { mtpAttribute 51 };

numberOfThresholdLevels ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NumberOfThresholdLevels;

BEHAVIOUR numberOfThresholdsLevelsBehaviour BEHAVIOUR DEFINED AS

"This attribute equals the number of threshold levels that are used for congestion control.";;

REGISTERED AS { mtpAttribute 52 };

opc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR opcBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the pointcode of an SP from which a certain message has been originated.";;

REGISTERED AS { mtpAttribute 53 };

periodicLinkTestFlag ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PeriodicLinkTestFlag;

BEHAVIOUR periodicLinkTestFlagBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates whether the periodical signalling link test procedure of Recommendation Q.707 is applied to a link set.";;

REGISTERED AS { mtpAttribute 54 };

pointCode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR pointCodeBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the pointcode of a certain SP.";;

REGISTERED AS { mtpAttribute 55 };

priorityMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PriorityMode;

BEHAVIOUR priorityModeBehaviour BEHAVIOUR DEFINED AS

"This attribute is necessary in case of creating a signRouteNePart instance or modifiying its priority attribute. It describes the influence of the management action on the priority attributes of other signRouteNePart instances contained in the same signRouteSetNePart.

This attribute is only writable, and this only together with the writing of attribute flexiblePriority.

The meanings of the attribute values are:

EQUAL: If by modifying the priority with priorityMode 'EQUAL' a 'priority gap' would come into existence, then for the priorities of all created signRouteNePart instances from the same signRouteSetNePart with priorities lower than the 'gap priority' the priority value has to be decremented by one.

INSERT: Priority values of all created signRouteNePart instances from the same signRouteSetNePart with equal or lower priorities are incremented by one.

EXCHANGE_SINGLE: The priority values of all created signRouteNePart instances from the same signRouteSetNePart which have the specified 'target' priority value, are exchanged for the last ('starting') priority value of the specified signRouteNePart instance.

EXCHANGE_GROUP: The priority values of all created signRouteNePart instances from the same signRouteSetNePart which have the specified 'target' priority value, are exchanged for the last ('starting') priority value of the specified signRouteNePart instance. Additionally the priority values of all created signRouteNePart instances from the same signRouteSetNePart, which have the same value as the 'starting' priority value of the specified signRouteNePart instance, are changed to the 'target' value.";;

REGISTERED AS { mtpAttribute 56 };

q704-t1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t1Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T1 'Delay to avoid message mis-sequencing on changeover from this link' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.The same value applies for all links associated with an instance of the procedure.";;

REGISTERED AS { mtpAttribute 57 };

q704-t2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t2Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T2 'Waiting for changeover acknowledgement' defined in Recommendation Q.704. The value range is from 700 to 2000 ms. For routes with long propagation delays a minimum value of 1400 ms is used.";;

REGISTERED AS { mtpAttribute 58 };

q704-t3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t3Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T3 'Time controlled diversion-delay to avoid mis-sequencing on changeback from this link' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used. The value of this attribute is network implementation dependent.";;

REGISTERED AS { mtpAttribute 59 };

q704-t4 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t4Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T4 'Waiting for changeback acknowledgement (first attempt)' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.";;

REGISTERED AS { mtpAttribute 60 };

q704-t5 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t5Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T5 'Waiting for changeback acknowledgement (second attempt)' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.";;

REGISTERED AS { mtpAttribute 61 };

q704-t6 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t6Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T6 'Delay to avoid message mis-sequencing on controlled rerouting' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.";;

REGISTERED AS { mtpAttribute 62 };

q704-t7 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t7Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T7 'Waiting for signalling data link connection acknowledgement' defined in Recommendation Q.704. The value range is from 1 to 2 seconds.";;

REGISTERED AS { mtpAttribute 63 };

q704-t8 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t8Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T8 'Transfer prohibited inhibition timer (transient solution)' defined in Recommendation Q.704. The value range is from 800 to 1200 ms.";;

REGISTERED AS { mtpAttribute 64 };

q704-t10 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t10Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T10 'Waiting to repeat signalling route set test message' defined in Recommendation Q.704. The value range is from 30 to 60 seconds. The maximum value may be extended at the discretion of the management function in certain situations, e.g. many signalling points being unavailable or signalling points of known long term unavailability.";;

REGISTERED AS { mtpAttribute 65 };

q704-t11 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t11Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T11 'Transfer restricted timer' defined in Recommendation Q.704. The value range is from 30 to 90 seconds. This is one way of implementing the function described in 13.4/Q.704 and mainly intended to simplify STPs.";;

REGISTERED AS { mtpAttribute 66 };

q704-t12 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t12Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T12 'Waiting for uninhibit acknowledgement' defined in Recommendation Q.704. The value range is from 800 to 1500 ms.";;

REGISTERED AS { mtpAttribute 67 };

q704-t13 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t13Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T13 'Waiting for force uninhibit' defined in Recommendation Q.704. The value range is from 800 to 1500 ms.";;

REGISTERED AS { mtpAttribute 68 };

q704-t14 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t14Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T14 'Waiting for inhibition acknowledgement' defined in Recommendation Q.704. The value range is from 2 to 3 seconds.";;

REGISTERED AS { mtpAttribute 69 };

q704-t15 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t15Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T15 'Waiting to start signalling route set congestion test' defined in Recommendation Q.704. The value range is from 2 to 3 seconds.";;

REGISTERED AS { mtpAttribute 70 };

q704-t16 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t16Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T16 'Waiting for route set congestion status update' defined in Recommendation Q.704. The value range is from 1.4 to 2 seconds.";;

REGISTERED AS { mtpAttribute 71 };

q704-t17 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t17Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T17 'Delay to avoid oscillation of initial alignment failure and link restart' defined in Recommendation Q.704. The value range is from 800 to 1500 ms. The value should be less than the corresponding timer 2 in 12.2.1.2/Q.703.";;

REGISTERED AS { mtpAttribute 72 };

q704-t18 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t18Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T18 defined in Recommendation Q.704. T18 is the Timer within a signalling point whose MTP restarts, for supervising link and linkset activation as well as the receipt of routing information. The value is implementation and network dependent. Criteria to choose T18 are given in 9.2/Q.704.";;

REGISTERED AS { mtpAttribute 73 };

q704-t19 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t19Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T19 'Supervision timer during MTP restart to avoid possible ping-pong of TFP, TFR and TRA messages.' defined in Recommendation Q.704. The value range is from 67 to 69 seconds.";;

REGISTERED AS { mtpAttribute 74 };

q704-t20 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t20Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T20 'Overall MTP restart timer at the signalling point whose MTP restarts.' defined in Recommendation Q.704. The value range is from 59 to 61 seconds.";;

REGISTERED AS { mtpAttribute 75 };

q704-t21 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t21Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T21 'Overall MTP restart timer at a signalling point adjacent to one whose MTP restarts.' defined in Recommendation Q.704. The value range is from 63 to 65 seconds.";;

REGISTERED AS { mtpAttribute 76 };

q704-t22 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t22Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T22 'Local inhibit test timer' defined in Recommendation Q.704. The value range is from 3 to 6 minutes (provisional value).";;

REGISTERED AS { mtpAttribute 77 };

q704-t23 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t23Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T23 'Remote inhibit test timer' defined in Recommendation Q.704. The value range is from 3 to 6 minutes (provisional value).";;

REGISTERED AS { mtpAttribute 78 };

q704-t24 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t24Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T24 'Stabilising timer after removal of local processor outage, used in LPO latching to RPO (national option)' defined in Recommendation Q.704. The value is 500 ms (provisional value).";;

REGISTERED AS { mtpAttribute 79 };

q707-t1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q707-t1Behaviour BEHAVIOUR DEFINED AS

"This attribute defines a timer value of timer T1 which is defined in Recommendation Q.707.";;

REGISTERED AS { mtpAttribute 80 };

q707-t2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q707-t2Behaviour BEHAVIOUR DEFINED AS

"This attribute defines a timer value of timer T2 which is defined in Recommendation Q.707.";;

REGISTERED AS { mtpAttribute 81 };

receiveCongestionThresholdAbatement ATTRIBUTE

WITH ATTRIBUTE SYNTAX

MTPDefinedTypesModule.ReceiveCongestionThresholdAbatement;

BEHAVIOUR receiveCongestionThresholdAbatementBehaviour BEHAVIOUR DEFINED AS

"If the number of MSUs in the reception buffer gets smaller than this number, Level 2 flow control measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 82 };

receiveCongestionThresholdOnset ATTRIBUTE

WITH ATTRIBUTE SYNTAX

MTPDefinedTypesModule.ReceiveCongestionThresholdOnset;

BEHAVIOUR receiveCongestionThresholdOnsetBehaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the reception buffer gets bigger than this value, Level 2 flow control measures begin. MSUs are still accepted.";;

REGISTERED AS { mtpAttribute 83 };

relatedLinkGroupNumber ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.RelatedLinkGroupNumber;

BEHAVIOUR relatedLinkGroupNumberBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the link group which the signalling link belongs to.";;

REGISTERED AS { mtpAttribute 84 };

remoteExchangeLabel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.RemoteExchangeLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR remoteExchangeLabelBehaviour BEHAVIOUR DEFINED AS

"This attribute specifies the remote exchange which contains the mtpSignPoint with the point code equal to the signRouteSetNePartId.";;

REGISTERED AS { mtpAttribute 85 };

screeningByLinkSetOrByOpc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.ScreeningByLinkSetOrByOpc;

BEHAVIOUR screeningByLinkSetOrByOpcBehaviour BEHAVIOUR DEFINED AS

"This attribute defines which of the different options of 8.2/Q.705 are used for identifying unauthorized SS No. 7 messages.";;

REGISTERED AS { mtpAttribute 86 };

signDataLinkTpId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 87 };

signDataLinkTpList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointerList;

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpListBehaviour BEHAVIOUR DEFINED AS

"This attribute lists the signDataLink instances that can be used for dynamical allocation to the signLinkTp.";;

REGISTERED AS { mtpAttribute 88 };

signDataLinkTpName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signDataLinkTp managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 89 };

signDataLinkTpPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute references a particular signDataLinkTp instance. The following restriction applies to this attribute: modification is only allowed if the administrative state of the instance that contains this attribute is 'locked'.";;

REGISTERED AS { mtpAttribute 90 };

signLinkSetTpId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkSetTpIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 91 };

signLinkSetTpName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkSetTpNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signLinkSetTp managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 92 };

signLinkSetTpPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkSetTpPointerBehaviour BEHAVIOUR DEFINED AS

"References the signLinkSetTp which is intended to be used as first segment of the succession of linksets, which form the signalling route on the network level.

If during creation of a signRouteNePart instance the value of this attribute is referencing a signLinkSetTp instance, which is not created or is already referenced by a signRouteNePart of the same signRouteSetNePart, then the creation of the signRouteNePart is rejected and not carried out.

It is not allowed to reference a signLinksetTp, which is not contained in the same mtpSignPoint as the signRouteSetNePart where the signRouteNePart is contained in.";;

REGISTERED AS { mtpAttribute 93 };

signLinkTpName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkTpNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signLinkTp managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 94 };

signRouteNePartId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteNePartIdBehaviour BEHAVIOUR DEFINED AS

"This is the naming attribute of signRouteNePart. It identifies a signRouteNePart instance within the signRouteSetNePart.";;

REGISTERED AS { mtpAttribute 96 };

signRouteNePartName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteNePartNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signRouteNePart managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 97 };

signRouteSetNePartId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteSetNePartIdBehaviour BEHAVIOUR DEFINED AS

"This is the naming attribute of signRouteSetNePart. It identifies a signRouteSetNePart instance within the mtpSignPoint and equals the signalling point code of the respective destination signalling point.";;

REGISTERED AS { mtpAttribute 98 };

signRouteSetNePartName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteSetNePartNameBehaviour BEHAVIOUR DEFINED AS

"Identifies a signRouteSetNePart instance within the managed object class; its value is unique within the signRouteSetNePart object class. In this attribute implementation or administration dependent information - like used for naming schemes - can be stored. If at creation time the value of this attribute given for the instance to be created already exists in another signRouteNePart instance, then the creation is rejected and not carried out.";;

REGISTERED AS { mtpAttribute 99 };

signTermId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signTermIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 100 };

signTermList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointerList;

BEHAVIOUR signTermListBehaviour BEHAVIOUR DEFINED AS

"This attribute lists the signTerm instances that can be used for dynamical allocation to the signLinkTp.";;

REGISTERED AS { mtpAttribute 101 };

signTermName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signTermNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signTerm managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 102 };

signTermPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR signTermPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies a particular signTerm instance.";;

REGISTERED AS { mtpAttribute 103 };

slCode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SlCode;

MATCHES FOR EQUALITY;

BEHAVIOUR slCodeBehaviour BEHAVIOUR DEFINED AS

"If one SLC has been assigned, it should be checked that this SLC will not be assigned again.";;

REGISTERED AS { mtpAttribute 104 };

slsCodeCurrentList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SLSCodeCurrentList;

MATCHES FOR SET-INTERSECTION;

BEHAVIOUR slsCodeCurrentListBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates, which SLSs are currently assigned to this signLinkTp. It has to be ensured, that all SLSs are covered and no SLS exists more than one time within the slsCodeCurrentList attributes of the signLinkTp instances contained within one signLinkSetTp. The content of this attribute can be different from that of the attribute slsCodeNormalList. This attribute can be changed from system inside.";;

REGISTERED AS { mtpAttribute 105 };

slsCodeNormalList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SlsCodeNormalList;

MATCHES FOR SET-INTERSECTION;

BEHAVIOUR slsCodeNormalListBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates, which SLSs are administratively assigned to this signLinkTp for the case of normal operation.";;

REGISTERED AS { mtpAttribute 106 };

slsList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SlsCodeList;

BEHAVIOUR slsListBehaviour BEHAVIOUR DEFINED AS

"This attribute has to be used if loadsharing over link sets toward a specific destination is done. It indicates which SLSs are assigned to this signRouteNePart, in case it is a current route (i.e. the messages with these SLSs are sent over the allocated link set).

It has to be ensured that all SLSs are covered and no SLS exists more than one time within the slsList attributes of the current routes.

The attribute can be written from system inside.";;

REGISTERED AS { mtpAttribute 107 };

slTimersProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR slTimersProfileIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 108 };

slTimersProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR slTimersProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the slTimersProfile managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 109 };

slTimersProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR slTimersProfilePointerBehaviour BEHAVIOUR DEFINED AS

"This attribute points to the instance of the class slTimersProfile, containing the Q.704 timers for the signalling link.";;

REGISTERED AS { mtpAttribute 110 };

spTimersProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR spTimersProfileIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 111 };

spTimersProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR spTimersProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the spTimersProfile managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 112 };

spTimersProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR spTimersProfilePointerBehaviour BEHAVIOUR DEFINED AS

"This attribute points to the instance of the class spTimersProfile, containing the Q.704 and Q.707 timers for the signalling point.";;

REGISTERED AS { mtpAttribute 113 };

spType ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SpType;

MATCHES FOR EQUALITY;

BEHAVIOUR spTypeBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the SP-type: SEP, STEP or STP.";;

REGISTERED AS { mtpAttribute 114 };

stmChannel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StmChannel;

MATCHES FOR EQUALITY;

BEHAVIOUR stmChannelBehaviour BEHAVIOUR DEFINED AS

"This attribute denotes the STM channel which defines the signalling datalink on the PCM transmission system.";;

REGISTERED AS { mtpAttribute 115 };

stpScreeningTableListMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StpScreeningTableListMode;

BEHAVIOUR stpScreeningTableListModeBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableListMode attribute indicates whether the STP access is allowed or not by the designated combinations contained in the subordinate stpScreeningTableLine's (see 8.2/Q.705).

TRUE:	STP access allowed

FALSE:	STP access inhibited.";;

REGISTERED AS { mtpAttribute 116 };

stpScreeningTableId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR stpScreeningTableIdBehaviour BEHAVIOUR DEFINED AS

"The STP Screening Table Id is an attribute used for naming purposes.";;

REGISTERED AS { mtpAttribute 117 };

stpScreeningTableLineContent ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StpScreeningTableLineContent;

BEHAVIOUR stpScreeningTableLineContentBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableLineContent attribute represents the content of one line of the STP ScreeningTable. The content consists of elements indicating the designated DPCs which are to be screened in combination with the OPC or linkset given by the stpScreeningTableLineId and an indication how a message fulfilling such a combination is to be treated.";;

REGISTERED AS { mtpAttribute 118 };

stpScreeningTableLineId ATTRIBUTE

MATCHES FOR EQUALITY;

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StpScreeningTableLineId;

BEHAVIOUR stpScreeningTableLineIdBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableLineId is an attribute used for naming purposes. Depending on the value of attribute screeningByLinkSetOrByDpc of the superior stpScreeningTable instance it represents the

OPC or linkset which is designated to be screened for STP traffic.";;

REGISTERED AS { mtpAttribute 119 };

stpScreeningTableLineName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR stpScreeningTableLineNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the stpScreeningTableLine managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 120 };

stpScreeningTableName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR stpScreeningTableNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the stpScreeningTable managed object class. 
	A set request is rejected if the name is already used by another instance of the same class.";;

REGISTERED AS { mtpAttribute 121 };

timerTx ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

MATCHES FOR EQUALITY;

BEHAVIOUR timerTxBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the Tx timer value for the 'multiple link congestion states without congestion priority' mechanism as described in 3.8.2.3/Q.704.";;

REGISTERED AS { mtpAttribute 122 };

timerTy ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR timerTyBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the Ty timer value for the 'multiple link congestion states without congestion priority' mechanism as described in 3.8.2.3/Q.704.";;

REGISTERED AS { mtpAttribute 123 };

transmissionCongestionThresholdAbatementLevel1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdAbatementLevel1;

BEHAVIOUR transmissionCongestionThresholdAbatementLevel1Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets smaller than this number, Level 2 flow control measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 124 };

transmissionCongestionThresholdAbatementLevel2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdAbatementLevel2;

BEHAVIOUR transmissionCongestionThresholdAbatementLevel2Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transm./retransm. buffer of a link in the congestionLevel 2 gets smaller than this number, the resp. link is not any more in congestionLevel 2. CongestionLevel 2 measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 125 };

transmissionCongestionThresholdAbatementLevel3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdAbatementLevel3;

BEHAVIOUR transmissionCongestionThresholdAbatementLevel3Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transm./retransm. buffer of a link in the congestionLevel 3 gets smaller than this number, the resp. link is not any more in congestionLevel 3. CongestionLevel 3 measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 126 };

transmissionCongestionThresholdDiscardLevel1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdDiscardLevel1;

BEHAVIOUR transmissionCongestionThresholdDiscardLevel1Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer of a link in the congestion level 1 gets bigger than this number, MSUs are discarded (this threshold equals the respective buffer size). The value has to be bigger than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 127 };

transmissionCongestionThresholdDiscardLevel2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdDiscardLevel2;

BEHAVIOUR transmissionCongestionThresholdDiscardLevel2Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transm./retransm. buffer of a link in the congestion level 2 gets bigger than this number, MSUs are discarded (this threshold equals the respective buffer size). The value has to be bigger than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 128 };

transmissionCongestionThresholdDiscardLevel3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdDiscardLevel3;

BEHAVIOUR transmissionCongestionThresholdDiscardLevel3Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer of a link in the congestion level 3 gets bigger than this number, MSUs are discarded (this threshold equals the respective buffer size). The value has to be bigger than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 129 };

transmissionCongestionThresholdOnsetLevel1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdOnsetLevel1;

BEHAVIOUR transmissionCongestionThresholdOnsetLevel1Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 1. CongestionLevel 1 measures start.";;

REGISTERED AS { mtpAttribute 130 };

transmissionCongestionThresholdOnsetLevel2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdOnsetLevel2;

BEHAVIOUR transmissionCongestionThresholdOnsetLevel2Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 2. CongestionLevel 2 measures start.";;

REGISTERED AS { mtpAttribute 131 };

transmissionCongestionThresholdOnsetLevel3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdOnsetLevel3;

BEHAVIOUR transmissionCongestionThresholdOnsetLevel3Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 3. CongestionLevel 3 measures start.";;

REGISTERED AS { mtpAttribute 132 };

transmissionRate ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TransmissionRate;

MATCHES FOR EQUALITY;

BEHAVIOUR transmissionRateBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the transmissionRate and implies the transmissionType (i.e. 56 and 64 kbits imply digital, 4.8 kbits implies analog transmission).";;

REGISTERED AS { mtpAttribute 133 };

treatmentOfOutsideRanges ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TreatmentOf;

BEHAVIOUR treatmentOfOutsideRangesBehaviour BEHAVIOUR DEFINED AS

"The treatmentOfOutsideRanges attribute specifies the treatment of the messages outside the designated ranges given the stpScreeningTable managed object (see 8.3/Q.705, part two).

normal : allow all STP messages outside the designated ranges

discard : discard all STP messages outside the designated ranges.";;

REGISTERED AS { mtpAttribute 134 };

## 7.5	Action definitions

getScreenedOpcsOrLinkSetsByDpc ACTION

BEHAVIOUR getScreenedOpcsOrLinkSetsByDpcBehaviour BEHAVIOUR DEFINED AS

"This action gets all OPCs or linksets (depending on the value of the screeningByLinkSetOrByOPC attribute) together with their respective messageTreatments which are screened in combination with a specific DPC within all the stpScreeningTableLineContents of all stpScreeningTableLine's contained in the stpScreeningTable.

The resulting list in the reply is empty, if no OPC or Linkset within all the stpScreeningTableLine (Contents of all stpScreeningTableLine's contained in the stpScreeningTable is screened in combination with the DPC - given in the information syntax), or if no stpScreeningTableLine contained in the stpScreeningTable does exist.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX MTPDefinedTypesModule.SpecificDpc

WITH REPLY SYNTAX MTPDefinedTypesModule.ScreenedOpcsOrLinkSetsList;

REGISTERED AS { mtpAction 1 };

localInhibit ACTION

BEHAVIOUR localInhibitBehaviour BEHAVIOUR DEFINED AS

"This action inhibits the signalling link termination point. If the inhibition procedure completes successfully the linkTpStatus contains localInhibit. This action is not performed if the linkTpStatus contains localInhibit.";;

MODE CONFIRMED;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 2 };

localUninhibit ACTION

BEHAVIOUR localUninhibitBehaviour BEHAVIOUR DEFINED AS

"This action uninhibits the signalling link termination point. If the uninhibition procedure completes successfully the localInhibit is removed from linkTpStatus. This action is not performed if the linkTpStatus does not contain localInhibit.";;

MODE CONFIRMED;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 3 };

signLinkTest ACTION

BEHAVIOUR signLinkTestBehaviour BEHAVIOUR DEFINED AS

"This test is performed to analyse the signalling link.";;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 5 };

## 7.6	Notification definitions

mtpRestarting NOTIFICATION

WITH INFORMATION SYNTAX MTPDefinedTypesModule.MtpRestarting;

REGISTERED AS { mtpNotification 1 };

ss7OnOccEvent NOTIFICATION

BEHAVIOUR ss7OnOccEventBhv BEHAVIOUR DEFINED AS

"This notification is used to report the Q.752 on-occurrence measurement results and other MTP events, which are not reported as a communication alarm.";;

WITH INFORMATION SYNTAX MTPDefinedTypesModule.SS7OnOccEventInfo;

REGISTERED AS { mtpNotification 2 };

## 7.7	Naming binding definitions

Naming is specified by structure rules and name bindings. The structure rules define the distinguished names that the managed objects shall have and the ways in which the objects are related to one another in the Management Information Tree (MIT).

Each structure rule defines a sequence of name bindings. The name binding selects the attribute to be used in the relative distinguished name of an object. The structure rule herewith selects all the attributes to be used in the distinguished name of an object.

For the labels that are used for the name binding templates, the convention "superiorClass-subordinateClass" is followed.

Creation of new object class instances may be rejected due to lack of system resources, e.g. the system specific maximal number of instances of this object class per superior object instance have been exceeded.

managedElement- managedSwitchingElement NAME BINDING

SUBORDINATE OBJECT CLASS	managedSwitchingElement;

NAMED BY

SUPERIOR OBJECT CLASS	managedElement;

WITH ATTRIBUTE	managedSwitchingElementId;

BEHAVIOUR managedElement - managedSwitchingElementBehaviour BEHAVIOUR DEFINED AS

"If the managedSwitchingElementName package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 39 };

managedSwitchingElement-mtpSignPoint NAME BINDING

SUBORDINATE OBJECT CLASS	mtpSignPoint;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE mtpSignPointId;

BEHAVIOUR managedSwitchingElement-mtpSignPointBehaviour BEHAVIOUR DEFINED AS

"An mtpSignPoint can be created by an operator or automatically.

In case the spTimersProfilePointerPackage is used, a create or set request is rejected, if

i)	the spTimersProfilePointer does not reference an spTimersProfile

OR	ii)	the spTimersProfilePointer would reference an instance which does not exist

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

An mtpSignPoint can be deleted if and only if it does not contain any other managed object class instances, except for contained measurements.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 1 };

mtpSignPoint-signRouteSetNePart NAME BINDING

SUBORDINATE OBJECT CLASS	signRouteSetNePart;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE signRouteSetNePartId;

BEHAVIOUR mtpSignPoint-signRouteSetNePartBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

A delete request is rejected, if the administrativeState of the signRouteSetNePart does not equal locked.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 2 };

signRouteSetNePart-signRouteNePart NAME BINDING

SUBORDINATE OBJECT CLASS 	signRouteNePart;

NAMED BY

SUPERIOR OBJECT CLASS	signRouteSetNePart;

WITH ATTRIBUTE signRouteNePartId;

BEHAVIOUR signRouteSetNePart-signRouteNePartBehaviour BEHAVIOUR DEFINED AS

"A create request is rejected, if the signlinkSetTpPointer would reference an instance, which

i)	does not belong to the signLinkSetTp object class

OR	ii)	does not exist

OR	iii)	is not contained in the same mtpSignPoint as the signRouteNePart

OR	iv)	is already referenced by a signRouteNePart contained within the same signRouteSetNePart

A delete request is rejected if its administrativeState does not equal locked.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 3 };

signLinkSetTp-signLinkTp NAME BINDING

SUBORDINATE OBJECT CLASS	signLinkTp;

NAMED BY

SUPERIOR OBJECT CLASS	signLinkSetTp;

WITH ATTRIBUTE slCode;

BEHAVIOUR signLinkSetTp-signLinkTpBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 4 };

mtpSignPoint-mtpAccessPoint NAME BINDING

SUBORDINATE OBJECT CLASS	mtpAccessPoint;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE	"ITU-T Rec. X.723 (1993) | ISO/IEC 10165-5 : 1993":sapId;

BEHAVIOUR mtpSignPoint-mtpAccessPointBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 5 };

mtpSignPoint-mtpLevel3 NAME BINDING

SUBORDINATE OBJECT CLASS	mtpLevel3;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE mtpLevel3Id;

BEHAVIOUR mtpSignPoint-mtpLevel3Behaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 6 };

mtpSignPoint-signDataLinkTp NAME BINDING

SUBORDINATE OBJECT CLASS	signDataLinkTp;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE signDataLinkTpld;

BEHAVIOUR mtpSignPoint-signDataLinkTpBehaviour BEHAVIOUR DEFINED AS

"A create request is rejected if the equipmentPointer would reference equipment that does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

WITH ATTRIBUTEsignDataLinkId;
REGISTERED AS { mtpNameBinding 7 };

mtpSignPoint-signLinkSetTp NAME BINDING

SUBORDINATE OBJECT CLASS	signLinkSetTp;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE signLinkSetId;
	BEHAVIOUR mtpSignPoint-signLinkSetTpBehaviour BEHAVIOUR DEFINED AS

"A creation request is only successful if a signRouteSetNePart exists within the mtpSignPoint for the value of attribute adjPC

In case the lsTimersProfilePointerPackage is used, create or set request is rejected, if

i)	the lsTimersProfilePointer does not reference an lsTimersProfile

OR	ii)	the lsTimersProfilePointer would reference an instance which does not exist

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If it is tried to delete a signLinkSetTp, which is referenced by at least one signRouteNePart, the deletion of the signLinkSetTp is rejected and not carried out.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 8 };

managedSwitchingElement-signTerm NAME BINDING

SUBORDINATE OBJECT CLASS	signTerm;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE signTermId;

BEHAVIOUR managedSwitchingElement-signTermBehaviour BEHAVIOUR DEFINED AS

"A create request is rejected if the equipmentPointer would reference equipment that does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 9 };

mtpSignPoint-stpScreeningTable NAME BINDING

SUBORDINATE OBJECT CLASS	stpScreeningTable;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE stpScreeningTableId;

BEHAVIOUR mtpSignPoint-stpScreeningTableBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

It is only allowed to delete an instance of this object class if its administrative state equals 'locked'.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 10 };

managedSwitchingElement-lsTimersProfile NAME BINDING

SUBORDINATE OBJECT CLASS 	lsTimersProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE		lsTimersProfileId;

BEHAVIOUR managedSwitchingElement-lsTimersProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an lsTimersProfile which is still referenced by a signLinkSetTp the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 11 };

managedSwitchingElement-mtpL2ProtocolProfile NAME BINDING

SUBORDINATE OBJECT CLASS	mtpL2ProtocolProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE mtpL2ProtocolProfileId;

BEHAVIOUR managedSwitchingElement-mtpL2ProtocolProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an mtpL2ProtocolProfile which is still referenced by a signLinkTP the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 12 };

managedSwitchingElement-slTimersProfile NAME BINDING

SUBORDINATE OBJECT CLASS	slTimersProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE		slTimersProfile Id;

BEHAVIOUR managedSwitchingElement-slTimersProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is 	
already used by another instance of the same object class will be rejected.

If an attempt is made to delete an slTimersProfile which is still referenced by a signLinkTp the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 13 };

managedSwitchingElement-spTimersProfile NAME BINDING

SUBORDINATE OBJECT CLASS	spTimersProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE	spTimersProfileId;

BEHAVIOUR managedSwitchingElement-spTimersProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an spTimersProfile which is still referenced by an mtpSignPoint the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 14 };

stpScreeningTable-stpScreeningTableLine NAME BINDING

SUBORDINATE OBJECT CLASS	stpScreeningTableLine;

NAMED BY

SUPERIOR OBJECT CLASS	stpScreeningTable;

WITH ATTRIBUTE stpScreeningTableLineId;

BEHAVIOUR stpScreeningTable-stpScreeningTableLineBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected. The total number of screened DPC's across all stpScreeningTableLine instances contained by an stpScreeningTable must be less or equal to maxNoOfScreenedDpcs. A respectively unallowed create request is rejected. If one tries to create an instance with an stpScreeningTableLineId which would represent a linkset which is not contained within the same signPoint as the stpScreeningTable, the create request is rejected. Instances of this object class can be deleted automatically if the superior stpScreeningTable is deleted.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 15 };

## 7.8	Abstract syntax productions

-- The syntax of this ASN.1 is based on Recommendation X.208. However, from Recommendation X.680,

-- the "ellipsis notation" is used.

MTPDefinedTypesModule

{itu-t(0) recommendation q(17) omap(751) mtp(1) informationModel(0) asn1Modules(2)

mtpDefinedTypesModule(0)}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

EXPORTS EVERYTHING

IMPORTS

AdministrativeState, SimpleNameType, ProbableCause, SpecificProblems, PerceivedSeverity,

NotificationIdentifier, CorrelatedNotifications, AdditionalText, AdditionalInformation FROM Attribute-

ASN1Module

{joint-iso-itu-t ms(9) smi(3) part2(2) asn1Module(2) 1}

NameType, Pointer, ObjectList, PointerOrNull FROM ASN1DefinedTypesModule

{itu-t recommendation m gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0)}

ObjectInstance FROM CMIP-1

{joint-iso-itu-t ms(9) cmip(1) modules(0) protocol(3)};

mtpInformationModel OBJECT IDENTIFIER ::= {itu-t recommendation q(17) omap(751) mtp(1) informationModel(0)}

mtpObjectClass OBJECT IDENTIFIER ::= {mtpInformationModel managedObjectClass(3)}

mtpPackage OBJECT IDENTIFIER ::= {mtpInformationModel package(4)}

mtpParameter OBJECT IDENTIFIER ::= {mtpInformationModel parameter(5)}

mtpAttribute OBJECT IDENTIFIER ::= {mtpInformationModel attribute(7)}

mtpNameBinding OBJECT IDENTIFIER ::= {mtpInformationModel nameBinding(6)}

mtpAction OBJECT IDENTIFIER ::= {mtpInformationModel action(9)}

mtpNotification OBJECT IDENTIFIER ::= {mtpInformationModel notification(10)}

mtpSpecificExtensions OBJECT IDENTIFIER ::= { mtpInformationModel specificExtensions(0) }

AdditionalName ::= IA5String -- the size of this string is implementation dependent

administrativeStateLocked	AdministrativeState ::= locked

AlternativeSLCodeList ::= SEQUENCE SIZE (0..15) OF SLCode

CapacityLink ::= INTEGER (0..maxOctetsPerSecondLink)

CapacityLS ::= INTEGER (0..maxOctetsPerSecondLS)

ChangeInLsToAdjSpInfo ::= SEQUENCE {

oldSignLinkSetTp NameType,

newSignLinkSetTp NameType }

Cic ::= INTEGER (0..4095)

CongestedState ::= ENUMERATED {

notCongested	(0),

congested	(1)}

CongestionControlMethod ::= ENUMERATED {

unknown	(0),

ccmQ704International	(1),

ccmQ704NationalWithPrio	(2),

ccmQ704NationalNoPrioBufferMechanism	(3),

ccmQ704NationalNoPrioTimerMechanism	(4)}

CongestionCounting ::= ENUMERATED {

octet	(0),

message	(1)}

CongestionLevel ::= ENUMERATED {

none	(0),

congestionLevel1	(1),

congestionLevel2	(2),

congestionLevel3	(3)}

CongestionReportingBaseObject ::= ENUMERATED {

routeSet	(0),

linkSetOfCongestedRouteSet	(1),

linkOfCongestedRouteSet	(2),

congestedLinkOfCongestedRoute	(3)}

DpcComponent ::= CHOICE {

dpc	[0]	PointCode,

dpcRange	[1]	SEQUENCE {

firstDPC	PointCode,

lastDPC	PointCode }}

ErrorCorrectionMethod ::= ENUMERATED

{basicRetr	(0),

prevCyclRetr	(1) }

EventsCongestionLevel ::= SEQUENCE {

events	INTEGER,

conglevel	CongestionLevel }

EventsUP ::= SEQUENCE {

events	INTEGER,

userpart	UserPart}

FixedPriority ::= Priority

FlexiblePriority ::= Priority

InaccessibleSpInfo ::= SEQUENCE {

networkIndicator	NetworkIndicator,

pointCode		PointCode }

InClsLoadsharingAlgorithm ::= SET OF SlsLinkPriorityList

InfoRequest ::= BITSTRING {

pointCode	(0),

pointCodeList 	(1),

routePriorityList	(2),

...}

InitialLevelIfCongested ::= INTEGER (1..3) -- This equals the value s from B.1.2.9.2

InLsLoadShareAlgorithm ::= OBJECT IDENTIFIER

L2TimerT1 ::= INTEGER (13000..600000)

-- time, in milliseconds

L2TimerT2 ::= INTEGER (5000..150000)

-- time, in milliseconds

L2TimerT3 ::= INTEGER (1000..14000)

-- time, in milliseconds

L2TimerT4E ::= INTEGER (400..8000)

-- time, in milliseconds

L2TimerT4N ::= INTEGER (7500..120000)

-- time, in milliseconds

L2TimerT5 ::= INTEGER (80..120)

-- time, in milliseconds

L2TimerT6 ::= INTEGER (3000..12000)

-- time, in milliseconds

L2TimerT7 ::= INTEGER (500..6000)

-- time, in milliseconds

LinkSetPriorityListSntx ::= SEQUENCE OF NameType

LinkTPStatus ::= BIT STRING {

localBlocked	(0),

remoteBlocked	(1),

localInhibited	(2),

remoteInhibited	(3),

failed	(4),

deactivated	(5)}

LoadsharingInformationRouteNePart ::= IA5String

-- the size of this string is implementation dependent

LoadsharingInformationRouteSetNePart ::= IA5String

-- the size of this string is implementation dependent

LocalUpUnavailable ::= UserPart

LoopDelay ::= INTEGER

-- time, in milliseconds

ManagementExtension ::= SEQUENCE {

identifier	OBJECT IDENTIFIER,

significance	[1]	BOOLEAN DEFAULT FALSE,

information	[2]	ANY DEFINED BY identifier }

maxLinksInLs INTEGER :: = 16

MaxMSUsRetransN1 ::= INTEGER (0..127)

-- lower range is implementation dependent

MaxMSUsRetransN2 ::= INTEGER

maxNoOfScreenedDpcs INTEGER ::= 128

-- This value is suggested by clause 8/Q.705. It is likely that this value is too low for international networks

maxNoOfScreenedOpcs INTEGER ::= 128

-- This value is suggested by clause 8/Q.705. It is likely that this value is too low for international networks

MaxNumOfTests ::= INTEGER

maxOctetsPerSecondLink INTEGER ::= 8000

maxOctetsPerSecondLS INTEGER ::= 128000

MaxOctRetransN2 ::= INTEGER

MRVTActionResult ::= CHOICE {

resultOfMRTVT	[0]	ResultOfMRVT,

reasonForRejection	[1]	ReasonForRejection}

MRVTFailureString ::= BIT STRING {

detectedLoop	(0),

excessiveLengthRoute	(1),

unknownDestination	(2),

routeInaccessible	(3),

processingFailure	(4),

unknownInitiatingSP	(5),

timerExpired	(6),

spNotAnSTP	(7),

indirectRoute	(8),

maxNrMRVTestsAlready	(9), -- Info from adjacent SP

... }

MtpRestarting ::= PointCode

MtpRouteVerifTestResultSntx ::= SEQUENCE {

testedDestination	PointCode,	-- 2.2.2.3 b)/Q.753

routeTestResult	ResultOfMRVT,	-- 2.2.2.3 c)/Q.753

failureType		[1] MRVTFailureString OPTIONAL,  -- 2.1.1.3.1/Q.754

routeTrace		[2] RouteTrace OPTIONAL,	-- 2.2.2.3 d)/Q.753

pointCodeInfo	[3] PointCode OPTIONAL,
	pointCodeListInfo	[4] PointCodeList OPTIONAL,
	routePriorityListInfo	[5] RoutePriorityListInfo OPTIONAL,
	copyData		[6] OCTET STRING OPTIONAL  -- 2.2.2.3 e)/Q.753

}

NetworkIndicator ::= ENUMERATED{

internationalNetwork1	(0),

internationalNetwork2	(1),

nationalNetwork 1	(2)

nationalNetwork 2	(3)}

NumCongStates ::= INTEGER (0..3)

NumberOfNormallyActiveSignLinksTps ::= INTEGER (0..16)

NumberOfThresholdLevels ::= INTEGER (2 | 3)

PeriodicLinkTestFlag ::= ENUMERATED {

on	(0),

off	(1)}

periodicTestFlagDefault PeriodicLinkTestFlag ::= off

PointCode ::= INTEGER

PointCodeList ::= SEQUENCE OF PointCode

PointerList ::= SEQUENCE OF Pointer

Priority ::= INTEGER

-- maximum value is implementation dependent

PriorityInfo ::= INTEGER {

unknown 	(0),

firstChoice 	(1),

secondChoice 	(2),

thirdChoice 	(3),

...} (0..255)

PriorityMode ::= ENUMERATED {

insert	(0),

equal	(1),

exchangeSingle	(2),

exchangeGroup	(3),

priorityModeDefault PriorityMode ::= insert

localUserPartUnavailable ProbableCause ::= globalValue: {mtpSpecificExtensions 001}

q752Item1p02 ProbableCause ::= globalValue: {mtpSpecificExtensions 102}

q752Item1p10 ProbableCause ::= globalValue: {mtpSpecificExtensions 110}

q752Item2p10 ProbableCause ::= globalValue: {mtpSpecificExtensions 210}

q752Item2p16 ProbableCause ::= globalValue: {mtpSpecificExtensions 216}

q752Item2p18 ProbableCause ::= globalValue: {mtpSpecificExtensions 218}

q752Item3p06 ProbableCause ::= globalValue: {mtpSpecificExtensions 306}

q752Item3p11 ProbableCause ::= globalValue: {mtpSpecificExtensions 311}

q752Item4p03 ProbableCause ::= globalValue: {mtpSpecificExtensions 403}

q752Item4p05 ProbableCause ::= globalValue: {mtpSpecificExtensions 405}

q752Item4p11 ProbableCause ::= globalValue: {mtpSpecificExtensions 411}

q752Item4p13 ProbableCause ::= globalValue: {mtpSpecificExtensions 413}

q752Item5p01 ProbableCause ::= globalValue: {mtpSpecificExtensions 501}

q752Item5p05 ProbableCause ::= globalValue: {mtpSpecificExtensions 505}

q752Item5p06 ProbableCause ::= globalValue: {mtpSpecificExtensions 506}

q752Item5p07 ProbableCause ::= globalValue: {mtpSpecificExtensions 507}

q752Item5p08 ProbableCause ::= globalValue: {mtpSpecificExtensions 508}

remoteUserPartUnavailable ProbableCause:: = globalValue: {mtpSpecificExtensions 002}

abnormalFIBRorBSNR SpecificProblems ::= {{mtpSpecificExtensions 003}}

excessiveAckDelay SpecificProblems ::= {{mtpSpecificExtensions 004}}

excessiveCongDuration SpecificProblems ::= {{mtpSpecificExtensions 006}}

excessiveErrorRate SpecificProblems ::= {{mtpSpecificExtensions 005}}

remoteUPInaccessible SpecificProblems ::= {{mtpSpecificExtensions 009}}

remoteUPUnequipped SpecificProblems ::= {{mtpSpecificExtensions 008}}

remoteUPUnknown SpecificProblems ::= {{mtpSpecificExtensions 007}}

ReasonForRejection ::= ENUMERATED {

routeTestInstanceLocked	(10),

routeTestInstanceDisabled	(11),

routeTestInstanceBusy	(12),

routeTestAlreadyInProgress	(13)}

ReceiveCongestionThresholdAbatement ::= INTEGER

ReceiveCongestionThresholdOnset ::= INTEGER

RelatedLinkGroupNumber ::= INTEGER (0..15)

RemoteExchangeLabel ::= IA5String

-- the size of this string is implementation dependent

RemoteUpUnavailable ::= SEQUENCE {

pointCode	[0]		PointCode,

userPart	[1]		UserPart }

ResultMtpRouteTest ::= SEQUENCE {

testedDestination	PointCode,

routeTestActionResult	MRVTActionResult,

failureType	[1]	MRVTFailureString OPTIONAL,

...}

ResultOfAction ::= ENUMERATED {

success	(0),

unsuccessful	(1),

notPerformed	(2),

linkAlreadyInhibited	(3),

linkNotInhibited	(4),

wrongClassReferenced	(5),

signTermNonExisting	(6),

signTermAlreadyReferenced	(7),

wrongSignLinkTpStatus	(8),

...}

ResultOfMRVT ::= ENUMERATED {

success 	(0),

partialSuccess	(1), -- 2.1.1.3.2/Q.754, Partial Success

failure 	(2), -- 2.1.1.3.1/Q.754, Failure

}

ReturnUnknownParams ::= BIT STRING {

tag15	(0),

tag16	(1),

...}

RoutePriorityListInfo ::= SEQUENCE OF PriorityInfo

RouteSetUnavailable ::= OBJECT IDENTIFIER

RouteTrace ::= CHOICE {

success	[0] IMPLICIT PointCodeList,

detectedLoop	[1] IMPLICIT PointCodeList,

excessiveLengthRoute	[2] IMPLICIT PointCodeList,

unknownDestination	[3] IMPLICIT NULL,

routeInaccessible	[4] IMPLICIT PointCodeList,

processingFailure	[5] IMPLICIT NULL,

unknownInitiatingSP	[6] IMPLICIT PointCode,

timerExpired	[7] IMPLICIT PointCodeList,

spNotAnSTP	[8] IMPLICIT PointCodeList,

indirectRoute	[9] IMPLICIT PointCode}

ScreenedOpcsOrLinkSetsList ::= SET SIZE (0..maxNoOfScreenedOpcs) OF

SEQUENCE {

screenedOpcOrLinkset	StpScreeningTableLineId,

messageTreatment	TreatmentOf }

ScreeningByLinkSetOrByOpc ::= ENUMERATED {

byOpc	(0),

byIncomingLinkset	(1)

}

Seconds ::= INTEGER

SignLinkSetTpId ::= NameType

SIOType ::= SEQUENCE {

service ENUMERATED {

signallingNetworkManagement	(0),

signallingNetworkTesting	(1),

sccp	(3),

tup	(4),

isup	(5),

dup	(6),

mt	(8),

b-isup	(9),

siup	(10),

...} ,

subService ENUMERATED {

internationalNetwork	(0),

nationalNetwork	(2),

...}

}

SignRouteSetNePartName ::= IA5String

-- the size of this string is implementation dependent

SlCode ::= INTEGER (0..15)

Sls ::= INTEGER (0..15)

SLSCodeCurrentList ::= SlsCodeList

SlsCodeList ::= SET SIZE (0..16) OF Sls

-- Each Sls value can occur at most once in a given SET

SlsCodeNormalList ::= SlsCodeList

SlsLinkPriorityList ::= SEQUENCE {

slsCode	[0]	Sls,

normalSlcCode	[1]	SlCode,

alternativeSlCodeList	[2]	AlternativeSlCodeList

}

StmChannel ::= INTEGER (1..31)

SpecificDpc ::= PointCode

SpType ::= ENUMERATED

{sep	(0),

step	(1),

stp	(2)}

SS7OnOccEventInfo ::= SEQUENCE{

probableCause	ProbableCause,

specificProblems	[1]	SpecificProblems OPTIONAL,

perceivedSeverity	PerceivedSeverity,

notificationIdentifier	[5]	NotificationIdentifier OPTIONAL,

correlatedNotifications	[6]	CorrelatedNotifications OPTIONAL,

additionalText	AdditionalText OPTIONAL,

additionalInformation	[10]	AdditionalInformation OPTIONAL }

StartMtpRouteTest ::= SEQUENCE {

testDestination	PointCode,

traceRequested	BOOLEAN,

threshold	ThresholdN,

infoRequest	[0]	InfoRequest OPTIONAL,

returnUnknownParams	[1]	ReturnUnknownParams OPTIONAL,

directRouteCheck	[2]	BOOLEAN OPTIONAL}

StpScreeningElement ::= SEQUENCE{

designatedDPC	DpcComponent,

messageTreatment	TreatmentOf }

StpScreeningTableLineContent ::= SET SIZE (1..maxNoOfScreenedDpcs) OF StpScreeningElement

StpScreeningTableLineId ::= CHOICE {

designatedLinkSet	[0]	SignLinkSetTpId,

designatedOPC	[1]	PointCode}

StpScreeningTableListMode ::= BOOLEAN

SuermNSU ::= INTEGER (0..256)

-- lower range is implementation dependent

SuermThresh ::= INTEGER (32 | 64) -- (minSuermThresh or maxSuermThres)

TestResult ::= BIT STRING

{detectedLoop	(0),

excessiveLengthRoute	(1),

unknownDestination	(2),

routeInaccessible	(3),

processingFailure	(4),

unknownInitiatingSP	(5),

timerExpired	(6),

spNotAnSTP	(7),

indirectRoute	(8),

maxNrMRVTestsAlready	(9),

...}

ThresholdN ::= INTEGER

TimeD ::= INTEGER

TimerValue ::= INTEGER (0..360000)

TransmissionCongestionThresholdAbatementLevel1 ::= INTEGER

TransmissionCongestionThresholdAbatementLevel2 ::= INTEGER

TransmissionCongestionThresholdAbatementLevel3 ::= INTEGER

TransmissionCongestionThresholdDiscardLevel1 ::= INTEGER

TransmissionCongestionThresholdDiscardLevel2 ::= INTEGER

TransmissionCongestionThresholdDiscardLevel3 ::= INTEGER

TransmissionCongestionThresholdOnsetLevel1 ::= INTEGER

TransmissionCongestionThresholdOnsetLevel2 ::= INTEGER

TransmissionCongestionThresholdOnsetLevel3 ::= INTEGER

TransmissionRate ::= ENUMERATED {

kbits4point8	(0),

kbits56	(1),

kbits64	(2)}

TreatmentOf ::= ENUMERATED {

normal	(0),

discard	(1) }

UserPart ::= ENUMERATED

{ isup	(5),

sccp	 	(3),

tup		(4),

dup		(6),

mt		(8),

b-isup	(9),

siup		(10),
	...}

END -- end of MTPDefinedTypesModule

## E.3	Managed Object Class definitions

Please pay attention to the fact, that the modelling of measurements 3.6, 3.9, 4.5, 4.6, 5.1 and 5.4 of Q.752 is for further study.

The following managed object class definitions are given in alphabetical order.

### E.3.1	All 30 Minutes Signalling Link Duration Data

all30MinSignallingLinkDurationData MANAGED OBJECT CLASS
	DERIVED FROM oblSignallingLinkDurationData;
	CHARACTERIZED BY all30MinSignallingLinkDurationDataPackage PACKAGE
		BEHAVIOUR all30MinSignallingLinkDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link duration data with
			granularityPeriod 30 minutes. The attributes are read-only.";;

ATTRIBUTES

slLocalInhibition	GET SET BY CREATE, -- Measurement Q.752/2.5
			slRemoteInhibition	GET SET BY CREATE, -- Measurement Q.752/2.6
			slFailed	GET SET BY CREATE, -- Measurement Q.752/2.7
			slRemoteProcOutage	GET SET BY CREATE;;; -- Measurement Q.752/2.9
REGISTERED AS { mtpObjectClass 17 };

### E.3.2	All 5 And 30 Minutes Signalling Link Duration Data

all5And30MinSignallingLinkDurationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY all5And30MinSignallingLinkDurationDataPackage PACKAGE
		BEHAVIOUR all5And30MinSignallingLinkDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link duration data with 
			preferred granularityPeriod 5 and 30 minutes. The attributes are read-only.";;

ATTRIBUTES

localMgntInhibit	GET SET BY CREATE, -- Measurement Q.752/2.13
			localMgntUninhibit	GET SET BY CREATE, -- Measurement Q.752/2.14
			localBusy	GET SET BY CREATE;;; -- Measurement Q.752/2.15
REGISTERED AS { mtpObjectClass 18 };

### E.3.3	All Signalling Link Utilization Data

allSignallingLinkUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM oblSignallingLinkUtilizationData;
	CHARACTERIZED BY allSignallingLinkUtilizationDataPackage PACKAGE
		BEHAVIOUR allSignallingLinkUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link utilization data. The preferred
			granularityPeriods are 5 and 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			slAlignment	GET SET BY CREATE, -- Measurement Q.752/1.7
			negAckReceived	GET SET BY CREATE, -- Measurement Q.752/1.9
			retransmittedOctets	GET SET BY CREATE, -- Measurement Q.752/3.2
			transmittedMSUs	GET SET BY CREATE, -- Measurement Q.752/3.3
			receivedMSUs	GET SET BY CREATE;;; -- Measurement Q.752/3.5
	CONDITIONAL PACKAGES

localChangeOversPackage
	PRESENT IF "the granularity period is 30 minutes";
REGISTERED AS { mtpObjectClass 19 };

### E.3.4	All Signalling Point Utilization Data

allSignallingPointUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM oblSignallingPointUtilizationData;
	CHARACTERIZED BY allSignallingPointUtilizationDataPackage PACKAGE
		BEHAVIOUR allSignallingPointUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling point utilization data. The preferred
			granularityPeriods are 5 or 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			adjacentInaccessibleEvents	GET SET BY CREATE;;; -- Measurement Q.752/5.1
				--Modelling of Measurement Q.752/5.1 is for further study

REGISTERED AS { mtpObjectClass 20 };

### E.3.5	Handled MSUs per OPC/DPC/SIO

handledMSUsOpcDpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY handledMSUsOpcDpcSioMeasurementPackage PACKAGE
		BEHAVIOUR handledMSUsOpcDpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC/DPC/SIO combination.
The preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements
should be limited to a small number of OPC/DPC combinations at a given time. The measurement attribute is read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			dpc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			handledMSUsOpcDpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.7
REGISTERED AS { mtpObjectClass 21 };

### E.3.6	Handled Octets per OPC/DPC/SIO

handledOctetsOpcDpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY handledOctetsOpcDpcSioMeasurementPackage PACKAGE
		BEHAVIOUR handledOctetsOpcDpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC/DPC/SIO combination. The preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements should be limited to a small number of OPC/DPC combinations at a given time. The attribute is read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			dpc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			handledOctetsOpcDpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.6
REGISTERED AS { mtpObjectClass 22 };

### E.3.7	Handled Octets per SIO

handledOctetsSIOMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY handledOctetsSIOMeasurementPackage PACKAGE
		BEHAVIOUR handledOctetsSIOMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific SIO. The preferred
			granularityPeriods are 5 and 30 minutes. The attribute is read-only.";;

ATTRIBUTES
			sio	GET SET BY CREATE,
			handledOctetsSIO	GET SET BY CREATE;;; -- Measurement Q.752/6.3
REGISTERED AS { mtpObjectClass 23 };

### E.3.8	Obligatory Signalling Link Duration Data

oblSignallingLinkDurationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY oblSignallingLinkDurationDataPackage PACKAGE
		BEHAVIOUR oblSignallingLinkDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains the obligatory signalling link duration data. The
			preferred granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			slUnavailabilityDuration	GET SET BY CREATE, -- Measurement Q.752/2.1
			slInServiceDuration	GET SET BY CREATE;;; -- Measurement Q.752/1.1
REGISTERED AS { mtpObjectClass 24 };

### E.3.9	Obligatory Signalling Link Utilization Data

oblSignallingLinkUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY oblSignallingLinkUtilizationDataPackage PACKAGE
		BEHAVIOUR oblSignallingLinkUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all obligatory signalling link utilization data. The
			granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			signUnitsReceived	GET SET BY CREATE, -- Measurement Q.752/1.8
			transmittedOctetsSIFSIO	GET SET BY CREATE, -- Measurement Q.752/3.1
			receivedOctetsSIFSIO	GET SET BY CREATE, -- Measurement Q.752/3.4
			discardedMSUs	GET SET BY CREATE;;; -- Measurement Q.752/3.10
REGISTERED AS { mtpObjectClass 25 };

### E.3.10	Obligatory Signalling Point Utilization Data

oblSignallingPointUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY oblSignallingPointUtilizationDataPackage PACKAGE
		BEHAVIOUR oblSignallingPointUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all obligatory signalling point utilization data. The
		preferred granularityPeriods are 5 minutes. The attributes are read-only.";;

ATTRIBUTES
			adjacentInaccessibleDuration	GET SET BY CREATE;;; -- Measurement Q.752/5.2
REGISTERED AS { mtpObjectClass 26 };

### E.3.11	Received Octets per OPC

receivedOctetsOPCMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY receivedOctetsOPCMeasurementPackage PACKAGE

BEHAVIOUR receivedOctetsOPCMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC. The preferred
			granularityPeriods are 5 and 30 minutes. Activation of these measurements should be
			limited to a small number of signalling point codes at a given time. The attribute is
			read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			receivedOctetsOPC	GET SET BY CREATE;;; -- Measurement Q.752/6.1
REGISTERED AS { mtpObjectClass 27 };

### E.3.12	Received Octets per OPC/SIO

receivedOctetsOpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY receivedOctetsOpcSioMeasurementPackage PACKAGE
		BEHAVIOUR receivedOctetsOpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC/SIO combination. The	
			preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements should

be limited to a small number of signalling point codes at a given time. The attribute is
			read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			receivedOctetsOpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.4
REGISTERED AS { mtpObjectClass 28 };

### E.3.13	Route Set Information Data

routeSetInformationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY routeSetInformationDataPackage PACKAGE
		BEHAVIOUR routeSetInformationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all permanent signalling point duration data. The
			granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			routeSetUnavailable	GET SET BY CREATE, -- Measurement Q.752/4.9
			routeSetUnavailableDuration	GET SET BY CREATE;;; -- Measurement Q.752/4.10
REGISTERED AS { mtpObjectClass 29 };

### E.3.14	Signalling Link Congestion Data

signallingLinkCongestionData MANAGED OBJECT CLASS
	DERIVED FROM ss7FirstAndIntervalCurrentData;
	CHARACTERIZED BY signallingLinkCongestionDataPackage PACKAGE
		BEHAVIOUR signallingLinkCongestionDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains the optional signalling link congestion data. The preferred granularityPeriods are 5 and 30 minutes. The attributes slCongestedStarts, slCongestionStops and congestionEventsMSULoss can be referenced from an instance of ss7FirstAndIntervalThresholdData when they are used for a first and interval measurement. The attributes are read-only.";;

ATTRIBUTES
			slCongestedStarts	GET SET BY CREATE, -- Measurement Q.752/3.6
				-- Modelling of Measurement Q.752/3.6 is for further study

slCongestionStops	GET SET BY CREATE, -- Measurement Q.752/3.9

-- Measurement 3.9 was meanwhile cancelled in Q.752!
			congestionEventsMSULoss	GET SET BY CREATE;;; -- Measurement Q.752/3.11

CONDITIONAL PACKAGES
	slCongestedDurationPackage PRESENT IF "the granularity period is 30 minutes";
REGISTERED AS { mtpObjectClass 30 };

### E.3.15	Signalling Link Set Duration Data

signallingLinkSetDurationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY signallingLinkSetDurationDataPackage PACKAGE
		BEHAVIOUR signallingLinkSetDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link set duration data. The
granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			slsUnavailable	GET SET BY CREATE;;; -- Measurement Q.752/4.2
REGISTERED AS { mtpObjectClass 31 };

### E.3.16	Signalling Point Data

signallingPointData MANAGED OBJECT CLASS
	DERIVED FROM ss7FirstAndIntervalCurrentData;

CHARACTERIZED BY signallingPointDataPackage PACKAGE
		BEHAVIOUR signallingPointDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains information about signalling point. The preferred
granularityPeriods are 5 or 30 minutes. All attributes are read-only and are referenced by an instance of ss7FirstAndIntervalThresholdData managed object class so does discardedMSUs attribute when it is used as a first and interval measurement.";;

ATTRIBUTES
			discardedMSUs	GET SET BY CREATE, -- Measurement Q.752/5.5
			transmittedUPUnavailable	GET SET BY CREATE, -- Measurement Q.752/5.6
			receivedUPUnavailable	GET SET BY CREATE, -- Measurement Q.752/5.7
			receivedTFC	GET SET BY CREATE;;; -- Measurement Q.752/5.8
REGISTERED AS { mtpObjectClass 32 };

### E.3.17	SS No. 7 Current Data

ss7CurrentData MANAGED OBJECT CLASS
	DERIVED FROM "ITU-T Rec. Q.822 (1994)":currentData;
	CHARACTERIZED BY ss7CurrentDataPackage PACKAGE
		BEHAVIOUR ss7CurrentDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class is used for subtyping of Q.752 duration measurements.
Subtypes will have to contain one or more attributes that can be scanned.";;;;

REGISTERED AS { mtpObjectClass 33 };

### E.3.18	SS No. 7 First and Interval Current Data

ss7FirstAndIntervalCurrentData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY ss7FirstAndIntervalCurrentDataPackage PACKAGE
		BEHAVIOUR ss7FirstAndIntervalCurrentDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class is used for subtyping of Q.752 first and interval measurements.
Subtypes will have to contain one or more attributes that can be scanned. The start of a
measurement can possibly generate an X.721:qualityOfServiceAlarm with an indication of the probable cause in an instance of another class.";;;;

REGISTERED AS { mtpObjectClass 34 };

### E.3.19	SS No. 7 First and Interval Threshold Data

ss7FirstAndIntervalThresholdData MANAGED OBJECT CLASS
	DERIVED FROM "ITU-T Rec. Q.822 (1994)": thresholdData;
	CHARACTERIZED BY
		"ITU-T Rec. Q.822 (1994)": counterThresholdListPackage,

ss7FirstAndIntervalThresholdDataPackage PACKAGE
		BEHAVIOUR ss7FirstAndIntervalThresholdDataBehaviour BEHAVIOUR DEFINED AS

"The conditional package counterThresholdListPackage is mandatory for this subclass. The
threshold values of the attributes in the counterThresholdAttributeList will be 1, indicating that the first event should be notified to the managed using a qualityOfServiceAlarm notification. The attributes being referenced will be part of instances of ss7FirstAndIntervalCurrentData or one of its subclasses.";;;;

REGISTERED AS { mtpObjectClass 35 };

### E.3.20	Transmitted Octets per DPC

transmittedOctetsDPCMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY transmittedOctetsDPCMeasurementPackage PACKAGE
		BEHAVIOUR transmittedOctetsDPCMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific DPC. The preferred
granularityPeriods are 5 and 30 minutes. Activation of these measurements should be
limited to a small number of signalling point codes at a given time. The attribute is read-only.";;

ATTRIBUTES
			dpc	GET SET BY CREATE,
			transmittedOctetsDPC	GET SET BY CREATE;;; -- Measurement Q.752/6.2
REGISTERED AS { mtpObjectClass 36 };

### E.3.21	Transmitted Octets per DPC/SIO

transmittedOctetsDpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY transmittedOctetsDpcSioMeasurementPackage PACKAGE
		BEHAVIOUR transmittedOctetsDpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific DPC/SIO combination. The
preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements should
be limited to a small number of signalling point codes at a given time. The attribute is
read-only.";;

ATTRIBUTES
			dpc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			transmittedOctetsDpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.5
REGISTERED AS { mtpObjectClass 37 };

## E.4	Package definitions

localChangeOversPackage PACKAGE
	ATTRIBUTES
		localChangeOvers	GET SET BY CREATE; -- Measurement Q.752/1.10
REGISTERED AS { mtpPackage 53 };

slCongestedDurationPackage PACKAGE
		ATTRIBUTES
			slCongestedDuration	GET SET BY CREATE; -- Measurement Q.752/3.7
REGISTERED AS { mtpPackage 54 };

## E.5	Attribute Definitions

adjacentInaccessibleDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR adjacentInaccessibleDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.2.";;

REGISTERED AS { mtpAttribute 135 };

adjacentInaccessibleEvents ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR adjacentInaccessibleEventsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.1.";;

-- Modelling of Measurement Q.752/3.6 is for further study

REGISTERED AS { mtpAttribute 136 };

congestionEventsMSULoss ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR congestionEventsMSULossBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.11.";;

REGISTERED AS { mtpAttribute 137 };

discardedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR discardedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.10 and Q.752/5.5.";;

REGISTERED AS { mtpAttribute 138 };

handledOctetsOpcDpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR handledOctetsOpcDpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.6.";;

REGISTERED AS { mtpAttribute 139 };

handledOctetsSIO ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR handledOctetsSIOBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.3.";;

REGISTERED AS { mtpAttribute 140 };

handledMSUsOpcDpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR handledMSUsOpcDpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.7.";;

REGISTERED AS { mtpAttribute 141 };

localBusy ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR localBusyBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.15. See also 9.3/Q.704.";;

REGISTERED AS { mtpAttribute 142 };

localChangeOvers ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR localChangeOversBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.10. See also 5/Q.704.";;

REGISTERED AS { mtpAttribute 143 };

localMgntInhibit ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Counter;
	MATCHES FOR EQUALITY;
	BEHAVIOUR localMgntInhibitBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.13. See also 10.2/Q.704.";;

REGISTERED AS { mtpAttribute 144 };

localMgntUninhibit ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Counter;
	MATCHES FOR EQUALITY;
	BEHAVIOUR localMgntUninhibitBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.14. See also 10.3/Q.704.";;

REGISTERED AS { mtpAttribute 145 };

negAckReceived ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR negAckReceivedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.9.";;

REGISTERED AS { mtpAttribute 146 };

receivedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.751/3.5.";;

REGISTERED AS { mtpAttribute 147 };

receivedOctetsOPC ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedOctetsOPCBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.1.";;

REGISTERED AS { mtpAttribute 148 };

receivedOctetsOpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedOctetsOpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.4.";;

REGISTERED AS { mtpAttribute 149 };

receivedOctetsSIFSIO ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedOctetsSIFSIOBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.751/3.4.";;

REGISTERED AS { mtpAttribute 150 };

receivedTFC ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.EventsCongestionLevel;
	BEHAVIOUR receivedTFCBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.8.";;

REGISTERED AS { mtpAttribute 151 };

receivedUPUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.EventsUP;
	BEHAVIOUR receivedUPUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.7. See also Recommendation Q.704/11.7.2.";;

REGISTERED AS { mtpAttribute 152 };

retransmittedOctets ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR retransmittedOctetsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.2. See also 5/Q.704.";;

REGISTERED AS { mtpAttribute 153 };

routeSetUnavailable ATTRIBUTE
	DERIVED FROM "ITU Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR routeSetUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.9. See also Recommendation Q.704/11.2.2.";;

REGISTERED AS { mtpAttribute 154 };

routeSetUnavailableDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR routeSetUnavailableDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.10. See also Recommendation Q.704/11.2.1.";;

REGISTERED AS { mtpAttribute 155 };

signUnitsReceived ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR signUnitsReceivedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.8. See also 4/Q.704.";;

REGISTERED AS { mtpAttribute 156 };

sio ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SIOType;
	MATCHES FOR EQUALITY;
REGISTERED AS { mtpAttribute 157 };

slAlignment ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR slAlignmentBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.7.";;

REGISTERED AS { mtpAttribute 158 };

slCongestedDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slCongestedDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.7.";;

REGISTERED AS { mtpAttribute 159 };

slCongestedStarts ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR slCongestedStartsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.6. See also 3.8/Q.704.";;

-- Modelling of Measurement Q.752/3.6 is for further study

REGISTERED AS { mtpAttribute 160 };

slCongestionStops ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR slCongestionStopsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.9. See also 3.8/Q.704.";;

REGISTERED AS { mtpAttribute 161 };

slFailed ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slFailedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.7. See also 3.2.2/Q.704.";;

REGISTERED AS { mtpAttribute 162 };

slInServiceDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slInServiceDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.1.";;

REGISTERED AS { mtpAttribute 163 };

slLocalInhibition ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slLocalInhibitionBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.5. See also 3.2.8/Q.704.";;

REGISTERED AS { mtpAttribute 164 };

slRemoteInhibition ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slRemoteInhibitionBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.6. See also 3.2.2/Q.704.";;

REGISTERED AS { mtpAttribute 165 };

slRemoteProcOutage ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slRemoteProcOutageBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.9. See also 3.2.6/Q.704.";;

REGISTERED AS { mtpAttribute 166 };

slsUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	BEHAVIOUR slsUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.2.";;

REGISTERED AS { mtpAttribute 167 };

slUnavailabilityDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slUnavailabilityDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.1.";;

REGISTERED AS { mtpAttribute 169 };

transmittedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.3.";;

REGISTERED AS { mtpAttribute 170 };

transmittedOctetsSIFSIO ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedOctetsSIFSIOBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.1. See also 2.3.8/Q.704.";;

REGISTERED AS { mtpAttribute 171 };

transmittedOctetsDPC ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedOctetsDPCBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.2.";;

REGISTERED AS { mtpAttribute 172 };

transmittedOctetsDpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedOctetsDpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.5.";;

REGISTERED AS { mtpAttribute 173 };

transmittedUPUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.EventsUP;
	BEHAVIOUR transmittedUPUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.6. See also Recommendation Q.704/11.7.2.";;

REGISTERED AS { mtpAttribute 174 };

## E.6	Name Binding Definitions

managedElement-ss7FirstAndIntervalThresholdData NAME BINDING
	SUBORDINATE OBJECT CLASS ss7FirstAndIntervalThresholdData;
	NAMED BY
	SUPERIOR OBJECT CLASS managedElement AND SUBCLASSES;
	WITH ATTRIBUTE "ITU-T Rec. Q.822 (1994)": thresholdDataId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 41};

mtpSignPoint-handledMSUsOpcDpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	handledMSUsOpcDpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 16 };

mtpSignPoint-handledOctetsOpcDpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	handledOctetsOpcDpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 17 };

mtpSignPoint-handledOctetsSIOMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	handledOctetsSIOMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 18 };

signRouteSetNePart-allSignallingPointUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS allSignallingPointUtilisationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS signRouteSetNePart;
	WITH ATTRIBUTE 		"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 40 };

signRouteSetNePartmtpSignPoint-oblSignallingPointUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS	oblSignallingPointUtilisationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS	signRouteSetNePart;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;

DELETE ;
REGISTERED AS { mtpNameBinding 19 };

mtpSignPoint-receivedOctetsOPCMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	receivedOctetsOPCMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 20 };

mtpSignPoint-receivedOctetsOpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	receivedOctetsOpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 21 };

routeSetNePart-routeSetInformationData NAME BINDING
	SUBORDINATE OBJECT CLASS	routeSetInformationData;
	NAMED BY
	SUPERIOR OBJECT CLASS	routeSetNePart;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 22 };

mtpSignPoint-signallingPointData NAME BINDING
	SUBORDINATE OBJECT CLASS	signallingPointData;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 23 };

mtpSignPoint-transmittedOctetsDPCMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	transmittedOctetsDPCMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 24 };

mtpSignPoint-transmittedOctetsDpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	transmittedOctetsDpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 25 };

signLinkSetTP-signallingLinkSetDurationData NAME BINDING
	SUBORDINATE OBJECT CLASS	signallingLinkSetDurationData;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkSetTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 26 };

signLinkTp-all5And30MinSignallingLinkDurationData NAME BINDING
	SUBORDINATE OBJECT CLASS	all5And30MinSignallingLinkDurationData;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 27 };

signLinkTp-oblSignallingLinkDurationData NAME BINDING
	SUBORDINATE OBJECT CLASS	oblSignallingLinkDurationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 28 };

signLinkTp-oblSignallingLinkUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS	oblSignallingLinkUtilisationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 29 };

signLinkTp-signallingLinkCongestionData NAME BINDING
	SUBORDINATE OBJECT CLASS	signallingLinkCongestionData;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 30 };

## E.7	ASN.1 productions

The ASN.1 productions for this annex can be found in 7.8.

### F.2.1	Managed object classes definitions

#### F.2.1.1	MTP Route Verification Test

mtpRouteVerifTest MANAGED OBJECT CLASS
	DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;
	CHARACTERIZED BY	mtpRouteTestPackage PACKAGE
		BEHAVIOUR	mtpRouteTestBehaviour BEHAVIOUR DEFINED AS

"The route verification test is used to determine whether the data of the MTP route tables throughout the network is consistent. The test is started from a certain SP by its OS and this SP will report the test result to its OS. The test checks whether a certain destination can be reached via all possible routes without loops and excessive length routes.

Administrative State:

–	Unlocked: The MRVT is allowed to perform its normal test functions. ACTIONS will be accepted to
start an MRVT for a certain destination.

–	Locked: The MRVT is not allowed to perform its normal functions. No ACTIONS will be accepted.

–	Shutting down: The usage of the MRVT is administratively limited to the running tests. ACTIONS to start a new MRVT will be rejected. If all running tests are finished, the administrative state becomes locked.

Operational State:

–	Enabled: When the MRVT is in the enabled operational state, it is fully operational.

–	Disabled: The MRVT is incapable of performing its normal function. ACTIONS to start an MRVT will be rejected.

Availability Status:

The supported values for this attribute are:

–	Dependency: The MRVT cannot operate because a server resource on which it depends is unavailable (e.g. OMASE, TCAP, SCCP). In this case the operational state is disabled.

–	Empty SET

Usage State:
			The supported values for this attribute are:
			–	Idle: No test is running.
			–	Active: One or more tests are running, and the instance has spare capacity to provide
				additional MRVTs simultaneously.
			–	Busy: The instance cannot provide an additional test, because the maximum number of 				tests
				that can be run simultaneously has been reached. The number of tests is limited by the 				value
				of the 'maxNumOfTests' attribute. ACTIONS to start an additional MRVT will be 					rejected.

			A change in the operational state shall cause a state change notification. A change in
			the administrative state or the usage state shall not emit a state change notification.";;

ATTRIBUTES

mtpRouteTestId			GET SET BY CREATE,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":administrativeState			GET-REPLACE,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":operationalState			GET,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":usageState			GET,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":availabilityStatus			GET,
			"ITU-T Rec. M.3100 (1992)":version			GET SET BY CREATE,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":providerObject			GET-REPLACE,
			maxNumOfTests			GET SET BY CREATE,
			timeD			GET SET BY CREATE;
		ACTIONS

startMtpRouteTest;
		NOTIFICATIONS
			mtpRouteTestResult,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the
attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

mtpRouteVerifTestNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 1 };

### F.2.2	Package definitions

mtpRouteVerifTestNamePackage PACKAGE
	ATTRIBUTES
		mtpRouteVerifTestName		GET-REPLACE;
REGISTERED AS { mtpPackage 1 };

### F.2.3	Attribute definitions

maxNumOfTests ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.MaxNumOfTests;

BEHAVIOUR maxNumOfTestsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the maximum number of simultaneous route tests at one SP.";;

REGISTERED AS { mtpAttribute 2 };

mtpRouteTestId ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;
	MATCHES FOR EQUALITY, SUBSTRINGS;
REGISTERED AS { mtpAttribute 3 };

mtpRouteVerifTestName ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;
	MATCHES FOR EQUALITY;
	BEHAVIOUR mtpRouteVerifTestNameBehaviour BEHAVIOUR DEFINED AS
		"This attribute is an additional name for instances of the mtpRouteVerifTest managed object
		class.";;
REGISTERED AS { mtpAttribute 4 };

timeD ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimeD;
	BEHAVIOUR timeDBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the maximum performance time D for an STP to process MRVT
		and MRVA messages.";;

REGISTERED AS { mtpAttribute 5 };

### F.2.4	Action definitions

startMtpRouteTest ACTION
	BEHAVIOUR
	startMtpRouteTestBehaviour BEHAVIOUR DEFINED AS

"This action is used by the Management Process to initiate an MRVT procedure. The MRVT
procedure sends an MRVT message for each signalling route which is contained in the MTP routing
tables to reach the test destination. The destination (DPC) of each of these messages is the adjacent
signalling point within the particular route under test. For each MRVT message a corresponding
MRVA message is expected.
When the MRVT procedure is initiated, a timer T1 is started.

The value of timer T1 depends on the value of the attribute 'timeD' and the value of 'threshold' given
in the action parameters (2.4/Q.753).
When all MRVA messages expected have been received or when T1 expires, the test is complete

and the result will be returned to the Management Process in the action reply.
The Action will be rejected, if
–	the administrative state is locked.
–	the operational state is disabled.
–	the usage state is busy.
–	an MRVT for the test destination given in the action parameters is already in progress.
The reason for the rejection will be returned to the Management Process in the action reply.";;

MODE CONFIRMED;
	WITH INFORMATION SYNTAX	MTPDefinedTypesModule.StartMtpRouteTest;
	WITH REPLY SYNTAX	MTPDefinedTypesModule.ResultMtpRouteTest;
REGISTERED AS { mtpAction 1 };

### F.2.5	Notification definitions

mtpRouteTestResult NOTIFICATION
	BEHAVIOUR	mtpRouteTestResultBehaviour BEHAVIOUR DEFINED AS

"The reception of an MRVR message regardless of whether or not the receiving SP was the initiator
causes this notification type. The notification includes the information contained in the received
MRVR message.";;

WITH INFORMATION SYNTAX MTPDefinedTypesModule.MtpRouteVerifTestResultSntx;

REGISTERED AS { mtpNotification 2 };

### F.2.6	Naming binding definitions

mtpSignPoint-mtpRouteVerifTest NAME BINDING
	SUBORDINATE OBJECT CLASS	mtpRouteVerifTest;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE mtpRouteTestId;
	CREATE;
	DELETE;
REGISTERED AS { mtpNameBinding 6 };

### F.2.7	Abstract syntax productions

The ASN.1 productions for this annex can be found in 7.8.

______________