| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 107-E June 1998 Original: English |

Question: 10/11

Texte disponible seulement enText available only inE

STUDY GROUP 11 – REPORT R 107

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (05/98) FOR RECOMMENDATION Q.771 (06/97)

________

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.771 - 06/97 - Signalling System No 7, Functional Description of Transaction Capabilities.

## 1.2	Background

This guide is a compilation of reported defects, their resolutions and minor upgrades to the 1997 edition of the ITU-T Recommendation Q.771. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the Guide

This guide records the resolutions of defects in the following categories:

•	editorial errors;

•	technical errors, such as omissions, inconsistencies etc.; and

•	ambiguities.

In addition, this guide records minor enhancement to the Recommendations in the following categories:

•	increased interoperability; and

•	deployment possibility in new environments.

## 1.4	Contacts

Editor Q.771 :	J. D. Humphrey		Tel:	+44 1202 853757
			GPT Limited			Fax:	+44 1202 853405
			Discovery Court		Email:	janeh@ncp.gpt.co.uk
			Canford House
			Wallisdown Road
			Poole, Dorset
			United Kingdom

# 2	Editorial Errors

None identified.

# 3	Technical Errors

None identified.

# 4	Ambiguities

Synopsis:

The TC Quality of Service Parameter contains the acceptable quality of Service parameters for the connectionless SCCP network service. Currently, the description of this parameter in Recommendation Q.771 implies that the parameter only consists of "Return Option" and "Sequence Control", this is an incorrect assumption. The intention was that this parameter should contain all information pertaining to Quality of Service for the connectionless SCCP network service and this would include "Return Option", "Sequence Control" and "Importance Indication". To correct this the following modifications should be applied:

Modification a): Section 3.1.2.1 Definition of parameters

For the parameter "Quality of Service" modify the definition as follows:

"–	"Quality of Service" - the TC-user indicates the acceptable Quality of Service. The "Quality of Service" parameters for the connectionless SCCP network service may contain, for example, "Return Option", "Sequence Control" and "Importance Indication".

Delete definitions of Return Option and Sequence Control."

Modification b)	Section 3.2.1 Overview of the Transaction sub-layer primitives

For the parameter "Quality of Service" modify the definition as follows:

"–	"Quality of Service" - the TC-user indicates the acceptable Quality of Service. The "Quality of Service" parameters for the connectionless SCCP network service may contain, for example, "Return Option", "Sequence Control" and "Importance Indication".

Delete definitions of Return Option and Sequence Control."

# 5	Increased interoperability

None.

# 6	Deployment possibility in new environments

Improved.

_____________