|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | H.323 System Implementors’ Guide |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (30 January 2004) |
|  | SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS Infrastructure of audiovisual services – Communication procedures |  |  |  |
|  | Implementors’ Guide for Recommendations of the H.323 System (“Packet-based multimedia communications systems”): H.323, H.225.0, H.245, H.246, H.283, H.235, H.341, H.450 Series, H.460 Series, and H.500 Series |  |  |  |

Summary

This document is a compilation of reported defects identified in the versions of ITU-T Recommendation H.323 and its related Recommendations currently in force. It must be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementers. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.323-series Recommendations.

This revision contains all updates submitted upto and including those at Study Group 16 meeting, January 2004, in Geneva (TD 46/PLEN). This Implementors’ Guide provides corrections and clarifications for implementations of the H.323-series Recommendations approved in May 2003.

Contact Information

| ITU-T Study Group 16 / Rapporteur Question 2/16 | Paul E. Jones Cisco Systems, Inc. 7025 Kit Creek Road Research Triangle Park, NC 27709. USA | Tel: +1 919 392 6948 Fax: +1 919 392 2177 E-mail: paulej@packetizer.com |
| --- | --- | --- |
| ITU-T Study Group 16 / Rapporteur Question 3/16 | Christian Groves Ericsson Australia Pty. Ltd. 37/360 Elizabeth Street Victoria 3000. Australia | Tel: +61 3 9301 6116 Fax: +61 3 9301 1499 E-mail: Christian.Groves@ericsson.com |
| ITU-T Study Group 16 / Rapporteur Question G/16 Editor ITU-T Rec. H.235 | Martin Euchner Siemens AG / ICN M NT 18 Hofmannstr. 51 D-81359 Muenchen. Germany | Tel: +49 89 722 5 57 90 Fax: +49 89 722 4 68 41 E-mail: martin.euchner@ties.itu.int |
| Editor ITU-T Rec. H.323 | Roni Even Polycom Israel Ltd. 94 Derech Em Hamoshavot P.O. Box 3654 Petach-Tikva 49130 Israel | Tel: +972 3 925 1200 Fax: +972 3 921 1571 E-mail: roni.even@polycom.co.il |
| Editor ITU-T Rec. H.341 | Craig Blasberg Cisco Systems, Inc. 7025 Kit Creek Road Research Triangle Park, NC 27709. USA | Tel: +1 919 392 5760 Fax: +1 919 392 6801 E-mail: blasberg@cisco.com |
| Editor ITU-T Rec. H.225.0 Editor ITU-T Rec. H.323 Series Implementors’ Guide | Vivek Bhargava Cisco Systems, Inc. 7025 Kit Creek Road Research Triangle Park, NC 27709. USA | Tel: +1 919 392 6823 Fax: +1 919 392 2177 E-mail: vbhargava@cisco.com |
| Editor ITU-T Rec. H.225.0 Annex G | Miner Gleason Cisco Systems, Inc. 7025 Kit Creek Road Research Triangle Park, NC 27709. USA | Tel: +1 919 392 8752 Fax: +1 919 392 7065 E-mail: mgleason@cisco.com |
| Editor ITU-T Rec. H.245 | Mike Nilsson BT Labs Ipswitch. United Kingdom | Tel: +44 1 473 645413 Fax: +44 1 473 643791 E-mail: mike.nilsson@bt.com |
| Editor ITU-T Rec. H.246 | Ogishi Hisataka | E-mail: ogishi.hisataka@lab.ntt.co.jp |
| ITU-T Study Group 16 / Rapporteur Question 5/16 & Editor ITU-T Rec. for H.450.{1,2,3,4,5, 6,9,10,11,12} | Ernst Horvath Siemens Austria Gudrunstrasse 11 A-1101 Vienna. Austria | Tel: +43 5 1707 45897 Fax: +43 5 1707 56992 E-mail: ernst.horvath@siemens.at |
| Editor ITU-T Rec. H.450.7 | Dave Walker SS8 Networks 135 Michael Cowpland Drive, Suite 200 Kanata, Ontario, K2M 2E9. Canada | Tel: +1 613 592 8450 Fax: +1 613 592 9634 E-mail: dwalker@ss8networks.com |
| Editor ITU-T Rec. H.450.8 | Glen Freundlich Avaya Communication 1300 W. 120th Avenue Westminster, CO 80234. USA | Tel: +1 303 538 2899 Fax: +1 303 538 3007 E-mail: ggf@avaya.com |
| Editor ITU-T Rec. H.460.1 | P. Cordell | E-mail: pete@tech-know-ware.com |
| Editor ITU-T Rec. H.460.4 | Gary Thom | E-mail: gthom@delta-info.com |
| Editor ITU-T Rec. H.460.5 | Sasha Ruditsky | E-mail: sasha@radvision.com |
| Editor ITU-T Rec. H.460.6 | Bob Gilman | E-mail: rrg@avaya.com |
| Editor ITU-T Rec. H.460.{2,7,8} | Paul Jones (see above for Q.2/16) | E-mail: paulej@packetizer.com |
| Editor ITU-T Rec. H.460.3 | Louis Fourie | E-mail: lfourie@cisco.com |
| Editor ITU-T Rec. H.460.9 | Ernst Horvath (see above for Q.5/16) | E-mail: ernst.horvath@siemens.at |

Note: Not all Recommendations indicated above have IG issues in this document. The information above is provided for completeness.

Table of Contents

1	Scope	1

2	Introduction	1

3	References	1

4	Nomenclature	1

5	Technical and Editorial Corrections to H.323 Series Recommendations	1

5.1	Technical and Editorial Corrections to ITU-T Recommendation H.323 (2003)	1

5.2	Technical and Editorial Corrections to ITU-T Recommendation H.225.0 (2003)	1

5.3	Technical and Editorial Corrections to ITU-T Recommendation H.245 (7/2003)	1

5.4	Technical and Editorial Corrections to ITU-T Recommendation H.246 (1998)	1

5.5	Technical and Editorial Corrections to ITU-T Recommendation H.235 (2003)	1

5.6	Technical and Editorial Corrections to ITU-T Recommendation H.450 Series	1

5.7	Technical and Editorial Corrections to ITU-T Recommendation H.341 (1999)	1

5.8	Technical and Editorial Corrections to Annex C/H.246 (2003)	1

5.9	Technical and Editorial Corrections to ITU-T Recommendation H.283 (1999)	1

5.10	Technical and Editorial Corrections to ITU-T Recommendation H.460 Series	1

5.11	Technical and Editorial Corrections to ITU-T Recommendation Annex P/H.323 (2003)	1

6	Implementation Clarifications	1

6.1	Token Usage in H.323 Systems	1

6.2	H.235 Random Value Usage in H.323 Systems	1

6.3	Gateway Resource Availability Messages	1

6.4	OpenLogicalChannel in fastStart	1

6.5	Clarification in Q.931 (1993)	1

6.6	Graceful Closure of TCP Connections	1

6.7	Race Condition on Simultaneous Close of Channels	1

6.8	Acceptance of Fast Connect	1

6.9	Semantic Differences between Lightweight RRQs and IRQ/IRR Messages	1

6.10	Specifying the Payload Format for a Channel	1

6.11	Version Dependencies in Annexes	1

6.12	Routing through Signaling Entities and Detecting Loops	1

6.13	Packetization for G.729, G.729a, G.711, and G.723.1	1

6.14	Checking versions for T.38 and V.150.1	1

7	Allocated Object Identifiers and Port Numbers	1

7.1	Allocated Object Identifiers	1

7.2	Allocated Port Numbers	1

8	Use of E.164 and ISO/IEC 11571 Numbering Plans	1

8.1	E.164 Numbering plan	1

8.2	Private Network Number	1

9	ASN.1 Usage, Guidelines, and Conventions	1

9.1	NULL, BOOLEAN, and NULL/BOOLEAN OPTIONAL	1

9.2	ASN.1 Usage in H.450-Series Recommendations	1

Annex A: H.323 Recommendation Series Defect Report Form	1

Implementors’ Guide for ITU-T H.323, H.225.0, H.245, H.246, H.283, H.235, H.341, H.450 Series, H.460 Series, and H.500 Series Recommendations

# 1	Introduction

This document is a compilation of reported defects identified in the versions of ITU-T Recommendation H.323 and its related Recommendations currently in force. It must be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.323-series Recommendations.

Upon discovering technical defects with any components of the H.323 Recommendations series, please provide a written description directly to the editors of the affected Recommendations with a copy to the Q13/16 or Q14/16 Rapporteur.  The template for a defect report is located at the end of the Guide.  Contact information for these parties is included at the front of the document.  Return contact information should also be supplied so a dialogue can be established to resolve the matter and an appropriate reply to the defect report can be conveyed.  This defect resolution process is open to any interested party.  Formal membership in the ITU is not required to participate in this process.

# 2	Scope

This guide resolves defects in the following categories:

- editorial errors

- technical errors, such as omissions and inconsistencies

- ambiguities

In addition, the Implementors’ Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions, or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories.  Proposals for new features should be made in through contributions to the ITU-T.

# 3	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation

–	ITU-T Recommendation H.323 (2003), Packet-Based multimedia communications systems

–	ITU-T Recommendation H.225.0 (2003), Call signaling protocols and media stream packetization for packet based multimedia communications Systems

–	ITU-T Recommendation H.245 (7/2003), Control protocol for multimedia communication

–	ITU-T Recommendation H.246 (1998), Interworking of H-Series multimedia terminals with H-Series multimedia terminals and voice/voiceband terminals on GSTN and ISDN

–	ITU-T Recommendation H.246 – Annex C (2003), ISDN User Part Function - H.225.0 Interworking

–	ITU-T Recommendation H.235 (2003), Security and encryption for H Series (H.323 and other H.245 based) multimedia terminals

–	ITU-T Recommendation H.235 Ammedment 1 (2004)

–	ITU-T Recommendation H.450.1 (1998), Generic functional protocol for the support of supplementary services in H.323

–	ITU-T Recommendation H.450.2 (1998), Call transfer supplementary service for H.323

–	ITU-T Recommendation H.450.3 (1998), Call diversion supplementary service for H.323

–	ITU-T Recommendation H.450.4 (1999), Call hold supplementary service for H.323

–	ITU-T Recommendation H.450.5 (1999), Call park and call pickup supplementary services for H.323

–	ITU-T Recommendation H.450.6 (1999), Call waiting supplementary service for H.323

–	ITU-T Recommendation H.450.7 (1999), Message waiting indication supplementary service for H.323

–	ITU-T Recommendation H.450.8 (2000), Name identification supplementary service for H.323

–	ITU-T Recommendation H.450.9 (2000), Call Completion Supplementary Services for H.323

–	ITU-T Recommendation H.450.10 (2001), Call offer supplementary service for H.323

–	ITU-T Recommendation H.450.11 (2001), Call intrusion supplementary services

–	ITU-T Recommendation H.450.12 (2001), Call Information Additional Network Feature for H.323

–	ITU-T Recommendation H.460.1 (2002), Guidelines for the use of generic extensibility framework

–	ITU-T Recommendation H.460.2 (2001), Number Portability interworking between H.323 and SCN networks

–	ITU-T Recommendation H.460.3 (2002), Circuit status map within H.323 systems

–	ITU-T Recommendation H.460.4 (2002), Call priority designation for H.323 calls

–	ITU-T Recommendation H.460.5 (2002), H.225.0 transport of multiple Q.931 IE of the same type

–	ITU-T Recommendation H.460.6 (2002), Extended Fast Connect Feature

–	ITU-T Recommendation H.460.7 (2002), Digit Maps Within H.323 Systems

–	ITU-T Recommendation H.460.8 (2002), Querying for alternate routes within H.323 systems

–	ITU-T Recommendation H.460.9 (2002), Support for online QoS-Monitoring report

–	ISO/IEC 11571 (1998), Information technology – Telecommunications and information exchange between systems – Private Integrated Services Networks – Addressing

–	ITU-T Recommendation Q.931 (1998), ISDN user-network interface layer 3 specification for basic call control

–	ITU-T Recommendation H.283, Remote device control logical channel transport

# 4	Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

| Symbol | Description |
| --- | --- |
| [Begin Correction] | Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| [End Correction] | Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| ... | Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity. |
| --- SPECIAL INSTRUCTIONS --- {instructions} | Indicates a set of special editing instructions to be followed. |

# 5	Technical and Editorial Corrections to H.323 Series Recommendations

## 5.1	Technical and Editorial Corrections to ITU-T Recommendation H.323 (2003)

### 5.1.1	H.245 Request Mode Initiation

| Description: | Figures D.8 through D.11/H.323 show the H.245 procedures necessary to establish a fax call. The endpoints shown are labeled as Originating and Terminating. However, this distinction is not important and any endpoint that detects the fax tone may initiate the procedures, regardless of whether it originated or terminated the call. The following text should be added to Annex D/H.323 to further clarify this. |
| --- | --- |

[Begin Correction]

D.5	Replacing an existing audio stream with a T.38 fax stream

…

Figure D.8 illustrates a successful switchover from voice to fax when a separate H.245 channel is already open for two unidirectional media channels. Note that in this diagram and in ones that follow, originating and terminating endpoints do not necessarily refer to sending and receiving or calling and called endpoints. Any endpoint that detects the fax tone initiates H.245 procedures to switch to fax mode.

[End Correction]

## 5.2	Technical and Editorial Corrections to ITU-T Recommendation H.225.0 (2003)

### 5.2.1	Error Codes Mapping is Unspecified

| Description: | Mapping from LocationRejectReason and AccessRejectionReason codes to AdmissionRejectReason codes is not specified within H.225.0. The following new section provides this mapping. |
| --- | --- |

[Begin Correction]

7.22	Error Code Mapping

A Gatekeeper that needs to return an AdmissionReject message in response to an AdmissionRequest from an endpoint, as a result of having received a LocationReject or an H.501 AccessRejection in response to its sending an LocationRequest or AccessRequest message, should use the following tables to map the error code that it returns in the AdmissionReject message.

Table 24/H.225.0 – LocationRejectReason to AdmissionRejectReason

| LocationRejectReason | Corresponding AdmissionRejectReason |
| --- | --- |
| notRegistered | calledPartyNotRegistered |
| invalidPermission | invalidPermission |
| requestDenied | requestDenied |
| undefinedReason | undefinedReason |
| securityDenial | securityDenial |
| aliasInconsistent | aliasesInconsistent |
| routeCallToSCN | routeCallToSCN |
| resourceUnavailable | resourceUnavailable |
| genericDataReason | genericDataReason |
| neededFeatureNotSupported | neededFeatureNotSupported |
| hopCountExceeded | noRouteToDestination |
| incompleteAddress | incompleteAddress |
| securityWrongSyncTime | securityWrongSyncTime |
| securityReplay | securityReplay |
| securityWrongGeneralID | securityWrongGeneralID |
| securityWrongSendersID | securityWrongSendersID |
| securityMessageIntegrityFailed | securityMessageIntegrityFailed |
| securityWrongOID | securityWrongOID |
| securityDHmismatch | securityDHmismatch |
| noRouteToDestination | noRouteToDestination |
| unallocatedNumber | unallocatedNumber |

Table 25/H.225.0 – AccessRejectionReason to AdmissionRejectReason

| AccessRejectionReason | Corresponding AdmissionRejectReason |
| --- | --- |
| noMatch | noRouteToDestination |
| packetSizeExceeded | undefinedReason |
| security | securityDenial |
| hopCountExceeded | noRouteToDestination |
| needCallInformation | undefinedReason |
| noServiceRelationship | noRouteToDestination |
| undefined | undefinedReason |
| neededFeature | neededFeatureNotSupported |
| genericDataReason | genericDataReason |
| destinationUnavailable | resourceUnavailable |
| aliasesInconsistent | aliasesInconsistent |
| resourceUnavailable | resourceUnavailable |
| incompleteAddress | incompleteAddress |
| unknownServiceID | noRouteToDestination |
| usageUnavailable | undefinedReason |
| cannotSupportUsageSpec | undefiedReason |
| unknownUsageSendTo | undefinedReason |

[End Correction]

### 5.2.2	Clarification on Usage of additionalSourceAddresses

| Description: | The interworking function between ISUP and H.323 uses Calling Party IE and additionSourceAddresses in H.225.0 Call Signalling Setup message to carry more than one calling party address. The usage by a call signaling routed GK is noted as below. |
| --- | --- |

[Begin Correction]

7.2.2.6	Calling party number

…

H.323 endpoints shall not send multiple Calling Party Number IEs in the same message. Gateways may provide support for interworking with Q.931 SETUP messages that contain multiple Calling Party Number IEs. Gateways that provide such support shall map the first Q.931 Calling Party Number IE to the Calling Party Number IE of the H.225.0 Setup message, and map subsequent Q.931 Calling Party Number IEs to the additionalSourceAddresses field of the H.225.0 Setup message. Gatekeepers that route H.225.0 Setup messages initiated by an H.323 endpoint may insert a number in the additionalSourceAddresses field before forwarding it to its next recipient.

[End Correction]

### 5.2.3	Gatekeeper Assignment of Aliases

| Description: | The text in H.225.0v4 removed the capability of a Gatekeeper to assign an E.164 address to an endpoint that does not register any itself. The following corrections restore this capability. |
| --- | --- |

[Begin Correction]

7.9.1	RegistrationRequest (RRQ)

…

terminalAlias – This optional value is a list of alias addresses, by which other terminals may identify this terminal. This field may be used in addition to or as an alternative to the terminalAliasPattern and supportedPrefixes fields. If the terminalAlias is null, a terminalAlias address may be assigned by the gatekeeper and included in the RCF. If the terminalAlias does not contain any dialedDigits or partyNumber address, a dialedDigits or partyNumber address may be assigned by the gatekeeper and included in the RCF. If an email-ID is available for the endpoint, it should be registered. Note that multiple alias addresses may refer to the same transport addresses. All of the endpoint's aliases that it desires to register shall be included in this list unless the additiveRegistration option is specified, in which case the endpoint aliases in an RRQ shall be added to the list of aliases currently registered for the endpoint.

[End Correction]

### 5.2.4	Use of Facility to initiate H.245

| Description: | The usage of Facility message to carry h245Address to initiate H.245 procedures is not correctly reflected in the text. The following sections in H.225.0 should be amended as shown below |
| --- | --- |

[Begin Correction]

7.3.1	Alerting

…

h245Address – This is a specific transport address on which the called endpoint or gatekeeper handling the call would like to establish H.245 signalling. This address may also be sent in Call Proceeding, Progress, Connect, or Facility.

…

7.3.3	Connect

…

h245Address – This is a specific transport address on which the called endpoint or gatekeeper handling the call would like to establish H.245 signalling. This address shall be sent if sent earlier in Alerting, Progress, Call Proceeding, or Facility.

…

7.3.7	Progress

…

h245Address – This is a specific transport address on which the called endpoint or gatekeeper handling the call would like to establish H.245 signalling. This address shall be sent if sent earlier in Call Proceeding, Alerting, Connect, or Facility.

…

[End Correction]

### 5.2.5	Length Field of UUIE

| Description: | The current text describing the length of the UUIE field and the value supplied in Table 7/H.225.0 through Table 18/H.225.0 can be misleading. The text below supplies the correction. |
| --- | --- |

[Begin Correction]

7.3	Q.931 based H.225.0 call signalling message details

Note that the lengths of the information elements specified in the tables below refer to messages that are generated by H.323 terminals only. The size of the User-user information element, not explicitly shown, is understood as the size of the PER encoded H323-UserInformation sequence. The total size of H323-UserInformation is limited to 65 536 octets. Regardless of the specified sizes, messages forwarded from the SCN side may have different (larger) sizes.

[End Correction]

[Begin Correction]

[Editorial Note] The correction below to the User-User Information Element should be applied to all tables from Table 7/H.225.0 through to Table 18/H.225.0.

Table 7/H.225.0 – Alerting

| Information element | H.225.0 status (M/F/O) | Length in H.225.0 |
| --- | --- | --- |
| Protocol discriminator | M | 1 |
| Call reference | M | 3 |
| Message type | M | 1 |
| Bearer capability | O | 5-6 |
| Extended facility | O | 8-* |
| Channel identification | FFS | NA |
| Facility | O | 8-* |
| Progress indicator | O | 2-4 |
| Notification indicator | O | 2-* |
| Display | O | 2-82 |
| Signal | O | 2-3 |
| High layer compatibility | FFS | NA |
| User-user | M | * |

[End Correction]

## 5.3	Technical and Editorial Corrections to ITU-T Recommendation H.245 (7/2003)

There are currently no corrections to H.245.

## 5.4	Technical and Editorial Corrections to ITU-T Recommendation H.246 (1998)

### 5.4.1	Annex A Corrections

| Description: | The H.245 equivalents defined for H.230 commands MCV and Cancel-MCV were incorrectly defined in H.246. The following text corrects those table entries. |
| --- | --- |

[Begin Correction]

A.5.2.4.1  Multipoint Control C&I

| H.230 command/indication | H.245 equivelent |
| --- | --- |
| MCV | Send either conferenceRequest.broadcastMyLogicalChannel or conferenceCommand.broadcastMyLogicalChannel with the LCN of the video channel in the direction from the gateway to the H.323 endpoint. If the gateway has previously both sent and received the MVC capability to/from the H.230 side (indicating that both ends of the terminal-MCU or inter-MCU link have declared the MVC capability or the H.245 equivalent), then the H.245 side shall use the conferenceRequest form of the message. Otherwise, it shall use the conferenceCommand form of the message. |
| Cancel-MCV | Send conferenceCommand.cancelBroadcastMyLogicalChannel |

[End Correction]

| Description: | New H.243 codepoints MVC, MVA, and MVR were approved in February 2000. To support those new codepoints, the following additions shall be added to the table in A.5.2.4.1 as shown below |
| --- | --- |

[Begin Correction]

A.5.2.4.1  Multipoint Control C&I

| H.230 command/indication | H.245 equivelent |
| --- | --- |
| MVC | Send conferenceCapability.multipointVisualizationCapability |
| MVA | Send conferenceResponse.broadcastMyLogicalChannel.grantedBroadcastMyLogicalChannel |
| MVR | Send conferenceResponse.broadcastMyLogicalChannel.deniedBroadcastMyLogicalChannel |

[End Correction]

| Description: | A minor inconsistency has been discovered in section A.5.2.4.4 of H.246 Annex A. The H.245 equivalent continuous presence BAS codes were not included in H.245v3 so continuous presence processing cannot be translated through a H.320-H.323 gateway. To correct this, commands are added to H.245 and the following corrected translations amend H.246. |
| --- | --- |

[Begin Correction]

A.5.2.4.4  Multipoint Control C&I

| H.230 command/indication | H.245 equivelent |
| --- | --- |
| VIN | Send terminalYouAreSeeing |
| VCB/Cancel-VCB | Send makeTerminalBroadcaster / CancelMakeTerminalBroadcaster |
| VCS/Cancel-VCS | Send sendThisSource / CancelSendThisSource |
| VCR | Send videoCommandReject |
| VIN2 | Send terminalYouAreSeeingInSubPictureNumber |
| VIC | Send videoIndicateCompose |
| VIM | Send videoIndicateMixingCapability |

[End Correction]

### 5.4.2	Reference to ATM Forum Document

| Description: | To help clarify the usage of H.246 with respect to ATM, a reference to an ATM Forum document has been proposed. This reference shall appear in next H.246 publication from the ITU. |
| --- | --- |

[Begin Correction]

1	Scope

...

Voice/Voiceband terminals on GSTN use the appropriate national standards for call control and G.711 or analogue signals for voice. Voice/Voiceband terminals on ISDN use the appropriate national variant of Q.931 for call control and G.711 for voice.

Interworking of H.323 over ATM with H.323 over non-ATM IP networks is possible through the use of an H.323-H.323 gateway. Transport of H.323 media streams over ATM is described in AF-SAA-0124.000.

[End Correction]

[Begin Correction]

2	Normative References

...

-	ATM Forum Technical Committee, AF-SAA-0124.000, Gateway for H.323 Media Transport Over ATM, 1999

[End Correction]

## 5.5	Technical and Editorial Corrections to ITU-T Recommendation H.235 (2003)

H.235 Amendment 1 holds clarifications and corrections of ITU-T Recommendation H.235 (2003). Please refer to that document for further details.

## 5.6	Technical and Editorial Corrections to ITU-T Recommendation H.450 Series

### 5.6.1	Technical and Editorial Corrections to H.450.1 (1998)

#### 5.6.1.1	Actions at a Destination Entity

| Description: | Typographical errors have been discovered in section 6.6 of H.450.1 (1998). The text below outlines the necessary changes. |
| --- | --- |

[Begin Correction]

1)	Section 6.6, line 6

Change:

"rejectUnrecognizedInvokePdu"

to

"rejectAnyUnrecognizedInvokePdu"

2)	Section 6.6, line 12

Change:

"discardAnyUnrecognizedInvokePDU"

to

"discardAnyUnrecognizedInvokePdu"

[End Correction]

#### 5.6.1.2	Corrections to the ASN.1

| Description: | H.225.0 (1999) introduces redundancy with H.450.1 in that both H.225.0 (1999) and H.450.1 have screening and presentation information. To remove the redundancy, it was decided that H.225.0 was the proper place for this information and the redundant elements shall be removed from H.450.1. Below shows the revision to the ASN.1 found in Table 6/H.450.1. |
| --- | --- |

[Begin Correction]

Addressing-Data-Elements

{ itu-t recommendation h 450 1 version1(0) addressing-data-elements(9)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS	AliasAddress, PartyNumber, PresentationIndicator, Screening Indicator FROM H323-MESSAGES; -- see H.225.0

...

-- PartyNumber defined in Recommendation H.225.0

-- PublicPartyNumber defined in Recommendation H.225.0

-- PrivatePartyNumber defined in Recommendation H.225.0

-- NumberDigits defined in Recommendation H.225.0

-- PublicTypeOfNumber defined in Recommendation H.225.0

-- PrivateTypeOfNumber defined in Recommendation H.225.0

-- PresentationIndicator defined in Recommendation H.225.0 (v3 and beyond)

-- ScreeningIndicator defined in Recommendation H.225.0 (v3 and beyond)

EndpointAddress			::=	SEQUENCE{

destinationAddress			SEQUENCE OF AliasAddress,

-- multiple alias addresses may be used to address the same H.323 endpoint

remoteExtensionAddress			AliasAddress OPTIONAL,

…,

destinationAddressPresentationIndicator		PresentationIndicator OPTIONAL,

-- Note 1, 2

destinationAddressScreeningIndicator			ScreeningIndicator OPTIONAL,

remoteExtensionAddressPresentationIndicator	PresentationIndicator OPTIONAL,

-- Note 1, 2

remoteExtensionAddressScreeningIndicator		ScreeningIndicator OPTIONAL

}

-- Note 1: If this element is not available, presentation allowed shall be assumed.

-- Note 2: If an H.450 APDU that carries this element EndpointAddress also

-- contains an element PresentationAllowedIndicator, then the setting of the

-- element PresentationAllowedIndicator shall take precedence in case of

-- conflicting presentation information.

...

[End Correction]

#### 5.6.1.3 Clarifications to ROS APDUs

| Description: | The ASN.1 specification of ROS APDUs has caused some uncertainty over the correct encoding of invoke identifiers. A correct encoding is essential for interoperability between different implementations. The text below attempts to clarify the uncertainty. |
| --- | --- |

Add the following note below Table 4/H.450.1:

[Begin Correction]

Note:

In the Invoke APDU, the invokeID is an INTEGER constrained by a PER-visible constraint (InvokeIdSet = 0..65535) and is therefore encoded as a constrained INTEGER (16 bits, no length field). In the ReturnResult and ReturnError APDUs, however, the invokeID is encoded as an unconstrained INTEGER (with explicit length field) because the applicable constraint (“must be that for an outstanding operation…“) is not PER-visible. In the Reject APDU the invokeID is also encoded as an unconstrained INTEGER (with explicit length field) since no constraint applies.

[End Correction]

#### 5.6.1.4	Error Definitions

| Description: | Error description as defined in Table 8 need to be clarified. The text below provides the needed clarifications. |
| --- | --- |

[Begin Correction]

10.3	General error list

Table 8 contains the definitions of general errors that may be used within H.450.x Recommendations.

Note – Every operation defined in any H.450.x Recommendation lists all permitted error values explicitly. This means that the errors defined here are not automatically part of another H.450.x Recommendation. An H.450.x Recommendation that uses one of the error values below must import it from the module defined below before it can be used for an operation of H.450.x.

Table 8/H.450.1 – H.450.1 General Error List

H4501-General-Error-List { itu-t recommendation h 450 1 version1(0) general-error-list (1) } DEFINITIONS AUTOMATIC TAGS ::= BEGIN IMPORTS ERROR FROM Remote-Operations-Information-Objects { joint-iso-itu-t remote-operations(4) informationObjects(5) version1(0) }; -- The following errors are based on the error definitions of Recommendation Q.950. userNotSubscribed ERROR ::= {CODE local:0} -- is an indication that the user has not subscribed to this service. rejectedByNetwork ERROR ::= {CODE local:1} -- is an indication that the requested service is rejected by the network -- (e.g. gatekeeper). rejectedByUser ERROR ::= {CODE local:2} -- is an indication that the remote user has rejected this service request. notAvailable ERROR ::= {CODE local:3} -- is an indication that the -- requested service is not available under the specific circumstances . insufficientInformation ERROR ::= {CODE local:5} -- is an indication that some essential information is missing, --although the operation argument is formally correct. invalidServedUserNumber ERROR ::= {CODE local:6} -- is an indication that the requested service cannot be performed -- because of the usage of an invalid served user number.

Table 8/H.450.1 – H.450.1 General Error List (concluded)

invalidCallState ERROR ::= {CODE local:7} -- is an indication that the service request is -- incompatible with the current H.225.0 call state .

basicServiceNotProvided ERROR ::= {CODE local:8} -- is an indication that the service request refers to an unsupported -- terminal capability. notIncomingCall ERROR ::= {CODE local:9} -- is an indication that the service request has been invoked for an -- outgoing call, which is not permitted for that service. supplementaryServiceInteractionNotAllowed ERROR ::= {CODE local:10} -- is an indication that the service request is not permitted in -- combination with either a further requested or active supplementary -- service.

resourceUnavailable ERROR ::= {CODE local:11} -- is an indication that the service provider has temporarily no -- resource available for the provision of the requested service.

callFailure ERROR ::= {CODE local:25} -- is an indication that the requested supplementary service was not executable by virtue -- of a Basic Call Failure.

proceduralError ERROR ::= {CODE local:43} -- is an indication that a -- received operation APDU --is not compatible with the procedural context in which it is received . END -- of H4501-General-Error-List

[End Correction]

### 5.6.2	Technical and Editorial Corrections to H.450.2 (1998)

#### 5.6.2.1	Editorial Corrections

| Description: | Typographical errors have been discovered in sections 11.4.2, 11.5.2, 11.6.2, and 13.4 of H.450.2. The text below outlines the necessary changes. |
| --- | --- |

[Begin Correction]

1) 	Editorial - Clause 11.4.2, line 4 c)

Change:

"The CTSetup.request primitive is used to request call establishment from TRTSE."

to

"The CTSetup.request primitive is used to request call establishment to TRTSE"

2)	Editorial - Clause 11.4.2, line 5 d)

Change:

"The CTSetup.confirm primitive is used to indicate success of call establishment to TRTSE."

to

"The CTSetup.confirm primitive is used to indicate success of call establishment from TRTSE."

3)	Editorial - Clause 11.5.2, line 6 e)

Change:

"The CTIdentify.indication primitive is used to request a call identification."

to

"The CTIdentify.indication primitive is used to indicate a call identification."

4)	Editorial - Clause 11.5.2, line 11,12 j)

Change:

"The CTComplete.request primitive may be used by GKs to request sending of call transfer information to the transferred-to user."

to

"The CTComplete.request primitive may be used by GKs to request sending of call transfer information to the transferred-to endpoint."

5)	Editorial - Clause 11.5.2, line 13,14 k)

Change:

"The CTComplete.indication primitive is used to indicate call transfer information to the transferred-to endpoint."

to

"The CTComplete.indication primitive is used to indicate call transfer information to the transferred-to user."

6)  	Editorial - Clause 11.6.2, line 2

Change:

"CT-T1 - Timer CT-T1 shall operate at the TRGSE during state CT-Await-Identify-Response. Its purpose is to protect against the absence of response to the CTIdentify.request."

to

"CT-T1 - Timer CT-T1 shall operate at the TRGSE during state CT-Await-Identify-Response. Its purpose is to protect against the absence of response to the CTIdentify.invoke."

7) 	Editorial – Clause 13.4, FIGURE 25 (sheet 2 of 3, 4th branch) of H.450.2

(i.e. FIGURE 22/H.450.2 (sheet 2 of 3, 4th branch) of H.450.2 (2/98) publication)

Change:

"T4 Timeout"

to

"CT-T4 Timeout"

In addition, the type of symbol was mistake. Time-Out event is an internal event.

[End Correction]

#### 5.6.2.2	Clarification of CallIdentifier and ConferenceIdentifier

| Description: | A clarification of the setting of H.225.0 elements CallIdentifier and ConferenceIdentifier values in conjunction with H.450.2 transferred calls has been added within a new clause 10.7 "Interactions with H.225.0 parameters". Special Note: This section appeared in the May 1999 Implementors’ Guide, but stated that the CallIdentifier should be the same for transferred calls. That definition contradicted H.323v2's definition of the CallIdentifier, so this section has been changed to align with H.323v2 and higher. |
| --- | --- |

[Begin Correction]

10.7		Interactions with H.225.0 parameters

The H.225.0 CallIdentifier value of the transferred call shall use a new value, rather than the value that was used in the primary call.

The H.225.0 ConferenceIdentifier of a transferred call may use a new value. However, the ConferenceIdentifier of an existing conference (multipoint conference) shall not be altered.

[End Correction]

#### 5.6.2.3	Transfer without Consultation

| Description: | An exceptional procedure for a transferred endpoint B actions has been added in clause 8.2.1 to allow call transfer without consultation to take place successfully even if the transferred-to endpoint C does either not support H.450.2 or not support H.450 at all. Furthermore, clause 6 was enhanced to allow a different Interpretation APDU setting. |
| --- | --- |

[Begin Correction]

6	Messages and Information elements

...

When conveying the invoke APDU of operation callTransferSetup, the Interpretation APDU shall contain value clearCallIfAnyInvokePduNotRecognized in case of Transfer with Consultation. In case of Call Transfer without Consultation, the Interpretation APDU shall be set to value discardAnyUnrecognizedInvokePdu.

[End Correction]

[Begin Correction]

8.2.1	Transfer without Consultation with transferred-to endpoint C not supporting H.450.2

a) When receiving a CONNECT message from endpoint C (that does not include a response to the callTransferSetup Invoke APDU) while being in state CT-Await-Setup-Response, the transferred endpoint B should continue as if a callTransferSetup Return Result APDU would have been received. This allows endpoint B to successfully continue with the Call Transfer procedures (including appropriate internal call transfer state handling and clearing of the primary call to the transferring endpoint A). This exceptional procedure enables successful Call Transfer even if the transferred-to endpoint C does not support H.450 at all.

b) When a RELEASE COMPLETE message as a response to a SETUP message containing callTransferSetup Invoke APDU is received in endpoint B on the transferred call attempt, possibly containing callTransferSetup Return Error or Reject APDU, then endpoint B may retry call establishment to endpoint C using a normal basic call. Upon receiving the CONNECT message from endpoint C, endpoint B may continue with the procedures as described in a) above.

Note that this procedure may apply if endpoint C supports H.450.1 but no H.450.2 and if endpoint B has not selected the recommended Interpretation APDU value discardAnyUnrecognizedInvokePdu but has set the value to clearCallIfAnyInvokePduNotRecognized.

[End Correction]

### 5.6.3	Technical and Editorial Corrections to H.450.3 (1998)

#### 5.6.3.1	Editorial Correction in H.450.3

| Description: | Typographical errors have been discovered in H.450.3 clause 12 SDLs. |
| --- | --- |

[Begin Correction]

Editorial – Clause 12 SDL FIGURES 21 (most right branch), 22 (most right branch), 23 (most right branch), 28 (sheet 1 of 4, second right branch) of H.450.3

(i.e. FIGURES 19,20,21 and 24 (sheet 1 of 4)  of H.450.3 of H.450.3 (2/98) published).

The type of symbol was mistake. Time-Out event is an internal event.

Note: The text within the referred symbols remains unchanged.

[End Correction]

#### 5.6.3.2	Clarification of the CallIdentifier and ConferenceIdentifier

| Description: | A clarification of the setting of H.225.0 elements CallIdentifier and ConferenceIdentifier values in conjunction with H.450.3 forwarded calls has been added within a new clause 9.9.3 "Interactions with H.225.0 parameters". Special Note: This section appeared in the May 1999 Implementors’ Guide, but stated that the CallIdentifier should be the same for diverted calls. That definition contradicted H.323v2's definition of the CallIdentifier, so this section has been changed to align with H.323v2 and higher. |
| --- | --- |

[Begin Correction]

9.9.3 Interactions with H.225.0 parameters

The H.225.0 CallIdentifier of a forwarded call shall use a new value, rather than the value that was used in the forwarding call.

The H.225.0 ConferenceIdentifier of a forwarded call may use a new value. However, the ConferenceIdentifier of an existing conference (multipoint conference) shall not be altered.

[End Correction]

#### 5.6.3.3	Correction to the ASN.1

| Description: | A typographical error has been discovered in the ASN.1 definitions presented in H.450.3, Chapter 11. |
| --- | --- |

[Begin Correction]

H225InformationElement FROM H225-generic-parameters-definition

[End Correction]

### 5.6.4	Technical and Editorial Corrections to H.450.4 (1999)

#### 5.6.4.1	Change Relating to Interpretation APDU

| Description: | In order to align H.450.4 with other H.450-series A modified description of the Call Hold Interpretation APDU (i-apdu) setting has been added in clause 6 of Recommendation H.450.4. This information will be contained in the revision 2 of H.450.4 Recommendation to be published by the ITU-T. The modified text is shown below. |
| --- | --- |

[Begin Correction]

6	Messages and Information elements

...

When conveying the Invoke APDU of operations remoteHold and remoteRetrieve, the Interpretation APDU shall be omitted or shall contain the value rejectAnyUnrecognizedInvokePdu.

[End Correction]

#### 5.6.4.2	Feature Interaction between H.450.4 and H.450.2

| Description: | A modified description of the Call Hold interaction with Call Transfer has been added in clause 9.2.1 of Recommendation H.450.4. This information will be contained in the revision 2 of H.450.4 Recommendation to be published by the ITU-T. The modified text is shown below. |
| --- | --- |

[Begin Correction]

9.2.1  	Call Transfer (H.450.2)

If prior to Consultation, the first call has been put on hold, the served User endpoint shall decide whether or not to automatically retrieve the held User before Call Transfer is invoked.

- If the served User endpoint decides for the automatic retrieve option, a retrieveNotific Invoke APDU (in case of near end call hold) or a remoteRetrieve Invoke APDU (in case of remote-end call hold) may either be sent by the served user prior to the message containing the callTransferInitiate Invoke APDU or may be sent within the same message containing the callTransferInitiate Invoke APDU.

If call transfer fails after retrieval from hold was successful (i.e. if callTransferInitiate Return Error or Reject APDU is received or if timer CT-T3 expires), the served user endpoint may automatically re-invoke SS-Hold.

If remote-end call hold retrieval is unsuccessful, in order to proceed with call transfer the remoteRetrieve Return Error or remoteRetrieve Reject APDU should be disregarded.

- If the served User endpoint decides to not choose the automatic retrieve option, call hold applies to the primary call until call transfer has been completed successfully (i.e. until the primary call is cleared). If transfer fails, the primary call remains being held by User A.

[End Correction]

### 5.6.5	Technical and Editorial Corrections to H.450.5 (1999)

#### 5.6.5.1	Clarification of the CallIdentifier

| Description: | A clarification of the setting of H.225.0 element CallIdentifier in conjunction with H.450.5 parked calls has been added within clause 8.3 "Interactions with H.225.0 parameters". This information will be contained in the revision 2 of H.450.5 Recommendation to be published by the ITU-T. The modified text is shown below. |
| --- | --- |

[Begin Correction]

8.3	Interaction with H.225.0 parameters

The H.225.0 CallIdentifier value within a parked call shall use a new value, rather the CallIdentifier value that was used in the primary call.  For all other SETUP messages carrying SS-PARK or SS-PICKUP related APDUs as defined within this recommendation, new CallIdentifier values shall be used. Note that the CallIdentifier value of the parked/alerting call is preserved during the SS-PARK / SS-PICKUP procedure within the H.450 APDUs.

[End Correction]

### 5.6.6	Technical and Editorial Corrections to H.450.7 (1999)

#### 5.6.6.1	Change Relating to Interpretation APDU

| Description: | In order to align H.450.7 with other H.450-series, a modified description of the Message Waiting Indication Interpretation APDU (i-apdu) setting has been added in clause 7.1.1 of Recommendation H.450.7. This information will be contained in the revision 2 of H.450.7 Recommendation to be published by the ITU-T. The modified text is shown below. |
| --- | --- |

[Begin Correction]

7.1.1   H.450.1 Supplementary Service APDU

...

When conveying the Invoke APDU of operations mwiActivate, mwiDeactivate, and mwiInterrogate, the interpretation APDU shall be omitted or shall contain the value rejectAnyUnrecognizedInvokePdu.

[End Correction]

### 5.6.7	Technical and Editorial Corrections to H.450.8 (2000)

#### 5.6.7.1	Usage of CalledName and AlertingName

| Description: | An editorial error has been found in the H.450.8 (2000) Recommendation in the usage of calledName and alteringName. The following text corrects the errors. |
| --- | --- |

[Begin Correction]

7.2 Terminals or MCU as Originating Endpoint

…

A terminal or MCU in receipt of an H.225.0 Connect, Alerting, or Release Complete message containing a connectedName, alertingName, or busyName APDU should not present name information if the Name element indicates namePresentationRestricted.

8.2 Terminals or MCU as Terminating Endpoint

A terminal or MCU in receipt of the H.225.0 Setup message may include name information in the Connect, Alerting or Release Complete as described above in 6.2, 6.3 or 6.4. If presentation of the name to the calling party is desirable, the Name element in the alertingName, connectedName, or busyName operation should indicate namePresentationAllowed. If presentation of the name to the called party is to be restricted, the Name element in the alertingName, connectedName, or busyName operation should indicate namePresentationRestricted.

[End Correction]

### 5.6.8	Technical and Editorial Corrections to H.450.12 (2001)

#### 5.6.8.1	Technical Correction

| Description: | The receipt of a CmnInform APDU at User A’s Endpoint is not described. Therefore add the text below at the end of section 7.1.1.1 ANF-CMN invocation. |
| --- | --- |

[Begin Correction]

7.1.1.1	ANF-CMN invocation

...

Upon receipt of a CmnInform invoke APDU in any message, the Originating endpoint shall remain in the current state.

[End Correction]

#### 5.6.8.2	Add definition of the states CMN-Wait-Response and CMN-Wait-Answer-Response

| Description: | The states CMN-Wait-Response and CMN-Wait-Answer-Response are used only in the SDL diagrams but are not defined anywhere. To avoid confusion, a definition of their meaning is added in section 13. |
| --- | --- |

[Begin Correction]

13.	Specification and Description Language (SDL) Diagrams for ANF-CMN

...

In the following SDLs the states CMN-Wait-Response and CMN-Wait-Answer-Response are used to describe the behavior of the Endpoints using explicit primitive exchange.

The state CMN-Wait-Response is entered at the Endpoint after a primitive CMNRequest indication is received and the previous state was CMN-Idle.

The state CMN-Wait-Answer-Response is entered at the Endpoint after a primitive CMNRequest indication is received and the previous state was CMN-Wait-Answer.

[End Correction]

#### 5.6.8.3	Redesign the SDL Diagrams, add two missing collision branches and delete an erroneous message symbol

| Description: | Two collision branches are missing: add in section 13.1 Figure 8/H.450.12 the possible receipt of a CMNInform request from the application in state CMN-Wait-Answer and in Figure 9/H.450.12 the possible receipt of a CMNRequest request in state CMN-Wait-Response. In Figure 9/H.450.12 the receipt of a CMNInform Request in state CMN-Wait-Response shall be ignored and the message with CMNInform invoke APDU shall not be forwarded to endpoint B. |
| --- | --- |

[Begin Correction]

Editorial - Replace the indicated diagrams by the following:

Figure 8/H.450.12 – SDL Representation of ANF-CMN at Endpoint A (Part 3)

Figure 9/H.450.12 – SDL Representation of ANF-CMN at Endpoint A (Part 3)

[End Correction]

#### 5.6.8.4	 Message Flow of ANF-CMN

| Description: | Timer T1 is started if cmnRequest invoke is sent in FACILITY message, but not if it is sent in a SETUP message. However, the message flow diagram in Figure 2/H.450.12 erroneously contains timer T1. The erroneous diagram should be replaced by the corrected diagram as below. |
| --- | --- |

[Begin Correction]

[End Correction]

## 5.7	Technical and Editorial Corrections to ITU-T Recommendation H.341 (1999)

### 5.7.1	Corrections to the RAS MIB in H.341

| Description: | A few editorial errors have been identified in the RAS MIB in H.341. The following text describes the necessary corrections. |
| --- | --- |

1)	RasAdmissionTableEntry SEQUENCE, the field RASAdmissionCallIdentifier is inserted twice.  The second entry shall be removed.

2)	Each field in CallSignalStatsEntry SEQUENCE referred to the number of messages received ("In") and the number of messages transmitted ("Out").  These counters shall be combined.  The new CallSignalStatsEntry SEQUENCE is shown below:

[Begin Correction]

CallSignalStatsEntry::= SEQUENCE {

callSignalStatsCallConnections

Counter32,

callSignalStatsAlertingMsgs

Counter32,

callSignalStatsCallProceedings

Counter32,

callSignalStatsSetupMsgs

Counter32,

callSignalStatsSetupAckMsgs

Counter32,

callSignalStatsProgressMsgs

Counter32,

callSignalStatsReleaseCompleteMsgs

Counter32,

callSignalStatsStatusMsgs

Counter32,

callSignalStatsStatusInquiryMsgs

Counter32,

callSignalStatsFacilityMsgs

Counter32,

callSignalStatsInfoMsgs

Counter32,

callSignalStatsNotifyMsgs

Counter32,

callSignalStatsAverageCallDuration

Integer32

}

[End Correction]

3)	In RasRegistrationTableEntry SEQUENCE, rasRegistrationEndpointType is defined to be type "Integer32" and should be defined as type "MmH323EndpointType".

### 5.7.2	Support for Expanded Country Code Values in T.35

| Description: | T.35 (1999) expanded the available country codes from one octet to two octets. In order to support the expanded country codes going forward, it is recommended that implementers make the following changes to these definitions in H.341. |
| --- | --- |

[Begin Correction]

h323TermSystemt35CountryCode OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Country code, per T.35 Annex A."

::= { h323TermSystemEntry 5 }

h323TermSystemt35CountryCodeExtention OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Assigned nationally, unless the country code

is 255, in which case this value shall contain

the country code found in T.35 Annex B."

::= { h323TermSystemEntry 6 }

[End Correction]

## Technical and Editorial Corrections to Annex C/H.246 (2003)

### 5.8.1	Interworking for Conveying Two Calling Party Numbers

| Description: | ISUP networks can convey 2 calling party numbers: One in the calling party number, and another one in the generic number (which has the qualification “additional calling party number”). H.323 describes H.460.5 to convey multiple IEs. However this does not allow Gatekeepers to add a “network provided number” and is un-necessarily complex for this simple function. The interworking function to use the existing H.225.0 field additionalSourceAddresses to support this functionality is described using the following set of corrections and additions. |
| --- | --- |

[Begin Correction]

C.6.2.1.1	Special arrangement applies

…

Setup Received from Gatekeeper

A) If only the additionalSourceAddresses is present, or

If only the Calling Party Number is present, or

If only the sourceAddress is present, then Table C.20.1 applies:

Table C.20.1/H.246 – Calling Party Number

| SETUP | IAM |
| --- | --- |
| additionalSourceAddresses or Calling Party Number or sourceAddress | Calling Party Number |

B) If the Calling Party Number (or the sourceAddress in case the Calling Party Number IE is absent) and the additionalSourceAddresses are present, then Table C.20.2 applies:

Table C.20.2/H.246 – Calling Party Number

| SETUP | IAM |
| --- | --- |
| Calling Party Number or sourceAddress | Generic Number (– additional Calling Party Number) |
| additionalSourceAddresses | Calling Party Number |

The address presentation restricted indicator of the calling party number and generic number parameters shall be set according to the CLIR supplementary service.

[End Correction]

[Begin Correction]

C.6.2.1.2	Special arrangement does not apply

…

Table C.20.1 and Table C.20.2 apply.

The address presentation restricted indicator of the calling party number parameter shall be set according to the CLIR supplementary service.

[End Correction]

[Begin Correction]

C.6.2.2	Calling Party Name Restriction (H.450.8)/Calling Line Identification Restriction (CLIR)

If additionalSourceAddresses field is not present then the address presentation restricted indicator of the calling party number and of the generic number parameters is coded as described in Table C.23.

NOTE – If the calling user does not have the Calling Party Name Restriction, the address presentation restricted indicator of the calling party number parameter is set to presentation allowed (see 4.10/Q.951.x [19]).

Table C.23/H.246 – Coding of the address presentation restricted indicator
of the calling party number and generic number parameters

| Internal data (User profile data) |  | SETUP | IAM |
| --- | --- | --- | --- |
| Permanent mode | Temporary mode | Calling party number information element/ User-user information element | Calling party number/ generic number parameter |
|  | default setting | Presentation indicator | Address presentation restricted indicator |
| Yes | Value non-significant | Value non-significant | Presentation restricted |
|  |  | Presentation restricted | Presentation restricted |
|  | Restricted | Absent | Presentation restricted |
| No |  | Presentation allowed | Presentation allowed |
|  |  | Presentation allowed | Presentation allowed |
|  | Allowed | Absent | Presentation allowed |
|  |  | Presentation restricted | Presentation restricted |
| NOTE – The presentationIndicator field in Setup-UUIE carries information identical to the presentation indicator found in the Calling Party Number IE. If both presentationIndicator and the presentation indicator of the Calling Party Number IE are present and are in conflict, the presentation indicator of the Calling Party Number IE shall be used. The meaning and use of the presentation indicator is defined in Q.951.x. |  |  |  |

If additionalSourceAddresses field is present then the address presentation restricted indicator of the calling party number and of the generic number parameters is coded as described in Table C.23bis.

Table C.23bis/H.246 – Coding of the address presentation restricted indicator
of the calling party number and generic number parameters

| Internal data (User profile data) |  | SETUP | IAM |
| --- | --- | --- | --- |
| Permanent mode | Temporary mode default setting |  |  |
| Yes | Value non-significant | Calling party number information element/ User-user information element presentation indicator Value non-significant | generic number presentation indicator Presentation restricted |
|  |  | additionalSourceAddresses presentation indicator Value non-significant | Calling party number presentation indicator Presentation restricted |
|  |  | Calling party number information element/ User-user information element presentation indicator | generic number presentation indicator |
|  |  | Presentation restricted | Presentation restricted |
|  | Restricted | Absent | Presentation restricted |
| No |  | Presentation allowed | Presentation allowed |
|  |  | additionalSourceAddresses presentation indicator | Calling party number presentation indicator |
|  |  | Presentation restricted | Presentation restricted |
|  |  | Absent | Presentation restricted |
|  |  | Presentation allowed | Presentation allowed |
|  |  | Calling party number information element/ User-user information element presentation indicator | generic number presentation indicator |
|  |  | Presentation allowed | Presentation allowed |
|  | Allowed | Absent | Presentation allowed |
|  |  | Presentation restricted | Presentation restricted |
|  |  | additionalSourceAddresses presentation indicator | Calling party number presentation indicator |
|  |  | Presentation allowed | Presentation allowed |
|  |  | Absent | Presentation allowed |
|  |  | Presentation restricted | Presentation restricted |
| NOTE – The presentationIndicator field in Setup-UUIE carries information identical to the presentation indicator found in the Calling Party Number IE. If both presentationIndicator and the presentation indicator of the Calling Party Number IE are present and are in conflict, the presentation indicator of the Calling Party Number IE shall be used for the ISUP generic number with the qualification “additional calling party number”. The meaning and use of the presentation indicator is defined in Q.951.x. |  |  |  |

[End Correction]

[Begin Correction]

C.7.2.3	Calling Line Identification Presentation (CLIP)/Calling Party Name Presentation (H.450.8)

If the called user has CLIP, one or two calling party number information elements are sent in the SETUP message by performing the conversion specified in tables C.56 and C.60.

SETUP message sent to Terminal or Gateway

See Table C.56.

Table C.56/H.246 – CLIP information sent to the called user

| IAM |  | SETUP |
| --- | --- | --- |
| Calling party number parameter Address | Generic number parameter with number qualifier set to |  |
| presentation restricted indicator | additional calling party number | CPAP information sent to the calling user |
| Presentation allowed | Absent | sourceAddress or Calling party number IE (see Table C.57) |
| Presentation allowed | Present | sourceAddress or Calling party number IE (Notes 1) (see Table C.58) additionalSourceAddresses (Notes 1) (see Table C.58bis) |
| Presentation restricted (Note 2) | Value non-significant | sourceAddress or Calling party number IE And additionalSourceAddresses Option 1: Type of number As received Numbering plan As received Presentation ind. Presentation restricted Screening ind. As received Number digits No digit Option 2: Type of number Unknown Numbering plan Unknown Presentation ind. Presentation restricted Screening ind. Network provided Number digits No digit |

Table C.56/H.246 – CLIP information sent to the called user (concluded)

| IAM |  | SETUP |
| --- | --- | --- |
| Address not available or No calling party number parameter | Value non-significant | sourceAddress or Calling party number IE Type of number Unknown Numbering plan Unknown Presentation ind. Not available due to interworking Screening ind. Network provided Number digits No digit |
| NOTE 1 – If the "two calling party number delivery option" does not apply: – only one calling party number information element is sent on H.225.0 side. The generic number is used (see Table C.58). If the "two calling party number delivery option" applies: – - The additionalSourceAddresses is sent and is coded according to the calling party number parameter received, and the Calling Party Number IE is coded according to the generic number parameter (see Table C.58) |  |  |
| NOTE 2 – As a national option, the presentation restriction indication received in the calling party number parameter can be overridden for specific calling access categories. In such a case, the same actions are taken as if presentation allowed was received, except for the presentation restriction indication, which is passed transparently into the calling party number information element. |  |  |

Table C.57/H.246 – Coding of the calling party number information element according 
to the calling party number parameter

| IAM | SETUP |
| --- | --- |
| Calling party number parameter | sourceAddress or Calling party number IE |
| Nature of address indicator National number International number | Type of number (Note 1) National number International number |
| Numbering plan indicator ISDN/Telephony numbering plan | Numbering plan identification ISDN/Telephony numbering plan |
| Address presentation restricted indicator Presentation allowed Presentation restricted | Presentation indicator (Note 2) Presentation allowed Presentation restricted |
| Screening indicator User provided, verified and passed Network provided | Screening indicator (Note 3) User provided, verified and passed Network provided |
| Address signals | Number digits |
| NOTE 1 – As a network option, the type of number may be coded unknown when a prefix is added to the number. NOTE 2 – The Presentation Indicator may be coded as part of the Calling Party Number or as a H.225.0 presentationIndicator IE. NOTE 3 – The Screening Indicator may be coded as part of the Screening Indicator or as a H.225.0 screeningIndicator IE. |  |

Table C.58/H.246 – Coding of the calling party number information element
according to the generic number parameter

| IAM | SETUP |
| --- | --- |
| Generic number parameter with number qualifier set to additional calling party number | sourceAddress or Calling party number IE |
| Nature of address indicator National number International number | Type of number (Note 1) National number International number |
| Numbering plan indicator ISDN/Telephony numbering plan | Numbering plan identification ISDN/Telephony numbering plan |
| Address presentation restricted indicator Presentation allowed Presentation restricted | Presentation indicator (Note 2) Presentation allowed Presentation restricted |
| Screening indicator User provided, not verified | Screening indicator (Note 3) User provided, not verified |
| Address signals | Number digits |
| NOTE 1 – As a network option, the type of number may be coded unknown when a prefix is added to the number. NOTE 2 – The Presentation Indicator may be coded as part of the Calling Party Number or as a H.225.0 presentationIndicator IE. NOTE 3 – The Screening Indicator may be coded as part of the Screening Indicator or as a H.225.0 screeningIndicator IE. |  |

Table C.58bis /H.246 – Coding of the additionalSourceAddresses information element
according to the calling party number parameter

| IAM | SETUP |
| --- | --- |
| Calling party number parameter | additionalSourceAddresses |
| Nature of address indicator National number International number | Type of number (Note 1) National number International number |
| Numbering plan indicator ISDN/Telephony numbering plan | Numbering plan identification ISDN/Telephony numbering plan |
| Address presentation restricted indicator Presentation allowed Presentation restricted | Presentation indicator Presentation allowed Presentation restricted |
| Screening indicator User provided, verified and passed Network provided | Screening indicator User provided, verified and passed Network provided |
| Address signals | Number digits |
| NOTE 1 – As a network option, the type of number may be coded unknown when a prefix is added to the number. |  |

[End Correction]

[Begin Correction]

SETUP Message sent to Gatekeeper

A) If the calling party number is absent in ISUP then Table C.60.1 applies:

Table C.60.1/H.246 – Calling Party Number

| SETUP | IAM |
| --- | --- |
| sourceAddress or Calling Party Number IE Type of number Unknown Numbering plan Unknown Presentation ind. Not available due to interworking Screening ind. Network provided Number digits No digit |  |

B) If only the calling party number is present in ISUP, then Table C.60.2 applies:

Table C.60.2/H.246 – Calling Party Number

| SETUP | IAM |
| --- | --- |
| Calling Party Number | Calling Party Number |

C) If the calling party number and the generic number (with the qualification additional Calling Party Number) are present, then Table C.60.3 applies:

Table C.60.3/H.246 – Calling Party Number

| SETUP | IAM |
| --- | --- |
| Calling Party Number | Generic Number (– additional Calling Party number) |
| additionalSourceAddresses | Calling Party Number |

Table C.60/H.246 – Calling Party Number

[End Correction]

### 5.8.2	ACM sending without cause when PI=8

| Description: | The current text seems to indicate an SS7 ACM message should not be sent when the Progress Indicator had value 8 (in-band information or an appropriate pattern is now available) in a Call Proceeding or Progress message. This is incorrect since the SS7 backward talk path will not be set up without sending ACM and there will be no way for the in-band information to make it to the calling party without sending ACM. |
| --- | --- |

[Begin Correction]

##### C.7.1.3.2	ACM without cause parameter

The following cases are possible trigger conditions of sending the address complete message (ACM) without cause parameter:

a)	The destination has determined independently of access indications that the complete called party number has been received.

b)	Overlap receiving is used on the H.225.0 side and a CALL PROCEEDING is received.

c)	En bloc receiving is used on the H.225.0 side and a Progress indicator information element  is received in a CALL PROCEEDING message or in a PROGRESS message.

d)	The first ALERTING message is received.

On speech or 3.1 kHz calls, the awaiting answer indication (e.g. ring tone) is sent to the calling party upon receipt of the first ALERTING message.

NOTE 1 – In all cases, it is assumed that no Address Complete Message (ACM) has already been sent.

NOTE 2 – The case of the sending of the Address Complete Message (ACM) when the call is forwarded is not described hereafter: see C.7.2.

[End Correction]

## 5.9	Technical and Editorial Corrections to ITU-T Recommendation H.283 (1999)

### 5.9.1	Support for Expanded Country Code Values in T.35

| Description: | T.35 (1999) expanded the available country codes from one octet to two octets. In order to support the expanded country codes going forward, it is recommended that implementers take note of the following usage guidelines for fields in H.283. |
| --- | --- |

[Begin Correction]

...

H221NonStandard ::= SEQUENCE

{

t35CountryCode 	INTEGER(0..255), 		-- country, as per T.35 Annex A

t35Extension      	INTEGER(0..255), 		-- assigned nationally, unless the

-- t35CountryCode is binary 1111 1111,

-- in which case this field shall

-- contain the country code found

-- in T.35 Annex B

manufacturerCode 	INTEGER(0..65535) 	-- assigned nationally

}

...

[End Correction]

## 5.10	Technical and Editorial Corrections to ITU-T Recommendation H.460 Series

### 5.10.1	Technical and Editorial Corrections to H.460.1 (2002)

#### 5.10.1.1	Encoding rules

| Description: | Currently, H.460.1 requires each individual recommendation to specify the encoding that it will use for parameters in the raw format. It would be better to specify the default encoding in H.460.1 and let individual recommendations change it only if needed. The textual changes are shown below. |
| --- | --- |

[Begin Correction]

7.2	Encoded in Raw Method

…

If the feature is defined using ASN.1, then it is recommended that the basic aligned variant of the PER encoding rules be used. However, irrespective of this, encoding rules that are used, if different from the above, shall be explicitly stated in the specification of the feature.

[End Correction]

### 5.10.2	Technical and Editorial Corrections to H.460.2 (2001)

#### 5.10.2.1	Typographical Error in Section 4.1

| Description: | A typographical error has been discovered in that the parameter qorPortedNumber in the ASN.1 is referred to as qorPortedAddress in Section 4.1 of H.460.2 (2001). The text below outlines the necessary change. |
| --- | --- |

[Begin Correction]

4.1	Messages and Signaling

…

5)	When a Gatekeeper receives an ARQ or LRQ and determines that the destination number is ported out of the network and it may wish to invoke number portability Query on Release (QoR) procedures (as specified in Annex C/Q.769.1). In such cases, the Gatekeeper must respond with ARJ or LRJ that contains a reject reason of genericDataReason. The Gatekeeper should include the genericData of the ARJ/LRJ that contains the NumberPortabilityGenericData with the numberPortabilityRejectReason. The numberPortabilityRejectReason now will have a value of qorPortedNumber (=1). This maps to the ISUP release cause value = #14 (QoR: ported number) as specified in Addendum 1/Q.850.

[End Correction]

#### 5.10.2.2	Cardinality of Number

| Description: | A typographical error has been discovered in that the parameter qorPortedNumber in the ASN.1 is referred to as qorPortedAddress in Section 4.1 of H.460.2 (2001). The text below outlines the necessary change. |
| --- | --- |

[Begin Correction]

5	H.225.0 Generic Data Usage

…

| Generic Extensibility Type | Fields | Field name | Value |
| --- | --- | --- | --- |
| EnumeratedParameter |  |  |  |
| GenericIdentifier | id | standard | 1 |
| Contents | content | raw | ASN.1 PER encoding of the NumberPortabilityInfo |
| Parameter Cardinality |  |  | Once and Only Once |

[End Correction]

### 5.10.3	Technical and Editorial Corrections to H.460.6 (2002)

#### 5.10.3.1	Close All Channels

| Description: | The intent of the Close All Media Channels request described in section 4.1.2 is to close all open media channels and cancel all available sessions, as described in section 4.5. To this end, text in sections 4.1.2 and 4.5.2 should be changed as follows. |
| --- | --- |

[Begin Correction]

4.1.2	Close All Channels

This parameter may be used by a party to request that the receiving endpoint close all open media channels and cancel all available sessions.  Support for this parameter is optional, and shall be negotiated during EFC feature negotiation.

…

4.5.2	Requesting Close-All-Channels

An endpoint or a third party may request that the other endpoint close all open media channels and cancel all available sessions by sending a genericData element with the EFC featureID and parameter 2 present in any convenient call signalling message (e.g., FACILITY).  The receiving endpoint is expected to silently close all open channels without any response (e.g., without issuing any Null-OLCs.)

[End Correction]

#### 5.10.3.2	Signaling of EFC Support in supportedFeatures

| Description: | It is held that signalling of EFC in supportedFeatures by the originating party is unnecessary. The text in section 4.2 should be corrected as below. |
| --- | --- |

[Begin Correction]

4.2	Invocation of Extended Fast Start

An originating party shall indicate its desire to use EFC when it issues a SETUP message.  The SETUP shall contain a request for EFC support in the desiredFeatures element, or a requirement for EFC support in the neededFeatures element.  The EFC feature is symmetric, hence requestor support for the feature may be inferred from a request for EFC, and the supportedFeatures element need not be included to indicate support for EFC. In addition, the SETUP message shall include a genericData element specifying EFC Proposal (parameter 1) and a fastStart element containing one or more proposals.   That is, EFC procedures shall include the standard Fast Connect procedures.

[End Correction]

#### 5.10.3.3	Prevention of Race Condition in Master/Slave Determination

| Description: | There is a possible race condition that may occur, depending on the order in which an endpoint processes fastStart elements versus tunnelled H.245 master/slave negotiation messages embedded in the same H.225.0 message. Thus, it is suggested that the following paragraph be added to the end of section 4.2.1. |
| --- | --- |

[Begin Correction]

4.2.1	Master/Slave Determination

Parties supporting Extended Fast Connect should use the H.245 tunnel to carry out master/slave negotiation.  For the initial Fast Connect exchange, the caller (sender of the SETUP with proposals) shall be considered the slave, and the called party (acceptor of proposals) shall act as the master.  Although this convention will suffice for simple A-to-B calls, it can lead to complications in more complex call scenarios.

Different implementations may process fastStart elements and tunnelled H.245 messages in different orders.  EFC proposals or acceptances shall not be included in any H.225.0 message that carries an H.245 MasterSlaveDeterminationAck message that conveys a change in master/slave status.  Doing so could lead to temporary confusion about which party is master and how to respond to the EFC elements.

[End Correction]

#### 5.10.3.4	Remote Endpoint Type and Version after Re-routing

| Description: | An endpoint may not be aware of the H.323 protocol version number supported by the remote endpoint, especially if the call gets re-routed one or more times. In some cases it might be helpful for the endpoint to have this information. The following additions should be made to H.460.6 document. |
| --- | --- |

[Begin Correction]

5.5	EFC Third-party Pause and Rerouting

EFC supports third-party pause and rerouting, as described in H.323 Annex F for SETs,  when used by a routing gatekeeper.  The third party (the gatekeeper in the example in Figure 5) may idle the caller’s transmit and/or receive channels via Null-OLCs, then supply the caller’s proposal fastStart to a new party (e.g., in a SETUP).  The acceptance fastStart will appear to the caller as a redirection or reconfiguration, asillustrated in Figure 5.

Figure 5/H.460.6 - Third-Party Redirection

In the above diagram, the Gatekeeper, or the entity that re-routes a call should send a Facility message containing the destinationInfo field upon completion of the re-routing to the entity that gets re-routed, i.e., Caller.  An endpoint should examine this message for the H.225.0 version information at any point that a Facility message is received containing this field.

After coming out of the “paused” state an endpoint should examine the version-id fields in TCS messages to determine the H.245 version supported by the remote endpoint.

In addition, an endpoint interested in knowing the version of the remote endpoint should send a Status Inquiry message and wait for the receipt of the Status message to determine the version of the H.225.0 in use when it exits the paused state when the above Facility message is not received within a reasonable amount of time. The length of this time is left to the implementation.

[End Correction]

### 5.10.4	Technical and Editorial Corrections to H.460.7 (2002)

#### 5.10.4.1	Compound Type Parameter Usage

| Description: | The contents of the compound parameter in Section 6.4, Table 9 are not well defined. The text below clarifies its usage. |
| --- | --- |

[Begin Correction]

6.4	Digit map string parameters

…

Table 1/H.460.7 – Type of Number Associated Digit Maps

| Parameter name: | ToN Associated Digit Map |
| --- | --- |
| Parameter description: | This compound type conveys Digit Map associated with a particular Type of Number |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 5 |
| Parameter type: | Compound |
| Parameter cardinality: | Zero or more |

Within the compound type defined in Table 9, the parameters defined in Table 2 and Table 11 shall be included to convey one or more Digit Map strings for a particular Type of Number:

Table 2/H.460.7 – Type of Number Parameter

| Parameter name: | Type of Number (ToN) |
| --- | --- |
| Parameter description: | This parameter indicates the type of number |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | Number8 |
| Parameter valid values: | 1 International number 2 National number 3 Network specific number 4 Subscriber number 6 Abbreviated number |
| Parameter cardinality: | Once |

The Digit Map strings comprising the Digit Map associated with a Type of Number are conveyed as additional parameters within the compound type of the Type of Number Associated Digit Maps parameter shown in Table 9. This is shown in Table 11.

Table 3/H.460.7 - Digit Map strings for ToN Parameter

| Parameter name: | Digit Map Strings for ToN |
| --- | --- |
| Parameter description: | This parameter contains a single Digit Map string |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 2 |
| Parameter type: | Text |
| Parameter cardinality: | One or more |

The syntax of the text field, which holds a single Digit Map string, is described in section 10.

The order of the Digit Map strings in the parameters field has no significance.

[End Correction]

#### 5.10.4.2	Duplicate Parameters

| Description: | Section 6.3 (Table 3) and Section 6.5 (Table 12) both define a parameter with ID 2. They can be used in the same place (in an RCF), so it is not possible to distinguish between them. The parameter identifier value should be changed to 2 as below. |
| --- | --- |

[Begin Correction]

6.5	URL parameter

…

Table 4/H.460.7 - URL Parameter

| Parameter name: | Digit Map URL |
| --- | --- |
| Parameter description: | This parameter contains a URL to Digit Map information accessible via HTTP |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 6 |
| Parameter type: | Alias |
| Parameter cardinality: | Zero or one |

[End Correction]

### 5.10.5	Technical and Editorial Corrections to H.460.8 (2002)

#### 5.10.5.1	CallTerminationCause Parameter Usage

| Description: | Table 3 defines a parameter for "Call Termination Cause" but does not explicitly define its format. The text below clarifies this. |
| --- | --- |

[Begin Correction]

6	Querying For Alternate Routes

…

Table 5

Parameter to contain the Call Termination Cause

| Parameter name: | Call Termination Cause |
| --- | --- |
| Parameter description: | The CallTerminationCause structure contains the reason for the previously failed call attempt |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 2 |
| Parameter type: | Raw |
| Parameter cardinality: | Zero or one |

The contents of the raw parameter shall be encoded the same as CallTerminationCause as defined in the ASN.1 in Annex H/H.225.0.

[End Correction]

## 5.11	Technical and Editorial Corrections to ITU-T Recommendation Annex P/H.323 (2003)

### 5.11.1	Allowance for multiple Audio and VBD Streams

| Description: | Inconsistencies exist between the current Annex P text and the text found in V.150.1 related to the same functionality for SDP-based systems. In particular, the text for SDP-based systems allows for any number of audio streams and multiple VBD streams. The text below provides the correction. |
| --- | --- |

[Begin Correction]

While there are no strict limitations on the number of streams that may be contained within any MPS channel, the MPS channel used for MoIP shall contain zero or more audio streams, one or more VBD streams, no more than one SSE stream, and no more than one SPRT stream. If the SPRT stream is opened as a separate channel, the MPS channel shall not also include an SPRT stream. In addition, there shall be different payload types for audio, VBD, SSE, and SPRT streams within the MPS. It is possible that more than four payload types may be utilized for audio, VBD, SSE, and SPRT streams. For example, if the VBD stream is protected with Forward Error Correction (FEC), and if those FEC packets are contained within a Redundancy Encoding packet, there may be not just one payload type value for the VBD stream, but three: one used in the RTP header to signify that the packet contains a redundantly encoded payload, one for the primary payload (the VBD data), and one for the FEC data carried as the secondary encoding.

To optionally protect a VBD stream, an endpoint may utilize forward error correction and/or redundancy encoding. A stream that utilizes forward error correction shall be signalled via the fec field of the DataType structure within the MultiplePayloadStreamElement structure. A stream that utilizes redundancy encoding shall be signalled via the redundancyEncoding field in the DataType structure within the MultiplePayloadStreamElement structure.

[End Correction]

# 6	Implementation Clarifications

## 6.1	Token Usage in H.323 Systems

There has been some confusion on the usage of individual CryptoH323Tokens as passed in RAS messages.  There are two main categories of CryptoH323Tokens; those used for H.235 procedures and those used in an application specific manner. The use of these tokens should be according to the following rules:

- All H.235 defined (e.g. cryptoEPPwdHash, cryptoGKPwdHash, cryptoEPPwdEncr, cryptoGKPwdEncr, cryptoGKCert, and cryptoFastStart). shall be utilized with the procedures and algorithms as described in H.235.

- Application specific or proprietary use of tokens shall utilize the nestedcryptoToken for their exchanges.

- Any nestedcryptoToken used should have a tokenOID (object identifier) which unambiguously identifies it.

## 6.2	H.235 Random Value Usage in H.323 Systems

The random value that is passed in xRQ/xCF sequence between endpoints and Gatekeepers may be updated by the Gatekeeper.  As described in section 4.2 of H.235 this random value may be refreshed in any xCF message to be utilized by a subsequent xRQ messages from the endpoint.  Due to the fact that RAS messages may be lost (including xCF/xRJ) the updated random value may also be lost.  The recovery from this situation may be the reinitializing of the security context but is left to local implementation.

Implementations that require the use of multiple outstanding RAS requests will be limited by the updating of the random values used in any authentication.  If the updating of this value occurs on every response to a request, parallel requests are not possible.  One possible solution, is to have a logical "window" during which a random value remains constant.  This issue is a local implementation matter.

## 6.3	Gateway Resource Availability Messages

The Resources Available Indication (RAI) is a notification from a gateway to a gatekeeper of its current call capacity for each H-series protocol and data rate for that protocol. The gatekeeper responds with a Resources Available Confirmation (RAC) upon receiving a RAI to acknowledge its reception.  A Gatekeeper should ignore any RAI notifications (e.g. send no RAC) upon receiving a RAI which contains bogus information (i.e. a bad endpointIdentifier).

## 6.4	OpenLogicalChannel in fastStart

In the H.225.0 ASN.1, fastStart is defined as SEQUENCE OF OCTET STRING OPTIONAL. The text definition states "This uses the OpenLogicalChannel structure defined in H.245…" Each OCTET STRING in fastStart is to contain the OpenLogicalChannel structure, not an entire request message.

## 6.5	Clarification in Q.931 (1993)

Table 4-3/Q.931 (1993) (Information Element Identifier Coding) shows that the Progress Indicator IE identifier is 0x1e, but Figure 4-29/Q.931 (octet layout of Progress Indicator IE) shows the identifier as 0x1f. Note that the identifier should be 0x1e.

## 6.6	Graceful Closure of TCP Connections

When a TCP connection is closed, the graceful closure procedure documented in section 3.5 of RFC 793 should always be used.

## 6.7	Race Condition on Simultaneous Close of Channels

Section 8.5 of H.323 describes the procedures that an endpoint follows to terminate a call.  It should be noted that as prescribed in Step 6, both endpoints shall issue a Release Complete simultaneously.  Endpoints should be prepared for this potential race condition.

## 6.8	Acceptance of Fast Connect

When an endpoint accepts the Fast Connect procedure, it may select from the proposed channels as specified in section 8.1.7.1/H.323.  The Recommendation clearly specifies what fields shall be modified by the endpoint to accept both the forward and the reverse channels.  An endpoint shall not modify any fields other than those specified in 8.1.7.1/H.323 when returning the proposed channels.

Newer versions of H.245 may introduce new fields into the OpenLogicalChannel sequence or one of the structures contained therein, as well as new procedures.  An older endpoint is obviously not required to decode such new fields or to return such new fields when accepting any proposal.  Implementers should consider the consequences of transmitting a newer H.245 OLC to an older endpoint.  For the purposes of Fast Connect, the calling endpoint shall assume that the called endpoint's version of H.245 is the minimum version of H.245 necessary to be complaint with an H.323 device that advertises the version of H.225.0 transmitted in the messages from the called endpoint (refer to the "Summary" section of H.323).

## 6.9	Semantic Differences between Lightweight RRQs and IRQ/IRR Messages

The lightweight RRQ and the IRR message serve two different functions with an H.323 system.  While both are a means of allowing the Gatekeeper to discover that an endpoint is alive, they also each serve separate, unique functions.

The lightweight RRQ is intended to prevent a registration with a Gatekeeper from expiring.  The message is generated by the endpoint and does not require the Gatekeeper to poll each endpoint on a regular interval. This message is also a means of allowing the Gatekeeper to provide updated registration information, such as a new list of Alternate Gatekeepers, after the initial registration.

Version 1 of H.323 did not have the concept of a lightweight RRQ, so the IRQ/IRR exchange is the only mechanism available to determine endpoint status of Version 1 devices.  However, the lightweight RRQ may be a better choice for determining endpoint status for Version 2 and higher devices.

The IRQ/IRR exchange allows the Gatekeeper to poll the endpoint periodically to discover if the endpoint is still alive.  However, an IRR is also intended to convey details about current active calls.  This can be used by the Gatekeeper to discover calls that have terminated, which may happen if the endpoint fails to properly send a DRQ message for a call.  The IRR message also provides specific details about active calls.

## 6.10	Specifying the Payload Format for a Channel

Implementers should be conscientious of the fact that there are possibly multiple payload formats defined for media formats.  For example, two payload formats are defined for H.263—one is defined for the Recommendation H.263 (1996) and one for Recommendation H.263 (1998).  Other payload formats may be defined for existing codecs or revisions of those codecs. For interoperability, it is strongly advised that implementers provide the mediaPacketization element of the h2250LogicalChannelParameters sequence in the OpenLogicalChannel message so that there is no ambiguity at to which payload format is being used.

## 6.11	Version Dependencies in Annexes

It was noted that the Annexes to H.323 often fail to indicate the minimum version of H.323 and H.245 required for the Annex.  This table is an attempt to clarify the version relationships:

| H.323 Annex | Minimum H.323 Version | Minimum H.245 Version |
| --- | --- | --- |
| Annex Dv1 (1998) | 1998 (Version 2) | 1998 (Version 4) |
| Annex Dv2 (2000) | 2000 (Version 4) | 2000 (Version 7) |
| Annex E | 1998 (Version 2) | N/A |
| Annex F | 1998 (Version 2) | N/A |
| Annex G | 1998 (Version 2) | 1998 (Version 4) |
| Annex J | 1998 (Version 2) | N/A |
| Annex M.1 | 2000 (Version 4) | N/A |
| Annex M.2 | 2000 (Version 4) | N/A |
| Annex P | 2000 (Version 4) | 2003 (Version 9) |
| Annex R | 2000 (Version 4) | N/A |

## 6.12	Routing through Signaling Entities and Detecting Loops

In some call scenarios, a call may be routed though a signaling entity multiple times.  For example, a call from Endpoint 1 (EP1) may be routed through Gatekeeper 1 (GK1) and Gatekeeper 2 (GK2) to Endpoint 2 (EP2) as shown in the Figure 1.

Figure 1 - Call placed through multiple gatekeepers

If EP2 redirects the call to a third endpoint, such as Endpoint 3 (EP3), signaling entities such as GK1 and GK2 should be prepared to handle such call rerouting.  For this example, assume that EP2 returned a Facility message with a reason of callForwarded upon receiving a Setup message.  Rather than propagate that response back to EP1, GK2 may choose to handle the call forward operation.  GK2 would send a Release Complete to EP2 and begin rerouting the call.  Suppose that GK2 sends an LRQ message to GK1 for EP3 and that GK1 replies with its address so that that calls routed to EP3 are routed through it.  GK2 would then send a Setup message for this call to GK1 as shown in Figure 2.

Figure 2 - Gatekeeper 2 re-routes call back to Gatekeeper 1

When GK1 receives the Setup message from GK2, it may inadvertently mistake the call as "bogus", since the Call Identifier will match an already existing call within the Gatekeeper.  Implementers should consider this type of call scenario and be prepared to receive incoming calls that contain Call Identifiers for calls that are already being routed through the routing entity.  The routing entity should examine not only the Call Identifier, but also the destination address of the call  (the call signaling address, aliases, or Called Party Number of the destination).  In this case, the call is routed through GK1 with a destination address of EP2 is rerouted by GK2 to GK1, but with a destination address of EP3.  In this way, the GK1 will properly handle call routing and rerouting, as well as prevent loops in the call signaling path.

In this example, there was a dependency on the H.323v2 Call Identifier.  Unfortunately, H.323 version 1 systems did not have Call Identifiers.  For this reason, these  loop detection and rerouting procedures are not possible.  Nonetheless, it is advisable for routing entities to make an effort to prevent loops properly.  For example, if the entities in Figure 2 were version 1 devices, the GK1 may examine the source address, destination address, and Conference Identifier (CID) of the call.  The first time the call is presented to the Gatekeeper, the destination address is EP2, just as before.  However, when GK re-routes the call back to GK1, the destination address is EP3.  In this way, GK1 may allow proper rerouting of the call to EP3.

The logic for Version 1 devices seems similar to that for Version 2 and higher devices, but there are issues when EP2 and EP3 are MCUs, for example.  Suppose that EP2 is an MCU that is directing all calls to EP3.  The first time a call is redirected to GK1, GK1 may realize that this is, indeed, a call redirection as described above.  However, when the second call is redirected, GK1 has no means of distinguishing between the first redirected call and the second: the source address may be the same, the destination address is the same as the previously rerouted call (EP3), and the Conference ID is the same.  So in this case, GK1 may have no choice but to assume that a loop has occurred and release the offending call.  Although this is unfortunate, H.323v2 and higher systems do not suffer from this problem.  What is important, though, is that loop detection is possible—even with version 1 systems.

## 6.13	Packetization for G.729, G.729a, G.711, and G.723.1

The delay associated with codec processing and packetization should be kept as short as possible. To accomplish this objective when G.729 or G.729A is used, two frames per packet should be considered as the maximum packet size. Similarly, G.711 may be used with packet sizes of 10 ms (80 frames) or 20 ms (160 frames) to achieve this objective. Finally, when G.723.1 is used, only one frame should be included in each packet. The 30 ms frame size of G.723.1 results in speech collection and coding delay of at least 60 ms, contributing to difficulty of interactive communications.

## 6.14	Checking versions for T.38 and V.150.1

It is important that devices properly negotiate the version of the T.38 or V.150.1 to be used and agree to use the same version. At the present time there are few guidelines for version negotiation. Until the guidelines are developed the following note applies:

Devices supporting multiple versions of T.38 and V.150.1 may offer multiple proposals in Fast Connect, each with a different version specified. A device shall not accept a proposal for a version that it does not support.

# 7	Allocated Object Identifiers and Port Numbers

Information in this section is provided for informational purposes and convenience.  This section does not supercede nor replace proper references in H.225.0, H.225, H.235, or other Recommendations.

## 7.1	Allocated Object Identifiers

The following object identifiers have been allocated for protocols associated with H.323.  Any future object IDs that are allocated should be indexed here to prevent duplication.

Note that object IDs below that are allocated below the arc { itu-t(0) recommendation(0) } are show with an abbreviated prefix of  "0 0" below.

| { 0 0 h(8) 2250 version(0) [v] } | H225.0 version numbers |
| --- | --- |
| Assigned values of v: 1-4 |  |
| { 0 0 h(8) 2250 annex(1) g(7) version(0) [v] } | H225.0 Annex G version numbers |
| Assigned values of v: 1-2 |  |
| { 0 0 h(8) 2250 annex(1) g(7) usage(1) [u] } | H225.0 Annex G usage tags |
| Assigned values of u: none |  |
| { 0 0 h(8) 245 version(0) [v] } | H245 version numbers |
| Assigned values of v: Please refer to Table D.1/H.245 |  |
| { 0 0 h(8) 245 generic-capabilities(1) video(0) [c] } | Generic video capabilities |
| Assigned values of c: Please refer to Table D.1/H.245 |  |
| { 0 0 h(8) 245 generic-capabilities(1) audio(1) [c] } | Generic audio capabilities |
| Assigned values of c: Please refer to Table D.1/H.245 |  |
| { 0 0 h(8) 245 generic-capabilities(1) data(2) [c] } | Generic data capabilities |
| Assigned values of c: Please refer to Table D.1/H.245 |  |
| { 0 0 h(8) 245 generic-capabilities(1) control(3) [c] } | Generic control capabilities |
| Assigned values of c: Please refer to Table D.1/H.245 |  |
| { 0 0 h(8) 245 generic-capabilities(1) multiplex(4) [c] } | Generic multiplex capabilities |
| Assigned values of c: Please refer to Table D.1/H.245 |  |
| { 0 0 h(8) 283 generic-capabilities(1) 0 } | H.283 Capability |
| {iso (1) identified-organization (3) icd-ecma (0012) private-isdn-signalling-domain (9)} | Identifies QSIG as the tunneled protocol within an H.225.0 Call Signalling Channel |

## 7.2	Allocated Port Numbers

The following IP port numbers have been allocated for various components of H.323:

| 1300 | TLS secured call signalling |
| --- | --- |
| 1718 | Multicast RAS Signalling |
| 1719 | Unicast RAS Signalling |
| 1720 | TCP call signalling |
| 2099 | Annex G/H.225.0 Signalling |
| 2517 | Annex E/H.323 Signalling |

# 8	Use of E.164 and ISO/IEC 11571 Numbering Plans

## 8.1	E.164 Numbering plan

ITU-T Recommendation defines E.164 numbers the following way for geographic areas:

Figure – International public telecommunication number structure for geographic areas

Similar descriptions are also defined for non-geographic areas. Recommendation E.164 further defines country codes (CC) for all the countries and regions of the world.

An international E.164 number always starts with a country code and its total length is always 15 digits or less. More importantly, it does not include any prefixes that are part of a dialing plan (for example, "011" for an international call placed in North America, or "1" for a long-distance call), nor does it include "#" or "*". The number "49 30 345 67 00" is an E.164 number with CC=49 for Germany. A national number is the international number stripped of the country code, "30 345 67 00" in this case. The subscriber number is the national number stripped of the national destination code, "345 67 00" in this case.

An E.164 number has global significance: any E.164 number can be reached from any location in the world. A "dialed digit sequence", however, only has significance within a specific domain. Within a typical private numbering plan in an enterprise, for example, a prefix, such as "9", may indicate that a call goes "outside", at which point the local telephone company's dialing plan takes over. Each telephone company or private network is free to choose its own dialing plan. It is also free to change it as it pleases—and frequently does so (adding new area codes, for example).

In a typical geographically determined network where users input telephone numbers manually and where users do not travel too much, having different dialing plans everywhere is usually a problem. However, when a user travels, the user must determine the other network's numbering plan in order to place calls. When computer systems perform the dialing automatically, the user is usually required to customize the dialing software for every region or network.

Because of these issues with varying dialing plans and automated dialing, it is essential to be able to refer to an absolute "telephone number" instead of  "what you have to dial to reach it from a specific location." Proper usage of E.164 numbers can resolve these issues. Many systems use E.164 numbers instead of dialed digits: for example, a PBX may gather the dialed digits from a user on a telephone and then initiate a call to the local phone company using an E.164 number in the Called Party Number information element in Q.931.  When completing the Called Party Number IE, specifying the numbering plan as "ISDN/telephony numbering plan (Recommendation E.164)" indicates an E.164 number.  Specifying the type of number as "unknown" and the specifying the numbering plan as "unknown" indicates dialed digits.

The following are a set of definitions from E.164:

number

A string of decimal digits that uniquely indicates the public network termination point. The number contains the information necessary to route the call to this termination point.

A number can be in a format determined nationally or in an international format. The international format is known as the International Public Telecommunication Number which includes the country code and subsequent digits, but not the international prefix.

numbering plan

A numbering plan specifies the format and structure of the numbers used within that plan. It typically consists of decimal digits segmented into groups in order to identify specific elements used for identification, routing and charging capabilities, e.g. within E.164 to identify countries, national destinations, and subscribers.

A numbering plan does not include prefixes, suffixes, and additional information required to complete a call.

The national numbering plan is the national implementation of the E.164 numbering plan.

dialing plan

A string or combination of decimal digits, symbols, and additional information that define the method by which the numbering plan is used. A dialing plan includes the use of prefixes, suffixes, and additional information, supplemental to the numbering plan, required to complete the call.

address

A string or combination of decimal digits, symbols, and additional information which identifies the specific termination point(s) of a connection in a public network(s) or, where applicable, in interconnected private network(s).

prefix

A prefix is an indicator consisting of one or more digits, that allows the selection of different types of number formats, networks and/or service.

international prefix

A digit or combination of digits used to indicate that the number following is an International Public Telecommunication Number.

country code (CC) for geographic areas

The combination of one, two or three digits identifying a specific country, countries in an integrated numbering plan, or a specific geographic area.

national (significant) number [N(S)N]

That portion of the number that follows the country code for geographic areas. The national (significant) number consists of the National Destination Code (NDC) followed by the Subscriber Number (SN). The function and format of the N(S)N is nationally determined.

national destination code (NDC)

A nationally optional code field, within the E.164 number plan, which combined with the Subscriber's Number (SN) will constitute the national (significant) number of the international public telecommunication number for geographic areas. The NDC will have a network and/or trunk code selection function.

The NDC can be a decimal digit or a combination of decimal digits (not including any prefix) identifying a numbering area within a country (or group of countries included in one integrated numbering plan or a specific geographic area) and/or network/services.

national (trunk) prefix

A digit or combination of digits used by a calling subscriber, making a call to a subscriber in his own country but outside his own numbering area. It provides access to the automatic outgoing trunk equipment.

subscriber number (SN)

The number identifying a subscriber in a network or numbering area.

## 8.2	Private Network Number

Private Network Numbers are used in private or virtual private telephony networks, e.g., a corporate network of PBXs and virtual private lines.

ISO/IEC 11571 defines Private Network Number (PNP) as having up to three regional levels.

A PNP Number shall comprise a sequence of x decimal digits (0,1,2,3,4,5,6,7,8,9) with the possibility that different PNP Numbers within the same PNP can have different values of x. The maximum value of x shall be the same as for the public ISDN numbering plan, see ITU-T Recommendation E.164.

Figure – H.323 - Structure of a PNP Number with three levels of regions

A level n Regional Number (RN) shall have significance only within the level n region to which it applies. When that number is used outside that level n region, it shall be in the form of an RN of level greater than n. Only a Complete Number shall have significance throughout the entire PNP.

A typical example in North America would be a 4-digit "extension" as the Level 0 Regional Number: a 3-digit "location code" combined with the 4 digit "extension" would form the Level 1 Regional Number. The Level 2 Regional Number would be nil.

A prefix could also be used to signal which regional number is used, and would not be part of the regional number per se, but only part of the dialing plan. Again, a typical example would be the use of digit "6" to access a Level 1 Regional Number, and no digit for a Level 0 Regional Number.

The following are a set of definitions from ISO/IEC 11571:

Private Numbering Plan (PNP)

The numbering plan explicitly relating to a particular private numbering domain, defined by the PISN Administrator of that domain.

PNP Number

A number belonging to a PNP.

Region

The entire domain or a sub-domain of a PNP. A region does not necessarily correspond to a geographical area of a PISN.

Region Code (RC)

The leading digits of a PNP Number which identify a region. The RC may be omitted to yield a shortened form of a PNP Number for use internally to that region.

Regional Number (RN)

A particular form of a PNP Number which is unambiguous in the region concerned.

Complete Number

A number which is unambiguous in the entire PNP, i.e. which corresponds to the highest regional level employed in that PISN.

# 9	ASN.1 Usage, Guidelines, and Conventions

## 9.1	NULL, BOOLEAN, and NULL/BOOLEAN OPTIONAL

Throughout the ASN.1 used in H.323-series documents, the reader will see the types NULL and BOOLEAN used, along with the modifier OPTIONAL in some cases.  People have questioned when NULL should be used or when BOOLEAN should be used and what the semantic differences are.

The BOOLEAN type allows a TRUE or FALSE value to be conveyed in the protocol.  When used in conjunction with OPTIONAL, it actually allows three values to be conveyed through the protocol: TRUE, FALSE, and absent.  The question is what does absent mean?  In some instances, the absence of a BOOLEAN OPTIONAL means should be interpreted as FALSE, while in other cases, it should be interpreted as "I don't care" or "I don't know"—but not always.  For example, the additiveRegistration field in the RRQ of H.225.0 Version 4 is defined as a BOOLEAN OPTIONAL.  When present, it clearly indicates that the endpoint supports the feature or does not support the feature.  However, absence of this field shall also be interpreted as FALSE.  The reason is that an older endpoint would not know anything about the field and would obviously not be able to include it.  Moreover, they certainly do not support the feature.  Another example is the originator field in the perCallInfo sequence.  When present, the meaning is quite clear: the caller is the originator or the terminator of the call.  However, if the field is not present, it may mean that the endpoint does not know or cannot supply this information for some reason.

The NULL type is often used to select one of several CHOICE options.  NULL carries no particular value, as it merely indicates presence.  In selecting the conference goal in a Setup message, for example, the goal CHOICEes are simply NULL types to allow the endpoint to indicate a selection.  Another common use of NULL is with the OPTIONAL modifier.  A NULL OPTIONAL type allows an endpoint to indicate support for a feature, for example.  It is similar in semantics to a BOOLEAN in that the presence of a NULL field indicates TRUE and absence of the NULL field indicates a FALSE.  As an example, the fastConnectRefused field in the Alerting message is a NULL OPTIONAL.  Absence of the field is interpreted as FALSE—Fast Connect is not (yet) refused.  Presence of the field, though, clearly indicates refusal of Fast Connect.  So why was BOOLEAN not used as the type for this field?  It would not have made the encoding any clearer, because the field is past the extension marker (ellipsis).  A version 1 and 2 device, for example, would not know to send this field, so there would be three values to consider if BOOLEAN were used: TRUE, FALSE, and absent.

Ideally, a field will convey no more values than makes sense.  In most cases, these types indicate only two possible values: TRUE/present or FALSE/absent.  However, there may be cases where three values are intended and the reader should refer to the appropriate Recommendation to determine if, indeed, there is significance in tri-state fields.

## 9.2	ASN.1 Usage in H.450-Series Recommendations

This section summarizes the use of ASN.1 in the current H.450.x recommendations.  This information is provided for implementers of the H.450.x protocols, as well as authors of new H.450.x Recommendations.

### 9.2.1	ASN.1 version and encoding rules

The ASN.1 code in H.450.x is based on the 1994 version of X.680-683, including the amendments on “Rules of extensibility”.

The basic aligned variant of packed encoding rules (PER) is used as specified in X.691 (1995).

### 9.2.2	Tagging

All modules defined in Recommendations H.450.x use the tag default AUTOMATIC TAGS.

The ROS APDUs (see below) are defined in H.450.1 as tagged types within the CHOICE type ROS. No other type defined in H.450.x is a tagged type, i.e. all sets, sequences and choices (except ROS) are automatically tagged.

### 9.2.3	Basic ASN.1 Types

The following types occur in ASN.1 definitions of H.450.x:

| BMPString, NumericString | NULL |
| --- | --- |
| BOOLEAN | OBJECT IDENTIFIER |
| CHOICE | OCTET STRING |
| CLASS (see below) | Open type (see below) |
| ENUMERATED | SEQUENCE |
| GeneralizedTime | SEQUENCE OF |
| INTEGER | SET OF |

No use is currently foreseen for the following basic types (needs consideration on a case-by-case basis):

| CHARACTER STRING | ObjectDescriptor |
| --- | --- |
| EMBEDDED PDV | REAL |
| EXTERNAL | UTCTime |
| GeneralString, GraphicString, PrintableString, TeletexString (T61String), UniversalString, VideotexString, VisibleString (ISO646String) |  |

Use of the following basic types in future recommendations H.450.x should not be precluded (needs consideration on a case-by-case basis):

| BIT STRING | Selection Type (out of a CHOICE) |
| --- | --- |
| IA5String | SET |
| INSTANCE OF | TYPE-IDENTIFIER (see X.681) |

Note: Some of these types are already used by other recommendations in the H.323 universe, e.g. BIT STRING and TYPE-IDENTIFIER in H.235.

### 9.2.4	Value sets, subtyping and constraints used in H.450.x:

H.450.x recommendations use size constraints (strings, set-of and sequence-of) and value range constraints (integers). In H.450.1 inner subtyping (“WITH COMPONENTS”) is used occasionally.

The use of value sets, single values, contained subtypes and permitted alphabets should be possible if needed by future services. The type constraint (for restricting an open type) may be useful, too.

Explicit set arithmetic (UNION, INTERSECTION, EXCEPT, ALL EXCEPT) is currently not used on subtype specifications.

### 9.2.5	Object classes, parameterization, general constraints, and ROS

H.450.1 defines a remote operations service (ROS) based on X.880.  ROS uses object classes (X.681), parameterization (X.683) and constraints (X.682) for its generic part.

Two object classes OPERATION and ERROR are defined and then used to define four PDU types (Invoke, ReturnResult, ReturnError and Reject) as sequences containing individual parts of these classes. The first three PDU types contain an optional open type component which is tied by a table constraint (“at (@)” notation) to the code value identifying the particular operation or error.

For each supplementary service the actual operations and errors are then defined as object instances of the generic classes OPERATION and ERROR in the corresponding Rec. H.450.x. Each operation and error is identified uniquely (within the context of the H.450.x series) by a code value (type INTEGER). A list of currently assigned operation and error values is contained in section 10.8 below.

Each supplementary service defines an object set containing all operations defined for that service.

### 9.2.6	Extensibility and non-standard information

Wherever meaningful, an extension marker (ellipsis “...”) is included in the definitions.

All operations, and some errors, include placeholders for non-standard (e.g. manufacturer-specific) information. This non-standard information can either be of type NonStandardParameter (imported from H.225.0) or of type Extension, which is defined in H.450.1 and consists of an object identifier followed by an open type. The definition of the Extension type uses an object class (EXTENSION) with parameterization and constraints similar to the ROS definition.

Usually there is space for more than one addition of non-standard information in an operation. Additions of both types (NonStandardParameter and Extension) can be mixed in any order.

### 9.2.7	List of Operation and Error Codes

Table 10.1: ASN.1 Operation values used in H.450 series

| Value number | Value name | Defined in standard: |
| --- | --- | --- |
| 0 | callingName | H.450.8 |
| 1 | alertingName | H.450.8 |
| 2 | connectedName | H.450.8 |
| 3 | busyName | H.450.8 |
| 7 | callTransferIdentify | H.450.2 |
| 8 | callTransferAbandon | H.450.2 |
| 9 | callTransferInitiate | H.450.2 |
| 10 | callTransferSetup | H.450.2 |
| 11 | callTransferActive | H.450.2 |
| 12 | callTransferComplete | H.450.2 |
| 13 | callTransferUpdate | H.450.2 |
| 14 | subaddressTransfer | H.450.2 |
| 15 | activateDiversionQ | H.450.3 |
| 16 | deactivateDiversionQ | H.450.3 |
| 17 | interrogateDiversionQ | H.450.3 |
| 18 | checkRestriction | H.450.3 |
| 19 | callRerouting | H.450.3 |
| 20 | divertingLegInformation1 | H.450.3 |
| 21 | divertingLegInformation2 | H.450.3 |
| 22 | divertingLegInformation3 | H.450.3 |
| 23 | cfnrDivertedLegFailed | H.450.3 |
| 27 | ccnrRequest | H.450.9 |
| 28 | ccCancel | H.450.9 |
| 29 | ccExecPossible | H.450.9 |
| 31 | ccRingout | H.450.9 |
| 32 | ccSuspend | H.450.9 |
| 33 | ccResume | H.450.9 |
| 34 | callOfferRequest | H.450.10 |
| 40 | ccbsRequest | H.450.9 |
| 43 | callIntrusionRequest | H.450.11 |
| 44 | callIntrusionGetCIPL | H.450.11 |
| 45 | callIntrusionIsolate | H.450.11 |
| 46 | callIntrusionForcedRelease | H.450.11 |
| 47 | callIntrusionWOBRequest | H.450.11 |
| 49 | cfbOverride | H.450.10 (re-used in H.450.11) |
| 80 | mwiActivate | H.450.7 |
| 81 | mwiDeactivate | H.450.7 |
| 82 | mwiInterrogate | H.450.7 |
| 84 | cmnRequest | H.450.12 |
| 85 | cmnInform | H.450.12 |
| 100 | divertingLegInformation4 | H.450.3 |
| 101 | holdNotific | H.450.4 |
| 102 | retrieveNotific | H.450.4 |
| 103 | remoteHold | H.450.4 |
| 104 | remoteRetrieve | H.450.4 |
| 105 | callWaiting | H.450.6 (re-used in H.450.10, H.450.11) |
| 106 | cpRequest | H.450.5 |
| 107 | cpSetup | H.450.5 |
| 108 | groupIndicationOn | H.450.5 |
| 109 | groupIndicationOff | H.450.5 |
| 110 | pickrequ | H.450.5 |
| 111 | pickup | H.450.5 |
| 112 | pickExe | H.450.5 |
| 113 | cpNotify | H.450.5 |
| 114 | cpickupNotify | H.450.5 |
| 115 | remoteUserAlerting | H.450.10 (re-used in H.450.11) |
| 116 | callIntrusionSilentMonitor | H.450.11 |
| 117 | callIntrusionNotification | H.450.11 |

Table 10.2: ASN.1 Error Values used in H.450 series

| Value number | Value name | Defined in standard: |
| --- | --- | --- |
| 0 | userNotSubscribed | H.450.1 |
| 1 | rejectedByNetwork | H.450.1 |
| 2 | rejectedByUser | H.450.1 |
| 3 | notAvailable | H.450.1 |
| 5 | insufficiantInformation | H.450.1 |
| 6 | invalidServedUserNumber | H.450.1 |
| 7 | invalidCallState | H.450.1 |
| 8 | basicServiceNotProvided | H.450.1 |
| 9 | notIncomingCall | H.450.1 |
| 10 | supplementaryServiceInteractionNotAllowed | H.450.1 |
| 11 | resourceUnavailable | H.450.1 |
| 12 | invalidDivertedNumber | H.450.3 |
| 14 | specialServiceNumber | H.450.3 |
| 15 | diversionToServedUserNumber | H.450.3 |
| 24 | numberOfDiversionsExceeded | H.450.3 |
| 25 | callFailure | H.450.1 |
| 31 | notActivated | H.450.7 |
| 43 | proceduralError | H.450.1 |
| 1000 | temporarilyUnavailable | H.450.3, H.450.11 |
| 1004 | invalidReroutingNumber | H.450.2 |
| 1005 | unrecognizedCallIdentity | H.450.2 |
| 1006 | establishmentFailure | H.450.2 |
| 1007 | notAuthorized | H.450.3, H.450.11 |
| 1008 | unspecified | H.450.2, H.450.3 |
| 1009 | notBusy | H.450.11 |
| 1010 | shortTermRejection | H.450.9 |
| 1011 | longTermRejection | H.450.9 |
| 1012 | remoteUserBusyAgain | H.450.9 |
| 1013 | failureToMatch | H.450.9 |
| 1018 | invalidMsgCentreId | H.450.7 |
| 2000 | callPickupIdUnvalid | H.450.5 |
| 2001 | callAlreadyPickedUp | H.450.5 |
| 2002 | undefined | H.450.4, H.450.5, H.450.7(re-used in H.450.9, H.450.11, H.450.12) |

_____________________

# Annex A: H.323 Recommendation Series Defect Report Form

| DATE: |  |
| --- | --- |
| CONTACT INFORMATION NAME: COMPANY: ADDRESS: TEL: FAX: EMAIL: |  |
| AFFECTED RECOMMENDATIONS: |  |
| DESCRIPTION OF PROBLEM: |  |
| SUGGESTIONS FOR RESOLUTION: |  |

NOTE - Attach additional pages if more space is required than is provided above.

_______________________________