| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2001 - 2004 | COM 7-63-E November 2001 Original: English |

- Question: 12/7

|  | Texte disponible seulement en Text available only in Texto disponible solamente en | E |
| --- | --- | --- |

- STUDY GROUP 7 – CONTRIBUTION 63

SOURCE:	ISO RAPPORTEUR ON DIRECTORY and EDITOR OF THE DIRECTORY IMPLEMENTORS' GUIDE (Q.12/7)

TITLE:	DIRECTORY IMPLEMENTORS' GUIDE - VERSION 15 - August 2001

____________________

NOTE:	This version applies to the 3rd (1997|1998) and 4th (2000|2001) editions of the ITU-T X.500-series of Recommendations and the ISO/IEC 9594 International Standards. It includes all approved and some draft corrigenda to these two editions. Readers still using the 1st edition (1988|1990) are advised to keep version 9 of the Directory Implementors' Guide as that is the last version that contains corrections to the 1st edition text. Readers still using the 2nd edition (1993|1995) are advised to keep version 14 of the Directory Implementors' Guide as that is the last version that contains corrections to the 2nd edition text.

- Agreed at the 7 September 2001 meeting of ITU-T Study Group 7.

CONTENTS

Introduction	3

Defect Report and Resolution Procedures	4

3rd Edition Technical Corrigenda	6

4th Edition Technical Corrigenda	72

Summary of Defect Reports	82

Defect Report Form	88

Defect Resolution Committee	89

Register of ASN.1 Modules Specified External to the Standard	91

# 1	Introduction

## 1.1	Background

This Guide is a compilation of reported defects and their resolutions to the 3rd (1997|1998) and 4th (2000|2001) editions of the ITU-T X.500-series Recommendations and ISO/IEC 9594 International Standards. It includes all approved corrigenda, and may include draft corrigenda, to the editions of the Directory specifications.  It is intended to be an additional authoritative source of information for implementers to be read in conjunction with the Recommendations | International Standards themselves.

This Guide itself is not an ITU-T Recommendation or ISO/IEC International Standard. However, the appendixes of the Guide reproduce approved Technical Corrigenda, which are formal corrections to the Directory specifications. They may also include draft Technical Corrigenda which have no formal standing and which may be overturned or altered during the ballot process.

## 1.2	Scope of the Guide

The Guide records the resolution of defects in the following categories:

•	editorial errors

•	technical errors such as omissions or inconsistencies

•	ambiguities

NOTE:  This Guide does not address proposed additions, deletions, or modifications to the Recommendations or International Standard that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions by national delegates to Question 9/17 within Study Group 17 of the ITU-T or JTC 1/SC 6/WG 7 Directory group of the ISO/IEC.

## 1.3	Contacts and Distribution of the Guide

This Guide is distributed through ITU-T normal contributions, and ISO/IEC JTC 1/ SC 6 N-series documents. It is also available on-line from the ITU (http://www.itu.int) and from a server maintained by the ISO Rapporteur for Directory (ftp://ftp.bull.com/pub/OSIdirectory/).

Contacts:

- ITU Rapporteur for Q.9/17 Directory Services and Systems 2001-2004

- Erik Andersen

CEN/ISSS/WS-DIR

Copenhagen

Denmark

Fax:		+45 39 45 07 77

E-mail:	era@tdcadsl.dk

ISO/IEC Directory Rapporteur and International Defect Report Editor & Editor - Directory Implementors' Guide

Hoyt L. Kesterson II

7625 West Villa Rita Drive

Glendale, Arizona   85318

U.S.A.

Fax:		+1 602 978 6750

E-mail:	hoytkesterson@earthlink.net

- ISO/IEC JTC 1/SC 6 Secretariat

- Jooran Lee

SC 6 Secretariat

Korean Standards Association

#13-31 Yoido-dong, Youngdeungpo-gu

Seoul, 150-010

Republic of Korea

Fax:		+82 2 369 8349

E-mail:	secretariat@jtc1sc06.org

- ITU-T Study Group 17 Secretariat

- Georges Sebek

ITU/TSB

Place des Nations

CH-1211 Geneva 20

Switzerland

Tel:		+41 22 730 5994

Fax:		+41 22 730 5853

E-mail:	sebek@itu.int

# 2	Defect Report and Resolution Procedures

## 2.1	Submission of Defects

Any implementer of the 1997|1998 or 2000|2001 editions of the X.500-series Recommendations or the ISO/IEC 9594 International Standard is invited to submit a Directory defect report using the form found in Appendix D of the Guide. The defect report should be submitted to the appropriate National Defect Report Editor, listed in Appendix E. Each form should cover a single defect. It is important that the form is completed accurately, especially the sections that relate to the base material against which the defect report is being raised.

## 2.2	Resolution of Defects

A collaborative Directory Defect Resolution Committee has been established to resolve reported defects. In the case of most countries, a single representative has been nominated to the committee from the ITU Administration and the ISO/IEC JTC 1 National Body.

Following agreement on a resolution, within the collaborative Defect Resolution Committee, the proposed resolution may require approval via ballot of ISO/IEC and by the ITU-T.

Please note that no individual responses can be given to those submitting reports, and that the procedure is not intended as a consulting service.

# 3.	Guide to Appendixes

The six appendixes of this Guide are organized as follows:

Appendix A is a collection of the approved Technical Corrigenda (TC) and Draft Technical Corrigenda (DTC) to the 3rd edition of the Directory specifications. The Directory specifications are arranged in the ISO/IEC order (Parts 1 to 10).

After the ballot comments on a DTC are resolved it is either published as a TC or incorporated into an edition of the Directory standard. The number of the TC may be different from the DTC; the mapping is documented in summary at the beginning of the annex.

Appendix B is a collection of the approved Technical Corrigenda and Draft Technical Corrigenda to the 4th edition of the Directory specifications. The Directory specifications are arranged in the ISO/IEC order (Parts 1 to 10).

After the ballot comments on a DTC are resolved it is either published as a TC or incorporated into an edition of the Directory standard. The number of the TC may be different from the DTC; the mapping is documented in summary at the beginning of the annex.

Appendix C is a summary of the Defect Reports to the 3rd and 4th editions. Defect reports up to and including 074 apply to the 1st edition only and are not documented in this version of the Implementors' Guide – see Version 9 for a description of those defects. Defects 075–156, 158, 160, 161, 165, 168, 171, 172, 174, and 175 apply to the 2nd edition only and are not documented in this version of the Implementors' Guide – see Version 14 for a description of those defects.

Appendix D is a pro forma defect reporting form. This form, or one like it, should be used for reporting defects. The defect should be submitted as an electronic copy to ease the editor’s task.

- Appendix E is a list of Defect Editors with their contact information.

- Appendix F is a register of OIDs for the module names of ASN.1 modules that are specified outside of the directory specifications.

Appendix A

Technical Corrigenda to Rec. X.500 (1997) | ISO/IEC 9594:1998 3rd Edition

## Summary of 3rd Edition Technical Corrigenda

| DTC # | Defect Reports resolved | Ballot Close | Published As | History |
| --- | --- | --- | --- | --- |
| ITU-T Rec. X.500 (1997) ¦ ISO/IEC 9594-1:1998 |  |  |  |  |
| 1-DTC1 | 228 | 10 Jan 2001 | withdrawn | Erik after Orlando 2000 |
| ### ITU-T Rec. X.501 (1997) ¦ ISO/IEC 9594-2:1998 |  |  |  |  |
| 2-DTC1 | 173, 179, 189, 205 |  | 2-TC1 | Patrick after Orlando 99 |
| 2-DTC2 | 211 |  | 2-TC1 | Hoyt after Orlando 99 |
| 2-DTC3 | 229, 230 |  | 2-TC2 |  |
| 2-DTC4 | 228, 242, 255, 260, 261, 267, 269 | 10 Jan 2001 | 2-TC2 | Erik after Orlando 2000 |
| ### ITU-T Rec. X.511 (1997) ¦ ISO/IEC 9594-3:1998 |  |  |  |  |
| 3-DTC1 | 166, 179, 188, 202, 206, 217 |  | 3-TC1 | Patrick after Orlando 99 |
| 3-DTC2 | 211 |  | 3-TC1 | Hoyt after Orlando 99 |
| 3-DTC3 | 231, 232 |  | 3-TC2 |  |
| 3-DTC4 | 247 |  | 3-TC2 |  |
| 3-DTC5 | 224, 228, 242, 263 | 10 Jan 2001 | 3-TC2 | Erik after Orlando 2000 |
| ### ITU-T Rec. X.518 (1997) ¦ ISO/IEC 9594-4:1998 |  |  |  |  |
| 4-DTC1 | 157,159,162,180, 190, 198, 206, 209 |  | 4-TC1 | Patrick after Orlando 99 |
| 4-DTC2 | 211 |  | 4-TC1 | Hoyt after Orlando 99 |
| 4-DTC3 | 233, 235 |  | 4-TC2 |  |
| 4-DTC4 | 234, 248 |  | 4-TC2 |  |
| 4-DTC5 | 228, 242, 265 | 10 Jan 2001 | 4-TC2 | Erik after Orlando 2001 |
| ### ITU-T Rec. X.519 (1997) ¦ ISO/IEC 9594-5:1998 |  |  |  |  |
| 5-DTC1 | 221 |  | 5-TC1 | Ella after Orlando 99 |
| 5-DTC2 | 236 |  | 5-TC2 |  |
| 5-DTC3 | 228, 242, 266 | 10 Jan 2001 | 5-TC2 | Erik after Orlando 2000 |
| ### ITU-T Rec. X.520 (1997) ¦ ISO/IEC 9594-6:1998 |  |  |  |  |
| 6-DTC1 | 211 |  | 6-TC1 | Hoyt after Orlando 99 |
| 6-DTC2 | 237, 238, 241 |  | 6-TC2 |  |
| 6-DTC3 | 270 | 10 Jan 2001 | 6-TC2 | Erik after Orlando 2001 |
| ### ITU-T Rec. X.521 (1997) ¦ ISO/IEC 9594-7:1998 |  |  |  |  |
| 7-DTC1 | 239 |  | 7-TC1 |  |
| ### ITU-T Rec. X.509 (1997) ¦ ISO/IEC 9594-8:1998 |  |  |  |  |
| 8-DTC1 | 183, 194 |  | 3rd edition | Incorporated into published edition |
| 8-DTC3 | 200, 201, 212, 213, 218, 220 |  | 8-TC1 | Sharon after Orlando 99 |
| 8-DTC4 | 185 |  | 8-TC1 | Sharon after Orlando 99 |
| 8-DTC5 | 204 |  | 8-TC1 | Sharon after Orlando 99 |
| 8-DTC7 | 222 |  | 8-TC1 | Sharon after Orlando 99 |
| 8-DTC8 | 226, 227, 240 |  | 8-TC2 |  |
| 8-DTC9 | 244, 256, 257, 258 |  | 8-TC2 | Sharon after Orlando 2000, comments resolved at Geneva 2001 |
| ### ITU-T Rec. X.525 (1997) ¦ ISO/IEC 9594-9:1998 |  |  |  |  |
| 9-DTC1 | 182, 186 |  | 9-TC1 | Processed at Helsinki 97 and produced by Hoyt after Orlando 99 |
| 9-DTC2 | 187, 208, 243 |  | 9-TC2 |  |
| 9-DTC3 | 245 |  | 9-TC2 |  |
| 9-DTC4 | 228, 242 | 10 Jan 2001 | 9-TC2 | Erik after Orlando 2001 |
| ### ITU-T Rec. X.530 (1997) ¦ ISO/IEC 9594-10:1998 |  |  |  |  |
| 10-DTC1 | 252 | 10 Jan 2001 | 10-TC1 | Erik after Orlando 2001 |

RECOMMENDATION X.501 (1997) | ISO/IEC 9594-2:1998

Information technology – Open Systems Interconnection –
The Directory: Models

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 1 and 2.

# Defect reports resolved by Draft Technical Corrigendum 1

(defect reports 173, 179, 189)

____________________________________________

# 1)	Resolution to defect report 9594/173

## Subclause 20.5 First Level DSAs

Change the text of bullet c) as follows:

c)	it holds subordinate references (of category master and/or shadow) and non-specific subordinate references (of category master and/or shadow) which account for all the naming contexts immediately subordinate to the root of the DIT which it does not itself hold.

# 2)	Resolution to defect report 9594/179

## Annex J, Table J.1

In the second column called "Entry Protected Item Permissions Required", add the following texts for the Read and the Search operations:

for the Read operation:

"ReturnDN for distinguished name"

for the Search Operation:

"returnDN for each returned distinguished name"

# 3)	Resolution to defect report 9594/189

## Subclause 26.3 Modify Operational Binding and Annex F

Add OPTIONAL to the ASN.1 of newAgreement as follows:

NewAgreement		[7]		OPERATIONAL-BINDING.&Agreement
({OpBindingSet}{@bindingType}) OPTIONAL,

# 4)	Resolution to defect report 9594/205

## Subclause 20.3.2 Knowledge Reference Types

Change the first bullet point after "A DSA may hold the following types of knowledge reference:" to read:

–	superior references;

## Subclause 20.3.2.1 Superior Reference

Change the title and second sentence to read:

#### 20.3.2.1	Superior References

A superior reference consists of:

–	the Access Point of a DSA.

Each non-first level DSA (see 20.5) shall maintain at least one superior reference.

## Subclause 20.4.1 Superior Knowledge

Change the first sentence to read:

Each DSA that is not a first level DSA shall maintain at least one superior reference.

And add the following second sentence:

Additional superior references may be held for operational reasons as alternative paths to the root of the DIT.

## Subclause 20.5 First Level DSAs

Change the second sentence to read:

"A DSA referenced by other DSAs may itself maintain one or more superior references."

Change the last sentence to read:

"They therefore may serve as a superior reference for non-first level DSAs."

## Subclause 21.4.2 DSE Types

Change the text of bullet h) to read as follows:

h)	supr: A DSE that holds a specific knowledge attribute to represent the DSAs superior references.

## Subclause 22.2.1.2 Superior Knowledge

Change the first sentence to the plural and the ATTRIBUTE WITH SYNTAX to read as follows:

The superiorKnowledge operational attribute type is used by a non-first level DSA to represent its superior references.

superiorKnowledge			ATTRIBUTE				::=		{
						WITH SYNTAX					SET OF AccessPoint
						.....

## Subclause 22.2.2.2 Superior Reference

Insert a new second sentence:

Since a superiorKnowledge attribute value may contain the access points of several DSAs, it may therefore represent several superior references.

# Defect reports resolved by Draft Technical Corrigendum 2

(defect report 211)

____________________________________________

# 5)	Resolution to defect report 9594/211

## Subclause 26.2

Change the two occurrences of UTCTime to Time:

Insert the following after the ASN.1 definition of Validity

Time ::= CHOICE {
		utcTime 			UTCTime,
		generalizedTime		GeneralizedTime  }

Before a value of Time is used in any comparison operation and if the syntax of Time has been chosen as the UTCTime type, the value of the two-digit year field shall be rationalized into a four-digit year value as follows:

–	If the 2-digit value is 00 through 49 inclusive, the value shall have 2000 added to it.

–	If the 2-digit value is 50 through 99 inclusive, the value shall have 1900 added to it.

NOTE – The use of GeneralizedTime may prevent interworking with implementations unaware of the possibility of choosing either UTCTime or GeneralizedTime. It is the responsibility of those specifying the domains in which this Directory Specification will be used, e.g. profiling groups, as to when the GeneralizedTime may be used. In no case shall UTCTime be used for representing dates beyond 2049.

## Subclause 26.4

Change UTCTime to Time:

## Subclause 26.5

Change UTCTime to Time:

Also make the ASN.1 changes to Annex F.

RECOMMENDATION X.501 (1997) | ISO/IEC 9594-2:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 3, and 4.

# 1)	Defect reports covered by Draft Technical Corrigendum 3

(Covering resolutions to defect reports 229 and 230.)

## 1.1)	This corrects the defects reported in defect reports 9594/229-230

In 2.1:

Replace:

–	ITU-T Recommendation X.525 (1997) | ISO/IEC 9594-8:1999, Information technology – Open Systems Interconnection – The Directory: Replication.

with:

–	ITU-T Recommendation X.525 (1997) | ISO/IEC 9594-9:1998, Information technology – Open Systems Interconnection – The Directory: Replication.

In 17.4.3:

In the attributeValueSecurityLabelContext specification replace SYNTAX with WITH SYNTAX

Delete the KeyIdentifier type.

Introduce the same changes in Annex P.

In 18.1.2:

Change the 4th paragraph to:

Digital signatures applied to the whole entry do not include operational, collective attributes or the attributeIntegrityInfo itself. Any attribute value contexts are included.

Delete the 5th paragraph (beginning "Additional control information …").

Change the attributeIntegrityInfo attribute definition and its supporting definitions to:

attributeIntegrityInfo ATTRIBUTE  ::=  {
		WITH SYNTAX				AttributeIntegrityInfo
		ID							id-at-attributeIntegrityInfo}

AttributeIntegrityInfo  ::=  SIGNED { SEQUENCE  {
		scope			Scope,						-- Identifies the attributes protected
		signer			Signer	OPTIONAL,			-- Authority or data originators name
		attribsHash  		AttribsHash } }				-- Hash value of protected attributes

Signer  ::=  CHOICE {
		thisEntry		[0]	EXPLICIT ThisEntry,
		thirdParty	[1]	SpecificallyIdentified }

ThisEntry  ::=  CHOICE  {
		onlyOne  NULL,
		specific IssuerAndSerialNumber }

IssuerAndSerialNumber  ::=  SEQUENCE {
		issuer	Name,
		serial CertificateSerialNumber }

SpecificallyIdentified  ::=  SEQUENCE  {
		name   	GeneralName,
		issuer	GeneralName  OPTIONAL,
		serial   	CertificateSerialNumber  OPTIONAL }
		( WITH COMPONENTS { …, issuer PRESENT, serial PRESENT } |
		( WITH COMPONENTS { …, issuer ABSENT, serial ABSENT } ) )

Scope  ::=  CHOICE  {
		wholeEntry  	[0]	NULL,		-- Signature protects all attribute values in this entry
		selectedTypes	[1]	SelectedTypes
						-- Signature protects all attribute values of the selected attribute types 
		}

SelectedTypes  ::=  SEQUENCE SIZE (1..MAX) OF AttributeType

AttribsHash  ::=  HASH { SEQUENCE SIZE (1..MAX) OF Attribute }
			-- Attribute type and values with associated context values for the selected Scope

Add the following text after the above ASN.1:

An AttributeIntegrityInfo value can be created in three different ways:

a)	An administrative authority can create and sign the value, and the public key to verify the signature is known by off-line means.

b)	The owner of the entry, i.e. the object represented by the entry, can create and sign the value. If the owner has several certificates, or expected to have that in the future, the certificate has to be identified by the CA issuing the certificate together with the certificate serial number.

c)	A third party may create and sign the value. The name of the signer, the name of the CA issuing the certificate and the certificate serial number is required.

If the scope is wholeEntry, all the applicable attributes shall be ordered as specified for a set-of type in 6.1 of ITU-T Rec. X.509 | ISO/IEC 9594-8. If scope is selectedTypes, the ordering shall be the same as the one given in the SelectedTypes.

NOTE – If a user does not retrieve all the complete attributes that are defined within the Scope data type, it will not be possible for the user to verify the integrity of the attributes.

Delete 18.1.2.1.

Introduce the same changes to ASN.1 in Annex P.

Replace 18.1.3 with the following:

### 18.1.3	Context for Protection of a Single Attribute Value

The following defines a context to hold a digital signature, along with associated control information, which provides integrity for a single attribute value. Any attribute value contexts are included in the integrity check, excluding the context used to hold signatures.

attributeValueIntegrityInfoContext  CONTEXT  ::=  {
	WITH SYNTAX	AttributeValueIntegrityInfo
	ID			id-avc-attributeValueIntegrityInfoContext }

AttributeValueIntegrityInfo  ::=  SIGNED { SEQUENCE {
	signer			Signer	OPTIONAL,  		-- Authority or data originators name
	aVIHash		AVIHash } }  				-- Hash value of protected attribute

AVIHash  ::=  HASH { AttributeTypeValueContexts }
			-- Attribute type and value with associated context values

AttributeTypeValueContexts ::= SEQUENCE {
	type			ATTRIBUTE.&id ({SupportedAttributes}),
	value  			ATTRIBUTE.&Type ({SupportedAttributes}{@type}),
	contextList  	SET SIZE (1..MAX) OF Context OPTIONAL }

The contextList shall be ordered as specified for a set-of type in 6.1 of ITU-T Rec. X.509 | ISO/IEC 9594-8.

Change the ASN.1 in Annex P as above and delete AVIAssertion data type.

In Annex B:

Delete OPTIONALLY-SIGNED import from DirectoryAbstractService

In Annex C:

In the application component of AttributeTypeInformation replace userApplication with userApplications

In Annex D:

Add directoryAbstractService to the import from UsefulDefinitions

Add SupportedAttributes to the import from InformationFramework

Add:

Filter
			FROM DirectoryAbstractService directoryAbstractService

In Annex F:

Add enhancedSecurity to the import from UsefulDefinitions

Delete OPTIONALLY-PROTECTED and DIRQOP from the import from EnhancedSecurity. Add instead OPTIONALLY-PROTECTED-SEQ.

In Annex P:

All the changes to Annex P have been subsumed by the resolution of defect report 228.

# 2)	Defect reports covered by Draft Technical Corrigendum 4

(Covering resolutions to defect reports 228, 242, 255, 260, 261, 267 and 269.)

## 2.1)	This corrects the defects reported in defect report 9594/228

Insert the following between 15.3 and 15.3.1:

Warning – Subclauses 15.3.1 and 15.3.2 are known to contain invalid specifications. These subclauses are therefore deprecated. A future edition will either remove the deprecated specifications or provide updated text.

The following specifications are provided to preserve the optionally signed capability provided by edition 2 of these Directory Specifications and to allow that capability to be extended to all operations and to errors:

OPTIONALLY-PROTECTED is a parameterized data type where the parameter is a data type whose values may, at the option of the generator, be accompanied by their digital signature. This capability is specified by means of the following type:

OPTIONALLY-PROTECTED { Type }  ::=  CHOICE {
	unsigned		Type,
	signed			SIGNED {Type} }

The OPTIONALLY-PROTECTED-SEQ is used instead of OPTIONALLY-PROTECTED when the protected data type is a sequence data type that is not tagged.

OPTIONALLY-PROTECTED-SEQ { Type } ::= CHOICE {
	unsigned		Type,
	signed	[0]		SIGNED { Type } }

The SIGNED parameterized data type, which describes the form of the signed form of the information, is specified in ITU-T Rec. X.509 | ISO/IEC 9594-8.

Insert the following 18.2 and 18.2.1:

Warning – This subclause is known to contain invalid specifications. This subclause is therefore deprecated. A future edition will either remove the deprecated specifications or provide updated text.

In Annex A, add ASN.1 comment items as shown:

-- securityExchange				ID	::=	{ds 32}

-- directorySecurityExchanges			ID	::=	{module directorySecurityExchanges (29) 1}

-- id-se   						ID	::=	securityExchange

In clause 26, delete all occurrence of:

,
	DIRQOP.&…-QOP{@dirqop}

and change all occurrences of:

OPTIONALLY-PROTECTED

to:

OPTIONALLY-PROTECTED-SEQ

Introduce the same changes to Annex F.

Replace Annex P with the following:

Annex  P

Enhanced security

(This annex forms an integral part of this Recommendation | International Standard)

This module is known to contain invalid specifications. Part of this module is therefore deprecated. The deprecated part is indicated by ASN.1 comment items. A future edition will either remove the deprecated specifications or provide updated specifications.

EnhancedSecurity { joint-iso-itu-t ds(5) modules(1) enhancedSecurity(28) 1 }

DEFINITIONS IMPLICIT TAGS  ::=

BEGIN

-- EXPORTS All --

IMPORTS

-- from ITU-T Rec. X.501 | ISO/IEC 9594-2

authenticationFramework, basicAccessControl, certificateExtensions, id-at, id-avc, id-mr,
	informationFramework, upperBounds
		FROM UsefulDefinitions { joint-iso-itu-t ds(5) module(1) usefulDefinitions(0) 3 }

Attribute, ATTRIBUTE, AttributeType, Context, CONTEXT, MATCHING-RULE, Name,
	objectIdentifierMatch, SupportedAttributes
		FROM InformationFramework informationFramework

AttributeTypeAndValue

FROM BasicAccessControl basicAccessControl

-- from ITU-T Rec. X.509 | ISO/IEC 9594-8

AlgorithmIdentifier, CertificateSerialNumber, ENCRYPTED{}, HASH{}, SIGNED{}

FROM AuthenticationFramework authenticationFramework

GeneralName, KeyIdentifier

FROM CertificateExtensions certificateExtensions

ub-privacy-mark-length
		FROM UpperBounds upperBounds	;

-- from GULS

-- SECURITY-TRANSFORMATION, PROTECTION-MAPPING, PROTECTED
--	FROM Notation { joint-iso-ccitt genericULS (20) modules (1) notation (1) }

-- dirSignedTransformation, KEY-INFORMATION

--	FROM GulsSecurityTransformations { joint-iso-ccitt genericULS (20) modules (1)

--		gulsSecurityTransformations (3) }

-- signed

--	FROM GulsSecurityTransformations { joint-iso-ccitt genericULS (20) modules (1)

--		dirProtectionMappings (4) };

-- The "signed" Protection Mapping and associated "dirSignedTransformations" imported

-- from the Generic Upper Layers Security specification (ITU-T Rec. X.830 | ISO/IEC 11586-1)

-- results in identical encoding as the same data type used with the SIGNED as defined in

-- ITU-T REC. X.509 | ISO/IEC 9594-8

-- The three statements below are provided temporarily to allow signed operations to be supported as in edition 3.

OPTIONALLY-PROTECTED { Type }  ::=  CHOICE {
	unsigned		Type,
	signed		SIGNED {Type} }

OPTIONALLY-PROTECTED-SEQ { Type } ::= CHOICE {
	unsigned		Type,
	signed	[0]	SIGNED { Type } }

-- The following out-commented ASN.1 specifications are known to be erroneous and are therefore deprecated.

-- genEncryptedTransform {KEY-INFORMATION: SupportedKIClasses }  SECURITY-TRANSFORMATION  ::=  
--	{
--	IDENTIFIER  			{ enhancedSecurity  gen-encrypted(2) }
--	INITIAL-ENCODING-RULES	{ joint-iso-itu-t  asn1(1)  ber(1) }

-- This default for initial encoding rules may be overridden
						-- using a static protected parameter (initEncRules).

--	XFORMED-DATA-TYPE  	SEQUENCE {

--			initEncRules	OBJECT IDENTIFIER DEFAULT { joint-iso-itu-t  asn1(1)  ber(1) },
--			encAlgorithm	AlgorithmIdentifier OPTIONAL, --  -- Identifies the encryption algorithm,
--			keyInformation	SEQUENCE {
--				kiClass	KEY-INFORMATION.&kiClass ({SupportedKIClasses}),
--				keyInfo	KEY-INFORMATION.&KiType ({SupportedKIClasses} {@kiClass})
--						} OPTIONAL,

-- Key information may assume various formats, governed by supported members
				-- of the KEY-INFORMATION information object class (defined in ITU-T
				-- Rec. X.830 | ISO/IEC 11586-1)

--			encData	BIT STRING ( CONSTRAINED BY {

-- the encData value must be generated following 
				-- the procedure specified in 17.3.1-- })
--			}
--	}

-- encrypted  PROTECTION-MAPPING  ::=  {

--	SECURITY-TRANSFORMATION { genEncryptedTransform } }

-- signedAndEncrypt  PROTECTION-MAPPING  ::=  {

--	SECURITY-TRANSFORMATION  { signedAndEncryptedTransform } }

-- signedAndEncryptedTransform {KEY-INFORMATION: SupportedKIClasses}
--	SECURITY-TRANSFORMATION  ::=  {
--		IDENTIFIER 			{ enhancedSecurity  dir-encrypt-sign (1) }
--		INITIAL-ENCODING-RULES	{ joint-iso-itu-t asn1 (1) ber-derived (2) distinguished-encoding (1) }
--		XFORMED-DATA-TYPE
--			PROTECTED
--			{
--				PROTECTED
--				{
--				ABSTRACT-SYNTAX.&Type,
--				signed
--				},
--			encrypted
--			}
--	}

-- OPTIONALLY-PROTECTED {ToBeProtected, PROTECTION-MAPPING:generalProtection} ::=

--	CHOICE {
--		toBeProtected		ToBeProtected,

-- no DIRQOP specified for operation

--		signed					PROTECTED {ToBeProtected, signed},

-- DIRQOP is Signed

--		protected			[APPLICATION 0]
--										PROTECTED { ToBeProtected, generalProtection } }

-- DIRQOP is other than Signed

-- defaultDirQop ATTRIBUTE  ::=  {
--	WITH SYNTAX							OBJECT IDENTIFIER
--	EQUALITY MATCHING RULE  	objectIdentifierMatch
--	USAGE										directoryOperation
--	ID						id-at-defaultDirQop }

-- DIRQOP  ::=  CLASS
-- This information object class is used to define the quality of protection 
-- required throughout directory operation. 
-- The Quality Of Protection can be signed, encrypted, signedAndEncrypt
-- {
--	&dirqop-Id    												OBJECT IDENTIFIER UNIQUE,
--	&dirBindError-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dirErrors-QOP												PROTECTION-MAPPING:protectionReqd,
--	&dapReadArg-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dapReadRes-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dapCompareArg-QOP  								PROTECTION-MAPPING:protectionReqd,
--	&dapCompareRes-QOP  								PROTECTION-MAPPING:protectionReqd,
--	&dapListArg-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dapListRes-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dapSearchArg-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dapSearchRes-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dapAbandonArg-QOP 				 				PROTECTION-MAPPING:protectionReqd,
--	&dapAbandonRes-QOP  								PROTECTION-MAPPING:protectionReqd,
--	&dapAddEntryArg-QOP  								PROTECTION-MAPPING:protectionReqd,
--	&dapAddEntryRes-QOP								PROTECTION-MAPPING:protectionReqd,
--	&dapRemoveEntryArg-QOP							PROTECTION-MAPPING:protectionReqd,
--	&dapRemoveEntryRes-QOP							PROTECTION-MAPPING:protectionReqd,
--	&dapModifyEntryArg-QOP								PROTECTION-MAPPING:protectionReqd,
--	&dapModifyEntryRes-QOP							PROTECTION-MAPPING:protectionReqd,
--	&dapModifyDNArg-QOP								PROTECTION-MAPPING:protectionReqd,
--	&dapModifyDNRes-QOP								PROTECTION-MAPPING:protectionReqd,
--	&dspChainedOp-QOP										PROTECTION-MAPPING:protectionReqd,
--	&dispShadowAgreeInfo-QOP				  			PROTECTION-MAPPING:protectionReqd,
--	&dispCoorShadowArg-QOP							PROTECTION-MAPPING:protectionReqd,
--	&dispCoorShadowRes-QOP							PROTECTION-MAPPING:protectionReqd,
--	&dispUpdateShadowArg-QOP						PROTECTION-MAPPING:protectionReqd,
--	&dispUpdateShadowRes-QOP						PROTECTION-MAPPING:protectionReqd,
--	&dispRequestShadowUpdateArg-QOP				PROTECTION-MAPPING:protectionReqd,
--	&dispRequestShadowUpdateRes-QOP				PROTECTION-MAPPING:protectionReqd,
--	&dopEstablishOpBindArg-QOP					PROTECTION-MAPPING:protectionReqd,
--	&dopEstablishOpBindRes-QOP					PROTECTION-MAPPING:protectionReqd,
--	&dopModifyOpBindArg-QOP   						PROTECTION-MAPPING:protectionReqd,
--	&dopModifyOpBindRes-QOP   						PROTECTION-MAPPING:protectionReqd,
--	&dopTermOpBindArg-QOP							PROTECTION-MAPPING:protectionReqd,
--	&dopTermOpBindRes-QOP							PROTECTION-MAPPING:protectionReqd
-- }
-- WITH SYNTAX
-- {
--	DIRQOP-ID   												&dirqop-Id
--	DIRECTORYBINDERROR-QOP						&dirBindError-QOP
--	DIRERRORS-QOP										&dirErrors-QOP
--	DAPREADARG-QOP										&dapReadArg-QOP
--	DAPREADRES-QOP										&dapReadRes-QOP
--	DAPCOMPAREARG-QOP								&dapCompareArg-QOP
--	DAPCOMPARERES-QOP								&dapCompareRes-QOP
--	DAPLISTARG-QOP										&dapListArg-QOP
--	DAPLISTRES-QOP										&dapListRes-QOP
--	DAPSEARCHARG-QOP  				 				&dapSearchArg-QOP
--	DAPSEARCHRES-QOP   								&dapSearchRes-QOP
--	DAPABANDONARG-QOP								&dapAbandonArg-QOP
--	DAPABANDONRES-QOP								&dapAbandonRes-QOP
--	DAPADDENTRYARG-QOP								&dapAddEntryArg-QOP
--	DAPADDENTRYRES-QOP								&dapAddEntryRes-QOP
--	DAPREMOVEENTRYARG-QOP						&dapRemoveEntryArg-QOP
--	DAPREMOVEENTRYRES-QOP						&dapRemoveEntryRes-QOP
--	DAPMODIFYENTRYARG-QOP  						&dapModifyEntryArg-QOP
--	DAPMODIFYENTRYRES-QOP 			 			&dapModifyEntryRes-QOP
--	DAPMODIFYDNARG-QOP								&dapModifyDNArg-QOP
--	DAPMODIFYDNRES-QOP								&dapModifyDNRes-QOP
--	DSPCHAINEDOP-QOP									&dspChainedOp-QOP
--	DISPSHADOWAGREEINFO-QOP					&dispShadowAgreeInfo-QOP
--	DISPCOORSHADOWARG-QOP						&dispCoorShadowArg-QOP
--	DISPCOORSHADOWRES-QOP						&dispCoorShadowRes-QOP
--	DISPUPDATESHADOWARG-QOP					&dispUpdateShadowArg-QOP
--	DISPUPDATESHADOWRES-QOP					&dispUpdateShadowRes-QOP
--	DISPREQUESTSHADOWUPDATEARG-QOP		&dispRequestShadowUpdateArg-QOP
--	DISPREQUESTSHADOWUPDATERES-QOP		&dispRequestShadowUpdateRes-QOP
--	DOPESTABLISHOPBINDARG-QOP 			 		&dopEstablishOpBindArg-QOP
--	DOPESTABLISHOPBINDRES-QOP   				&dopEstablishOpBindRes-QOP
--	DOPMODIFYOPBINDARG-QOP						&dopModifyOpBindArg-QOP
--	DOPMODIFYOPBINDRES-QOP						&dopModifyOpBindRes-QOP
--	DOPTERMINATEOPBINDARG-QOP  		 		&dopTermOpBindArg-QOP
--	DOPTERMINATEOPBINDRES-QOP   				&dopTermOpBindRes-QOP
-- }

attributeValueSecurityLabelContext CONTEXT ::= {
	WITH SYNTAX	SignedSecurityLabel	--  At most one security label context can be assigned to an
								--  attribute value
	ID			id-avc-attributeValueSecurityLabelContext }

SignedSecurityLabel  ::=  SIGNED {SEQUENCE  {
	attHash		HASH {AttributeTypeAndValue},
	issuer		Name   		OPTIONAL,  -- name of labelling authority
	keyIdentifier	KeyIdentifier	OPTIONAL,
	securityLabel	SecurityLabel } }

SecurityLabel  ::=  SET {
	security-policy-identifier	SecurityPolicyIdentifier   	OPTIONAL,
	security-classification		SecurityClassification		OPTIONAL,
	privacy-mark			PrivacyMark			OPTIONAL,
	security-categories		SecurityCategories		OPTIONAL }
		(ALL EXCEPT ( {-- none, at least one component shall be present -- } ) )

SecurityPolicyIdentifier  ::=  OBJECT IDENTIFIER

SecurityClassification ::= INTEGER {
	unmarked		(0),
	unclassified	(1),
	restricted		(2),
	confidential   	(3),
	secret		(4),
	top-secret		(5) }

PrivacyMark  ::=  PrintableString (SIZE (1..ub-privacy-mark-length))

SecurityCategories ::= SET SIZE (1..MAX) OF SecurityCategory

clearance  ATTRIBUTE  ::=  {
	WITH SYNTAX	Clearance
	ID			id-at-clearance }

Clearance  ::=  SEQUENCE  {
	policyId			OBJECT IDENTIFIER,
	classList			ClassList				DEFAULT {unclassified},
	securityCategories  	SET SIZE (1..MAX) OF SecurityCategory	OPTIONAL }

ClassList  ::=  BIT STRING {
	unmarked		(0),
	unclassified	(1),
	restricted		(2),
	confidential  	(3),
	secret		(4),
	topSecret		(5) }

SecurityCategory  ::=  SEQUENCE {
	type		[0]	SECURITY-CATEGORY.&id ({SecurityCategoriesTable}),
	value   	[1]	EXPLICIT SECURITY-CATEGORY.&Type ({SecurityCategoriesTable} {@type}) }

SECURITY-CATEGORY  ::=  TYPE-IDENTIFIER

SecurityCategoriesTable SECURITY-CATEGORY ::= { ... }

attributeIntegrityInfo ATTRIBUTE  ::=  {
	WITH SYNTAX				AttributeIntegrityInfo
	ID						id-at-attributeIntegrityInfo }

AttributeIntegrityInfo  ::=  SIGNED { SEQUENCE  {
	scope		Scope,				-- Identifies the attributes protected
	signer		Signer	OPTIONAL,  	-- Authority or data originators name
	attribsHash  	AttribsHash } }			-- Hash value of protected attributes

Signer  ::=  CHOICE {
	thisEntry	[0]	EXPLICIT ThisEntry,
	thirdParty	[1]	SpecificallyIdentified }

ThisEntry  ::=  CHOICE  {
	onlyOne	NULL,
	specific	IssuerAndSerialNumber }

IssuerAndSerialNumber  ::=  SEQUENCE {
	issuer	Name,
	serial   	CertificateSerialNumber }

SpecificallyIdentified  ::=  SEQUENCE  {
	name   	GeneralName,
	issuer	GeneralName			OPTIONAL,
	serial   	CertificateSerialNumber	OPTIONAL }
	( WITH COMPONENTS { …, issuer PRESENT, serial PRESENT } |
	( WITH COMPONENTS { …, issuer ABSENT, serial ABSENT } ) )

Scope  ::=  CHOICE  {
	wholeEntry  	[0]	NULL,		-- Signature protects all attribute values in this entry
	selectedTypes	[1]	SelectedTypes
					-- Signature protects all attribute values of the selected attribute types 
	}

SelectedTypes  ::=  SEQUENCE SIZE (1..MAX) OF AttributeType

AttribsHash  ::=  HASH { SEQUENCE SIZE (1..MAX) OF Attribute }
			-- Attribute type and values with associated context values for the selected Scope

attributeValueIntegrityInfoContext  CONTEXT  ::=  {
	WITH SYNTAX	AttributeValueIntegrityInfo
	ID			id-avc-attributeValueIntegrityInfoContext }

AttributeValueIntegrityInfo  ::=  SIGNED { SEQUENCE {
	signer		Signer	OPTIONAL,		-- Authority or data originators name
	aVIHash		AVIHash } }  			-- Hash value of protected attribute

AVIHash  ::=  HASH { AttributeTypeValueContexts }
			-- Attribute type and value with associated context values

AttributeTypeValueContexts ::= SEQUENCE {
	type			ATTRIBUTE.&id ({SupportedAttributes}),
	value  		ATTRIBUTE.&Type ({SupportedAttributes}{@type}),
	contextList  	SET SIZE (1..MAX) OF Context OPTIONAL }

-- The following out-commented ASN.1 specification are known to be erroneous and are therefore deprecated.

-- EncryptedAttributeSyntax {AttributeSyntax}  ::=  SEQUENCE  {
--	keyInfo	SEQUENCE OF KeyIdOrProtectedKey,
--	encAlg	AlgorithmIdentifier,
--	encValue	ENCRYPTED { AttributeSyntax } }

-- KeyIdOrProtectedKey  ::=  SEQUENCE  {
--	keyIdentifier	[0]	KeyIdentifier	OPTIONAL,
--	protectedKeys	[1]	ProtectedKey	OPTIONAL }
		-- At least one key identifier or protected key must be present

-- ProtectedKey  ::=  SEQUENCE  {
--	authReaders	AuthReaders,--	-- if absent, use attribute in authorized reader entry
--	keyEncAlg   	AlgorithmIdentifier  OPTIONAL, --	--  algorithm to encrypt encAttrKey
--	encAttKey		EncAttKey  }

-- confidentiality key protected with authorized user's
					-- protection mechanism

-- AuthReaders  ::=  SEQUENCE OF Name

-- EncAttKey  ::=  PROTECTED {SymmetricKey, keyProtection}

-- SymmetricKey  ::=  BIT STRING

-- keyProtection PROTECTION-MAPPING  ::=  {
--	SECURITY-TRANSFORMATION {genEncryption} }

-- confKeyInfo  ATTRIBUTE  ::=  {
--	WITH SYNTAX				ConfKeyInfo
--	EQUALITY MATCHING RULE   	readerAndKeyIDMatch
--	ID						id-at-confKeyInfo }

-- ConfKeyInfo  ::=  SEQUENCE  {
--	keyIdentifier	KeyIdentifier,
--	protectedKey	ProtectedKey }

-- readerAndKeyIDMatch MATCHING-RULE  ::=  {
--	SYNTAX	ReaderAndKeyIDAssertion
--	ID		id-mr-readerAndKeyIDMatch }

-- ReaderAndKeyIDAssertion  ::=  SEQUENCE {
--	keyIdentifier	KeyIdentifier,
--	authReaders	AuthReaders OPTIONAL }

-- Object identifier assignments --

-- attributes --

id-at-clearance							OBJECT IDENTIFIER	::=	{id-at 55}

-- id-at-defaultDirQop						OBJECT IDENTIFIER	::=	{id-at 56}

id-at-attributeIntegrityInfo					OBJECT IDENTIFIER	::=	{id-at 57}

-- id-at-confKeyInfo							OBJECT IDENTIFIER	::=	{id-at 60}

-- matching rules --

-- id-mr-readerAndKeyIDMatch					OBJECT IDENTIFIER	::=	{id-mr 43}

-- contexts--

id-avc-attributeValueSecurityLabelContext		OBJECT IDENTIFIER	::=	{id-avc 3}

id-avc-attributeValueIntegrityInfoContext			OBJECT IDENTIFIER	::=	{id-avc 4}

END  -- EnhancedSecurity

## 2.2)	This corrects the defects reported in defect report 9594/242

Add size limit SIZE (1..MAX) to all optional SET OF and SEQUENCE OF constructs.

## 2.3)	This corrects the defects reported in defect reports 9594/255.

In 12.7.2 and in Annex A, change in the CONTENT-RULE information object class from:

&structuralClass   			OBJECT-CLASS.&id	UNIQUE,

to:

&structuralClass   			OBJECT-CLASS		UNIQUE,

## 2.4)	This corrects the defects reported in defect reports 9594/260

Update the AttributeTypeAndDistinguishedValue as shown:

AttributeTypeAndDistinguishedValue  ::=  SEQUENCE  {
	type					ATTRIBUTE.&id ({SupportedAttributes}),
	value  				ATTRIBUTE.&Type({SupportedAttributes}{@type}),
	primaryDistinguished		BOOLEAN DEFAULT TRUE,
	valuesWithContext		SET SIZE (1 .. MAX) OF SEQUENCE {
		distingAttrValue		[0]	ATTRIBUTE.&Type ({SupportedAttributes}{@type}) OPTIONAL,
		contextList  			SET SIZE (1 .. MAX) OF Context } OPTIONAL }

## 2.5)	This corrects the defects reported in defect reports 9594/261

Replace CommonResults with CommonResultsSeq in all ASN.1 constructs and in the import in Annex F.

In last paragraph of 26.5 replace CommonResults with CommonResultsSeq.

## 2.6)	This corrects the defects reported in defect reports 9594/267

In Note 1 of 14.7.3, replace ITU-T Rec. X.680 | ISO/IEC 8824-1 with ITU-T Rec. X.682 | ISO/IEC 8824-3.

Replace Note 1 in 14.7.10 with a copy of NOTE 1 in 14.7.3, but keep the last sentence.

In 25.2, swap Figures 19 and 20, but not the figure text.

In 22.2.1.2, make the superiorKnowledge attribute multi-valued and return to the old syntax (AccessPoint).

## 2.7)	This corrects the defects reported in defect reports 9594/269

In 12.5.2, item a), replace:

... rule is applied to;

with:

... rule is applied to unless the matching rule specifies otherwise;

In 14.7.3 add OPTIONAL to the information component of MatchingRuleDescription.

RECOMMENDATION X.511 (1997) | ISO/IEC 9594-3:1998

Information technology – Open Systems Interconnection –
The Directory:  Abstract service definition

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 1 and 2.

# Defect reports resolved by Draft Technical Corrigendum 1

(defect reports 166, 179, 188, 202, 206, 217)

# 1)	Resolution to defect report 9594/166

## Subclause 7.11.1.1 Alias derefencing

Change the second to last sentence of the first paragraph as follows:

If the DSA chains the request to another DSA and receives back a referral from it, then the access controls shall be applied to the referral if the targetObject in the referral is the same as in the chained request.

# 2)	Resolution to defect report 9594/179

## Annex B, Figure B.4

In the flow chart "Return of DN" add under the question "alias name available?/No" an additional question:

"Read operation?"

with the following outputs :

Yes: Name Error

No: go to next question: entry corresponds to (base) object of DAP operation?

## Annex B, Figure B.5

In the flow chart "Read Operation" change on the right part the text of the last step of handling "selection empty = yes"

from:

"return Read result"

to:

"return Read result or nameError"

# 3)	Resolution to defect report 9594/188

## Subclause 11.1.5 Add operation decision points for basic-access-control

Reword NOTE 2 to read as follows:

"The Add permission must be provided as prescriptiveACI when attempting to add an entry, and as prescriptiveACI, or subentryACI, when attempting to add a subentry."

# 4)	Resolution to defect report 9594/202

## Subclause 7.10 Security Parameters

Replace the paragraph describing CertificationPath with the following:

The CertificationPath component is a sequence containing the signer's user certificate, and, optionally, a sequence of one or more certification authority (CA) certificates. (See clause 8 in ITU-T Rec. X.509 | ISO/IEC 9594-8). The user certificate is used to bind the signer's public key and distinguished name, and may be used to verify the signature on a request argument, response, or error. This parameter shall be present and contain the signer's user certificate if the request argument, response, or error is signed. Additional certificates may be present and may be used to determine if the signer's user certificate is valid. Additional certificates are not required if the recipient shares the same certification authority as the signer. If the recipient requires a certification path for validation, and an acceptable parameter is not present, whether the recipient rejects the signature, or attempts to determine a certification path, is a local matter.

Replace the paragraph describing time with the following:

The time is the intended expiry time for the validity of the request, response, or error. It is used in conjunction with the random number to enable the detection of replay attacks.

Replace the 1st paragraph in the paragraph describing random with the following:

The random value is a number that should be different for each request, response, or error. It is used in conjunction with the time parameter to enable the detection of replay attacks. If sequence integrity is required then the random argument may be used to carry a sequence integrity number as follows:

# Defect reports resolved by Draft Technical Corrigendum 2

(defect reports 206, 217, 211)

____________________________________________

# 5)	Resolution to defect report 9594/206

## Subclause 10.1.3 List results

In the second to last paragraph, change the first part of the first sentence "When a DUA has requested a protection request of signed, the uncorrelatedListInfo prameter…" as follows:

"When the DUA has requested a protection request of signed, or if the Directory for other reasons are not able to correlate information, the uncorrelatedListInfo parameter..."

# 6)	Resolution to defect report 9594/217

## Subclause 7.10 Security parameters

Replace syntax for operationCode in SecurityParameters as follows:

operationCode	[6]	Code OPTIONAL

Code should be imported from:

Remote-Operations-Information-Objects

{joint-iso-ccitt remote-operations(4) informationObjects(5) version1(0)}

In the paragraph describing operationCode delete "object identifier". Also, at the end of the same paragraph, change:

"or results"

to:

"results or errors".

Add to the SecurityParameters syntax:

errorCode	[9]	Code OPTIONAL

and add the following description:

The errorCode is used to secure the error code where an error is returned in response to an operation.

# 7)	Resolution to defect report 9594/211

## Subclause 7.10

Change UTCTime to Time:

Insert the following after the ASN.1 definition of  ProtectionRequest:

Time ::= CHOICE {

utcTime	UTCTime,

generalizedTime	GeneralizedTime  }

Insert the following after the last paragraph of 7.10:

If the syntax of Time has been chosen as the UTCTime type, the value of the two-digit year field shall be rationalized into a four-digit year value as follows:

–	If the 2-digit value is 00 through 49 inclusive, the value shall have 2000 added to it.

–	If the 2-digit value is 50 through 99 inclusive, the value shall have 1900 added to it.

NOTE – GeneralizedTime shall be used if the negotiated version is v2 or greater. The use of GeneralizedTime when v1 has been negotiated may prevent interworking with implementations unaware of the possibility of choosing either UTCTime or GeneralizedTime. It is the responsibility of those specifying the domains in which this Directory Specification will be used, e.g. profiling groups, as to when the GeneralizedTime may be used. In no case shall UTCTime be used for representing dates beyond 2049.

## Subclause 8.1.1

Change the value of validity in the ASN.1 type SimpleCredentials to:

Validity	[1]	SET {

validityPeriod	CHOICE  {

COMPONENTS OF ValidityPeriodUTC,  -- UTC when v1

COMPONENTS OF ValidityPeriodGT },  -- GT when > v1

random1	[2]	BIT STRING  OPTIONAL,

random2	[3]	BIT STRING  OPTIONAL} OPTIONAL,

Insert the following after the ASN.1 type SimpleCredentials to:

ValidityPeriodUTC	::=	SET {

time1	[0]	UTCTime OPTIONAL,

time2	[1]	UTCTime OPTIONAL }

ValidityPeriodGT	::=	SET {

time1	[0]	GeneralizedTime OPTIONAL,

time2	[1]	GeneralizedTime OPTIONAL }

## Subclause 8.1.2

Insert the following after the second paragraph.

Note 1 – ValidityPeriodGT shall be used if the negotiated version is v2 or greater. The use of ValidityPeriodGT when v1 has been negotiated may prevent interworking with implementations unaware of the possibility of choosing either ValidityPeriodUTC or ValidityPeriodGT. It is the responsibility of those specifying the domains in which this Directory Specification will be used, e.g. profiling groups, as to when the ValidityPeriodGT may be used. In no case shall ValidityPeriodUTC be used for representing dates beyond 2049.

Change the value of time in the ASN.1 type Token to:

time	[2]	Time,

Also make the ASN.1 changes to Annex A.

RECOMMENDATION X.511 (1997) | ISO/IEC 9594-3:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 3, 4, and 5.

# 1)	Defect reports covered by Draft Technical Corrigendum 3

(Covering resolutions to defect reports 231 and 232)

## 1.1)	This corrects the defects reported in defect report 9594/231

This Technical Corrigendum makes modifications to Technical Corrigendum 1 (Draft Technical Corrigendum 2).

Instead of the ASN.1 suggested in Corrigendum 1, use the following data type:

SimpleCredentials  ::=	SEQUENCE  {
	name  		[0]	DistinguishedName,
	validity		[1]	SET {
		time1  		[0]	CHOICE {
			utc			UTCTime,
			gt			GeneralizedTime } OPTIONAL,
		time2  		[1]	CHOICE {
			utc			UTCTime,
			gt			GeneralizedTime } OPTIONAL,
		random1		[2]	BIT STRING  OPTIONAL,
		random2		[3]	BIT STRING  OPTIONAL },
	password	[2]	CHOICE {
		unprotected			OCTET STRING,
		protected				SIGNATURE {OCTET STRING} } OPTIONAL}

Change the Note suggested for 7.10 to normative text.

## 1.2)	This corrects the defects reported in defect report 9594/232

General:

Change all occurrences of  joint-iso-ccitt to  joint-iso-itu-t.

In 7.2  "Information types defined elsewhere":

Replace OPTIONALLY-SIGNED with OPTIONALLY-PROTECTED and OPTIONALLY-PROTECTED-SEQ.

In Annex A:

Add basicAccessControl and enhancedSecurity to the import from UsefulDefinitions.

Add a new import:

AttributeTypeAndValue
	FROM BasicAccessControl  basicAccessControl

Add ENCRYPTED to the import from AuthenticationFramework.

Move the semicolon from the end of the import from Remote-Operations-Generic-ROS-PDUs to the end of import from SpkmGssTokens.

In the import from SpkmGssTokens, change SPKM-REP-IT to SPKM-REP-TI.

# 2)	Defect reports covered by Draft Technical Corrigendum 4

(Covering resolutions to defect report 247)

## 2.1)	This corrects the defects reported in defect report 9594/247

In the Introduction, change from:

Annex B, which is an integral part of this Recommendation | International Standard,

to:

Annex B, which is not an integral part of this Recommendation | International Standard,

In 7.4, add the following construct and explanatory Note after CommonResults:

CommonResultsSeq  ::=  SEQUENCE {
	securityParameters	[30]		SecurityParameters				OPTIONAL,
	performer			[29]		DistinguishedName				OPTIONAL,
	aliasDereferenced		[28]		BOOLEAN						DEFAULT FALSE }

NOTE – CommonResults and CommonResultsSeq consist of the same components. The former is used when included in set types by the COMPONENT OF type, while the latter is used similarly in sequenced types.

In the AbandonResult, AddEntryResult, RemoveEntryResult, ModifyEntryResult and ModifyDNResult change CommonResults to CommonResultsSeq.

# 3)	Defect reports covered by Draft Technical Corrigendum 5

(Covering resolutions to defect reports 224, 228, 242 and 263)

## 3.1)	This corrects the defects reported in defect report 9594/224

In 7.8.1, change "undefined" to "UNDEFINED" in all places to indicate parity with "TRUE" and "FALSE" for the three-valued logic defined in this subclause.

In 7.8.2, add to the end of the 3rd paragraph:

When these conditions are not met, the FilterItem shall evaluate to the logical value UNDEFINED.

Delete Note 1 and change Note 2 (which is now Note 1) to:

NOTE 1 – Access control restrictions may affect the evaluation of the FilterItem and may cause the FilterItem to evaluate to UNDEFINED.

Insert the following new paragraph after the new Note 1:

An assertion which is defined by these conditions additionally evaluates to UNDEFINED if it relates to an attribute value and the attribute type is not present in an attribute against which the assertion is being tested. An assertion which is defined by these conditions and relates to the presence of an attribute type evaluates to FALSE.

## 3.2)	This corrects the defects reported in defect report 9594/228

Delete any occurrence of:

,
	DIRQOP.&…-QOP{@dirqop}

In 9.3, change OPTIONALLY-PROTECTED to OPTIONALLY-PROTECTED-SEQ in both AbandonArgument and AbandonResult.

In 11.1.1, change PROTECTED to OPTIONALLY-PROTECTED-SEQ in AddEntryResult.

In 11.2.1, change PROTECTED to OPTIONALLY-PROTECTED-SEQ in RemoveEntryResult.

In 11.3.1, change OPTIONALLY-PROTECTED to OPTIONALLY-PROTECTED-SEQ in ModifyEntryResult.

In 11.4.1, change OPTIONALLY-PROTECTED to OPTIONALLY-PROTECTED-SEQ in ModifyDNResult.

In Annex A, make the changes as indicated above.

In Annex A, add OPTIONALLY-PROTECTED-SEQ to and delete DIRQOP from the import from EnhancedSecurity.

## 3.3)	This corrects the defects reported in defect report 9594/242

Add size limit SIZE (1..MAX) to all optional SET OF and SEQUENCE OF constructs.

## 3.4)	This corrects the defects reported in defect report 9594/263

Change the last sentence of the second paragraph of 7.1 to:

Each of the subclauses 7.3 through 7.10 identifies and defines an information type.

Replace Note 1 in 8.1.2 with the following paragraph.

GeneralizedTime shall be used for time1 and time2 if the negotiated version is v2 or greater. The use of GeneralizedTime when v1 has been negotiated may prevent interworking with implementations unaware of the possibility of choosing either UTCTime or GeneralizedTime. It is the responsibility of those specifying the domains in which this Directory Specification will be used, e.g. profiling groups, as to when the GeneralizedTime may be used. UTCTime shall not be used for representing dates beyond 2049.

RECOMMENDATION X.518 (1997) | ISO/IEC 9594-4:1998

Information technology – Open Systems Interconnection –
The Directory:  Procedures for distributed operation

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 1 and 2.

# Defect reports resolved by Draft Technical Corrigendum 1

(defect reports 157, 159, 162, 180, 190, 198, 206, 209)

# 1)	Resolution to defect report 9594/157

Subclause 19.1.4 Modify DN operation

After the first paragraph of bullet 9), add the following new paragraph:

If the entry, alias entry or subentry was within the UnitOfReplication of one or more shadowing agreements held by the DSA, and the superior of the renamed entry, alias entry or subentry is not within this UnitOfReplication, the shadow consumers shall be updated using the procedures of the Directory shadow service specified in ITU-T Rec. X.525 | ISO/IEC 9594-9; in this case the shadowed entry and all its subordinates shall be removed.

If the entry, alias entry or subentry was not within the UnitOfReplication of one or more shadowing agreements held by the DSA, and the renamed entry, alias entry or subentry is now within this UnitOfReplication, the shadow consumers shall be updated using the procedures of the Directory shadow service specified in ITU-T Rec. X.525 | ISO/IEC 9594-9; in this case the shadowed entry and all its subordinates shall be shadowed.

# 2)	Resolution to defect report 9594/159

Subclause 19.3.2.2.1 Search procedure (I)

Replace the whole text of the subclause 19.3.2.2.1 1) b) i) with the following text:

i)	If e is unsuitable, make a continuationReference as follows and add it to SRContinuationList:

–	targetObject set to the DN of DSE e;

–	operationProgress with nameResolutionPhase set to proceeding and nextRDNtoBeResolved set to the number of RDNs in e;

–	all other components of continuationReference are unchanged.

Then return.

In the Note following subclause 19.3.2.2.1.1) b) i), remove the brackets with their content.

Subclause 18.3.1 Find DSE procedure

Delete in step 9) the first paragraph and NOTE 5.

# 3)	Resolution to defect report 9594/162

Subclause 20.4.5 APInfo procedure

Replace the text of the second to last dash of 5) c), "ChainingArguments.exclusions absent" with the following:

–	"chainingArguments.exclusions is set to either the relevant exclusions for the current target object if called by the Search Continuation Reference procedure, or absent if the APInfo procedure was called by the Name Resolution or the List Continuation procedures."

# 4)	Resolution to defect report 9594/180

Subclause 10.3 Chaining Arguments

Delete bullet g) (which is a duplicate of o)) and renumber the following items. Modify the order of m) n) and o) to o), n) m) which, with the renumbering will become:

l)	The entryOnly ...

m)	 uniqueIdentifier ...

n)	 authenticationLevel ...

# 5)	Resolution to defect report 9594/190

Subclause 19.3.1.2.2 List procedure (II)

In bullet 1) add a new step before a) and renumber the subsequent steps accordingly:

a)	If e' is not an entry or alias, continue with the next immediate subordinate.

b)	Check ACI ...

# 6)	Resolution to defect report 9594/198

Subclause 17.3.3.1 DUA request

Insert two new bullets e) and f) into 17.3.3.1 after bullet d) and renumber the existing e), f), g) as g), h), i) to read as follows:

d)	ChainingArguments.AuthenticationLevel and ChainginArgument.UniqueID are set according to the local security policy.

e)	ChainingArguments.nameResolveOnMaster is copied from CommonArguments.nameResolveOnMaster.

f) 	ChainingArguments.exclusions, ChainingArguments.entryOnly and ChainingArguments.referenceType are copied from CommonArguments.exclusions, CommonArguments.entryOnly and CommonArguments.referenceType if they are present, otherwise they are omitted.

g)	 If the manageDSAIT option is set …

# 7)	Resolution to defect report 9594/206

Clause 21 Results Merging procedure

Add the following Note after bullet 6):

"NOTE – In case a DSA receives search or list results from other DSAs, and such results have parameters unknown to the DSA, the uncorrelated results shall be returned. Otherwise, the DSA shall perform merging, if the search results are not signed.

A DSA which received unsigned, uncorrelated results from a DSA not able to perform consolidation, shall perform merging, if it has the proper knowledge of all parameters of the uncorrelated results."

# 8)	Resolution to defect report 9594/209

Subclause 12.1 Chained operations and Annex A

Modify the ASN.1 of ERRORS as follows:

ERRORS	{operation.&Errors Except referral| dsaReferral}

# Defect reports resolved by Draft Technical Corrigendum 2

(defect reports 211)

# 9)	Resolution to defect report 9594/211

Subclause 10.3

Change timeLimit in ChainingArguments to:

timeLimit	[9]	Time OPTIONAL,

Insert the following after the ASN.1 definition of ChainingArguments:

Time ::= CHOICE {
		utcTime 	UTCTime,
		generalizedTime    GeneralizedTime  }

Add the following to k):

Before a value of Time is used in any comparison operation, and if the syntax of Time has been chosen as the UTCTime type, the value of the two-digit year field shall be rationalized into a four-digit year value as follows:

–	If the 2-digit value is 00 through 49 inclusive, the value shall have 2000 added to it.

–	If the 2-digit value is 50 through 99 inclusive, the value shall have 1900 added to it.

NOTE – The use of GeneralizedTime may prevent interworking with implementations unaware of the possibility of choosing either UTCTime or GeneralizedTime. It is the responsibility of those specifying the domains in which this Directory Specification will be used, e.g. profiling groups, as to when the GeneralizedTime may be used. In no case shall UTCTime be used for representing dates beyond 2049.

Also make the ASN.1 changes to Annex A.

RECOMMENDATION X.518 (1997) | ISO/IEC 9594-4:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 3, 4, and 5.

# 1)	Defect reports covered by Draft Technical Corrigendum 3

(Covering resolutions to defect reports 233 and 235)

## 1.1)	This corrects the defects reported in defect report 9594/233

In Annex A:

Change all occurrences of joint-iso-ccitt to joint-iso-itu-t

Add enhancedSecurity to the import from UsefulDefinitions

Add a semicolon to the end of import from DirectoryAccessProtocol.

## 1.2)	This corrects the defects reported in defect report 9594/235

Replace the last three paragraphs of 10.8 as follows:

An AccessPointInformation value identifies one or more access points to the Directory.

AccessPointInformation  ::=  SET {
	COMPONENTS OF		MasterOrShadowAccessPoint,
	additionalPoints		[4]	MasterAndShadowAccessPoints OPTIONAL }

In the case of 1988 edition DSAs producing an AccessPointInformation value, the optional component of the set is absent. In the case of 1988 edition DSAs interpreting an AccessPointInformation value, any MasterAndShadowAccessPoints value present is ignored.

In the case of post-1988 edition DSAs, the MasterOrShadowAccessPoint value component produced for an AccessPointInformation value may be of category master or shadow, as determined by the knowledge selection procedure of the DSA producing the value. It may be viewed as a suggested access point provided by the DSA generating the value to the DSA receiving it. A MasterAndShadowAccessPoints value may optionally also be produced for an AccessPointInformation value. This constitutes additional information which may be employed by the receiving DSA's knowledge selection procedure to determine an alternative access point.

Change the ASN.1 in Annex A.

# 2)	Defect reports covered by Draft Technical Corrigendum 4

(Covering resolutions to defect reports 234 and 248)

## 2.1)	This corrects the defects reported in defect report 9594/234

Delete the last sentence of 15.3.1 (If protection is performed on the arguments, request decomposition shall not be used.)

## 2.2)	This corrects the defects reported in defect report 9594/248

In 25.1.4 and in Annex D replace:

NHOBSubordinateToSuperior  ::=  SubordinateToSuperior (
		WITH COMPONENTS { ..., alias ABSENT, entryInfo ABSENT})

with:

NHOBSubordinateToSuperior  ::= SEQUENCE {
		accessPoints		[0]	MasterAndShadowAccessPoints OPTIONAL,
		subentries		[3]	SET OF SubentryInfo OPTIONAL }

# 3)	Defect reports covered by Draft Technical Corrigendum 5

(Covering resolutions to defect reports 228, 242 and 265)

## 3.1)	This corrects the defects reported in defect report 9594/228

Delete the last paragraph of 16.3.9 and clause 21.

Delete any occurrence of:

,
	DIRQOP.&…-QOP{@dirqop}

Insert the following at the begining of 15.5.5

Warning – This subclause refers to specifications that have been deprecated with respect to encryption. Signing of requests is not deprecated.

In Annex A, remove the DIRQOP from the import.

## 3.2)	This corrects the defects reported in defect report 9594/242

Add size limit SIZE (1..MAX) to all optional SET OF and SEQUENCE OF constructs.

## 3.3)	This corrects the defects reported in defect report 9594/265

In the first paragraph of 14.5, replace subordinate DSA with those DSAs.

Add the following new paragraph and Note to the end of 15.3.1:

The argument of a chained request (see 12.1) or subrequest shall be the unmodified operation argument if the operation was initiated by a DUA. A DSA receiving a chained request shall not change argument when doing request decomposition.

NOTE – The following subclauses specify that requirement for individual components of argument. This should not be interpreted to mean that a component not explicitly mentioned can be changed.

At the beginning of the last paragraph of 15.5.2, add it does not support after "If a DSA encounters an extension". Change execution phase to evaluation phase.

Delete 19.3.1.1.3.

RECOMMENDATION X.519 (1997) | ISO/IEC 9594-5:1998

Information technology – Open Systems Interconnection –
The Directory:  Protocol specifications

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 1.

# Defect reports resolved by Draft Technical Corrigendum 1

# 1)	Resolution to defect report 9594/221

Subclause 9.1.1 Statement Conformance

Add to 9.1.1 b):

and whether conformance for signed operations is claimed.

Add the following new bullet:

e)	If conformance is claimed for strong authentication, signed operations, or protected operations, identification of the Certificate and CRL extensions for which conformance is claimed.

Subclause 9.1.2 Static Conformance

Add the following new bullet:

d)	conform to clause 12 of ITU-T Rec. X.509 | ISO/IEC 9594-8 for the Certificate and CRL extensions for which conformance was claimed in bullet 9.1.1 e).

Subclause 9.2.1 Statement Conformance

Add to 9.2.1 e):

and whether conformance for signed operations is claimed.

Add the following new bullet:

ad)	If conformance is claimed for strong authentication, signed operations, or protected operations, identification of the Certificate and CRL extensions for which conformance is claimed.

Subclause 9.2.2 Static Conformance

Add the following new bullet:

x)	conform to clause 12 of ITU-T Rec. X.509 | ISO/IEC 9594-8 for the Certificate and CRL extensions for which conformance was claimed in 9.2.1 ad).

RECOMMENDATION X.519 (1997) | ISO/IEC 9594-5:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 2 and 3.

# 1)	Defect reports covered by Draft Technical Corrigendum 2

(Covering resolutions to defect report 236)

## 1.1)	This corrects the defects reported in defect report 9594/236

In Annexes A, B, C, D imports:

Change Remote-Operations-Realisations and realisations(8) or (9) to
Remote-Operations-Realizations and realizations(9)

Change:

{joint-iso-ccitt remote-operations(4) remote-Operations-Abstract-Syntaxes(12) version1(0)}

to:

{joint-iso-itu-t remote-operations(4) remote-operations-abstract-syntaxes(12) version1(0)}

In Annex A:

In the DAP-Invokable  OPERATION construct replace addEtry with addEntry

In Annex C:

Replace InvokeID with InvokeId

In Annex D:

Change the object identifier for the module to:

{joint-iso-itu-t ds(5) module(1) dop(17) 3}

Annex G:

Changes to Annex G have been subsumed by the resolution to defect report 228.

# 2)	Defect reports covered by Draft Technical Corrigendum 3

(Covering resolutions to defect reports 228 and 266)

## 2.1)	This corrects the defects reported in defect report 9594/228

In the Introduction, delete the second last paragraph and change Annex H to Annex G in the last paragraph.

In 2.1, delete references to Generic upper layers security.

In clause 4, delete the GULS and SESE abbreviations.

Delete the last paragraph of 6.1.

In 6.7.3:

In the 3rd paragraph, delete but not SESE.

In the 4th paragraph, replace If the RTSE and SESE are both with If the RTSE is.

Delete the 5th and 6th paragraphs including the two letter-numbered lists.

In 6.7.4:

In the 5th paragraph, delete but not SESE.

In the 6th paragraph, replace If the RTSE and SESE are both with If the RTSE is.

Delete the 7th and 8th paragraphs.

Delete 6.7.5

In 8.1.1, delete the last paragraph.

In 8.1.1.1.2:

Delete in the first paragraph if SESE is not used.

Delete the last paragraph including its letter-numbered list.

In 8.1.1.1.4, delete the last paragraph.

Delete 8.1.3.

In 8.2.1.1.2:

Delete If SESE is not used,

Delete the second (last) paragraph.

In 8.2.1.1.4:

Replace this subclause with the following new paragraph:

The initiator of the association shall supply the Presentation Context Definition List in the RT-OPEN request primitive which shall contain the ACSE abstract-syntax (id-as-acse) and the DISP abstract-syntax that includes the RTSE (id-as-directoryReliableShadowAS).

Delete 8.2.3.

In 9.1.1:

In item a), delete or directoryAccessWith2or3seAC.

Delete item d) and renumber next item.

In 9.1.2, item a), delete or directoryAccessWith2or3seAC.

In 9.1.3, replace item a) with:

a)	shall conform to the mapping onto the used service defined in clause 8 or clause 10 or both; and

In 9.2.1:

In item a), delete directoryAccessWith2or3seAC, directorySystemWith2or3seAC,

In item d), delete or directorySystemWith2or3seAC.

In 9.2.3:

In item c), delete or directoryAccessWith2or3seAC.

In item d), delete or directorySystemWith2or3seAC.

In 9.3.1, item a), delete shadowSupplierInitiatedWith2or3seAC, and shadowConsumerInitiatedWith2or3seAC.

In 9.4.1, item a), delete shadowSupplierInitiatedWith2or3seAC, and shadowConsumerInitiatedWith2or3seAC.

In Annex A:

Remove directorySecurityExchanges import from UsefulDefinitions.

Delete the id-ac-directoryAccessWith2or3seAC import from ProtocolObjectIdentifiers.

Delete the import from directorySecurityExchanges.

Delete the directoryAccessWith2or3seAC application-context.

In Annex B:

Remove directorySecurityExchanges import from UsefulDefinitions.

Delete the id-ac-directorySystemWith2or3seAC import from ProtocolObjectIdentifiers.

Delete the import from directorySecurityExchanges.

Delete the directorySystemWith2or3seAC application-context.

In Annex C:

Remove directorySecurityExchanges import from UsefulDefinitions.

Delete the id-ac-shadowSupplierInitiatedWith2or3seAC, id-ac-shadowConsumerInitiatedWith2or3seAC, id-ac-reliableShadowSupplierInitiatedWith2or3seAC and id-ac-reliableShadowConsumerInitiatedWith2or3seAC imports from ProtocolObjectIdentifiers

Delete the import from directorySecurityExchanges.

Delete the shadowSupplierInitiatedWith2or3seAC, shadowConsumerInitiatedWith2or3seAC, reliableShadowSupplierInitiatedWith2or3seAC and reliableShadowConsumerInitiatedWith2or3seAC application-contexts.

In Annex D:

Remove directorySecurityExchanges import from UsefulDefinitions.

Delete the id-ac-directoryOperationalBindingManagementWith2or3seAC import from ProtocolObjectIdentifiers.

Delete the import from directorySecurityExchanges.

Delete the directoryOperationalBindingManagementWith2or3seAC application-context.

In Annex E:

Delete the id-se import from UsefulDefinitions.

Delete the object identifiers id-se-threewayse and id-se-spkmthreewayse.

Delete Annex G and rename Annex H as Annex G.

## 2.2)	This corrects the defects reported in defect report 9594/266

Reinstate the 9.1.1, item c) from edition 2 and change the current item c) to d).

Disregard the updates to 9.1.1 b) and 9.2.1 e) as required by Technical Corrigendum 1 to ITU-T Rec. X.519 (1997) | ISO/IEC 9594-5:1998.

RECOMMENDATION X.520 (1997) | ISO/IEC 9594-6:1998

Information technology – Open Systems Interconnection – 
The Directory:  Selected attribute types

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigendum 1.

# Defect reports resolved by Draft Technical Corrigendum 1

(defect report 211)

# 1)	Resolution to defect report 9594/211

Subclause 6.3.2

Add the following to the last paragraph:

The value of the two-digit year field shall be rationalized into a four-digit year value as follows:

–	if the 2-digit value is 00 through 49 inclusive, the value shall have 2000 added to it; and

–	if the 2-digit value is 50 through 99 inclusive, the value shall have 1900 added to it.

RECOMMENDATION X.520 (1997) | ISO/IEC 9594-6:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 2 and 3.

# 1)	Defect reports covered by Draft Technical Corrigendum 2

(Covering resolutions to defect reports 228, 237, 238 and 241)

## 1.1)	This corrects the defects reported in defect report 9594/228

In Annex C, add  ub-privacy-mark-length		INTEGER	::=	128

## 1.2)	This corrects the defects reported in defect report 9594/237

In 5.2.1:

Replace the attribute definition with:

name ATTRIBUTE	::=	{

WITH SYNTAX		DirectoryString {ub-name}
		EQUALITY MATCHING RULE		caseIgnoreMatch
		SUBSTRINGS MATCHING RULE		caseIgnoreSubstringsMatch
		ID		id-at-name }

In 5.2.9:

Change the upper bound to  ub-serial-number and the object identifier to  id-at-serialNumber.

In 7.3:

Change localeContextSyntax to  LocaleContextSyntax in two places and remove the spaces between  "::" and  =.

Add  {ub-locale-context-syntax} after DirectoryString

Introduce the same changes to  Annex A.

In Annex C, add  ub-locale-context-syntax		INTEGER	::=	128 to the end of the list.

In Annex A:

Remove  TeletexNonBasicParameters from the import from  MTSAbstractService.

Remove one occurrence of  ub-name from the import from  UpperBounds.

Add  CONTEXT to the import from  InformationFramework.

In the  FacsimileTelephoneNumber type definition, add a comma after  TelephoneNumber.

In the  x121Address attribute type definition, replace  X.121Address with  X121Address.

In X121Address  ::=  NumericString (SIZE(1  ub-x121-address)) change the two spaces in the size specification to points, i.e.  SIZE(1..ub-x121-address).

Add a right curly parenthesis at the end of the  languageContext context definition.

Add a right curly parenthesis in the  Period type as shown:

bitDay	BIT STRING { sunday (0), monday (1), tuesday (2),
				wednesday (3), thursday (4), friday (5), saturday (6) },

In the  NamedDay type, replace  ENUMARATED with  ENUMERATED.

Add two hypens to the start of the second line of:
-- id-at-encryptedTeletexTerminalIdentifier

Add two hypens to the start of both lines of:
-- id-at-encryptedCollectiveTeletexTerminalIdentifier

In Annex C:

Change the last component of the object identifier for the module from 2 to 3

Change the last occurrence  ub-name to  ub-surname.

## 1.3)	This corrects the defects reported in defect report 9594/238

In 6.1.1, change in the first paragraph from:

attribute value of type PrintableString, NumericString, TeletexString, BMPString, UniversalString, or DirectoryString

to:

attribute value of type DirectoryString and each data type appearing in the choice type DirectoryString, e.g. UTF8String.

In 6.1.2 through 6.1.6, in the first paragraph change from:

attribute value whose type is one of the ones listed in 6.1.1

to:

attribute value of type DirectoryString and each data type appearing in the choice type DirectoryString, e.g. UTF8String.

## 1.4)	This corrects the defects reported in defect report 9594/241

In 5.2.9, replace "of a device" with "of an object".

# 2)	Defect reports covered by Draft Technical Corrigendum 3

(Covering resolutions to defect report 270)

## 2.1)	This corrects the defects reported in defect report 9594/270

In 5.8.1, replace preferredDeliveryMethod attribute type with:

preferredDeliveryMethod  ATTRIBUTE  ::=  {
	WITH SYNTAX		PreferredDeliveryMethod
	SINGLE VALUE		TRUE
	ID					id-at-preferredDeliveryMethod }

PreferredDeliveryMethod  ::=  SEQUENCE OF INTEGER {
		any-delivery-method	(0),
		mhs-delivery			(1),
		physical-delivery		(2),
		telex-delivery			(3),
		teletex-delivery		(4),
		g3-facsimile-delivery	(5),
		g4-facsimile-delivery	(6),
		ia5-terminal-delivery	(7),
		videotex-delivery  		(8),
		telephone-delivery	(9) }

In 6.1.10, replace caseIgnoreListMatch matching rule with:

caseIgnoreListMatch  MATCHING-RULE  ::=  {
		SYNTAX		CaseIgnoreList
		ID			id-mr-caseIgnoreListMatch }

CaseIgnoreList  ::=  SEQUENCE OF DirectoryString {ub-match}

RECOMMENDATION X.521 (1997) | ISO/IEC 9594-7:1998

Information technology – Open Systems Interconnection – 
The Directory:  Selected object classes

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigendum 1.

# Defect reports resolved by Draft Technical Corrigendum 1

(Covering resolutions to defect report 239)

This corrects the defects reported in defect report 9594/239

In Annex A:

Add certificateExtensions to the import from UsefulDefinitions.

Remove supportedAlgorithms and deltaRevocationList from the import from AuthenticationFramework.

Add a new import:

supportedAlgorithms, deltaRevocationList

FROM CertificateExtensions certificateExtensions	;

RECOMMENDATION X.509 (1997) | ISO/IEC 9594-8:1998

Information technology – Open Systems Interconnection – 
The Directory:  Authentication framework

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 3, 4, 5, and 7.

# Defect reports resolved by Draft Technical Corrigendum 3

(defect reports 200, 201, 212, 213, 218, and 220)

# 1)	Resolution to defect report 9594/200

## Subclause 12.6.2

Add the following at the end of the paragraph beginning with "If this extension is flagged critical ...":

"Where the distribution points are used to distribute CRL information for all revocation reason codes and all certificates issued by the CA include the crlDistributionPoint as a critical extension, the CA is not required to also publish a full CRL at the CA entry."

# 2)	Resolution to defect report 9594/201

## Subclause 12.6.3.1

Move the second sentence of the second paragraph "If this field is absent …CRL issuer." to the first paragraph immediately before the sentence "This field is defined as follows:":

Add a paragraph break following the relocated sentence, making "This field is defined as follows:" as an independent paragraph immediately before the ASN.1.

# 3)	Resolution to defect report 9594/212

## Subclause 12.7.6

Add the following to subclause 12.7.6:

"g)	authorityKeyIdentifier matches if the value of this component in the stored attribute value equals that in the presented value; there is no match if the stored attribute value contains no authority key identifier extension or if not all components in the presented value are present in the stored attribute value."

# 4)	Resolution to defect report 9594/213

## Subclause 12.7.6 d)

Replace the text of 12.7.6 d) with the following:

"d)	reasonFlags matches if any of the bits that are set in the presented value are also set in the onlySomeReasons components of the issuing distribution point extension of the stored attribute value; there is also a match if the stored attribute value contains no reasonFlags in the issuing distribution point extension, or if the stored attribute value contains no issuing distribution point extension;

NOTE – Even though a CRL matches on a particular value of reasonFlags, the CRL may not contain any revocation notices with that reason code."

# 5)	Resolution to defect report 9594/218

## Subclause 12.7.2 j)

Replace the text of 12.7.2 j) with the following:

"j)	policy matches if at least one member of the CertPolicySet presented appears in the certificate policies extension in the stored attribute value; there is no match if there is no certificate policies extension in the stored attribute value;"

# 6)	Resolution to defect report 9594/220

## Subclause 11.2, Note 3

In Note 3, in the second sentence, replace "shall be absent" with "may be absent".

In Note 3, at the beginning of the 3rd sentence, replace "This may permit" with "If version is absent, this may permit".

In Note 3, at the beginning of the 4th sentence, replace "An implementation that supports version 2 (or greater) CRLs may" with "An implementation that supports version 2 (or greater) CRLs, in the absence of version, may also ...".

# Defect reports resolved by Draft Technical Corrigendum 4

(defect report 185)

# 7)	Resolution to defect report 9594/185

## Clause 8

Add the following text immediately following the ASN.1 for certificatePair:

"The cACertificate attribute of a CA's directory entry shall be used to store self-issued certificates (if any) and certificates issued to this CA by CAs in the same realm as this CA.

The forward elements of the crossCertificatePair attribute of a CA's directory entry shall be used to store all, except self-issued certificates issued to this CA. Optionally, the reverse elements of the crossCertificatePair attribute, of a CA's directory entry may contain a subset of certificates issued by this CA to other CAs. When both the forward and the reverse elements are present in a single attribute value, issuer name in one certificate shall match the subject name in the other and vice versa, and the subject public key in one certificate shall be capable of verifying the digital signature on the other certificate and vice versa.

When a reverse element is present, the forward element value and the reverse element value need not be stored in the same attribute value; in other words, they can be stored in either a single attribute value or two attribute values.

In the case of v3 certificates, none of the above CA certificates shall include a basicConstraints extension with the cA value set to FALSE.

The definition of realm is purely a matter of local policy."

Also, replace Figure 4 with the following:

# Defect reports resolved by Draft Technical Corrigendum 5

(defect reports 204)

# 8)	Resolution to defect report 9594/204

## Subclause 12.6.3.1

In the first sentence following the ASN.1, delete "unexpired".

Add the following as a new second sentence in the first paragraph following the ASN.1:

"After a certificate appears on a CRL, it may be deleted from a subsequent CRL after the certificate’s expiry."

# Defect reports resolved by Draft Technical Corrigendum 7

(defect report 222)

# 9)	Resolution to defect report 9594/222

Add the following to subclause 12.1:

##### "Certificate policy

The authentication framework contains three types of entity: the certificate user, the certification authority and the certificate subject (or end-entity). Each entity operates under obligations to the other two entities and, in return, enjoys limited warranties offered by them. These obligations and warranties are defined in a certificate policy. A certificate policy is a document (usually in plain-language). It can be referenced by a unique identifier, which may be included in the certificate policies extension of the certificate issued by the certification authority, to the end-entity and upon which the certificate user relies. A certificate may be issued in accordance with one or more than one policy. Definition of the policy, and assignment of the identifier, are performed by a policy authority. And the set of policies administered by a policy authority is called a policy domain. All certificates are issued in accordance with a policy, even if the policy is neither recorded anywhere nor referenced in the certificate. The Recommendation | International Standard does not prescribe the style or contents of the certificate policy.

The certificate user may be bound to its obligations under the certificate policy by the act of importing an authority public key and using it as a trust anchor, or by relying on a certificate that includes the associated policy identifier. The certification authority may be bound to its obligations under the policy by the act of issuing a certificate that includes the associated policy identifier. And, the end-entity may be bound to its obligations under the policy by the act of requesting and accepting a certificate that includes the associated policy identifier and by using the corresponding private key. Implementations that do not use the certificate policies extension should achieve the required binding by some other means.

For an entity to simply declare conformance to a policy does not generally satisfy the assurance requirements of the other entities in the framework. They require some reason to believe that the other parties operate a reliable implementation of the policy. However, if explicitly so stated in the policy, certificate users may accept the certification authority’s assurances that its end-entities agree to be bound by their obligations under the policy, without having to confirm this directly with them. This aspect of certificate policy is outside the scope of the Recommendation | International Standard.

A certification authority may place limitations on the use of its certificates, in order to control the risk that it assumes as a result of issuing certificates. For instance, it may restrict the community of certificate users, the purposes for which they may use its certificates and/or the type and extent of damages that it is prepared to make good in the event of a failure on its part, or that of its end-entities. These matters should be defined in the certificate policy.

Additional information, to help affected entities understand the provisions of the policy, may be included in the certificate policies extension in the form of policy qualifiers.

##### Cross-certification

A certification authority may be the subject of a certificate issued by another certification authority. In this case, the certificate is called a cross-certificate, the certification authority that is the subject of the certificate is called the subject certification authority and the certification authority that issues the cross-certificate is called an intermediate certification authority (see Figure 1). Both the cross-certificate and the end-entity’s certificate may contain a certificate policies extension.

The warranties and obligations shared by the subject certification authority, the intermediate certification authority and the certificate user are defined by the certificate policy identified in the cross-certificate, in accordance with which the subject certification authority may act as, or on behalf of, an end-entity. And the warranties and obligations shared by the certificate subject, the subject certification authority and the intermediate certification authority are defined by the certificate policy identified in the end-entity’s certificate, in accordance with which the intermediate certification authority may act as, or on behalf of, a certificate user.

A certification path is said to be valid under the set of policies that are common to all certificates in the path.

An intermediate certification authority may, in turn, be the subject of a certificate issued by another certification authority, thereby creating certification paths of length greater than two certificates. And, since trust suffers dilution as certificate paths grow in length, controls are required to ensure that end-entity certificates with an unacceptably low associated trust level will be rejected by the certificate user. This is part of the function of the certification path processing procedure.

In addition to the situation described above, there are two special cases to be considered:

1)	the certification authority does not use the certificate policies extension to convey its policy requirements to certificate users; and

2)	the certificate user or intermediate certification authority delegates the job of controlling policy to the next authority in the path.

In the first case, the certificate should not contain a certificate policies extension at all. As a result, the set of policies under which the path is valid will be null. But, the path may be valid nonetheless. Certificate users must still ensure that they are using the certificate in conformance with the policies of the authorities in the path.

In the second case, the certificate user or intermediate certification authority should include the special value any-policy in the initial-policy-set or cross-certificate. Where a certificate includes the special value any-policy, it should not include any other certificate policy identifiers. The identifier any-policy should not have any associated policy qualifiers.

The certificate user can ensure that all its obligations are conveyed in accordance with the Recommendation | International Standard by setting the initial-explicit-policy indicator. In this way, only authorities that use the standard certificate policies extension as their way of achieving binding are accepted in the path, and certificate users have no additional obligations. Because authorities also attract obligations when they act as, or on behalf of, a certificate user, they can ensure that all their obligations are conveyed in accordance with the Recommendation | International Standard by setting requireExplicitPolicy in the cross-certificate.

##### Policy mapping

Some certification paths may cross boundaries between policy domains. The warranties and obligations according to which the cross-certificate is issued may be materially equivalent to some or all of the warranties and obligations according to which the subject certification authority issues certificates to end-entities, even though the policy authorities under which the two certification authorities operate may have selected different unique identifiers for these materially equivalent policies. In this case, the intermediate certification authority may include a policy mappings extension in the cross-certificate. In the policy mappings extension, the intermediate certification authority assures the certificate user that it will continue to enjoy the familiar warranties, and that it should continue to fulfill its familiar obligations, even though subsequent entities in the certification path operate in a different policy domain. The intermediate certification authority should include one or more mappings for each of a subset of the policies under which it issued the cross-certificate, and it should not include mappings for any other policies. If one or more of the certificate policies according to which the subject certification authority operates is identical to those according to which the intermediate certification authority operates (i.e. it has the same unique identifier), then these identifiers should be excluded from the policy mapping extension, but included in the certificate policies extension.

Policy mapping has the effect of converting all policy identifiers in certificates further down the certification path to the identifier of the equivalent policy, as recognized by the certificate user.

Policies should not be mapped either to or from the special value any-policy.

Certificate users may determine that certificates issued in a policy domain other than its own should not be relied upon, even though a trusted intermediate certification authority may determine its policy to be materially equivalent to its own. It can do this by setting the initial-policy-mapping-inhibit input to the path validation procedure. Additionally, an intermediate certification authority may make a similar determination on behalf of its certificate users. In order to ensure that certificate users correctly enforce this requirement, it can set inhibitPolicyMapping in a policy constraints extension.

##### Certification path processing

The certificate user faces a choice between two strategies:

1)	it can require that the certification path be valid under at least one of a set of policies pre-determined by the user; or

2)	it can ask the path validation module to report the set of policies for which the certification path is valid.

The first strategy may be most appropriate when the certificate user knows, a priori, the set of policies that are acceptable for its intended use.

The second strategy may be most appropriate when the certificate user does not know, a priori, the set of policies that are acceptable for its intended use.

In the first instance, the certification path validation procedure will indicate the path to be valid only if it is valid under one or more of the policies specified in the initial-policy-set, and it will return the sub-set of the initial-policy-set under which the path is valid. In the second instance, the certification path validation procedure may indicate that the path is invalid under the initial-policy-set, but valid under a disjoint set: the authorities-constrained-policy-set. Then the certificate user must determine whether its intended use of the certificate is consistent with one or more of the certificate policies under which the path is valid. By setting the initial-policy-set to any-policy, the certificate user can cause the procedure to return a valid result if the path is valid under any (unspecified) policy.

##### Self-issued certificates

There are three circumstances under which a certification authority may issue a certificate to itself:

1)	as a convenient way of encoding its public key for communication to, and storage by, its certificate users;

2)	for certifying key usages other than certificate and CRL signing (such as time-stamping); and

3)	for replacing its own expired certificates.

These types of certificate are called self-issued certificates, and they can be recognized by the fact that the issuer and subject names present in them are identical. For purposes of path validation, self-issued certificates of type one are verified with the public key contained in them, and if they are encountered in the path, they shall be ignored.

Self-issued certificates of type two may only appear as end certificates in a path, and shall be processed as end certificates.

Self-issued certificates of type three (also known as self-issued intermediate certificates) may appear as intermediate certificates in a path. As a matter of good practice, when replacing a key that is on the point of expiration, a certification authority should request the issuance of any in-bound cross-certificates that it requires for its replacement public key before using the key. Nevertheless, if self-issued certificates are encountered in the path, they shall be processed as intermediate certificates, with the following exception: they do not contribute to the path length for purposes of processing the pathLenConstraint component of the basicConstraints extension and the skip-certificates values associated with the policy-mapping-inhibit-pending and explicit-policy-pending indicators."

In subclause 12.2.2.6, after the 2nd sentence of the 1st paragraph, add the following:

"The presence of this extension in an end-entity certificate indicates the certificate policies for which this certificate is valid. The presence of this extension in a certificate issued by one CA to another CA indicates the certificate policies for which this certificate can be used to validate certification paths."

Add the following text in subclause 12.2.2.6, after the 1st sentence of the 1st paragraph:

"The list of certificate policies is used in determining the validity of a certification path, as described in 12.4.3. The optional qualifiers are not used in the certification path processing procedure, but relevant qualifiers are provided as an output of that process to the certificate using application to assist in determining whether a valid path is appropriate for the particular transaction."

In subclause 12.2.2.7, replace the sentence "This extension is always non-critical." with the following:

"This extension may, at the option of the certificate issuer, be either critical or non-critical. It is recommended that it be critical, otherwise a certificate user may not correctly interpret the stipulation of the issuing CA."

Add the following new subclause 12.4.2.4:

#### "12.4.2.4	Inhibit any policy field

This field specifies a constraint that indicates any-policy is not considered an explicit match for other certificate policies for the remainder of the certification path.

inhibitAnyPolicy ::= EXTENSION {

SYNTAX SkipCerts

IDENTIFIED BY {id-ce-inhibitAnyPolicy }}

This extension may, at the option of the certificate issuer, be either critical or non-critical. It is recommended that it be critical, otherwise a certificate user may not correctly interpret the stipulation of the issuing CA."

Add the following to the list of OIDs in the certificateExtensions module in Annex A:

"id-ce-inhibitAnyPolicy			OBJECT IDENTIFIER ::=	{id-ce 54}"

Replace subclause 12.4.3 with the following:

### "12.4.3	Certification path processing procedure

Certification path processing is carried out in a system which needs to use the public key of a remote end entity, e.g. a system which is verifying a digital signature generated by a remote entity. The certificate policies, basic constraints, name constraints, and policy constraints extensions have been designed to facilitate automated, self-contained implementation of certification path processing logic.

The following is an outline of a procedure for validating certification paths. A conformant implementation shall be functionally equivalent to the external behaviour resulting from this procedure. But, the algorithm used by a particular implementation to derive the correct output(s) from the given inputs is not standardized.

The inputs to the certification path processing procedure are:

a)	a set of certificates comprising a certification path;

b)	a trusted public key value or key identifier (if the key is stored internally to the certification path processing module), for use in verifying the first certificate in the certification path;

c)	an initial-policy-set comprising one or more certificate policy identifiers, indicating that any one of these policies would be acceptable to the certificate user for the purposes of certification path processing; this input can also take the special value any-policy;

d)	an initial-explicit-policy indicator value, which indicates whether an acceptable policy identifier must appear in the certificate policies extension field of all certificates in the path;

e)	an initial-policy-mapping-inhibit indicator value, which indicates whether policy mapping is forbidden in the certification path;

f)	an initial-inhibit-policy indicator value, which indicates if the special value anyPolicy, if present in a certificate policies extension, is considered a match for any specific certificate policy value in a constrained set; and

g)	the current date/time (if not available internally to the certification path processing module).

The values of c), d), e) and f) will depend upon the policy requirements of the user-application combination that needs to use the certified end-entity public key.

Note that because these are individual inputs to the path validation process, a certificate user may limit the trust it places in any given trusted public key to a given set of certificate policies. This can be achieved by ensuring that a given public key is the input to the process only when initial-policy-set input includes policies for which the certificate user trusts that public key. Since another input to the process is the certification path itself, this control could be exercised on a transaction by transaction basic.

The outputs of the procedure are:

a)	an indication of success or failure of certification path validation;

b)	if validation failed, a diagnostic code indicating the reason for failure;

c)	the set of authorities-constrained policies and their associated qualifiers in accordance with which the certification path is valid, or the special value any-policy;

d)	the set of user-constrained policies, formed from the intersection of the authorities-constrained-policy-set and the initial-policy-set;

e)	explicit-policy-indicator, indicating whether the certificate user or an authority in the path requires that an acceptable policy be identified in every certificate in the path; and

f)	details of any policy mapping that occurred in processing the certification path.

NOTE – If validation is successful, the certificate-using system may still choose not to use the certificate as a result of values of policy qualifiers or other information in the certificate.

The procedure makes use of the following set of state variables:

a)	authorities-constrained-policy-set: A table of policy identifiers and qualifiers from the certificates of the certification path (rows represent policies, their qualifiers and mapping history, and columns represent certificates in the certification path);

b)	permitted-subtrees: A set of subtree specifications defining subtrees within which all subject names in subsequent certificates in the certification path must fall, or may take the special value unbounded;

c)	excluded-subtrees: A (possibly empty) set of subtree specifications (each comprising a subtree base name and maximum and minimum level indicators) defining subtrees within which no subject name in a subsequent certificate in the certification path may fall;

d)	explicit-policy-indicator: Indicates whether an acceptable policy must be explicitly identified in every certificate in the path;

e)	path depth: An integer equal to one more than the number of certificates in the certification path for which processing has been completed;

f)	policy-mapping-inhibit-indicator: Indicates whether policy mapping is inhibited;

g)	inhibit-any-policy-indicator: Indicates whether the special value anyPolicy is considered a match for any specific certificate policy;

h)	pending-constraints: Details of explicit-policy, inhibit-policy-mapping and/or inhibit-any-policy constraints which have been stipulated but have yet to take effect. There are three one-bit indicators called explicit-policy-pending, and policy-mapping-inhibit-pending and inhibit-any-policy-pending together with, for each, an integer called skip-certificates which gives the number of certificates yet to skip before the constraint takes effect.

The procedure involves an initialization step, followed by a series of certificate-processing steps. The initialization step comprises:

a)	Write any-policy in the zeroth and first columns of the zeroth row of the authorities-constrained-policy-set table.

b)	Initialize the permitted-subtrees variable to unbounded.

c)	Initialize the excluded-subtrees variable to an empty set.

d)	Initialize the explicit-policy-indicator to the initial-explicit-policy value.

e)	Initialize path-depth to one.

f)	Initialize the policy-mapping-inhibit-indicator to the initial-policy-mapping-inhibit value.

g)	Initialize the three pending-constraints indicators to unset.

Each certificate is then processed in turn, starting with the certificate signed using the input trusted public key. The last certificate is considered to be the end certificate; any other certificates are considered to be intermediate certificates.

The following checks are applied to a certificate:

a)	Check that the signature verifies, that dates are valid, that the certificate subject and certificate issuer names chain correctly, and that the certificate has not been revoked.

b)	For an intermediate certificate, if the basic constraints extension field is present in the certificate, check that the cA component is present and set to true. If the pathLenConstraint component is present, check that the current certification path does not violate that constraint (ignoring intermediate self-issued certificates).

c)	If the certificate policies extension is not present, then set the authorities-constrained-policy-set to null by deleting all rows from the authorities-constrained-policy-set table.

d)	If the certificate policies extension is present, then for each policy, P, in the extension other than anyPolicy, attach the policy qualifiers associated with P to each row in the authorities-constrained-policy-set table whose [path-depth] column entry contrains the value P. If no row in the authorities-constrained-policy-set table contains P in its [path-depth] colums entry but the value in authorities-constrained-policy-set[0, path-depth] is any-policy, then add a new row to the table by duplicating the zeroth row and writing the policy identifier P along with its qualifiers in the [path-depth] column entry of the new row.

e)	If the certificate policies extension is present and does not include the value anyPolicy or if the inhibit-any-policy-indicator is set, then delete any row for which the [path-depth] column entry contains the value any-policy along with any row for which the [path-depth] column entry does not contain one of the values in the certificate policies extension.

f)	If the certificate policies extension is present and includes the value anyPolicy and the inhibit-any-policy-indicator is not set, then attach the policy qualifiers associated with anyPolicy to each row in the authorities-constrained-policy-set table whose [path-depth] column entry contains the value any-policy or contains a value that does not appear in the certificate policies extension.

g)	If the certificate is not an intermediate self-issued certificate, check that the subject name is within the name-space given by the value of permitted-subtrees and is not within the name-space given by the value of excluded-subtrees.

For an intermediate certificate, the following constraint recording actions are then performed, in order to correctly set up the state variables for the processing of the next certificate:

a)	If the nameConstraints extension with a permittedSubtrees component is present in the certificate, set the permitted-subtrees state variable to the intersection of its previous value and the value indicated in the certificate extension.

b)	If the nameConstraints extension with an excludedSubtrees component is present in the certificate, set the excluded-subtrees state variable to the union of its previous value and the value indicated in the certificate extension.

c)	If policy-mapping-inhibit-indicator is set:

–	process any policy mappings extension by, for each mapping identified in the extension, locate all rows in the authorities-constrained-policy-set table whose [path-depth] column entry is equal to the issuer domain policy value in the extension and delete the row.

d)	If policy-mapping-inhibit-indicator is not set:

–	process any policy mappings extension by, for each mapping identified in the extension, locate all rows in the authorities-constrained-policy-set table whose [path-depth] column entry is equal to the issuer domain policy value in the extension, and write the subject domain policy value from the extension in the [path-depth+1] column entry of the same row. If the extension maps an issuer domain policy to more than one subject domain policy, then the affected row must be copied and the new entry added to each row. If the value in authorities-constrained-policy-set[0, path-depth] is any-policy, then write each issuer domain policy identifier from the policy mappings extension in the [path-depth] column, making duplicate rows as necessary and retaining qualifiers if they are present, and write the subject domain policy value from the extension in the [path-depth+1] column entry of the same row;

–	if the policy-mapping-inhibit-pending indicator is set and the certificate is not self-issued, decrement the corresponding skip-certificates value and, if this value becomes zero, set the policy-mapping-inhibit-indicator;

–	if the inhibitPolicyMapping constraint is present in the certificate, perform the following. For a SkipCerts value of 0, set the policy-mapping-inhibit-indicator. For any other SkipCerts value, set the policy-mapping-inhibit-pending indicator, and set the corresponding skip-certificates value to the lesser of the SkipCerts value and the previous skip-certificates value (if the policy-mapping-inhibit-pending indicator was already set).

e)	For any row not modified in either step c) or d), above (and every row in the case that there is no mapping extension present in the certificate), write the policy identifier from [path-depth] column in the [path-depth+1] column of the row.

f)	If inhibit-any-policy-indicator is not set:

–	If the inhibit-any-policy-pending indicator is set and the certificate is not self-issued, decrement the corresponding skip-certificates value and, if this value becomes zero, set the inhibit-any-policy-indicator.

–	If the inhibitAnyPolicy constraint is present in the certificate, perform the following. For a SkipCerts value of 0, set the inhibit-any-policy-indicator. For the other SkipCerts value, set the inhibit-any-policy-pending indicator, and set the corresponding skip-certificates values to the lesser of the SkipCerts value and the previous skip-certificates value (if the inhibit-any-policy-pending indicator was already set).

g)	Increment path-depth.

For all certificates, the following actions are then performed:

a)	If explicit-policy-indicator is not set:

–	If the explicit-policy-pending indicator is set and the certificate is not a self-issued intermediate certificate, decrement the corresponding skip-certificates value and, if this value becomes zero, set explicit-policy-indicator.

–	If the requireExplicitPolicy constraint is present in the certificate, perform the following. For a SkipCerts value of 0, set the explicit-policy-indicator. For any other SkipCerts value, set the explicit-policy-pending indicator, and set the corresponding skip-certificates values to the lesser of the SkipCerts value and the previous skip-certificates value (if the inhibit-any-policy-pending indicator was already set).

–	If the requireExplicitPolicy component is present, and the certification path includes a certificate issued by a nominated CA, it is necessary for all certificates in the path to contain, in the certificate policies extension, an acceptable policy identifier. An acceptable policy identifier is the identifier of the certificate policy required by the user of the certification path, the identifier of a policy which has been declared equivalent to it through policy mapping, or any-policy. The nominated CA is either the issuer CA of the certificate containing this extension (if the value of requireExplicitPolicy is 0) or a CA which is the subject of a subsequent certificate in the certification path (as indicated by a non-zero value).

For the end-certificate, the following actions are then performed:

a)	If explicit-policy-indicator is set, check that the authorities-constrained-policy-set table is not empty. If any of the above checks were to fail, then the procedure shall terminate, returning a failure indication, an appropriate reason code, explicit-policy-indicator and null values in the user-constrained-policy-set and the authorities-constrained-policy-set table.

If none of the above checks were to fail on the end certificate, then the user-constrained-policy-set shall be calculated by forming the intersection of the authorities-constrained-policy-set and the initial-policy-set. If the authorities-constrained-policy-set[0, path-depth] is any-policy, then the authorities-constrained-policy-set is any-policy  Otherwise, the authorities-constrained-policy-set is, for each row in the table, the value in the left-most cell which does not contain the identifier any-policy. Then the procedure shall terminate, returning a success indication together with the explicit-policy-indicator, the authorities-constrained-policy-set table and the user-constrained-policy-set. If the intersection of the authority constrained set and user constrained set is null, the path is valid under authority constrained policy(s), but none is acceptable to the user."

RECOMMENDATION X.509 (1997) | ISO/IEC 9594-8:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers Draft Technical Corrigenda 8 and 9.

###### 1)	Defect reports resolved by Draft Technical Corrigendum 8

(covering resolutions to defect reports 226, 227 and 240)

## 1.1)	This corrects the defects reported in defect report 226

In 11.2, delete the 2nd paragraph:

The production of a certificate … compromise unlikely.

## 1.2)	This corrects the defects reported in defect report 227

In 12.2.2.1, add the following 2 sentences to the end of the paragraph that begins with "Certification authorities shall assign…":

The keyIdentifier form can be used to select CA certificates during path construction.  The authorityCertIssuer, authoritySerialNumber pair can only be used to provide preference to one certificate over others during path construction.

## 1.3)	This corrects the defects reported in defect report 240

The following corrections should be made to the 1997 edition authenticationFramework module in Annex A:

1)	Add id-mr to the list of objects imported from UsefulDefinitions module in the authenticationFramework module.

2)	Add AttributeType, Attribute, and MATCHING-RULE to the set of objects imported into the authenticationFramework module from the InformationFramework module.

3)	Add GeneralNames to the set of objects imported into the authenticationFramework module from the CertificateExtensions module.

4)	Add the following definition to the authenticationFramework module because this is imported into other modules in the X.500 series of Recommendations, but had never been included in the 1997 text of this Recommendation:

HASH {ToBeHashed}	::=				   SEQUENCE {

algorithmIdentifier		AlgorithmIdentifier,

hashValue				BIT STRING ( CONSTRAINED BY {

-- must be the result of applying a hashing procedure to the DER-encoded octets --

-- of a value of --ToBeHashed } ) }

5)	Add the following OID assignments in the authenticationFramework module:

id-at-attributeCertificateRevocationList	OBJECT IDENTIFIER 	::=	{id-at 59}

id-mr-attributeCertificateMatch			OBJECT IDENTIFIER 	::=	{id-mr 42}

6)	Add Time to the set of objects imported into the certificateExtensions module from the authenticationFramework module.

7)	In the certificateExtensions module, and in the main text of 12.7.2 replace:

CertPolicySet ::= SEQUENCE (1..MAX) OF CertPolicyId

with:

CertPolicySet ::= SEQUENCE SIZE (1..MAX) OF CertPolicyId

2)	Defect reports resolved by Draft Technical Corrigendum 9

(covering resolutions to defect reports 244, 256, 257 and 258)

## 2.1)	This corrects the defects reported in defect report 244

In clause 8:

In the paragraph that begins "The extensions field allows addition of new ...", add the following two sentences to the end of the paragraph:

When a certificate-using implementation recognizes and is able to process an extension, then the certificate-using implementation shall process the extension regardless of the value of the criticality flag. Note that any extension that is flagged non-critical will cause inconsistent behaviour between certificate-using systems that will process the extension and certificate-using systems that do not recognize the extension will ignore it.

Add the following immediately after the paragraph that begins "If unknown elements appear within the extension …":

A CA has three options with respect to an extension:

i)	it can exclude the extension from the certificate;

ii)	it can include the extension and flag it non-critical;

iii)	it can include the extension and flag it critical.

A validation engine has two possible actions to take with respect to an extension:

i)	it can ignore the extension and accept the certificate (all other things being equal);

ii)	it can process the extension and accept or reject the certificate depending on the content of the extension and the conditions under which processing is occuring (e.g. the current values of the path processing variables).

Some extensions can only be marked critical. In these cases, a validation engine that understands the extension processes it and acceptance/rejection of the certificate is dependent (at least in part) on the content of the extension. A validation engine that does not understand the extension rejects the certificate.

Some extensions can only be marked non-critical. In these cases, a validation engine that understands the extension processes it and acceptance/rejection of the certificate is dependent (at least in part) on the content of the extension. A validation engine that does not understand the extension accepts the certificate (unless factors other than this extension cause it to be rejected).

Some extensions can be marked critical or non-critical. In these cases, a validation engine that understands the extension processes it and acceptance/rejection of the certificate is dependent (at least in part) on the content of the extension, regardless of the criticality flag. A validation engine that does not understand the extension accepts the certificate if the extension is marked non-critical (unless factors other than this extension cause it to be rejected) and rejects the certificate if the extension is marked critical.

When a CA considers including an extension in a certificate, it does so with the expectation that its intent will be adhered to wherever possible. If it is necessary that the content of the extension be considered prior to any reliance on the certificate, a CA would flag the extension critical. This must be done with the realization that any validation engine that does not process the extension will reject the certificate (probably limiting the set of applications that can verify the certificate). The CA may mark certain extensions non-critical to achieve backward compatibility with validation applications that cannot process the extensions. Where the need for backward compatibility and interoperability with validation applications incapable of processing the extensions is more vital than the ability of the CA to enforce the extensions, then these optionally critical extensions would be marked non-critical.  It is most likely that CAs would set optionally critical extensions as non-critical during a transition period while the verifiers' certificate processing applications are upgraded to ones that can process the extensions.

In clause 12.1:

In the paragraph that begins "In a certificate or CRL, an extension is flagged ...", add the following immediately after the third sentence that ends with "... ignoring the extension":

If an extension is flagged non-critical, a certificate-using system that does recognize the extension, shall process the extension.

In clause 12.2.2.3:

In the paragraph that begins "If the extension is flagged non-critical ...", replace the second sentence with the following:

If this extension is present, and the certificate-using system recognizes and processes the keyUsage extension type, then the certificate using system shall ensure that the certificate shall be used only for a purpose for which the corresponding key usage bit is set to one.

In clause 12.2.2.4:

In the paragraph that begins "If the extension is flagged non-critical ...", replace the second sentence with the following:

If this extension is present, and the certificate-using system recognizes and processes the extendedKeyUsage extension type, then the certificate using system shall ensure that the certificate shall be used only for one of the purposes indicated.

In clause 12.4.2.1:

In the 4th paragraph following the ASN.1, replace: "If this extension is present and is flagged critical then:" with the following:

If this extension is present and is flagged critical, or is flagged non-critical but is recognized by the certificate-using system, then:

In clause 12.4.2.2:

Replace the last sentence "If this extension is present and is flagged critical ..." with the following:

If this extension is present and is flagged critical, or is flagged non-critical but is recognized by the certificate-using system, then the certificate-using system shall check that the certification path being processed is consistent with the value in this extension.

## 2.2)	This corrects the defects reported in defect report 256

In clause 8:

In the first paragraph of the description of the cross certificate pair attribute (that begins "The forward elements …"), add the following as a new 3rd sentence:

If a CA issues a certificate to another CA, and the subject CA is not a subordinate to the issuer CA in a hierarchy, then the issuer CA must place that certificate in the reverse element of the crossCertificatePair attribute of its own directory entry.

## 2.3)	This corrects the defects reported in defect report 257

In clause 8 in the ASN.1 construct CertificatePair:

Replace forward with issuedByThisCA and

Replace reverse with issuedToThisCA and make changes to the associated text as outlined below.

In the descriptive text, throughout ITU-T X.509, update the text accordingly to reflect these new terms. This includes the following specific clauses:

–	general descriptive text in clause 8;

–	ASN.1 and descriptive text for the cross certificate pair attribute in clause 8;

–	ASN.1 and descriptive text for the associated matching rules in clauses 12.7.3 and 12.7.4, and

–	the duplicate ASN.1 constructs in Annex A.

Also, add the following text to the end of the paragraph that begins "The forward elements ...":

The term forward was used in previous editions for issuedByThisCA, and the term reverse was used in previous editions for issuedToThisCA.

## 2.4)	This corrects the defects reported in defect report 258

In clause 8, add the following as a new paragraph at the end of the clause, immediately before the first subclause 8.1:

Each certificate in a certification path shall be unique. No certificate may appear more than once in a value of theCACertificates component of CertificationPath or in a value of certificate in the CrossCertificates component of ForwardCertificationPath.

In clause 12.4.3 add the following Note immediately after bullet a), "a set of certificates":

NOTE – Each certificate in a certification path is unique. A path that contains the same certificate two or more times is not a valid certification path.

RECOMMENDATION X.509 (1997) | ISO/IEC 9594-8:1998

TECHNICAL  CORRIGENDUM  3 (DTC 10)

(covering resolutions to defect reports 272, 273, 275 and 277)

# 1)	This corrects the defects reported in defect report 272

In 12.4.2.1, add the following text to the end of the paragraph that begins with "The pathLenConstraint component shall be present only if…"

The constraint takes effect beginning with the next certificate in the path. The constraint restricts the length of the segment of the certification path between the certificate containing this extension and the end-entity certificate. It has no impact on the number of CA-certificates in the certification path between the trust anchor and the certificate containing this extension. Therefore, the length of a complete certification path may exceed the maximum length of the segment constrained by this extension. The constraint controls the number of non self-issued CA certificates between the CA certificate containing the constraint and the end-entity certificate. Therefore the total length of this segment of the path, excluding self-issued certificates, may exceed the value of the constraint by as many as two certificates. (This includes the certificates at the two endpoints of the segment plus the CA certificates between the two endpoints that are constrained by the value of this extension.)

# 2)	This corrects the defects reported in defect report 273

Replace 12.4.2.2 with the following:

#### 12.4.2.2	Name constraints extension

This field, which shall be used only in a CA-certificate, indicates a name space within which all subject names in subsequent certificates in a certification path must be located. This field is defined as follows:

nameConstraints EXTENSION ::= {
	SYNTAX			NameConstraintsSyntax
	IDENTIFIED BY 	id-ce-nameConstraint }

NameConstraintsSyntax ::= SEQUENCE {
	permittedSubtrees		[0]	GeneralSubtrees OPTIONAL,
	excludedSubtrees		[1]	GeneralSubtrees OPTIONAL,
	requiredNameForms	[2]	NameForms OPTIONAL  }

GeneralSubtrees ::= SEQUENCE SIZE (1..MAX) OF GeneralSubtree

GeneralSubtree ::= SEQUENCE {
	base				GeneralName,
	minimum		[0]	BaseDistance DEFAULT 0,
	maximum		[1]	BaseDistance OPTIONAL }

BaseDistance ::= INTEGER (0..MAX)

NameForms  ::= SEQUENCE {

basicNameForms		[0]	BasicNameForms OPTIONAL,

otherNameForms		[1]	SEQUENCE SIZE (1..MAX) OF OBJECT IDENTIFIER OPTIONAL }

(ALL EXCEPT ({ -- none; i.e.:at least one component shall be present -- }))

BasicNameForms  ::= BIT STRING {

rfc822Name 					(0),

dNSName					(1),

x400Address					(2),

directoryName				(3),

ediPartyName				(4),

uniformResourceIdentifier 	(5),

iPAddress					(6),

registeredID					(7) }  (SIZE (1..MAX))

If present, the permittedSubtrees and excludedSubtrees components each specify one or more naming subtrees, each defined by the name of the root of the subtree and optionally, within that subtree, an area that is bounded by upper and/or lower levels. If permittedSubtrees is present, subject names within these subtrees are acceptable.  If excludedSubtrees is present, any certificate issued by the subject CA or subsequent CAs in the certification path that has a subject name within these subtrees is unacceptable. If both permittedSubtrees and excludedSubtrees are present and the name spaces overlap, the exclusion statement takes precedence for names within that overlap. If neither permitted nor excluded subtrees are specified for a name form, then any name within that name form is acceptable. If requiredNameForms is present, all subsequent certificates in the certification path must include a name of at least one of the required name forms.

If permittedSubtrees is present, the following applies to all subsequent certificates in the path. If any certificate contains a subject name (in the subject field or subjectAltNames extension) of a name form for which permitted subtrees are specified, the name must fall within at least one of the specified subtrees. If any certificate contains only subject names of name forms other than those for which permitted subtrees are specified, the subject names are not required to fall within any of the specified subtrees. For example, assume that two permitted subtrees are specified, one for the DN name form and one for the rfc822 name form, no excluded subtrees are specified, but requiredNameForms is specified with the directoryName bit and rfc822Name bit present. A certificate that contained only names other than a directory name or rfc822 name would be unacceptable. If requiredNameForms were not specified, however, such a certificate would be acceptable. For example, assume that two permitted subtrees are specified, one for the DN name form and one for the rfc822 name form, no excluded subtrees are specified, and requiredNameForms is not present. A certificate that only contained a DN and where the DN is within the specified permitted subtree would be acceptable. A certificate that contained both a DN and an rfc822 name and where only one of them is within its specified permitted subtree would be unacceptable. A certificate that contained only names other than a DN or rfc822 name would also be acceptable.

If excludedSubtrees is present, any certificate issued by the subject CA or subsequent CAs in the certification path that has a subject name (in the subject field or subjectAltNames extension) within these subtrees is unacceptable. For example, assume that two excluded subtrees are specified, one for the DN name form and one for the rfc822 name form. A certificate that only contained a DN and where the DN is within the specified excluded subtree would be unacceptable. A certificate that contained both a DN and an rfc822 name and where at least one of them is within its specified excluded subtree would be unacceptable.

When a certificate subject has multiple names of the same name form (including, in the case of the directoryName name form, the name in the subject field of the certificate if non-null), then all such names shall be tested for consistency with a name constraint of that name form.

If requiredNameForms is present, all subsequent certificates in the certification path must include a subject name of at least one of the required name forms.

Of the name forms available through the GeneralName type, only those name forms that have a well-defined hierarchical structure may be used in the permittedSubtrees and excludedSubtrees fields. The directoryName name form satisfies this requirement; when using this name form a naming subtree corresponds to a DIT subtree.

The minimum field specifies the upper bound of the area within the subtree. All names whose final name component is above the level specified are not contained within the area. A value of minimum equal to zero (the default) corresponds to the base, i.e. the top node of the subtree. For example, if minimum is set to one, then the naming subtree excludes the base node but includes subordinate nodes.

The maximum field specifies the lower bound of the area within the subtree. All names whose last component is below the level specified are not contained within the area. A value of maximum of zero corresponds to the base, i.e. the top of the subtree. An absent maximum component indicates that no lower limit should be imposed on the area within the subtree. For example, if maximum is set to one, then the naming subtree excludes all nodes except the subtree base and its immediate subordinates.

This extension may, at the option of the certificate issuer, be either critical or non-critical. It is recommended that it be flagged critical, otherwise a certificate user may not check that subsequent certificates in a certification path are located in the name space intended by the issuing CA.

Conformant implementations are not required to recognize all possible name forms.

If the extension is present and is flagged critical, a certificate-using implementation must recognize and process all name forms for which there is both a subtree specification (permitted or excluded) in the extension and a corresponding value in the subject field or subjectAltNames extension of any subsequent certificate in the certification path. If an unrecognized name form appears in both a subtree specification and a subsequent certificate, that certificate shall be handled as if an unrecognized critical extension was encountered. If any subject name in the certificate falls within an excluded subtree, the certificate is unacceptable. If a subtree is specified for a name form that is not contained in any subsequent certificate, that subtree can be ignored. If the requiredNameForms component specifies only unrecognized name forms, that certificate shall be handled as if an unrecognized critical extension was encountered. Otherwise, at least one of the recognized name forms must appear in all subsequent certificates in the path.

If the extension is present and is flagged non-critical and a certificate-using implementation does not recognize a name form used in any base component, then that subtree specification may be ignored. If the extension is flagged non-critical and any of the name forms specified in the requiredNameForms component are not recognized by the certificate-using implementation, then the certificate shall be treated as if the requiredNameForms component was absent.

In 12.4.3, add a new path processing variable as follows and renumber subsequent bullets accordingly:

d)	required-name-forms: A (possibly empty) set of sets of name forms. For each set of name forms, every subsequent certificate must contain a name of one of the name forms in the set.

In 12.4.3, add a new initialization step as follows and renumber subsequent bullets accordingly:

d)	Initialize the required-name-forms to an empty set;

In 12.4.3, add a step to the checks applied to all certificates as follows:

h)	If the certificate is not an intermediate self-issued certificate, and if required-name-forms is not an empty set, for each set of name forms in required-name-forms check that there is a subject name in the certificate of one of the name forms in the set.

In 12.4.3, add a step to the constraint recording actions applied to intermediate certificates as follows:

c)	If the nameConstraints extension with a requiredNameForms component is present in the certificate, set the required-name-forms variable to the union of its previous value and the set consisting of the set of name forms specified in the certificate extension. If the requiredNameForms component contains more than one name form, the required-name-forms variable shall signal that a name of at least one of the indicated name forms in this extension shall be present in all subsequent certificates. The union of a previous value of the required-name-forms variable with the value from the current certificate extension is a set of sets signalling requirements for all subsequent certificates. For example if the current required-name-forms is set to requiring that either a DN or an rfc822 name must be present in certificates and the current extension in the certificate being processed indicates that either rfc822 names or DNS names are required, the resulting union that is the new required-name-forms indicates that each of the subsequent certificates must have either an rfc822 name or both a DN and a DNS name.

In Annex A, certificateExtensions module update the ASN.1 for nameConstraints extension as above.

In Annex A, certificateExtensions module add the following:

id-ce-nameConstraint			OBJECT IDENTIFIER	::=	{id-ce 30 1}

In Annex A, certificateExtensions module, delete the following:

id-ce-nameConstraints			OBJECT IDENTIFIER	::=	{id-ce 30}

In Annex A, certificateExtensions module, add the following to the set of OIDs not used in this Specification:

id-ce 30

# 3)	This corrects the defects reported in defect report 275

In 12.2.2.4, add the following as a new second paragraph following the ASN.1 for the extKeyUsage extension.

A CA may assert any-extended-key-usage by using the anyExtendedKeyUsage identifier. This enables a CA to issue a certificate that contains OIDs for extended key usages that may be required by certificate-using applications, without restricting the certificate to only those key usages.  If extended key usage would restrict key usage, then the inclusion of this OID removes that restriction.

anyExtendedKeyUsage  OBJECT IDENTIFIER  ::= { 2 5 29 37 0 }

# 4)	This corrects the defects reported in defect report 277

In 12.4.2.3, in the last sentence of the second paragraph:

Replace "which is the subject of a subsequent certificate" with "which is the issuer of a subsequent certificate".

RECOMMENDATION X.525 (1997) | ISO/IEC 9594-9:1998

Information technology – Open Systems Interconnection –
The Directory:  Replication

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigendum 1.

(defect reports 182, 186)

# 1)	Resolution to defect report 9594/182

Subclause 7.2.2.3

Insert as a fourth new paragraph:

If subordinates is specified, then the supplier shall send subordinate entries and a subordinate reference, and the SDSEs will be of type subr, entry, and cp. The subordinate entries shall contain attributes according to the attribute selection. In addition, if the supplying DSE is of type admPoint, then the SDSE shall additionally be of type admPoint and the administrativeRole attribute shall be supplied. All appropriate subentries, with only the appropriate information, below the admPoint DSE shall also be supplied as SDSEs in the shadowed information.

Subclause 9.2 and Annex A

Replace the UnitOfReplication ASN.1 type as follows (thereby adding subordinates):

UnitOfReplication				::=		SEQUENCE {
	area							AreaSpecification,
	attributes					AttributeSelection,
	knowledge					Knowledge OPTIONAL,
	subordinates					BOOLEAN DEFAULT FALSE }

Insert the following after the description of  knowledgetype:

subordinates is used to indicate that subordinate entries, rather than simply subordinate references, are to be copied to the consumer DSA. subordinates may only be TRUE if knowledge is requested and extendedKnowledge is FALSE.

# 2)	Resolution to defect report 9594/186

Subclause 7.2.2.2

Append the following to a) in the fifth paragraph:

If the entryACI operational attribute is present and holds relevant ACI, e.g. naming, then the attribute (containing at least the relevant ACI) shall always be included in the SDSE.

Subclause 9.2.4.1

Add a new list element d) as follows:

d)	If the entry is refined out, the replacement glue SDSE shall contain the necessary access control information, e.g. reading.

Delete "prescriptive" from Note 2.

RECOMMENDATION X.525 (1997) | ISO/IEC 9594-9:1998

TECHNICAL  CORRIGENDUM  2

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigenda 2, 3, and 4.

# 1)	Defect reports covered by Draft Technical Corrigendum 2

(Covering resolutions to defect report 187, 208 and 243)

## 1.1)	This corrects the defects reported in defect report 9594/187

In 7.2.1.1, add  root to the list of SDSE types.

In 11.3.1.1, delete  root from the list of SDSE types.

## 1.2)	This corrects the defects reported in defect report 9594/208

Insert the following text into 7.2.2.3, at the end of both the second paragraph and the first sentence of the third paragraph (after "appropriate knowledge"):

- and access control information.

Insert a new third paragraph into 7.2.2.3 as follows:

If subordinate knowledge is supplied, and the supplying DSE (of type subr) is also of type admPoint, then the SDSE shall additionally be of type admPoint and the administrativeRole attribute shall be supplied.  If  such a DSE has any immediately subordinate subentries containing PrescriptiveACI relating to the administrative point, then they shall also be supplied as SDSEs in the shadowed information.

NOTE – A DSE can be of type subr and admPoint in a superior DSA, when the naming context in the subordinate DSA is the start of a new administrative area.

Update Figure 3 as follows to show a subentry immediately below a subordinate reference. The subentry contains prescriptiveACI and is part of the shadowed information.

Add supporting text to 7.2 in the paragraph after Figure 3. Insert the following sentences after the sentence "Subordinate knowledge may also be replicated":

Implicit in the subordinate knowledge is the access control information which governs access to the RDN of the subordinate knowledge. When the subordinate entry is an administrative point in another DSA, then part of this access control information may be held in prescriptiveACI subentries beneath the subordinate knowledge.

Add the following new point d) to 9.2.4.1:

d)	if subordinate knowledge (not extended knowledge) is shadowed then any prescriptiveACI in subordinate subentries shall also be copied.

## 1.3)	This corrects the defects reported in defect report 9594/243

In 2.1, change all references ISO/IEC 9594-x:1997 to ISO/IEC 9594-x:1998.

In clause 6, change ITU-T Rec. X.518 | ISO/IEC 9594-5 to ITU-T Rec. X.519 | ISO/IEC 9594-5.

In 9.2 in the UnitOfReplication type, change ContextType to CONTEXT.&id.

In 11.1:

Change CoordinateShadowUpdate to coordinateShadowUpdate.

Remove the last right curly parenthesis in the CoordinateShadowUpdateArgument.

Replace the ASN.1 in Annex A with:

DirectoryShadowAbstractService
	{joint-iso-itu-t ds(5) module(1) directoryShadowAbstractService(15) 3} 
DEFINITIONS   IMPLICIT TAGS   ::=
BEGIN

-- EXPORTS All --

-- The types and values defined in this module are exported for use in the other ASN.1 modules contained 
-- within the Directory Specifications, and for the use of other applications which will use them to access 
-- directory services. Other applications may use them for their own purposes, but this will not constrain
-- extensions and modifications needed to maintain or improve the directory service.

IMPORTS

-- from ITU-T Rec. X.501 | ISO/IEC 9594-2

directoryAbstractService, directoryOperationalBindingTypes, informationFramework,
	disp, distributedOperations, dsaOperationalAttributeTypes, enhancedSecurity,
	opBindingManagement

FROM UsefulDefinitions {joint-iso-itu-t ds(5) module(1) usefulDefinitions(0) 3}

Attribute, AttributeType, CONTEXT, DistinguishedName, RelativeDistinguishedName,
	SubtreeSpecification
		FROM InformationFramework informationFramework

OPERATIONAL-BINDING, OperationalBindingID

FROM OperationalBindingManagement opBindingManagement

DSEType, SupplierAndConsumers

FROM DSAOperationalAttributeTypes dsaOperationalAttributeTypes

OPTIONALLY-PROTECTED, OPTIONALLY-PROTECTED-SEQ

FROM EnhancedSecurity enhancedSecurity

-- from ITU-T Rec. X.511 | ISO/IEC 9594-3

CommonResultsSeq, ContextSelection, directoryBind, directoryUnbind, EntryModification,
	SecurityParameters
		FROM DirectoryAbstractService directoryAbstractService

-- from ITU-T Rec. X.518 | ISO/IEC 9594-4

AccessPoint

FROM DistributedOperations distributedOperations

-- from ITU-T Rec. X.519 | ISO/IEC 9594-5

id-op-binding-shadow
		FROM DirectoryOperationalBindingTypes directoryOperationalBindingTypes

id-errcode-shadowError, id-opcode-coordinateShadowUpdate, id-opcode-requestShadowUpdate,
	id-opcode-updateShadow, reliableShadowSupplierInitiatedAC,

reliableShadowConsumerInitiatedAC,
	shadowConsumerInitiatedAC, shadowSupplierInitiatedAC
		FROM DirectoryInformationShadowProtocol disp

-- from ITU-T Rec. X.880 | ISO/IEC 13712-1

ERROR, OPERATION

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t remote-operations(4) informationObjects(5) version1(0) }	;

-- bind and unbind operations --

dSAShadowBind  		OPERATION	::=	directoryBind

dSAShadowUnbind	OPERATION	::=	directoryUnbind

-- shadow operational binding --

shadowOperationalBinding OPERATIONAL-BINDING  ::=  {
	AGREEMENT				  ShadowingAgreementInfo
	APPLICATION CONTEXTS {
		{ shadowSupplierInitiatedAC
			APPLIES TO 		{ All-operations-supplier-initiated } } |
		{ shadowConsumerInitiatedAC
			APPLIES TO 		{ All-operations-consumer-initiated } } |
		{ reliableShadowSupplierInitiatedAC
			APPLIES TO 		{ All-operations-supplier-initiated } } |
		{ reliableShadowConsumerInitiatedAC
			APPLIES TO 		{ All-operations-consumer-initiated } } }
	ASYMMETRIC
		ROLE-A		{	-- shadow supplier role
			ESTABLISHMENT-INITIATOR   		TRUE
			ESTABLISHMENT-PARAMETER		NULL
			MODIFICATION-INITIATOR			TRUE
			TERMINATION-INITIATOR			TRUE }
		ROLE-B		{	-- shadow consumer role
			ESTABLISHMENT-INITIATOR    		TRUE
			ESTABLISHMENT-PARAMETER		NULL
			MODIFICATION-INITIATOR			TRUE
			MODIFICATION-PARAMETER   		ModificationParameter
			TERMINATION-INITIATOR			TRUE }
	ID	id-op-binding-shadow }

-- types --

ModificationParameter  		::=  	SEQUENCE {
	secondaryShadows	SET OF SupplierAndConsumers }

AgreementID  						::=  	OperationalBindingID

ShadowingAgreementInfo  		::=  	SEQUENCE {
	shadowSubject				UnitOfReplication,
	updateMode					UpdateMode DEFAULT supplierInitiated : onChange : TRUE,
	master						AccessPoint OPTIONAL,
	secondaryShadows	[2]	BOOLEAN DEFAULT FALSE }

UnitOfReplication  			::=  	SEQUENCE {
	area					AreaSpecification,
	attributes			AttributeSelection,
	knowledge   			Knowledge OPTIONAL,
	subordinates			BOOLEAN DEFAULT FALSE,
	contextSelection		ContextSelection OPTIONAL,
	supplyContexts		[0]	CHOICE {
		allContexts   				NULL,
		selectedContexts  		SET SIZE (1..MAX) OF CONTEXT.&id } OPTIONAL }

AreaSpecification  			::=  SEQUENCE  {
	contextPrefix		DistinguishedName,
	replicationArea	SubtreeSpecification }

Knowledge  					::=  SEQUENCE  {
	knowledgeType		ENUMERATED {

master		(0),
			shadow		(1),
			both			(2) },

extendedKnowledge	BOOLEAN DEFAULT FALSE }

AttributeSelection  					::=  SET OF ClassAttributeSelection

ClassAttributeSelection  		::=  SEQUENCE {
	class  				OBJECT IDENTIFIER OPTIONAL,
	classAttributes		ClassAttributes DEFAULT allAttributes : NULL }

ClassAttributes  				::=  CHOICE {
	allAttributes			NULL,
	include				[0]	AttributeTypes,
	exclude				[1]	AttributeTypes }

AttributeTypes  						::=  SET OF AttributeType

UpdateMode  					::=  CHOICE {
	supplierInitiated		[0]	SupplierUpdateMode,
	consumerInitiated		[1]	ConsumerUpdateMode }

SupplierUpdateMode  			::=  CHOICE {
	onChange			BOOLEAN,
	scheduled			SchedulingParameters }

ConsumerUpdateMode  				::=  SchedulingParameters

SchedulingParameters  		::=  SEQUENCE {
	periodic				PeriodicStrategy OPTIONAL,  -- must be present if othertimes is set to FALSE --
	othertimes			BOOLEAN DEFAULT FALSE }

PeriodicStrategy  				::=  SEQUENCE {
	beginTime			Time OPTIONAL,
	windowSize			INTEGER,
	updateInterval		INTEGER }

Time  ::=  GeneralizedTime

-- as per 41.3 b) and c) of ITU-T Rec. X.680 | ISO/IEC 8824-1

-- shadow operations, arguments, and results --

All-operations-consumer-initiated  OPERATION  ::=  {

requestShadowUpdate | updateShadow }

All-operations-supplier-initiated  OPERATION  ::=  {

coordinateShadowUpdate | updateShadow }

coordinateShadowUpdate  OPERATION  ::=  {
	ARGUMENT		CoordinateShadowUpdateArgument
	RESULT			CoordinateShadowUpdateResult
	ERRORS			{ shadowError }
	CODE			id-opcode-coordinateShadowUpdate }

CoordinateShadowUpdateArgument  ::=  OPTIONALLY-PROTECTED { [0] SEQUENCE {
	agreementID		AgreementID,
	lastUpdate			Time OPTIONAL,
	updateStrategy		CHOICE {
		standard				ENUMERATED {
			noChanges   			(0),
			incremental  			(1),
			total					(2) },
		other   				EXTERNAL },
	securityParameters	SecurityParameters OPTIONAL } }

CoordinateShadowUpdateResult  ::=  CHOICE {
	null				NULL,
	information  		OPTIONALLY-PROTECTED { [0] SEQUENCE {
		agreementID 			AgreementID,
		lastUpdate				Time OPTIONAL,
		COMPONENTS OF		CommonResultsSeq } } }

requestShadowUpdate 	OPERATION  ::=  {
	ARGUMENT  	RequestShadowUpdateArgument
	RESULT		RequestShadowUpdateResult
	ERRORS		{ shadowError }
	CODE		id-opcode-requestShadowUpdate }

RequestShadowUpdateArgument  ::=  OPTIONALLY-PROTECTED { [0] SEQUENCE {
	agreementID		AgreementID,
	lastUpdate			Time OPTIONAL,
	requestedStrategy	CHOICE {
		standard	ENUMERATED {
			incremental  	(1),
			total			(2) },
		other   	EXTERNAL },
	securityParameters	SecurityParameters OPTIONAL } }

RequestShadowUpdateResult   ::=   CHOICE {
	null			NULL,
	information		OPTIONALLY-PROTECTED { [0] SEQUENCE {
		agreementID		AgreementID, 
		lastUpdate			Time OPTIONAL,
		COMPONENTS OF	CommonResultsSeq } } }

updateShadow  OPERATION  ::=  {
	ARGUMENT	UpdateShadowArgument
	RESULT		UpdateShadowResult
	ERRORS		{ shadowError }
	CODE		id-opcode-updateShadow }

UpdateShadowArgument  ::=	OPTIONALLY-PROTECTED { [0] SEQUENCE {
	agreementID		AgreementID,
	updateTime  		Time,
	updateWindow		UpdateWindow OPTIONAL,
	updatedInfo		RefreshInformation,
	securityParameters	SecurityParameters OPTIONAL } }

UpdateShadowResult  ::=	CHOICE {
	null				NULL,
	information   		OPTIONALLY-PROTECTED { [0] SEQUENCE {
		agreementID		AgreementID, 
		lastUpdate			Time OPTIONAL,
		COMPONENTS OF	CommonResultsSeq } } }

UpdateWindow  ::=  SEQUENCE {
	start		Time,
	stop		Time }

RefreshInformation  ::=	CHOICE {
	noRefresh			NULL,
	total			[0]	TotalRefresh,
	incremental		[1]	IncrementalRefresh,
	otherStrategy		EXTERNAL }

TotalRefresh  ::=	SEQUENCE  {
	sDSE  	SDSEContent OPTIONAL,
	subtree	SET SIZE (1..MAX) OF Subtree OPTIONAL }

SDSEContent  ::=  SEQUENCE {
	sDSEType			SDSEType,
	subComplete	[0]	BOOLEAN DEFAULT FALSE,
	attComplete	[1]	BOOLEAN OPTIONAL,
	attributes			SET OF Attribute,
	attValIncomplete   	SET OF AttributeType DEFAULT {} }

SDSEType  				::=  DSEType

Subtree  				::=  SEQUENCE {
	rdn				RelativeDistinguishedName,
	COMPONENTS OF	TotalRefresh }

IncrementalRefresh  		::=  SEQUENCE OF IncrementalStepRefresh

IncrementalStepRefresh 	::=  SEQUENCE {
	sDSEChanges		CHOICE {
		add				[0]	SDSEContent,
		remove				NULL,
		modify			[1]	ContentChange } OPTIONAL,
	subordinateUpdates	SEQUENCE SIZE (1..MAX) OF SubordinateChanges OPTIONAL }

ContentChange  			::=  SEQUENCE {
	rename			CHOICE {
		newRDN		RelativeDistinguishedName,
		newDN		DistinguishedName } OPTIONAL,
	attributeChanges 	CHOICE {
		replace		[0]		SET SIZE (1..MAX) OF Attribute,
		changes		[1]		SEQUENCE SIZE (1..MAX) OF EntryModification } OPTIONAL,
	sDSEType					SDSEType,
	subComplete		[2]		BOOLEAN DEFAULT FALSE,
	attComplete		[3]		BOOLEAN OPTIONAL,
	attValIncomplete  	SET OF AttributeType DEFAULT {} }

SubordinateChanges  		::=  SEQUENCE {
	subordinate		RelativeDistinguishedName,
	changes			IncrementalStepRefresh }

-- errors and parameters --

shadowError  ERROR  ::=  {
	PARAMETER		OPTIONALLY-PROTECTED-SEQ { SEQUENCE {
		problem				ShadowProblem,
		lastUpdate   			Time OPTIONAL,
		updateWindow			UpdateWindow OPTIONAL,
		COMPONENTS OF		CommonResultsSeq } }
	CODE		id-errcode-shadowError }

ShadowProblem  ::=  INTEGER {
	invalidAgreementID			(1),
	inactiveAgreement			(2),
	invalidInformationReceived		(3),
	unsupportedStrategy			(4),
	missedPrevious				(5),
	fullUpdateRequired			(6),
	unwillingToPerform			(7),
	unsuitableTiming  			(8),
	updateAlreadyReceived			(9),
	invalidSequencing			(10),
	insufficientResources			(11) }

END  -- DirectoryShadowAbstractService

# 2)	Defect reports covered by Draft Technical Corrigendum 3

(Covering resolutions to defect report 245)

## 2.1)	This corrects the defects reported in defect report 9594/245

In 9.2,  UnitOfReplication, change the supplyContexts component to:

supplyContexts			[0]	CHOICE {
		allContexts   					NULL,
		selectedContexts  			SET OF CONTEXT.&id } OPTIONAL

In clause 11.1, CoordinateShadowUpdateResult, clause 11.2,  RequestShadowUpdateResult, clause 11.3, UpdateShadowResult, clause 12, shadowError and associated text, change CommonResults to CommonResultsSeq.

(Changes to Annex A are subsumed by resolution to defect report 243.)

# 3)	Defect reports covered by Draft Technical Corrigendum 4

(Covering resolutions to defect reports 228 and 242)

## 3.1)	This corrects the defects reported in defect report 9594/228

Delete any occurrence of:

,
		DIRQOP.&…-QOP{@dirqop}

In 11.1, change CoordinateShadowUpdateResult to:

CoordinateShadowUpdateResult  ::=  CHOICE {
	null				NULL,
	information  		OPTIONALLY-PROTECTED { [0] SEQUENCE {
		agreementID  		AgreementID,
		lastUpdate			Time OPTIONAL,
		COMPONENTS OF	CommonResultsSeq } } }

In 11.2, change RequestShadowUpdateResult  to:

RequestShadowUpdateResult   ::=   CHOICE {
	null				NULL,
	information   		OPTIONALLY-PROTECTED { [0] SEQUENCE {
		agreementID			AgreementID, 
		lastUpdate			Time OPTIONAL,
		COMPONENTS OF	CommonResultsSeq } } }

In 11.3, change UpdateShadowResult to:

UpdateShadowResult  ::=  CHOICE {
	null				NULL,
	information   		OPTIONALLY-PROTECTED { [0] SEQUENCE {
		agreementID			AgreementID, 
		lastUpdate			Time OPTIONAL,
		COMPONENTS OF	CommonResultsSeq } } }

In clause 12 in the shadowError construct, change OPTIONALLY-PROTECTED to OPTIONALLY-PROTECTED-SEQ.

(Changes to Annex A are subsumed by resolution to Defect Report 243.)

## 3.2)	This corrects the defects reported in defect report 9594/242

Add size limit SIZE (1..MAX) to all optional SET OF and SEQUENCE OF constructs.

RECOMMENDATION X.530 (1997) | ISO/IEC 9594-10:1998

Information technology – Open Systems Interconnection – 
The Directory:  Use of systems management for administration of the Directory

TECHNICAL  CORRIGENDUM  1

NOTE – This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigendum 1.

# Defect reports resolved by Draft Technical Corrigendum 1

(Covering resolutions to defect report 252)

### This corrects the defects reported in defect report 9594/252

In A.9:

Replace the module identification with:

DirectoryManagement {joint-iso-itu-t ds(5) module(1) directoryManagement(27) 1 }

Add basicAccessControl and upperBounds to the import from UsefulDefinitions.

Remove ub-common-name from the import from SelectedAttributeTypes

Add a new import:

ub-common-name
		FROM UpperBounds upperBounds

Remove AttributeTypeAndValue from the import from InformationFramework.

Replace:

Id-mat-foundLocalEntries			OBJECT IDENTIFIER	::=	{id-mat 6}

with:

id-mat-foundLocalEntries			OBJECT IDENTIFIER	::=	{id-mat 6}

Appendix B
	
Technical Corrigenda to Rec. X.500 (2000&2001) | ISO/IEC 9594:2000&2001
4th Edition

## Summary of 4th Edition Technical Corrigenda

| DTC # | Defect Reports resolved | Ballot Close | Published As | History |
| --- | --- | --- | --- | --- |
| ### ITU-T Rec. X.501 (2001) ¦ ISO/IEC 9594-2:2001 |  |  |  |  |
| 2-DTC1 | 250, 259 | 10 Jan 2001 | 4th edition | Erik after Orlando 2000. Incorporated into published edition. |
| ### ITU-T Rec. X.511 (2001) ¦ ISO/IEC 9594-3:2001 |  |  |  |  |
| 3-DTC1 | 249, 262, 268 | 10 Jan 2001 | 4th edition | Erik after Orlando 2000. Incorporated into published edition. |
| ### ITU-T Rec. X.518 (2001) ¦ ISO/IEC 9594-4:2001 |  |  |  |  |
| 4-DTC1 | 251, 253, 254, 264 | 10 Jan 2001 | 4th edition | Erik after Orlando 2000. Incorporated into published edition. |
| ### ITU-T Rec. X.519 (2001) ¦ ISO/IEC 9594-5:2001 |  |  |  |  |
| 5-DTC1 | 271 | 10 Jan 2001 | 4th edition | Erik after Orlando 2000. Incorporated into published edition. |
| ### ITU-T Rec. X.520 (2001) ¦ ISO/IEC 9594-6:2001 |  |  |  |  |
| 6-DTC2 | 251, 253, 270 | 10 Jan 2001 | 4th edition | Erik after Orlando 2000. Incorporated into published edition. |
| ### ITU-T Rec. X.509 (2000) ¦ ISO/IEC 9594-8:2001 |  |  |  |  |
| 8-DTC1 | 244, 256, 257, 258 | 10 Jan 2001 | 4th edition | Sharon after Orlando 2000. Comments resolved at Geneva 2001. Incorporated into published edition |
| 8-DTC2 | 272 - 279 | 9 August 2001 6N 11966 | 8-TC1 | Sharon in April 2001 SOV SC6N12012 comments resolved by editor |
| 8-DTC3 | 280-282 | 27 Nov 2001 6N 12016 |  | Sharon 28 June 2001 |

RECOMMENDATION X.509 (2000) | ISO/IEC 9594-8:2001

Information technology – Open Systems Interconnection – 
The Directory:  Public-key and attribute certificate frameworks

TECHNICAL  CORRIGENDUM  1

NOTE :	This Technical Corrigendum covers the result of the ballot resolutions of Draft Technical Corrigendum 2.

(covering resolutions to defect reports 272, 273, 274, 275, 276, 277, 278 & 279)

# 1)	This corrects the defects reported in defect report 272

In 8.4.2.1, add the following text to the end of the paragraph that begins with "The pathLenConstraint component shall be present only if…"

The constraint takes effect beginning with the next certificate in the path. The constraint restricts the length of the segment of the certification path between the certificate containing this extension and the end-entity certificate. It has no impact on the number of CA-certificates in the certification path between the trust anchor and the certificate containing this extension. Therefore, the length of a complete certification path may exceed the maximum length of the segment constrained by this extension. The constraint controls the number of non self-issued CA certificates between the CA certificate containing the constraint and the end-entity certificate. Therefore the total length of this segment of the path, excluding self-issued certificates, may exceed the value of the constraint by as many as two certificates. (This includes the certificates at the two endpoints of the segment plus the CA certificates between the two endpoints that are constrained by the value of this extension.)

In 15.5.2.1, in the paragraph that begins with "The pathLenConstraint component is meaningful only if…", replace the last two sentences of this paragraph with the following:

The constraint restricts the length of the segment of the delegation path between the certificate containing this extension and the end-entity certificate. It has no impact on the number of AA-certificates in the delegation path between the trust anchor and the certificate containing this extension. Therefore, the length of a complete delegation path may exceed the maximum length of the segment constrained by this extension. The constraint controls the number of AA certificates between the AA certificate containing the constraint and the end-entity certificate. Therefore the total length of this segment of the path may exceed the value of the constraint by as many as two certificates. (This includes the certificates at the two endpoints of the segment plus the AA certificates between the two endpoints that are constrained by the value of this extension.)

# 2)	This corrects the defects reported in defect report 273

Replace 8.4.2.2 with the following:

#### 8.4.2.2	Name constraints extension

This field, which shall be used only in a CA-certificate, indicates a name space within which all subject names in subsequent certificates in a certification path must be located. This field is defined as follows:

nameConstraints EXTENSION ::= {
	SYNTAX		NameConstraintsSyntax
	IDENTIFIED BY	id-ce-nameConstraint }

NameConstraintsSyntax ::= SEQUENCE {
	permittedSubtrees	[0]	GeneralSubtrees OPTIONAL,
	excludedSubtrees	[1]	GeneralSubtrees OPTIONAL,
	requiredNameForms	[2]	NameForms OPTIONAL  }

GeneralSubtrees ::= SEQUENCE SIZE (1..MAX) OF GeneralSubtree

GeneralSubtree ::= SEQUENCE {
	base			GeneralName,
	minimum	[0]	BaseDistance DEFAULT 0,
	maximum	[1]	BaseDistance OPTIONAL }

BaseDistance ::= INTEGER (0..MAX)

NameForms  ::= SEQUENCE {

basicNameForms	[0]	BasicNameForms OPTIONAL,

otherNameForms	[1]	SEQUENCE SIZE (1..MAX) OF OBJECT IDENTIFIER OPTIONAL }

(ALL EXCEPT ({ -- none; i.e.: at least one component shall be present -- }))

BasicNameForms  ::= BIT STRING {

rfc822Name			 	(0),

dNSName				(1),

x400Address			(2),

directoryName			(3),

ediPartyName			(4),

uniformResourceIdentifier 	(5),

iPAddress				(6),

registeredID			(7) }  (SIZE (1..MAX))

If present, the permittedSubtrees and excludedSubtrees components each specify one or more naming subtrees, each defined by the name of the root of the subtree and optionally, within that subtree, an area that is bounded by upper and/or lower levels. If permittedSubtrees is present, subject names within these subtrees are acceptable.  If excludedSubtrees is present, any certificate issued by the subject CA or subsequent CAs in the certification path that has a subject name within these subtrees is unacceptable. If both permittedSubtrees and excludedSubtrees are present and the name spaces overlap, the exclusion statement takes precedence for names within that overlap. If neither permitted nor excluded subtrees are specified for a name form, then any name within that name form is acceptable. If requiredNameForms is present, all subsequent certificates in the certification path must include a name of at least one of the required name forms.

If permittedSubtrees is present, the following applies to all subsequent certificates in the path. If any certificate contains a subject name (in the subject field or subjectAltNames extension) of a name form for which permitted subtrees are specified, the name must fall within at least one of the specified subtrees. If any certificate contains only subject names of name forms other than those for which permitted subtrees are specified, the subject names are not required to fall within any of the specified subtrees. For example, assume that two permitted subtrees are specified, one for the DN name form and one for the rfc822 name form, no excluded subtrees are specified, but requiredNameForms is specified with the directoryName bit and rfc822Name bit present. A certificate that contained only names other than a directory name or rfc822 name would be unacceptable. If requiredNameForms were not specified, however, such a certificate would be acceptable. For example, assume that two permitted subtrees are specified, one for the DN name form and one for the rfc822 name form, no excluded subtrees are specified, and requiredNameForms is not present. A certificate that only contained a DN and where the DN is within the specified permitted subtree would be acceptable. A certificate that contained both a DN and an rfc822 name and where only one of them is within its specified permitted subtree would be unacceptable. A certificate that contained only names other than a DN or rfc822 name would also be acceptable.

If excludedSubtrees is present, any certificate issued by the subject CA or subsequent CAs in the certification path that has a subject name (in the subject field or subjectAltNames extension) within these subtrees is unacceptable. For example, assume that two excluded subtrees are specified, one for the DN name form and one for the rfc822 name form. A certificate that only contained a DN and where the DN is within the specified excluded subtree would be unacceptable. A certificate that contained both a DN and an rfc822 name and where at least one of them is within its specified excluded subtree would be unacceptable.

When a certificate subject has multiple names of the same name form (including, in the case of the directoryName name form, the name in the subject field of the certificate if non-null), then all such names shall be tested for consistency with a name constraint of that name form.

If requiredNameForms is present, all subsequent certificates in the certification path must include a subject name of at least one of the required name forms.

Of the name forms available through the GeneralName type, only those name forms that have a well-defined hierarchical structure may be used in the permittedSubtrees and excludedSubtrees fields. The directoryName name form satisfies this requirement; when using this name form a naming subtree corresponds to a DIT subtree.

The minimum field specifies the upper bound of the area within the subtree. All names whose final name component is above the level specified are not contained within the area. A value of minimum equal to zero (the default) corresponds to the base, i.e. the top node of the subtree. For example, if minimum is set to one, then the naming subtree excludes the base node but includes subordinate nodes.

The maximum field specifies the lower bound of the area within the subtree. All names whose last component is below the level specified are not contained within the area. A value of maximum of zero corresponds to the base, i.e. the top of the subtree. An absent maximum component indicates that no lower limit should be imposed on the area within the subtree. For example, if maximum is set to one, then the naming subtree excludes all nodes except the subtree base and its immediate subordinates.

This extension may, at the option of the certificate issuer, be either critical or non-critical. It is recommended that it be flagged critical, otherwise a certificate user may not check that subsequent certificates in a certification path are located in the name space intended by the issuing CA.

Conformant implementations are not required to recognize all possible name forms.

If the extension is present and is flagged critical, a certificate-using implementation must recognize and process all name forms for which there is both a subtree specification (permitted or excluded) in the extension and a corresponding value in the subject field or subjectAltNames extension of any subsequent certificate in the certification path. If an unrecognized name form appears in both a subtree specification and a subsequent certificate, that certificate shall be handled as if an unrecognized critical extension was encountered. If any subject name in the certificate falls within an excluded subtree, the certificate is unacceptable. If a subtree is specified for a name form that is not contained in any subsequent certificate, that subtree can be ignored. If the requiredNameForms component specifies only unrecognized name forms, that certificate shall be handled as if an unrecognized critical extension was encountered. Otherwise, at least one of the recognized name forms must appear in all subsequent certificates in the path.

If the extension is present and is flagged non-critical and a certificate-using implementation does not recognize a name form used in any base component, then that subtree specification may be ignored. If the extension is flagged non-critical and any of the name forms specified in the requiredNameForms component are not recognized by the certificate-using implementation, then the certificate shall be treated as if the requiredNameForms component was absent.

In 10.3, add a new path processing variable as follows and renumber subsequent bullets accordingly:

d)	required-name-forms: A (possibly empty) set of sets of name forms. For each set of name forms, every subsequent certificate must contain a name of one of the name forms in the set.

In 10.4, add a new initialization step as follows and renumber subsequent bullets accordingly:

d)	Initialize the required-name-forms to an empty set;

In 10.5.1, add a step to the checks applied to all certificates as follows:

h)	If the certificate is not an intermediate self-issued certificate, and if required-name-forms is not an empty set, for each set of name forms in required-name-forms check that there is a subject name in the certificate of one of the name forms in the set.

In 10.5.2, add a step to the constraint recording actions applied to intermediate certificates as follows:

- If the nameConstraints extension with a requiredNameForms component is present in the certificate, set the required-name-forms variable to the union of its previous value and the set consisting of the set of name forms specified in the certificate extension. If the requiredNameForms component contains more than one name form, the required-name-forms variable shall signal that a name of at least one of the indicated name forms in this extension shall be present in all subsequent certificates. The union of a previous value of the required-name-forms variable with the value from the current certificate extension is a set of sets signalling requirements for all subsequent certificates. For example if the current required-name-forms is set to requiring that either a DN or an rfc822 name must be present in certificates and the current extension in the certificate being processed indicates that either rfc822 names or DNS names are required, the resulting union that is the new required-name-forms indicates that each of the subsequent certificates must have either an rfc822 name or both a DN and a DNS name.

In Annex A, certificateExtensions module update the ASN.1 for nameConstraints extension as above.

In Annex A, certificateExtensions module add the following:

id-ce-nameConstraint	OBJECT IDENTIFIER::=	{id-ce 30 1}

In Annex A, certificateExtensions module, delete the following:

id-ce-nameConstraints	OBJECT IDENTIFIER::=	{id-ce 30}

In Annex A, certificateExtensions module, add the following to the set of OIDs not used in this Specification:

id-ce 30

# 3)	This corrects the defects reported in defect report 274

In Annex A,  replace  the AttCertVersion ASN.1 production with:

AttCertVersion	::=	INTEGER  {v2(1) }

In clause 12.1, replace the first paragraph that follows the ASN.1 with the following:

The version differentiates between different versions of the attribute certificate. For attribute certificates issued in accordance with the syntax in this Specification, version must be v2.

# 4)	This corrects the defects reported in defect report 275

In 8.2.2.4, add the following as a new second paragraph following the ASN.1 for the extKeyUsage extension:

A CA may assert any-extended-key-usage by using the anyExtendedKeyUsage identifier. This enables a CA to issue a certificate that contains OIDs for extended key usages that may be required by certificate-using applications, without restricting the certificate to only those key usages.  If extended key usage would restrict key usage, then the inclusion of this OID removes that restriction.

anyExtendedKeyUsage  OBJECT IDENTIFIER  ::= { 2 5 29 37 0 }

# 5)	This corrects the defects reported in defect report 276

In 8.1.5:

In the last sentence, replace "and explicit-policy-pending indicators" with "explicit-policy-pending and inhibit-any-policy indicators".

In 8.4.2.4, in the first sentence:

Replace "for all certificates in the certification path" with "for all non-self-issued certificates in the certification path".

In 10.5.1, step e):

Replace "or if the inhibit-any-policy-indicator is set, then delete" with "or if the inhibit-any-policy-indicator is set and the certificate is not a self-issued intermediate certificate, then delete".

# 6)	This corrects the defects reported in defect report 277

In 8.4.2.3, in the last sentence of the second paragraph:

Replace "which is the subject of a subsequent certificate" with "which is the issuer of a subsequent certificate".

# 7)	This corrects the defects reported in defect report 278

In 8.6.2.6, in the first sentence:

Replace "shall be used only as a certificate extension and may be…" with "may be used either as a certificate or CRL extension. Within certificates, this extension may be…"

# 8)	This corrects the defects reported in defect report 279

In clause 7, add the following immediately after the ASN.1 CrossCertificates production:

PkiPath	::=	SEQUENCE OF Certificate

PkiPath is used to represent a certification path. Within the sequence, the order of certificates is such that the subject of the first certificate is the issuer of the second certificate, etc.

In 11.1.6:

Replace "object class pkiCA" with "pkiCA or pkiUser".

In the last sentence of the last paragraph of clause 7:

Replace "component of CertPath" with "component of CertPath or a value of Certificate in PkiPath."

In 11.2.10:

Delete the PkiPath ASN.1 production.

In the first sentence of 11.2.10:

Replace "cross-certificates" with "certificates".

In 11.2.10, replace the text following the ASN.1 with the following:

This attribute can be stored in a directory entry of object class pkiCA or pkiUser.

When stored in pkiCA entries, values of this attribute contain certification paths excluding end-entity certificates. As such, the attribute is used to store certification paths that are frequently used by relying parties associated with that CA. A value of this attribute can be used in conjunction with any end-entity certificate issued by the last certificate subject in the attribute value.

When stored in pkiUser entries, values of this attribute contain certification paths that include the end-entity certificate. In this case, the end-entity is the user whose entry holds this attribute. The values of the attribute represent complete certification paths for certificates issued to this user.

In 11.3.9, in the last sentence of the first paragraph:

Replace "issued to the CA that issued the end-entity certificate being validated." with "issued to the specified subject."

RECOMMENDATION X.509 (2000) | ISO/IEC 9594-8:2001

TECHNICAL CORRIGENDUM 3

# 1)	From draft Technical Corrigendum 3

(covering resolutions to defect reports 280, 281 & 282)

## 1.1)	This corrects the defects reported in defect report 280

Replace the existing subclause 8.6.2.2 with the following and make associated changes to the ASN.1 in Annex A:

#### 8.6.2.2	Issuing distribution point extension

This CRL extension field identifies the CRL distribution point for this particular CRL, and indicates if the CRL is indirect, or if it is limited to covering only a subset of the revocation information. The limitation may be based on a subset of the certificate population or on a subset of revocation reasons. The CRL is signed by the CRL issuer's private key – CRL distribution points do not have their own key pairs. However, for a CRL distributed via the Directory, the CRL is stored in the entry of the CRL distribution point, which may not be the directory entry of the CRL issuer. If issuing distribution point extension and the CRL scope field are both absent, the CRL shall contain entries for all revoked unexpired certificates issued by the CRL issuer.

After a certificate appears on a CRL, it may be deleted from a subsequent CRL after the certificate's expiry.

This field is defined as follows:

issuingDistributionPoint		EXTENSION	::=		{
	SYNTAX			IssuingDistPointSyntax
	IDENTIFIED BY 	id-ce-issuingDistributionPoint }

IssuingDistPointSyntax ::= SEQUENCE {

-- If containsUserPublicKeyCerts, containsCACerts, containsUserAttributeCerts, containsAACerts, and
-- containsSOAPublicKeyCerts s are all absent, or not set to TRUE, (), the CRL covers all these certificate types

distributionPoint					[0] DistributionPointName OPTIONAL,
containsUserPublicKeyCerts		[1] BOOLEAN DEFAULT FALSE,
containsCACerts					[2] BOOLEAN DEFAULT FALSE,
onlySomeReasons					[3] ReasonFlags OPTIONAL,
indirectCRL						[4] BOOLEAN DEFAULT FALSE,
containsUserAttributeCerts			[5] BOOLEAN DEFAULT FALSE,
containsAACerts					[6] BOOLEAN DEFAULT FALSE,
containsSOAPublicKeyCerts		[7] BOOLEAN DEFAULT FALSE }

The distributionPoint component contains the name of the distribution point in one or more name forms.

If containsUserPublicKeyCerts is true, the CRL contains revocations for end-entity public-key certificates. If containsCACerts is true, the CRL contains revocations for CA certificates.

If onlySomeReasons is present, the CRL only contains revocations for the identified reason or reasons, otherwise the CRL contains revocations for all reasons.

If indirectCRL is true, then the CRL may contain revocation notifications from authorities other than the issuer of the CRL. The particular authority responsible for each entry is as indicated by the certificate issuer CRL entry extension in that entry or in accordance with the defaulting rules described in 8.6.2.3. In such a CRL, it is the responsibility of the CRL issuer to ensure that the CRL is complete in that it contains all revocation entries, consistent with containsUserPublicKeyCerts, containsCACerts, containsUserAttributeCerts, containsAACerts, containsSOAPublicKeyCerts and onlySomeReasons indicators, from all authorities that identify this CRL issuer in their certificates.

If containsUserAttributeCerts is true, the CRL contains revocations for attribute certificates issued to end-entities that are not themselves AAs. If containsAACerts is true, the CRL contains revocations for attribute certificates issued to subjects that are themselves AAs.

If containsSOAPublicKeyCerts is true, the CRL contains revocations for public-key certificates issued to an entity that is an SOA for purposes of privilege management (i.e., certificates that contain the SOAIdentifier extension).

For CRLs distributed via the Directory the following rules apply. If the CRL is a dCRL it shall be distributed via the deltaRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the deltaRevocationList attribute of the CRL issuer entry, regardless of the settings for certificate types covered by the CRL. Unless the CRL is a dCRL:

•	A CRL which has only containsCACerts set shall be distributed via the authorityRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the authorityRevocationList attribute of the CRL issuer entry.

•	A CRL which has only containsAACerts and/or containsSOAPublicKeyCerts set shall be distributed via the attributeAuthorityRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the attributeAuthorityRevocationList attribute of the CRL issuer entry.

•	A CRL which has only containsAACerts and containsUserAttributeCerts set (with or without containsSOAPublicKeyCerts also set) shall be distributed via the attributeCertificateRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the attributeCertificateRevocationList attribute of the CRL issuer entry.

•	A CRL which has only containsCACerts as well as one or both of containsAACerts and/or containsSOAPublicKeyCerts set shall be distributed via the authorityRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the authorityRevocationList attribute of the CRL issuer entry.

•	A CRL which has only containsUserPublicKeyCerts set shall be distributed via the certificateRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the certificateRevocationList attribute of the CRL issuer entry.

•	A CRL which has only containsUserAttributeCerts set shall be distributed via the attributeCertificateRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the attributeCertificateRevocationList attribute of the CRL issuer entry.

•	A CRL which contains any other combination of these certificate type components, including absence of all five, shall be distributed via the certificateRevocationList attribute of the associated distribution point or, if no distribution point is identified, via the certificateRevocationList attribute of the CRL issuer entry.

If the issuingDistributionPoint extension is present in a CRL, and all five certificate type components are absent, or none of these five components are set to TRUE, (containsUserPublicKeyCerts, containsCACerts, containsUserAttributeCerts, containsAACerts, containsSOAPublicKeyCerts), the CRL covers all certificate types, regardless of whether any certificates of a given type have ever been issued by the associated authority. For example, if an authority issues only public-key certificates and issues them both to the users and to other CAs, and issues a single CRL covering revocations for both certificate types, there is no need to set the containsUserPublicKeyCerts and containsCACerts components to TRUE. If compatibility with 3rd edition systems is necessary, it is recommended that the default value FALSE be used for both the containsUserPublicKeyCerts and containsCACerts components. Absence of all five components is adequate in this example. If an authority issued both public-key and attribute certificates to users and to other authorities, but wished to issue partitioned CRLs for those, many combinations are possible. One combination might be a division into two CRLs with one covering all public-key certificates (containsUserPublicKeyCerts and containsCACerts both set to TRUE) and one covering all attribute certificates (containsUserAttributeCerts and containsAACerts both set to TRUE). An alternative for this same authority might be to issue one CRL for all certificates issued to end users (containsUserPublicKeyCerts and containsUserAttributeCerts both set to TRUE) and another CRL for all certificates issued to other authorities (containsCACerts and containsAACerts both set to TRUE).

This extension is always critical. A certificate user which does not understand this extension cannot assume that the CRL contains a complete list of revoked certificates of the identified authority. CRLs not containing critical extensions shall contain all current CRL entries for the issuing authority, including entries for all revoked user certificates and authority certificates.

NOTE 1 – The means by which revocation information is communicated by authorities to CRL issuers is beyond the scope of this Recommendation | International Standard.

NOTE 2 – If an authority publishes a CRL with containsCACerts set to TRUE and with containsUserPublicKeyCerts absent or not set to TRUE, then the authority shall ensure that all CA certificates covered by this CRL contain the basicConstraints extension.

NOTE 3 – If an authority publishes a CRL with containsAACerts set to TRUE and containsUserAttributeCerts absent or not set to TRUE, then the authority shall ensure that all AA certificates covered by this CRL contain the basicAttConstraints extension.

NOTE 4 – If an authority publishes a CRL containsSOAPublicKeyCerts set to TRUE, then the authority shall ensure that all SOA certificates covered by this CRL contain the SOAIdentifier extension.

In subclause 8.5.2.5, and in Annex A, replace the OnlyCertificateTypes ASN.1 construct with the following:

OnlyCertificateTypes	::= BIT STRING {

userPublicKey	(0),

CA			(1),

userAttribute	(2),

AA			(3),

SOAPublicKey	(4) }

## 1.2)	This corrects the defects reported in defect report 281

In subclause 8.6.2.6, add the following paragraph after the ASN.1:

The value of type CRLDistPointsSyntax is as defined in the CRL distribution points extension in subclause 8.6.2.1 of this Specification.

Replace the existing subclause B.5.1.4 with the following:

In order to determine that a CRL is one of the CRLs indicated by a distribution point in the CRL distribution point extension or freshest CRL extension, all of the following conditions shall be true:

–	Either the distribution point field in the CRL’s issuing distribution point extension shall be absent (only when not looking for a critical CRL DP), or one of the names in the distribution point field of the CRL DP or freshest CRL extension shall match one of the names in the distribution point field in the issuing distribution point extension of the CRL. Alternatively, one of the names in the cRLIssuer field of the CRL DP or freshest CRL extension can match one of the names in DP of the IDP; and

–	If the certificate is an end entity certificate, the CRL shall not contain onlyContainsAuthorityCerts field set to TRUE in the issuing distribution point extension of the CRL; and

–	If onlyContainsAuthorityCerts is set to TRUE in the issuing distribution point extension of the CRL, then the certificate being checked shall contain the basicConstraints extension with the cA component set to TRUE; and

–	If the reasons field is present in the CRL DP or freshest CRL extension, the onlySomeReasons field shall be either absent from the issuing distribution point extension of the CRL or contain at least one of the reason codes asserted in the CRL DP or freshest CRL extension; and

–	If the cRLIssuer field is absent from the relevant extension (either CRL DP or freshest CRL), the CRL shall be signed by the same CA that signed the certificate; and

–	If the cRLIssuer field is present in the relevant extension (CRL DP or freshest CRL), the CRL shall be signed by the CRL issuer identified in the cRLIssuer field and the CRL shall contain the issuing distribution point extension with the indirectCRL field set to TRUE.

NOTE – When testing the reasons and cRLIssuer field for presence, the test succeeds only if the field is present in the same DistributionPoint of the CRL DP or freshest CRL extension for which there is a name match in the corresponding distribution point field of the IDP extension in the CRL.

## 1.3)	This corrects the defects reported in defect report 282

In clause 7, in the paragraph immediately following the definition of the version field and in the paragraph immediately following the definition of the extensions field, replace:

"documented in 7.5.2.2 in ITU-T Rec. X.519 | ISO/IEC 9594-5.".

with:

"documented in 12.2.2 in ITU-T Rec. X.519 | ISO/IEC 9594-5.".

In subclause 7.3, immediately following Note 6 and in subclause 12.1 immediately following the definition of the extensions field add the following new paragraph:

"If unknown elements appear within the extension, and the extension is not marked critical, those unknown elements shall be ignored according to the rules of extensibility documented in 12.2.2 in ITU-T Rec. X.519 | ISO/IEC 9594-5.".

Appendix C

Summary of Defect Reports

Defects numbered 001 to 074 apply to the 1st edition only and are not documented here; for these see Version 9 of the Implementor’s Guide.

Defects numbered 075–156, 158, 160, 161, 165, 168, 171, 172, 174, and 175 apply to the 2nd edition only and are not documented here; for these see Version 14 of the Implementors' Guide.

The third edition (1997 | 1998) is identified by the mark 3rd. The 4th edition (2000 for X.509|9594-8 and 2001 for all others) is identified by the mark 4th.

| DR # | Description of Defect | Part # and clause | DR doc # | Source | Status |
| --- | --- | --- | --- | --- | --- |
| 157 | ModifyDN and UnitOfReplication | 4/19.1.4 | 8N363 | Germany | 4-TC1 (3rd) |
| 159 | targetObject in Search (I) procedure | 4/19.3.2.2.1 | 8N363 | Germany | 4-TC1 (3rd) |
| 162 | APInfo procedure | 4/20.4.5 | 8N363 | Germany | 4-TC1 (3rd) |
| 163 | Shadowed information procedure | 9/7.2, 9.Fig.3 | 8N363 | Germany | Accepted Source solution |
| 164 | ASN.1 of SupplierUpdateMode |  |  |  | Rejected |
| 166 | Alias control by alias dereferencing | 3/7.11.1 | 8N363 | Germany | 3-TC1 (3rd) |
| 168 | Protected password |  |  |  | Rejected |
| 169 | Permutable property for PKCS | 8/Clause 7, 8/10.2, 8/10.3 |  | UK | Accepted Not in DTC |
| 170 | Entry selection in search procedure | 3/Fig. B-11 |  | UK | Open |
| 173 | NSSRs in the root entry | 2/18.5 |  | ITU Rapp. | 2-TC1 (3rd) |
| 176 | Access controls on aliases | 3/7.11.1 |  | ITU Rapp. | Open |
| 177 | Distinguished encoding of UTCTime | 8/Clause 9 |  | ISO Rapp. | 8-TC3 |
| 178 | Duplicate of 209 |  |  |  |  |
| 179 | Clarification in returnDN handling | 2/Table K-1 3/7.4.11, B4, B5 |  | Germany | 2-TC1 (3rd) 3-TC1 (3rd) |
| 180 | entryOnly inconsistency | 3/7.3 4/10.3(g)-(o) |  | Germany | Rejected But editorial to part 4 was accepted - 4-TC1 (3rd) |
| 181 | Shadowing access controls |  |  |  | Withdrawn |
| 182 | Shadowing and one-level searching | 9/7.2.2.3 and 9.2 |  | IETF | 9-TC1 (3rd) |
| 183 | Public key usage | 8/12.2.2..3 |  | UK | 8-TC1 (3rd) |
| 184 | CertificationPath | 8/8 |  | UK | Rejected Helsinki97 |
| 185 | Forward and reverse certificates | 8/8 |  | UK | 8-TC4 (3rd) |
| 186 | Entry ACI and shadowing | 9/7.2.2 |  | UK | 9-TC1 (3rd) |
| 187 | sdseType of root | 9/7.2.1.1 |  | UK | 9-DTC2(3rd) |
| 188 | Add permission and prescriptive ACI | 3/11.1.5 (3) |  | UK | 3-TC1 (3rd) |
| 189 | Modify operational binding | 2/24.3 |  | UK | 2-TC1 (3rd) |
| 190 | Access controls | 4/19.3.1.2.2 1b |  | UK | 4-TC1 (3rd) |
| 191 | Alias loops | 4/18.3.1 |  | UK | Rejected |
| 192 | Collective attributes and content rules | 2/11.7, 2/12.7.1, 2/13.6 |  | UK | Open |
| 193 | Policy constraints |  |  |  | Rejected |
| 194 | Validity date | ? |  |  | 8-TC1 (3rd) |
| 195 | Shadowing agreement parameters | 9/9.1 |  | UK | Rejected |
| 196 | Validity period |  |  |  | Withdrawn |
| 197 | DSE type bits | 2/19.4.2 4/24.3.1.2 |  | Defect Group | Open |
| 198 | Additions to chaining arguments | 4/17.3.3.1 |  | UK | 4-TC1 (3rd) |
| 199 | Presence Filter | 3/7.8.2 |  | US | Accepted with mod Not in DTC!! |
| 200 | CRL dist pts & full crls | 8/12.6.2 |  | Defect Group | 8-TC3 (3rd) |
| 201 | Issuing distribution point | 8/12.6.3.1 |  | UK | 8-TC3 (3rd) |
| 202 | Clarification of CertificationPath in SecurityParameters | 3/7.10 |  | Defect Group | 3-TC1 (3rd) |
| 203 | Entry Information Selection | 3/7.6 |  | Defect Group | Rejected |
| 204 | Revoked certificates on CRL past expiry time | 8/12.6.3.1 and 8/11.2 |  | Defect Group | 8-TC5 (3rd) |
| 205 | Definition of Superior Reference |  |  | US | 2-TC1 (3rd) |
| 206 | Handling extensions for search results | 3/10.1.3 4/21 |  | EIDQ/FDAS & ISSS/WS DIR | 3-TC1 (3rd) 4-TC1 (3rd) |
| 207 | Problem in the use of the Algorithm object Class | 8/8 & Annex A |  | Rapporteur | 8-DTC6(97) ? |
| 208 | Needed ACI when processing List using knowledge held in superior DSA | 9/7.2.2.3 & 9.2.4.1 |  | IETF IDS | 9-DTC2(3rd) |
| 209 | DSA referrals (duplicate registration 178) |  |  | ITU rapporteur | 4-TC1 (3rd) |
| 210 | Shadowing attribute selection |  |  | Defect Group | Open |
| 211 | Y2K corrections | Parts 2, 3,4, & 6 |  | US | 2-TC2, 3-TC3, 6-TC1 (3rd) |
| 212 | CRL matching rules | 8/12.7.6 |  | US | 8-TC3 (3rd) |
| 213 | CRL matching rules | 8/12.7.6d |  | US | 8-TC3 (3rd) |
| 214 | Use of the term “canonical” | 8/ |  | Rapporteur | 8-DTC6(97) ? |
| 215 | Access control to changing RDN |  |  | Rapporteur (UK) | Open |
| 216 | CertificateAssertion |  |  | Australia | rejected |
| 217 | Use of Operation and Error Code in Security Parameters | 3/7.10 |  | UK | 3-TC1 (3rd) |
| 218 | Certificate Policy Match | 8/12.7.2 |  | UK | 8-TC3 (3rd) |
| 219 | CA certificate and Basic Constraints | 8/ |  | IETF | rejected |
| 220 | CRL version number | 8/ |  | IETF/ISO rapporteur | 8-TC3 (3rd) |
| 221 | Conformance for Certificate Extensions | 5/9 |  | Rapporteur’s meeting | 5-TC1 (97 3rd) |
| 222 | Policy Mapping | 8/12.1 & 12.4.3 |  | US (Santosh and Moses) | 8-TC7 (3rd) |
| 223 | The naming attribute for an entry should always be shadowed. | 9/9.2.2 |  | UK | Open |
| 224 | The evaluation of a filter to UNDEFINED needs to be made consistent for the case where access control is/is not present. | 3/7.8.2 |  | UK | 3-DTC5(3rd) |
| 225 | Entry Information Selection and extraAttributes | 3/7.6 |  | Australia | Open |
| 226 | CA system operational characteristics | 8/11.2 |  | Editor | 8-DTC8(3rd) |
| 227 | Authority Key Identifier format | 8/12.2.2.1 |  | US | 8-DTC8(3rd) |
| 228 | ASN.1 errors in protection feature in X.501 | 2/15.3.2, P |  | Editor | 1-DTC1(3rd), 2-DTC4(3rd), 3-DTC5(3rd), 4-DTC5(3rd), 5-DTC3(3rd), 9-DTC4(3rd) |
| 229 | Wrong references and minor ASN.1 errors in X.501 | 2/17.4.3, 18.1.2 – 3, B, F, P |  | Editor | 2-DTC3(3rd) |
| 230 | The X.501 ASN.1 type Issuer is unknown | 2/18.1.2.1 |  | Editor | 2-DTC3(3rd) |
| 231 | Simple credential ASN.1 error in X.511 | 3/8.1.1, A |  | Editor | 3-DTC3(3rd) |
| 232 | Small ASN.1 editorial errors in X.511 | 3/7.2, 8.11, 9.3, A |  | Editor | 3-DTC3(3rd) |
| 233 | Minor ASN.1 editorials in the import section of X.518 ASN.1 Module | 4/ A |  | Editor | 4-DTC3(3rd) |
| 234 | Wrong limitation on request decomposition | 4/15.3.1 |  | Editor | 4-DTC4(3rd) |
| 235 | Error in X.518 ASN.1 datatype AccessPointInformation | 4/10.8 |  | Editor | 4-DTC3(3rd) |
| 236 | Editorial mistakes in X.519 ASN.1 modules | 5/A, B,C, D, G |  | Editor | 5-DTC2(3rd) |
| 237 | ASN.1 errors in X.520 | 6/5.2.9, 7.6, A |  | Editor | 6-DTC2(3rd) |
| 238 | Wrong reference of string types in X.520 | 6/6.1.1 - 6 |  | Editor | 6-DTC2(3rd) |
| 239 | Missing imports in X.521 ASN.1 module | 7/A |  | Editor | 7-DTC1(3rd) |
| 240 | Miscellaneous errors in X.509 | A |  | Editor | 8-DTC8(3rd) |
| 241 | SerialNumber attribute | 6/5.2.9 |  | Rapporteur | 6-DTC2(3rd) |
| 242 | Size constraint on SET OF and SEQUENCE OF | 8/ |  | Rapporteur | 2-DTC4(3rd), 3-DTC5(3rd), 4-DTC5(3rd), 5-DTC3(3rd), 9-DTC4(3rd) |
| 243 | Miscellaneous errors in X.525 | 9/2.1, 6, 9.2, 11.1-3, A |  | Editor | 9-DTC2(3rd) |
| 244 | Clarification of conformance to criticality | 8/see proposal |  | Sharon | 8-DTC9(3rd), 8-DTC1(4th) |
| 245 | Duplicate Tags | 9/9.2 |  | Erik | 9-DTC3(3rd) |
| 246 | Miscellaneous errors | 6/5.12.2, 5.12.5, 6.8, A, C |  | Erik |  |
| 247 | Miscellaneous errors | 3/Introduction, 12.4 |  | Erik | 3-DTC4(3rd) |
| 248 | ASN.1 error in NHOBSubordinateToSuperior | 4/25.1.4, D |  | Erik | 4-DTC4(3rd) |
| 249 | Miscellaneous errors | 3/3.7.4, 7.3.2, 7.7, 7.8.2, 7.8.3 |  | Erik | 3-DTC1(4th) |
| 250 | Miscellaneous errors | 2/various |  | Erik | 2-DTC1(4th) |
| 251 | AdministrativeLimit | 4/16.1.4.4, 6/5.12.1 |  | Erik | 4-DTC1(4th), 6-DTC1(4th) |
| 252 | ASN.1 errors | 10/A.9 |  | Erik | 10-DTC1(3rd) |
| 253 | Hierarchy selections problems | 4/19.3.3.2.4 (old 19.3.3.2.1. 6/5.12. |  | Erik | 4-DTC1(4th), 6-DTC1(4th) |
| 254 | chainingRequired component misplaced | 4/10.4, 10.8, A |  | Erik | 4-DTC1(4th) |
| 255 | Inconsistency in CONTENT-RULE information object class | 2/12.7.2 |  | Erik | 2-DTC4(3rd) |
| 256 | Populating reverse element | 8/ |  | Sharon | 8-DTC9(3rd), 8-DTC1(4th) |
| 257 | Renaming forward & reverse | 8/ |  | Sharon | 8-DTC9(3rd), 8-DTC1(4th) |
| 258 | Certificate path loops | 8/ |  | Sharon | 8-DTC9(3rd), 8-DTC1(4th) |
| 259 | PartialOutcomeQualifier and ContextCombination errors | 4th 2/13.6.1, 16.10 |  | Erik | 2-DTC1(4th) |
| 260 | Ambiguity in AttributeTypeAndDistinguishedValue | 2/9.3, B |  | Erik | 2-DTC4(3rd) |
| 261 | CommonResults is wrong data dytpe | 2/26.5 |  | Erik | 2-DTC4(3rd) |
| 262 | Signal hierarchy selection not supported | 4th 3/13.3 |  | Erik | 3-DTC1(4th) |
| 263 | Incorrect clause references; test does not match ASN.1 for SimpleCredentials | 3/7.1, 8.12 |  | Erik | 3-DTC5(3rd) |
| 264 | Optionally signal chaining; search constrained by service specific administrative area | 4th 4/16.1.4.2, 19.3.2.2.4 |  | Erik | 4-DTC1(4th) |
| 265 | Various errors | 4/14.5, 15.3.1, 19.3.1.1.3 |  | Erik | 4-DTC5(3rd) |
| 266 | Invalid updates of conformance clause | 5/9 |  | Erik | 5-DTC3(3rd) |
| 267 | Various errors | 2/14.7.3, 14.7.10, 25.2, 22.2.1.2 |  | Erik | 2-DTC4(3rd) |
| 268 | noSubtypeSelection in Entry information selection | 4th 3/7.6 |  | Erik | 3-DTC1(4th) |
| 269 | Error in MatchingRuleDescription dasta type | 2/12.5.2 b), 14.7.3 |  | Erik | 2-DTC4(3rd) |
| 270 | Data types in attribute syntaxes and matching rule assertion syntaxes | 6/5.8.1, 6.1.1, 6.1.10, 6.5.3.1 |  | Erik | 6-DTC3(3rd), 6-DTC1(4th) |
| 271 | Use of term "packet" | 4th 5/9.7 |  | Erik | 5-DTC1(4th) |
| 272 | Certification Path Length | 3rd 8/12.4.2.1 & 4th 8/8.4.2, 15.5.2.1 |  | Sharon | 8-DTC10(3rd), 8-DTC2(4th) |
| 273 | Name constraints conformance | 3rd 12.4.2.2 4th 8.4.2.2 |  | Sharon | 8-DTC10(3rd), 8-DTC2(4th) |
| 274 | Attribute Certificate version | 4th 12.1, A |  | Sharon | 8-DTC2(4th) vote for confirmation of solution already incorporated into published 4th edition |
| 275 | ExtendedKeyUsage | 3rd 8/12.2.2.4 & 4th 8/8.4.2.1, 15.5.2.1 |  | Sharon | 8-DTC10(3rd), 8-DTC2(4th) |
| 276 | Use of anyPolicy in self issued certificates | 4th 8/8.1.5, 8.4.2.4, 10.5 |  | Sharon | 8-DTC2(4th) |
| 277 | Requires explicit policy skip certificates value | 3rd 8/12.4.2.3 & 4th 8/8.4.2.3, 10 |  | Sharon | 8-DTC10(3rd), 8-DTC2(4th) |
| 278 | FreshestCRL extension | 4th 8/8.6.2.6 |  | Sharon | 8-DTC2(4th) |
| 279 | Certification path syntaxes | 4th 8/7, 11.1.6, 11.2.10, 11.3.9 |  | Sharon/Mullan | 8-DTC2(4th) |
| 280 | IDP extension: CA/AA split in CRLs | 4th 8/8.6.2.2 |  | Sharon/Polk/Housley/Cooper | 8-DTC3(4th) |
| 281 | FreshestCRL extension | 4th 8/8.6.2.6 and B.5.1.4 |  | Cooper | 8-DTC34th) |
| 282 | Invalid references to 4th edition of X.519/9594-5 | 4th 8/7, 7.3 & 12.1 |  | Sharon | 8-DTC3(4th) |

Appendix D

Defect Report Form

Please also send a soft copy of the defect in Microsoft Word format to the Defect Editor (hoytkesterson@earthlink.net).

- DEFECT REPORT FORM

1.	Defect Report Number:

Title:

2.	Source:

3.	Addressed to:	ISO/IEC JTC1/SC6 and ITU-T SG 17

Editor Group on the Directory

4.	(a)  ISO/IEC JTC 1/SC 6 Secretariat:	Fax: +82 2 369 8349

Email: secretariat@jtc1sc06.org

(b)  ITU-T Study Group 17 Secretariat:	Fax: +41 22 730 5853

Email: sebek@itu.int or tsbsg17@itu.int

5.	Date Circulated by WG Secretariat:

6.	Deadline for Response from Editor:

7.	Defect Report Concerning:

(number and title of IS or DIS final text/ITU Recommendation)

8.	Qualifier:   (e.g.: error, omission, clarification required)

9.	References in Document:  (e.g.: page, clause/section, figure, and/or table numbers)

10.	Nature of Defect:  (complete, concise explanation of the perceived problem)

11.	Solution Proposed by the Source:  (optional)

12.	Editor's Response:

(any material proposed for processing as an erratum to, an amendment to, or a commentary on the IS or DIS final text/ITU Recommendation or Draft Recommendation is attached separately to this completed report).

Appendix E

Defect Resolution Committee Members

The following representatives have been nominated to the Collaborative Defect Resolution Committee.

- International Defect Report Editor

Hoyt L. Kesterson II	Tel: 	+1 602 316 1985

7625 West Villa Rita Drive	Fax: 	+1 602 978 6750

Glendale, Arizona  85308	Email: hoytkesterson@earthlink.net

USA

- Australia

Rolf Exner		Tel:	+61 3 9253 6718

Telstra Research Laboratories	Fax:	+61 3 9253 6352

770 Blackburn Road	Email: rolf.exner@team.telstra.com

Clayton  Victoria 3168

Australia

- Canada

Sharon Boeyen		Tel:	 +1 613 270 3181

Entrust Technologies	Fax: 	+1 613 270 2503

1000 Innovation Drive	Email: boeyen@entrust.com

Ottawa Ontario  K2K 3E7

Canada

- Denmark

Erik Andersen		Tel:	+45 3947 0736

Fischer & Lorenzo	Fax:	+45 3947 0777

Leopold Damms Alle 3	Email: era.als@get2net.dk

DK-2900 Hellerup

Denmark

- France

Anh Hoang-Van	Tel:	+33 1 45 29 4597

France Telecom	Fax:	+33 1 45 29 6070

38-40, rue du General Leclerc	Email:  anh.hoangvan@francetelecom.com

92131 Issy Les Moulineaux

France

- Germany

Patrick Fantou		Tel:	+49 89 722 53243

Siemens			Fax:	+49 89 722 53249

ICN ISA TNA 4	Email:  patrick.fantou@icn.siemens.de

Otto-Hahn-Ring 6

D-81739 Munich

Germany

- Japan

(to be designated)

- Norway

(to be designated)

- Sweden

(to be designated)

- United Kingdom

(to be designated)

United States of America

John (Skip) Slone	Tel:	+1 407 306 7102

Lockheed Martin	Fax:	+1 407-306-1392

MP 166			Email:  skip.slone@lmco.com

12506 Lake Underhill Road

Orlando, FL 32825

U.S.A.

Appendix F

Register of ASN.1 Modules
Specified External to the Standard

Annex A of ITU-T Rec. X.501|ISO/IEC 9594-2 defines the OIDs of the ASN.1 modules that are specified in the X.500 series of recommendations | parts of 9594. There is a need to define ASN.1 modules that are associated with this standard but are not defined therein.

This appendix of the Implementor’s guide is the definitive register of the module OIDs for those ASN.1 modules. Currently, there are no modules registered.

The root OID of the values in this register is defined in the aforesaid Annex A as

externalDefinitions   ID   : ::= { module externalDefinitions(34) }

OIDS constructed from the entries in this table have the value

{ joint-iso-itu-t ds(5) module(1) externalDefinitions(34) x Version(y) }

where x is the value specified in the following table and y is the revision number of the module being named.

| OID x | Date | Source |
| --- | --- | --- |
| 0 | 5 September 2001 | The module containing the OIDs defined in this register |

ExternalDefinitions {joint-iso-itu-t ds(5) module(1) externalDefinitions(34) 0 version(0) }

DEFINITIONS ::=

BEGIN

-- EXPORTS All –

-- This module specifies the OIDs of ASN.1 modules that are not defined within the Directory Specifications
-- This module is specified in Appendix F of the Directory Implementor’s Guide 
-- A source for the externally defined ASN.1 module may be found in the register in Appendix F of the

-- Directory Implementor’s Guide

IMPORTS

-- from ITU-T Rec. X.501 | ISO/IEC 9594-2

externalDefinitions
		FROM UsefulDefinitions { joint-iso-itu-t ds(5) module(1) usefulDefinitions(0) 4 }

ID				::=	OBJECT IDENTIFIER

exModule ID			::= 	{ externalDefinition }

-- categories of information object --

-- exampleModule ID ::= { exModule x(1) version(0) }

END  -- ExternalDefinitions

_____________________

TSB NOTE: Extract from Technical Corrigendum 3 to Recommendation X.509 (2000) | ISO/IEC 9594-8:2001 (prepublished).