ITU - Telecommunication Standardization Sector	Temporary Document 46 (PLEN)

STUDY GROUP 4

Geneva, 2 – 13 July 2001

Question(s):	12/4

SOURCE*:	Rapporteur Q12/4

TITLE:		M.3100 SERIES - TMN IMPLEMENTORS' GUIDE - DEFECTS AND RESOLUTIONS; VERSION 11 (13/07/2001)

__________________

ABSTRACT

This document is a compilation of reported defects  in the 1995 editions of the ITU-T M.3100 series of Recommendations. It is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendations themselves.  Further, the Guide includes defect resolutions, that will be corrected in the next editions of the relevant Recommendations.

Preface

The purpose of this Guide is to help the implementors of the 1995 ITU-T Recommendations on TMN principles and architecture, management services and generic information models. This Guide is not part of those Recommendations, but may be used in their ongoing maintenance.

The first version of the guide was produced following the Sydney 1995 WP 3/4 (now part of WP 4/4) expert groups meeting.

Items marked with an asterisk (*) were added to the Guide or modified for this Version.

Every change to the Recommendations are identified with bullets having reference numbers of the form Yx where Y is a letter which corresponds to one of the ITU-T Recommendations and x is a number which identifies the particular change in the context of that Recommendation.

At the end of each bullet a defect report number (DR) is included if appropriate. This defect report number is used in Appendix A, which is  a register of defect reports raised and their current status. Note that a single bullet may relate to more than one defect report, or a single defect report may result in more than one bullet being generated.

Wide distribution of this document is expected and encouraged.  The latest version of this Guide will be available on the Word Wide Web server of the ITU (http://www.itu.ch) below the ITU-T SG 4 entry.

This Guide is published in the spirit of international communication and co-operation. However the authors assume no responsibility for the accuracy of the information it contains or for the consequences arising from its use.

TABLE OF CONTENTS

1.	Introduction	4

1.1	Background	4

1.2	Scope of the Guide	4

1.3	Contacts and Distribution of the Guide	4

2.	Defect Report and Resolution Procedures	6

2.1	Submission of defects	6

2.2	Resolution of Defects	6

2.3	Defect report register	6

3.	Implementation Guidance	7

3.1	Changes to Rec M.3100 (1995)	7

3.2	Changes to Rec M.3101 (1995)	7

3.3	Changes to Rec M.3120 (2001)	7

# Introduction

## Background

This Guide concerns the ITU-T M.3100 series of Recommendations on Telecommunications Management Network (TMN).

This Guide is informal in nature and the Guide is not an ITU-T Recommendation. The information it contains will serve as an information source for the ITU-T SG 4, who is responsible for maintaining the M.3100 series of TMN Recommendations, and other users both within and outside of the ITU-T. The changes are expected to be included into future versions of the Recommendations.

## Scope of the Guide

This guide resolves defects in (only) the following categories:

- editorial errors;

- technical errors, such as omissions or inconsistencies;

- ambiguities.

In addition the Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

Note:	This Guide will not address proposed additions, deletions or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories.

Initially, the Guide is limited to defects concerning information models and implementation conformance statements proformas.

## Contacts and Distribution of the Guide

This Guide will be made available at ITU-T SG4 meetings as well as meeting of ITU-T WP4/4. In addition copies of this Guide, can in general, also be made available from one's national representative for ITU-T. Copies may also be obtained from other agencies.

Contacts:

ITU-T WP 3/4 Chairman

| Nobuo FIJII NTT Network Innovation Japan | Tel: +81 468 593 510 Fax: +81 468 551 282 E-mail: nobuo@exa.onlab.ntt.co.jp |
| --- | --- |

Implementors' Guide Editor

Knut Johannessen			Tel. +47 23 25 09 29

Telenor Nett					Fax. +47 23 25 05 06

Pb. 6701 St. Olavs plass		Email:	knut-hakon.johannessen@telenor.com

N-0130 OSLO

NORWAY

ITU-T SG4 Secretariat

Mr. Y.H. Choe							Tel. +41 22 730 59 70

Study Group 4								Fax. +41 22 730 58 53

Telecommunications Standardization		Email:tsbsg4 @itu.int
Bureau

International Telecommunications Union

Place des Nations

CH-1211 Geneva 20

Switzerland

Defect group leaders

This list includes the names of leaders for Recommendations. Not all of the Recommendations may have defects raised against them and leaders are not identified for all Recommendations. However, whenever a defect is raised, a defect group leader will be identified for the relevant Recommendation.

| Recommendation | Defect group leader |  |
| --- | --- | --- |
| Rec. M.3100 Generic Network Information Model (including amendments) | Knut Johannessen Telenor Nett Pb. 6701 St. Olavs plass N-0130 OSLO NORWAY | Tel. +47 23 25 09 29 Fax. +47 23 25 05 06 Email: knut-hakon.johannessen@telenor.com |
| Rec. M.3101 Conformance Statement Proformas for Rec. M.3100 Generic Network Information Model | Knut Johannessen Telenor Nett Pb. 6701 St. Olavs plass N-0130 OSLO NORWAY | Tel. +47 23 25 09 29 Fax. +47 23 25 05 06 Email: knut-hakon.johannessen@telenor.com |
| Rec. M.3120 CORBA Generic Network And Network Element Level Information Model | Kam Lam Lucent Technologies United States | Tel: +1 732 615-4118 Fax: +1 732 615-4610 E-mail: hklam@lucent.com |

1.4	History log

| Item | Location | Reason |
| --- | --- | --- |
| 1.0 | Geneva, January 1996 | First version of the Guide |
| 2.0 | Tokyo, April 1996 | Second version of the Guide |
| 3.0 | Edinburgh, July 1996 | Third version of the Guide |
| 4.0 | Geneva, December 1996 | Correction of editorial errors (e.g. missing semicolons) Addition of new defect resolutions including modifications to existing entries. |
| 4.0a | Oslo, January 1997 | Correction of editorial errors (e.g. syntax errors) |
| 4.0b | Oslo, May 1997 | Correction of editorial errors (e.g. syntax errors) |
| 4.1 | Beijing, June 1997 | Correction of editorial errors (e.g. missing semicolons) Addition of new defect resolutions including modifications to existing entries. |
| 5.0 | Geneva, October 1997 | SG approval Note - All approved corrections to Rec. M.3100 is published in a technical corrigendum. |
| 6.0 | Geneva, June 1998 | All resolved defects are documented in M.3100 Technical Corrigendum 1. The resolutions are removed from the implementors guide (this document). |
| 7.0 | Geneva, February 2000 | Addition of user guidelines, correction to registration etc. |
| 8.0 | Torrance, August 2000 | Additional defects discovered as part of work on CORBA/IDL (M.3120) as well as defects moved from the M.3100 status document |
| 9.0 | Geneva, January 2001 | No additional defect reports |
| 10.0 | Pleasanton, May 2001 | Incorrect ArcAlarmDetailSet syntax in Amendment 3 Defect report on M.3120 on missing string identifiers for conditional packages. |
| 11.0 | Geneva, July 2001 | Incorrect reference to TC-3 replaced with TC-2 for defect resolutions DR-M3100-31 to DR-M3100-42. Combined into a single table for all M.3100 defects (including amendments). |

# Defect Report and Resolution Procedures

## Submission of defects

Any implementor of the TMN Recommendations is invited to submit TMN defect report using the form in Appendix B of the Guide. The defect should be submitted to the ITU-T SG4 Secretariat and copied to the ITU-T WP4/4 Chairman. Each form should cover a single defect. It is important that the form is completed accurately, especially the sections which relate to the base material against which the defect report is being raised.

## Resolution of Defects

A TMN defect resolution group is established for each of the Recommendations. In some cases a group covers more than one Recommendation. Following agreement on a resolution, within the defect resolution group, the proposed resolution may require approval of ITU-T WP4/4 and ITU-T SG4.

This Guide will contain resolutions as they are agreed by the defect resolution group. The status of each will be reflected in Appendix A of the Guide and any modifications required to the resolutions themselves prior to final approval, will be reflected in Section 3 of the Guide.

Please note that individual responses can not be given to an individual submitting defect reports, and that the procedure is not intended as a consulting service.

## Defect report register

New defect reports will be included in the report of the relevant Question or Working Party.

# Implementation Guidance

Remember that this Guide is intended to be an authoritative source of information for implementors of the TMN Recommendations, however it is not itself an ITU-T Recommendation.

Items marked with an asterisk (*) were added to the Guide or modified for this version of the Guide.

Bullets have reference numbers of the form Yx where Y is a letter which corresponds to one of the ITU-T Recommendations, and x is a number which identified the bullet in the context of that Recommendation.

At the end of each bullet title a defect report number (DR) is included if appropriate. This defect report number is used in Annex A, which is  a register of defect reports raised and their current status. Note that a single bullet may relate to more than one defect report, or a single defect report may result in more than one bullet being generated.  Text contained in this clause shall only be considered final when the associated changes are part of a revised Recommendation.

## Changes to Rec M.3100 (1995)

See

- Rec. M.3100 Technical corrigendum 1

- Rec. M.3100 Technical corrigendum 2

for published corrections.

A1	DR-M3100-43 	(*)

In section 4.1.6,	GDMO/ASN.1 (amendment 3)

Replace

ArcAlarmDetailSet ::= SEQUENCE {

ArcAlarmDetail

}

with

ArcAlarmDetailSet ::= SET OF ArcAlarmDetail

## Changes to Rec M.3101 (1995)

None

## Changes to Rec M.3120 (2001)

B1	DR-M3120-1

Add new section section 6.8 “Conditional Package Identifiers” after old section 6.7 as follows:

/**

6.8 Conditional Package Identifiers

This collection of constants represent the conditional packages defined in this recommendation. When a conditional package is supported in a particular managed object instance, one of these strings will be contained in the packages attribute

*/

module itut_m3120 {

const string administrativeOperationalStatesPackage =

"itut_m3120::administrativeOperationalStatesPackage";

const string administrativeStatePackage =

"itut_m3120::administrativeStatePackage";

const string affectedObjectListPackage =

"itut_m3120::affectedObjectListPackage";

const string alarmSeverityAssignmentPointerPackage =

"itut_m3120::alarmSeverityAssignmentPointerPackage";

const string arcPackage = "itut_m3120::arcPackage";

const string arcRetrieveAlarmDetailPackage =

"itut_m3120::arcRetrieveAlarmDetailPackage";

const string attributeValueChangeNotificationPackage =

"itut_m3120::attributeValueChangeNotificationPackage";

const string audibleVisualLocalAlarmPackage =

"itut_m3120::audibleVisualLocalAlarmPackage";

const string availabilityStatusPackage =

"itut_m3120::availabilityStatusPackage";

const string basicConnectionPerformerPackage =

"itut_m3120::basicConnectionPerformerPackage";

const string channelNumberPackage = "itut_m3120::channelNumberPackage";

const string characteristicInformationPackage =

"itut_m3120::characteristicInformationPackage";

const string circuitPackConfigurationPackage =

"itut_m3120::circuitPackConfigurationPackage";

const string circuitPackResetPackage =

"itut_m3120::circuitPackResetPackage";

const string clientConnectionListPackage =

"itut_m3120::clientConnectionListPackage";

const string clientCTPListPackage = "itut_m3120::clientCTPListPackage";

const string clientLinkConnectionPointerListPackage =

"itut_m3120::clientLinkConnectionPointerListPackage";

const string clientLinkEndPointerPackage =

"itut_m3120::clientLinkEndPointerPackage";

const string clientLinkPointerPackage =

"itut_m3120::clientLinkPointerPackage";

const string clientTrailPackage = "itut_m3120::clientTrailPackage";

const string componentPointerPackage =

"itut_m3120::componentPointerPackage";

const string compositePointerPackage =

"itut_m3120::compositePointerPackage";

const string configuredConnectivityPackage =

"itut_m3120::configuredConnectivityPackage";

const string connectivityPointerPackage =

"itut_m3120::connectivityPointerPackage";

const string containedAccessGroupListPackage =

"itut_m3120::containedAccessGroupListPackage";

const string containedBoardPackage =

"itut_m3120::containedBoardPackage";

const string containedInSubnetworkListPackage =

"itut_m3120::containedInSubnetworkListPackage";

const string containedLinkEndListPackage =

"itut_m3120::containedLinkEndListPackage";

const string containedLinkListPackage =

"itut_m3120::containedLinkListPackage";

const string containedNetworkTPListPackage =

"itut_m3120::containedNetworkTPListPackage";

const string containedSubnetworkListPackage =

"itut_m3120::containedSubnetworkListPackage";

const string createDeleteNotificationsPackage =

"itut_m3120::createDeleteNotificationsPackage";

const string crossConnectionPointerPackage =

"itut_m3120::crossConnectionPointerPackage";

const string currentProblemListPackage =

"itut_m3120::currentProblemListPackage";

const string environmentalAlarmR2Package =

"itut_m3120::environmentalAlarmR2Package";

const string equipmentsEquipmentAlarmR2Package =

"itut_m3120::equipmentsEquipmentAlarmR2Package";

const string externalTimePackage = "itut_m3120::externalTimePackage";

const string layerConnectionListPackage =

"itut_m3120::layerConnectionListPackage";

const string linkConnectionPointerListPackage =

"itut_m3120::linkConnectionPointerListPackage";

const string linkPointerListPackage =

"itut_m3120::linkPointerListPackage";

const string locationNamePackage = "itut_m3120::locationNamePackage";

const string logicalLinkEndHandlerPackage =

"itut_m3120::logicalLinkEndHandlerPackage";

const string logicalLinkHandlerPackage =

"itut_m3120::logicalLinkHandlerPackage";

const string maximumLinkConnectionCountPackage =

"itut_m3120::maximumLinkConnectionCountPackage";

const string maximumNetworkCTPCountPackage =

"itut_m3120::maximumNetworkCTPCountPackage";

const string namedCrossConnectionPackage =

"itut_m3120::namedCrossConnectionPackage";

const string neAssignmentPointerPackage =

"itut_m3120::neAssignmentPointerPackage";

const string networkCTPPackage = "itut_m3120::networkCTPPackage";

const string networkCTPsInLinkEndListPackage =

"itut_m3120::networkCTPsInLinkEndListPackage";

const string networkLevelPackage = "itut_m3120::networkLevelPackage";

const string networkTPPointerPackage =

"itut_m3120::networkTPPointerPackage";

const string normalControlStatePackage =

"itut_m3120::normalControlStatePackage";

const string numberOfPortPackage = "itut_m3120::numberOfPortPackage";

const string objectManagementNotificatoinsPackage =

"itut_m3120::objectManagementNotificatoinsPackage";

const string operationalStatePackage =

"itut_m3120::operationalStatePackage";

const string portAssociationsPackage =

"itut_m3120::portAssociationsPackage";

const string potentialLinkCapacityPackage =

"itut_m3120::potentialLinkCapacityPackage";

const string potentialLinkEndCapacityPackage =

"itut_m3120::potentialLinkEndCapacityPackage";

const string processingErrorAlarmR2Package =

"itut_m3120::processingErrorAlarmR2Package";

const string protectedPackage = "itut_m3120::protectedPackage";

const string provisionedLinkCapacityPackage =

"itut_m3120::provisionedLinkCapacityPackage";

const string provisionedLinkConnectionCountPackage =

"itut_m3120::provisionedLinkConnectionCountPackage";

const string provisionedLinkEndCapacityPackage =

"itut_m3120::provisionedLinkEndCapacityPackage";

const string provisionedNetworkCTPCountPackage =

"itut_m3120::provisionedNetworkCTPCountPackage";

const string quailityOfConnectivityServicePackage =

"itut_m3120::quailityOfConnectivityServicePackage";

const string redlinePackage = "itut_m3120::redlinePackage";

const string relatedRoutingProfilePackage =

"itut_m3120::relatedRoutingProfilePackage";

const string serverConnectionListPackage =

"itut_m3120::serverConnectionListPackage";

const string serverTrailListPackage =

"itut_m3120::serverTrailListPackage";

const string serverTTPPointerPackage =

"itut_m3120::serverTTPPointerPackage";

const string sncPointerPackage = "itut_m3120::sncPointerPackage";

const string softwareProcessingErrorAlarmR2Package =

"itut_m3120::softwareProcessingErrorAlarmR2Package";

const string stateChangeNotificationPackage =

"itut_m3120::stateChangeNotificationPackage";

const string subordinateCircuitPackPackage =

"itut_m3120::subordinateCircuitPackPackage";

const string supportableClientListPackage =

"itut_m3120::supportableClientListPackage";

const string supportedByPackage = "itut_m3120::supportedByPackage";

const string systemTimingSourcePackage =

"itut_m3120::systemTimingSourcePackage";

const string tmnCommunicationsAlarmInformationR1Package =

"itut_m3120::tmnCommunicationsAlarmInformationR1Package";

const string topologicalLinkEndHandlerPackage =

"itut_m3120::topologicalLinkEndHandlerPackage";

const string topologicalLinkHandlerPackage =

"itut_m3120::topologicalLinkHandlerPackage";

const string totalLinkCapacityPackage =

"itut_m3120::totalLinkCapacityPackage";

const string totalLinkEndCapacityPackage =

"itut_m3120::totalLinkEndCapacityPackage";

const string trafficDescriptorPackage =

"itut_m3120::trafficDescriptorPackage";

const string usageCostPackage = "itut_m3120::usageCostPackage";

const string usageStatePackage = "itut_m3120::usageStatePackage";

const string userLabelPackage = "itut_m3120::userLabelPackage";

const string vendorNamePackage = "itut_m3120::vendorNamePackage";

const string versionPackage = "itut_m3120::versionPackage";

}

B2	DR-M3120-2

In Section 6, Information Model IDL, the pragma prefix directive is located before included definitions. This is incorrect, and the pragma prefix shall be moved below the include directives.

B3	DR-M3120-3

Object references based on IOR cannot be assumed to be correct or valid over time. Hence, object references using IOR are replaced with object names. This approach also allow for support of the facade approach using the same based model.

The combined resulution of this defect will result in a revision of M.3120. The excerpt below is an indication of the resulting changes.

typedef itut_x780::MONameSeqType MONameSeqType;

typedef itut_x780::MONameSetType MONameSetType;

/**

Typedefs forward declarations

*/

typedef MONameType AbstractLinkNameType;

typedef MONameType AbstractLinkEndNameType;

typedef MONameType AccessGroupNameType;

typedef MONameType AlarmSeverityAssignmentProfileNameType;

typedef MONameType ArcIntervalProfileNameType;

typedef MONameType BasicLayerNDNameType;

typedef MONameType BasicSubnetworkNameType;

typedef MONameType CircuitEndPointSubgroupNameType;

typedef MONameType CircuitPackNameType;

typedef MONameType ControlPointNameType;

typedef MONameType CTPAbstractNameType;

typedef MONameType CTPBidNameType;

typedef MONameType CTPSinkNameType;

typedef MONameType CTPSourceNameType;

typedef MONameType CrossConnectionNameType;

typedef MONameType EquipmentNameType;

typedef MONameType EquipmentHolderNameType;

typedef MONameType ExternalPointNameType;

typedef MONameType FabricNameType;

typedef MONameType GTPNameType;

typedef MONameType LayerNDNameType;

typedef MONameType LinkConnectionNameType;

typedef MONameType LogicalLinkNameType;

typedef MONameType LogicalLinkEndNameType;

typedef MONameType ManagedElementNameType;

typedef MONameType ManagedElementComplexNameType;

typedef MONameType MPCrossConnectionNameType;

typedef MONameType NetworkNameType;

typedef MONameType NetworkCTPAbstractNameType;

typedef MONameType NetworkCTPBidNameType;

typedef MONameType NetworkCTPSinkNameType;

typedef MONameType NetworkCTPSourceNameType;

typedef MONameType NetworkTPNameType;

typedef MONameType NetworkTTPAbstractNameType;

typedef MONameType NetworkTTPBidNameType;

typedef MONameType NetworkTTPSinkNameType;

typedef MONameType NetworkTTPSourceNameType;

typedef MONameType PipeNameType;

typedef MONameType ScanPointNameType;

typedef MONameType SoftwareNameType;

typedef MONameType SubnetworkNameType;

typedef MONameType SNCNameType;

typedef MONameType TopLinkNameType;

typedef MONameType TopLinkEndNameType;

typedef MONameType TPNameType;

typedef MONameType TPPoolNameType;

typedef MONameType TrailNameType;

typedef MONameType Trail95NameType;

typedef MONameType TTPAbstractNameType;

typedef MONameType TTPBidNameType;

typedef MONameType TTPSinkNameType;

typedef MONameType TTPSourceNameType;

Also, replace construct such as

typedef sequence<AbstractLink> AbstractLinkSetType;

with

typedef sequence<AbstractLinkNameType> AbstractLinkNameSetType;

TMN Defect Report Register

The defects reported to date are listed below. The status of each is indicated according to the classification outlined below:

O	Open

-	The defect has been submitted, a solution may have been proposed, but the Defect Resolution Group has not yet come to an agreement.

A/U	Agreed/Unanimous

- 	Proposed solution agreed by everyone in the Defect Resolution Group

- 	Pending approval by affected Study Group

A/C	Agreed/Consensus

-	Consensus solution agreed by the Defect Resolution Group and documented in this version of the Implementors’ Guide.

-	Pending approval by affected Study Group

A	Agreed

-	Proposed solution agreed by the Defect Resolution Group

-	Pending approval by affected Study Group

Note: This status value is no longer to be used and has been replaced with the A/U and A/C status 	values.

C	Complete

-	Defect resolution approval by full Study Group

-	Final resolution reflected in this version of Implementor's Guide

P	Published

-	Change included in published version

R	Rejected

-	As a defect (may be misinterpretation, request for extension or have already been corrected in subsequent version of text)

W	Withdrawn

-	Defect report withdrawn by source.

The severity of each is indicated according to the classification outlined below:

m	Minor

The following defects are classified as minor:

-	Pure syntax errors in GDMO templates (e.g. missing semicolons)

-	Obvious misalignment in names of attributes in the GDMO and ASN.1

-	Missing ASN.1 productions where the text is clear as to the syntax, semantics and data type

-	Clarifications of ambiguities in the text if supported by the formal definitions

-	Typographical errors.

M	Major

-	All other defects are classified as major.

| ITU-T Recommendation M.3100 (1995) - including amendments |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Status | Severity | Defect Number | Source | Guide Entry | Other reference | Subject Matter |
| R | M | DR-M3100-1 | Deutsche Telecom | None | SYD-46, M.3100/§3.5.5 | Missing ToTermination in Multi-point Cross-Connection. In the case of a multi-point cross-connection, the to-termination points are not identified directly by the Multi-point Cross-Connection object instance, but by the Cross-Connection object instances that are contained by the Multi-point Cross-Connection instance. |
| P | M | DR-M3100-2 | Deutsche Telecom | TC 1 | SYD-46, M.3100/Fig I.6 | Information flow in Figure I.6 should be from left to right and the sink and source labels should be corrected |
| P | M | DR-M3100-3 | Australia | TC 1 | SYD WD-M7(R1) TD-6 (4/4) | Modify the subordinateCircuitPackSoftwareLoad attribute behaviour to add clarification |
| P | M | DR-M3100-4 | Australia | TC 1 | SYD WD-M7 TD-6 (4/4) | Add DELETE to (new version of) autocreated circuitPack name binding. |
| P | m | DR-M3100-5 | Australia | TC 1 | SYD WD-M7 | Additional user guidelines on use of supportedByObjectList |
| P | m | DR-M3100-6 | Japan | TC 1 | Geneve WD-M11/12 | Multiple use of package label in OID assignments |
| P | M | DR-M3100-7 | Bellcore | TC 1 | TOK-42 | Addition of Error Parameter to (some) name bindings |
| P | M | DR-M3100-8 | Bellcore | TC 1 | TOK-41 | Missing resource type attribute in equipmentR1 object class |
| P | M | DR-M3100-9 | Bellcore | TC 1 | Tokyo WD-MM | Missing ProblemCause value |
| P | M | DR-M3100-10 | ETSI | TC 1 | TOK -58 | Missing probable cause values (derived from work on GSM) NOTE: The applicable generic or otherwise useful set of new probable cause values has been added to M.3100. Too generic (e.g. equipmentFailure) and too specific (or implementation oriented) probable cause values are not considered useful and are not included. |
| P | M | DR-M3100-11 | Nokia | TC 1 | EDI-11 | Missing probable cause values for real time clock |
| P | m | DR-M3100-12 | Nokia | TC 1 | EDI-51 | Duplicated behaviour definition in name binding. |
| W | M | DR-M3100-13 | Nokia | TC 1 | EDI-52 | Use of crossConnectionPackage |
| P | m | DR-M3100-14 | Nokia | TC 1 | EDI-53 | Typographical and editorial errors. |
| P | M | DR-M3100-15 | Nokia | TC 1 | EDI-54 | Revised crossConnection |
| O | M | DR-M3100-16 | Fujitsu |  | EDI-29 | Identification of physical connectivity in circuitPack NOTE: Also addressed by DR-M3100-20 |
| P | M | DR-M3100-17 | Q.15/4 | TC 1 | WD-x | Missing namebinding from managedElementComplex |
| P | m | DR-M3100-18 | Italy (ETSI) | TC 1 | D-012 | Possible values of equipmentHolderType |
| P | M | DR-M3100-19 | Italy (ETSI) | TC 1 | D-012 | Need of possibleCircuitPackList attribute |
| O | M | DR-M3100-20 | Italy (ETSI) |  | D-012 | Modelling of cables, connectors and backplane NOTE: Modelling of cables and connectors are also addressed by DR-M3100-16 |
| P | M | DR-M3100-21 | Q.15/4 | TC 1 | WD-x | Missing name bindings from managedElement (and subclasses) to managed objects named from system in X.700 series |
| P | M | DR-M3100-22 | Nokia | TC 1 | D-022 | Assignment of probable cause values related to radio relay management NOTE: A subset of the proposed values has been included in version 4.0 of the implementors’ guide. |
| P | M | DR-M3100-23 | USA | TC 1 | D-008 | Enhancement of the generalErrorParameter with addition of new cause codes. |
| P | m | DR-M3100-24 | WP 4/4 | TC 1 | TD-71 | Several editors errors are identified. NOTE: Some issues have been addressed in previous defect reports. |
| P | m | DR-M3100-25 | ETSI | TC 1 | WD-5 | Definition of object identifier of e0-e4 levels. |
| P | M | DR-M3100-26 | Lucent | TC 1 | Beijing-30 | The defect report proposes clarification of delete behaviour of circuitPack as follows: “Circuit packs are deleted only by the manager. It is not automatically deleted when removed”. |
| P | M | DR-M3100-27 | ETSI | TC 1 | Beijing-24 | Modification to behaviour of equipmentHolder-equipmentHolder name binding to allow an equipmentHolder to contain both equipmentHolder and circuitPack objects. |
| P | M | DR-M3100-28 | USA | TC 1 | Beijing-45 | Addition of missing notifications to fabricR1and gtp managed object class. NOTE: No object identifier values has been assigned in this document as further changes to these object classes is expected before publication of the next version of M.3100. |
| P | m | DR-M3100-29 | China | TC 1 | TD-38 (4/4) | Missing name binding of managedElementComplex to network. |
| C | m | DR-M3100-30 | USA | TC 2 | D.185 (GEN) | Clarification on non-alarmed behaviour |
| C | M | DR-M3100-31 | Rapporteur | TC 2 |  | Duplication of registration OID. |
| C | M | DR-M3100-32 | Joint experts group on CORBA/IDL | TC 2 | WD 15 (Torrance) | The abstractLink (M.3100 Amd-1) object has the usageCost attribute that is get only. It was suggested to change this to GET-REPLACE. |
| C | m | DR-M3100-33 | Joint experts group on CORBA/IDL | TC 2 | WD 15 (Torrance) | In the networkCTPSource object, the last paragraph of the behaviour should be “… the Network Connection Termination Point Sink or Bi-directional, that receives information from…” |
| C | m | DR-M3100-34 | Joint experts group on CORBA/IDL | TC 2 | WD 15 (Torrance) | The node was not intended to be included in M.3100 Amendment 1 (was taken out of input draft document) and should be removed. |
| C | m | DR-M3100-35 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.1 | In spite that linkEnd of G.853.1(CIV) has signalIdentification, abstractLinkEnd of M.3100 does not have signalId. |
| C | m | DR-M3100-36 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.2 | clientLinkEndPointerPackage is defined in networkTTPSource. However, it is missing in networkTTPSink. |
| C | m | DR-M3100-37 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.3 | According to present M.3100 Amd1, networkCTPPackage is defined based on subnetworkTPPoolIsMadeOfSubnetworkTP of G.853.1(CIV). However, as this package is addressing partitioning, subnetworTPIsRelatedToExtremity may be the corresponding relationship definition. |
| C | m | DR-M3100-38 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.4 | In the componentPointerPackage of subNetworkConnection the last paragraph of the behavior clause should be ”The componentListPointerPackage is supported where the Sub-network Connection is made up of a number of component Sub-network Connections, and Link Connections, within the same layer”. |
| C | m | DR-M3100-39 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.5 | Figure x.3/M.3100 (Entity Relationships) should be updated to be aligned with the text. |
| C | m | DR-M3100-40 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.6 | The behaviour definition of the signal identification attribute in M.3100 Am. 1 is inconsistent with the syntax definition for SignalId. |
| C | m | DR-M3100-41 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§4.5.7 | In the name binding accessGroup-layerNetworkDomain, correct the wording in the DELETE statement to be consistent with the behaviour statement. |
| C | m | DR-M3100-42 | Joint experts group on CORBA/IDL | TC 2 | TD 42/4,§5.2.3 | Topology Objects Creation Behaviour with Respect to Signal Identification |
| C | M | DR-M3100-43 | Experts group | TC 3 |  | Incorrect ASN.1 syntax ArcAlarmDetailSet |

ITU-T Recommendation M.3101 (1995)

| Status | Severity | Defect Number | Source | Guide Entry | Other reference | Subject Matter |
| --- | --- | --- | --- | --- | --- | --- |

| ITU-T Recommendation M.3120 (01/01) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Status | Severity | Defect Number | Source | Guide Entry | Other reference | Subject Matter |
| C | m | DR-M3120-1 | Experts group | B1 | EGPL05, EGPL06 | Missing string identifiers for conditional packages |
| C | m | DR-M3120-2 | Experts group | B2 |  | Incorrect placement of pragma prefix |
| C | m | DR-M3120-3 | Experts group | B3 |  | Object reference persistence |

TMN Defect Report Form

| DEFECT REPORT FORM |  |  |
| --- | --- | --- |
|  | Defect Report Number | Recommendation code/numeric Note: Only the recommendation is identified by the defect report submitter |
|  | Source | country, member etc Note: Filled out by the defect report submitter |
|  | Addressed to | Defect editors group reference Note: Filled out by the defect report submitter |
|  | - ITU-T TSB - ITU-T WP | administrative body reference Note: Not filled out by the defect report submitter |
|  | Date circulated by administrative body | date Note: Filled out by the TSB or WP |
|  | Deadline for response from editor | date Note: Determined by the Defect Resolution Group |
|  | Defect Report Concerning | Recommendation number and publication date Note: Filled out by the defect report submitter |
|  | Qualifier | e.g. error, omission, clarification required Note: Filled out by the defect report submitter |
|  | Reference in document | clause number Note: Filled out by the defect report submitter |
|  | Nature of defect | complete, concise explanation of the perceived problem Note: Filled out by the defect report submitter |
|  | Solution proposed by source | optional |
|  | Editors response | any material proposed for processing as an erratum to, an amendment to or a commentary on the final Recommendation text. This will be included in Chapter 3 of a later version of this document. |

_________________

Defect resolutions are published in M.3100 Technical Corrigendum 1 (TC-1).

Defect resolutions are published in M.3100 Technical Corrigendum 2 (TC-2).

Defect resolutions are published in M.3100 Technical Corrigendum 3 (TC-3).