|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.126 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Implementor’s Guide (January 2005) |
|  | SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS Interactive systems for digital television distribution |  |  |  |

Implementor’s Guide for
ITU-T Recommendation J.126

Embedded Cable Modem device specification

Implementor’s Guide

During the Study Group 9 meeting (Geneva, 17-21 January 2005) it was agreed to publish the present implementor’s guide in view of correcting editorial faults in the current Recommendation J.126.

(Based on Temporary Document 92, this document is a follow-up to the Contribution COM 9-D010 presented during the meeting by the Rapporteur for Question 18/9*)

Summary

This Guide contains a series of engineering changes to update J.126 to match what is currently being built in the industry.  These changes were proposed by implementers and CableLabs, and were approved to apply to CableLabs' eDOCSIS specification (I04) by the CableLabs' engineering change process.

Summary of Proposed Changes to J.126

| CableLabs' Engineering Change # | Location in this document | Date Accepted | J.126 - Clauses Affected | Company | Summary Description |
| --- | --- | --- | --- | --- | --- |
| Technical Changes |  |  |  |  |  |
| N-04.0122.2 | Annex A | 2/11/04 | 5.2.5.2.1, 5.2.5.2.2, | CableLabs | EDOCSIS (SLED) CLARIFICATIONS |
| N-04.0122-2 | Annex B | 10/29/03 | Annex A of J.126 | CableLabs, Broadcom Corp. | To incorporate Software Loopback for eDOCSIS (SLED) into eDOCSIS devices that do not have an exposed CMCI and/or have multiple eSAFEs |
| N-04.0150-2 | Annex C | 6/30/04 | 5.2.2 | YAS Broadband Ventures for Comcast | eDOCSIS clarification on the effect of NACO state on eSAFE traffic |
| N-04.0152-5 N-04.0161-1 N-04.0172-6 N-04.0176-2 | Annex D | 7/7/04 7/28/04 10/6/04 10/27/04 | New Annex B of J.126 | CableLabs C-COR Inc. | Insert new Annex B - eSAFE MIB definition for eDOCSIS, including enhancements specifically related to safe reboots |
| N-04.0154-2 | Annex E | 6/30/04 | 2.2, 3, 5.2.6, | CableLabs | Modify eDOCSIS firmware download requirements to account for eSTB |
| N-04.0158-2 | Annex F | 7/7/04 | 5.2.3.3, | YAS Broadband Ventures for Comcast | Additional RFC 1493 Requirements for eDOCSIS devices |
| N-04.0182-2 | Annex G | 10/27/04 | 5.2.2, 5.2.3, | CableLabs | Clarification of MaxCPE count for DSG tunnel MAC addresses |
| N-04.0123-2 N-04.0134-2 | Annex H | 2/11/04 2/18/04 | 5.2.4.1, 5.2.4.3, 5.2.4.4 | Broadcom Corp Cisco | eDOCSIS DHCP Option 43 clarifications |
| Editorial Changes |  |  |  |  |  |
| N-04.0160-1 | Appendix I | 7/28/04 | 1.2, 3, 4, 5.2.3.2, | CableLabs | Miscellaneous editorial changes |
| N-04.0138-1 | Appendix II | 4/7/04 | 5.2.4.3 | CableLabs | Reserved DHCP 43 Suboptions for eSafes |

Attachment: ITU-T Recommendation J.126 – Implementor’s Guide

| *Contact: | Greg White CableLabs United States of America | Tel:+1 303 661 3822 Email:gwhite@cablelabs.com |
| --- | --- | --- |
| All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU. |  |  |

Annex A

Proposed Changes

Summary: eDOCSIS (SLED) Clarifications

Change #1

NEW TEXT: Section 5.2.5.2.1  (New paragraph to be added after the third paragraph)

The SLED MIB values revert to power-on values when the CM de-registers or loses Operational state; the sledGlobalEnable will revert to ‘false’ and in-progress packet generation or loopback will be stopped.

Change #2

ORIGINAL TEXT: Section 5.2.5.2.2  (add footnote after step #5)

5. When operating in loopback mode, all Ethernet packets forwarded across the indicated LCI by the eCM will be processed as follows:

NEW TEXT: Section 5.2.5.2.2

5. When operating in loopback mode, all Ethernet packets forwarded across the indicated LCI by the eCM will be processed as follows:1

1 Note: The eCM MUST remove the Ethernet FCS/CRC32 before encapsulating and processing the packet for loopback.  Because the CRC32 may not be present,  may be incorrect, and is not relevant to the SLED loopback tests, it is omitted from the packet that is looped back.

Change #3

ORIGINAL: Section 5.2.5.2.2 - Figure 5-7

NEW Figure:

Annex B

Proposed Changes

Summary:  Minor corrections to Annex a - SLED MIB Definition

Revise Text as shown below (only changes indicated; red strikethrough is deleted text, blue double underline is inserted new text):

Annex A

SLED MIB Definition

SLED-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

Integer32,

OBJECT-TYPE		        FROM SNMPv2-SMI

TruthValue,

TimeStamp			          FROM SNMPv2-TC

OBJECT-GROUP,

MODULE-COMPLIANCE  	FROM SNMPv2-CONF

clabProjDocsis        	FROM CLAB-DEF-MIB

InterfaceIndex        	FROM IF-MIB

;

sledMib MODULE-IDENTITY

LAST-UPDATED    "200312300000Z" -- December 30, 2003       "200411240000Z" -- November 24, 2004

ORGANIZATION       "CableLabs eDOCSIS Working Group"

CONTACT-INFO

"John Eng

Postal:    	Cable Television Laboratories, Inc

400 Centennial Parkway

858 Coal Creek Circle

Louisville, CO 80027

U.S.A.

Phone:  +1 303-661-9100

Fax:    +1 303-661-9199

E-mail: mibs@cablelabs.com"

DESCRIPTION

"This MIB module provides the management objects necessary

to configure and invoke the Software Loopback Application

for eDOCSIS (SLED) functionality."

REVISION        "200312300000Z" -- December 30, 2003 "200411240000Z" -- November 24, 2004

DESCRIPTION

"InitialSecond version of the ITU-T eDOCSIS SLED MIB module

Acknowledgement:

Greg Nakanishi of Motorola, Inc."

::=  { clabProjDocsis 1317 }

-- Administrative assignments

sledNotifications             OBJECT IDENTIFIER ::= { sledMib 0 }

sledMibObjects                OBJECT IDENTIFIER ::= { sledMib 1 }

sledMibNotificationsObjects   OBJECT IDENTIFIER ::= { sledMib 2 }

sledMibConformance            OBJECT IDENTIFIER ::= { sledMib 3 }

-- Object Groups

sledGlobal		        OBJECT IDENTIFIER ::= { sledMibObjects 1 }

sledLoopback		      OBJECT IDENTIFIER ::= { sledMibObjects 2 }

sledPktGen		        OBJECT IDENTIFIER ::= { sledMibObjects 3 }

--

-- The following group describes the objects that apply to

-- both loopback and packet generator SLED functionality

--

sledGlobalEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object allows the SLED functionality to be

enabled/disabled.

This object may only be updated prior to

device registration.  If	   the device has completed

registration, any attempt to update	   the value of this

object returns 'notWritable'.	   Prior to registration, if the

value of this object is set to        'true', the SLED

functionality is enabled and access to this MIB is        allowed.

Prior to registration, if the value of this object is set

to        'false', the SLED functionality is disabled and any

attempt to         update other objects in this MIB returns

'noAccess'.	  	  "

DEFVAL { false }

::= { sledGlobal 1 }

--

-- The following group describes the loopback objects

--

sledLoopbackInterface OBJECT-TYPE

SYNTAX		      InterfaceIndex

MAX-ACCESS	  read-write

STATUS		      current

DESCRIPTION

"The index of the logical CPE interface (LCI) that the SLED

loopback function is attached to.  If the index does not

correspond to a LCI supported by this device, 'wrongValue'

is returned.

Any attempt to set this object while sledLoopbackEnable is

set to 'true' returns 'notWritable'."

::= { sledLoopback 1 }

sledLoopbackEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Setting this object to 'true' enables the loopback

function. Setting this object to 'false' disables the

loopback function. When enabled, allthe eCM removes the

Ethernet FCS/CRC32 from the original packets. All Ethernet

packets received by the SLED from the LCI are then

processed

as follows:

1. If the received Ethernet packet is greater than 1472

octets, the Ethernet packet is split into two fragments,

the first consisting of the first 1472 octets of the

Ethernet packet and the second containing the remaining

octets, resulting in two payloads that are processed as

described below. If the received Ethernet packet is less

than or equal to 1472 octets, the entire packet will be

processed as a single payload.

2. For each payload generated in step 1, the payload is

appended to the contents of sledLoopbackPktHdr.

3. The mutable fields within sledLoopbackPktHdr MUST be

recomputed. The mutable fields are IP Header Checksum, IP

Total Length, UDP Checksum, and UDP  Length.

4. If the Ethernet packet was fragmented in step 1, the

appropriate IP header fields (Flags and Fragment Offset)

are updated to indicate IP fragmentation. These IP

fragmentation header values will differ depending on

if this is the first or second fragment being processed.

5. The Ethernet FCS is computed and appended.

6. The resulting Ethernet packet is transmitted to the

LCI."

DEFVAL { false }

::= { sledLoopback 2 }

sledLoopbackPktHdr OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(42))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"A properly formatted Ethernet(DIX)+IP+UDP headers for use

in SLED loopback processing as described in

sledLoopbackEnable. The object value contains mutable

fields that are recomputed: the IP Header Checksum,

IP Total Length, UDP Length, and UDP Checksum. Any attempt

to set this object while sledLoopbackEnable is set to

'true' returns 'notWritable'."

::= { sledLoopback 3 }

--

-- The following group describes the packet generation objects

--

sledPktGenInterface OBJECT-TYPE

SYNTAX		      InterfaceIndex

MAX-ACCESS	  read-write

STATUS		      current

DESCRIPTION

"The index of the logical CPE interface (LCI) that the SLED

packet generation function is attached to.  If the index

does	    not correspond to a LCI supported by the device,

'wrongValue'	    is returned. Any attempt to set this object

while         sledPktGenTrigger is set to 'start' returns

'notWritable'."

::= { sledPktGen 1 }

sledPktGenPayload OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(64..1518))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The properly formatted Ethernet packet payload to be

generated.        Any attempt to set this object while

sledPktGenTrigger  is set        to 'start' returns

'notWritable'."

::= { sledPktGen 2 }

sledPktGenRate OBJECT-TYPE

SYNTAX      IntegerUnsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The packet rate (in packets per second) that the SLED is

to        transmit the packet specified in the sledPktGenPayload.

Any        attempt to set this object while sledPktGenTrigger is

set to        'start' returns 'notWritable'."

DEFVAL { 10 }

::= { sledPktGen 3 }

sledPktGenNumPkts OBJECT-TYPE

SYNTAX      IntegerUnsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of packets to be generated at the rate specified by

sledPktGenRate. Any attempt to set this object while

sledPktGenTrigger  has been set to 'start' will return

'notWritable'."

DEFVAL { 1 }

::= { sledPktGen 4 }

sledPktGenTrigger OBJECT-TYPE

SYNTAX      INTEGER {

start(1),

stop(2)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object controls the packet generation.  Setting this

object to 'start' causes the packet generation to begin.

Reading this  object will return 'start' if a packet

generation is in progress, otherwise it will return 'stop'.

Setting this object to 'stop' while packet generation is in

progress aborts the packet  generation.  `

to 'start' while packet  generation is in progress,

'wrongValue' is returned."

DEFVAL { stop }

::= { sledPktGen 5 }

sledPktGenLastTrigger OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Value of sysUptime when the  packet generation was

last triggered."

::= { sledPktGen 6 }

-- Conformance information *******************************************

sledMibCompliances  OBJECT IDENTIFIER ::= { sledMibConformance 1 }

sledMibGroups       OBJECT IDENTIFIER ::= { sledMibConformance 2 }

-- Compliance statements

sledMibCompliance MODULE-COMPLIANCE

STATUS      current

DESCRIPTION

"The compliance statement for SLED."

MODULE

-- unconditionally mandatory groups

MANDATORY-GROUPS {

sledMibBaseGroup

}

::= { sledMibCompliances 1 }

sledMibBaseGroup OBJECT-GROUP

OBJECTS {

sledGlobalEnable,

sledLoopbackInterface,

sledLoopbackEnable,

sledLoopbackPktHdr,

sledPktGenInterface,

sledPktGenPayload,

sledPktGenRate,

sledPktGenNumPkts,

sledPktGenTrigger,

sledPktGenLastTrigger

}

STATUS      current

DESCRIPTION

"Group of object in SLED MIB."

::= { sledMibGroups 1 }

END

Annex C

Proposed Changes

Summary: eDOCSIS clarification on the effect of NACO state on eSAFE traffic

Change #1

ORIGINAL TEXT: Section 5.2.2, Interface Requirements

5.2.2 Interface Requirements

The bridging function between RF port and the CPE interfaces (logical or physical) MUST be equivalent to that of a multi-port learning bridge. Each CPE interface MUST comply with the CM Forwarding Rules defined in [DOCSISx-RFI]7. In particular:

- MAC addresses seen on the interface are learned or provisioned as defined in [DOCSISx-RFI] specification, and are counted toward the total allowed by the Maximum Number of CPEs configuration setting.

- Packet forwarding and filtering rules defined in [DOCSISx-RFI] specification apply to both logical and physical CPE interfaces as defined in this specification and in [DOCSISx-OSSI]8.

- Data forwarding through the interface will obey the Network Access Control Object as defined in [DOCSISx-RFI].

NEW TEXT: Section 5.2.2, Interface Requirements

5.2.2 Interface Requirements

The bridging function between RF port and the CPE interfaces (logical or physical) MUST be equivalent to that of a multi-port learning bridge. Each CPE interface MUST comply with the CM Forwarding Rules defined in [DOCSISx-RFI]7. In particular:

- MAC addresses of each eSAFE MUST be counted toward the total allowed by the Maximum Number of CPEs configuration setting.

- Packet forwarding and filtering rules defined in [DOCSISx-RFI] specification MUST apply to both logical and physical interfaces to eSAFEs as defined in this specification and in [DOCSISx-OSSI]8.

- Data forwarding through the interfaces to all eSAFEs - with the exception of the interface to the DSG Client (ifIndex 18) - MUST obey the Network Access Control Object as defined in [DOCSISx-RFI].  NACO state does not affect the forwarding of DSG traffic (through ifIndex 18) in an eDOCSIS device containing an eSTB.

Annex D

Proposed Changes

Summary: eSAFE MIB definition for eDOCSIS

Change #1

NEW TEXT: Annex B, eSAFE MIB Definition as follows:

Annex B

eSAFE MIB Definition

An eCM in an eDOCSIS device implementing an embedded CableHome Portal Services logical element  (ePS) or/and embedded PacketCable Multimedia Terminal Adapter logical element (eMTA) as an eSAFE MUST implement the following eSAFE MIB.

eSAFE-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

Unsigned32,

BITS,

OBJECT-TYPE             FROM SNMPv2-SMI  --RFC 2578

OBJECT-GROUP,

MODULE-COMPLIANCE       FROM SNMPv2-CONF  -- RFC 2580

TruthValue,

DateAndTime

FROM SNMPv2-TC  -- RFC 2579

SnmpAdminString         FROM SNMP-FRAMEWORK-MIB --RFC 3411

ifIndex                 FROM IF-MIB

clabProjDocsis          FROM CLAB-DEF-MIB

;

esafeMib MODULE-IDENTITY

LAST-UPDATED "200411240000Z" –- November 24, 2004

ORGANIZATION "CableLabs eDOCSIS Working Group"

CONTACT-INFO

"John Eng

Postal: Cable Television Laboratories, Inc

858 Coal Creek Circle

Louisville, CO 80027

U.S.A.

Phone: +1 303-661-9100

Fax: +1 303-661-9199

E-mail: mibs@cablelabs.com"

DESCRIPTION

"This MIB module provides the management objects necessary

to configure functionality of eSAFE components of a device

implementing an eDOCSIS compliant cable modem and one or

more eSAFE elements."

REVISION "200411240000Z" -- November 24, 2004

DESCRIPTION

"Initial version of the eDOCSIS eSAFE MIB module."

::= { clabProjDocsis 14 }

-- Administrative assignments

-- esafeNotifications  OBJECT IDENTIFIER ::= { esafeMib 0 }

esafeMibObjects     OBJECT IDENTIFIER ::= { esafeMib 1 }

esafeBase           OBJECT IDENTIFIER ::= { esafeMibObjects 1 }

esafePsMibObjects   OBJECT IDENTIFIER ::= { esafeMibObjects 2 }

-- Placeholders

-- esafeMtaMibObjects  OBJECT IDENTIFIER ::= { esafeMibObjects 3 }

-- esafeStbMibObjects  OBJECT IDENTIFIER ::= { esafeMibObjects 4 }

-- Object Groups

--

-- eSAFE Base Objects

--

esafeProvisioningStatusTable OBJECT-TYPE

SYNTAX      SEQUENCE OF EsafeProvisioningStatusEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the current provisioning

status of each implemented eSAFE, and information

about the last failure or exception condition in

the eSAFE provisioning process, if applicable."

::= { esafeBase 1 }

esafeProvisioningStatusEntry OBJECT-TYPE

SYNTAX      EsafeProvisioningStatusEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry in this table is created for

each eSAFE implemented in the eDOCSIS

compliant device."

INDEX { ifIndex }

::= { esafeProvisioningStatusTable 1 }

EsafeProvisioningStatusEntry ::=SEQUENCE

{

esafeProvisioningStatusProgress          INTEGER,

esafeProvisioningStatusFailureFound      TruthValue,

esafeProvisioningStatusFailureFlow       SnmpAdminString,

esafeProvisioningStatusFailureEventID    Unsigned32,

esafeProvisioningStatusFailureErrorText  SnmpAdminString,

esafeProvisioningStatusLastUpdate        DateAndTime

}

esafeProvisioningStatusProgress OBJECT-TYPE

SYNTAX      INTEGER {

notInitiated(1),

inProgress(2),

finished(3)

}

MAX-ACCESS   read-only

STATUS       current

DESCRIPTION

"The current state of the eSAFE provisioning process.

notInitiated(1) indicates that the eSAFE has not yet

begun its provisioning process.

inProgress(2) indicates that the eSAFE is in the process

of provisioning, has not yet completed, and has not

encountered an error or exception condition.

finished(3) indicates that the eSAFE completed

its provisioning process. Provisioning success

or failure information is provided by other

objects, including

esafeProvisioningStatusFailureFound and

cabhPsDevProvState."

::={ esafeProvisioningStatusEntry 1 }

esafeProvisioningStatusFailureFound OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"true(1) indicates that the eSAFE encountered

an error condition during the provisioning

process. An eSAFE could start a looping process

from a previous flow step after a failure,

therefore this value is retained until the

flow step that initially failed eventually

passes or is updated with another error

condition."

REFERENCE

"PacketCable Provisioning specification,

Provisioning Overview section; CableHome

specification, Provisioning Processes

section."

::={ esafeProvisioningStatusEntry 2 }

esafeProvisioningStatusFailureFlow OBJECT-TYPE

SYNTAX      SnmpAdminString

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If esafeProvisioningStatusFailureFound

is true(1) this object contains the

label for the provisioning flow step

in which the error condition was

encountered, otherwise an empty value

is reported.

The value of this object corresponds to

the provisioning sequence 'Flow Step'

designator for the associated eSAFE, as

defined in the eSAFE specification. For

example, an ePS will report a value like

CHPSWMD-1, and an eMTA will report a value

such as MTA-1."

REFERENCE

"PacketCable Provisioning specification,

Provisioning Overview section; CableHome

specification, Provisioning Processes

section."

::={ esafeProvisioningStatusEntry 3 }

esafeProvisioningStatusFailureEventID OBJECT-TYPE

SYNTAX      Unsigned32 (0..4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If esafeProvisioningStatusFailureFound

is true(1) this object contains the

eSAFE log error Event Identifier defined

in the eSAFE specification, otherwise

it returns a value '0'. For an eMTA type

eSAFE, this object reports the PacketCable

EventID value from the Provisioning Events

table, e.g., 65535. For an ePS type eSAFE,

this object reports the EventID value from

the Defined Events for CableHome table, e.g.,

68000100."

REFERENCE

"PacketCable Provisioning specification,

Appendix I Provisioning Events; CableHome

specification, Appendix II Format and Content

for Event, SYSLOG, and SNMP Trap."

::={ esafeProvisioningStatusEntry 4 }

esafeProvisioningStatusFailureErrorText OBJECT-TYPE

SYNTAX      SnmpAdminString

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If esafeProvisioningStatusFailureFound

is true(1) this object contains the eSAFE

log error messages based on the eSAFE

specification, otherwise it returns an

empty value. For an eMTA type eSAFE, this

object reports the value from the

'Comments' column of the Provisioning

Events table, e.g., 'DNS Srv request

has been transmitted and no reply has

yet been received.' For an ePS type

eSAFE this object reports the value

from the 'Event Text' column of the

Defined Events for CableHome table,

e.g., 'DHCP Failed - Discover sent,

no offer received."

REFERENCE

"PacketCable Provisioning specification,

Appendix I Provisioning Events; CableHome

specification, Appendix II Format and Content

for Event, SYSLOG, and SNMP Trap."

::={ esafeProvisioningStatusEntry 5 }

esafeProvisioningStatusLastUpdate OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of the eCM docsDevDateTime when

this row entry was last updated."

::={ esafeProvisioningStatusEntry 6 }

esafeDevStatusTable OBJECT-TYPE

SYNTAX      SEQUENCE OF EsafeDevStatusEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains entries that provide the SNMP manager

with status information pertaining to each implemented

eSAFE device. While this table MUST be implemented by all

eDOCSIS devices, the support for reporting such information

and the status conditions supported will be determined by

the corresponding eSAFE specifications. It is highly

recommended that the eSAFE MIBs themselves have objects

to specify more detailed information."

::= { esafeBase 2 }

esafeDevStatusEntry OBJECT-TYPE

SYNTAX      EsafeDevStatusEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry in this table MUST be created for each eSAFE

device behind the eCM. The index needs to be the

corresponding index in the ifTable for the associated

eSAFE device."

INDEX { ifIndex }

::= { esafeDevStatusTable 1 }

esafeDevStatusEntry ::=SEQUENCE

{

esafeDevServiceIntImpact INTEGER,

esafeDevServiceIntImpactInfo SnmpAdminString

}

esafeDevServiceIntImpact OBJECT-TYPE

SYNTAX      INTEGER {

significant(1),

none(2),

unsupported(3)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this MIB object indicates the service

interruption impact assessment of the corresponding eSAFE

device as determined by the current status of the eSAFE

device, in accordance with the directives provided in the

eSAFE specification.

If esafeDevServiceIntImpact is set to significant (1), it

indicates that the corresponding eSAFE device (as per the

eSAFE specification) identifies a significant impact on the

active services at the given point in time. This impact

level is highly recommended for critical or real-time

services, though the impact assesment is left to the

directives provided by the associated eSAFE specification.

If esafeDevServiceIntImpact is set to none (2), it

indicates that the corresponding eSAFE device (as per the

eSAFE specification) identifies no significant impact on

the services offered at the given point in time.

If esafeDevServiceIntImpact is unsupported(3), it indicates

that the corresponding eSAFE device has no known interfaces

to support this feature or the eSAFE specification does not

recommend this feature.

If the eSAFE specification specifies the use of this

mechanism then it MUST define definitive states for the

impacts (significant or none) and the value of

unsupported(3) MUST not be used by the eDOCSIS device

for that eSAFE interface.

However, if the corresponding eSAFE specification does not

provide any directives then the value MUST be set to

unsupported(3).

If there exists multiple services being offered by an eSAFE

device (Either multiple services or multiple instances of

the same service), this MIB MUST indicate the highest

possible impact and other impact information SHOULD be

populated in the associated esafeDevServiceIntImpactInfo

table."

::={ esafeDevStatusEntry 1 }

esafeDevServiceIntImpactInfo OBJECT-TYPE

SYNTAX      SnmpAdminString

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides more information to the SNMP Managers

regarding the condition reported in

esafeDevServiceIntImpact. The eSAFE device vendor could use

this to fill in specific vendor strings or values that

could add value or provide more information related to the

status.

Examples:

For eMTA devices:

Lines 1 and 3 have active connections, Line 2 is not

provisioned.

<Value of the corresponding MIB object in the eMTA

MIBs, if applicable>

For other eSAFE devices:

Critical video streaming in progress, please wait for

5.30 minutes

<Value of the corresponding MIB object in the eSAFE

MIBs, if applicable>.

The device MUST report 'No Additional Information' in case

the associated  eSAFE vendor cannot obtain information from

the eSAFE device."

::={ esafeDevStatusEntry 2 }

--

-- Objects that apply to an eCM with an ePS type eSAFE.

--

esafePsCableHomeModeControl OBJECT-TYPE

SYNTAX      INTEGER {

disabledMode(1),

provSystem(2),

dormantCHMode(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object provides control over the mode of

operation of the CableHome ePS eSAFE element

of the eDOCSIS compliant device.

When this object is set to disabledMode(1), the

ePS eSAFE element is instructed to switch to

CableHome Disabled Mode operation.

When this object is set to provSystem(2), the ePS

eSAFE element restarts its provisioning process.

When this object is set to dormantCHMode(3), the ePS

eSAFE element is instructed to switch to CableHome

Dormant Mode operation. In this mode the ePS restarts

its provisioning process omitting CableHome-specific

DHCP Options 60 and 43 in the DHCP DISCOVER and

DHCP REQUEST messages, acquires an IP address lease from

the cable operator's DHCP server, and operates in unmanaged

Dormant CableHome Mode regardless of the values of the file

and siaddr fields or of the values of DHCP options that

might otherwise configure the ePS to operate in DHCP

Provisioning Mode or in SNMP Provisioning Mode.

The value of this object MUST persist across cable modem

resets."

REFERENCE

"CableHome specifications, CableHome Operational

Models section."

DEFVAL { dormantCHMode }

::= { esafePsMibObjects 1 }

esafePsCableHomeModeStatus OBJECT-TYPE

SYNTAX      INTEGER {

disabledMode(1),

dormantCHMode(2),

cableHomeMode(3)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides visibility to the current

mode of operation of the CableHome ePS eSAFE

element of the eDOCSIS compliant device.

If the value of this object is disabledMode(1), the

ePS eSAFE element is currently operating in CableHome

Disabled Mode.

If the value of this object is dormantCHMode(2), the

ePS is currently operating in Dormant CableHome Mode.

If the value of this object is cableHomeMode(3), the ePS

is currently operating in CableHome mode."

REFERENCE

"CableHome specification, CableHome Operational Models

section."

::= { esafePsMibObjects 2 }

-- Conformance information *******************************************

esafeMibConformance     OBJECT IDENTIFIER ::= { esafeMib 2 }

esafeMibCompliances     OBJECT IDENTIFIER ::= { esafeMibConformance 1 }

esafeMibGroups          OBJECT IDENTIFIER ::= { esafeMibConformance 2 }

-- Compliance statements

esafeMibBasicCompliance MODULE-COMPLIANCE

STATUS      current

DESCRIPTION

"The compliance statement for eSAFE MIB objects."

MODULE    -- eSAFE-MIB

-- unconditionally mandatory groups

MANDATORY-GROUPS {

esafeBaseGroup

}

-- conditionally mandatory groups

GROUP esafePsMibGroup

DESCRIPTION

"This group is implemented only by eDOCSIS devices

that implement an embedded Portal Services logical

element (ePS) compliant with CableLabs

CableHome specifications."

::= { esafeMibCompliances 1 }

-- eSAFE Base Group Declarations

esafeBaseGroup OBJECT-GROUP

OBJECTS {

esafeProvisioningStatusProgress,

esafeProvisioningStatusFailureFound,

esafeProvisioningStatusFailureFlow,

esafeProvisioningStatusFailureEventID,

esafeProvisioningStatusFailureErrorText,

esafeProvisioningStatusLastUpdate,

esafeDevServiceIntImpact,

esafeDevServiceIntImpactInfo

}

STATUS      current

DESCRIPTION

"Group of eSAFE Base objects in the eSAFE MIB."

::= { esafeMibGroups 1 }

--  PS MIB Group

esafePsMibGroup OBJECT-GROUP

OBJECTS {

esafePsCableHomeModeControl,

esafePsCableHomeModeStatus

}

STATUS      current

DESCRIPTION

"Group of embedded PS-specific objects

in the eSAFE MIB."

::= { esafeMibGroups 2 }

END

Annex E

Proposed Changes

Summary: Modify eDOCSIS firmware download requirements to account for eSTB

Change #1

ADD NEW TEXT:

Appendix I - Bibliography (Informative)

[OC]	OpenCable Host Core Functional Requirements 2.0 Specification, OC-SP-HOST2.0-CFR-I02-041119, CableLabs.

[OC2]	OpenCable Common Download Specification, OC-SP-CD-IF-I07-040402”, April 2, 2004, CableLabs.

Change #2

NEW TEXT: (Add the following reference to Subclause 2.2 References - Informative)

[BPI+]	ITU-T Recommendation J.125 (04/2004), Link privacy for cable modem implementations

Change #3

NEW TEXT: (Add the following to Section 3, Glossary)

Embedded Security eSTB	An eSTB with integrated security functions

OpenCable Host eSTB		An eSTB device built to OpenCable Host specifications

Change #4

ORIGINAL TEXT: None

NEW TEXT: Section 5.2.6, Firmware Download

5.2.6 Firmware Download

All eDOCSIS devices  MUST support a single secured firmware image download that MUST be used for the entire eDOCSIS device.

With the exception of eDOCSIS devices that contain eSTBs, the download MUST be controlled by the eCM using the DOCSIS Secure Software Download (SSDL) mechanisms as specified in [BPI+].

Firmware download for eDOCSIS devices that contain eSTBs (referred to as a Set-top Device) is defined in the following subsection.

5.2.6.1 Set-top Device Firmware Download

5.2.6.1.1 Firmware Download for OpenCable Advanced Host Set-top Devices

An OpenCable Advanced Host Set-top Device MUST support both or either one of the following firmware download methods:

- OpenCable Common Download (OCCD) mechanisms as defined in the OpenCable Common Download specification [OC2],

2.		DOCSIS Secure Software Download (SSDL) mechanisms as defined in [BPI+].

5.2.6.1.2 OSS Requirements for OCCD

If the Set-top Device employs OCCD firmware download mechanisms, it’s eCM MUST set the docsDevSwOperStatus to “other” (5) and MUST respond to an attempt to trigger a TFTP upgrade (initiated through SNMP or Configuration File) as follows:

- The SNMP Set or Config File TLV MUST be ignored and the Device MUST remain capable of accepting new software through the non-DOCSIS firmware download mechanism.

- The eCM MUST report the attempt to trigger a TFTP firmware upgrade by logging the appropriate event (via an entry in the docsDevEvTable).  The text of this event MUST explicitly indicate that the TFTP upgrade is not applicable for this Set-top Device.

- The docsDevSwOperStatus MUST remain other (5).

If the Set-top Device employs the OCCD firmware download mechanism, then all the download-related requirements as specified in [BPI+] do not apply.  Additionally the eCM in this type of Set-top Device MUST support the BPI+ MIB docsBpi2CodeDownloadControl objects with the following constraints (other objects within docsBpi2CodeDownloadControl are as defined within the MIB):

- docsBpi2CodeDownloadStatusCode always reports other (7)

- docsBpi2CodeDownloadStatusString always returns the string “OpenCable Common Download”

If the Set-top Device employs the OCCD firmware download mechanism, its eCM MUST ignore any firmware upgrade related information in the CM Configuration File and MUST log the appropriate event (via an entry in the docsDevEvTable).  The text of this event MUST explicitly indicate that the firmware upgrade related information is not applicable. The message must be sent at level “4 Error”.

Annex F

Proposed Changes

Summary: Additional RFC 1493 Requirements for eDOCSIS devices

Change #1

ORIGINAL TEXT: Section 5.2.3.3 [RFC 1493] Requirements (entire subclause)

Ports associated with SAFEs MUST be added to dot1dBasePortTable.

The dot1dTpFdbTable MUST contain information about learned or provisioned CPEs through the logical CPE interfaces with learned(3) or mgmt(5) status accordingly.

All bridge statistics of dot1dTpPortTable MUST be supported for logical CPE interfaces.

NEW TEXT: Section 5.2.3.3 [RFC 1493] Requirements

The eCM MUST add ports associated with eSAFEs to its [RFC 1493] dot1dBasePortTable.

The eCM MUST support all bridge statistics of the [RFC 1493] dot1dTpPortTable for all ports associated with eSAFEs.

The eCM MUST create a row entry in its dot1dTpFdbTable for each active eSAFE MAC address within the eDOCSIS device.

Within each dot1dTpFdbTable entry for an eSAFE, the dot1dTpFdbAddress value MUST be the eSAFE MAC address, the dot1dTpFdbPort value MUST be the port associated with the ifIndex of that eSAFE from the dot1dBasePortTable, and the dot1dTpFdbStatus value MUST be mgmt(5).  The eCM MUST prevent row entries for eSAFEs from being aged-out or overwritten.

An eDOCSIS device containing an eSTB MUST implement the objects from the [RFC 1493] dot1dStaticTable with a MAX-ACCESS of read-only.

In an eDOCSIS device containing an eSTB, the eCM will acquire the DSG MAC addresses from the eSTB in an implementation-dependant manner.  For each DSG MAC address acquired by the eCM, the eCM MUST create a corresponding row entry in the dot1dStaticTable.

For a dot1dStaticTable row entry for a DSG MAC address, the dot1dStaticAddress value MUST be the DSG MAC address, the dot1dStaticReceivePort value MUST be the port associated with ifIndex 2 (CATV-MAC interface) from the dot1dBasePortTable, the dot1dStaticAllowedToGoTo value MUST be the port associated with the ifIndex 18 (interface to the DSG Client of the eSTB) from the dot1dBasePortTable, and the dot1dStaticStatus value MUST be deleteOnReset(4).

Table 5-X	dot1dStaticTable Entries for DSG MAC Addresses

| MIB Object | Value |
| --- | --- |
| dot1dStaticAddress | DSG MAC Address |
| dot1dStaticReceivePort | port associated with ifIndex 2 in dot1dBasePortTable |
| dot1dStaticAllowedToGoTo | port associated with ifIndex 18 in dot1dBasePortTable |
| dot1dStaticStatus | deleteOnReset(4) |

Each row entry in the dot1dStaticTable describes a bridging rule in which the eCM only bridges a frame with a destination address of the value of dot1dStaticAddress - that is received from the port represented by the value of dot1dStaticReceivePort - if the frame is to be bridged out a port included in the value of dot1dStaticAllowedToGoTo.

An eCM MUST NOT bridge a frame having a destination address equal to an active DSG MAC address if that frame was received from any port other than the one associated with ifIndex 2 (CATV-MAC).

An eCM MUST NOT bridge a frame having a destination address equal to an active DSG MAC address to any port other than the one associated with ifIndex 18 (the interface to the DSG Client of the eSTB).

Annex G

Proposed Changes

Summary:  . Clarification of MaxCPE count for DSG tunnel MAC addresses

Change #1

ORIGINAL TEXT: None

NEW TEXT: Section 5.2.2 Interface Requirements

--add the following bullet after the 1st bullet--

- The DSG tunnel MAC addresses associated with ifIndex=18 MUST NOT be counted in the MaxCPE count of the eCM configuration file.

Change #2

ORIGINAL TEXT: None

NEW TEXT: Section 5.2.3, [RFC 1493] Requirements

--Insert the following after the last paragraph (these two paragraphs should be the last prior to subclause 5.2.4):

If a CPE MAC address is acquired by the eCM via the eCM Configuration  File or via the address learning process, and the eCM is later informed  that the same address is a DSG tunnel MAC address, the eCM SHOULD remove  the DSG tunnel MAC address from its list of acquired CPE MAC addresses.

Also, the eCM SHOULD NOT populate a CPE MAC address from the eCM Configuration File into its list of acquired CPE MAC addresses if that MAC address matches that of a DSG tunnel MAC addresses already established via DSG operation.

Annex H

Spec Change #1

Clause 5.2.4.1 - General Requirements  (both paragraphs)

ORIGINAL SPEC TEXT:

Each eCM DHCP DISCOVER MUST use Option 43 and its Sub-options 1 through 10 for Vendor Specific Information to identify embedded components as specified in Table 5-4.

Similarly, each eSAFE may issue its own DHCP DISCOVER with Option 43 after eCM has been successfully registered and operational; details are specified in each eSAFE’s specification.

NEW SPEC TEXT:

Each eCM DHCP DISCOVER and DHCP REQUEST MUST use Option 43 and its Sub-options 2 through 10 for Vendor Specific Information to identify embedded components as specified in Table 5-4.

Similarly, each eSAFE may issue its own DHCP DISCOVER and DHCP REQUEST with Option 43 after eCM has been

successfully registered and operational; details are specified in each eSAFE’s specification.

Spec Change #2

Clause 5.2.4.3 - DHCP Option 43 Sub-option Encoding - 2nd paragraph

ORIGINAL SPEC TEXT:

The eCM MUST encode DHCP Option 43 sub-option 1 by the number of octets equal to the value of the length octet of this sub-option, with each octet codifying a requested sub-option.

NEW SPEC TEXT:

The eCM MAY encode DHCP Option 43 sub-option 1 by the number of octets equal to the value of the length octet of this sub-option, with each octet codifying a requested sub-option.  If the length octet of this sub-option is 0 (because there are no requested sub-options), this sub-option SHOULD be omitted from DHCP Option 43.

Spec Change #3

Clause 5.2.4.3 - DHCP Option 43 Sub-option Encoding - 11th paragraph

ORIGINAL SPEC TEXT:

An eCM MUST send DHCP Option 43 sub-option 8 containing a 3-octet, hexidecimally-encoded, vendor specific Organization Unique Identifier (OUI) that uniquely identifies the eCM manufacturer. A vendor MAY use the same OUI as in the eCM’s MAC address, and MAY use a single OUI to identify all its eDOCSIS products.

NEW SPEC TEXT:

An eCM MUST send DHCP Option 43 sub-option 8 containing a 6-octet, hexidecimally-encoded, vendor specific Organization Unique Identifier (OUI) that uniquely identifies the eCM manufacturer. A vendor MAY use the same OUI as in the eCM’s MAC address, and MAY use a single OUI to identify all its eDOCSIS products.

Spec Change #4 - Clause 5.2.4.3 - DHCP Option 43 Sub-option Encoding

ORIGINAL SPEC TEXT in Table 5-4/J.126 – DHCP Option 43 syntax - (Row 8  - Description column):

A 3-octet, hexidecimally-encoded, vendor-specific Organization Unique Identifier (OUI) that may match the OUI in eCM’s MAC address

.

NEW SPEC TEXT:

A 6-octet, hexidecimally-encoded, vendor-specific Organization Unique Identifier (OUI) that may match the OUI in eCM’s MAC address.

Change #5  -  Section 5.2.4.3 - 2nd paragraph

ORIGINAL TEXT:

The eCM MAY encode DHCP Option 43 sub-option 1 by the number of octets equal to the value of the length octet of this sub-option, with each octet codifying a requested sub-option.  If the length octet of this sub-option is 0 (because there are no requested sub-options), this sub-option SHOULD be omitted from DHCP Option 43.

NEW TEXT:

The eCM MAY include Option 43 sub-option 1 in DHCPDISCOVER and DHCPREQUEST messages. If DHCP Option 43 sub-option 1 is included in these DHCP client messages, the eCM MUST encode this sub-option by the number of octets equal to the value of the length octet of this sub-option, with each octet codifying a requested sub-option.  If the length octet of this sub-option is 0 (because there are no requested sub-options), this sub-option SHOULD be omitted from DHCP 
Option 43.

Appendix I

Proposed Changes

Summary: Miscellaneous Editorial Changes

Change #1

ORIGINAL TEXT: Section 3, Glossary

Logical CPE Interface (Logical CMCI)	A bi-directional, data-only 802.3/Ethernet MAC fram interface between eCM and an eSAFE

NEW TEXT: Section 3, Glossary

Logical CPE Interface	A bi-directional or unidirectional data-only logical 802.3/Ethernet MAC frame interface between eCM and an eSAFE

Change #2

NEW TEXT: Section 4, Abbreviations  (Add the following new abbreviations:)

LCI	Logical CPE Interface

SLED	Software Loopback for eDOCSIS

eSTB	embedded Set-top Box

DSG	DOCSIS Set-top Gateway

Change #3

(Update title of subclause 5.2.3.2)

ORIGINAL TEXT:

5.2.3.2 	IpNetToMedia Table Requirements

NEW TEXT:

5.2.3.2 [RFC 2011] IpNetToMedia Table Requirements

Appendix II

Proposed Changes

Summary: Reserved DHCP 43 suboptions for eSAFEs

Change #1

ORIGINAL TEXT: Section 5.2.4.3

In eDOCSIS, DHCP Option 43 sub-option 11-127 are reserved for future definitions and MUST NOT be used for other purpose.

NEW TEXT:

In eDOCSIS, DHCP Option 43 sub-option 11-127 are reserved for eSAFEs and future definitions, and MUST NOT be used for other purpose. The partitions are as follows:

- Sub-options 11 to 30:		reserved for IPCable2Home use,

- Sub-options 31 to 50: 	reserved for IPCablecom use,

- Sub-options 51 to 127:	reserved for future definitions use.

_________