Attachment 3 To document 6M/TEMP/66

Source:	Document 6M/TEMP/72

Subject:	Question ITU-R 13/6

INTERFACES BETWEEN AN APPLICATION AND THE SYSTEM

# Description of Interactive Receivers

Applications use the Application Programming Interface (API) to access the actual resources of the receiver, including e.g. CPU, databases, streamed media decoders, static content decoders and communications. These resources are functional entities of the receiver and are mapped onto the hardware of the receiver in some manner.

Figure 1

The conceptual diagram in figure 1 shows the interfaces and their relationship to media and information flows within the system. The API is depicted as the border between the application and the system.

Digital TV Receivers have the following key HW elements:

•	Tuner

•	Demodulator (decodes the tuner signal and makes the TS available for the MPEG decoder)

•	MPEG decoder (de-multiplexes the components of the TS, like video and audio)

•	Graphics engine (blends video and graphics and modulates for output)

•	Audio decoder (decodes audio and modulates for output)

•	CPU (runs applications, initializes and controls the other components)

•	Flash ROM (stores the middleware and resident applications in compressed form, and user profiles)

•	RAM (working memory for the CPU and graphics rendering)

Here follows a list of the interactive TV systems considered in this document:

TV+PC	This architecture is one where the TV broadcast and TV receiver have no enhanced content. Instead the programming is being enhanced through the use of the Internet and a web-based computer (PC). “Normal” viewers are not provided any extra experience. Alternatively, the computer could have a video receiver built in (PC with TV add-on card), which would provide the TV programme on the same screen as the web browsing.

DDE-1-A/B	IEC/PAS 62292 Ed 1.0 (2001-12): Draft SMPTE Engineering Guideline SMPTE XXX-Declarative Data Essence.

IEC62298-4		TeleWeb application: Interaction Profile

DASE-DA	DTV application software environment level 1 (DASE-1), part 2: declarative applications and environment

DASE-PA	DTV application software environment level 1 (DASE-1), part 3: procedural applications and environment

IEC62298-3		TeleWeb application: Superteletext Profile

MHEG-5		MHEG-5 UKEngineProfile1

MHP-1.0		ETSI TS 101 812

MHP-1.1		ETSI TS 102 812

OCAP			OCAP 2.0 Profile: OC-SP-OCAP2.0-I01-020419

Table 1

Definition of integrated Digital TV and mapping to interactive TV systems

The differences between different DTV systems are in software and memory requirements.

•	A low-end interactive profile MHP receiver has been estimated as requiring 8Mbyte of flash ROM and 8-16Mbyte of RAM, see “The cost of MHP” whitepaper [http://www.mhp.org/news_events/pdf_and_other_files/CostsMHP.pdf]. Additional RAM is required for graphics. Actual low-end MHP products are equipped with 16Mbyte RAM or more. For further information see Annex A.

•	A low-end MHEG-box has 2MByte flash ROM and 2MByte RAM. Top of the range boxes have 8MByte RAM, in which they can store the whole object carousel.

•	ATSC-DASE boxes …TBD.

•	BML boxes in practice have 2-16MByte RAM, ?? Mbyte Flash ROM.

CPU requirements depend very much on the software stack. An optimized stack will run many times faster than a non-optimized one. The first MHP-boxes have 80-130 MHz processors. It is expected that in high-end boxes 2-5 times more power will be provided, i.e. in the order of 300 MIPS.

A recommendation for good practice in broadcasting and reception of digital services is given in International Standard IEC 62216, which specifies the baseline receiver for the DVB-T system. Its objective is to define the behavior required from receivers to work well with these broadcasts in order to be attractive to consumers.

## Mapping Application Scenarios to Resources

Document 6M/46-Annex 6 deals with the development of a framework model for harmonization of digital interactive television standards. The approach, based upon a "top down view" of the full end-to-end environment, is driven by the application developer's perspective as a starting point.

Services/Applications can be evaluated in respect to their character, their required system functionality and their related User Experiences/Expectations. Each Service/Application can be described by a set of unique "Descriptor Elements" with their own range of values. A first example of such a set is listed in table 2. This list will need to be further developed and agreed upon.

Table 2

Initial Application Description

Document NIST GCR 01-818 shortly describes a number of interactive TV applications. The feasibility of running those applications on different interactive TV platforms is explored in table 9-1 in that document. Table 3 below extends that table with more interactive TV platforms, and also with the applications from document 6M/79.

Table 3

Interactive Applications for Emerging iDTV

Document 6M/79 provides scenarios of interactive television programming that the content community expects to be monetarily effective in delivering to television viewers in the next 3-5 years. The scenarios are provided in two appendices, the first of which is an overview of all possible scenarios in seven categories: advertising, games shows, news shows, sports shows, drama shows, broadcaster service layer, and non-linear programming. The second appendix provides details of one to three scenarios in each of the seven categories. By applying above described method each of the possible scenarios can be transferred into its application descriptor and these are used as input data for a mapping process to a gross list of all kind of possible resources and processor specifications. This process leads to a one-to-one relation between an application and the required resources and processing power to run this application. It is expected that using this process for all described scenarios will lead to a limited number of target receivers. Figure 2 gives an overview of the mapping process.

Figure 2

Mapping applications to receiver requirements

Tables 4-15 derive receiver requirements from application descriptions. The applications are those listed in 6M/79-AppendixB. The tables have three segments:

- Application characteristics

- Broadcast requirements, Communication channel requirements, System & I/O requirements

- Condensed application summary

The requirements in the 2nd segment are derived from the application characteristics listed in the 1st segment according to formulas. These formulas are listed here:

Broadcast Requirements Formula

Trigger:

IF( Data == Realtime ) Trigger = Required

ELSE Not Useful

Communication Channel Requirements formulas

Certification:

IF( Features == E-commerce ) Certification = Server Certification

ELSE IF( Communication Channel == Transaction ) Certification = Server Certification

ELSE Certification = None

Encryption:

IF( Features == E-commerce ) Encryption = Yes

ELSE IF( Communication Channel == Transaction ) Encryption = Yes

ELSE Encryption = No

Upstream Bandwidth:

Upstream Bandwidth  = Low

None of the application neither send nor receives large amounts of data through the communication channel.

Downstream Bandwidth:

Downstream Bandwidth = Low

None of the application neither send nor receives large amounts of data through the communication channel.

System & I/O requirements formulas

Memory Size:

IF( Memory Intensity == Low ) Memory Size = Low

ELSE IF( Memory Intensity == Moderate ) Memory Size = Moderate

ELSE IF( Memory Intensity == High ) Memory Size = High

ELSE Memory Size = Undefined

NV Memory Size:

IF( Features == Banking ) NV Memory Size = Moderate

ELSE NV Memory Size = Low

Processing Power:

IF( Processing Intensity == Low ) Processing Power = Low

ELSE IF(Processing Power == Moderate ) Processing Power = Moderate

ELSE IF(Processing Power == High ) Processing Power = High

ELSE Processing Power = Undefined

Procedural Capability:

IF(Local Calculation & Open Outcome == No ) Procedural Capability = No

ELSE Procedural Capability = Yes

Display Input

Audio Output

Printer:

IF(Printed Output == No ) Printer = No

ELSE Printer  = Yes

Removable Storage:

IF(Save to Device == No ) Removable Storage = No

ELSE Removable Storage = Yes

UI Device:

IF( Typing == No ) UI Device = RC

ELSE UI Device = KB

## Content Types vs Receiver Requirements

Figure 3 below shows an abstraction of a TV or STB. To a consumer the user interface and the possibility of connecting the TV to a recording or playback device is of crucial importance. Less important to the consumer is how content enters the TV. It can be an analog broadcast, or a digital broadcast or through the interaction channel.

Figure 3

Abstraction of an ITV

Receiver requirements can be deduced from the type of applications that the receiver is required to handle. An application can be classified according to the type of content it consists of.

From the tutorial paper on interactive TV [SMPTE journal , June 2002, p280-p284] the following interface model has been extracted.

FIGURE 4

Interactive content in DTV

The three rows in the table below are extracted from figure 4. Another crucial aspect of an application is whether it needs an interaction channel or not. This aspect defines the two columns in the table. In this way we can define a total of six different content types.

TABLE 4

Content types

At the receiver side the different content types can be handled according to the following list of six receiver types:

TABLE 5

Receiver types

# Annex A

Example of MHP STB supplier, see [http://www.espial.com/index.php?page=faq_itv]

The range of MHP 1.1 capable Set-Top Boxes are:

Low End Set-Top Box:

Video Stream content

16MB RAM

8MB Flash

50-120MHz

Mid Range Set-Top Box:

Walled Garden content

Email client

Video Stream content

32MB RAM

16MB Flash

150-200MHz

High End Set-Top Box:

Full Internet

Browsed mail (AOL mail, Hotmail)

Walled Garden content

Email client

Video Stream content

64+MB RAM

16-32MB Flash

200+MHz

# Annex B: Application Requirement tables

Table 6

Simple Advert

Table 7

Complex Advert

Table 8

Simple Gaming

Table 9

Moderate to Complex Gaming

Table 10

Complex Gaming

Table 11

Simple News

Table 12

Moderate News

Table 13

Complex News

Table 14

Simple Sports

Table 15

Moderate Sports

Table 16

Simple Drama

Table 17

Complex Drama

References are available in electronic form only as parts 2 and 3. See :

http://web.itu.ch/md/meetingdoc.asp?type=sitems&lang=e&parent=R00-WP6M-C-0141