Attachment 1 to document 6M/TEMP/66

Questions:	ITU-T 8/9, ITU-R 13/6

Source:		Document 6M/TEMP/62(Rev.1) (Edited)

Working Party 6M

Working document towards a
Preliminary Draft New Recommendation

Harmonization of procedural content formats for interactive TV applications

Summary

This Recommendation defines APIs, semantic guarantees and system aspects of platform behaviour for harmonized procedural content formats for interactive TV applications.

WORKING DOCUMENT TOWARDS A
PRELIMINARY DRAFT NEW RECOMMENDATION

Harmonization of procedural content formats for interactive TV applications

(Questions ITU-T 8/9 and ITU-R 13/6)

The ITU Radiocommunication Assembly,

considering

a)	the need to harmonize the applications environment to achieve interoperability, interchange of content and economies of scale;

b)	the substantial commonalities that have been identified in proposals contributed to ITU which are summarized in Appendix A;

c)	the need for an integrated approach to procedural and declarative aspects to which attention is drawn in Appendix A;

d)	the possibilities for migration to a common platform from legacy solutions,

recommends

[1	the use of the terminal specification framework given in Annex 1 which provides semantic guarantees and system aspects of platform behaviour for the APIs in the common core given in Appendix A;]

[2	the use of service-specific additions such as those given in Appendix A, where necessary];

[3	a close interrelationship between procedural and declarative aspects];

[4	the development of migration strategies from legacy systems, assisted, where necessary, by additional functionalities such as the example given in Appendix B].

Annex 1

Globally executable MHP

This document specifies a set of interfaces, and the semantic guarantees underlying those interfaces to enable binary interoperability of applications between different receiver specifications and/or standards.

The full text of this document (approx. 70 pages) will be referenced as an ETSI document in the final draft Recommendation.

Meanwhile:

a)	the content pages only are reproduced in this Appendix.

b)	the full text is available on the ITU-R WP 6M/ITU-T SG 9 JRG-1 e-mail reflector as Mug 033r4.pdf.

c)	Those not having access to the JRG-1 reflector requiring to review the full text should apply to DVB for a copy of GEM 1.0 document Mug 033r4

Contents

Contents	1

Intellectual Property Rights	6

Foreword	6

0	Introduction	6

0.1	Purpose	6

0.2	Format	7

0.3	Inclusion of MHP Features	7

0.3.1	Subsetting Prohibited	7

0.3.2	Supersetting Permitted	7

0.4	Addition of Non-GEM Interfaces	7

0.4.1	DVB-J enumerations	7

0.5	Application Areas	8

0.6	Profiles	8

1	Scope	9

2	References	9

3	Definitions and abbreviations	11

3.1	Definitions	11

3.1.1	Definitions From MHP	11

3.1.2	Definitions Introduced by GEM	11

3.2	Abbreviations	11

3.2.1	Abbreviations From MHP	11

3.2.2	Abbreviations Introduced by GEM	11

4	Conventions	11

4.1	References within the MHP Specification	11

4.2	Terminology in the MHP Specification	12

4.2.1	MHP	12

4.2.2	Resident Navigator	12

4.2.3	DVB Service	12

4.3	Inclusion of Sections of the MHP specification	12

5	Basic Architecture	13

6	Transport Protocols	14

6.1	Introduction	14

6.2	Broadcast Channel Protocols	14

6.2.1	MPEG-2 Transport Stream	14

6.2.2	MPEG-2 Sections	14

6.2.3	DSM-CC Private Data	14

6.2.4	DSM-CC Data Carousel	14

6.2.5	DSM-CC User-to-User Object Carousel.	14

6.2.5.1	DVB-J Class Files	14

6.2.5.2	DVB-HTML Document Files	15

6.2.5.3	Loss of Carousel Behaviour	15

6.2.6	Protocol for Delivery of IP Multicast over the Broadcast Channel	15

6.2.7	Internet Protocol (IP)	15

6.2.8	User Datagram Protocol (UDP)	15

6.2.9	DVB Service Information	15

6.3	Interaction Channel Protocols	15

7	Content formats	16

7.1	Static Formats	16

7.2	Broadcast Streaming Formats	16

7.2.1	Audio	16

7.2.2	Video	16

7.2.3	Subtitles	16

7.3	Resident fonts	16

7.4	Downloadable Fonts	16

7.5	Colour Representation	16

7.6	MIME Types	16

8	DVB-HTML	17

9	Application Model	18

9.1	Broadcast GEM Applications	18

9.2	DVB-J Model	18

9.3	DVB-HTML Model	18

9.4	Inter-Application Resource Management	18

10	Application Signalling	19

10.1	Introduction	19

10.1.1	Summary of Requirements on Common Signalling	19

10.1.2	Summary of additional signalling for DVB-J applications	19

10.2	Program Specific Information	19

10.3	Locators Within an Application Description	19

10.4	Application Description	19

10.4.1	Application Description Transmission and Monitoring	20

10.4.2	Visibility of Application Description	20

10.4.3	Content of the Application Description	20

10.4.4	Applications from Previously Selected Services	21

10.5	DVB-J Specific Application Description	21

10.5.1	Content of DVB-J application description	21

11	DVB-J Platform	23

11.1	The Virtual Machine	23

11.2	General Issues	23

11.3	Fundamental DVB-J APIs	23

11.4	Presentation APIs	23

11.5	Data Access APIs	24

11.6	Service Information and Selection APIs	24

11.6.1	DVB Service Information API	24

11.6.2	Service Selection API	24

11.6.3	Tuning API	24

11.6.4	Conditional Access API	24

11.6.5	Protocol Independent SI API	24

11.7	Common Infrastructure APIs	25

11.7.1	APIs to Support DVB-J Application Lifecycle	25

11.7.2	Application Discovery and Launching APIs	25

11.7.3	Inter-Application Communication API	26

11.7.4	Basic MPEG Concepts	26

11.7.5	Resource Notification	26

11.7.6	Content Referencing	26

11.7.7	Common Error Reporting	26

11.8	Security	27

11.9	Other APIs	27

11.9.1	Timer Support	27

11.9.2	User Settings and Preferences API	27

11.9.3	Profile and Version Properties	27

11.10	Java Permissions	27

11.11	Content Referencing	28

11.11.1	Transport stream	28

11.11.2	Network	28

11.11.3	Bouquet	28

11.11.4	Service	28

11.11.4.1	MPEG/GEM Specific Service	28

11.11.4.2	Generic Service	29

11.11.5	Program Event	29

11.11.6	MPEG Elementary Stream	29

11.11.7	File	29

11.11.8	Directory	29

11.11.9	Drip Feed Decoder	29

11.11.10	Irrelevant	29

11.11.11	Methods Working on Many Locator Types	30

11.11.12	Support for the HTTP Protocol in DVB-J	30

12	Security	31

12.1	Introduction	31

12.1.1	Overview of the security framework for applications	31

12.1.2	Overview of return channel security	31

12.1.3	Extensions to MHP application signing framework	31

12.2	Authentication of applications	31

12.3	Message transport	31

12.4	Detail of application authentication messages	32

12.5	Profile of X.509 certificates for authentication of applications	32

12.6	Security policy for applications	32

12.7	Example of creating an application that can be authenticated	32

12.8	GEM/MHP certification procedures	33

12.9	Certificate management	33

12.10	Security on the return channel	33

12.11	the internet profile of X.509 (informative)	33

12.12	Platform minima	33

13	Graphics reference model	34

13.1	Supported graphics resolutions	34

13.2	Broadcast streaming formats	34

13.3	Subtitles	34

14	System integration aspects	35

14.1	Namespace mapping	35

14.2	Reserved names	35

14.3	XML notation	35

14.4	Network signalling	35

14.5	Text encoding of application identifiers	35

14.6	Reserved names for persistent storage	35

14.7	Files and file names	35

14.8	Locators and content referencing	35

14.9	Service identification	36

14.10	CA system	36

15	Detailed platform profile definitions	37

15.1	PNG - restrictions	38

15.2	Minimum media formats supported by DVB-J APIs	39

15.3	JPEG - restrictions	39

15.4	Locale support	39

15.5	Video raster format dependencies	39

15.5.1	Standard Definition (PAL/SECAM or NTSC resolution)	39

15.5.1.1	Logical pixel resolution	39

16	Registry of Constants	40

16.1	System constants	40

16.2	DVB-J constants	40

Annex A (normative):	External references; errata, clarifications and exemptions	41

Annex B (normative):	Broadcast Filesystem and Trigger Transport	42

B.1	Service Domain	42

B.2	Filesystem Requirements	42

B.2.1	Static Requirements	42

B.2.1.1	Caching behaviour	43

B.2.2	Filesystem Updates	43

B.3	Stream Description	43

B.4	Trigger Signalling	44

B.4.1	Trigger Object	44

B.4.2	Trigger Event	44

B.4.2.1	Extrapolation of NPT Values	44

B.4.2.2	Monitoring of trigger events	45

Annex C (informative):	References	46

C.1	Informative References from MHP	46

C.2	Additional Informative References	46

Annex D (normative):	Text presentation	47

Annex E (normative):	Character set	48

Annex F (informative):	Authoring & Implementation Guidelines	49

Annex G (normative):	Minimum Platform Capabilities	50

G.1	Graphics	50

G.2	Audio	50

G.3	Video	50

G.4	Resident fonts and text rendering	50

G.5	Input events	50

G.6	Memory	50

G.7	Other resources	50

Annex H (normative):	Extensions	51

Annex I (normative):	DVB-J fundamental classes	52

Annex J (normative):	DVB-J event API	53

Annex K (normative):	DVB-J persistent storage API	54

Annex L (normative):	User Settings and Preferences API	55

Annex M :	Void	56

Annex N (normative):	Streamed Media API Extensions	57

Annex O (normative):	Integration of the JavaTV SI API	58

Annex P (normative):	Broadcast Transport Protocol Access	59

P.1	Overview	59

P.2	The org.dvb.dsmcc Package	59

P.2.1	DSMCCObject	60

P.2.2	DSMCCStream	60

P.2.2.1	isAudio() method	60

P.2.2.2	isData() method	60

P.2.2.3	isMPEGProgram() method	60

P.2.2.4	isVideo() method	60

P.2.3	DSMCCStreamEvent	60

P.2.4	InvalidFormatException	60

P.2.5	ServiceDomain	60

P.2.5.1	ServiceDomain.attach(byte[])	60

P.2.5.2	ServiceDomain.attach(Locator) and attach(Locator, int)	61

P.2.5.3	ServiceDomain.getLocator()	61

P.2.5.4	ServiceDomain.getNSAPAddress()	61

P.2.5.5	ServiceDomain.getURL(Locator)	61

P.2.6	ServiceXFRErrorEvent	61

P.2.7	ServiceXFRException	61

P.2.8	ServiceXFRReference	61

P.2.9	StreamEvent	62

Annex Q (normative):	Datagram Socket Buffer Control	63

Annex R (normative):	DVB-J Return Channel Connection Management API	64

Annex S (normative):	Application Listing and Launching	65

Annex T (normative):	Permissions	66

Annex U (normative): 	Extended graphics APIs	67

Annex V :	Void	68

Annex W (informative):	DVB-J examples	69

W.1	DVB-J examples from MHP	69

W.2	Example of enumeration extension	69

Annex X (normative):	Test support	71

Annex Y (normative):	Inter-application and Inter-Xlet communication API	72

Annex Z (informative):	Services, Service Contexts and Applications in an MHP Environment	73

Appendix A

# 1	Scope

This Recommendation is intended to harmonize the application environment for interactive TV applications. There is substantial potential for commonality in the Procedural Application Environment. Such commonality would benefit service providers through interoperability of procedural content and would provide economies of scale for the manufacture of consumer equipment.

This Recommendation is the first in a series of closely interrelated Recommendations on the harmonization of the applications environment. It must be used in conjunction with one dealing with the declarative part and one describing the overall architecture which defines the relationship and interfaces between the procedural and declarative parts. It is published separately to give potential users early information about the recommended approach which is stable and has widespread support in all three ITU Regions. It must be emphasized that in implementation the procedural and declarative aspects form parts of the integrated total application environment.

# 2	Analysis of procedural APIs

The following set of tables identify the opportunities for commonality between the various procedural approaches as well as solutions for the market-specific requirements, where necessary.

Common core APIs for ARIB B24, DASE-1, MHP 1.0.2, 
MHP 1.1, and OCAP 1.0

java.awt

java.awt.event

java.awt.image

java.beans

java.io

java.lang

java.lang.reflect

java.net

java.security

java.security.cert

java.util

java.util.zip

javax.media

javax.media.protocol

javax.tv.graphics

javax.tv.locator

javax.tv.media

javax.tv.media.protocol

javax.tv.net

javax.tv.service

javax.tv.service.guide

javax.tv.service.navigation

javax.tv.service.selection

javax.tv.service.transport

javax.tv.util

javax.tv.xlet

org.davic.media

org.davic.resources

org.havi.ui

org.havi.ui.event

Additional APIs common to ARIB B24, MHP 1.0.2, 
MHP 1.1, and OCAP 1.0

java.math

java.rmi

java.security.spec

javax.net

javax.net.ssl

javax.security.cert

org.davic.mpeg

org.davic.mpeg.sections

org.davic.net

org.davic.net.dvb

org.davic.net.tuning

org.dvb.application

org.dvb.dsmcc

org.dvb.event

org.dvb.io.ixc

org.dvb.io.persistent

org.dvb.lang

org.dvb.media

org.dvb.net

org.dvb.net.tuning

org.dvb.net.rc

org.dvb.test

org.dvb.ui

org.dvb.user

Specific additional APIs common to MHP 1.0.2 and MHP 1.1

org.davic.mpeg.dvb

org.davic.net.ca

org.dvb.net.ca

org.dvb.si

OCAP 1.0 specific additional APIs

org.ocap.application

org.ocap.event

org.ocap.hardware

org.ocap.hardware.pod

org.ocap.media

org.ocap.net

org.ocap.resource

org.ocap.service

org.ocap.system

org.ocap.system.error

org.ocap.ui.event

ARIB Bxx specific additional APIs

jp.or.arib.si

jp.or.arib.*

MHP 1.1 specific additional APIs

java.applet

java.awt.datatransfer

java.text

org.dvb.application.inner

org.dvb.application.plugins

org.dvb.application.storage

org.dvb.dom.bootstrap

org.dvb.dom.css

org.dvb.dom.dvbhtml

org.dvb.dom.environment

org.dvb.dom.event

org.dvb.dom.inner

org.dvb.internet

org.dvb.smartcard

org.w3c.dom

org.w3c.dom.css

org.w3c.dom.events

org.w3c.dom.html

org.w3c.dom.stylesheets

org.w3c.dom.views

DASE-1 specific additional APIs

com.sun.awt

com.sun.lang

java.text

java.util.jar

javax.tv.carousel

org.atsc.application

org.atsc.carousel

org.atsc.data

org.atsc.dom

org.atsc.dom.environment

org.atsc.dom.html

org.atsc.dom.views

org.atsc.graphics

org.atsc.management

org.atsc.net

org.atsc.preferences

org.atsc.registry

org.atsc.security

org.atsc.si

org.atsc.system

org.atsc.trigger

org.atsc.user

org.atsc.xlet

org.w3c.dom

org.w3c.dom.css

org.w3c.dom.events

org.w3c.dom.html

org.w3c.dom.stylesheets

org.w3c.dom.views

Appendix B

Migration from legacy platforms

Since there are significant numbers of digital terminal equipment with legacy platforms such as BML and MHEG-5 it is important to develop migration strategies so that advantage can be taken of the enhanced functionalities defined in Annex A without disenfranchizing the users.

For example, migration from MHEG-5 to MHP could be facilitated by the provision of a standard interoperable MHP plug-in. Migration would be further assisted by some modifications to the advanced system. For example, in the case of MHEG-5 migration to MHP, the process would be assisted by the following:

1)	Extension of the graphics APIs to support drawing lines thicker than 1 pixel for all primitives. This could be implemented by extending the underlying PersonalJava specification or by making DVB-specific extensions.

2)	Addition of 14:9 font support to the DVBTextLayoutManager. If this is not feasible, then it may be possible to use a defensive solution of 16:9 for all display types. This would distort the font (by compressing it horizontally) but would ensure the expected text flow.

However, the logical widths calculated when using this aspect ratio would be different, resulting in the line breaks being inserted at different points in the body of text. More importantly, anything other than very basic formatted text (relying on tabulation) would have a slim chance of being rendered correctly.

3)	Addition of VK_CANCEL to the set of minimum supported key events, or removal of the requirement for this in MHEG.

4)	Synchronization of the character repertoire to MHEG repertoire in UKProfile1.

5)	Provision of CI AppMMI extensions; the ability for an interoperable plug-in to register itself as a handler for specific application domains, and the ability to operate a data pipe to the source module.

__________