| Comparison table for Reference model and Monomedia (Work in Progress) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| References: |  | ARIB |  | STD-B24(Ver. 3.2), STD-B21(Ver.3.0), STD-B25(ver.3.0), STD-B10(Ver.3.2), TR-B15(Ver.2.0) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ATSC DASE-1 |  | CS/100-1, CS/100-2, CS/100-3, CS/100-4, CS/100-5, CS/100-6, CS/100-7, CS/100-8 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DVB MHP |  | TS 101812 (current:1.0).+. (draft 1.0.1).+. (soon 1.1, including DVB-HTML) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | OCAP |  | OpenCable Application Platform Specification, OCAP 1.0 Profile, OC-SP-OCAP1.0-I02-020405, OpenCable Application Platform Specification, OCAP 2.0 Profile, OC-SP-OCAP2.0-I01-020419 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MHEG-5 UK Profile 1 |  | Digital Terrestrial Television MHEG-5 Specification (1.05) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SMPTE DDE1（IEC62292） |  | SMPTE EG39, 363M, 357M |  |  |  |  |  |  |  |  |  |  |  |
|  |  | IEC 62298(TeleWEB) |  | IEC 62298-3, IEC 62298-4 |  |  |  |  |  |  |  |  |  |  |  |
| note: TD015 indicated as a reference in the CableLabs-OCAP column is out of date (Nov 2000, Beaverton Oregon) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Reference Model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ARIB | ATSC DASE-1 |  | DVB-MHP |  | CableLabs-OCAP |  | MHEG-5 UK Profile 1 | SMPTE DDE1 |  | IEC 62298-3 IEC 62298-4 |  |  |  |
| System Reference |  | see Vol.1, Part.1, Chap.4 of B24 (Digital satellite/terrestrial/cable broadcasting system with asymmetrical interaction channel.) | see section 1.2 of CS/100-1- (Any transport systems) |  | see section 5.1 of TS 101812 (applicable to all DVB defined transmission media and networks such as satellite, cable, terrestrial, and microwave. The application areas are Enhanced broadcasting, Interactive broadcasting and Internet access.) |  | * see section 7.2.1.1 of OCAP 1.0 Profile |  | (Used in the UK for Digital Terrestrial Services) | Transport independent. Uses IP Multicast and bindings defined for analog and digital networks. |  | IEC62298, HTML 3.2 with extensions |  |  |  |
| Protocol Stack |  | see Vol.1, Part.1, Chap.5 of B24 (Data carousel and Independent PES transmission schemes are specified.) | see CS/100-7 (ref. to A90. The ATSC document A90 specifies data carousel for asynchronous data, PES for syncronized streaming data and data piping scheme) |  | see section 6.2 and section 6.3 of TS 101812 (Object carousel with data carousel, DVB multiprotocol encapsulation and data piping) |  | see section 8 of OCAP 1.0 Profile |  | (DSMCC Object carousel) | Transport indpendent. Original use on UHTTP. |  | IEC62297, Data Carousel in VBI, PES or DSMCC sections |  |  |  |
| SI |  | ref. To ARIB-B10 | see T3-535r0 (ref. to A65) |  | ref. to ETS 300 468 2nd Edition |  | see Annex T of OCAP 1.0 Profile |  | ref. to ?? | Transport independent. |  | IEC62298-2, section 7 |  |  |  |
| Conditional Access |  | ref. To ARIB-B25 featuring IC card interface | none specified |  |  |  | CA external to OCAP (see http://www.opencable.com/specifications.html for various specifications) |  |  | Transport dependent. |  | Not specified |  |  |  |
| App->App Navigation |  | - SI is used to generate EPG. EPG applications depend on the receivers. Using the EPG application, Application can be selected. - Input of the service ID with remote controller results the selection of Application. | PA (procedural app) may indirectly cause another app to load by selecting a service containing an auto-launch app. DA (declarative app) may cause another app to load by performing hypertext navigation to app or by selecting a service containing an auto-launch app. |  |  |  | see section 10 and 11 in OCAP 1.0 Profile |  | MHEG-5 allows an application to be replaced temporarily or permanently by any other application on a filesystem that is available without changing service. | New document trees can be loaded programmatically. |  | Hyperlinks, History, Bookmarks |  |  |  |
| content/content Navigation |  | Hyperlink can be embedded in the declarative contents | Hyperlink navigation may occur in DA. |  |  |  | see section 15.2.1.5 in OCAP 1.0 profile |  | Programmable hyperlinks for navigation | Hyperlink navigations |  | Hyperlinks, History, Bookmarks |  |  |  |
| Broadcasting signalling |  | Event message scheme of MPEG-2 DSM-CC | application/dase-trigger specified in CS/100-1 |  |  |  | see section 10 and 11 in OCAP 1.0 Profile |  | Event message scheme of MPEG-2 DSM-CC | Triggers as defined in SMPTE 363. |  | Triggers as specified in IEC62297-2 |  |  |  |
| Locators |  | RFC 1738, RFC 2396, Extension of schemes such as arib: and arib-dc: | RFC 2396, RFC 2838 (tv:), SMPTE 343M (lid:). Extensions of "archive:" |  |  |  | see sections 15.2.1.1, 15.2.1.5, and annex G in OCAP 1.0 Profile |  | dvb:// for service selection, DSM:// for DSMCC object references | Tv:, lid: and http:, javascript: |  | RFC-2396, tw: ttx: nextview: |  |  |  |
| Decoder Model |  | see Vol.1, Part.1, Chap.6 of B24 (It features PE, modem and IC card interface) | see sections 4.3 and 5.2 of CS/100-1. |  | see section 5.2 and section 5.3 of TS 101812 |  | see section 7.2.1.5.1 in OCAP 1.0 profile |  | See section 6.7 of UK MHEG-5 specifications | See SMPTE 363M. |  | IEC62298-1, Section 4 |  |  |  |
| tuner |  | - Satellite (12GHz band) - Terrestrial (UHF) - Cable | Terrestrial (via CS/100-7) |  |  |  | QAM and QPSK for cable; DOCIS for OCAP 2.0 |  | Terrestrial | Transport independent. |  | Transport independent |  |  |  |
| Screen resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Logical coordinate and Display coordinate) |  | see Vol.1, Part.1, Chap.7 of B24 (Mapping is defined for both square pixel and non-square pixel) | see section 5.2..4 of CS/100-1 and figure 1 of CS/100-2. |  | see section 13 of TS 101812 |  | see section 14 of OCAP 1.0 Profile |  | Fixed 720x576 pels with selectable 4:3 or 16:9 aspect ratios | See HTML4 and CSS1. |  | Fixed at 640x480 |  |  |  |
| (Colorimetry) |  | see Vol.1, Part.1, Chap.7 of B24 (ITU-R BT709) | not defined (see section 5.2.5.4.4 of CS/100-1) |  | see section 7.5 of TS 101812 |  | see section 9 of OCAP 1.0 Profile |  | See section 4.3 of UK MHEG-5 specifications | See HTML4. |  | IEC62298-3, Annex B, default color palette. |  |  |  |
| (Composition of planes) |  | see Vol.1, Part.1, Chap.7 of B24 (It consists of video plane, still picture plane, swithing plane for video and still picture, text and graphics plane and subtitle plane) | see section 5.2 of CS/100-1 (It consists of Background plane for graphic or still picture, Video plane, Graphics plane and Cursor plane. Alpha blending of graphic and video is possible.) |  | see section 13 of TS 101812 |  | see section 14 of OCAP 1.0 Profile |  | Video or still picture plane behind graphics plane with alpha blending | See PNG alpha channel support. |  | IEC62298-1, Section 7 |  |  |  |
| Non-volatile storage |  | not specified (The size is specified as 65 kB in ARIB TR-B15) | not specified |  |  |  | minimum of 2KB for NVRAM |  | 1024 bytes shared between broadcast services | HTTP Cookies.(1KB for storage) |  | Size not specified |  |  |  |
| Volatile storage |  | not specified (The size is specified as 2 MB in ARIB B21) | application can specify required/requested size; see section 6.1.1.6.4.1.4 of CS/100-1 |  |  |  |  |  | See section 6.7 of UK MHEG-5 specifications (a minimum of 1MB is required) | 1MB content cache. |  | 5Mbyte for content storage |  |  |  |
| Network resources | ref. to ARIB B21 (specifies V.22bis as basic modem for PSTN and variety of connections such as ISDN. xDSL, cable modem, FTTH, mobile-phone and ethernet) |  | none required; none assumed |  |  |  |  |  | None required, hardware requirement specifies V.22bis? basic PSTN modem | Optional IP return channel. |  | IEC62298-4 has http and IP return channel |  |  |  |
| User interfaces | Remote control unit with “select”, “up”, “down”, “left”, “right” and other buttons |  | -Navigation device with “select”, “up”, “down”, “left”, “right” and other buttons |  | -A virtual keyboard or a physical keyboard | remote control unit and keyboard |  |  | Remote control unit with “select”, cursor and coloured keys | Navigation device (RC or KB) for selecting hyperlinks. |  | At least remote control with 4 arrow keys, 4 color keys and Ok key |  |  |  |
| Boot configuration | Not specified (depends on the receivers) |  | not specified |  |  |  |  |  | ? |  |  | Not specified |  |  |  |
| System Registry | Not specified (depends on the receivers) |  | not specified |  |  |  |  |  | not required |  |  | Not specified |  |  |  |
| 2.Monomedia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ARIB |  | ATSC DASE-1 |  | DVB | OCAP |  |  | MHEG-5 UK Profile 1 | SMPTE DDE1 |  | TeleWeb |  |  |  |
| Application Metadata | None |  | application/dase (XML based), specified by CS/100-1. |  | ? | AIT/XAIT |  |  | ? | See SDP record definition in SMPTE 357. |  | IEC62298-3, Section 12 |  |  |  |
| Video(streaming) | MPEG2 MP@HL, MP@ML 1920 x 1080 1280 x 720 720 x 480 352 x 240 |  | MPEG2 |  | MPEG2 | MPEG2 |  |  | MPEG2 | MPEG2 Format independent. ANALOG |  | PAL, MPEG-2 |  |  |  |
|  | MPEG1 352 x 240 176 x 120 |  |  |  |  |  |  |  |  | MPEG1 |  |  |  |  |  |
|  | MPEG4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Video(non-streaming) | MNG (limited colour depth) |  | MNG |  | MPEG-2Video “drips” | MPEG-2Video “drips” |  |  | Not required |  |  | Animated GIF |  |  |  |
| Still picture | JPEG MPEG2-I 1920 x 1080 1280 x 720 720 x 480 352 x 240 |  | JPEG |  | JPEG MPEG2-I | JPEG MPEG2-I |  |  | MPEG2-I | JPEG |  | JPEG |  |  |  |
| Graphics (broadcast profile, ‘Web profile’) | PNG (limited colour depth) |  | PNG |  | PNG [GIF optional] | PNG GIF (legacy) Animated GIF |  |  | PNG | PNG |  | GIF |  |  |  |
| Graphics resolution | 1920x1080 (16:9) 1280x720 (16:9) 940x540 (16:9) 720x480 (16:9, 4:3) |  | 640x480 (required). 1920x1080, 1280x720 and 960x540 (optional) |  |  | 640x480, 704x480, 1280x720 |  |  | 720 x 576 | Resolution independent. |  | 640x480 |  |  |  |
| Audio(streaming) | MPEG2 AAC MPEG2 BC MPEG4 |  | AC-3 |  | MPEG1 | MPEG1 AC3 |  |  | MPEG1 ? | Audio independent. |  | Not specified |  |  |  |
| Audio(non-streaming) | PCM (AIFF-C) |  | PCM (8bit mu-law) |  | MPEG1 | WAVE with MPEG-1 and MS PCM format, AC3 |  |  |  | PCM (8bit mu-law) |  | Not specified |  |  |  |
|  |  |  |  |  |  |  |  | MPEG1 |  |  |  |  |  |  |  |
| Plain Text | EUC-J, Shift-JIS, UTF-16 |  | none |  | UTF-8 | UTF-8, ISO 8859-1 |  | UTF-8 |  | 8859-1 |  | ISO-8859-1 |  |  |  |
| Font | DRCS |  | Portable Font Resource, ISO/IEC 16500-6 :1999 |  | Portable Font Resource defined by DAVIC (BitStream) | Portable Font Resource defined by DAVIC (BitStream) |  |  |  | Built in |  | IEC62298-3 Annex F Generic proportional or monospaced |  |  |  |
| Font Mapping | None |  |  |  | Font map file |  |  |  |  |  |  | Font map file |  |  |  |
| Archive content | (RFC2068:HTTP1.1 is supported) |  | ZIP (specified by T3-CS/100-5), and JAR (specified by CS/100-3 and Sun Manifest format). |  | ZIP files (optional) | ZIP files Packaged files |  |  |  | None |  | ZLIB |  |  |  |
| Trigger Content | (DSM-CC Event message) |  | Trigger (XML based), specified by CS/100-1.. |  | (DSM-CC Event message) | (DSM-CC Event message) |  | DSM-CC Event message |  | Trigger defined in SMPTE 363M. |  | IEC62297-1, URL based |  |  |  |
| 2-dimensional table data | CSV table, Binary table |  | none |  |  |  |  |  |  |  |  | IEC62298-3, Section 6.9 |  |  |  |
| External XML document | Specified (loadXMLdocument etc.) |  | none |  |  |  |  |  |  |  |  | Not specified |  |  |  |
| Colour look-up table | supported |  | none |  |  |  |  |  |  |  |  | IEC62298-3, Annex B |  |  |  |
| Signature | None |  | none |  | Signature, certificate, CRL | Signature, certificate, CRL |  |  |  | None |  | Not specified |  |  |  |
| Manifest | Resourse list |  | none |  | Hash files | Hash files |  |  |  | None |  | Not specified |  |  |  |
| Permission request | (Parental rating?) |  | Permission Request (XML based), specified by CS/100-6. |  | Permission request files | Permission request files |  |  |  | None |  | IEC62298-3, Parental rating Sections 9 and 12 |  |  |  |
| Subtitles | ARIB Subtitle |  | EIA-708 (closed caption) |  | DVB Subtitles Teletext defined in EN 300 472 | EIA-708 EIA-608 (legacy) |  | DVB Subtitles |  |  |  | IEC62298-3, IEC62297 |  |  |  |