#### Cambodia (country code +855)

##### Communication of 1.V.2006:

The Ministry of Posts and Telecommunications, Phnom-Penh, announces the following National Numbering Plan (NNP) for Cambodia:

###### Structure of the Numbering Plan

The numbering plan is based on a closed numbering scheme, with each subscriber having a fixed number of digits. The number consists of area codes or operator codes corresponding to the province or the operator and the subscriber number corresponding to individual subscriber numbers.

The numbering plan is structured in the following manner:

Number known internationally:	CC + NDC + SN
Number known nationwide:	NDC + SN
Number known locally (within an area):	SN

where:

CC:	Country Code (855 for Cambodia)
NDC:	National Destination Code or Area Code
SN:	Subscriber Number

Fixed PSTN Area Codes

For fixed PSTN (Public Switched Telephone Network) including wire-line and www networks, the following Area Codes apply.  When calling from a different NDC, subscribers need to dial “0” as the first digit before NDC.

| Province | Area Code |
| --- | --- |
| Reserved | 20 – 22 |
| Phnom-Penh | 23 |
| Kandal | 24 (Takhamau is considered as part of Phnom-Penh, i.e. 23) |
| Kampong Speu | 25 |
| Kampong Chhang | 26 |
| Reserved | 27 – 29 |
| Reserved | 30 – 31 |
| Takeo | 32 |
| Kampot | 33 |
| Sihanoukville | 34 |
| Koh kong | 35 |
| Kep Reserved Reserved | 36 37 – 39 40 – 41 |
| Kampong Cham | 42 |
| Prey Veng | 43 |
| Scvay Rieng Reserved Reserved | 44 45 – 59 50 – 51 |
| Pursat | 52 |
| Battambang | 53 |
| Banteay Meanchey | 54 |
| Pailin Reserved Reserved | 55 56 – 59 60 – 61 |
| Kampong Thom | 62 |
| Siem Reap | 63 |
| Preah Vihear | 64 |
| Udar Meanchey Reserved Reserved | 65 66 – 69 70 – 71 |
| Kratie | 72 |
| Mondul Kiri | 73 |
| Stung Treng | 74 |
| Ratanakiri Reserved Reserved | 75 76 – 79 80 – 89 |

Operator Assigned Numbers

There are currently three fixed PSTN operators, namely MPTC, Camshin and Camintel. The numbers allocated to the three operators are:

| Number block | Operator |
| --- | --- |
| 2XX XXX | MPTC |
| 3XX XXX | Camshin |
| 4XX XXX | MPTC |
| 5XX XXX | Reserved for future digit expansion |
| 6XX XXX | Reserved for future digit expansion |
| 7XX XXX | MPTC |
| 8XX XXX | MPTC |
| 9XX XXX | Camintel |

Mobile services

The numbering for mobile networks is as follows: 	1N + AXXXXX and
9N + AXXXXX

where:

1N and 9N:	as mobile operator code,

A:	can take digit between 2 and 9 and
X:	can take digit between 0 and 9.

Note:	9N can take 93 and 96 – 98 as shown below.

As of today the mobile operator access codes are:

| 010 | Reserved |
| --- | --- |
| 011 | Camshin (Shinawatra) DCS 1800 |
| 012 | CamGSM |
| 013 | Reserved |
| 014 | Reserved |
| 015 | Casacom (Samart) NMT 900 (it will be changed to 016 GSM 900) |
| 016 | Casacom (Samart) GSM 900 |
| 017 | Reserved |
| 018 | Camtel AMPS |
| 019 | S Telecom (South Korea) |
| 090 | Reserved |
| 091 | MPTC TLMRS (Trunked land mobile Radio) |
| 092 | Mobitel Tango Service |
| 093 | Reserved |
| 094 | Maritime Radio |
| 095 | National paging service |
| 096 | Reserved |
| 097 | Reserved |
| 098 | Reserved |
| 099 | Nationwide Limited TLMRS |

Codes Used for International Roaming

The following number is to be assigned by each GSM mobile operator to a foreign phone that has roamed to Cambodia:

1N 11XXXX (011 11XXXX, 012 11XXXX, 016 11XXXX, ...)
9N 11XXXX Reserved

where:

1N:	is the operator code specified above and
XXXX:	is the number assigned for each roamer phone; X can take digit 0 – 9.

This is to enable that phone to receive calls from foreign countries and for it to call other phones locally in Cambodia.

In accordance with GSM MoU a number of other codes are necessary to effect international roaming. Those codes are:

|  | Country initials | KHM |  |
| --- | --- | --- | --- |
|  | Mobile Country Code | 456 (as per ITU-T Rec. E.212) |  |
|  | Mobile Network Code | Mobitel | 01 |
|  |  | Samart | 02 |
|  | Reserved | S Telecom | 03 |
|  |  | Camshin | 18 |
|  | Network Colour Code | Mobitel | 5 |
|  |  | Samart | 6 |
|  |  | Camshin | 7 |
|  | Reserved | S Telecom | 8 |

Special service numbers

| 100 | Reserved |
| --- | --- |
| 101 | International test number for transmission |
| 102 | International test number for 1 KHz tone |
| 103 | International test number for loop back |
| 104 – 109 | Reserved |
| 110 – 116 | Reserved |
| 117 | Police |
| 118 | Fire Service |
| 119 | Hospital |
| 12XX | Telephone operator numbers |
| 13X | Short code for paging service number |
| 14XY | Operator short codes, where: X is unique for one operator and Y is assigned to operator themselves except that Y = 1 must be for General Enquiry and Y = 2 must be for Faults. The Xs are assigned as follows: |
| 140Y | Telecom Cambodia, e.g. (where Y = 0, 1, 2 …, 9) |
| 1400 1401 1402 1403 1404 | Assigned to Telecom Cambodia |
| 1405 – 1409 | Access codes assigned to BCC/VoIP for prepaid, post-paid and calling card services |
| 141Y | Camsbin |
| 142Y | Mobitel |
| 143Y | Spare |
| 144Y | S Telecom, reserved |
| 145Y | Samart |
| 146Y | Spare |
| 147Y | Spare |
| 148Y | Camtel |
| 149Y | Camintel |
| 150 | Dummy number for MPTC prepaid service |
| 151 – 154 | Reserved |
| 155 – 157 | MPTC prepaid service |
| 158 – 159 | Reserved |
| 1270 – 1279 1280 – 1289 1290 – 1299 | Reserved |

National

| 1200 | Directory assistance | Reserved |
| --- | --- | --- |
| 1212 | Call enquiries | Reserved |
| 1213 | Directory assistance | Reserved |
| 1214 | Fault and service difficulties | Reserved |
| 1215 | Making operator assisted calls | Reserved |
| 1216 | Operator connected conference calls | Reserved |
| 1217 | Wake-up and reminder calls | Reserved |

Paging Operator Codes

| Within a local area | 130 – 139 | Reserved |
| --- | --- | --- |
| Nationwide | 950 | Reserved |
| International | +855 950 | Reserved |

International Access Codes

| 001 | Gateway 1 (MPTC or Telecom Cambodia) |
| --- | --- |
| 002 – 006 | Reserved |
| 007 | Gateway 2 (Royal Telecam International) |
| 008 | VoIP Gateway |
| 009 | Reserved |

International Telephone Operator Service for Gateway 1

| 1200 |  |
| --- | --- |
| 1201 | Reserved |
| 1202 | Call enquiries |
| 1203 | Directory assistance |
| 1204 | Fault and service difficulties |
| 1205 | Making operator assisted calls |
| 1206 | Operator connected conference calls |
| 1207 – 1209 | Reserved |
| 1210 – 1211 | Reserved |
| 1212 – 1217 | Reserved |
| 1218 – 1219 | Reserved |
| 1220 – 1229 | Reserved |
| 1230 – 1239 | Reserved |
| 1240 – 1249 | Reserved |
| 1250 – 1259 | Reserved |
| 1260 – 1261 | Reserved |
| 1262 | Call enquiries (Reserved) |
| 1263 | Directory assistance (Reserved) |
| 1264 | Fault and service difficulties (Reserved) |
| 1265 | Making operator assisted calls (Reserved) |
| 1266 | Operator connected conference calls (Reserved) |
| 170 – 179 | Reserved |

Home country direct

| 1800 881 X1 X2 X3 X4 | Reserved |
| --- | --- |

To date the following home country direct numbers have been allocated:

| 1800 881 001 X | XATT & T USA |
| --- | --- |
| 1800 881 061 X | Australia |
| 1800 881 065 X | Singapore |
| 1800 881 033 X | France |

Free Call numbers (Free phone service)

| 1800 10XXXX | Reserved |
| --- | --- |
| 1800 11XXXX | Reserved for Camshin |
| 1800 12XXXX | Reserved for Cam GSM |
| 1800 13XXXX | Reserved |
| 1800 14XXXX | Reserved |
| 1800 15XXXX | Reserved for Casacom |
| 1800 16XXXX | Reserved |
| 1800 17XXXX | Reserved |
| 1800 18XXXX | Reserved for Camtel |
| 1800 19XXXX | Reserved for S Telecom |
| 1800 20XXXX | Reserved for MPTC |
| 1800 29XXXX | Reserved for Camintel |

Charged call numbers

| 1900 10XXXX | Reserved |
| --- | --- |
| 1900 11XXXX | Reserved for Camshin |
| 1900 12XXXX | Reserved for Cam GSM |
| 1900 13XXXX | Reserved |
| 1900 14XXXX | Reserved |
| 1900 15XXXX | Reserved for Casacom |
| 1900 16XXXX | Reserved |
| 1900 17XXXX | Reserved |
| 1900 18XXXX | Reserved for Camtel |
| 1900 19XXXX | Reserved for S Telecom |
| 1900 20XXXX | Reserved for MPTC |
| 1900 29XXXX | Reserved for Camintel |

Contact:

Mr Sok Nareth
Regulations Directorate
Ministry of Posts and Telecommunications 
Corner of Str. 13 and 102
PHNOM-PENH
Cambodia
Fax:	+855 23 428 986 / +855 23 426 011
URL:	www.mptc.gov.kh / numbering_plan.html