#### Slovenia (country code +386)

##### Communication of 22.II.2006:

The Post and Electronic Communications Agency of the Republic of Slovenia, Ljubljana, announces that a new telephone numbering plan has been introduced in Slovenia, effective since 30 June 2005.

The structure of the international telephone numbers is: 		+386 (0) ABCDEFGH

Country Code (CC):		+386

National (Significant) Number (N(S)N):		AB CD EF GH

Maximum length of the National (Significant) Number (N(S)N):	eight digits

National prefix:		0

International prefix:		00

The structure of National (Significant) Numbers (N(S)N) can vary as follows:

| National Destination Code (NDC) | Subscriber Number (SN) |
| --- | --- |
| A | BCD EF GH |
| AB | CD EF GH |
| ABC | DE FGH |
| ABCD | E FGH |

–	Geographical services

The National Destination Codes (NDC) in geographical numbering ranges are:

| Geographical area | National Destination Code (NDC) (Area (trunk) code) A |
| --- | --- |
| Ljubljana | 1 |
| Maribor, Ravne na Koroskem, Murska Sobota | 2 |
| Celje, Trbovlje | 3 |
| Kranj | 4 |
| Nova Gorica, Koper, Postojna | 5 |
| Novo mesto, Krsko | 7 |

The length of the Subscriber Number (SN) in geographical numbering ranges is seven (7) digits and the format of the National (Significant) Number (N(S)N) is the following: A XXXXXXX.

–	Non-geographical services

The National Destination Codes (NDC) for public mobile telecommunication networks are:

AB  30, 31, 40, 41, 51, 64, 65, 66, 67, 68, 69, 70 and 71. Their length can be two (2), three 	(3) or four (4) digits (AB, ABC or ABCD).

The length of the Subscriber Number (SN) is therefore six (6), five (5) or four (4) digits and National (Significant) Numbers (N(S)N) have the following formats:

AB XXXXXX,
	ABC XXXXX,
	ABCD XXXX.

–	Other numbering ranges

| 80CDEF(GH) | Freephone services (six- or eight-digit length) |
| --- | --- |
| 90CDEF(GH) | Premium rate services (six- or eight-digit length) |
| 88CDEFGH | Access to special networks (data, internet, etc.) |
| 89CDE(F)(G)(H) C = 1,2,3 | Access to services like mass calling, televoting, etc. |
| 49CDEFGH 59CDEFGH 81CDEFGH 82CDEFGH 83CDEFGH | Non-geographical numbering ranges for PATS on fixed location (e.g.: Voice telephony over IP (VoIP)) – see Note 2) National (Significant) Numbers (N(S)N) in these ranges may have one of the following formats NDC + SN: AB XXXXXX, ABC XXXXX, ABCD XXXX |
| 10PR | Carrier selection codes |
| 11P(Q(R)) 19P(Q(R)) | Short codes |
| 98MN | Routing codes |
| 6BCDEFGH B = 1,2,3 | Reserved numbering ranges for public mobile telecommunication networks |
| 20CDEFGH 21CDEFGH 29CDEFGH 39CDEFGH 50CDEFGH 60CDEFGH 79CDEFGH 8BCDEFGH B = 4,5,6,7 89CDEFGH C=0,4,5,6,7,8,9 9BCDEFGH B=1,2,3,4,5,6,7,9 | Reserved numbering ranges for non-geographic numbers |

Note 1:  Mobile network

The following National Destination Codes (NDC) have been allocated thus far to the following mobile operators in Slovenia:

| National Destination Code AB | Operator of network | Service |
| --- | --- | --- |
| 30 | SI.MOBIL d.d. | Mobile telephone service (GSM) |
| 31 | MOBITEL d.d. | Mobile telephone service (GSM) |
| 40 | SI.MOBIL d.d. | Mobile telephone service (GSM) |
| 41 | MOBITEL d.d. | Mobile telephone service (GSM) |
| 51 | MOBITEL d.d. | Mobile telephone service (GSM and UMTS) |
| 70 | Western Wireless International d.o.o. | Mobile telephone service (GSM) |
| 71 | MOBITEL d.d. | Mobile telephone service (GSM and UMTS) |

Note 2:  VoIP

The following National Destination Codes (NDC) in non-geographical numbering ranges have been allocated thus far to operators in Slovenia which ensure Public Available Telephone Services (PATS) on the fixed network (e.g. Voice telephony over IP (VoIP)):

| National Destination Code (NDC) (=AB(C)(D)) + SN | Operator |
| --- | --- |
| 590 XXXXX | T -2 d.o.o. |
| 592 XXXXX | IN.LIFE d.d. |
| 595 XXXXX | KRS ROTOVŽ d.d. |
| 596 XXXXX | MEDINET d.o.o. |
| 597 XXXXX | VOLJATEL d.d. |
| 598 XXXXX | PERFTECH d.o.o. |
| 599 XXXXX | Slovenija Online – SiOl d.o.o. |
| 816 XXXXX | SOFTNET d.o.o. |
| 822 XXXXX | SINFONIKA d.o.o. |

Note 3:  Geographical numbers

The following blocks of geographical numbers have been allocated thus far to operators of Public Available Telephone Services (PATS):

| Geographical area | National Destination Code (NDC) (Area (trunk) code) A | Numbering blocks | Operator |
| --- | --- | --- | --- |
| Ljubljana | 1 | 320XXXX 620XXXX 400XXXX 880XXXX 600XXXX (Note*) | MEDINET d.o.o. MEDINET d.o.o. SINFONIKA d.o.o. STELKOM d.o.o. VOLJATEL d.d. TELEKOM SLOVENIJE d.d. |
| Maribor, Ravne na Koroskem, Murska Sobota | 2 | 620XXXX 880XXXX 600XXXX (Note*) | MEDINET d.o.o. STELKOM d.o.o. VOLJATEL d.d. TELEKOM SLOVENIJE d.d. |
| Celje, Trbovlje | 3 | 620XXXX 880XXXX 600XXXX (Note*) | MEDINET d.o.o. STELKOM d.o.o. VOLJATEL d.d. TELEKOM SLOVENIJE d.d. |
| Kranj | 4 | 620XXXX 880XXXX 600XXXX (Note*) | MEDINET d.o.o. STELKOM d.o.o. VOLJATEL d.d. TELEKOM SLOVENIJE d.d. |
| Nova Gorica, Koper, Postojna | 5 | 620XXXX 880XXXX 600XXXX (Note*) | MEDINET d.o.o. STELKOM d.o.o. VOLJATEL d.d. TELEKOM SLOVENIJE d.d. |
| Novo mesto, Krsko | 7 | 620XXXX 880XXXX 600XXXX (Note*) | MEDINET d.o.o. STELKOM d.o.o. VOLJATEL d.d. TELEKOM SLOVENIJE d.d. |

*	Note: The incumbent operator of Public Available Telephone Services (PATS) in Slovenia TELEKOM SLOVENIJE d.d. has numerous blocks of geographical numbers in all six geographical areas.

Contact:

Post and Electronic Communications Agency of the Republic of Slovenia
Stegne 7, POB 418
1001 LJUBLJANA
Slovenia
Tel:	+386 1 583 63 00
Fax:	+386 1 511 11 01
E-mail:	info.box@apek.si
URL:	www.apek.si