| Committee 5 has finished its consideration of agenda items 1.12 and 1.14. As a result of these deliberations, it has unanimously adopted, at its second meeting, the attached text that is submitted for your consideration with a view to its subsequent submission to the Plenary. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 1 to Document 224-E |
|  |  |  | 29 May 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
| Source: Document 227 |  |  | COMMITTEE 6 |
| FIRST SERIES OF TEXTS SUBMITTED BY COMMITTEE 5 TO THE EDITORIAL COMMITTEE |  |  |  |

Chris Van DIEPENBEEK
	Chairperson, Committee 5

ADD

RESOLUTION  [COM5/1]  (WRC-2000)

Use of the frequency band 35.5-35.6 GHz by spaceborne precipitation radar

The World Radiocommunication Conference (Istanbul, 2000),

considering

a)	that the frequency band 35.5-36.0 GHz is allocated to the Earth exploration-satellite (active) service as a primary service with footnote S5.551A and allocated to meteorological aids and radiolocation services on a primary basis;

b)	that before WRC-97, operation on a primary basis of radars located on spacecraft was allowed in the band 35.5-35.6 GHz;

c)	that performance criteria and interference criteria for spaceborne precipitation radar within 35.5-36.0 GHz is included in Recommendation ITU-R SA.1166-2;

d)	that spaceborne precipitation radar is very important to measure rain rate globally and study global water circulation;

e)	that minimum observable rain rate around 35 GHz is less than 0.2 mm/h;

f)	that combination between part of 35.5-35.6 GHz and 13.4-13.75 GHz is very useful to measure rain rate precisely;

g)	that studies have shown that sharing between spaceborne active sensors and radiolocation systems in 35.5-36 GHz is feasible as shown in section 5.7.2.1 of Chapter 5 of the CPM-97 Report,

resolves

1	to invite ITU-R to study sharing between the spaceborne precipitation radar and other services in 35.5-35.6 GHz;

2	that WRC-02 would review the results of these studies and consider exclusion of footnote S5.551A in 35.5-35.6 GHz used for spaceborne precipitation radar.

__________