| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 273-E |
|  |  |  | 18 May 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
|  |  |  | WORKING GROUP 5B |
| Sub-Working Group 5B-2 |  |  |  |

SPACE-TO-SPACE ALLOCATION FOR THE RADIONAVIGATION-SATELLITE SERVICE

(AGENDA ITEM 1.15.2)

Sub-Working Group 5B-2 has prepared the attached text concerning space-to-space allocation to the radionavigation-satellite service and is submitting it to Working Group 5B for consideration and approval.

Takeshi MIZUIKE
	Chairperson, Sub-Working Group 5B-2
	Box 132

MOD

890-1 350 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 215-1 240 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) S5.329 ADD S5.329A SPACE RESEARCH (active) S5.330 S5.331 S5.332 |  |  |
| 1 240-1 260 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) S5.329 ADD S5.329A SPACE RESEARCH (active) Amateur S5.330 S5.331 S5.332 S5.334 S5.335 |  |  |

MOD

1 525-1 610 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 559-1 610 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) ADD S5.329A S5.341 S5.355 S5.359 S5.363 |  |  |

ADD

S5.329A	[The radionavigation-satellite service (space-to-space) in frequency bands 1 215-1 260 MHz and 1 559-1 610 MHz is not considered a safety service as defined in No. S9.59 and the provisions of No. S5.10 do not apply.] Use of systems in the radionavigation-satellite service (space-to-space) operating in the bands 1 215-1 260 MHz and 1 559-1 610 MHz shall not impose any additional constraints on other systems or services operating in accordance with the Table of Frequency Allocations.

______________