| RESOLUTION [com5/6] (WRC-2000) INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 376-E |
|  |  |  | 23 May 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
|  |  |  | COMMITTEE 5 |
| Working Group 5D |  |  |  |

Protection of GSO FSS and GSO BSS networks from the maximum aggregate equivalent power flux-density produced by multiple non-GSO FSS systems in frequency bands where epfd limits have been adopted

The World Radiocommunication Conference (Istanbul, 2000),

considering

a)	that WRC-97 has adopted, in Article S22, provisional epfd limits to be met by non-GSO FSS systems in order to protect GSO FSS and GSO BSS networks in parts of the frequency range 10.7-30 GHz;

b)	that WRC-2000 has revised Article S22 to ensure the limits contained therein provide adequate protection to GSO systems without causing undue constraints to any of the systems and services sharing these frequency bands;

c)	that WRC-2000 decided that a combination of single-entry validation, single-entry operational and for certain antenna sizes single-entry additional operational epfd limits, which are included in Article S22, along with the aggregate limits in Tables COM5/6-1A through COM5/6-1D, which apply to non-GSO FSS systems protect GSO networks in these bands;

d)	that these single-entry validation limits have been derived from aggregate equivalent power flux-density (epfd) masks contained in Tables COM5/6-1A through COM5/6-1D, assuming a maximum effective number of non-GSO FSS systems of 3.5;

e)	that the aggregate interference caused by all co-frequency non-GSO FSS systems in these bands into GSO FSS systems should not exceed the aggregate epfd levels in Tables COM5/6-1A through COM5/6-1D;

f)	that WRC-97 decided, and WRC-2000 confirmed, that non-GSO FSS systems in these bands are to coordinate the use of these frequencies between themselves under the provisions of No. [S9.10] of the Radio Regulations;

g)	that the orbital characteristics of such systems are likely to be inhomogeneous;

h)	that as a result of this likely inhomogeneity, the aggregate epfd levels from multiple non-GSO FSS systems are not directly related to the number of actual systems sharing a frequency band, and the number of such systems operating co-frequency is likely to be small;

i)	that the possible misapplication of single-entry limits should be avoided,

recognizing

a)	that non-GSO FSS systems are likely to need to implement interference mitigation techniques to share frequencies among themselves;

b)	that because the use of such interference mitigation techniques will likely keep the number of non-GSO systems small, the aggregate interference caused by non-GSO FSS systems into GSO systems will also likely be small;

c)	that notwithstanding considering d), considering e) and recognizing b), there may be instances where the aggregate interference from non-GSO systems could exceed the interference levels given in Tables COM5/6-1A through COM5/6-1D;

d)	that administrations operating GSO systems may wish to ensure that the aggregate epfd produced by all operating co-frequency non-GSO FSS systems in the frequency bands referred to in considering a) above into GSO FSS and/or GSO BSS networks does not exceed the aggregate interference levels given in Tables COM5/6-1A through COM5/6-1D,

resolves

1	that administrations operating or planning to operate non-GSO FSS systems, for which coordination or notification information, as appropriate, was received after 21 November 1997, in the frequency bands referred to in considering a) above, individually or in collaboration, take all possible steps, including by means of appropriate modifications to their systems if necessary, to ensure that the aggregate interference into GSO FSS and GSO BSS networks caused by such systems operating co-frequency in these frequency bands does not cause the aggregate power levels shown in Tables COM5/6-1A through COM5/6-1D to be exceeded [(see No. S22.5K)];

2	that, in the event that the aggregate interference levels in Tables COM5/6-1A through COM5/6-1D are exceeded, administrations operating non-GSO FSS systems in these frequency bands shall expeditiously take all necessary measures to reduce the aggregate epfd levels to those in Tables COM5/6-1A through COM5/6-1D or to reduce such interference to levels that are acceptable to the affected GSO administration [(see No. S22.5K)],

requests ITU-R

1	to develop, as a matter of urgency, and complete, in time for consideration by the next WRC, a suitable methodology for calculating the aggregate epfd produced by all non-GSO FSS systems operating or planning to operate co-frequency in the frequency bands referred to in considering a) above into GSO FSS and GSO BSS networks, which may be used to determine whether the systems are in compliance with the aggregate power levels shown in Tables COM5/6-1A through COM5/6-1D;

2	to continue its studies and to develop, as a matter of urgency, a recommendation on the accurate modelling of interference from non-GSO FSS systems into GSO FSS and GSO BSS networks in the frequency bands referred to in considering a) above in order to assist the administrations planning or operating non-GSO FSS systems in their efforts to limit the aggregate epfd levels produced by their systems into GSO networks and to provide guidance to GSO network designers on the maximum epfddown levels expected to be produced by all non-GSO FSS systems when accurate modelling assumptions are used;

3	to develop a recommendation, as a matter of urgency, that contains procedures to be used amongst administrations to ensure that the aggregate epfd limits contained in Tables COM5/6-1A through COM5/6-1D are not exceeded by operators of non-GSO FSS systems;

4	to attempt to develop measurement techniques to identify the interference levels from non-GSO systems in excess of the “aggregate” limits given in Tables COM5/6-1A through COM5/6-1D of this Resolution, and to confirm compliance with these limits,

requests the Director of the Radiocommunication Bureau

1	to assist in the development of the methodology referred to in requests ITU-R 1 above;

2	to report to WRC-02/03 on the results of studies in requests ITU-R 1 and 3 above.

ANNEX 1 (to Resolution [COM5/6] (WRC-2000))

TABLE COM5/6-1A1, 3, 4

Limits to the aggregate epfddown radiated by non-GSO FSS systems in 
certain frequency bands

| Frequency band (GHz) | epfddown dB(W/m2) | Percentage of time during which epfddown may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter, and reference radiation pattern2 |
| --- | --- | --- | --- | --- |
|  | –170.0 –168.6 –165.3 –160.4 –160.0 –160.0 | 0 90 99 99.97 99.99 100 | 40 | 60 cm Recommendation ITU-R S.1428 |
| 10.7-11.7 in all Regions 11.7-12.2 in Region 2 12.2-12.5 in Region 3 and 12.5-12.75 in Regions 1 and 3 | –176.5 –173.0 –164.0 –161.6 –161.4 –160.8 –160.5 –160 –160 | 0 99.5 99.84 99.945 99.97 99.99 99.99 99.9975 100 | 40 | 1.2 m Recommendation ITU-R S.1428 |
|  | –185 –184 –182 –168 –164 –162 –160 –160 | 0 90 99.5 99.9 99.96 99.982 99.997 100.00 | 40 | 3 m3bis Recommendation ITU-R S.1428 |
|  | –190 –190 –166 –160 –160 | 0 99 99.99 99.998 100 | 40 | 10 m3bis Recommendation ITU-R S.1428 |

For certain GSO FSS receive earth stations, see also ADD S9.7A and ADD S9.7B.

2 	Under this section, reference patterns are to be used only for the calculation of interference from non-GSO FSS systems into GSO FSS systems.

3 	In addition to the limits shown in Table COM5/6-1A, the following aggregate epfddown limits apply to all antenna sizes greater than 60 cm in the frequency bands listed in Table COM5/6-1A.

| 100% of the time epfddown dB(W/(m2·40 kHz)) | Latitude (North or South) (°) |
| --- | --- |
| –160 | 0 < ¦Latitude¦  57.5 |
| –160 + 3.4(57.5 – ¦Latitude¦)/4 | 57.5 < ¦Latitude¦  63.75 |
| –165.3 | 63.75  ¦Latitude¦ |

3bis	The values for the 3 metre and 10 metre antennas are applicable only for the aggregation methodologies referred to requests ITU-R 1.

4 	For each reference antenna diameter, the limit consists of the complete curve on a plot which is linear in decibels for the epfd levels and logarithmic for the time percentages, with straight lines joining the data points.

MOD

TABLE COM5/6-1B1, 3

Limits to the aggregate epfddown radiated by non-GSO FSS systems in 
certain frequency bands

| Frequency band (GHz) | epfddown dB(W/m2) | Percentage of time during which epfddown may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter, and reference radiation pattern2 |
| --- | --- | --- | --- | --- |
| 17.8-18.6 | –170 –170 –164 –164 | 0 90 99.9 100 | 40 | 1 m Recommendation ITU-R S.1428 |
|  | –156 –156 –150 –150 | 0 90 99.9 100 | 1 000 |  |
| 17.8-18.6 | –173 –173 –166 –164 –164 | 0 99.4 99.9 99.92 100 | 40 | 2 m Recommendation ITU-R S.1428 |
|  | –159 –159 –152 –150 –150 | 0 99.4 99.9 99.92 100 | 1 000 |  |
| 17.8-18.6 | –180 –180 –172 –164 –164 | 0 99.8 99.8 99.992 100 | 40 | 5 m Recommendation ITU-R S.1428 |
|  | –166 –166 –158 –150 –150 | 0 99.8 99.8 99.992 100 | 1 000 |  |

For certain GSO FSS receive earth stations, see also ADD S9.7A and ADD S9.7B.

2 	Under this section, reference patterns are to be used only for the calculation of interference from non-GSO FSS systems into GSO FSS systems.

3 	For each reference antenna diameter, the limit consists of the complete curve on a plot which is linear in decibels for the epfd levels and logarithmic for the time percentages, with straight lines joining the data points.

3bis	A non-GSO system shall meet the limits of this table in both the 40 kHz and the 1 MHz reference bandwidth.

MOD

TABLE COM5/6-1C1, 3

Limits to the aggregate epfddown radiated by non-GSO FSS systems in 
certain frequency bands

| Frequency band (GHz) | epfddown dB(W/m2) | Percentage of time during which epfddown may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter, and reference radiation pattern2 |
| --- | --- | --- | --- | --- |
| 19.7-20.2 | –182 –172 –154 –154 | 0 90 99.94 100 | 40 | 70 cm Recommendation ITU-R S.1428 |
|  | –168 –158 –140 –140 | 0 90 99.94 100 | 1 000 |  |
| 19.7-20.2 | –185 –176 –165 –160 –154 –154 | 0 91 99.8 99.8 99.99 100 | 40 | 90 cm Recommendation ITU-R S.1428 |
|  | –171 –162 –151 –146 –140 –140 | 0 91 99.8 99.8 99.99 100 | 1 000 |  |
| 19.7-20.2 | –191 –162 –154 –154 | 0 99.933 99.998 100 | 40 | 2.5 m Recommendation ITU-R S.1428 |
|  | –177 –148 –140 –140 | 0 99.933 99.998 100 | 1 000 |  |
| 19.7-20.2 | –195 –184 –175 –161 –154 –154 | 0 90 99.6 99.984 99.9992 100 | 40 | 5 m Recommendation ITU-R S.1428 |
|  | –181 –170 –161 –147 –140 –140 | 0 90 99.6 99.984 99.9992 100 | 1 000 |  |

For certain GSO FSS receive earth stations, see also ADD S9.7A and ADD S9.7B.

2 	Under this section, reference patterns are to be used only for the calculation of interference from non-GSO FSS systems into GSO FSS systems.

3 	For each reference antenna diameter, the limit consists of the complete curve on a plot which is linear in decibels for the epfd levels and logarithmic for the time percentages, with straight lines joining the data points.

3bis	A non-GSO system shall meet the limits of this table in both the 40 kHz and the 1 MHz reference bandwidth.

TABLE COM5/6-1D2, 3

Limits to the aggregate epfddown radiated by non-GSO FSS 
systems in certain frequency bands 30 cm, 45 cm, 60 cm, 
90 cm, 120 cm, 180 cm, 240 cm and 300 cm BSS antennas

| Frequency band (GHz) | epfddown dB(W/m2) | Percentage of time during which epfddown level may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter, and reference radiation pattern1 |
| --- | --- | --- | --- | --- |
| 11.7- 12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz in Region 2 | –160.4 –160.1 –158.6 –158.6 –158.33 –158.33 | 0 25 96 98 98 100 | 40 | 30 cm Recommendation ITU-R BO.1443 Annex 1 |
| 11.7-12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz in Region 2 | –170 –167 –164 –160.75 –160 –160 | 0 66 97.75 99.33 99.95 100 | 40 | 45 cm Recommendation ITU-R BO.1443 Annex 1 |
| 11.7-12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz in Region 2 | –171 –168.75 –167.75 –162 –161 –160.2 –160 –160 | 0 90 97.8 99.6 99.8 99.9 99.99 100 | 40 | 60 cm Recommendation ITU-R BO.1443 Annex 1 |
| 11.7-12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz in Region 2 | –173.75 –173 –171 –165.5 –163 –161 –160 –160 | 0 33 98 99.1 99.5 99.8 99.97 100 | 40 | 90 cm Recommendation ITU-R BO.1443 Annex 1 |
| 11.7-12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz In Region 2 | –177 –175.25 –173.75 –173 –169.5 –167.8 –164 –161.9 –161 –160.4 –160 | 0 90 98.9 98.9 99.5 99.7 99.82 99.9 99.965 99.993 100 | 40 | 120 cm Recommendation ITU-R BO.1443 Annex 1 |

| 11.7-12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz in Region 2 | –179.5 –178.66 –176.25 –163.25 –161.5 –160.35 –160 –160 | 0 33 98.5 99.81 99.91 99.975 99.995 100 | 40 | 180 cm Recommendation ITU-R BO.1443 Annex 1 |
| --- | --- | --- | --- | --- |
| 11.7-12.5 GHz in Region 1 11.7-12.2 GHz and 12.5-12.75 GHz in Region 3 12.2-12.7 GHz in Region 2 | –182 –180.9 –178 –164.4 –161.9 –160.5 –160 –160 | 0 33 99.25 99.85 99.94 99.98 99.995 100 | 40 | 240 cm Recommendation ITU-R BO.1443 Annex 1 |
| 11.7-12.5 GHz In Region 1 11.7-12.2 GHz and 12.5-12.75 GHz In Region 3 12.2-12.7 GHz In Region 2 | –186.5 –184 –180.5 –173 –167 –162 –160 –160 | 0 33 99.5 99.7 99.83 99.94 99.97 100 | 40 | 300 cm Recommendation ITU-R BO.1443 Annex 1 |

1	Under this section, reference patterns are to be used only for the calculation of interference from non-GSO FSS systems into GSO BSS systems.

2	For BSS antenna diameters 180 cm, 240 cm and 300 cm, in addition to the aggregate limit shown in Table COM5/6-1D, the following aggregate 100% of the time epfddown limit also applies:

| 100% of the time epfddown dB(W/(m2·40 kHz)) | Latitude (North or South) (°) |
| --- | --- |
| –160 | 0  Latitude  57.5 |
| –160 + 3.4 * (57.5 – Latitude)/4 | 57.5  Latitude  63.75 |
| –165.3 | 63.75  Latitude |

3	For each reference antenna diameter, the limit consists of the complete curve on a plot which is linear in decibels for the epfd levels and logarithmic for the time percentages, with straight lines joining the data points.

For BSS antenna diameter 240 cm, in addition to the above aggregate 100% of the time epfddown limit, a –167 dB(W/(m2 · 40 kHz)) aggregate 100% of the time operational epfddown limit also applies to receive antennas located in Region 2, west of 140° W, north of 60° N, pointing toward GSO BSS satellites at 91° W, 101° W, 110° W, 119° W and 148° W with elevation angles greater than 5°. [This limit is implemented during a transition period of 15 years.]*

________________

*	 	Comment - This transitional regime would be applicable only if the pfd limits in section 5c of Annex 1 to Appendix S30 are sufficiently relaxed.