| Please find the attached texts proposed to replace section 1 of Annex 1 of Appendix S30 and section 4 of Annex 1 of Appendix S30A appearing currently in square brackets in Document 493 (source: Document DT/116) submitted to the Editorial Committee. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 497-E |
|  |  |  | 31 May 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
|  |  |  | PLENARY MEETING |
| Note from the Chairperson of GT PLEN-1 |  |  |  |

R. ZEITOUN
	Chairperson, GT PLEN-1, Box 27

attachment

Proposed criteria for protection of the Regions 1 and 3 Plan or List
or assignments previously proposed for inclusion in the List

annex 1 to appendix s30

MOD

# 1	Limits for the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or into new or modified assignments in the Regions 1 and 3 List

NOTE – In performing the calculation, the effect at the receiver input of all the co-channel and adjacent-channel signals is expressed in terms of one equivalent co-channel interfering signal. This value is usually expressed in decibels.

ADD

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the List shall not exceed the value of –103.6 dB(W/m2/27 MHz).

With respect to paragraphs 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 shall be considered as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst case station keeping conditions, is less than 9°. However, an administration shall not be considered as affected if either of the following two conditions are met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values:14bis

–147 dB(W/m2/27 MHz)	for  0°        0.245°

–134.8 + 20 log  dB(W/m2/27 MHz)	for 0.245°      °

–135 + 1.66  dB(W/m2/27 MHz)	for 1.7°      °

–127.5 + 25 log dB(W/m2/27 MHz)	for 3.6°      °;

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin13bis corresponding to a test point of its entry in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 of this Appendix has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as the result of successful application of Article 4 procedures.

annex 1 to appendix s30A

MOD

# 4	Limits to the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or proposed new or modified assignments in the Regions 1 and 3 List

ADD

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the List shall not exceed the value of –76 dB(W/m2/27 MHz) at any point in the GSO orbit, and the relative off-axis e.i.r.p. of the associated feeder-link antenna shall be in compliance with Figure A (WRC-97 curves) of Annex 3 to this Appendix.

With respect to paragraphs 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 shall be considered as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst-case station-keeping conditions, is less than 9°.

However, an administration shall not be considered as affected if, under assumed free-space propagation conditions, the effect of the proposed new or modified assignments in the List is that the feeder-link equivalent protection margin13 corresponding to a test point of its entry in the Plan and the List or for which the procedure of Article 4 of this Appendix has been initiated, including the cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB, or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as the result of the successful application of Article 4 procedures.

For a proposed new or modified assignment to the List, in the interference analysis, for each test point, the antenna characteristics described in section 3.5 of Annex 3 to this Appendix shall apply.

14bis For the protection of analogue assignments brought in service before 17 October 1997, the following values shall be used and until 1 January 2015:

–147 dB(W/m2/27 MHz)	for  0°        0.44°

–138    25 log  dB(W/m2/27 MHz)	for  0.44°        9°.

13bis For the definition of the equivalent protection margin, see § 3.4 of Annex 5.

13 	For the definition of the equivalent protection margin, see § 1.7 of Annex 3.