| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 377-E |
|  |  |  | 1 July 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Report of ad hoc Group 6 of the Plenary |  |
| Recommendation 66 and agenda items 1.8.1 and 1.8.2 |  |

Ad hoc Group 6 of the Plenary met during the afternoon of 1 July to discuss the necessary modifications of Recommendation 66 (Rev.WRC-2000) with regard to possible actions at this Conference on agenda items 1.8.1 and 1.8.2.

A modified version of Recommendation 66 has been reproduced that reflects the current status of recommends that ITU-R in the Recommendation 66. recommends 1 through 4 and 7 may be suppressed in the Recommendation barred unforeseen actions on agenda item 1.8.1. Original recommends 5 and 6 have been placed in brackets because of the uncertainty of the outcome on agenda item 1.8.2. If the final results in the Conference on agenda item 1.8.2 request further action at WRC-07, then there is general support to drop these two recommends. Finally old recommends 8 would represent a broad agenda item for [WRC-07] that may be dropped as consequential action after suppression of recommends 5 and 6. The considering and noting of Recommendation 66 have not been considered in detail and are placed in square brackets awaiting the outcome on agenda item 1.8.2. In the text also all references to safety services have been removed.

It may be concluded that Recommendation 66 may be suppressed as a whole, if the final actions on agenda item 1.8.2 contain further action at WRC-07 for the EESS and the RAS. If this is not the case then the discussion on Recommendation 66 needs to be reopened.

Willem A. Baan
	Chairman, ad hoc Group 6 of the Plenary, Box 315

MOD

RECOMMENDATION  66  (Rev.WRC-03)

Studies of the maximum permitted levels of unwanted emissions

The World Radiocommunication Conference (Geneva, 2003),

considering

[a)	that Appendix 3 specifies the maximum permitted levels of spurious emissions, in terms of the mean power level of any spurious component supplied by a transmitter to the antenna transmission line;

b)	that the principal objective of Appendix 3 is to specify the maximum permitted levels of spurious emissions that, while being achievable, provide protection against harmful interference;

c)	that excessive levels of unwanted emissions may give rise to harmful interference;

d)	that while out-of-band emissions can also give rise to harmful interference, the Radio Regulations do not provide general limits for these emissions;

e)	that while Appendix 3 applies generally to the mean power of a transmitter and its spurious emissions, it also takes account of a variety of emissions where interpretation of the term “mean power”, and thus its measurement, would be difficult, particularly in the cases of  digital modulation broadband systems, pulsed modulation and narrow-band high-power transmitters;

f)	that unwanted emissions from transmitters operating in space stations may cause harmful interference, particularly emissions from wideband amplifiers which cannot be adjusted after launch;

g)	that unwanted emissions may cause harmful interference to radio astronomy and space services using passive sensors;

h)	that, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect specific services, such as passive services in specific bands or situations;

i)	that broadband digital modulation may cause unwanted emissions at frequencies far from the carrier frequency,

noting

a)	that passive services have in many cases been allocated frequencies adjacent or close to those of services employing high-power transmitters;

b)	that some administrations have adopted more stringent limits for spurious emissions than those specified in Appendix 3;

c)	that, at this time, in response to resolves 2.3.2 of Resolution 722 (WRC-97), ITU-R has decided to recommend not including general out-of-band limits in the Radio Regulations,]

[1	study those frequency bands and instances where, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect passive services such as radio astronomy, and the impact on all concerned services of implementing or not implementing such limits;]

[2	study those frequency bands and instances where, for technical or operational reasons, out-of-band limits may be required to protect passive services such as radio astronomy, and the impact on all concerned services of implementing or not implementing such limits;]

[3	report the results of these studies to [the next] competent world radiocommunication conference(s) [in WRC-07].]

__________