| INTERNATIONAL TELECOMMUNICATION UNION Radiocommunication Bureau (Direct Fax No. +41 22 730 5785) |  |
| --- | --- |

Circular Letter	15 February 1999

CR/116

To Administrations of Member States of the ITU

Subject:	Further request for administrations to supply data on GSO/FSS links and on BSS systems using the frequency bands covered by Appendices S30 and S30A

To the Director General

Dear Sir,

# 1	Introduction

In Circular Letter CR/92, Administrations were invited to supply data on GSO FSS links and on BSS systems using the frequency bands covered by Appendices S30 and S30A.  Responses were requested by 25 June 1998.  A number of administrations responded to that letter, and Joint Task Group 4-9-11 has assembled those responses into a database of GSO FSS and GSO BSS parameters.

At its meeting in January 1999, Joint Task Group 4-9-11 identified the desirability of obtaining additional sets of GSO FSS and GSO BSS link parameters.  This letter repeats the invitation, originally extended in CR/92, for Administrations to supply data on GSO FSS links and on BSS systems using the frequency bands covered by Appendices S30 and S30A.

# 2	Data on GSO/FSS links

## 2.1	Introduction

Resolution 130 (WRC-97) introduced provisional power limits on non-GSO FSS systems in certain bands, and requested the ITU-R to conduct the appropriate technical, operational and regulatory studies to review the regulatory conditions relating to the coexistence of non-GSO and GSO systems, in order to ensure that they do not pose undue constraints on the development of non-GSO and GSO systems. Joint Task Group 4-9-11 has been assigned the responsibility for reporting the results of those studies to the 1999 Conference Preparatory Meeting (CPM-99).

## 2.2	Request for data

Administrations could use the parameters listed in Annex 1 to perform an initial assessment of the impact of interference from a non-GSO system into existing and planned GSO FSS links. The use of Annex 1 would facilitate the identification by Administrations of GSO FSS links which may be affected by non-GSO/FSS interference.

Should this initial assessment indicate that the interference effects require further analysis, Administrations are strongly urged to use more detailed characteristics of the GSO FSS links, including, if practicable, the parameters listed in Annex 2.

Where this assessment indicates that the current provisional limits could have a significant impact on the performance of a GSO FSS link, administrations are requested to communicate to the ITU-R their technical analysis including the characteristics of the GSO FSS link. Administrations are strongly encouraged to provide as much information as possible: as a minimum the parameters of Annex 1, and preferably at least the parameters of Annex 2 should be supplied.

Annex 3 provides a spread sheet and associated formulas which may be used for calculation of link budgets taking into account the characteristics listed in Annex 2. In order to ensure consistency between the parameters that will be used for interference calculations, administrations are encouraged to use this spread sheet in developing the parameters they intend to provide under Annex 2. However, administrations may use instead alternative spread sheets in performing their link budget calculations. (An Excel version of Annexes 6 and 7 may be downloaded from http://www.itu.int/itudoc/itu-r/cl/cr/116/cr116e3.xls and cr116e4.xls respectively).

# 3	Data on BSS systems using the frequency bands covered by Appendices S30 and S30A

## 3.1	Introduction

In considering the use of the frequency bands covered by RR Appendices S30 and S30A by non-geostationary satellite systems in the fixed-satellite service, WRC-97 added new provisional limits in terms of equivalent power flux-density (epfd) and aggregate power flux-density (apfd) to be applied to such systems. These limits appear in RR Article S22 of the Radio Regulations and in the Annex to Resolution 538 (WRC-97).

Resolution 538 (WRC-97) requests the ITU-R to conduct, as a matter of urgency and in time for consideration by WRC-99 (now WRC-2000), the appropriate technical, operational and regulatory studies to review the regulatory provisions concerning the operation of non-GSO FSS systems in the frequency bands covered by RR Appendices S30 and S30A, in order to ensure that these conditions ensure appropriate protection of the Plans and their future modifications and do not place unreasonable constraints on the development of non-GSO FSS systems in these bands. Joint Task Group 4-9-11 has been assigned the responsibility for reporting the results of those studies to the CPM-99.

## 3.2	Request for data

In order for the Joint Task Group to perform the studies needed, Administrations are encouraged to provide details of characteristics of BSS systems using the Plans as well as those being considered for future Plan modifications. The level of detail required is based on the requirement to provide enough information to enable the assessment of the impact on such system by any non-GSO FSS system, as well as the impact of multiple non-GSO FSS systems.

Annex 4 provides the list of the BSS system characteristics which might be needed in order  to evaluate the potential impact of non-GSO FSS systems. Administrations are therefore invited to provide this information in relation to their planned BSS systems and associated BSS feeder links in these bands.

In providing this information, administrations may consider that the impact of the feeder link on the overall system can be accounted for in the downlink data provided. It is also understood that certain characteristics, such as the C/I caused by other services and systems sharing the band may be provided using the administration's best assessment of the expected level.

Annex 5 provides the formulae associated to a spread sheet that may be used for the calculation of link budgets taking into account the characteristics listed in Annex 1. This spread sheet is available in Excel 95 format and can be downloaded from the following address: http://www.itu.int/itudoc/itu-r/cl/cr/116/cr116e2.xls (see Annex 5). In order to ensure consistency between the parameters that will be used for interference calculations, administrations are encouraged to use this spread sheet in developing these parameters. Administrations may also use instead alternative spread sheets in performing their link budget calculations. Administrations are also invited to provide any additional information pertaining to their systems, that they may consider relevant to the studies referred to in § 2 above.

# 4	Submission of contributions

The ITU-R studies are to be completed in time for consideration by the WRC-99. However, in order to allow time- for the completion and translation of the documentation for the CPM-99, the ITU-R studies must be completed by the end of May 1999. For this reason, Administrations are urged to submit the information requested in § 2.2 and § 3.2 by 15 March 1999.

Contributions should be sent to Mr. Jinxing Li (Counsellor Study Group 4, Radiocommunication Bureau, Tel.: +41 22 730 5990, Fax: +41 22 730 5806,   e-mail: jinxing.li@itu.int).

Contributions in response to § 3.2 should also be sent to the Special Rapporteur Group 10-11S/SRG-2 (Mr. E. Reinhart, Telecommunication Consultant, 6449 Linway Terrace, MCLEAN, VA 22101, Tel.: +1 703 4489552; Fax: +1 703 4485920; e-mail: ereinhart@erols.com).

Yours faithfully,

Robert W. Jones

Director,

Radiocommunication Bureau

Annexes: 7

Distribution:

-	Administrations of Member States of the ITU

-	Members of the Radio Regulations Board

ANNEX 1

List of parameters characterizing the GSO FSS link

Type of Link:	Transparent ("Bent Pipe") or Regenerative Repeater

fu:	(GHz)	Frequency of the Uplink

fd:	(GHz)	Frequency of the Downlink

Te:	(K)	Receiving System Noise Temperature of the Earth Station referred to the 	input of the earth station receiver (LNA (or LNB))

Ts:	(K)	Receiving System Noise Temperature of the Space Station referred to the 	input of the space station receiver

g:	(dB)	Transmission Gain of a specific satellite link as defined in Appendix 29 of the 	Radio Regulations

T:	(K)	Equivalent Satellite Link Noise Temperature referred to the input of the earth 	station receiver including allowances for contributions from all internal and 	external (except non-GSO FSS) noise-like interference sources referred to the 	input of the earth station receiver (for the transparent link). In the case of a 	regenerative repeater, the uplink and the downlink, would similarly include 	respective allowances for contributions from internal and external sources and 	be treated separately.

Mcs:	(dB)	Required Minimum Clear Sky Margin above the threshold carrier-to-noise 	ratio which must be exceeded for all but p% of an average year.

p:	(%)	Percentage of time in an average year that the carrier-to-noise3 is below the 	threshold carrier-to-noise4 ratio.

Gs:	(dBi)	Maximum Antenna Gain of the Receiving GSO FSS Space Station

De:	(m)	Diameter of the Receiving GSO FSS Earth Station

Ge:	(dBi)	Maximum Antenna Gain of the Receiving GSO FSS Earth Station

X:	(dB)	The maximum allowable degradation to Mcs which must not be exceeded for 	more than p% of an average year.

Annex 2

Template for the collection of detailed characteristics of GSO/FSS networks

Section 1 of this Annex provides a template for the collection of re-modulating satellite characteristics, while Section 2 provides a template for the collection of transparent satellite characteristics.

1	Template for transparent geostationary networks

| GEOSTATIONARY NETWORK | Name of GSO |
| --- | --- |
| PERFORMANCE OBJECTIVES |  |
| Threshold #1 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| Threshold #2 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| Threshold #3 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| WAVEFORM DESCRIPTION |  |
| Access type (TDMA, CDMA, FDMA,...) |  |
| Modulation type (e.g. FM, QPSK, BPSK) |  |
| Noise bandwidth per carrier (kHz) |  |
| TRANSMIT EARTH STATION CHARACTERISTICS |  |
| Altitude (km) |  |
| Latitude (+: North, -: South) from Equator (degrees) |  |
| Elevation angle (degrees) |  |
| Temperature at ground level (°C) |  |
| Relative humidity (%) |  |
| Rain model (ITU/Crane) |  |
| Rain zone (as per rain model ) |  |
| Rain fall rate exceeded for 0.01% of an average year (mm/h) if available |  |
| On-axis Earth station transmit e.i.r.p. (dBW) |  |
| Antenna pointing loss towards the geostationary satellite (dB) |  |
| Inter modulation earth stations C/I (dB) |  |
| Power control range (>0, 0 dB if none) (dB) |  |
| Power control accuracy (applicable only if up link power control used) (dB) |  |
| Polarisation isolation (C/I of wanted to unwanted polarisation) (dB) |  |
| RECEIVE EARTH STATION CHARACTERISTICS |  |
| Altitude (km) |  |
| Latitude (+: North, -: South) from Equator (degrees) |  |
| Temperature at ground level (°C) |  |
| Relative humidity (%) |  |
| Elevation angle (degrees) |  |
| Rain zone (as per rain model) |  |
| Rain fall rate exceeded for 0.01% of an average year (mm/h) if available |  |
| Earth station receive noise temperature (K) |  |
| On-axis antenna gain (dBi) |  |
| Antenna diameter(m) |  |
| Antenna pointing loss (dB) |  |
| Polarisation isolation (C/I of wanted to unwanted polarisation) (dB) |  |

| SPACE STATION RECEIVE CHARACTERISTICS |  |
| --- | --- |
| Transponder bandwidth (MHz) |  |
| Receive frequency (GHz) |  |
| Receive polarisation (H: horizontal, V: Vertical, C: Circular) |  |
| Automatic level control range (0 if none) (dB) |  |
| Peak receive antenna gain (dBi) |  |
| Receive satellite antenna gain in the direction of transmit earth station (dBi) |  |
| Satellite receive temperature (K) |  |
| Receive cross-polarisation isolation (C/I ratio, 100 if not applicable) (dB) |  |
| Receive frequency re-use isolation (C/I ratio, 100 if not applicable) (dB) |  |
| Transponder total input back-off (dB) |  |
| SPACE STATION TRANSMIT CHARACTERISTICS |  |
| Transmit frequency (GHz) |  |
| Transmit polarisation (H: horizontal, V: Vertical, C: Circular) |  |
| Transponder total output back-off (dB) |  |
| Satellite e.i.r.p. in the direction of the receive earth station (dBW) |  |
| Transmit cross-polarisation isolation (C/I ratio, 100 if not applicable) (dB) |  |
| Transmit frequency re-use isolation (C/I ratio, 100 if not applicable) (dB) |  |
| Transparent/remodulating transponder |  |
| Satellite adjacent transponder isolation (dB) |  |
| Transponder inter modulation C/I(dB) |  |
| INTERFERENCE FROM OTHER GSO NETWORKS AND TERRESTRIAL SERVICES |  |
| Up link clear-sky C/I due to other geostationary networks (dB) |  |
| Up link clear-sky C/I due to sharing with fixed service (dB) (100 dB if no sharing) |  |
| Down link clear-sky C/I due to other geostationary networks (dB) |  |
| Down link clear-sky C/I due to sharing with fixed services (dB) (100 dB if no sharing) |  |

2	Template for re-modulating geostationary network

| GEOSTATIONARY NETWORK | Name of GSO |
| --- | --- |
| UP LINK PERFORMANCE OBJECTIVES |  |
| Threshold #1 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| Threshold #2 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| Threshold #3 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| UP LINK WAVEFORM DESCRIPTION |  |
| Variable coding rate (Yes or No) |  |
| Access type (TDMA, CDMA, FDMA,...) |  |
| Modulation type (e.g. FM, QPSK, BPSK) |  |
| Noise bandwidth per carrier (kHz) |  |
| Repeater receive bandwidth (MHz) |  |
| DOWN LINK PERFORMANCE OBJECTIVES |  |
| Threshold #1 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| Threshold #2 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| Threshold #3 (N/A for not applicable): C/(N+I) (dB) |  |
| % of the year C/(N+I) should be exceeded |  |
| DOWN LINK WAVEFORM DESCRIPTION |  |
| Variable coding rate (Yes or No) |  |
| Access type (TDMA, CDMA, FDMA,...) |  |
| Modulation type (e.g. FM, QPSK, BPSK) |  |
| Noise bandwidth per carrier (kHz) |  |
| Repeater transmit bandwidth (MHz) |  |
| TRANSMIT EARTH STATION CHARACTERISTICS |  |
| Altitude (km) |  |
| Latitude (+: North, -: South) from Equator (degrees) |  |
| Elevation angle (degrees) |  |
| Temperature at ground level (°C) |  |
| Relative humidity (%) |  |
| Rain model (ITU/Crane) |  |
| Rain zone (as per rain model ) |  |
| Rain fall rate exceeded for 0.01% of an average year (mm/h) if available |  |
| On-axis Earth station transmit e.i.r.p. (dBW) |  |
| Antenna pointing loss towards the geostationary satellite (dB) |  |
| Inter modulation earth stations C/I (dB) |  |
| Power control range (>0, 0 dB if none) (dB) |  |
| Power control accuracy (applicable only if up link power control used) (dB) |  |
| Polarisation isolation (C/I of wanted to unwanted polarisation) (dB) |  |

| RECEIVE EARTH STATION CHARACTERISTICS |  |  |
| --- | --- | --- |
| Altitude (km) |  |  |
| Latitude (+: North, -: South) from Equator (degrees) |  |  |
| Temperature at ground level (°C) |  |  |
| Relative humidity (%) |  |  |
| Elevation angle (degrees) |  |  |
| Rain zone (as per rain model) |  |  |
| Rain fall rate exceeded for 0.01% of an average year (mm/h) if available |  |  |
| Earth station receive noise temperature (K) |  |  |
| On-axis antenna gain (dBi) |  |  |
| Antenna diameter(m) |  |  |
| Antenna pointing loss (dB) |  |  |
| Polarisation isolation (C/I of wanted to unwanted polarisation) (dB) |  |  |
| SPACE STATION RECEIVE CHARACTERISTICS |  |  |
| Receive frequency (GHz) |  |  |
| Receive polarisation (H: horizontal, V: Vertical, C: Circular) |  |  |
| Peak receive antenna gain (dBi) |  |  |
| Receive satellite antenna gain in the direction of transmit earth station (dBi) |  |  |
| Satellite receive temperature (K) |  |  |
| Receive cross-polarisation isolation (C/I ratio, 100 if not applicable) (dB) |  |  |
| Receive frequency re-use isolation (C/I ratio, 100 if not applicable) (dB) |  |  |
| SPACE STATION TRANSMIT CHARACTERISTICS |  |  |
| Transmit frequency (GHz) |  |  |
| Transmit polarisation (H: horizontal, V: Vertical, C: Circular) |  |  |
| Transponder total output back-off (dB) |  |  |
| Satellite e.i.r.p. in the direction of the receive earth station (dBW) |  |  |
| Transmit cross-polarisation isolation (C/I ratio, 100 if not applicable) (dB) |  |  |
| Transmit frequency re-use isolation (C/I ratio, 100 if not applicable) (dB) |  |  |
| Satellite adjacent transponder isolation (dB) |  |  |
| Transponder inter modulation C/I(dB) |  |  |
| INTERFERENCE FROM OTHER GSO NETWORKS AND TERRESTRIAL SERVICES |  |  |
| Up link clear-sky C/I due to other geostationary networks (dB) |  |  |
| Up link clear-sky C/I due to sharing with fixed service (dB) (100 dB if no sharing) |  |  |
| Down link clear-sky C/I due to other geostationary networks (dB) |  |  |
| Down link clear-sky C/I due to sharing with fixed services (dB) (100 dB if no sharing) |  |  |

ANNEX 3

Spread sheet to compute the characteristics of a geostationary satellite network

1	Introduction

This Annex provides a spread sheet and the associated formulas for calculation of link budgets taking into account the characteristics listed in Annexes 1 and 2. In order to ensure consistency between the parameters that will be used for interference calculations, ITU-R Sector Members are encouraged to use this spread sheet in developing the parameters that they intend to provide under Annex 2.

In order to permit the implementation of the spread sheet example input characteristics are shown in this spread sheet but should not be understood as being as set of a representative characteristics of any geostationary network.

Section 2 applies to transparent geostationary satellites, while Section 3 applies to remodulating satellites.

Both spread sheet make are used with macro's for the computation of the rain attenuation as in Recommendation ITU-R 618 (see Section 4) or in the Crane model (see Section 5), while Section 6 deals with the macro implementation of the gaseous attenuation, following Recommendation ITU-R 676.

2	Spread sheet for transparent satellites

See Annex 6.

3	Spread sheet for remodulating satellites

See Annex 7.

4	Macro implementation of Recommendation ITU-R P.618

Function ITU_Rain(region, polarization, stat_height, stat_lat, freq, el_angle, availability)

'Inputs

'variable:format:Infor: range

'region:  String: rain region for site of interest: (A,B,C,D,E,E,F,G,H,J,K,L,M,N,P,Q)

'         range input  can also be in lower case - Note I, and O are not valid entries!

'polarization: String : wave:"V","H","C"

'stat_height: Number  : Station  height above mean sea level in km: 0 to ~ N

'stat_lat:Number: Absolute value of Latitude of earth station in deg: (0 - 81.3 degrees)

'freq : Number : Frequency in GHz: Range is 1 GHz to 399.999 GHz

'el_angle: Number: Earth Station antenna elevation angle in deg.  (0 - 90)

'availability: Number:Desired link availability: i.e., 99.5,  (min. value is 99., max 99.999)

'Output/Return value is the attenuation in dB.

'Inputs range check

'Check availability (smallest allowed value will be 99., Max will be 99.999)

'If availability < 99# Then

'   MsgBox "Value for availability is too low, min value is 99"

'  Exit Function

'End If

'If availability > 99.99 Then

'   MsgBox "Value for availability is too high , max value is 99.999"

'  Exit Function

'End If

'Max, lat of 81.3 restricted given visibility limitations at ~0 elevation angles

'If stat_lat > 81.3  Then

'   MsgBox "Sat. not visible from station.  Choose a lat between 0 and  81.3 deg."

'Exit Function

'End If

'If el_angle < 0 Or el_angle > 90 Then

'   MsgBox "Elevation angle is out of limits. Must be with 0 to 90 deg."

'  Exit Function

'End If

'Max. freq. value slightly reduced to simplify arrays search Match command

If freq < 1 Or freq > 40 Then

MsgBox "Frequency is out of limits. Range is 1 GHz to 40 GHz"

Exit Function

End If

'Set up format of basic parameters used several times

' equivalent elevation angle in radians - Excel functions operate in radians

ele_rad = el_angle * Application.Pi / 180

'unavailability, (100 percent - given availability)

unavailability = 100 - availability

'__________________________________________________________

'First Step of Algorithm is to calculate the Isotherm height for the rain : km

' i.e., height at which rain is at 0 deg C

Select Case stat_lat

Case Is > 23 ' Northern Hemisphere

rain_height = 5 - 0.075 * (stat_lat - 23)

Case 0 To 23     ' Northern Hemi.

rain_height = 5

Case -21 To 0    ' Southern Hemi.

rain_height = 5

Case -71 To -21 ' Southern Hemi.

rain_height = 5 + 0.1 * (stat_lat + 21)

Case -90 To -71  ' Southern Hemi.

rain_height = 0

End Select

'Next determine the slant path length to isotherm, this is the Ls in the ITU Rec.

' Note the value of 8500 is the earth radius in km

If el_angle >= 5 Then

slant_path = (rain_height - stat_height) / Sin(ele_rad)

Else

' very low elevation angles

slant_path = 2 * (rain_height - stat_height) / (Sqr(Sin(ele_rad) ^ 2 _

+ 2 * (rain_height - stat_height) / 8500) + Sin(ele_rad))

End If

'Determine horizontal proj. to ground of slant path length.  (this is the LG in the ITU REC)

horz_slant_path = slant_path * Cos(ele_rad)

' Now determine the Rain Point intensity (mm/hr)for an exceedance of 0.01: R_one_hundreth

' select value for selected rain region

' only one of the .01  rates are  used (based on the rain region)

' Values taken from ITU-R, Rec. 837-1, 1994

Select Case region

Case "A", "a"

R_one_hundreth = 8

Case "B", "b"

R_one_hundreth = 12

Case "C", "c"

R_one_hundreth = 15

Case "D", "d"

R_one_hundreth = 19

Case "E", "e"

R_one_hundreth = 22

Case "F", "f"

R_one_hundreth = 28

Case "G", "g"

R_one_hundreth = 30

Case "H", "h"

R_one_hundreth = 32

Case "J", "j"

R_one_hundreth = 35

Case "K", "k"

R_one_hundreth = 42

Case "L", "l"

R_one_hundreth = 60

Case "M", "m"

R_one_hundreth = 63

Case "N", "n"

R_one_hundreth = 95

Case "P", "p"

R_one_hundreth = 145

Case "Q", "q"

R_one_hundreth = 115

Case Else

MsgBox "Invalid entry for rain region; range is  A to Q, except for I and O "

Exit Function

End Select

' correct R_one_hundreth value if above 100, only for path reduction calculation, for other calculations the value obtained from above is used

If R_one_hundreth > 100 Then

R_o = 100

Else

R_o = R_one_hundreth

End If

' calculate the reduction factor for availability r.01

L_o = 35 * Exp(-0.015 * R_o)

'reduction factor for .01% of the time

red_factor = 1 / (1 + (horz_slant_path / L_o))

' Now find the k and alpha factor per ITU-R  Rec.838

'array of frequencies 1 to 400 GHz, used to specify an index value for k and alpha

freq_array = Array(1, 2, 4, 6, 7, 8, 10, 12, 15, 20, 25, 30, 35, 40, 45, 50, 60, _

70, 80, 90, 100, 120, 150, 200, 300, 400)

'the following are arrays of specific values for kh, kv, alphah, and alphav.

'they are used for interpolating when calculating actual values k_H, k_V,

'alpha_H, and alpha_V.

kh = Array(0.0000387, 0.000154, 0.00065, 0.00175, 0.00301, 0.00454, 0.0101, _

0.0188, 0.0367, 0.0751, 0.124, 0.187, 0.263, 0.35, 0.442, 0.536, 0.707, _

0.851, 0.975, 1.06, 1.12, 1.18, 1.31, 1.45, 1.36, 1.32)

kv = Array(0.0000352, 0.000138, 0.000591, 0.00155, 0.00265, 0.00395, 0.00887, _

0.0168, 0.0335, 0.0691, 0.113, 0.167, 0.233, 0.31, 0.393, 0.479, 0.642, _

0.784, 0.906, 0.999, 1.06, 1.13, 1.27, 1.42, 1.35, 1.31)

alphah = Array(0.912, 0.963, 1.121, 1.308, 1.332, 1.327, 1.276, _

1.217, 1.154, 1.099, 1.061, 1.021, 0.979, 0.939, 0.903, 0.873, 0.826, _

0.793, 0.769, 0.753, 0.743, 0.731, 0.71, 0.689, 0.688, 0.683)

alphav = Array(0.88, 0.923, 1.075, 1.265, 1.312, 1.31, 1.264, _

1.2, 1.128, 1.065, 1.03, 1, 0.963, 0.929, 0.897, 0.868, 0.824, _

0.793, 0.769, 0.754, 0.744, 0.732, 0.711, 0.69, 0.689, 0.684)

' find index for frequency within the frequency array

freq_index = Application.Match(freq, freq_array)

freq1 = freq_index

freq2 = freq_index + 1

' interpolate to find the values for k_H and K_V .: Log (K_x)vs LOG(freq)

x1 = Application.Log10(freq_array(freq1))

x2 = Application.Log10(freq_array(freq2))

x3 = Application.Log10(freq)

y1 = Application.Log10(kh(freq1))

y2 = Application.Log10(kh(freq2))

k_H = 10 ^ (y1 - (x3 - x1) * (y1 - y2) / (x2 - x1))

y1 = Application.Log10(kv(freq1))

y2 = Application.Log10(kv(freq2))

k_v = 10 ^ (y1 - (x3 - x1) * (y1 - y2) / (x2 - x1))

'Interpolate to find the valued for alpha_H and alpha_V : Alpha_x vs log (freq)

y1 = alphah(freq1)

y2 = alphah(freq2)

alpha_H = y1 - (x3 - x1) * (y1 - y2) / (x2 - x1)

y1 = alphav(freq1)

y2 = alphav(freq2)

alpha_v = y1 - (x3 - x1) * (y1 - y2) / (x2 - x1)

'Determine tilt of wave based on polarization type, set polarization angle to radians

Select Case polarization

Case "C"

tau = Application.Pi / 4

Case "V"

tau = Application.Pi / 2

Case "H"

tau = 0

Case Else

MsgBox "Polarization types: circular, horizontal, or vertical"

End Select

'calculate the factor k

k = (k_H + k_v + (k_H - k_v) * Cos(2 * tau) * (Cos(ele_rad) ^ 2)) / 2

'calculate the factor alpha

alpha = (k_H * alpha_H + k_v * alpha_v + (k_H * alpha_H - k_v * alpha_v) _

* Cos(2 * tau) * (Cos(ele_rad) ^ 2)) / (2 * k)

'specific attenuation from frequency-dependent coefficients (dB/km)

gamma_R = k * (R_one_hundreth) ^ alpha

'predicted attenuation exceeded for .01% of an average year

A_one_hundreth = gamma_R * slant_path * red_factor

'estimated attenuation to be exceeded for other percentages of an average year

'in the range of .001% to 1%  is approximated by

A_p = A_one_hundreth * 0.12 * unavailability ^ (-1 * (0.546 + _

0.043 * Application.Log10(unavailability)))

ITU_Rain = A_p

End Function

5	Macro implementation of the Crane model

' This function performs the Crane/Global rain attenuation model

' Implementation per the NASA Propagation Effects Handbook for Satellite Systems Design

' Dated  1989 (fourth edition) Section 6.3.2.1 page 6-23,

' "Prediction of Rain Attenuation" by R. Crane, IEEE Transactions on Communications

' Sep. 9, 1980, and updated tables from the ITU Propagations Rec-338 1994

Function CRANE(region, polarization, stat_height, stat_lat, freq, el_angle, availability)

'Inputs

'variable:format:Infor: range

'region:  String: Crane rain region for site of interest: (A,B,B1,B2,C,D,D1,D2,D3,E,F,G,H)

'         range input  can also be in lower case

'polarization: String : wave:"V","H","C"

'stat_height: Number  : Station  height above mean sea level in km: 0 to ~ N

'stat_lat:Number: Absolute value of Latitude of earth station in deg: (0 - 81.3 degrees)

'freq : Number : Frequency in GHz: Range is 1 GHz to 399 GHz

'el_angle: Number: Earth Station antenna elevation angle in deg.  (0 - 90)

'availability: Number:Desired link availability: i.e., 99.5,  (min. value is 95.1, max 99.999)

'Output/Return value is the AVERAGE attenuation in dB.

'Inputs range check

'Check availability (smallest allowed value will be 95.01, Max will be 99.999)

'availability range was reduced slightly to simplify arrays search - Match command

'If availability < 95.01 Then

'   MsgBox "Value for availability is too low, min value is 95.01"

'  Exit Function

'End If

'If availability > 99.9989 Then

'   MsgBox "Value for availability is too high , max value is 99.9989"

'  Exit Function

'End If

'Max, lat of 81.3 restricted given visibility limitations at ~0 elevation angles

'If stat_lat > 81.3 Or stat_lat < 0 Then

'   MsgBox "Sat. not visible from station.  Choose a lat between 0 and  81.3 deg."

'Exit Function

'End If

'If el_angle < 0 Or el_angle > 90 Then

'   MsgBox "Elevation angle is out of limits. Must be with 0 to 90 deg."

'  Exit Function

'End If

'Max. freq. value slightly reduced to simplify arrays search Match command

If freq < 1 Or freq > 399.9 Then

MsgBox "Frequency is out of limits. Range is 1 GHz to 399.9 GHz"

Exit Function

End If

' equivalent elevation angle in radians - Excel functions operate in radians

ele_rad = el_angle * Application.Pi / 180

stat_lat = Abs(stat_lat)

' Major Step 1, find the Rp (Point Rain Rate Distribution) or XPD

'_______________________________________________________

unavailability = 100 - availability

'Array of "unavailability" numbers which is used to specify an index

'given the point rain rate

unavl = Array(0.001, 0.002, 0.005, 0.01, 0.02, 0.05, 0.1, 0.2, 0.5, 1, 2, 5)

'Selection of Point Rain Rate Distribution (Rp)  vs. Exceedance Prob.

'first select appropriate array, then interpolate for desired value

'select one possible array (named rates) based on CRANE Global rain region

'see table 6.3-1 of NASA handbook

Select Case region

Case "A", "a"

rates = Array(28.5, 21, 13.5, 10, 7, 4, 2.5, 1.5, 0.7, 0.4, 0.1, 0.03)

Case "B", "b"

rates = Array(57.5, 44, 28.5, 19.5, 13.5, 8, 5.2, 3.4, 1.9, 1.3, 0.7, 0.3)

Case "B1", "b1"

rates = Array(45, 34, 22, 15.5, 11, 6.4, 4.2, 2.8, 1.5, 1, 0.5, 0.2)

Case "B2", "b2"

rates = Array(70, 54, 35, 23.5, 16, 9.5, 6.1, 4, 2.3, 1.5, 0.8, 0.3)

Case "C", "c"

rates = Array(78, 62, 41, 28, 18, 11, 7.2, 4.8, 2.7, 1.8, 1.1, 0.5)

Case "D1", "d1"

rates = Array(90, 72, 50, 35.5, 24, 14.5, 9.8, 6.4, 3.6, 2.2, 1.2, 0.03)

Case "D2", "d2", "D", "d"

rates = Array(108, 89, 64.5, 49, 35, 22, 14.5, 9.5, 5.2, 3, 1.5, 0.3)

Case "D3", "d3"

rates = Array(126, 106, 80.5, 63, 48, 32, 22, 14.5, 7.8, 4.7, 1.9, 0.03)

Case "E", "e"

rates = Array(165, 144, 118, 98, 78, 52, 35, 21, 10.6, 6, 2.9, 0.5)

Case "F", "f"

rates = Array(66, 51, 34, 23, 15, 8.3, 5.2, 3.1, 1.4, 0.7, 0.2, 0.03)

Case "G", "g"

rates = Array(185, 157, 120.5, 94, 72, 47, 32, 21.8, 12.2, 8, 5, 1.8)

Case "H", "h"

rates = Array(253, 220.5, 178, 147, 119, 86.5, 64, 43.5, 22.5, 12, 5.2, 1.2)

Case Else

MsgBox "Invalid entry for rain region"

Exit Function

End Select

'Determine the indexes to be used for reading the rates array, depending on desired availability

'Find the closest index for the value inside the unavailable array that is less than or equal to desired value

u_index = Application.Match(unavailability, unavl)

u1 = u_index

u2 = u_index + 1

'Interpolate Log (Rp)  vs. log (unavailability)- if you plot this you will see that

'relationship is almost linear

x1 = Application.Log10(unavl(u1))

y1 = Application.Log10(rates(u1))

x2 = Application.Log10(unavl(u2))

y2 = Application.Log10(rates(u2))

x3 = Application.Log10(unavailability)

'Now - Linear Interpolate for the value of Rp - given (x,y) pairs

R_p = 10 ^ (y1 - (x3 - x1) * (y1 - y2) / (x2 - x1))

' Second major Step

'___________________________________________________

' Determine the Isotherm Height (0 deg. C)- height of frozen rain layer

' Dependent on Latitude, and unavailability -See figure 6.3-4 or NASA handbook

'array of latitudes used in determining isotherm height

lat_array = Array(0, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, _

75, 80, 85)

' array of unavailability for isotherm height determination

unavl2 = Array(0.001, 0.01, 0.1, 1, 5)

u_index = Application.Match(unavailability, unavl2)

u1 = u_index

u2 = u_index + 1

' select appropriate set of two curves to interpolate in between them

Select Case u_index

' input is between .001 and .01

Case 1

h_1 = Array(5.05, 5.1, 5.2, 5.2, 5.15, 5, 4.8, 4.45, 4, 3.6, 3.15, 2.65, _

2.25, 1.8, 1.55, 1.3, 1.1)

h_2 = Array(5.3, 5.35, 5.45, 5.5, 5.5, 5.45, 5.3, 5, 4.7, 4.4, 4, 3.6, _

3.15, 2.75, 2.4, 1.9, 1.6)

' input is between .01 and .1

Case 2

h_1 = Array(4.85, 4.85, 4.85, 4.8, 4.75, 4.6, 4.25, 3.8, 3.25, 2.65, 2.2, _

1.7, 1.3, 1, 0.75, 0.6, 0.5)

h_2 = Array(5.05, 5.1, 5.2, 5.2, 5.15, 5, 4.8, 4.45, 4, 3.6, 3.15, 2.65, _

2.25, 1.8, 1.55, 1.3, 1.1)

' input is between .1 and 1

Case 3

h_1 = Array(4.6, 4.6, 4.55, 4.5, 4.4, 4.2, 3.8, 3.25, 2.5, 1.75, 1.2, _

0.7, 0.4, 0.2, 0.1, 0.05, 0.001)

h_2 = Array(4.85, 4.85, 4.85, 4.8, 4.75, 4.6, 4.25, 3.8, 3.25, 2.65, 2.2, _

1.7, 1.3, 1, 0.75, 0.6, 0.5)

' input is between 1 and 5

Case 4

h_1 = Array(4.2, 4.2, 4.15, 4.1, 4, 3.8, 3.4, 2.8, 2.1, 1.35, 0.8, _

0.3, 0.1, 0.05, 0.01, 0.01, 0.01)

h_2 = Array(4.6, 4.6, 4.55, 4.5, 4.4, 4.2, 3.8, 3.2, 2.5, 1.75, 1.2, _

0.7, 0.4, 0.2, 0.1, 0.05, 0.001)

End Select

' find bracketing set of index within array for station latitudes

lat_index = Application.Match(stat_lat, lat_array)

lat1 = lat_index

lat2 = lat_index + 1

' interpolate Height for selected latitude at available curve 1 of prob

x1 = lat_array(lat1)

x2 = lat_array(lat2)

x3 = stat_lat

y11 = h_1(lat1)

y21 = h_1(lat2)

y31 = y21 + (x2 - x3) * (y11 - y21) / (x2 - x1)

' interpolate for selected latitude at available curve 2 of prob

y12 = h_2(lat1)

y22 = h_2(lat2)

y32 = y22 + (x2 - x3) * (y12 - y22) / (x2 - x1)

'Now interpolate the Height at desired lat. - relationship is Height vs log (Prob)

x1 = Application.Log10(unavl2(u1))

x2 = Application.Log10(unavl2(u2))

x3 = Application.Log10(unavailability)

y2 = y31

y1 = y32

'interpolate for isotherm height in km :  y=a+b*log(x)

H = y2 + (x2 - x3) * (y1 - y2) / (x2 - x1)

'horizontal path projection length (km)

If el_angle >= 10 Then

D = (H - stat_height) / Tan(ele_rad)

Else

'Earth Radius

E = 8500

Ho = stat_height

D = E * Application.Asin((Cos(ele_rad) / (H + E)) * (Sqr(((Ho + E) ^ 2) _

* (Sin(ele_rad) ^ 2) + 2 * E * (H - Ho) + H ^ 2 - Ho ^ 2) - (Ho + E) * _

Sin(ele_rad)))

End If

'if the value for D is larger than 22.5 km, scale out the outage percentage by

'(D/22.5)and make D=22.5, if this is the case, need to find Rp again

If D > 22.5 Then

unavailability = unavailability * (D / 22.5)

D = 22.5

If unavailability >= 5 Then

unavailability = 4.999

End If

u_index = Application.Match(unavailability, unavl)

u1 = u_index

u2 = u_index + 1

'Log (Rp)  vs. log (unavailability)

x1 = Application.Log10(unavl(u1))

y1 = Application.Log10(rates(u1))

x2 = Application.Log10(unavl(u2))

y2 = Application.Log10(rates(u2))

x3 = Application.Log10(unavailability)

R_p = 10 ^ (y1 - (x3 - x1) * (y1 - y2) / (x2 - x1))

End If

' Major Step 3: find Coefficients K and alpha as a function of frequency

'___________________________________________________________________

'array of frequencies 1 to 400 GHz, used to specify an index value for KH,KV _

' and alphaH, AlphaV.  These values were taken from the updated ITU Rec.838 1994

' almost identical than what appears in NASA handbook

freq_array = Array(1, 2, 4, 6, 7, 8, 10, 12, 15, 20, 25, 30, 35, 40, 45, _

50, 60, 70, 80, 90, 100, 120, 150, 200, 300, 400)

'the following are arrays of specific values for kh, kv, alphah, and alphav.

'they are used for interpolating when calculating actual values k_H, k_V,

'alpha_H, and alpha_V.

kh = Array(0.0000387, 0.000154, 0.00065, 0.00175, 0.00301, 0.00454, 0.0101, _

0.0188, 0.0367, 0.0751, 0.124, 0.187, 0.263, 0.35, 0.442, 0.536, 0.707, _

0.851, 0.975, 1.06, 1.12, 1.18, 1.31, 1.45, 1.36, 1.32)

kv = Array(0.0000352, 0.000138, 0.000591, 0.00155, 0.00265, 0.00395, 0.00887, _

0.0168, 0.0335, 0.0691, 0.113, 0.167, 0.233, 0.31, 0.393, 0.479, 0.642, _

0.784, 0.906, 0.999, 1.06, 1.13, 1.27, 1.42, 1.35, 1.31)

alphah = Array(0.912, 0.963, 1.121, 1.308, 1.332, 1.327, 1.276, _

1.217, 1.154, 1.099, 1.061, 1.021, 0.979, 0.939, 0.903, 0.873, 0.826, _

0.793, 0.769, 0.753, 0.743, 0.731, 0.71, 0.689, 0.688, 0.683)

alphav = Array(0.88, 0.923, 1.075, 1.265, 1.312, 1.31, 1.264, _

1.2, 1.128, 1.065, 1.03, 1, 0.963, 0.929, 0.897, 0.868, 0.824, _

0.793, 0.769, 0.754, 0.744, 0.732, 0.711, 0.69, 0.689, 0.684)

' find index for frequency within the frequency array

freq_index = Application.Match(freq, freq_array)

freq1 = freq_index

freq2 = freq_index + 1

' interpolate to find the values for k_H and K_V .: Log (K_x)vs LOG(freq)

x1 = Application.Log10(freq_array(freq1))

x2 = Application.Log10(freq_array(freq2))

x3 = Application.Log10(freq)

y1 = Application.Log10(kh(freq1))

y2 = Application.Log10(kh(freq2))

k_H = 10 ^ (y1 - (x3 - x1) * (y1 - y2) / (x2 - x1))

y1 = Application.Log10(kv(freq1))

y2 = Application.Log10(kv(freq2))

k_v = 10 ^ (y1 - (x3 - x1) * (y1 - y2) / (x2 - x1))

'Interpolate to find the valued for alpha_H and alpha_V : Alpha_x vs log (freq)

y1 = alphah(freq1)

y2 = alphah(freq2)

alpha_H = y1 - (x3 - x1) * (y1 - y2) / (x2 - x1)

y1 = alphav(freq1)

y2 = alphav(freq2)

alpha_v = y1 - (x3 - x1) * (y1 - y2) / (x2 - x1)

'Determine tilt of wave based on polarization type, set polarization angle to radians

Select Case polarization

Case "C"

tau = Application.Pi / 4

Case "V"

tau = Application.Pi / 2

Case "H"

tau = 0

Case Else

MsgBox "Polarization types: circular, horizontal, or vertical"

End Select

'calculate the factor k

k = (k_H + k_v + (k_H - k_v) * Cos(2 * tau) * Cos(ele_rad) ^ 2) / 2

'calculate the factor alpha

alpha = (k_H * alpha_H + k_v * alpha_v + (k_H * alpha_H - k_v * alpha_v) _

* Cos(2 * tau) * Cos(ele_rad) ^ 2) / (2 * k)

'compute empirical constants for each percentage value

x = 2.3 * R_p ^ -0.17

y = 0.026 - 0.03 * Log(R_p)

' Note that the NASA handbook has a typo for Z  in section 6: it is correct in sec. 3

Z = 3.8 - 0.6 * Log(R_p)

u = (Log(x * Exp(y * Z))) / Z

' Major Step 4

'____________________________________

'Calculate Rain attenuation

If D = 0 Or el_angle = 90 Then

A = (H - stat_height) * (k * R_p ^ alpha)

CRANE = A

Exit Function

End If

p_mult = (k * R_p ^ alpha) / Cos(ele_rad)

If D < Z Then

A = p_mult * (Exp(u * alpha * D) - 1) / (u * alpha)

CRANE = A

Exit Function

End If

If Z <= D Then

A = p_mult * ((Exp(u * Z * alpha) - 1) / (u * alpha) - (Exp(y * Z * alpha) _

* x ^ alpha) / (y * alpha) + (Exp(y * D * alpha) * x ^ alpha) / (y * alpha))

CRANE = A

Exit Function

End If

End Function

6	Macro implementation of Recommendation ITU-R 676 for the computation of the gaseous attenuation

Function Gas_Att_ITU(freq, Temp_surf, Humidity, Ele, env)

'This function estimates the attenuation due to atmospheric gases per ITU rec. 676

' So far this routine works only for frequencies below 57 GHz

'Freq= Frequency in GHz

'Temp_surf = surface temperature in degrees C

'Relative Humidity at site (%)

'Ele = Elevation angle in degrees

'env = environment : Clear or Rain

'Saturated Partial pressure of water vapour

ps = 206.43 * Exp(0.0354 * ((9 * Temp_surf / 5) + 32))

Rho = (Humidity / 100) * ps / (0.461 * (Temp_surf + 273))

'Attenuation due to Oxygen for frequencies less than 57 GHz

Alpha_o = (0.00719 + (6.09 / (freq ^ 2 + 0.0227)) + (4.81 / ((freq - 57) ^ 2 + 1.5))) * (freq ^ 2 / 1000)

' need to code for frequencies >57  in the future - maybe

'Attenuation due to other Water vapour at frequencies less than 350 GHz

Alpha_w = (0.05 + 0.0021 * Rho + (3.6 / ((freq - 22.2) ^ 2 + 8.5)) + (10.6 / ((freq - 183.3) ^ 2 + 9)) + (8.9 / ((freq - 325.4) ^ 2 + 26.3))) * freq ^ 2 * Rho / 10000

' equivalent height for oxygen at freq < 57 GHz

Ho = 6

' equivalent height for water vapour -

If env = "clear" Then

hwo = 1.6

End If

If env = "rain" Then

hwo = 2.1

End If

hw = hwo * (1 + (3# / ((freq - 22.2) ^ 2 + 5)) + (5 / ((freq - 183.3) ^ 2 + 6)) + (2.5 / ((freq - 325.4) ^ 2 + 4)))

Att = (Alpha_o * Ho + Alpha_w * hw) / Sin(Application.Radians(Ele))

If freq < 57 Then

Gas_Att_ITU = Att

Else

Gas_Att_ITU = "error, Freq > 57 GHz"

End If End Function

ANNEX 4

GSO BSS systems and feeder link characteristics needed to evaluate the potential impact 
of non-GSO FSS systems in the frequency bands 
covered by Appendix S30 and S30A

1	System characteristics

- Frequency (GHz)

- Availability objective (% of the time)

- Receiver Noise bandwidth (MHz)

- Modulation type

- C/I due to the other BSS networks in the Plan (dB)

- C/I due to GSO FSS networks (dB)

- Clear sky feeder link C/(N+I) (dB)

- C/N+I required at operating threshold (dB)

- Clear sky and 99.7% of time margin above operating threshold (dB)

2	Characteristics of the space station

- Longitude (°)

- Satellite e.i.r.p. in the direction of the earth station considered (dBW)

3	Characteristics of the receive earth station

- (Geographical and climatic information should be provided for a number of representative receive earth stations and locations within each beam, in order to fully characterize the interference impact of the non-GSO system)

- Antenna diameter (cm)

- On-axis antenna gain at antenna output (dBi)

- Off-axis antenna gain characteristics (dBi as a function of off-axis angle °)

- Maximum pointing loss (dB)

- Clear sky receive system noise temperature at antenna output (K)

- Latitude (° ')

- Longitude (° ')

- Altitude (m)

- Rain climatic zone

- Elevation angle

- Rain attenuation for 99.7% of the time (dB)

- Rain attenuation for the percentage of time corresponding to the availability objective (dB)

4	Feeder link earth station characteristics1

- Minimum feeder link e.i.r.p.

- Maximum uplink power control

- Latitude (°)

- Longitude (°)

- Altitude (m)

- Rain climatic zone

- Elevation angle

- Rain attenuation for 99.97% of the time (dB)

- Rain attenuation for the percentage of time corresponding to the availability objective (dB)

5	Characteristics of the space station receiver

- Frequency Band

- Satellite receive noise temperature

- Satellite receive antenna gain in the direction of feeder link station

- Automatic Gain Control range

- C/I from other assignments in the Plan

- C/I from other GSO FSS systems

# Comments on Annexes 6 and 7 (English only)

This information refers to the modifications to the spreadsheet that may be used to calculate link budgets with the parameters appearing in Annex 2 of CR-92.

1.	Transparent transponder

Line 103 (uplink rain attenuation)

Line 107 (Up link gazeous attenuation)

Line 108 (Wanted power at satellite receive antenna output)

Line 111 (Wanted carrier IBO variation at TWTA input)

- Add this information

- Line 112 (Total input back off at TWTA input)

- Line 114 (Wanted satellite eirp variation w.r.t. to clear sky condition)

- Refer multicarrier / monocarrier transponder mode to the adequate cell (C70 instead of C69)

- Line 117 (downlink rain attenuation)

- Take into account the given “ Rain fall rate exceeded for 0.01% of an average year (mm/h) ”, when available

- Update rain calculation macro accordingly

- Line 118 (Down link gazeous attenuation)

- Upgrade gaz calculation macro to take altitude into account (see Rec 676)

- Line 121 (Wanted power at earth station receive antenna output)

- Take gazeous attenuation into account when calculating wanted signal fadings

- Line 124 (Thermal temperature variation at earth station receive antenna output (K) due to down link rain fade and gases w.r.t. clear sky conditions)

- Line 125 (Thermal noise power at earth station receive antenna)

- Take gazeous attenuation into account when calculating antenna noise variations

- Line 128 (Combined cross-polarisation C/I)

- Line 129 (Combined frequency-reuse C/I)

- Line 130 (Combined intermodulation C/I)

- Line 134 (Combined other geostationary networks C/I)

- Line 135 (Combined fixed-service systems C/I)

- Take rain effects on internal and external interferences into account

- Take uplink wanted signal variations into account when calculating downlink wanted signal

- Line 132 (Total internal C/I)

- Delete C/N contribution to C/I

- Regenerative transponders:

Line 100:	Add of the macro ITU_Rain_mod which allows to calculate rain attenuation with the rain fall rate instead of ITU rain zone (the macro is also provided in the Excel file, of course).

Line 101:	for the UPC change ">" into "<" and change B36 into B40

Line 126:	the down link gaseous attenuation should be taken into account

Line 144:	Add of the macro ITU_Rain_mod which allows to calculate rain attenuation with the rain fall rate instead of ITU rain zone (the macro is also provided in the Excel file, of course).

Assumes that the transmitting and receiving earth stations and the transmitting and receiving spacecraft antenna(s) are all at their maximum respective pointing errors.  The transmitting earth station is transmitting at the minimum power level in its specified range of variability from the nominal set transmitter power level.  No margin for Satellite End-Of-Life (EOL) is included.

3, 4	Equivalent Satellite Link Noise.

1  This data is optional if the Administration considers that the impact of the feeder link is adequatedly accounted for by the downlink clear sky feeder link C/(N+I) provided in Section 1.