| Received: 8 May 2007 TECHNOLOGY | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8F/1193-E |
|  |  | 9 May 2007 |
|  |  | English only |

Subject:	Question ITU-R 229-1/8

Israel

Israeli EVALUATION GROUP (ISR-EVG) EVALUATION REPORT
of IP-OFDMA

# 1	Introduction

The IP-OFDMA proposal (Doc. 8F/1065) for IMT-2000 was evaluated by the Israeli Evaluation Group, based on Recommendation ITU-R M.1225. We present below the results of our evaluation.

# 2	ISR-EVG approach

Our group provides the evaluation results based on Annex 3 in Recommendation ITU-R M.1225.

# 3	Summary of evaluation results

# 3.1	Criterion A3.1: spectrum efficiency

3.1.1	Evaluation Results

We agree with the proponent evaluation.

# 3.2	Criterion A3.2: technology complexity-effect on cost of installation and operation

We consider that the complexity of the IP-OFDMA technology is adapted to the status of the radio and silicon technologies.

Peak transmitter/carrier (Pb) power (A3.2.2.1) for the BS is not limited by the RTT.

Broadband power amplifier (A3.2.2.2): A broadband power amplifier is needed; the specific requirements for the maximum power are provided by the licensing regulations.

Number of users per carrier/frequency channel (A3.2.8): We agree with proponent evaluation.

Base site implementation/installation requirements (A3.2.9): We consider that the site cost is similar for all the technologies.

Handover complexity (A3.2.10):  The IEEE 802.16 standard provides both simple and optimized handover procedures.

# 3.3	Criterion A3.3: Quality

This RTT supports the data rate requirements for the specified test environments. It implements voice communications using VoIP. The enhanced re-transmission procedures allow for improved voice quality.

Maximum user bit rate for data (A.3.3.5): The user bit rate is linearly scaled with channel bandwidth and MIMO transmission order. For a 10 MHz channel using SISO, the data rate is above 20 Mbit/s. For a 2*2 MIMO the data rate is double.

Voice quality (A3.3.7): The IP-OFDMA supports VoIP.

# 3.4	Criterion A3.4: flexibility of radio technologies

Multimedia capabilities (A3.4.1.7): Multimedia applications are provided due to the fact that 16e standard and Mobile WiMAX are optimised for large throughput/capacity DL and to natural broadcast capabilities to multiple users within coverage including massive DL traffic scenarios, such as popular sports events through MBS.

Flexibility in the use of the frequency band (A3.4.2.1.1): In principle the operating BW in an OFDMA system is determined by two factors, the FFT size and the sampling frequency. Both are scalable and thus the frequency allocation can fit into a variety of regulatory constraints.

Moreover, the different reuse factors supported, such as 1:3 and 1:1, allow optimal deployment in different cell configuration and an excellent means to mitigate interference.

Moreover, TDD offers more flexibility for regulators, to allocate RF spectrum.

Minimum frequency band required to operate (A3.4.2.1.3): The flexibility of OFDMA technology in terms of frequency use allows applying it to small BW such as 1.25 MHz, as exercised in satellite communications and narrowband CDMA. The factor that determines spectral efficiency and the robustness of the system in mobility environment and other conditions (Doppler) is the carrier spacing. Maintaining sufficient carrier spacing allows the application of OFDMA with adequate QoS in small BW as the 1.25 MHz mentioned above.

Frequency management between different layers (A3.4.2.3.1)

The technology allows using hierarchical layers.

Existing system migration capability (A3.4.2.4.3)

Being the first generation of this technology, there is no possibility for system migration.

# 3.5	Criterion A3.5: implication on network interfaces

The questions in this evaluation clause refer mainly to the TDM network interfaces, as ISDN and E1/T1. The IP-OFDMA technology may be interfaced to the legacy networks using Routers, Gateways, Soft VoIP Switches, which are now well deployed and used.

Examine the network modifications required for the RTT to pass the standard set of ISDN bearer services (A3.5.3.1):

The ISDN bearer services can be interfaced with the IP-OFDMA RTT using Routers or VoIP gateways, commercially available for a number of years.

# 3.6	Criterion A3.6: Hand-portable performance optimization capability

Peak transmission power (A3.6.4): Peak transmission power should not exceed 30 dbm (EIRP), subject to regulation (class 3). The increase in cell capacity will be achieved through sophisticated multiple antenna method in DL and UL, especially Receive Beamforming on the Base Station side.

Diversity schemes (A3.6.9): In WiMAX/IP-OFDMA the BS and MS are endowed with multiple antennae. This allows the application of diverse transmission and reception schemes, which include:

a)	Spatial Multiplexing (SM), in which multiple information streams are transmitted simultaneously using identical frequency and temporal resources, thus explicitly increasing link capacity. SM may be employed both at the DL and UL. In the UL case the simultaneous transmissions are done by two users each with a single Tx antenna.

b)	Diversity schemes, such as Maximal Ratio Combining, Space Time Coding and their combinations, are used in a way the identical information streams undergo multiple and diverse channels. Thus, diversity schemes significantly improve the channel condition which in turn allows increasing the link capacity through adaptive modulation and coding.

c)	Beamforming is a method to create spatial filtering with the use of multiple receives and/or transmit antennas by means of digital signal processing, thus maximizing the link CINR on receive and directional transmission beam on transmit. We further note that Tx and Rx Beamforming may be applied simultaneously to several MS, thus employing Spatial Division Multiple Access (SDMA).

The number of antennas (A3.6.10): The minimum number of antennas in order to employ the above mentioned MIMO and Diversity schemes is:

BS: 2Tx, 2Rx;  	MS: 1Tx, 2Rx

The above antenna number allows efficient employment of SM in the UL (Collaborative) and DL, as well as STS+MRC diversity schemes.

However, in order to have an efficient Beamforming gain a larger number of Tx and Rx antennas on the BS side are required. Moreover, the efficiency of Tx beamforming is significantly (through the combination with SM) increased when 2Tx antennas are employed on the MS side.

The number of receivers (A3.6.11): The number of receivers (receive chains) should correspond to the number of receive antennae as described in the previous item.

The ratio of “off (sleep)” time to “on” time (A3.6.13): The ratio is programmable and taken care of in MS implementation.

Digital signal processing requirements (A3.6.15): The requirements for DSP processing power are in line with the achievements of the commercial technology.

# 3.7	Criterion A3.7: coverage/power efficiency

Evaluation of coverage efficiency enhancement techniques (A3.7.1.2): A1.3.5, A1.3.6 Various methods exist to enhance the coverage. The most prominent are:

a)	Beamforming and other spatial diversity method that concentrate the transmitted energy towards specific users.

b)	The use of Pico-BS in malls and within buildings to increase QoS and coverage.

c)	The application of smart repeaters that allow creating another dimension of diversity and increased coverage (Cooperative Diversity).

d)	Coordinated and synchronized transmission in multi-cell deployment.

The use of the above methods is deployment scenario specific.

# 4	Conclusion

Based on the system evaluation performance, field tests and actual deployments we consider that the IP-OFDMA RTT is suitable to become a member of the IMT-2000 family of standards within ITU-R M.1457-7.

References

[1]	Document 8F/1065 – IEEE - proposed new IMT-2000 terrestrial radio interface intended for inclusion in recommendation ITU-R M.1457.

[2]	Document 8F/1079(Rev.1) – WiMAX Forum - Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial radio interface.

[3]	Document 8F/1082 - Considerations on adding IP-OFDMA as an IMT-2000 terrestrial radio interface.

# List of Annexes

Annex 1	Detailed evaluation procedures (as per Rec. ITU-R M.1225)

Annex 2	Israeli Evaluation Group evaluation criteria co-ordinators

Annex 3	Israeli comments

Annex 1

Detailed evaluation procedures (as per Rec. ITU-R M.1225)

| Number | Criteria | Most Important Technical Attributes (G1) |
| --- | --- | --- |
| A3.1 | Spectrum efficiency | – Voice traffic capacity (E/MHz/cell) – Information capacity (Mbit/s/MHz/cell) |
| A3.2 | Technology complexity – Effect on cost of installation and operation | – Peak transmitter/carrier (Pb) power – Broadband power amplifier (PA) – Number of users per RF carrier/frequency channel – Base site implementation/installation requirements – Handover complexity |
| A3.3 | Quality | – Maximum user bit rate for data (bit/s) – Voice quality |
| A3.4 | Flexibility of radio technologies | – Multimedia capabilities – Flexibility in the use of the frequency band – Minimum frequency band required to operate – Frequency management between different layers – Existing system migration capability |
| A3.5 | Implication on network interfaces | – Examine the network modifications required for the RTT to pass the standard set of ISDN bearer services |
| A3.6 | Handportable performance optimization capability | – Peak transmission power – Diversity schemes – The number of antennas – The number of receivers – The ratio of “off(sleep)” time to “on” time – Digital signal processing requirements |
| A3.7 | Coverage/power efficiency | – Base site coverage efficiency – Method to increase the coverage efficiency |

Annex 2

Israeli Evaluation Group (ISR-EVG) Evaluation Criteria Coordinators

| Number | Criteria | Co-ordinators |
| --- | --- | --- |
| A3.1 | Spectrum efficiency | Michael Livshitz (Schema) Alternate: Avi Freedman (Hexagon) |
| A3.2 | Technology complexity-effect on cost of installation and operation | Mariana Goldhamer (Alvarion) |
| A3.3 | Quality | Mariana Goldhamer (Alvarion) |
| A3.4 | Flexibility of radio technologies | Eli Sofer (Runcom) |
| A3.5 | Implication on network interfaces | Mariana Goldhamer (Alvarion) |
| A3.6 | Handportable performance optimisation capability | Eli Sofer (Runcom) |
| A3.7 | Coverage/power efficiency | Eli Sofer (Runcom) |

MoC Israel:     Haim Mazar, mazarh@moc.gov.il, mazar@ties.itu.int

Co-Chairs of the Israeli EVG:

Mariana Goldhamer, Alvarion, mariana.goldhamer@alvarion.com, +972544225548

Eli Sofer, Runcom, elisofer@runcom.co.il, +972544997996

Editor: 	 Avi Freedman, Hexagon, avif@hexagonltd.com, +972525620002

Annex 3

Israeli EVG Comments

# Source: Document 8F/1079(Rev. 1), Annex D Self Evaluation

| Index | Criteria and attributes | Q or q | Gn |  |  |  | Related attributes in Annex 1 |  |  |  |  |  |  |  |  |  |  |  | Proponents Comments | Israeli Evaluators Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3.1 | Spectrum efficiency : The following entries are considered in the evaluation of spectrum efficiency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.1.1 | For terrestrial environment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.1.1.1 | Voice traffic capacity (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. This metric must be used for a common generic continuous voice bearer with characteristics 8 kbit/s data rate and an average BER 1  10-3 as well as any other voice bearer included in the proposal which meets the quality requirements (assuming 50% voice activity detection (VAD) if it is used). For comparison purposes, all measures should assume the use of the deployment models in Annex 2, including a 1% call blocking. The descriptions should be consistent with the descriptions under criterion § 6.1.7 – Coverage/power efficiency. Any other assumptions and the background for the calculation should be provided, including details of any optional speech codecs being considered. | Q and q | G1 |  |  |  | A1.3.1.5.1 |  |  |  |  |  |  |  |  |  |  |  | TDD mode Voice capacity using VoIP: -90 Erlangs/MHz/cell for reuse 3, SIMO, 10 MHz PUSC Subchannelization -80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC Subchannelization Assumptions: -ITU vehicular path loss model -Pedestrian B3 channel model | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.1.1.2 | Information capacity (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. The information capacity is to be calculated for each test service or traffic mix for the appropriate test environments. This is the only measure that would be used in the case of multimedia, or for classes of services using multiple speech coding bit rates. Information capacity is the instantaneous aggregate user bit rate of all active users over all channels within the system on a per cell basis. If the user traffic (voice and/or data) is asymmetric and the system can take advantage of this characteristic to increase capacity, it should be described qualitatively for the purposes of evaluation. | Q and q | G1 |  |  |  | A1.3.1.5.2 |  |  |  |  |  |  |  |  |  |  |  | For the packet data bearer (UDD) service: Data capacity: -DL SIMO 5MHz= 3.45 Mbit/s/MHz/cell -DL SIMO 10MHz = 3.57 Mbit/s/MHz/cell -UL SIMO 5MHz = 1.6 Mbit/s/MHz/cell -DL MIMO 10MHz= 5.52 Mbit/s/MHz/cell -UL SIMO 10MHz= 1.59 Mbit/s/MHz/cell -UL MIMO 10MHz= 2.1 Mbit/s/MHz/cell Assumptions: - PUSC, ITU vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, -DL:UL=28:9 (payload only) | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.1.2 | For satellite environment These values (§ A3.1.2.1 and A3.1.2.2) assume the use of the simulation conditions in Annex 2. The first definition is valuable for comparing systems with identical user channel rates. The second definition is valuable for comparing systems with different voice and data channel rates. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.1.2.1 | Voice information capacity per required RF bandwidth (bit/s/Hz) | Q | G1 |  |  |  | A1.3.2.3.1 |  |  |  |  |  |  |  |  |  |  |  | NA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.1.2.2 | Voice plus data information capacity per required RF bandwidth (bit/s/Hz) | Q | G1 |  |  |  | A1.3.2.3.2 |  |  |  |  |  |  |  |  |  |  |  | NA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2 | Technology complexity – Effect on cost of installation and operation The considerations under criterion § 6.1.2 – Technology complexity apply only to the infrastructure, including BSs (the handportable performance is considered elsewhere). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.1 | Need for echo control The need for echo control is affected by the round trip delay, which is calculated as shown in Fig. 6. Referring to Fig. 6, consider the round trip delay with the vocoder (D1, ms) and also without that contributed by the vocoder (D2, ms). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. | Q | G4 |  |  |  | A1.3.7.2 A1.3.7.3 |  |  |  |  |  |  |  |  |  |  |  | Echo control is needed for voice applications. The voice delay is also dependent on the codec used. Selection of the codec is implementation dependent and no specific codec is mandated. Echo control is used on the MS and also optionally on a need basis at the BS or Gateways. The performance characteristics meet the delay requirements outlined in ITU-R M.1079. | We agree with this statement. We also note that the short MAC Frame duration of 5ms (see [1] clause 5.6.1.3.2) minimizes the requirements for the codec, such that no special voice codec is needed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.2 | Transmitter power and system linearity requirements NOTE 1 – Satellite e.i.r.p. is not suitable for evaluation and comparison of RTTs because it depends very much on satellite orbit. The RTT attributes in this section impact system cost and complexity, with the resultant desirable effects of improving overall performance in other evaluation criteria. They are as follows. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.2.1 | Peak transmitter/carrier (Pb) power (not applicable to satellite) | Q | G1 |  |  |  | A1.2.16.2.1 |  |  |  |  |  |  |  |  |  |  |  | This is not limited by RTT but rather by regulations for the specific RF bands. Mobile Station @ 2.5GHz 23 dBm EIRP (Power class I, QPSK, Refer to Section A3.2.2.2) | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Peak transmitter power for the BS should be considered because lower peak power contributes to lower cost. Note that Pb may vary with test environment application. This is the same peak transmitter power assumed in Appendix 2, link budget template (Error: Reference source not found). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | This is not limited by RTT but rather by regulations for the specific RF bands. | Agree; supplementary for cases in which the BS cell size is capacity limited, lower BS powers may be used. In this case the market will drive the range of available BS powers, as it is the case with other technologies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.2.2 | Broadband power amplifier (PA) (not applicable to satellite) Is a broadband power amplifier used or required? If so, what are the peak and average transmitted power requirements into the antenna as measured in watts. | Q | G1 |  |  |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 A1.5.5 A1.2.5 |  |  |  |  |  |  |  |  |  |  |  | A broadband power amplifier is required. Tx Power is not limited by RTT but by regulations. BS - Tx dynamic range = 10 dB - Spectral flatness as per conditions in A.1.4.10 - Peak Tx power on BS is limited only by regulations and not by the RTT. MS - Tx dynamic range = 45 dB - Spectral flatness as per conditions in A.1.4.10 - 4 power classes are supported as shown below: Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max | Agree; supplementary, this question is probably not relevant to broadband high-range technologies which obviously need to use a broadband amplifier. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.2.3 | Linear base transmitter and broadband amplifier requirements (not applicable to satellite) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.2.3.1 | Adjacent channel splatter/emission and intermodulation affect system capacity and performance. Describe these requirements and the linearity and filtering of the base transmitter and broadband PA required to achieve them. | q | G3 | A1.4.2 A1.4.10 |  |  |  |  |  |  |  |  |  |  | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |
| A3.2.2.3.2 | Also state the base transmitter and broadband PA (if one is used) peak to average transmitter output power, as a higher ratio requires greater linearity, heat dissipation and cost. | Q and q | G2 | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 |  |  |  |  |  |  |  |  |  |  | These are implementation dependent. The PAPR of the proposed RTT is around 12dB |  |  |  |  |  |  |  |  |  | Partially agree. The PAPR requirements are different for Base Station and mobile station. For the mobile station the regulatory requirements can be met with much lower PAPR. |  |  |  |  |  |  |  |  |  |  |
| A3.2.2.4 | Receiver linearity requirements (not applicable to satellite) Is BS receiver linearity required? If so, state the receiver dynamic range required and the impact of signal input variation exceeding this range, e.g., loss of sensitivity and blocking. | q | G4 | A1.4.11 A1.4.12 |  |  |  |  |  |  |  |  |  |  | BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBmBS/MS BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |
| A3.2.3 | Power control characteristics (not applicable to satellite) Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with BS technology complexity. | Q and q | G4 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.3 A1.2.22.4 A1.2.22.5 |  |  |  |  |  |  |  |  |  |  | Open loop and closed loop transmitter power control methods are used. Power control is done on the DL as well as the UL. Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. The power control cycle of closed-loop or open-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦± 1 dB ¦m¦ = 3dB¦± 1.5 dB 4dB <¦m¦< = 10 dB¦± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to ± 2 dB is allowed for any size step. The minimum power control dynamic range is 45 dB. The RTT supports 45 dB under the full power assumption |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |
| A3.2.4 | Transmitter/receiver isolation requirement (not applicable to satellite) If FDD is used, specify the noted requirement and how it is achieved. | q | G3 |  |  | A1.2.2 A1.2.2.2 A1.2.2.1 |  |  |  |  |  |  | Not Applicable as it is TDD. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.5 | Digital signal processing requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.5.1 | Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q | G2 |  |  | A1.4.13 |  |  |  |  |  |  | The Hardware requirements are implementation dependent. For 5 MHz a 512 FFT and for 10 MHz and 1024 FFT is required. Memory and Processing needs are very much specific to the type of hardware. |  |  |  |  |  |  |  |  |  | Agree. Technology progress allows today cost-effective implementations of systems having higher complexity as compared with 10 years ago, when the Rec. M.1225 was approved. |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.5.2 | What is the channel coding/error handling for both the forward and reverse links? Provide details and ensure that implementation specifics are described and their impact considered in DSP requirements described in § A3.2.5.1. | q | G4 |  |  | A1.2.12 A1.4.13 |  |  |  |  |  | An 8bit CRC is used for MAC PDU errors. Forward Error Correction schemes Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. |  |  |  |  |  |  |  |  |  | The adaptive modulation and coding is frequently used in the modern systems. We do not see a complexity factor here. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.6 | Antenna systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The implementation of specialized antenna systems while potentially increasing the complexity and cost of the overall system can improve spectrum efficiency (e.g. smart antennas), quality (e.g. diversity), and reduce system deployment costs (e.g. remote antennas, leaky feeder antennas). |  |  |  |  |  |  |  |  |  |  | MS: 1 Tx Antenna 2 Rx Antennas BS: 2 or more Tx Antennas 2 or more Rx Antennas Both MIMO and Beamforming support are mandatory at the Mobile Stations. Base Stations may support either MIMO or Beamforming. In general, it is expected for Beamforming to be deployed in scenarios where increased coverage is required (urban and suburban scenarios), while MIMO is expected to be employed in scenarios requiring high system capacity (urban scenarios). For MIMO operation: Adaptive switching between STC and SM is supported, see Section 1.3. 5 for a detailed description. Two transmit and two or more receive antennas are employed at the BS; one transmit and two receive antennas are supported at the MS. The typical antenna spacing at the BS and MS is 10 λ and 0.5 λ, respectively, where λ stands for the carrier wavelength. Regarding the type of equalizers for the SM MIMO mode, either minimum mean squared error (MMSE) or maximum-likelihhod (ML) based receivers will be implemented by MS vendors. Regarding the CSI, this is based either on physical or effective carrier-to-interference-and-noise ratio (CINR), while the communication of the MIMO mode is also enabled by the Mobile WiMAX system profiles. Please see also Section 1.3.5 for a detailed description. For Beamforming operation: Typically, a BS transceiver is equipped with 4 transmit and receive antennas but larger number of antennas can be used. The antenna spacing depends on the used Beamforming algorithm and can range from 0.5 λ to 3 λ. Regarding the weight update operation, see also Section 1.3.5, this is based on channel sounding, which is the process of channel estimation during the uplink operation for updating the antenna weights to be used for the subsequent transmission to a particular user in the downlink. Note that due to the channel reciprocity enabled by the TDD operation, the weights are accurate for low MS speeds, e.g., up to 30 km/h, while a graceful degradation of the performance is expected for higher speeds. Certainly, the accuracy of the antenna weights is also highly dependent on the specific Beamforming algorithm used at the BS, which may lead to smaller performance degradation at higher MS speeds. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1 – For the satellite component, diversity indicates the number of satellites involved; the other antenna attributes do not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.6.1 | Diversity : describe the diversity schemes applied (including micro and macro diversity schemes). Include in this description the degree of improvement expected, and the number of additional antennas and receivers required to implement the proposed diversity design beyond and omni-directional antenna. | Q | G2 |  |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  |  |  |  |  | When the MIMO option is deployed: In the downlink, both transmit diversity and receive diversity is supported through the use of STC (use of the Alamouti code which is a space-time block coding code for two transmit antennas, while two receive antennas are used at the MS for receive diversity). Note that when SM is used, although there is also inherent transmit and receive diversity due to the use of two antennas at both the BS and MS, the target is the increase of the peak rate by transmitting two data streams over one OFDMA symbol per subcarrier, see also Section 1.3.5 for a detailed description. In the uplink where CSM (collaborative spatial multiplexing) is supported, receive diversity is applied by the use of two or more receive antennas at the BS. Depending on the propagation environment (mainly characterized by the frequency and time diversity of the link-level channel model), the signal-to-noise ratio (SNR) gain of STC ranges from 4 dB to 7dB compared to a single antenna system; the SNR gain of SM ranges from 2 dB to 4 dB compared to a single antenna system, where there is double data throughput supported by SM compared to the single antenna system. Regarding the CSM mode, higher gains on the order of 1 dB to 2 dB are expected compared to the SM gains reported above. When the Beamforming option is applied: In the downlink, transmit diversity is supported, while receive diversity is also applied when two receive antennas are used at the MS. In the uplink, receive diversity is supported by using multiple antenna reception at the BS. For a typical implementation of 4 receive and transmit antennas for Bemaforming, the SNR gains at both the uplink and the downlink are expected to range from 6 dB to 12 dB. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.6.2 | Remote antennas : describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas. | q | G2 |  |  | A1.3.6 |  |  |  |  |  | These can be used for extending coverage. Performance is implementation and deployment scenario specific. |  |  |  |  |  |  |  |  |  | Agree. The distance between the remote antenna and the Base Station may go up to 150 meter. This is depending of product implementation and not of RT. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.6.3 | Distributed antennas : describe whether and how distributed antenna designs are used. | q | G3 |  |  | A1.3.6 |  |  |  |  |  | They can be used in microcellular environments. |  |  |  |  |  |  |  |  |  | Distributed antennas may be connected directly to the Base Station radio. This is depends on product implementation and not of RT. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.6.4 | Unique antenna : describe additional antenna systems which are either required or optional for the proposed system, e.g., beam shaping, leaky feeder. Include in the description the advantage or application of the antenna system. | q | G4 |  |  | A1.3.6 |  |  |  |  |  | MIMO and Beamforming types of Smart Antenna capability are supported. MIMO is used for capacity enhancements. Beamforming is used for coverage enhancement. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.7 | BS frequency synchronization/time alignment requirements Does the proposed RTT require base transmitter and/or receiver station synchronization or base-to-base bit time alignment? If so, specify the long term (1 year) frequency stability requirements, and also the required bit-to-bit time alignment. Describe the means of achieving this. | Q and q | G3 |  |  | A1.4.1 A1.4.3 |  |  |  |  | As it is a TDD system, BS synchronization is required. Methods used are implementation dependent. GPS based methods are typically used. BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing. Time alignment between BS and MS is achieved using the Downlink Preambles and the Uplink ranging operation which corrects time offset errors. The OFDMA Cyclic Prefix marks the Symbol level time alignment. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.8 | The number of users per RF carrier/frequency channel that the proposed RTT can support affects overall cost – especially as bearer traffic requirements increase or geographic traffic density varies widely with time. Specify the maximum number of user channels that can be supported while still meeting ITU-T Recommendation G.726 performance requirements for voice traffic. | Q | G1 |  |  | A1.2.17 |  |  |  |  | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The performance characteristics meet the delay and traffic requirements outlined in ITU-R M.1079. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.9 | Base site implementation/installation requirements (not applicable to satellite) BS size, mounting, antenna type and height can vary greatly as a function of cell size, RTT design and application environment. Discuss its positive or negative impact on system complexity and cost. | q | G1 |  |  | A1.4.17 |  |  |  |  | No RTT specific requirements exist. |  |  |  |  |  |  |  |  |  | We consider that the impact of antenna height and BS mounting cost are similar for all the technologies. The impact is not technology specific, but rather product specific. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.2.10 | Handover complexity Consistent with handover quality objectives defined in criterion § 6.1.3, describe how user handover is implemented for both voice and data services and its overall impact on infrastructure cost and complexity. | Q and q | G1 |  |  | A1.2.24 A1.4.6.1 |  |  |  |  | Simple Hard Handover and Optimized Hard Handover is supported. As the MS is only attached to one BS at a time significantly less complexity is expected. As voice is supported as an application over the IP data bearer the handover is always treated as a data connection. Base stations and Mobile stations implement the ability to buffer data during handover as well the protocols necessary for handover. See section 2.2.2.2 for handover performance analysis. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3 | Quality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.1 | Transparent reconnect procedure for dropped calls Dropped calls can result from shadowing and rapid signal loss. Air interfaces utilizing a transparent reconnect procedure – that is, the same as that employed for hand-off – mitigate against dropped calls whereas RTTs requiring a reconnect procedure significantly different from that used for hand-off do not. | q | G2 |  |  | A1.4.14 |  |  |  |  | Voice is supported as an application over the RTT. The RTT is primarily designed to support Voice using Voice Over IP Protocols. MAC connections that provide reliable Quality of Service for Voice Over IP data flows are supported. These data connections are managed using timers and well as MAC layer signaling to ensure a reliable connection is maintained. Transparent reconnects are provided by the application layer for the voice traffic. As the RTT supports Adaptive Modulation and Coding, and Link Adaptation methods, the MAC level transport connections are managed to make them reliable. |  |  |  |  |  |  |  |  |  | Agree. Supplementary, the air interface is not involved in call closing, which is at the IP level with VoIP protocols. The shadowing effect may be rather translated by some VoIP packet losses. The retransmissions will minimize these effects. The modern codecs have embedded the capability for mitigating the effect of 1-2 packet drops. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.2 | Round trip delay, D1 (with vocoder (ms)) and D2 (without vocoder (ms)) (See Fig. 6). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q | G2 |  |  | A1.3.7.1 A1.3.7.2 |  |  |  |  | Assuming G.729 with a vocoder delay of 20ms for a 20 Byte voice sample. D1 = 20ms (vocoder) + 50ms (max one-way air interface delay) x 2 = 120ms D2 = 50ms x 2 = 100ms |  |  |  |  |  |  |  |  |  | The delay parameter is actually implementation specific. We consider as a typical implementation parameters, for a 5ms MAC Frame duration: The air interface delay is max. 20ms. However, the voice coding process may take another 20ms and the voice decoding process may take also another 10ms. Our computation results: Coder delays: 50ms Two-way air interface: 50ms, including one retransmission Total delay: 100ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.3 | Handover/ALT quality Intra switch/controller handover directly affects voice service quality. Handover performance, minimum break duration, and average number of handovers are key issues. | Q | G2 |  |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  |  |  |  | Handover signalling is designed to minimize loss of data. Handover latency is <= 50ms if no network re-entry is required. This ensures minimum disruption to data transfer. If NW re-entry is required the latency is <= 85ms. Handover frequency is scenario specific. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.4 | Handover quality for data There should be a quantitative evaluation of the effect on data performance of handover. | Q | G3 |  |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  |  |  |  | Handover for voice and data are treated the same way in this RTT. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.5 | Maximum user bit rate for data (bit/s) A higher user bit rate potentially provides higher data service quality (such as high quality video service) from the user’s point of view. | Q | G1 |  |  | A1.3.3 |  |  |  |  | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.6 | Channel aggregation to achieve higher user bit There should also be a qualitative evaluation of the method used to aggregate channels to provide higher bit rate services. | q | G4 |  |  | A1.2.32 |  |  |  |  | No channel aggregation is necessary as IP-OFDMA can operate over the entire 10 MHz channel. However, flexible allocation of subchannels (in frequency domain) within an RF channel can be used to dynamically allocate bandwidth to individual users for various bit rate services (see also Section s 1.3.1 to 1.3.3) . |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.7 | Voice quality Recommendation ITU-R M.1079 specifies that FPLMTS speech quality without errors should be equivalent to ITU-T Recommendation G.726 (32 kbit/s ADPCM) with desired performance at ITU-T Recommendation G.711 (64 kbit/s PCM). NOTE 1 – Voice quality equivalent to ITU-T Recommendation G.726 error free with no more than a 0.5 degradation in MOS in the presence of 3% frame erasures might be a requirement. | Q and q | G1 |  |  | A1.2.19 A1.3.8 |  |  |  |  | The vocoder is independent of the RTT. Any suitable vocoder can be used as voice is supported over using Voice over IP protocol. Therefore the MOS values for the G.726 or any other vocoder used will apply. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.3.8 | System overload performance (not applicable to satellite) Evaluate the effect on system blocking and quality performance on both the primary and adjacent cells during an overload condition, at e.g. 125%, 150%, 175%, 200%. Also evaluate any other effects of an overload condition. | Q and q | G3 |  |  | A1.3.9.1 |  |  |  |  | System overload causes graceful degradation as data transmission bandwidth can be traded off for lower quality connections. As adaptive modulation and coding are supported the system adapts to the load conditions as per the policies implemented. |  |  |  |  |  |  |  |  |  | Agree. Supplementary, the traffic is classified and the services requiring QoS can be kept at same data rates, while the channel degradation will rather influence the Best-Effort data services. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4 | Flexibility of radio technologies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1 | Services aspects |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.1 | Variable user bit rate capabilities Variable user bit rate applications can consist of the following: – adaptive signal coding as a function of RF signal quality; – adaptive voice coder rate as a function of traffic loading as long as ITU-T Recommendation G.726 performance is met; – variable data rate as a function of user application; – variable voice/data channel utilization as a function of traffic mix requirements. Some important aspects which should be investigated are as follows: – how is variable bit rate supported? – what are the limitations? Supporting technical information should be provided such as – the range of possible data rates, – the rate of changes (ms). | q and Q | G2 |  |  | A1.2.18 A1.2.18.1 |  |  |  |  | The user bit rates are variable according to the number of subchannels assigned and modulation and coding rate used. The rates can be changed every 5ms which is every frame. The DL-MAP and UL-MAP signal the changes every frame. DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 Data rates: 9.6 kbit/s to 23040 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 Data rates: 9.6kbit/s to 6048 kbit/s |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.2 | Maximum tolerable Doppler shift, Fd (Hz) for which voice and data quality requirements are met (terrestrial only) Supporting technical information: Fd | q and Q | G3 |  |  | A1.3.1.4 |  |  |  |  | Fd ~500 Hz Voice and Data are treated the same way from the Physical layer perspective. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.3 | Doppler compensation method (satellite component only) What is the Doppler compensation method and residual Doppler shift after compensation? | Q and q | G3 |  |  | A1.3.2.2 |  |  |  |  | NA |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.4 | How the maximum tolerable delay spread of the proposed technology impact the flexibility (e.g., ability to cope with very high mobile speed)? | q | G3 |  |  | A1.3.1.3 A1.2.14 A1.2.14.1 A1.2.14.2 A1.3.10 |  |  |  |  | ~20µs of delay spread can be tolerated without an equalizer. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.5 | Maximum user information bit rate, Ru (kbit/s) How flexibly services can be offered to customers ? What is the limitation in number of users for each particular service? (e.g. no more than two simultaneous 2 Mbit/s users) | Q and q | G2 |  |  | A1.3.3 A1.3.1.5.2 A1.2.31 A1.2.32 |  |  |  |  | Assuming 10 MHz PUSC: - 23040 kbit/s for the Downlink (DL:UL=35:12) - 6048 kbit/s for the Uplink for (DL:UL=26:21) Services are very flexible as the Subchannels can be grouped to increase data rates. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.6 | Multiple vocoder rate capability – bit rate variability, – delay variability, – error protection variability. | Q and q | G3 |  |  | A1.2.19 A1.2.19.1 A1.2.7 |  |  |  |  | Yes. Vocoders are however independent of the RTT and are implementation specific. The data transports for voice can operate at varying levels of Packet error rate and using H-ARQ can significantly boost performance. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.1.7 | Multimedia capabilities The proponents should describe how multimedia services are handled. The following items should be evaluated: – possible limitations (in data rates, number of bearers), – ability to allocate extra bearers during of the communication, – constraints for handover. | Q and q | G1 |  |  | A1.2.21 A1.2.20 A1.3.1.5.2 A1.2.18 A1.2.24 A1.2.30 A1.2.30.1 |  |  |  |  | The Data bearers have no constraints on the type of media they can carry. However typically they are mapped to the QoS of the media type being transmitted. There are no limits on the number of bearers as long as bandwidth is available. Extra bearers can be allocated during communication. There are no handover constraints as long as coverage is available. |  |  |  |  |  |  |  |  |  | Multimedia applications such as video streaming, interactive applications, VoIPand high data throughput are addressed efficiently. This is due to the fact that 16e and Mobile WiMAX are optimised for large throughput/capacity DL and to natural broadcast capabilities to multiple users within coverage. WiMAX also supports massive DL traffic scenarios such as popular sports events through MBS, where resources of multiple BS`s are directed in coordinated manner to increase coverage and provide diversity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2 | Planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.1 | Spectrum related matters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.1.1 | Flexibility in the use of the frequency band The proponents should provide the necessary information related to this topic (e.g., allocation of sub-carriers with no constraints, handling of asymmetric services, usage of non-paired band). | q | G1 |  |  | A1.2.1 A1.2.2 A1.2.2.1 A1.2.3 A1.2.5.1 |  |  |  |  | A 5 MHz or 10 MHz TDD carrier may be deployed with 1:3:3 frequency re-use or 1:3:1 reuse. |  |  |  |  |  |  |  |  |  | In principle the operating BW in an OFDMA system is determined by two factors, the FFT size and the sampling frequency. Thus, using a small set of FFT sizes (512, 1k, 2k) and a large set of the simpler factor of sampling frequency lead to great flexibility in the sense of frequency usage. Moreover, WiMAX accommodates different Reuse factors such as 1/3 and 1 that allow optimal deployment in different cell configuration and a fine means to mitigate interference. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.1.2 | Spectrum sharing capabilities The proponent should indicate how global spectrum allocation can be shared between operators in the same region. The following aspects may be detailed: – means for spectrum sharing between operators in the same region, – guardband between operators in case of fixed sharing. | q and Q | G4 |  |  | A1.2.26 |  |  |  |  | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. So spectrum sharing is carried out using multiple channel carriers. The guard bands are RF band specific. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.1.3 | Minimum frequency band necessary to operate the system in good conditions Supporting technical information: – impact of the frequency reuse pattern, – bandwidth necessary to carry high peak data rate. | Q and q | G1 |  |  | A1.2.1 A1.4.15 A1.2.5 |  |  |  |  | 5 MHz or 10 MHz 1x3x3 PUSC or 1x3x1 PUSC may be used. 10 MHz gives the optimal data rate. |  |  |  |  |  |  |  |  |  | The flexibility of OFDMA technology in terms of frequency use allows applying it to small BW such as 1.25 MHz as exercised in satellite communications. The factor that determines spectral efficiency and the robustness of the system in mobility environment and other conditions (Doppler) is the carrier spacing. Maintaining sufficient carrier spacing allows the application of OFDMA with adequate QoS in small BW as the 1.25 MHz mentioned above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.2 | Radio resource planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.2.1 | Allocation of radio resources The proponents and evaluators should focus on the requirements and constraints imposed by the proposed technology. More particularly, the following aspects should be considered: – what are the methods used to make the allocation and planning of radio resources flexible? – what are the impacts on the network side (e.g. synchronization of BSs, signalling,)? – other aspects. Examples of functions or type of planning required which may be supported by the proposed technology: – DCA, – frequency hopping, – code planning, – time planning, – interleaved frequency planning. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. In some cases, no particular functions are necessary (e.g. frequency reuse  1). | q | G2 |  |  | A1.2.25 A1.2.27 A1.4.15 |  |  |  |  | Subchannelization schemes and zones namely PUSC and AMC are supported to provide flexibility in utilizing the frequency and time resources. Sectorized deployments are possible with flexible frequency re-use (1x3x3 or 1x3x1) using PUSC subchannelization schemes. Slots of multiple subchannels and OFDM symbols are used to manage the resource allocation granularity BSs need to be synchronized. This is typically done using GPS on the BS. No frequency planning is required across cells. |  |  |  |  |  |  |  |  |  | Agree. Supplementary: The PUSC and zoning gives flexibility to the planner. Frequency planning is not required, but, if used,provide advantages. TDD and zone boundaries are flexible, but should be synchronized across cells. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.2.2 | Adaptability to adapt to different and/or time varying conditions (e.g., propagation, traffic) How the proposed technology cope with varying propagation and/or traffic conditions? Examples of adaptive functions which may be supported by the proposed technology: – DCA, – link adaptation, – fast power control, – adaptation to large delay spreads. Some adaptivity aspects may be inherent to the RTT. | q | G2 |  |  | A1.3.10 A1.2.27 A1.2.22 A1.2.14 |  |  |  |  | Subchannelization and slot structure capability provides the ability to schedule frequency/time resources to mitigate the effects of propagation losses and also for traffic load balancing. Link adaptation schemes with CQI feedback capability allow operating the link more efficiently. H-ARQ also allows operations at high packet error rates resulting higher spectral efficiency as higher order coding and modulation rates can be used. The OFDMA symbol structure is designed to reduce the effects of delay spreads up to 20µs. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.3 | Mixed cell architecture (not applicable to satellite component) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.3.1 | Frequency management between different layers What kind of planning is required to manage frequencies between the different layers? e.g. – fixed separation, – dynamic separation, – possibility to use the same frequencies between different layers. Possible supporting technical information: – guard band. | q and Q | G1 |  |  | A1.2.28 A1.4.15 |  |  |  |  | Hierarchical layered cells are possible. The type of frequency planning is implementation/deployment scenario specific. The same frequencies can be used across layers by proper segmentation of the PUSC Subchannels. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.3.2 | User adaptation to the environment What are the constraints to the management of users between the different cell layers? e.g. – constraints for handover between different layers, – adaptation to the cell layers depending on services, mobile speed, mobile power. | q | G2 |  |  | A1.2.28 A1.3.10 |  |  |  |  | The RTT does not impose constraints on the management of users between different cell layers in such a hierarchical deployment. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.4 | Fixed-wireless access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.4.1 | The proponents should indicate how well its technology is suited for operation in the fixed wireless access environment. Areas which would need evaluation include (not applicable to satellite component): – ability to deploy small BSs easily, – use of repeaters, – use of large cells, – ability to support fixed and mobile users within a cell, – network and signaling simplification. | q | G4 |  |  | A1.1.3 A1.3.5 A1.4.17 A1.4.7 A1.4.7.1 |  |  |  |  | The RTT is very much suited for fixed wireless access as well. Pico or Micro cells or Macro cells and repeaters are possible. Both fixed and mobile users can work in the same cell. Network signaling for fixed devices are simpler compared to mobile devices. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.4.2 | Possible use of adaptive antennas (how well suited is the technology) (not applicable to satellite component) Is RTT suited to introduce adaptive antennas? Explain the reason if it is. | q | G4 |  |  |  |  | A1.3.6 |  |  |  |  |  | Yes the RTT supports adaptive antenna/Beamforming solutions. |  |  |  |  |  |  |  |  |  | The RTT supports a variety of spatial techniques, such as adaptive antennas, spatial multiplexing and diversity enhancement. Special means, such as private maps, dedicated pilots etc., were introduced to support that. |  |  |  |  |  |  |  |  |  |  |  |
| A3.4.2.4.3 | Existing system migration capability | q | G1 |  |  |  |  | A1.4.16 |  |  |  |  |  | NA |  |  |  |  |  |  |  |  |  | This is the first generation of this technology |  |  |  |  |  |  |  |  |  |  |  |
| A3.5 | Implication on network interface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.1 | Examine the synchronization requirements with respect to the network interfaces. Best case : no special accommodation necessary to provide synchronization. Worst case : special accommodation for synchronization is required, e.g. additional equipment at BS or special consideration for facilities. | q | G4 |  |  |  |  | A1.4.3 |  |  |  |  |  | Synchronization of the BSs across the network is required and this is typically accomplished using GPS. |  |  |  |  |  |  |  |  |  | We consider that the question refers to sync with E1/T1 network interfaces. A BS can implement various encapsulation methods of E1/T1 traffic (see TDM over IP draft RFC) and the synchronization will be acquired at the BS and propagated to MS/SS subscriber stations. |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.2 | Examine the RTTs ability to minimize the network infrastructure involvement in cell handover. Best case : neither PSTN/ISDN nor mobile switch involvement in handover. Worst case : landline network involvement essential for handover. | q | G3 |  |  |  |  | A1.2.24 A1.4.6.1 |  |  |  |  |  | Handover within the same ASN (Access Service Network) does not involve the CSN (Core Service Network). In most handover scenarios with neighboring cells there is minimal involvement of the CSN. Only the BS and ASN GW may need to be involved in these scenarios. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.3 | Landline feature transparency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.3.1 | Examine the network modifications required for the RTT to pass the standard set of ISDN bearer services. Best case : no modifications required. Worst case: substantial modification required, such as interworking functions. | q | G1 |  |  |  |  | A1.4.7.1 |  |  |  |  |  | ISDN is supported as an application running over the IP protocol and is not natively supported. As voice is supported using Voice over IP protocols, the use of ISDN is only involved interworking functions between the IP networks and PSTN. |  |  |  |  |  |  |  |  |  | Yes, the RTT is able to pass the basic bearer services: voice and data. The ISDN voice and data channels are converted to IP data at the Router levels, including VoIP Gateway functions. These routers/GWs are part of the operator infrastructure and are not related to the air interface. |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.3.2 | Examine the extent of the PSTN/ISDN involvement in switching functionality. Best case : all switching of calls is handled by the PSTN/ISDN. Worst case : a separate mobile switch is required. | q | G2 |  |  |  |  | A1.4.6 A1.4.8 |  |  |  |  | PSTN/ISDN is not used for switching within the IP network. |  |  |  |  |  |  |  |  |  | Agree. A VoIP Gateway may be needed to connect the legacy voice to the IP-OFDMA system. The combined IP/Legacy voice switching may need a VoIP “Soft Switch”, if a combination of VoIP and legacy voice needs to be supported. |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.3.3 | Examine the depth and duration of fading that would result in a dropped call to the PSTN/ISDN network. The robustness of an RTTs ability to minimize dropped calls could be provided by techniques such as transparent reconnect. | Q and q | G3 |  |  |  |  | A1.2.24 A1.4.14 |  |  |  |  | Voice is supported as an application over the RTT. The robustness of the link maintained is implementation dependent. The RTT supports HARQ and hence can operate in higher Packer Error Rates up to 10%. |  |  |  |  |  |  |  |  |  | Agree. Agree. |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.5.3.4 | Examine the quantity and type of network interfaces necessary for the RTT based on the deployment model used for spectrum and coverage efficiencies. The assessment should include those connections necessary for traffic, signalling and control as well as any special requirements, such as soft handover or simulcast. | Q | G2 |  |  |  |  |  | A1.2.30 A1.2.30.1 A1.4.9 |  |  |  |  |  |  |  | The RTT design is to minimize impacts on the network. All the connections necessary for traffic, signalling and control terminate on the BS for PHY/MAC layer. The Radio Resource Management functions implemented over the IP protocol reside in the ASN. So most RTT configuration parameters are controlled on the BS which is interfaced using an IP connection to the ASN-GW . |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |  |
| A3.6 | Handportable performance optimization capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A3.6.1 | Isolation between transmitter and receiver Isolation between transmitter and receiver has an impact on the size and weight of the handportable. | Q | G2 |  |  |  |  |  | A1.2.2 A1.2.2.1 A1.2.2.2 |  |  |  |  |  |  |  | As the RTT is a TDD based technology, no specific isolation requirements exist. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.2 | Average terminal power output P0 (mW) Lower power gives longer battery life and greater operating time. | Q | G2 |  |  |  |  |  | A1.2.16.1.2 |  |  |  |  |  |  |  | This is implementation dependent. The terminals have different power classes to which they belong as shown in A3.2.2.2.2. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.3 | System round trip delay impacts the amount of acoustical isolation required between hand portable microphone and speaker components and, as such, the physical size and mechanical design of the subscriber unit. NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q and q | G2 |  |  |  |  |  | A1.3.7 A1.3.7.1 A1.3.7.2 A1.3.7.3 |  |  |  |  |  |  |  | The Round trip delay will be well within the ITU-T specified limits for a typical Voice application that may be implemented using the RTT. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.4 | Peak transmission power | Q | G1 |  |  |  |  |  | A1.2.16.1.1 |  |  |  |  |  |  |  | This is not limited by RTT but by regulations. The peak terminal power output P0 = 1000 mW (Power class 3). Also see A3.2.2.2.2 for more details. |  |  |  |  |  |  |  |  |  | Peak transmission power should not exceed 30dbm (EIRP) subject to regulation (class 3). The increase in cell capacity will be achieved through sophisticated multiple antenna method in DL and UL, especially Receive Beamforming on the Base Station side. |  |  |  |  |  |  |  |  |
| A3.6.5 | Power control characteristics Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with mobile station technology complexity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes the RTT does utilize transmitter power control for both Downlink and Uplink. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.5.1 | Power control dynamic range Larger power control dynamic range gives longer battery life and greater operating time. | Q | G3 |  |  |  |  |  | A1.2.22 A1.2.22.3 A1.2.22.4 |  |  |  |  |  |  |  | The minimum power control dynamic range is 45 dB. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.5.2 | Power control step size, accuracy and speed | Q | G3 |  |  |  |  |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.5 |  |  |  |  |  |  |  | The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦ ± 1 dB ¦m¦ = 3dB¦ ± 1.5 dB 4dB< ¦m¦< = 10dB¦ ± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.6 | Linear transmitter requirements | q | G3 |  |  |  |  |  | A1.4.10 |  |  |  |  |  |  |  | Linear transmitters are used on the BS and MS. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.7 | Linear receiver requirements (not applicable to satellite) | q | G3 |  |  |  |  |  | A1.4.11 |  |  |  |  |  |  |  | Linear receivers are used on the BS and MS. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.8 | Dynamic range of receiver The lower the dynamic range requirement, the lower the complexity and ease of design implementation. | Q | G3 |  |  |  |  |  | A1.4.12 |  |  |  |  |  |  |  | 80dB for the MS receiver and 65dB for the BS receiver |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.9 | Diversity schemes Diversity has an impact on hand portable complexity and size. If utilized describe the type of diversity and address the following two attributes. | Q and q | G1 |  |  |  |  |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  |  |  |  |  |  |  | MIMO and Beamforming are supported. Within the MIMO scheme both Transmit Diversity and Spatial Multiplexing are supported. |  |  |  |  |  |  |  |  |  | In WiMAX/IP-OFDMA the BS and MS are endowed with multiple antennae. This enhanced physical condition implying multiple channels allow the application of diverse MIMO transmission and reception schemes. These may be divided into 3 categories: a. Spatial Multiplexing (SM), in which multiple information streams are transmitted simultaneously using identical frequency and temporal resources. In spatial multiplexing the link capacity is explicitly increased. SM may be employed both at the DL and UL. In the UL case the simultaneous transmissions are done by two users each endowed with a single Tx antenna. b. Diversity schemes such as Maximal Ratio Combining, Space Time Coding and their combinations are used in a way the identical information streams undergo multiple and diverse channels. Thus, diversity schemes significantly improve the channel condition which in turn allows increasing the link capacity through adaptive modulation and coding. c. Beamforming is a method to create spatial filtering with the use of multiple receives and/or transmit antennas by means of digital signal processing. In the case of Receive Beamforming (on the BS side) complex valued weights are calculated in order to maximize the link CINR. The implied counterpart in Transmit Beamforming in which the transmission weights are produced to create a directional transmission beam toward the MS. We further note that Tx and Rx Beamforming may be applied simultaneously to several MS thus employing Spatial Division Multiple Access (SDMA) |  |  |  |  |  |  |  |  |
| A3.6.10 | The number of antennas | Q | G1 |  |  |  |  |  | A1.2.23.1 |  |  |  |  |  |  |  | BS: 2 Tx, 2 Rx MS: 1 Tx, 2 Rx |  |  |  |  |  |  |  |  |  | The minimum number of antennas in order to employ the above mentioned MIMO and Diversity schemes is: BS: 2Tx, 2Rx; MS: 1Tx, 2Rx. The above antenna number allows efficient employment of SM in the UL (Collaborative) and DL as well as STS+MRC diversity schemes. However, in order to have an efficient Beamforming gain a larger number of Tx and Rx antennas on the BS side are required. Moreover, the efficiency of Tx beamforming is significantly (through the combination with SM) increased when 2Tx antennas are employed on the MS side. |  |  |  |  |  |  |  |  |
| A3.6.11 | The number of receivers | Q | G1 |  |  |  |  |  | A1.2.23.1 |  |  |  |  |  |  |  | BS: 2 Receivers MS : 2 Receivers |  |  |  |  |  |  |  |  |  | The number of receivers (receive chains) should correspond to the number of receive antennae as described in the previous item |  |  |  |  |  |  |  |  |
| A3.6.12 | Frequency stability Tight frequency stability requirements contribute to handportable complexity. | Q | G3 |  |  |  |  |  | A1.4.1.2 |  |  |  |  |  |  |  | BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.13 | The ratio of “off (sleep)” time to “on” time | Q | G1 |  |  |  |  |  | A1.2.29 A1.2.29.1 |  |  |  |  |  |  |  | This implementation dependent and is programmable by the BS or MS implementations. |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.14 | Frequency generator step size, switched speed and frequency range Tight step size, switch speed and wide frequency range contribute to handportable complexity. Conversely, they increase RTT flexibility. | Q | G2 |  |  |  |  |  | A1.4.5 |  |  |  |  |  |  |  | Frequency step size : 200 and 250 KHz Switched speed : 200 μsec Frequency range : 5, 10 MHz |  |  |  |  |  |  |  |  |  | Agree |  |  |  |  |  |  |  |  |
| A3.6.15 | Digital signal processing requirements Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q | G1 |  |  |  |  |  | A1.4.13 |  |  |  |  |  |  |  | These are again implementation dependent. |  |  |  |  |  |  |  |  |  | The requirements for DSP processing power are in line with the achievements of the commercial technology |  |  |  |  |  |  |  |  |
| A3.7.1.1 | Base site coverage efficiency The number of base sites required to provide coverage at system start-up and ongoing traffic growth significantly impacts cost. From § 1.3.2 of Annex 2, determine the coverage efficiency, C (km2/base sites), for the lowest traffic loadings. Proponent has to indicate the background of the calculation and also to indicate the maximum coverage range. | Q | G1 |  |  |  |  |  |  | A1.3.1.7 A1.3.1.7.1 A1.3.1.7.2 A1.3.4 |  |  |  |  |  |  |  | 80-95% at system startup 95-100% in a mature system See section 2.2.4.2 for more details. |  |  |  |  |  |  |  |  |  | Agree. |  |  |  |  |  |  |  |
| A3.7.1.2 | Method to increase the coverage efficiency Proponent describes the technique adopted to increase the coverage efficiency and drawbacks. Remote antenna systems can be used to economically extend vehicular coverage to low traffic density areas. RTT link budget, propagation delay system noise and diversity strategies can be impacted by their use. Distributed antenna designs – similar to remote antenna systems – interconnect multiple antennas to a single radio port via broadband lines. However, their application is not necessary limited to providing coverage, but can also be used to economically provide continuous building coverage for pedestrian applications. System synchronization, delay spread, and noise performance can be impacted by their use. | q | G1 |  |  |  |  |  |  | A1.3.5 A1.3.6 |  |  |  |  |  |  |  | MIMO and Beamforming can be used to increase coverage efficiency. Remote or Distributed antenna systems can also be used. However the use of these methods is deployment scenario specific based on the implementations. |  |  |  |  |  |  |  |  |  | Various methods exist to enhance the coverage. The most prominent are: a. Beamforming and other spatial diversity method that concentrate the transmitted energy towards specific users. b. The use of Pico-BS in malls and within buildings to increase QoS and coverage c. The application of smart repeaters that allow creating another dimension of diversity and increased coverage (cooperative Diversity). d. Coordinated and synchronized transmission in multi-cell deployment. The use of the above methods is deployment scenario specific. |  |  |  |  |  |  |  |
| A3.7.2 | Satellite Normalized power efficiency Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data mixed traffic. | Q | G1 |  |  |  |  |  |  | A1.3.2.4 A1.3.2.4.1 A1.3.2.4.2 |  |  |  |  |  |  |  | NA |  |  |  |  |  |  |  |  |  | N.A. |  |  |  |  |  |  |  |

________________