| Received: 15 May 2007 | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8F/1227-E |
|  |  | 15 May 2007 |
|  |  | English only |

Subject:		Question ITU-R 229-1/8

ARIB (Association of Radio Industries and Businesses)

ARIB Evaluation Report for “IP-OFDMA”

# Introduction

At the 21st meeting of ITU-R Working Party 8F, the Institute of Electrical and Electronics Engineers, (IEEE), in conjunction with the WiMAX Forum, submitted a proposed new IMT-2000 terrestrial radio interface named “IP-OFDMA” intended for inclusion in Rec. ITU-R M.1457 (“Detailed specifications of the radio interfaces of International Mobile Telecommunications-2000”).

The “IP-OFDMA” proposal was reviewed and considered at the 21st meeting of ITU-R WP 8F from 17 to 25 January 2007, in Yaoundé, Cameroon. As a result, Circular Letter 8/LCCE/153 was issued. In order to facilitate the consideration of the “IP-OFDMA” technology in an expeditious manner, Circular Letter kindly  requests contributions of evaluation results related to the “IP-OFDMA” submission be made on or before 8 May 2007 but WP 8F establishes the official contribution deadline as 16:00 hours UTC on 16 May 2007.

As a response to Circular Letter, ARIB formed ARIB Evaluation Group for performing IP-OFDMA evaluation in March 2007 with eighteen member companies under Standards Study Group of IMT-Advanced Subcommittee, Advanced Wireless Communications Study Committee (ADWICS).

Annex to this document is as follow:

Annex 1: Evaluation for Section 3 (“Self-Evaluation”) of Document 8F/1079(Rev.1)

# Scope

ARIB Evaluation Group discussed the scope of the work under Circular Letters 8/LCCE/95, 8/LCCE/47 and Recommendation ITU-R M.1225, and identified the necessity to evaluate Section 2.1 (“Radio Transmission Technology (RTT) Description Template”) and Section 3 (“Self-Evaluation” of Document 8F/1079(Rev.1) ) corresponding to Annex 1 and Annex 3 of Recommendation ITU-R M.1225. ARIB Evaluation Group also agreed that the assessment of Section 2.2 (“Requirements and Objective Template” of Document 8F/1079(Rev.1) ) and “Minimum Performance Capabilities” are the responsibility of ITU-R WP 8F. Thus, ARIB Evaluation Group did not include them in the scope of the discussion.

ARIB Evaluation Group took the approach to evaluate all items of Annex 1 and Annex 3 of Recommendation ITU-R M.1225 as stated above by members and discussed evaluator’s comments in face to face meetings. Thus, the ARIB evaluation report covers all the scope required under Circular Letters 8/LCCE/95, 8/LCCE/47 and Recommendation ITU-R M.1225.

After reviewing the proposal, ARIB Evaluation Group recognized that the simulations for the self-evaluation by the proponent do not use the assumptions defined in Recommendation ITU-R M.1225 in some cases. ARIB Evaluation Group also acknowledged that a self-evaluation doesn't necessarily need to be conducted based on the assumptions defined in Recommendation ITU-R M.1225.

ARIB Evaluation Group conducted evaluation based on the methodology defined in Recommendation ITU-R M.1225. ARIB Evaluation Group asked members to conduct coverage and spectral efficiency evaluations. Three members responded to the request and submitted the evaluation results. One member conducted the evaluations based on the assumptions defined in Recommendation ITU-R M.1225 and thus these are included in Chapter 4.3. The other two members conducted the evaluations based on the assumptions used in Document 8F/1079(Rev.1). As a result of the discussions, ARIB Evaluation Group concluded not to include these simulation results in Chapter 4.3 of the report. These two members wished to leave their questions and comments based on their simulation results and these are included in Chapter 4.4 of this report.

No member did have a chance to conduct simulation for voice capacity based on Recommendation ITU-R M.1225. Therefore, ARIB has no results on voice capacity evaluation and we need to depend on other evaluation groups for this evaluation. However, all other evaluation items have been through complete evaluation process, and thus the quality of the evaluation is reliable.

Two volunteered members of ARIB Evaluation Group participated in IEEE hosted IP-OFDMA workshop at IEEE 802 meeting in Orlando, Florida in March 2007. ARIB Evaluation Group had a face to face meeting with a WCA representative and shared information on the status of each evaluation group, which was held for gaining consensus between evaluation groups through coordination between evaluation groups recommended in Circular Letters 8/LCCE/95 and 8/LCCE/47.

# Objectives of Evaluation

The purposes of ARIB evaluation activities on “IP-OFDMA” proposed by IEEE are listed below;

- to confirm that all IMT-2000 requirements and objectives are fulfilled

- to contribute to fair and proper evaluation at ITU-R and other evaluation groups

# 4          Summary of Evaluation

## 4.1 	Evaluation for Section 3 (“Self-Evaluation”) of Document 8F/1079(Rev.1)

Evaluator’s comments for Self-Evaluation of Document 8F/1079(Rev.1) are found in Annex 1. This is a core part of this evaluation report.

## 4.2 	Evaluation for Section 2.1 (“Radio Transmission Technology (RTT) Description Template”) of Document 8F/1079(Rev.1)

Evaluator’s comments for Radio Transmission Technology (RTT) Description Template of Document 8F/1079(Rev.1) are found in Appendix 1. This is a complimentary part to Chapter 4.1.

## 4.3 	Evaluation for Section 2.3 (“Capacity and Coverage”) of Document 8F/1079(Rev.1)

Detail of coverage efficiency evaluations and simulation conditions for spectral efficiency calculation are attached in Appendix 2. This supports some of evaluator’s comments in Chapter 4.1.

## 4.4 	Questionnaire in the Evaluation Process

Appendix 3 shows the questions that were forwarded to IEEE and WiMAX Forum for clarification in the evaluation process, with the answers returned. Some of the questions are not answered when this version of this report was produced and ARIB is waiting for responses. However, the evaluation is completed without the help of these unanswered questions.

# 5	Conclusion

ARIB Evaluation Group announces the fact that all proponent’s comments in Section 3 of Document 8F/1079(Rev.1) corresponding to Annex 3 of Recommendation ITU-R M.1225 are satisfactory but for A3.1.1.1 and A3.1.1.2. ARIB expects that this evaluation report will be fully reflected to the discussions at the 22nd meeting of ITU-R WP 8F from 23 to 31 May 2007, in Kyoto, Japan, in consideration of the “IP-OFDMA” technology as the 6th IMT-2000 terrestrial radio interface in Recommendation ITU-R M.1457.

Through the evaluation process, ARIB acknowledged that some parameters in Recommendation ITU-R M.1225 do not entirely meet the current evaluation environment. ARIB proposes that this is considered during the evaluation process in WP 8F. ARIB also believes that it is desirable that the evaluation process defines that the proponent’s self-evaluation is based on Recommendation ITU-R M.1225.

Annex: 1

Annex 1

ARIB (Association of Radio Industries and Businesses)

Evaluation for Section 3 (“Self-Evaluation”) of Document 8F/1079(Rev.1)

This annex is the table of evaluator’s comments for Annex 3 of Document 8F/1079(Rev.1), “Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial Radio Interface“.

| Index | Criteria and attributes |  | Q or q |  | Gn |  | Related attributes in Annex 1 | Proponents Comments | Evaluators Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3.1 | Spectrum efficiency : The following entries are considered in the evaluation of spectrum efficiency |  |  |  |  |  |  |  |  |
| A3.1.1 | For terrestrial environment |  |  |  |  |  |  |  |  |
| A3.1.1.1 | Voice traffic capacity (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. This metric must be used for a common generic continuous voice bearer with characteristics 8 kbit/s data rate and an average BER 1 10-3 as well as any other voice bearer included in the proposal which meets the quality requirements (assuming 50% voice activity detection (VAD) if it is used). For comparison purposes, all measures should assume the use of the deployment models in Annex 2, including a 1% call blocking. The descriptions should be consistent with the descriptions under criterion § 6.1.7 – Coverage/power efficiency. Any other assumptions and the background for the calculation should be provided, including details of any optional speech codecs being considered. |  | Q and q |  | G1 |  | A1.3.1.5.1 | TDD mode Voice capacity using VoIP: -90 Erlangs/MHz/cell for reuse 3, SIMO, 10 MHz PUSC Subchannelization -80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC Subchannelization Assumptions: -ITU vehicular path loss model -Pedestrian B3 channel model | The ARIB evaluation group could not confirm values in proponent’s comments of this item, since no simulation based on the Recommendation ITU-R M.1225 was conducted by the ARIB evaluation group. The Q&A sessions on this item can be referred to ARIB-1, ARIB-2, ARIB-3 and ARIB-12 in Appendix 3. |
| A3.1.1.2 | Information capacity (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. The information capacity is to be calculated for each test service or traffic mix for the appropriate test environments. This is the only measure that would be used in the case of multimedia, or for classes of services using multiple speech coding bit rates. Information capacity is the instantaneous aggregate user bit rate of all active users over all channels within the system on a per cell basis. If the user traffic (voice and/or data) is asymmetric and the system can take advantage of this characteristic to increase capacity, it should be described qualitatively for the purposes of evaluation. |  | Q and q |  | G1 |  | A1.3.1.5.2 | For the packet data bearer (UDD) service: Data capacity: -DL SIMO 5MHz= 3.45 Mbit/s/MHz/cell -DL SIMO 10MHz = 3.57 Mbit/s/MHz/cell -UL SIMO 5MHz = 1.6 Mbit/s/MHz/cell -DL MIMO 10MHz= 5.52 Mbit/s/MHz/cell -UL SIMO 10MHz= 1.59 Mbit/s/MHz/cell -UL MIMO 10MHz= 2.1 Mbit/s/MHz/cell Assumptions: - PUSC, ITU vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, -DL:UL=28:9 (payload only) | The ARIB evaluation group conducted the spectral efficiency evaluation based on conditions stated as the ITU-R Recommendation M.1225. The evaluation results are as follows: Data capacity: - DL Indoor Office = 1.47 Mbit/s/MHz/cell - DL Outdoor-to-Indoor Pedestrian = 1.37 Mbit/s/MHz/cell - DL Pedestrian Outdoor = 1.37 Mbit/s/MHz/cell - DL Vehicular = 2.82 Mbit/s/MHz/cell - UL Indoor Office = 0.92 Mbit/s/MHz/cell - UL Outdoor-to-Indoor Pedestrian = 0.88 Mbit/s/MHz/cell - UL Pedestrian Outdoor = 0.90 Mbit/s/MHz/cell - UL Vehicular = 2.20 Mbit/s/MHz/cell Please see Appendix 2 for the detail simulation conditions. The Q&A sessions on this item can be referred to ARIB-10 and ARIB-11 in Appendix 3. |
| A3.1.2 | For satellite environment These values (§ A3.1.2.1 and A3.1.2.2) assume the use of the simulation conditions in Annex 2. The first definition is valuable for comparing systems with identical user channel rates. The second definition is valuable for comparing systems with different voice and data channel rates. |  |  |  |  |  |  |  |  |
| A3.1.2.1 | Voice information capacity per required RF bandwidth (bit/s/Hz) |  | Q |  | G1 |  | A1.3.2.3.1 | NA |  |
| A3.1.2.2 | Voice plus data information capacity per required RF bandwidth (bit/s/Hz) |  | Q |  | G1 |  | A1.3.2.3.2 | NA |  |
| A3.2 | Technology complexity – Effect on cost of installation and operation The considerations under criterion § 6.1.2 – Technology complexity apply only to the infrastructure, including BSs (the handportable performance is considered elsewhere). |  |  |  |  |  |  |  |  |
| A3.2.1 | Need for echo control The need for echo control is affected by the round trip delay, which is calculated as shown in Fig. 6. Referring to Fig. 6, consider the round trip delay with the vocoder (D1, ms) and also without that contributed by the vocoder (D2, ms). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. |  | Q |  | G4 |  | A1.3.7.2 A1.3.7.3 | Echo control is needed for voice applications. The voice delay is also dependent on the codec used. Selection of the codec is implementation dependent and no specific codec is mandated. Echo control is used on the MS and also optionally on a need basis at the BS or Gateways. The performance characteristics meet the delay requirements outlined in ITU-R M.1079. | Self-evaluation comments are satisfactory. The Q&A sessions on this item can be referred to ARIB-4 in Appendix 3. |
| A3.2.2 | Transmitter power and system linearity requirements NOTE 1 – Satellite e.i.r.p. is not suitable for evaluation and comparison of RTTs because it depends very much on satellite orbit. The RTT attributes in this section impact system cost and complexity, with the resultant desirable effects of improving overall performance in other evaluation criteria. They are as follows. |  |  |  |  |  |  |  |  |
| A3.2.2.1 | Peak transmitter/carrier (Pb) power (not applicable to satellite) |  | Q |  | G1 |  | A1.2.16.2.1 | This is not limited by RTT but rather by regulations for the specific RF bands. Mobile Station @ 2.5GHz 23 dBm EIRP (Power class I, QPSK, Refer to Section A3.2.2.2) | Self-evaluation comments are satisfactory. |
|  | Peak transmitter power for the BS should be considered because lower peak power contributes to lower cost. Note that Pb may vary with test environment application. This is the same peak transmitter power assumed in Appendix 2, link budget template (Error: Reference source not found). |  |  |  |  |  |  | This is not limited by RTT but rather by regulations for the specific RF bands. | Self-evaluation comments are satisfactory. |
| A3.2.2.2 | Broadband power amplifier (PA) (not applicable to satellite) Is a broadband power amplifier used or required? If so, what are the peak and average transmitted power requirements into the antenna as measured in watts. |  | Q |  | G1 |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 A1.5.5 A1.2.5 | A broadband power amplifier is required. Tx Power is not limited by RTT but by regulations. BS - Tx dynamic range = 10 dB - Spectral flatness as per conditions in A.1.4.10 - Peak Tx power on BS is limited only by regulations and not by the RTT. MS - Tx dynamic range = 45 dB - Spectral flatness as per conditions in A.1.4.10 - 4 power classes are supported as shown below: Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max | Self-evaluation comments are satisfactory. |
| A3.2.2.3 | Linear base transmitter and broadband amplifier requirements (not applicable to satellite) |  |  |  |  |  |  |  |  |
| A3.2.2.3.1 | Adjacent channel splatter/emission and intermodulation affect system capacity and performance. Describe these requirements and the linearity and filtering of the base transmitter and broadband PA required to achieve them. | q |  | G3 |  | A1.4.2 A1.4.10 |  | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. | Self-evaluation comments are satisfactory. |
| A3.2.2.3.2 | Also state the base transmitter and broadband PA (if one is used) peak to average transmitter output power, as a higher ratio requires greater linearity, heat dissipation and cost. | Q and q |  | G2 |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 |  | These are implementation dependent. The PAPR of the proposed RTT is around 12dB | Self-evaluation comments are satisfactory. |
| A3.2.2.4 | Receiver linearity requirements (not applicable to satellite) Is BS receiver linearity required? If so, state the receiver dynamic range required and the impact of signal input variation exceeding this range, e.g., loss of sensitivity and blocking. | q |  | G4 |  | A1.4.11 A1.4.12 |  | BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBmBS/MS BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. | Self-evaluation comments are satisfactory. The Q&A sessions on this item can be referred to ARIB-9 in Appendix 3. |
| A3.2.3 | Power control characteristics (not applicable to satellite) Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with BS technology complexity. | Q and q |  | G4 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.3 A1.2.22.4 A1.2.22.5 |  | Open loop and closed loop transmitter power control methods are used. Power control is done on the DL as well as the UL. Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. The power control cycle of closed-loop or open-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦± 1 dB ¦m¦ = 3dB¦± 1.5 dB 4dB <¦m¦< = 10 dB¦± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to ± 2 dB is allowed for any size step. The minimum power control dynamic range is 45 dB. The RTT supports 45 dB under the full power assumption | Self-evaluation comments are satisfactory. |
| A3.2.4 | Transmitter/receiver isolation requirement (not applicable to satellite) If FDD is used, specify the noted requirement and how it is achieved. | q |  | G3 |  | A1.2.2 A1.2.2.2 A1.2.2.1 |  | Not Applicable as it is TDD. | Self-evaluation comments are satisfactory. |
| A3.2.5 | Digital signal processing requirements |  |  |  |  |  |  |  |  |
| A3.2.5.1 | Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q |  | G2 |  | A1.4.13 |  | The Hardware requirements are implementation dependent. For 5 MHz a 512 FFT and for 10 MHz and 1024 FFT is required. Memory and Processing needs are very much specific to the type of hardware. | Self-evaluation comments are satisfactory. |
| A3.2.5.2 | What is the channel coding/error handling for both the forward and reverse links? Provide details and ensure that implementation specifics are described and their impact considered in DSP requirements described in § A3.2.5.1. | q |  | G4 |  | A1.2.12 A1.4.13 |  | An 8bit CRC is used for MAC PDU errors. Forward Error Correction schemes Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. | Self-evaluation comments are satisfactory. Doc. 8F/1183 mentioned that “8 bits CRC” should be corrected as “32 bits CRC”. |
| A3.2.6 | Antenna systems |  |  |  |  |  |  |  |  |
|  | The implementation of specialized antenna systems while potentially increasing the complexity and cost of the overall system can improve spectrum efficiency (e.g. smart antennas), quality (e.g. diversity), and reduce system deployment costs (e.g. remote antennas, leaky feeder antennas). |  |  |  |  |  |  | MS: 1 Tx Antenna 2 Rx Antennas BS: 2 or more Tx Antennas 2 or more Rx Antennas Both MIMO and Beamforming support are mandatory at the Mobile Stations. Base Stations may support either MIMO or Beamforming. In general, it is expected for Beamforming to be deployed in scenarios where increased coverage is required (urban and suburban scenarios), while MIMO is expected to be employed in scenarios requiring high system capacity (urban scenarios). For MIMO operation: Adaptive switching between STC and SM is supported, see Section 1.3. 5 for a detailed description. Two transmit and two or more receive antennas are employed at the BS; one transmit and two receive antennas are supported at the MS. The typical antenna spacing at the BS and MS is 10 λ and 0.5 λ, respectively, where λ stands for the carrier wavelength. Regarding the type of equalizers for the SM MIMO mode, either minimum mean squared error (MMSE) or maximum-likelihhod (ML) based receivers will be implemented by MS vendors. Regarding the CSI, this is based either on physical or effective carrier-to-interference-and-noise ratio (CINR), while the communication of the MIMO mode is also enabled by the Mobile WiMAX system profiles. Please see also Section 1.3.5 for a detailed description. For Beamforming operation: Typically, a BS transceiver is equipped with 4 transmit and receive antennas but larger number of antennas can be used. The antenna spacing depends on the used Beamforming algorithm and can range from 0.5 λ to 3 λ. Regarding the weight update operation, see also Section 1.3.5, this is based on channel sounding, which is the process of channel estimation during the uplink operation for updating the antenna weights to be used for the subsequent transmission to a particular user in the downlink. Note that due to the channel reciprocity enabled by the TDD operation, the weights are accurate for low MS speeds, e.g., up to 30 km/h, while a graceful degradation of the performance is expected for higher speeds. Certainly, the accuracy of the antenna weights is also highly dependent on the specific Beamforming algorithm used at the BS, which may lead to smaller performance degradation at higher MS speeds. | Self-evaluation comments are satisfactory. |
|  | NOTE 1 – For the satellite component, diversity indicates the number of satellites involved; the other antenna attributes do not apply. |  |  |  |  |  |  |  |  |
| A3.2.6.1 | Diversity : describe the diversity schemes applied (including micro and macro diversity schemes). Include in this description the degree of improvement expected, and the number of additional antennas and receivers required to implement the proposed diversity design beyond and omni-directional antenna. | Q |  | G2 |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  | When the MIMO option is deployed: In the downlink, both transmit diversity and receive diversity is supported through the use of STC (use of the Alamouti code which is a space-time block coding code for two transmit antennas, while two receive antennas are used at the MS for receive diversity). Note that when SM is used, although there is also inherent transmit and receive diversity due to the use of two antennas at both the BS and MS, the target is the increase of the peak rate by transmitting two data streams over one OFDMA symbol per subcarrier, see also Section 1.3.5 for a detailed description. In the uplink where CSM (collaborative spatial multiplexing) is supported, receive diversity is applied by the use of two or more receive antennas at the BS. Depending on the propagation environment (mainly characterized by the frequency and time diversity of the link-level channel model), the signal-to-noise ratio (SNR) gain of STC ranges from 4 dB to 7dB compared to a single antenna system; the SNR gain of SM ranges from 2 dB to 4 dB compared to a single antenna system, where there is double data throughput supported by SM compared to the single antenna system. Regarding the CSM mode, higher gains on the order of 1 dB to 2 dB are expected compared to the SM gains reported above. When the Beamforming option is applied: In the downlink, transmit diversity is supported, while receive diversity is also applied when two receive antennas are used at the MS. In the uplink, receive diversity is supported by using multiple antenna reception at the BS. For a typical implementation of 4 receive and transmit antennas for Bemaforming, the SNR gains at both the uplink and the downlink are expected to range from 6 dB to 12 dB. | Self-evaluation comments are satisfactory under ideal conditions. |
| A3.2.6.2 | Remote antennas : describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas. | q |  | G2 |  | A1.3.6 |  | These can be used for extending coverage. Performance is implementation and deployment scenario specific. | It is outside the object of evaluation. |
| A3.2.6.3 | Distributed antennas : describe whether and how distributed antenna designs are used. | q |  | G3 |  | A1.3.6 |  | They can be used in microcellular environments. | It is outside the object of evaluation. |
| A3.2.6.4 | Unique antenna : describe additional antenna systems which are either required or optional for the proposed system, e.g., beam shaping, leaky feeder. Include in the description the advantage or application of the antenna system. | q |  | G4 |  | A1.3.6 |  | MIMO and Beamforming types of Smart Antenna capability are supported. MIMO is used for capacity enhancements. Beamforming is used for coverage enhancement. | Self-evaluation comments are satisfactory. |
| A3.2.7 | BS frequency synchronization/time alignment requirements Does the proposed RTT require base transmitter and/or receiver station synchronization or base-to-base bit time alignment? If so, specify the long term (1 year) frequency stability requirements, and also the required bit-to-bit time alignment. Describe the means of achieving this. | Q and q |  | G3 |  | A1.4.1 A1.4.3 |  | As it is a TDD system, BS synchronization is required. Methods used are implementation dependent. GPS based methods are typically used. BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing. Time alignment between BS and MS is achieved using the Downlink Preambles and the Uplink ranging operation which corrects time offset errors. The OFDMA Cyclic Prefix marks the Symbol level time alignment. | Self-evaluation comments are satisfactory. |
| A3.2.8 | The number of users per RF carrier/frequency channel that the proposed RTT can support affects overall cost – especially as bearer traffic requirements increase or geographic traffic density varies widely with time. Specify the maximum number of user channels that can be supported while still meeting ITU-T Recommendation G.726 performance requirements for voice traffic. | Q |  | G1 |  | A1.2.17 |  | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The performance characteristics meet the delay and traffic requirements outlined in ITU-R M.1079. | Self-evaluation comments are satisfactory. |
| A3.2.9 | Base site implementation/installation requirements (not applicable to satellite) BS size, mounting, antenna type and height can vary greatly as a function of cell size, RTT design and application environment. Discuss its positive or negative impact on system complexity and cost. | q |  | G1 |  | A1.4.17 |  | No RTT specific requirements exist. | Self-evaluation comments are satisfactory. |
| A3.2.10 | Handover complexity Consistent with handover quality objectives defined in criterion § 6.1.3, describe how user handover is implemented for both voice and data services and its overall impact on infrastructure cost and complexity. | Q and q |  | G1 |  | A1.2.24 A1.4.6.1 |  | Simple Hard Handover and Optimized Hard Handover is supported. As the MS is only attached to one BS at a time significantly less complexity is expected. As voice is supported as an application over the IP data bearer the handover is always treated as a data connection. Base stations and Mobile stations implement the ability to buffer data during handover as well the protocols necessary for handover. See section 2.2.2.2 for handover performance analysis. | Self-evaluation comments are satisfactory. Doc. 8F/1183 mentioned that they correct the reference from “Section 2.2.2.2” to “Section 2.3.2”. |
| A3.3 | Quality |  |  |  |  |  |  |  |  |
| A3.3.1 | Transparent reconnect procedure for dropped calls Dropped calls can result from shadowing and rapid signal loss. Air interfaces utilizing a transparent reconnect procedure – that is, the same as that employed for hand-off – mitigate against dropped calls whereas RTTs requiring a reconnect procedure significantly different from that used for hand-off do not. | q |  | G2 |  | A1.4.14 |  | Voice is supported as an application over the RTT. The RTT is primarily designed to support Voice using Voice Over IP Protocols. MAC connections that provide reliable Quality of Service for Voice Over IP data flows are supported. These data connections are managed using timers and well as MAC layer signaling to ensure a reliable connection is maintained. Transparent reconnects are provided by the application layer for the voice traffic. As the RTT supports Adaptive Modulation and Coding, and Link Adaptation methods, the MAC level transport connections are managed to make them reliable. | Self-evaluation comments are satisfactory. |
| A3.3.2 | Round trip delay, D1 (with vocoder (ms)) and D2 (without vocoder (ms)) (See Fig. 6). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q |  | G2 |  | A1.3.7.1 A1.3.7.2 |  | Assuming G.729 with a vocoder delay of 20ms for a 20 Byte voice sample. D1 = 20ms (vocoder) + 50ms (max one-way air interface delay) x 2 = 120ms D2 = 50ms x 2 = 100ms | Self-evaluation comments are satisfactory. |
| A3.3.3 | Handover/ALT quality Intra switch/controller handover directly affects voice service quality. Handover performance, minimum break duration, and average number of handovers are key issues. | Q |  | G2 |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  | Handover signaling is designed to minimize loss of data. Handover latency is <= 50ms if no network re-entry is required. This ensures minimum disruption to data transfer. If NW re-entry is required the latency is <= 85ms. Handover frequency is scenario specific. | Self-evaluation comments are satisfactory. |
| A3.3.4 | Handover quality for data There should be a quantitative evaluation of the effect on data performance of handover. | Q |  | G3 |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  | Handover for voice and data are treated the same way in this RTT. | Self-evaluation comments are satisfactory. |
| A3.3.5 | Maximum user bit rate for data (bit/s) A higher user bit rate potentially provides higher data service quality (such as high quality video service) from the user’s point of view. | Q |  | G1 |  | A1.3.3 |  | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) | Self-evaluation comments are satisfactory. |
| A3.3.6 | Channel aggregation to achieve higher user bit There should also be a qualitative evaluation of the method used to aggregate channels to provide higher bit rate services. | q |  | G4 |  | A1.2.32 |  | No channel aggregation is necessary as IP-OFDMA can operate over the entire 10 MHz channel. However, flexible allocation of subchannels (in frequency domain) within an RF channel can be used to dynamically allocate bandwidth to individual users for various bit rate services (see also Section s 1.3.1 to 1.3.3) . | Self-evaluation comments are satisfactory. |
| A3.3.7 | Voice quality Recommendation ITU-R M.1079 specifies that FPLMTS speech quality without errors should be equivalent to ITU-T Recommendation G.726 (32 kbit/s ADPCM) with desired performance at ITU-T Recommendation G.711 (64 kbit/s PCM). NOTE 1 – Voice quality equivalent to ITU-T Recommendation G.726 error free with no more than a 0.5 degradation in MOS in the presence of 3% frame erasures might be a requirement. | Q and q |  | G1 |  | A1.2.19 A1.3.8 |  | The vocoder is independent of the RTT. Any suitable vocoder can be used as voice is supported over using Voice over IP protocol. Therefore the MOS values for the G.726 or any other vocoder used will apply. | Self-evaluation comments are satisfactory. The ARIB evaluation group points out that the low bit rate voice coder testing is subjective and may result in large variances from test to test. |
| A3.3.8 | System overload performance (not applicable to satellite) Evaluate the effect on system blocking and quality performance on both the primary and adjacent cells during an overload condition, at e.g. 125%, 150%, 175%, 200%. Also evaluate any other effects of an overload condition. | Q and q |  | G3 |  | A1.3.9.1 |  | System overload causes graceful degradation as data transmission bandwidth can be traded off for lower quality connections. As adaptive modulation and coding are supported the system adapts to the load conditions as per the policies implemented. | Self-evaluation comments are satisfactory. |
| A3.4 | Flexibility of radio technologies |  |  |  |  |  |  |  |  |
| A3.4.1 | Services aspects |  |  |  |  |  |  |  |  |
| A3.4.1.1 | Variable user bit rate capabilities Variable user bit rate applications can consist of the following: – adaptive signal coding as a function of RF signal quality; – adaptive voice coder rate as a function of traffic loading as long as ITU-T Recommendation G.726 performance is met; – variable data rate as a function of user application; – variable voice/data channel utilization as a function of traffic mix requirements. Some important aspects which should be investigated are as follows: – how is variable bit rate supported? – what are the limitations? Supporting technical information should be provided such as – the range of possible data rates, – the rate of changes (ms). | q and Q |  | G2 |  | A1.2.18 A1.2.18.1 |  | The user bit rates are variable according to the number of subchannels assigned and modulation and coding rate used. The rates can be changed every 5ms which is every frame. The DL-MAP and UL-MAP signal the changes every frame. DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 Data rates: 9.6 kbit/s to 23040 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 Data rates: 9.6kbit/s to 6048 kbit/s | Self-evaluation comments are satisfactory. |
| A3.4.1.2 | Maximum tolerable Doppler shift, Fd (Hz) for which voice and data quality requirements are met (terrestrial only) Supporting technical information: Fd | q and Q |  | G3 |  | A1.3.1.4 |  | Fd ~500 Hz Voice and Data are treated the same way from the Physical layer perspective. | Self-evaluation comments are satisfactory. |
| A3.4.1.3 | Doppler compensation method (satellite component only) What is the Doppler compensation method and residual Doppler shift after compensation? | Q and q |  | G3 |  | A1.3.2.2 |  | NA |  |
| A3.4.1.4 | How the maximum tolerable delay spread of the proposed technology impact the flexibility (e.g., ability to cope with very high mobile speed)? | q |  | G3 |  | A1.3.1.3 A1.2.14 A1.2.14.1 A1.2.14.2 A1.3.10 |  | ~20µs of delay spread can be tolerated without an equalizer. | Self-evaluation comments are satisfactory. The Q&A sessions on this item can be referred to ARIB-5 in Appendix 3. |
| A3.4.1.5 | Maximum user information bit rate, Ru (kbit/s) How flexibly services can be offered to customers ? What is the limitation in number of users for each particular service? (e.g. no more than two simultaneous 2 Mbit/s users) | Q and q |  | G2 |  | A1.3.3 A1.3.1.5.2 A1.2.31 A1.2.32 |  | Assuming 10 MHz PUSC: - 23040 kbit/s for the Downlink (DL:UL=35:12) - 6048 kbit/s for the Uplink for (DL:UL=26:21) Services are very flexible as the Subchannels can be grouped to increase data rates. | Self-evaluation comments are satisfactory. |
| A3.4.1.6 | Multiple vocoder rate capability – bit rate variability, – delay variability, – error protection variability. | Q and q |  | G3 |  | A1.2.19 A1.2.19.1 A1.2.7 |  | Yes. Vocoders are however independent of the RTT and are implementation specific. The data transports for voice can operate at varying levels of Packet error rate and using H-ARQ can significantly boost performance. | Self-evaluation comments are satisfactory. |
| A3.4.1.7 | Multimedia capabilities The proponents should describe how multimedia services are handled. The following items should be evaluated: – possible limitations (in data rates, number of bearers), – ability to allocate extra bearers during of the communication, – constraints for handover. | Q and q |  | G1 |  | A1.2.21 A1.2.20 A1.3.1.5.2 A1.2.18 A1.2.24 A1.2.30 A1.2.30.1 |  | The Data bearers have no constraints on the type of media they can carry. However typically they are mapped to the QoS of the media type being transmitted. There are no limits on the number of bearers as long as bandwidth is available. Extra bearers can be allocated during communication. There are no handover constraints as long as coverage is available. | Self-evaluation comments are satisfactory. |
| A3.4.2 | Planning |  |  |  |  |  |  |  |  |
| A3.4.2.1 | Spectrum related matters |  |  |  |  |  |  |  |  |
| A3.4.2.1.1 | Flexibility in the use of the frequency band The proponents should provide the necessary information related to this topic (e.g., allocation of sub-carriers with no constraints, handling of asymmetric services, usage of non-paired band). | q |  | G1 |  | A1.2.1 A1.2.2 A1.2.2.1 A1.2.3 A1.2.5.1 |  | A 5 MHz or 10 MHz TDD carrier may be deployed with 1:3:3 frequency re-use or 1:3:1 reuse. | Self-evaluation comments are satisfactory. |
| A3.4.2.1.2 | Spectrum sharing capabilities The proponent should indicate how global spectrum allocation can be shared between operators in the same region. The following aspects may be detailed: – means for spectrum sharing between operators in the same region, – guardband between operators in case of fixed sharing. | q and Q |  | G4 |  | A1.2.26 |  | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. So spectrum sharing is carried out using multiple channel carriers. The guard bands are RF band specific. | Self-evaluation comments are satisfactory. |
| A3.4.2.1.3 | Minimum frequency band necessary to operate the system in good conditions Supporting technical information: – impact of the frequency reuse pattern, – bandwidth necessary to carry high peak data rate. | Q and q |  | G1 |  | A1.2.1 A1.4.15 A1.2.5 |  | 5 MHz or 10 MHz 1x3x3 PUSC or 1x3x1 PUSC may be used. 10 MHz gives the optimal data rate. | Self-evaluation comments are satisfactory. |
| A3.4.2.2 | Radio resource planning |  |  |  |  |  |  |  |  |
| A3.4.2.2.1 | Allocation of radio resources The proponents and evaluators should focus on the requirements and constraints imposed by the proposed technology. More particularly, the following aspects should be considered: – what are the methods used to make the allocation and planning of radio resources flexible? – what are the impacts on the network side (e.g. synchronization of BSs, signalling,)? – other aspects. Examples of functions or type of planning required which may be supported by the proposed technology: – DCA, – frequency hopping, – code planning, – time planning, – interleaved frequency planning. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. In some cases, no particular functions are necessary (e.g. frequency reuse = 1). | q |  | G2 |  | A1.2.25 A1.2.27 A1.4.15 |  | Subchannelization schemes and zones namely PUSC and AMC are supported to provide flexibility in utilizing the frequency and time resources. Sectorized deployments are possible with flexible frequency re-use (1x3x3 or 1x3x1) using PUSC subchannelization schemes. Slots of multiple subchannels and OFDM symbols are used to manage the resource allocation granularity BSs need to be synchronized. This is typically done using GPS on the BS. No frequency planning is required across cells. | Self-evaluation comments are satisfactory. |
| A3.4.2.2.2 | Adaptability to adapt to different and/or time varying conditions (e.g., propagation, traffic) How the proposed technology cope with varying propagation and/or traffic conditions? Examples of adaptive functions which may be supported by the proposed technology: – DCA, – link adaptation, – fast power control, – adaptation to large delay spreads. Some adaptivity aspects may be inherent to the RTT. | q |  | G2 |  | A1.3.10 A1.2.27 A1.2.22 A1.2.14 |  | Subchannelization and slot structure capability provides the ability to schedule frequency/time resources to mitigate the effects of propagation losses and also for traffic load balancing. Link adaptation schemes with CQI feedback capability allow operating the link more efficiently. H-ARQ also allows operations at high packet error rates resulting higher spectral efficiency as higher order coding and modulation rates can be used. The OFDMA symbol structure is designed to reduce the effects of delay spreads up to 20µs. | Self-evaluation comments are satisfactory. The Q&A sessions on this item can be referred to ARIB-5 in Appendix 3. |
| A3.4.2.3 | Mixed cell architecture (not applicable to satellite component) |  |  |  |  |  |  |  |  |
| A3.4.2.3.1 | Frequency management between different layers What kind of planning is required to manage frequencies between the different layers? e.g. – fixed separation, – dynamic separation, – possibility to use the same frequencies between different layers. Possible supporting technical information: – guard band. | q and Q |  | G1 |  | A1.2.28 A1.4.15 |  | Hierarchical layered cells are possible. The type of frequency planning is implementation/deployment scenario specific. The same frequencies can be used across layers by proper segmentation of the PUSC Subchannels. | Self-evaluation comments are satisfactory. |
| A3.4.2.3.2 | User adaptation to the environment What are the constraints to the management of users between the different cell layers? e.g. – constraints for handover between different layers, – adaptation to the cell layers depending on services, mobile speed, mobile power. | q |  | G2 |  | A1.2.28 A1.3.10 |  | The RTT does not impose constraints on the management of users between different cell layers in such a hierarchical deployment. | Self-evaluation comments are satisfactory. |
| A3.4.2.4 | Fixed-wireless access |  |  |  |  |  |  |  |  |
| A3.4.2.4.1 | The proponents should indicate how well its technology is suited for operation in the fixed wireless access environment. Areas which would need evaluation include (not applicable to satellite component): – ability to deploy small BSs easily, – use of repeaters, – use of large cells, – ability to support fixed and mobile users within a cell, – network and signaling simplification. | q |  | G4 |  | A1.1.3 A1.3.5 A1.4.17 A1.4.7 A1.4.7.1 |  | The RTT is very much suited for fixed wireless access as well. Pico or Micro cells or Macro cells and repeaters are possible. Both fixed and mobile users can work in the same cell. Network signaling for fixed devices are simpler compared to mobile devices. | Self-evaluation comments are satisfactory. |
| A3.4.2.4.2 | Possible use of adaptive antennas (how well suited is the technology) (not applicable to satellite component) Is RTT suited to introduce adaptive antennas? Explain the reason if it is. | q |  | G4 |  | A1.3.6 |  | Yes the RTT supports adaptive antenna/Beamforming solutions. | Self-evaluation comments are satisfactory. |
| A3.4.2.4.3 | Existing system migration capability | q |  | G1 |  | A1.4.16 |  | NA |  |
| A3.5 | Implication on network interface |  |  |  |  |  |  |  |  |
| A3.5.1 | Examine the synchronization requirements with respect to the network interfaces. Best case : no special accommodation necessary to provide synchronization. Worst case : special accommodation for synchronization is required, e.g. additional equipment at BS or special consideration for facilities. | q |  | G4 |  | A1.4.3 |  | Synchronization of the BSs across the network is required and this is typically accomplished using GPS. | Self-evaluation comments are satisfactory. |
| A3.5.2 | Examine the RTTs ability to minimize the network infrastructure involvement in cell handover. Best case : neither PSTN/ISDN nor mobile switch involvement in handover. Worst case : landline network involvement essential for handover. | q |  | G3 |  | A1.2.24 A1.4.6.1 |  | Handover within the same ASN (Access Service Network) does not involve the CSN (Core Service Network). In most handover scenarios with neighboring cells there is minimal involvement of the CSN. Only the BS and ASN GW may need to be involved in these scenarios. | Self-evaluation comments are satisfactory. |
| A3.5.3 | Landline feature transparency |  |  |  |  |  |  |  |  |
| A3.5.3.1 | Examine the network modifications required for the RTT to pass the standard set of ISDN bearer services. Best case : no modifications required. Worst case: substantial modification required, such as interworking functions. | q |  | G1 |  | A1.4.7.1 |  | ISDN is supported as an application running over the IP protocol and is not natively supported. As voice is supported using Voice over IP protocols, the use of ISDN is only involved interworking functions between the IP networks and PSTN. | Self-evaluation comments are satisfactory. |
| A3.5.3.2 | Examine the extent of the PSTN/ISDN involvement in switching functionality. Best case : all switching of calls is handled by the PSTN/ISDN. Worst case : a separate mobile switch is required. | q |  | G2 |  | A1.4.6 A1.4.8 |  | PSTN/ISDN is not used for switching within the IP network. | Self-evaluation comments are satisfactory. |
| A3.5.3.3 | Examine the depth and duration of fading that would result in a dropped call to the PSTN/ISDN network. The robustness of an RTTs ability to minimize dropped calls could be provided by techniques such as transparent reconnect. | Q and q |  | G3 |  | A1.2.24 A1.4.14 |  | Voice is supported as an application over the RTT. The robustness of the link maintained is implementation dependent. The RTT supports HARQ and hence can operate in higher Packer Error Rates up to 10%. | Self-evaluation comments are satisfactory. |
| A3.5.3.4 | Examine the quantity and type of network interfaces necessary for the RTT based on the deployment model used for spectrum and coverage efficiencies. The assessment should include those connections necessary for traffic, signalling and control as well as any special requirements, such as soft handover or simulcast. | Q |  | G2 |  | A1.2.30 A1.2.30.1 A1.4.9 |  | The RTT design is to minimize impacts on the network. All the connections necessary for traffic, signaling and control terminate on the BS for PHY/MAC layer. The Radio Resource Management functions implemented over the IP protocol reside in the ASN. So most RTT configuration parameters are controlled on the BS which is interfaced using an IP connection to the ASN-GW . | Self-evaluation comments are satisfactory. |
| A3.6 | Handportable performance optimization capability |  |  |  |  |  |  |  |  |
| A3.6.1 | Isolation between transmitter and receiver Isolation between transmitter and receiver has an impact on the size and weight of the handportable. | Q |  | G2 |  | A1.2.2 A1.2.2.1 A1.2.2.2 |  | As the RTT is a TDD based technology, no specific isolation requirements exist. | Self-evaluation comments are satisfactory. |
| A3.6.2 | Average terminal power output P0 (mW) Lower power gives longer battery life and greater operating time. | Q |  | G2 |  | A1.2.16.1.2 |  | This is implementation dependent. The terminals have different power classes to which they belong as shown in A3.2.2.2.2. | Self-evaluation comments are satisfactory. |
| A3.6.3 | System round trip delay impacts the amount of acoustical isolation required between hand portable microphone and speaker components and, as such, the physical size and mechanical design of the subscriber unit. NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q and q |  | G2 |  | A1.3.7 A1.3.7.1 A1.3.7.2 A1.3.7.3 |  | The Round trip delay will be well within the ITU-T specified limits for a typical Voice application that may be implemented using the RTT. | Self-evaluation comments are satisfactory. |
| A3.6.4 | Peak transmission power | Q |  | G1 |  | A1.2.16.1.1 |  | This is not limited by RTT but by regulations. The peak terminal power output P0 = 1000 mW (Power class 3). Also see A3.2.2.2.2 for more details. | Self-evaluation comments are satisfactory. |
| A3.6.5 | Power control characteristics Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with mobile station technology complexity. |  |  |  |  |  |  | Yes the RTT does utilize transmitter power control for both Downlink and Uplink. | Self-evaluation comments are satisfactory. |
| A3.6.5.1 | Power control dynamic range Larger power control dynamic range gives longer battery life and greater operating time. | Q |  | G3 |  | A1.2.22 A1.2.22.3 A1.2.22.4 |  | The minimum power control dynamic range is 45 dB. | Self-evaluation comments are satisfactory. |
| A3.6.5.2 | Power control step size, accuracy and speed | Q |  | G3 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.5 |  | The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦ ± 1 dB ¦m¦ = 3dB¦ ± 1.5 dB 4dB< ¦m¦< = 10dB¦ ± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. | Self-evaluation comments are satisfactory. |
| A3.6.6 | Linear transmitter requirements | q |  | G3 |  | A1.4.10 |  | Linear transmitters are used on the BS and MS. | Self-evaluation comments are satisfactory. |
| A3.6.7 | Linear receiver requirements (not applicable to satellite) | q |  | G3 |  | A1.4.11 |  | Linear receivers are used on the BS and MS. | Self-evaluation comments are satisfactory. The Q&A sessions on this item can be referred to ARIB-9 in Appendix 3. |
| A3.6.8 | Dynamic range of receiver The lower the dynamic range requirement, the lower the complexity and ease of design implementation. | Q |  | G3 |  | A1.4.12 |  | 80dB for the MS receiver and 65dB for the BS receiver | Self-evaluation comments are satisfactory. |
| A3.6.9 | Diversity schemes Diversity has an impact on hand portable complexity and size. If utilized describe the type of diversity and address the following two attributes. | Q and q |  | G1 |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  | MIMO and Beamforming are supported. Within the MIMO scheme both Transmit Diversity and Spatial Multiplexing are supported. | Self-evaluation comments are satisfactory. |
| A3.6.10 | The number of antennas | Q |  | G1 |  | A1.2.23.1 |  | BS: 2 Tx, 2 Rx MS: 1 Tx, 2 Rx | Self-evaluation comments are satisfactory. |
| A3.6.11 | The number of receivers | Q |  | G1 |  | A1.2.23.1 |  | BS: 2 Receivers MS : 2 Receivers | Self-evaluation comments are satisfactory. |
| A3.6.12 | Frequency stability Tight frequency stability requirements contribute to handportable complexity. | Q |  | G3 |  | A1.4.1.2 |  | BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing | Self-evaluation comments are satisfactory. |
| A3.6.13 | The ratio of “off (sleep)” time to “on” time | Q |  | G1 |  | A1.2.29 A1.2.29.1 |  | This implementation dependent and is programmable by the BS or MS implementations. | Self-evaluation comments are satisfactory. |
| A3.6.14 | Frequency generator step size, switched speed and frequency range Tight step size, switch speed and wide frequency range contribute to handportable complexity. Conversely, they increase RTT flexibility. | Q |  | G2 |  | A1.4.5 |  | Frequency step size : 200 and 250 KHz Switched speed : 200 μsec Frequency range : 5, 10 MHz | Frequency switched speed is not specified in IEEE std 802.16e. It is preferable that it depends on regulations. |
| A3.6.15 | Digital signal processing requirements Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q |  | G1 |  | A1.4.13 |  | These are again implementation dependent. | Self-evaluation comments are satisfactory. |
| A3.7.1.1 | Base site coverage efficiency The number of base sites required to provide coverage at system start-up and ongoing traffic growth significantly impacts cost. From § 1.3.2 of Annex 2, determine the coverage efficiency, C (km2/base sites), for the lowest traffic loadings. Proponent has to indicate the background of the calculation and also to indicate the maximum coverage range. | Q |  | G1 |  | A1.3.1.7 A1.3.1.7.1 A1.3.1.7.2 A1.3.4 |  | 80-95% at system startup 95-100% in a mature system See section 2.2.4.2 for more details. | Self-evaluation comments are satisfactory. The ARIB evaluation group conducted the coverage efficiency evaluation based on conditions stated as the Recommendation ITU-R M.1225. The evaluation results are as follows: Coverage efficiency: - DL Indoor Office, Speech, Channel A = 1.51 km2/site - UL Indoor Office, Speech, Channel A = 1.93 km2/site - DL Outdoor-to-Indoor Pedestrian, Speech, Channel A = 0.40 km2/site - UL Outdoor-to-Indoor Pedestrian, Speech, Channel A = 0.49 km2/site - DL Pedestrian Outdoor, Speech, Channel A = 2.28 km2/site - UL Pedestrian Outdoor, Speech, Channel A = 2.81 km2/site - DL Vehicular, Speech, Channel A = 352.97 km2/site - UL Vehicular, Speech, Channel A = 81.41 km2/site - DL Indoor Office, Speech, Channel B = 1.65 km2/site - UL Indoor Office, Speech, Channel B = 2.32 km2/site - DL Outdoor-to-Indoor Pedestrian, Speech, Channel B = 0.50 km2/site - UL Outdoor-to-Indoor Pedestrian, Speech, Channel B = 0.60 km2/site - DL Pedestrian Outdoor, Speech, Channel B = 2.87 km2/site - UL Pedestrian Outdoor, Speech, Channel B = 3.42 km2/site - DL Vehicular, Speech, Channel B = 344.43 km2/site - UL Vehicular, Speech, Channel B = 80.42 km2/site - DL Indoor Office, LDD, Channel A = 0.04 km2/site - UL Indoor Office, LDD, Channel A = 0.02 km2/site - DL Outdoor-to-Indoor Pedestrian, LDD, Channel A = 0.10 km2/site - UL Outdoor-to-Indoor Pedestrian, LDD, Channel A = 0.06 km2/site - DL Pedestrian Outdoor, LDD, Channel A = 0.57 km2/site - UL Pedestrian Outdoor, LDD, Channel A = 0.33 km2/site - DL Vehicular, LDD, Channel A = 153.09 km2/site - UL Vehicular, LDD, Channel A = 19.09 km2/site - DL Indoor Office, LDD, Channel B = 0.05 km2/site - UL Indoor Office, LDD, Channel B = 0.02 km2/site - DL Outdoor-to-Indoor Pedestrian, LDD, Channel B = 0.13 km2/site - UL Outdoor-to-Indoor Pedestrian, LDD, Channel B = 0.07 km2/site - DL Pedestrian Outdoor, LDD, Channel B = 0.72 km2/site - UL Pedestrian Outdoor, LDD, Channel B = 0.40 km2/site - DL Vehicular, LDD, Channel B = 153.09 km2/site - UL Vehicular, LDD, Channel B = 18.86 km2/site - DL Indoor Office, UDD, Channel A = 0.04 km2/site - UL Indoor Office, UDD, Channel A = 0.02 km2/site - DL Outdoor-to-Indoor Pedestrian, UDD, Channel A = 0.13 km2/site - UL Outdoor-to-Indoor Pedestrian, UDD, Channel A = 0.06 km2/site - DL Pedestrian Outdoor, UDD, Channel A = 0.76 km2/site - UL Pedestrian Outdoor, UDD, Channel A = 0.36 km2/site - DL Vehicular, UDD, Channel A = 188.53 km2/site - UL Vehicular, UDD, Channel A = 22.39 km2/site - DL Indoor Office, UDD, Channel B = 0.04 km2/site - UL Indoor Office, UDD, Channel B = 0.02 km2/site - DL Outdoor-to-Indoor Pedestrian, UDD, Channel B = 0.15 km2/site - UL Outdoor-to-Indoor Pedestrian, UDD, Channel B = 0.07 km2/site - DL Pedestrian Outdoor, UDD, Channel B = 0.85 km2/site - UL Pedestrian Outdoor, UDD, Channel B = 0.38 km2/site - DL Vehicular, UDD, Channel B = 188.53 km2/site - UL Vehicular, UDD, Channel B = 22.11 km2/site Please see Appendix 2 for the detail simulation conditions. |
| A3.7.1.2 | Method to increase the coverage efficiency Proponent describes the technique adopted to increase the coverage efficiency and drawbacks. Remote antenna systems can be used to economically extend vehicular coverage to low traffic density areas. RTT link budget, propagation delay system noise and diversity strategies can be impacted by their use. Distributed antenna designs – similar to remote antenna systems – interconnect multiple antennas to a single radio port via broadband lines. However, their application is not necessary limited to providing coverage, but can also be used to economically provide continuous building coverage for pedestrian applications. System synchronization, delay spread, and noise performance can be impacted by their use. | q |  | G1 |  | A1.3.5 A1.3.6 |  | MIMO and Beamforming can be used to increase coverage efficiency. Remote or Distributed antenna systems can also be used. However the use of these methods is deployment scenario specific based on the implementations. | Self-evaluation comments are satisfactory. |
| A3.7.2 | Satellite Normalized power efficiency Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data mixed traffic. | Q |  | G1 |  | A1.3.2.4 A1.3.2.4.1 A1.3.2.4.2 |  | NA |  |

## Appendixes to this annex are as follows:

## Appendix 1: Evaluation for Section 2.1 (“Radio Transmission Technology (RTT) Description Template”) of Document 8F/1079(Rev.1)

## Appendix 2: Evaluation for Section 2.3 (“Capacity and Coverage”) of Documents 8F/1079(Rev.1)

## Appendix 3: Questionnaire in the Evaluation Process

Appendix 1

ARIB (Association of Radio Industries and Businesses)

Evaluation for Section 2.1 (“Radio Transmission Technology (RTT) Description Template”) of Document 8F/1079(Rev.1)

Evaluator’s notes

Note 1: Proponents should add the clause to be able to identify the information if it locates in the references, not in the main body of the proposal.

Note 2: Gray shaded index is the topic which is related to Annex 1.

| Index | Criteria and Attributes | Proponent’s Comments | Evaluator’s Comments |  |
| --- | --- | --- | --- | --- |
| A1.1 | Test environment support |  |  |  |
| A1.1.1 | In what test environments will the RTT operate? | - indoor - outdoor to indoor and pedestrian, - vehicular - mixed | Confirmed Described in section 2.3.3 in 8F/1079(Rev.1) |  |
| A1.1.2 | If the RTT supports more than one test environment, what test environment does this technology description template address? | One template for all | Confirmed Described in section 2.3.3 in 8F/1079(Rev.1) |  |
| A1.1.3 | Does the RTT include any features in support of FWA application? Provide detail about the impact of those features on the technical parameters provided in this template, stating whether the technical parameters provided apply for mobile as well as for FWA applications. | Yes (cf. Rec. ITU-R F.1763). Flexible mixed fixed and mobile design. - QoS - Dynamic bandwidth allocation - Continuous and variable bit rate support - Support of nomadic operation - Support of fixed wireless voice, image, video and data services. |  | Confirmed (Note1) Described in section 1.1 in REF[1] of 8F/1079(REV.1). |

| Index | Criteria and attributes | Proponent’s Comment | Evaluator’s Comment |
| --- | --- | --- | --- |
| A1.2 | Test environment support |  |  |
| A1.2.1 | What is the minimum frequency band required to deploy the system (MHz)? | 5 MHz or 10 MHz (10 MHz provides better performance). | Confirmed Described in section1.3.5 in 8F/1079(Rev.1). Minimum 5MHz, 10MHz provides better |
| A1.2.2 | What is the duplex method: TDD or FDD? | TDD | Confirmed Described in section1.3.4 in 8F/1079(Rev.1) |
| A1.2.2.1 | What is the minimum up/down frequency separation for FDD? | N/A |  |
| A1.2.2.2 | What is requirement of transmit/receive isolation? Does the proposal require a duplexer in either the mobile station (MS) or BS? | Does not require a duplexer. | Confirmed As a nature of TDD system |
| A1.2.3 | Does the RTT allow asymmetric transmission to use the available spectrum? Characterize. | Yes. The ratio of uplink to downlink transmission can be reconfigured on a system-wide basis. | Confirmed Described in section1.3.4 in 8F/1079(Rev.1) |
| A1.2.4 | What is the RF channel spacing (kHz)? In addition, does the RTT use an interleaved frequency plan? NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. If a proponent is going to employ an interleaved frequency plan, the proponent should state so in § A1.2.4 and complete § A1.2.15 with the protection ratio for both the adjacent and second adjacent channel. | 5 000 kHz or 10 000 kHz The RTT does not use an interleaved frequency plan | Confirmed Described in section1.3.5 in 8F/1079(Rev.1) |
| A1.2.5 | What is the bandwidth per duplex RF channel (MHz) measured at the 3 dB down points? It is given by (bandwidth per RF channel)  (1 for TDD and 2 for FDD). Provide detail. | For 5 MHz (TDD): about 4.7 MHz, depending on the permutation used. For 10 MHz (TDD): about 9.4 MHz, depending on the permutation used. | Confirmed See ARIB-6 in Appendix 3 |
| A1.2.5.1 | Does the proposal offer multiple or variable RF channel bandwidth capability? If so, are multiple bandwidths or variable bandwidths provided for the purposes of compensating the transmission medium for impairments but intended to be feature transparent to the end user? | The RTT offers variable RF channel bandwidth capability through the use of OFDMA sub-channelization. |  |
| A1.2.6 | What is the RF channel bit rate (kbit/s)? NOTE 1 – The maximum modulation rate of RF (after channel encoding, adding of in-band control signalling and any overhead signalling) possible to transmit carrier over an RF channel, i.e. independent of access technology and of modulation schemes. | DOWNLINK For the 10 MHz case this is the calculation: Distributed permutation of sub-carriers Assumptions: 32 data symbols per frame (35 symbols in sub-frame, 1 symbol for preamble, 2 symbols for control information), 5 ms frame duration, 64 QAM 5/6 code rate, 30 slots for 2 symbols, 48 data tones per slot. Maximum data rate: 23 040 kbit/s UPLINK Distributed permutation of sub-carriers Assumptions: 18 data symbols per frame (21 symbols in UL sub-frame, 3 symbols for control channels), 5 ms frame duration, 16 QAM 3/4 code rate, 35 slots for 3 symbols, 48 data tones per slot. Maximum data rate: 6 048 kbit/s | Confirmed (Note 1) Simulation result is shown in section 4.1 in REF[4] of 8F/1079(Rev.1) For TDD, one can not achieve both peaks rates at the same time since the maxim Dl/UL ratio implies minimum UL/DL ratio |
| A1.2.7 | Frame structure: describe the frame structure to give sufficient information such as: – frame length, – the number of time slots per frame, – guard time or the number of guard bits, – user information bit rate for each time slot, – channel bit rate (after channel coding), – channel symbol rate (after modulation), – associated control channel (ACCH) bit rate, - power control bit rate. NOTE 1 – Channel coding may include forward error correction (FEC), cyclic redundancy checking (CRC), ACCH, power control bits and guard bits. Provide detail. NOTE 2 – Describe the frame structure for forward link and reverse link, respectively. NOTE 3 – Describe the frame structure for each user information rate. | Frame length: 5 ms The number of time slots per frame: N/A The number of time symbols per frame: 48 symbols (including TTG and RTG gaps) The number of sub-carriers per each symbol: 1 024 FFT Resource allocation: 2 dimensional structure for frequency and time (see section 2.4 of the RTT System Description for more details) Sub-channel structure: see Section 2.2 of the RTT System Description for details Ratio of DL and UL sub-frame: Ranging from 35 symbols: 12 symbols to 26 symbols: 21 symbols (DL:UL) TTG / RTG : 105.7 μs / 60 μs Common control overhead : 1 symbol per frame for preamble (see section 2.4 of the RTT System Description for more details) DOWNLINK (See A1.2.5.1) Distributed permutation of sub-carriers The number of sub-carriers per slot : 48 (data) + 8 (pilots) Guard sub-carrier: 184 (including DC sub-carrier) The channel bit or symbol rate is variable, depending on the number of allocated slots, and the modulation and coding rate. Power control rate: no power control Adjacent permutation of sub-carriers The number of sub-carriers per slot : 48 (data) + 6 (pilots) Guard sub-carrier : 160 (including DC sub-carrier) UPLINK Distributed permutation of sub-carriers The number of subcarriers per slot : 48 (data) + 24 (pilots) Guard subcarrier : 184 (including DC subcarrier) The channel bit or symbol rate is variable, depending on the number of allocated slots, and the modulation and coding rate. Power control rate : 200 Hz Adjacent permutation of subcarriers The number of subcarriers per slot : 48 (data) + 6 (pilots) Guard subcarrier : 160 (including DC subcarrier) | Confirmed Described in section1.3.4/1.3.5 in 8F/1079(Rev.1) |
| A1.2.8 | Does the RTT use frequency hopping? If so, characterize and explain particularly the impact (e.g. improvements) on system performance. | No | Confirmed Described in 1.3 in 8F/1079(Rev.1). The IEEE 802.16 specification, however, does allow for rotation of the UL tiles (which distributed across the band). So in some sense this is like frequency hopping. Also, the tone assignment in DL PUSC and FUSC changes every OFDM symbol, so this is also in some sense like frequency hopping |
| A1.2.8.1 | What is the hopping rate? | N/A | Confirmed Based on the above comment (A1.2.8), the hopping rate for the UL is equal to the UL frame rate which is once every 5 ms and can be every symbol for the DL. |
| A1.2.8.2 | What is the number of the hopping frequency sets? | N/A | Confirmed Based on the above comment, one can claim that the number of frequency hopping sets is equal to the number of sub channels. |
| A1.2.8.3 | Are BSs synchronized or non-synchronized? | Synchronized in frequency and in time for TDD operation, even though frequency hopping is not used. | Confirmed Described in section1.3.4 in 8F/1079(Rev.1). Section 8.4.10.1.1 in REF [1,2] and the section 4.1.6.1 in REF[3] OF 8F/1079(Rev.1) provide more detail information |
| A1.2.9 | Does the RTT use a spreading scheme? | No | Confirmed Described in section1.3 in 8F/1079(Rev.1) |
| A1.2.9.1 | What is the chip rate (Mchip/s)? Rate at input to modulator. | N/A | Confirmed |
| A1.2.9.2 | What is the processing gain? 10 log (chip rate/information rate). | N/A | Confirmed |
| A1.2.9.3 | Explain the uplink and downlink code structures and provide the details about the types (e.g. personal numbering (PN) code, Walsh code) and purposes (e.g. spreading, identification, etc.) of the codes. | N/A | Confirmed |
| A1.2.10 | Which access technology does the proposal use: TDMA, FDMA, CDMA, hybrid, or a new technology? In the case of CDMA, which type of CDMA is used: frequency hopping (FH) or direct sequence (DS) or hybrid? Characterize. | OFDMA | Confirmed Described in section1.3 in 8F/1079(Rev.1) MC-CDMA for ranging and UL sounding |
| A1.2.11 | What is the baseband modulation technique? If both the data modulation and spreading modulation are required, describe in detail. What is the peak to average power ratio after baseband filtering (dB)? | DOWNLINK QPSK, 16 QAM, 64 QAM for data modulation. Spreading modulation does not apply. UPLINK QPSK, 16 QAM for data modulation. Spreading modulation does not apply. PAPR is about 12 dB without any PAPR reduction scheme. | Confirmed Described in section1.3.5 in 8F/1079(Rev.1) except PARP |
| A1.2.12 | What are the channel coding (error handling) rate and form for both the forward and reverse links? E.g., does the RTT adopt: – FEC or other schemes? – Unequal error protection? Provide details. – Soft decision decoding or hard decision decoding? Provide details. – Iterative decoding (e.g. turbo codes)? Provide details. – Other schemes? | Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. Unequal error protection: None Soft decision decoding and iterative decoding: It is an implementation issue not covered by the description specification. | Confirmed Described in section1.3.5 in 8F/10791(Rev.1) |
| A1.2.13 | What is the bit interleaving scheme? Provide detailed description for both uplink and downlink. | The bit interleaving scheme is the same for both uplink and downlink. All encoded data bits shall be interleaved by a block interleaver with a block size corresponding to the number of coded bits per the encoded block size. | Confirmed (Note 1) Section 8.3.3.3 in REF. [1,2] of 8F/1079(Rev.1) provides this information |
| A1.2.14 | Describe the approach taken for the receives (MS and BS) to cope with multipath propagation effects (e.g. via equalizer, Rake receiver, etc.). | To cope with the multipath propagation effect, the cyclic prefix and 1-tap equalizer are employed. The length of cyclic prefix is 1/8 of symbol duration thus 11.4 μs. | Confirmed Described in section1.3 in 8F/1079(Rev.1) |
| A1.2.14.1 | Describe the robustness to intersymbol interference and the specific delay spread profiles that are best or worst for the proposal. | The intersymbol interference can be removed by the use of sufficiently longer cyclic prefix than delay spread. | Confirmed Described in section1.3 in 8F/1079(Rev.1) |
| A1.2.14.2 | Can rapidly changing delay spread profile be accommodated? Describe. | Yes, delay spread variation within the length of cyclic prefix does not cause the intersymbol interference. | Confirmed Described in section1.3 in 8F/1079(Rev.1) |
| A1.2.15 | What is the adjacent channel protection ratio? NOTE 1 – In order to maintain robustness to adjacent channel interference, the RTT should have some receiver characteristics that can withstand higher power adjacent channel interference. Specify the maximum allowed relative level of adjacent RF channel power (dBc). Provide detail how this figure is assumed. | Min adjacent channel rejection at BER=10-6 for 3 dB degradation C/I 11 dB – 16 QAM, 3/4 coding rate 4 dB – 64 QAM, 2/3 coding rate Min non-adjacent channel rejection at BER=10-6 for 3 dB degradation C/I 30 dB – 16 QAM, 3/4 coding rate 23 dB - 64 QAM, 2/3 coding rate | Confirmed (Note 1) Section 4.2.3 in REF[3] OF 8F/1079(Rev.1) provides this information |
| A1.2.16 | Power classes | Mobile Station Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max | Confirmed (Note 1) Section 7 in REF[3] of 8F/1079(Rev.1) provides this information |
| A1.2.16.1 | Mobile terminal emitted power : what is the radiated antenna power measured at the antenna? For terrestrial component, give (dBm). For satellite component, the mobile terminal emitted power should be given in e.i.r.p. (effective isotropic radiated power) (dBm). | See A.1.2.16 | Confirmed |
| A1.2.16.1.1 | What is the maximum peak power transmitted while in active or busy state? | See A.1.2.16 | Confirmed |
| A1.2.16.1.2 | What is the time average power transmitted while in active or busy state? Provide detailed explanation used to calculate this time average power. | See A.1.2.16 | Confirmed See ARIB-7 in Appendix 3 |
| A1.2.16.2 | Base station transmit power per RF carrier for terrestrial component | See A.1.2.16 | Confirmed See ARIB-8 in Appendix 3 |
| A1.2.16.2.1 | What is the maximum peak transmitted power per RF carrier radiated from antenna? | Not limited by RTT | Confirmed |
| A1.2.16.2.2 | What is the average transmitted power per RF carrier radiated from antenna? | Not limited by RTT | Confirmed |
| A1.2.17 | What is the maximum number of voice channels available per RF channel that can be supported at one BS with 1 RF channel (TDD systems) or 1 duplex RF channel pair (FDD systems), while still meeting ITU-T Recommendation G.726 performance requirements? | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The capacity calculated assumes a blocking-limited scenario with Voice Activity Factor = 1, DL 64 QAM 5/6, and UL 16QAM 3/4. | Confirmed Related to the discussion on A3.2.8 |
| A1.2.18 | Variable bit rate capabilities: describe the ways the proposal is able to handle variable baseband transmission rates. For example, does the RTT use: – Adaptive source and channel coding as a function of RF signal quality? – Variable data rate as a function of user application? – Variable voice/data channel utilization as a function of traffic mix requirements? Characterize how the bit rate modification is performed. In addition, what are the advantages of your system proposal associated with variable bit rate capabilities? | Variable bit rate is supported by the flexible resource allocation. By assigning the variable number of sub-channels and using various modulations and coding rates frame by frame, the bit rate for each user can be variable frame by frame. Modulation and coding rate is usually defined by user's RF signal quality (CQI). For higher data rates, the bit rate information is provided to the receiver via scheduling mechanisms and associated control signaling every frame. | Confirmed Described in section 1.3.4 and 1.4.2 in 8F/1079(Rev.1) |
| A1.2.18.1 | What are the user information bit rates in each variable bit rate mode? | The user information bit rates are variable according to the number of sub-channels assigned and modulation and coding rate used. For 10 MHz: DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 3312 kbit/s (1/2, QPSK, (DL:UL)=(26:21) symbols) ~ 23040 kbit/s (5/6, 64 QAM, (DL:UL)=(35:12) symbols). See equation below. Note 1: The above numbers are calculated based on the maximum DL/UL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 11520 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 1008 kbit/s (1/2, QPSK, (DL:UL)=(35:12) symbols) ~ 6048 kbit/s (3/4, 16 QAM, (DL:UL)=(26:21) symbols). See equation below. Note 1: The above numbers are calculated based on the maximum UL/DL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 3024 kbit/s. Equation used: PHY Data Rate=(Data sub-carriers/Symbol period) × (information bits per symbol) | Confirmed (Note 1) Section 4.1 in REF [4] of 8F/1079(Rev.1) provides this information |
| A1.2.19 | What kind of voice coding scheme or codec is assumed to be used in proposed RTT? If the existing specific voice coding scheme or codec is to be used, give the name of it. If a special voice coding scheme or codec (e.g. those not standardized in standardization bodies such as ITU) is indispensable for the proposed RTT, provide detail, e.g. scheme, algorithm, coding rates, coding delays and the number of stochastic code books. | Due to the IP-based characteristics of the radio interface it can utilize any speech codec. | Confirmed |
| A1.2.19.1 | Does the proposal offer multiple voice coding rate capability? Provide detail. | Yes. The RTT supports flexible data rate for each user and also provide variety scheduling services. A constant bit rate is provided by UGS service, while a variable bit rate is provided by ErtPS service. See A.1.2.18, A1.2.20.1 and A1.2.20.2 | Confirmed |
| A1.2.20 | Data services: are there particular aspects of the proposed technologies which are applicable for the provision of circuit-switched, packet-switched or other data services like asymmetric data services? For each service class (A, B, C and D) a description of RTT services should be provided, at least in terms of bit rate, delay and BER/frame error rate (FER). NOTE 1 – See Recommendation ITU-R M.1224 for the definition of: – “circuit transfer mode”, – “packet transfer mode”, – “connectionless service”, and for the aid of understanding “circuit switched” and “packet switched” data services NOTE 2 – See ITU-T Recommendation I.362 for details about the service classes A, B, C and D. | Yes, a wide range of data services and applications with varied QoS requirements are supported. These are summarized in Error: Reference source not found of Attachment 2 Section in this submission. | Confirmed |
| A1.2.20.1 | For delay constrained, connection oriented (Class A). | The RTT provides UGS (unsolicited grant service), corresponding to the Class A. UGS is characterized as constant and low data rates and low delay data service. | Confirmed Described in section 1.4.1 in 8F/1079(Rev.1) |
| A1.2.20.2 | For delay constrained, connection oriented, variable bit rate (Class B). | The RTT provides rtPS (real-time polling service), corresponding to the Class B. rtPS is utilized for low to high data rate services. The RTT provides ErtPS (extended real-time polling service) as well. ErtPS is utilized for low data rate and low delay data services. | Confirmed Described in section 1.4.1 in 8F/1079(Rev.1) |
| A1.2.20.3 | For delay unconstrained, connection oriented (Class C). | The RTT provides nrtPS (non-real-time polling service), corresponding to the Class C. nrtPS is utilized for high data rate services. | Confirmed Described in section 1.4.1 in 8F/1079(Rev.1) |
| A1.2.20.4 | For delay unconstrained, connectionless (Class D). | The RTT provides BE (best effort service) corresponding to the Class D. BE is utilized for moderate data rate services. | Confirmed Described in section 1.4.1 in 8F/1079(Rev.1) |
| A1.2.21 | Simultaneous voice/data services: is the proposal capable of providing multiple user services simultaneously with appropriate channel capacity assignment? NOTE 1 – The following describes the different techniques that are inherent or improve to a great extent the technology described above to be presented. Description for both BS and MS are required in attributes from § A1.2.22 through § A1.2.23.2 | Yes, multiple parallel services are supported with different QoS requirements. Each service is associated with a set of QoS parameters that quantify aspects of its behavior. These parameters are managed using the dynamic service provisions, represented by the DSA and DSC message dialog. | Confirmed Described in section 1.4.1 in 8F/1079(Rev.1) |
| A1.2.22 | Power control characteristics : is a power control scheme included in the proposal? Characterize the impact (e.g. improvements) of supported power control schemes on system performance. | Yes. A closed loop power control scheme and an open loop power control scheme are included. By means of these power control schemes, the interference level is reduced and the uplink system level throughput is increased. | Confirmed Described in section 1.4.3 in 8F/1079(Rev.1) |
| A1.2.22.1 | What is the power control step size (dB)? | Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. | Confirmed (Note 1) Sections 8.4.12.1 and 11.6 in REF [1,2] of 8F/1079(Rev.1) provide this information |
| A1.2.22.2 | What are the number of power control cycles per second? | The power control cycle of closed-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. Due to TDD nature, the open loop power control cycle is inherently identical to the number of frames per seconds, thus 200 Hz. | Confirmed Described in section 1.3.5/ 1.4.3 in 8F/1079(Rev.1) |
| A1.2.22.3 | What is the power control dynamic range (dB)? | The minimum power control dynamic range is 45 dB. | Confirmed Described in 8F/1079(Rev.1). Section 8.4.12.1 in REF[1,2] and section 4.2.2 in REF[3] of 8F/1079(REV.1) provides this information |
| A1.2.22.4 | What is the minimum transmit power level with power control? | The RTT supports 45 dB under the full power assumption | Confirmed Described in 8F/1079(Rev.1). Section 8.4.12.1 in REF[1,2] of 8F/1079(Rev.1) provides this information |
| A1.2.22.5 | What is the residual power variation after power control when RTT is operating? Provide details about the circumstances (e.g. in terms of system characteristics, environment, deployment, MS-speed, etc.) under which this residual power variation appears and which impact it has on the system performance. | The accuracy for power level control can vary from  0.5 dB to  2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB ¦ +/- 0.5 dB ¦m¦ = 2dB ¦ +/- 1 dB ¦m¦ = 3dB ¦ +/- 1.5 dB Otherwise 4dB< ¦m¦< = 10dB ¦ +/- 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. | Confirmed Described in 8F/1079(Rev.1). Section 8.4.12.1 of 802.16-2004/Cor2 D3 includes related information. |
| A1.2.23 | Diversity combining in MS and BS : are diversity combining schemes incorporated in the design of the RTT? | Yes. | Confirmed Described in section 1.3.5 in 8F/1079(Rev.1) |
| A1.2.23.1 | Describe the diversity techniques applied in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – time diversity: repetition, Rake-receiver, etc., – space diversity: multiple sectors, multiple satellite, etc., – frequency diversity: FH, wideband transmission, etc., – code diversity: multiple PN codes, multiple FH code, etc., – other scheme. Characterize the diversity combining algorithm, for example, switch diversity, maximal ratio combining, equal gain combining. Additionally, provide supporting values for the number of receivers (or demodulators) per cell per mobile user. State the dB of performance improvement introduced by the use of diversity. For the MS: what is the minimum number of RF receivers (or demodulators) per mobile unit and what is the minimum number of antennas per mobile unit required for the purpose of diversity reception? These numbers should be consistent to that assumed in the link budget template of Annex 2 and that assumed in the calculation of the “capacity” defined at § A1.3.1.5. | The standard supports beamforming, transmit/receive diversity and MIMO. The receiver also supports maximal ratio combining. There is no need for a Rake receiver because it is an OFDM system. | Confirmed Described in section 1.3.5 in 8F/1079(Rev.1) except Receive Diversity |
| A1.2.23.2 | What is the degree of improvement expected (dB)? Also indicate the assumed conditions such as BER and FER. | Please refer to Section Error: Reference source not found. | Confirmed Described in Proponent’s comment in A3.2.6.1 |
| A1.2.24 | Handover/automatic radio link transfer (ALT) : do the radio transmission technologies support handover? Characterize the type of handover strategy (or strategies) which may be supported, e.g. MS assisted handover. Give explanations on potential advantages, e.g. possible choice of handover algorithms. Provide evidence whenever possible. | Yes. The RTT supports handover and also provides means for expediting handover. Each base station broadcasts the information on the list of neighboring base stations and their channel information such as the operating center frequency, preamble index and synchronization periodically. The channel information in this broadcasting is used for a mobile station to synchronize with the neighboring base station. After a mobile station monitors the signal strength of a neighboring base station and seeks suitable base station(s) for handover, the mobile station or its serving base station can initiate handover by handover request message. But only the mobile station can transmit handover indication message to the its serving base station. After transmitting handover indication message, the mobile station stops monitoring the downlink frame of its serving base station and performs network re-entry to target base station. To reduce the handover latency further, the serving base station provides the target base station with network entry information on a mobile station to be handed over the target base station. Further information is available in the IEEE 802.16 standard; Section 6.3.22 MAC layer handover procedures. | Confirmed Described in section 1.4.6 in 8F/1079(Rev.1) |
| A1.2.24.1 | What is the break duration (s) when a handover is executed? In this evaluation, a detailed description of the impact of the handover on the service performance should also be given. Explain how the estimate was derived. |  | Confirmed Section 2.3.2.2 in 8F/1079(Rev.1) provides the simulation result |
| A1.2.24.2 | For the proposed RTT, can handover cope with rapid decrease in signal strength (e.g. street corner effect)? Give a detailed description of: – the way the handover detected, initiated and executed, – how long each of this action lasts (minimum/maximum time (ms)), – the time-out periods for these actions. | Yes. A base station broadcasts the criterion which is being used for mobile station to request handover. The mobile station issues handover request message whenever the criterion is met. The handover criterion depends on the implementation but usually the received signal strength by a mobile station is used. Further information is available in the IEEE 802.16 standard; Section 11.1.7 MOB-NBR-ADV message encodings. | Confirmed (Note 1) REF[1,2] of 8F/1079(Rev.1) provides this information |
| A | Characterize how the proposed RTT reacts to the system deployment (e.g. necessity to add new cells and/or new carriers) particularly in terms of frequency planning. | All base stations can use the same frequency or different frequency depending on the frequency reuse deployment scenario. OFDMA sub-channelization allows various permutations of sub-carriers. A distributed permutation of sub-carriers, e.g., PUSC (partial usage of sub-carrier) in this RTT, minimizes interferences from neighboring cells and/or sectors in case of the frequency reuse of 1. Different operators usually use different frequencies. | Confirmed Described in section 1.3.2 in 8F/1079(Rev.1) |
| A1.2.26 | Sharing frequency band capabilities : to what degree is the proposal able to deal with spectrum sharing among IMT-2000 systems as well as with all other systems: – spectrum sharing between operators, – spectrum sharing between terrestrial and satellite IMT-2000 systems, – spectrum sharing between IMT-2000 and non-IMT-2000 systems, – other sharing schemes. | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. This capability, complemented by interference mitigation techniques described in Report ITU-R M.2045 such as use of appropriate filters and linear power amplifiers would ensure excellent potential for optimum spectrum sharing between the proposed RTT and other IMT-2000 systems. ITU-R WP 8F is in the process of performing sharing studies between fixed/nomadic and mobile IEEE 802.16 and IMT-2000. Preliminary results show similarities with the case of coexistence between IMT-2000 TDD and FDD technologies as captured in Reports ITU-R M.2030 and ITU-R M.2045. | Confirmed |
| A1.2.27 | Dynamic channel allocation : characterize the dynamic channel allocation (DCA) schemes which may be supported and characterize their impact on system performance (e.g. in terms of adaptability to varying interference conditions, adaptability to varying traffic conditions, capability to avoid frequency planning, impact on the reuse distance, etc.). | Various permutations of OFDMA sub-carriers enable dynamic usage of the spectrum among cells to balance the load and/or average interferences. | Confirmed |
| A1.2.28 | Mixed cell architecture : how well does the RTT accommodate mixed cell architectures (pico, micro and macrocells)? Does the proposal provide pico, micro and macro cell user service in a single licensed spectrum assignment, with handoff as required between them? (terrestrial component only). NOTE 1 – Cell definitions are as follows: – pico – cell hex radius: r < 100 m – micro: 100 m < r < 1 000 m – macro: r > 1 000 m. | The proposed RTT can support flexible frequency reuse operation thus mixed cell architecture is supported well on the same or different frequencies depending on the implementation. | Confirmed |
| A1.2.29 | Describe any battery saver/intermittent reception capability. |  |  |
| A1.2.29.1 | Ability of the MS to conserve standby battery power : provide details about how the proposal conserves standby battery power. | The battery power saving of mobile station is supported by the sleep mode and the idle mode operations. Since the RTT basically provides packet-based transmission, both two modes operate in a slotted mode. In those modes, a mobile station communicates to its serving base station only in a listening interval and saves its power consumption otherwise. The information on listening, sleep and idle intervals are determined by the negotiation between the base station and the mobile station before the mobile station transits to either of two modes. A mobile station maintains the connection to its serving base station even in the sleep mode, while a mobile station in the idle mode returns system resources relevant to the existing connection to a base station. In latter case, the mobile station is managed by the multiple base stations grouped in a paging zone. Further information can be found in the IEEE 802.16 standard Sections 6.3.21, Sleep Mode, and 6.3.24, Idle Mode. | Confirmed Described in section 1.4.5 in 8F/1079(Rev.1). |
| A1.2.30 | Signaling transmission scheme: if the proposed system will use RTTs for signaling transmission different from those for user data transmission, describe the details of the signaling transmission scheme over the radio interface between terminals and base (satellite) stations. | The same RTT is used for both user data and signaling transmission. | Confirmed Described in section 1.3.2 in 8F/1079(Rev.1)] |
| A1.2.30.1 | Describe the different signaling transfer schemes which may be supported, e.g. in connection with a call, outside a call. Does the RTT support: – new techniques? Characterize. – Signaling enhancements for the delivery of multimedia services? Characterize. | Flexible message-based signaling scheme is used. | [Not confirmed] Need clarification |
| A1.2.31 | Does the RTT support a bandwidth on demand (BOD) capability? BOD refers specifically to the ability of an end-user to request multi-bearer services. Typically, this is given as the capacity in the form of bits per second of throughput. Multi-bearer services can be implemented by using such technologies as multi-carrier, multi-time slot or multi-codes. If so, characterize these capabilities. NOTE 1 – BOD does not refer to the self-adaptive feature of the radio channel to cope with changes in the transmission quality (see § A1.2.5.1). | Yes. The scheduling service is provided for both downlink and uplink traffic. In order for the scheduler to make an efficient resource allocation and provide the desired QoS and data rate in the uplink, mobile stations must feedback accurate and timely information as to the traffic conditions and QoS requirements. To this end, multiple uplink bandwidth request mechanisms, such as bandwidth request through ranging channel, piggyback request and polling are provided to support uplink bandwidth requests. Frequency and time resource allocation in both downlink and uplink is on a per frame basis to duly react to the traffic and channel conditions. Additionally, the amount of resource in each allocation can range from one slot to the entire frame. Further information can be found in the IEEE 802.16 standard, Sections 6.3.6 Bandwidth Allocation and Request mechanism, 6.3.7.3 DL-MAP, 6.3.7.4 UL-MAP, and 8.4.4 Frame Structure. | Confirmed Described in section 1.4.2 in 8F/1079(Rev.1). |
| A1.2.32 | Does the RTT support channel aggregation capability to achieve higher user bit rates? | No. | Confirmed |

| Index | Criteria and attributes | Proponent’s Comments | Evaluator’s Comments |
| --- | --- | --- | --- |
| A1.3 | Expected performances. |  |  |
| A1.3.1 | For terrestrial test environment only. |  |  |
| A1.3.1.1 | What is the achievable BER floor level (for voice)? NOTE 1 – The BER floor level is evaluated under the BER measuring conditions defined in Annex 2 using the data rates indicated in § 1 of Annex 2. | Coded BER floor is implementation-dependent but achievable floor is significantly below GoS requirements (10-3) within the specified ranges of tolerable delay spread (20 µs) and Doppler shifts (250Hz). | Confirmed Checked by ARIB-5 Question and ANS in Appendix 3 Note: Need simulation results of evidence other than 1079(Rev.1) |
| A1.3.1.2 | What is the achievable BER floor level (for data)? | Coded BER floor is implementation-dependent but achievable floor is significantly below GoS requirements (10-6) within the specified ranges of tolerable delay spread (20 µs) and Doppler shifts (250 Hz). | Confirmed No description of the assumption (20uS) Checked by ARIB-5 Question and ANS in Appendix 3 |
|  | NOTE 1 – The BER floor level is evaluated under the measuring conditions defined in Annex 2 using the data rates indicated in § 1 of Annex 2. |  |  |
| A1.3.1.3 | What is the maximum tolerable delay spread (ns) to maintain the voice and data service quality requirements? | The maximum specified range of delay spread (20 µ s in Vehicular B) can be tolerated without an equalizer. | Confirmed Checked by ARIB-5 Question and ANS in Appendix 3 |
|  | NOTE 1 – The BER is an error floor level measured with the Doppler shift given in the BER measuring conditions of Annex 2. |  |  |
| A1.3.1.4 | What is the maximum tolerable Doppler shift (Hz) to maintain the voice and data service quality requirements? | At least 500 Hz, based on the observation that Doppler frequency shows about 570 Hz for 250 km/h at 2.5 GHz | Confirmed Note: Need simulation results or evidence other than 1079(Rev.1). fDT=0.05 In such a condition, ICI does not seem to be negligible. Thus, MCS which can be available for this will be limited. |
|  | NOTE 1 – The BER is an error floor level measured with the delay spread given in the BER measuring conditions of Annex 2. |  |  |
| A1.3.1.5 | Capacity : the capacity of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2 and technical parameters from § A1.2.22 through § A1.2.23.2. |  |  |
| A1.3.1.5.1 | What is the voice traffic capacity per cell (not per sector): provide the total traffic that can be supported by a single cell (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. Provide capacities for all penetration values defined in the deployment model for the test environment in Annex 2. The procedure to obtain this value is described in Annex 2. The capacity supported by not a standalone cell but a single cell within contiguous service area should be obtained here. | See Section 2.3 for details | Confirmed Checked by ARIB-1, 2 and 3. and their ANSs in Appendix 3 Note: Need to verify the data by our simulation results |
|  |  | Voice capacity (ITU Vehicular path loss model, Pedestrian B 3 channel model): |  |
|  |  | - 90 Erlangs/MHz/cell for reuse 3, SIMO,10 MHz PUSC |  |
|  |  | - 80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC |  |
| A1.3.1.5.2 | What is the information capacity per cell (not per sector): provide the total number of user-channel information bits which can be supported by a single cell (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. Provide capacities for all penetration values defined in the deployment model for the test environment in Annex 2. The procedure to obtain this value is described in Annex 2. The capacity supported by not a standalone cell but a single cell within contiguous service area should be obtained here. | See reference ‎0 for details : | Confirmed Note: Need to verify the data by our simulation results |
|  |  | Data capacity (PUSC, ITU Vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, DL:UL=28:9) |  |
|  |  | SIMO: |  |
|  |  | 10 MHz |  |
|  |  | DL = 3.57 Mbps/MHz/cell |  |
|  |  | UL = 1.59 Mbps/MHz/cell |  |
|  |  | 5 MHz |  |
|  |  | DL = 3.45 Mbps/MHz/cell |  |
|  |  | UL = 1.6 Mbps/MHz/cell |  |
|  |  | MIMO: |  |
|  |  | 10 MHz |  |
|  |  | DL = 5.52 Mbps/MHz/cell |  |
|  |  | UL = 2.10 Mbps/MHz/cell |  |
|  |  | - Beamforming technology increases the spectral efficiency of the system. |  |
| A1.3.1.6 | Does the RTT support sectorization? If yes, provide for each sectorization scheme and the total number of user-channel information bits which can be supported by a single site (Mbit/s/MHz) (and the number of sectors) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) in FDD mode or contiguous bandwidth of 30 MHz in TDD mode. | Yes, the RTT supports sectorization. The sectorization and frequency reuse schemes are implementation-dependent and consequently, so are the capacities achieved. The tri-sector scheme is the typical scenario with frequency reuse 1 or reuse 3. | Confirmed 802.16e-2005 Section 6.1 describes capability of the sectorization. Annex B.4 also says the RTT is suited to reuse factor >1, but it can deal with = 1. Reference information: 6.1 PMP The IEEE Std 802.16 wireless link operates with a central BS and a sectorized antenna that is capable of handling multiple independent sectors simultaneously. |
| A1.3.1.7 | Coverage efficiency: the coverage efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |  |
| A1.3.1.7.1 | What is the base site coverage efficiency (km2/site) for the lowest traffic loading in the voice only deployment model? Lowest traffic loading means the lowest penetration case described in Annex 2. | See Link Budget in Section 2.3.4 | Confirmed DL:1.61/UL:1.40 for PA DL:63.60/UL:49.92 for VA It depends on the defined TX power in M. |
| A1.3.1.7.2 | What is the base site coverage efficiency (km2/site) for the lowest traffic loading in the data only deployment model? Lowest traffic loading means the lowest penetration case described in Annex 2. | See Link Budget in Section 2.3.4 | Confirmed DL:0.54/UL:0.18 for PA DL:39.97/UL:13.73 for VA It depends on the defined TX power in M. |
| A1.3.2 | For satellite test environment only |  | N/A |
| A1.3.2.1 | What is the required C/N0 to achieve objective performance defined in Annex 2? |  | N/A |
| A1.3.2.2 | What are the Doppler compensation method and residual Doppler shift after compensation? |  | N/A |
| A1.3.2.3 | Capacity : the spectrum efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  | N/A |
| A1.3.2.3.1 | What is the voice information capacity per required RF bandwidth (bit/s/Hz)? |  | N/A |
| A1.3.2.3.2 | What is the voice plus data information capacity per required RF bandwidth (bit/s/Hz)? |  | N/A |
| A1.3.2.4 | Normalized power efficiency : the power efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  | N/A |
| A1.3.2.4.1 | What is the supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice? |  | N/A |
| A1.3.2.4.2 | What is the supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data? |  | N/A |
| A1.3.3 | Maximum user bit rate (for data) : specify the maximum user bit rate (kbit/s) available in the deployment models described in Annex 2. | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) | Confirmed FFTsize=1024, DL/UL ratio=2:1→number of DL symbol=28, PUSC=720 data subcarrier, 64QAM=6bit/symbol/subcarrier, R=5/6, frame duration = 5ms ∴The bit rates =6*(5/6)*720*28/0.005 = 20160 kbit/s |
| A1.3.4 | What is the maximum range (m) between a user terminal and a BS (prior to hand-off, relay, etc.) under nominal traffic loading and link impairments as defined in Annex 2? | See Link Budget in Section 2.3. The maximum range depends on the deployment and the QoS of a connection | Confirmed DL:720m/UL:670m for Voice for PA DL:360m/UL:230m for Data for PA |
| A1.3.5 | Describe the capability for the use of repeaters. | Repeaters can be used. There is nothing in the technology that precludes the use of repeaters. | Confirmed Implementation matter. Reference information: The 802.16e-based relay system is under study in the 802.16j standardization. |
| A1.3.6 | Antenna systems : fully describe the antenna systems that can be used and/or have to be used; characterize their impacts on systems performance, (terrestrial only); e.g., does the RTT have the capability for the use of: | The air-interface does not place any restrictions on the types of antenna systems such as smart antenna technologies, including Beamforming, Transmit/Receive diversity and MIMO, as well as a combination of these like Beamforming plus MIMO. | Confirmed The matrix for 2x2-MIMO (STC/SDM) is defined as Matrix A/B. For UL, collaborative SM (SDM for different MSs) is also defined. |
|  | – remote antennas: describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas; | The uses of remote and distributed antennas are not precluded. |  |
|  | – distributed antennas: describe whether and how distributed antenna designs are used, and in which IMT-2000 test environments; |  |  |
|  | – Smart antennas (e.g., switched beam, adaptive, etc.): describe how smart antennas can be used and what is their impact on system performance; |  |  |
|  | – other antenna systems. |  |  |
| A1.3.7 | Delay (for voice) | Voice services are provided in the PS-domain with appropriate QoS setting (UGS, rtPS or ErtPS) | Confirmed Network Latency (1-way) for priority Voice (MEVS) packets between 1) Between Customer Premise Equipment (CPE) Routers < 100 ms: propagation delay 2) Mouth to ear < 150 ms: total delay=processing delay + propagation delay + vocoder delay Reference:Application Architecture Task Group(AATG) Best Practices: VoIP over WiMAX Service Networks (DRAFT 1) [2006/07/12] P.20 |
| A1.3.7.1 | What is the radio transmission processing delay due to the overall process of channel coding, bit interleaving, framing, etc., not including source coding? This is given as transmitter delay from the input of the channel coder to the antenna plus the receiver delay from the antenna to the output of the channel decoder. Provide this information for each service being provided. In addition, a detailed description of how this parameter was calculated is required for both the uplink and the downlink. | The minimum delay is roughly 10ms assuming a 5ms TDD frame and the maximum is implementation and traffic load-dependent (scheduling metric, traffic load, buffer sizes, retransmission scheme etc) | Confirmed Checked by Question ARIB-4. and its ANS in Appendix 3 Maximum processing delay=2)-1)-vocoder delay =150-100-20=30ms>10ms |
| A1.3.7.2 | What is the total estimated round trip delay (ms) to include both the processing delay, propagation delay (terrestrial only) and vocoder delay? Give the estimated delay associated with each of the key attributes described in Fig. 6 that make up the total delay provided. | Assuming a 20 ms vocoder, 5ms frame and ignoring queuing delay (typically <30ms), the RTD delay is approximately 60 ms | Confirmed Checked by Question ARIB-4. and its ANS in Appendix 3 Assuming propagation (queuing?) delay =0, RTD={2)-1)}x2 =50[ms]x2 =100ms>60ms |
| A1.3.7.3 | Does the proposed RTT need echo control? | Yes | Confirmed Checking by Question ARIB-4 and its ANS in Appendix 3 |
| A1.3.8 | What is the MOS level for the proposed codec for the relevant test environments given in Annex 2? Specify its absolute MOS value and its relative value with respect to the MOS value of ITU-T Recommendation G.711 (64 k PCM) and ITU-T Recommendation G.726 (32 k ADPCM). | The RTT supports VoIP and is not limited to any particular codecs. Applications/implementations determine the choice of codec. | Confirmed |
|  | NOTE 1 – If a special voice coding algorithm is indispensable for the proposed RTT, the proponent should declare detail with its performance of the codec such as MOS level. (See § A1.2.19) |  |  |
| A1.3.9 | Description of the ability to sustain quality under certain extreme conditions. |  |  |
| A1.3.9.1 | System overload (terrestrial only) : characterize system behaviour and performance in such conditions for each test services in Annex 2, including potential impact on adjacent cells. Describe the effect on system performance in terms of blocking grade of service for the cases that the load on a particular cell is 125%, 150%, 175%, and 200% of full load. Also describe the effect of blocking on the immediate adjacent cells. Voice service is to be considered here. Full load means a traffic loading which results in 1% call blocking with the BER of 1 ´ 10–3 maintained. | The RTT provides many features that can be used to ensure optimal loading in the event of system overload. Among these are admission control, handover, rate adaptation, fractional frequency reuse and power control. | Confirmed Reference information: Fractional frequency reuse technique combining 1-reuse and 3-reuse can be employed. For UL, open/closed loop power control can be available. |
| A1.3.9.2 | Hardware failures : characterize system behaviour and performance in such conditions. Provide detailed explanation on any calculation. | This is implementation-dependent. The RTT does not preclude any means to build in redundancy or other reliability features. | Confirmed |
| A1.3.9.3 | Interference immunity : characterize system immunity or protection mechanisms against interference. What is the interference detection method? What is the interference avoidance method? | In addition to frequency reuse, and intelligent scheduling/RRM, the RTT’s TDD OFDM interface is inherently robust against delay spread, suitable for multi-user detection and supports various smart antenna schemes. Also, the RTT does not preclude any means to cancel interference or to protect against interference | Confirmed |
| A1.3.10 | Characterize the adaptability of the proposed RTT to different and/or time-varying conditions (e.g. propagation, traffic, etc.) that are not considered in the above attributes of § A1.3. | The RTT supports modulation and coding adaptation, HARQ, power control and opportunistic scheduling | Confirmed |

| A1.4 | Technology design constraints | Proponent’s Comments | Evaluator’s Comments |
| --- | --- | --- | --- |
| A1.4.1 | Frequency stability : provide transmission frequency stability (not oscillator stability) requirements of the carrier (include long term – 1 year – frequency stability requirements (ppm)). |  |  |
| A1.4.1.1 | For BS transmission (terrestrial component only). | BS frequency tolerance ≤  2ppm of carrier frequency BS to BS frequency accuracy ≤  1% of subcarrier spacing | Confirmed (Note 1) Described in 8.3.12 (REF[1] of 8F/1079(Rev.1)) Frequency and timing requirements |
| A1.4.1.2 | For MS transmission. | MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing | Confirmed (Note 1) Described in 8.3.12 (REF[1] of 8F/1079(Rev.1)) Frequency and timing requirements |
| A1.4.2 | Out-of-band and spurious emissions : specify the expected levels of base or satellite and mobile transmitter emissions outside the operating channel, as a function of frequency offset. | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. Similar to other IMT-2000 RTTs, terminals adhering to a single global mask will be used to provide global roaming. | Confirmed |
| A1.4.3 | Synchronization requirements : describe RTT’s timing requirements, e.g. – Is BS-to-BS or satellite land earth station (LES)-to-LES synchronization required? Provide precise information, the type of synchronization, i.e., synchronization of carrier frequency, bit clock, spreading code or frame, and their accuracy. – Is BS-to-network synchronization required? (terrestrial only). – State short-term frequency and timing accuracy of BS (or LES) transmit signal. – State source of external system reference and the accuracy required, if used at BS (or LES) (for example: derived from wireline network or GPS receiver). – State free run accuracy of MS frequency and timing reference clock. – State base-to-base bit time alignment requirement over a 24 h period (μs). | BS-to-BS synchronization: Yes. All BSs should be time and frequency synchronized to a common source signal. The common source signal is typically provided by GPS. BS-to-network synchronization: No. BS-to-network synchronization is not required. Frequency accuracy : BS frequency tolerance ≤  2ppm of carrier frequency Timing accuracy ≤ 1usec compared to reference timing. Source of external system reference and the accuracy : GPS (the synchronizing reference shall be a 1 pps timing pulse and a 10 MHz frequency reference) Free run accuracy : MS frequency tolerance ≤ maximum 2% of the subcarrier spacing Timing tolerance: 25% of minimum guard interval( (Tb/32)/4) The BS's timing accuracy is required to be 1 μs compared to reference timing when GPS locked. | Confirmed (Note 1) Described in 8.4.12.4 (REF[1] of 8F/1079(Rev.1)) Transmitter reference timing accuracy |
| A1.4.4 | Timing jitter : for BS (or LES) and MS give: – the maximum jitter on the transmit signal, – the maximum jitter tolerated on the received signal. Timing jitter is defined as r.m.s. value of the time variance normalized by symbol duration. | BS The BS's timing accuracy is required to be 1μsec compared to reference timing. MS MS Transmit symbol timing accuracy within ± (Tb/32)/4 | Confirmed (Note 1) Listed in 12.4.3.1.5 (REF[1] of 8F/1079(Rev.1)) Minimum performance requirements |
| A1.4.5 | Frequency synthesizer: what is the required step size, switched speed and frequency range of the frequency synthesizer of MSs? | Frequency step size : 200 and 250 KHz Switched speed : 200 μs Frequency range : 5, 10 MHz Start frequencies are various, depending on channel bandwidth and profile | Confirmed See Evaluator’s comment of A 3.6.14 concerning “Switched speed” |
| A1.4.6 | Does the proposed system require capabilities of fixed networks not generally available today? | No | Confirmed |
| A1.4.6.1 | Describe the special requirements on the fixed networks for the handover procedure. Provide handover procedure to be employed in proposed RTT in detail. | The RTT supports handover and also provides means for expediting handover. Each base station broadcasts the information on the list of neighboring base stations and their channel information such as the operating center frequency, preamble index and synchronization periodically. The channel information in this broadcasting is used for a mobile station to synchronize with the neighboring base station. After a mobile station monitors the signal strength of a neighboring base station and seeks suitable base station(s) for handover, the mobile station or its serving base station can initiate handover by handover request message. But only the mobile station can transmit handover indication message to the its serving base station. After transmitting handover indication message, the mobile station stops monitoring the downlink frame of its serving base station and performs network re-entry to target base station. To reduce the handover latency further, the serving base station provides the target base station with network entry information on a mobile station to be handed over the target base station. | Confirmed |
| A1.4.7 | Fixed network feature transparency |  |  |
| A1.4.7.1 | Which service(s) of the standard set of ISDN bearer services can the proposed RTT pass to users without fixed network modification. | Convergence Sub-layer in the proposed RTT supports interface to various fixed networks such as ATM, Ethernet, IP, and VLAN. | Confirmed |
| A1.4.8 | Characterize any radio resource control capabilities that exist for the provision of roaming between a private (e.g., closed user group) and a public IMT-2000 operating environment. | Handover between the different access networks is basically supported. Furthermore, Operator ID in the signaling during the handover enable mobile stations to recognize the operator of access network they are handed over to. | Confirmed |
| A1.4.9 A1.4.10 | Describe the estimated fixed signaling overhead (e.g., broadcast control channel, power control messaging). Express this information as a percentage of the spectrum which is used for fixed signaling. Provide detailed explanation on your calculations. Characterize the linear and broadband transmitter requirements for BS and MS (terrestrial only). | The fixed MAP overhead is typically about 10% in a 10 MHz channel with a 5ms frame size. BS - Tx dynamic Range = 10 dB - Spectral flatness according to the following: ≤ ±2 dB for spectral lines from – Nused/4 to 1 and +1 to Nused/4 Within +2/-4 dB for spectral lines from – Nused/2 to Nused/4 and +Nused/4 to Nused/2 - Per sub-carrier flatness ≤ 0.1 dB - Power difference between adjacent subcarriers according to the following: Tx downlink radio frame shall be time-aligned with the 1pps timing pulse within 1 μsec - Tx relative constellation error according to the following: QPSK-1/2 ≤ -15.0 dB QPSK-3/4 ≤ -18.0 dB 16QAM-1/2 ≤ -20.5 dB 16QAM-3/4 ≤ -24.0 dB 64QAM-1/2 (if 64-QAM supported) ≤ -26.0 dB 64QAM-2/3 (if 64-QAM supported) ≤ -28.0 dB 64QAM-3/4 (if 64-QAM supported)≤ -30.0 dB | Confirmed. Preamble[2symbols]+MAP[3symbols]/4710%. Confirmed (Note 1) Listed in 12.4.3.1.5 (REF[1] of 8F/1079(Rev.1)) Minimum performance4 requirements Value of per sub-carrier flatness is updated to 0.4dB. |
|  |  | MS - Tx dynamic Range = 45 dB - Tx power level min adjustment step = 1 dB - Tx power level min relative step accuracy = ± 0.5 dB - Spectral flatness according to the following: ≤ ±2 dB for spectral lines from –Nused/4 to –1 and +1 to Nused/4 Within +2/-4 dB for spectral lines from –Nused/2 to –Nused/4 and +Nused/4 to Nused/2 - Power difference between adjacent subcarriers ≤ 0.1 dB - Tx relative constellation error according to the following: QPSK-1/2 ≤ -15.0 dB QPSK-3/4 ≤ -18.0 dB 16QAM-1/2 ≤ -20.5 dB 16QAM-3/4 ≤ -24.0 dB |  |
| A1.4.11 A1.4.12 | Are linear receivers required? Characterize the linearity requirements for the receivers for BS and MS (terrestrial only). Specify the required dynamic range of receiver (terrestrial only). | BS No. The PAPR of the proposed RTT is around 12dB, and which is not required a stringent linear receiver. BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBm BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 | Confirmed There are no linear receivers required. However, the WiMAX devices are expected to have linear operation for providing good performance across all deployment scenarios, i.e., linear receivers are expected to be used at both BS and MS equipment. See ARIB-08 in Appendix3 Confirmed (Note 1) Described in 8.4.13.3.2 (REF[1] OF 8F/1079(REV.1) of 8F/1079(Rev.1)) BS receiver maximum input signal |
|  |  | Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. |  |
| A1.4.13 | What are the signal processing estimates for both the handportable and the BS? – MOPS (millions of operations per second) value of parts processed by DSP (digital signal processing), – gate counts excluding DSP, – ROM size requirements for DSP and gate counts (kbytes), – RAM size requirements for DSP and gate counts (kbytes). NOTE 1 – At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs etc.). NOTE 2 – The signal processing estimates should be declared with the estimated condition such as assumed services, user bit rate and etc. | It is an implementation issue not covered by the description. | Confirmed |
| A1.4.14 | Dropped calls : describe how the RTT handles dropped calls. Does the proposed RTT utilize a transparent reconnect procedure – that is, the same as that employed for handoff? | No specific process to handle call dropping recovery is defined. However, mobile station can recover the connection after call dropping by means of the Idle mode re-entry procedure. | Confirmed |
| A1.4.15 | Characterize the frequency planning requirements: – frequency reuse pattern: given the required C/I and the proposed technologies, specify the frequency cell reuse pattern (e.g. 3-cell, 7-cell, etc.) and, for terrestrial systems, the sectorization schemes assumed; – characterize the frequency management between different cell layers; – does the RTT use an interleaved frequency plan? – are there any frequency channels with particular planning requirements? – all other relevant requirements. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. If a proponent is going to employ an interleaved frequency plan, the proponent should state so in § A1.2.4 and complete § A1.2.15 with the protection ratio for both the adjacent and second adjacent channel. | The RTT supports frequency reuse configuration of 1 and 3. In order for MS to provide BS with a correct DL channel quality information, MS is required to properly measure CINR of preamble with considering the frequency reuse configuration: i.e. For frequency reuse of 3, consider the modulated subcarriers of the preamble only. For frequency reuse of 1, consider both the un-modulated and the modulated subcarriers of the preamble. There are 114 different preamble code sets in the proposed RTT to differentiate the cell ID and sector ID's per each sector. The RTT can use both the interleaved frequency plan and the non-interleaved frequency plan. | Confirmed WIMAX system configuration is properly described. |
| A1.4.16 | Describe the capability of the proposed RTT to facilitate the evolution of existing radio transmission technologies used in mobile telecommunication systems migrate toward this RTT. Provide detail any impact and constraint on evolution. |  | Confirmed Non applicable question |
| A1.4.17 | Are there any special requirements for base site implementation? Are there any features which simplify implementation of base sites? (terrestrial only) | No | Confirmed |

| A1.5 | Information required for terrestrial link budget template Proponents should fulfill the link budget template given in Table 6 and answer the following questions. | see Section Error: Reference source not found Link Budget | Remarks In the LB, they assume the receiver sensitivity is based on the bandwidth of bit rate (1bps/Hz) because Eb/N0 is required (not SNR) in M1225. We wonder in this section, if they used the data of the generic LB (2.3.4.2), not of M1225-based (2.3.4.1) |
| --- | --- | --- | --- |
| A1.5.1 | What is the BS noise figure (dB)? | 4 dB used for Section Error: Reference source not found Link Budget Max 8 dB Noise Figure is considered in RTT. | Confirmed 5dB is assigned in M1225 |
| A1.5.2 | What is the MS noise figure (dB)? | 7 dB see Section Error: Reference source not found Link Budget Max 8 dB Noise Figure is considered in RTT for MS clients supporting multi band operation. | Confirmed 5 dB is assigned in M1225 |
| A1.5.3 | What is the BS antenna gain (dBi)? | 15 dBi (see Section Error: Reference source not found Link Budget) | Confirmed 10dBi is assigned for Pedestrian and 13dBi is assigned for Vehicular in M1225 |
| A1.5.4 | What is the MS antenna gain (dBi)? | -1 dBi (see Section Error: Reference source not found Link Budget) | Confirmed 0dBi is assigned for both Pedestrian and Vehicular in M1225 |
| A1.5.5 | What is the cable, connector and combiner losses (dB)? | 0 dB (see Section Error: Reference source not found Link Budget) | Confirmed 2dB is assigned at BS side in M1225 |
| A1.5.6 | What are the number of traffic channels per RF carrier? | Function of required QoS | Confirmed |
| A1.5.7 | What is the RTT operating point (BER/FER) for the required Eb/N0 in the link budget template? | 1% FER | Confirmed |
| A1.5.8 | What is the ratio of intra-sector interference to sum of intra-sector interference and inter-sector interference within a cell (dB)? | Depends on environment and receiver implementation | Confirmed |
| A1.5.9 | What is the ratio of in-cell interference to total interference (dB)? | Negligible at low Doppler (<300 Hz) and depends on receiver implementation at high Doppler | Confirmed We consider they assume the perfect FDMA assumption by employing the scheduler. Thus, they conclude only ICI may be included in in-cell interference. |
| A1.5.10 | What is the occupied bandwidth (99%) (Hz)? | Depends on nominal bandwidth, permutation scheme, and on the subchannelization. For the case considered in Section Error: Reference source not found Link Budget, it is approximately 9.2 MHz on the downlink and 2.4 MHz on the uplink | Confirmed The answer may be arguable. The question is the acutual bandwidth at RF, considering the roll-off factor, and it is independent of the UP/DL channel. Ex. 11.2MHz x 851(Preamble/FUSC) /1024=9.30MHz 9.30 x 0.99 =9.21MHz |
| A1.5.11 | What is the information rate (dBHz)? | Depends on service rate with the maximum subject to the channel bandwidth employed. (see Section Error: Reference source not found Link Budget) | Confirmed |

APPENDIX 2

ARIB (Association of Radio Industries and Businesses)

Evaluation for Section 2.3 (“Capacity and Coverage”) of Documents 8F/1079(Rev.1)

This is the appendix of detail of coverage efficiency evaluations and simulation conditions for spectral efficiency evaluations

# Introduction

This document contains the results of simulations for IP-OFDMA coverage and capacity performance evaluation based on the guidelines provided in ITU-R M. [1]. The baseline model used is a 5 MHz bandwidth TDD system with one Tx antenna and two Rx antennas in both the downlink and uplink (1x2 SIMO). Further details of the deployment models considered and simulation assumptions are described in the corresponding sections below. Section 2 describes the results for coverage performance evaluation, and results for data capacity performance evaluation are summarized in Section 3.

# Coverage

The coverage analysis has been performed using the link budget template and default parameter values in M., except that the carrier frequency used is 2500 MHz. The link budgets tables are presented in Section  and the preceding sections describe the deployment models and assumptions.

## Deployment Models

The deployment models considered are Indoor, Outdoor-to-Indoor Pedestrian (OIP), Pedestrian and Vehicular. For each model, both Channels A and B [1] were evaluated and for each channel the speech, low-delay-data (LDD) and unconstrained-delay-data (UDD) cases were considered. Furthermore, since IP-OFDMA is a purely packet-based architecture (i.e., no circuit domain) with HARQ, the speech and LDD cases were emulated using lower target PER and lower number of HARQ retransmissions compared to UDD. The ‘circuit-switched long delay’ case was not evaluated as it was felt to be not relevant to IP-OFDMA.

## Assumptions

The bearer rates take into account the overhead due to retransmissions. Since the system is lightly-loaded per the M. requirements, the Rx interference density is negligible and the downlink repetition and uplink subchannelization gains are explicitly accounted for. The log-normal fade margins are calculated for a 95% area coverage. Note that a site is defined as an omni cell for the Indoor and Pedestrian cases and as a 3-sector cell for the Vehicular case.

The link budgets are for a 5 MHz bandwidth, PUSC SIMO (1Tx-2Rx antennas in both downlink and uplink) system. The diversity gain is included in the Eb/No, hence there is no explicit diversity gain in the link budget tables. The required QPSK-1/2 Eb/No values for the given target PER are obtained from link-level simulations that model the IP-OFDM PHY compliant with the IEEE 802.16e specifications and using the corresponding link channel models from M. [1] and an MRC receiver.

The cell ranges in the link budget tables (Table 1 to Table 6) are calculated using the corresponding path loss models from M.. The corresponding cell areas are calculated assuming omni cells for Indoor, OIP and Pedestrian, and 3-sector cells for Vehicular. In addition to the M. parameter values, the link budget is also calculated for optimized parameter values.

## Link Budgets

Table 1. Link Budgets for Speech (Channel A)

| Frequency | 2500 MHz | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test environment |  | Indoor | Indoor | OIP | OIP | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | A | A | A | A | A | A | A | A |
| Mobile speed |  | 0.5 km/h | 0.5km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | Speech | Speech | Speech | Speech | Speech | Speech | Speech | Speech |
| Note |  |  |  |  |  |  |  |  |  |
| Bit rate | kbps | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 |
| Average Tx power per traffic Ch | dBm | < 10 | < 4 | < 20 | < 14 | < 20 | < 14 | < 30 | < 24 |
| Maximum Tx power per traffic Ch | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Maximum total Tx power | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 2 | 0 | 10 | 0 | 10 | 0 | 13 | 0 |
| Tx EIRP per traffic Ch | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Total Tx EIRP | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Rx antenna gain | dBi | 0 | 2 | 0 | 10 | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Rx interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 |
| Required Eb/(No+Io) | dB | 9.2 | 9.1 | 8.9 | 8.6 | 8.9 | 8.6 | 7.3 | 7.8 |
| Rx sensitivity | dBm | -117.3 | -117.4 | -117.6 | -117.9 | -117.6 | -117.9 | -119.2 | -118.7 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 7.8 | 12.3 | 7.8 | 12.3 | 7.8 | 12.3 | 7.8 | 12.3 |
| Log-normal fade margin | dB | 15.4 | 15.4 | 26.3 | 26.3 | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 119.7 | 118.4 | 127.1 | 125.9 | 142.2 | 141.1 | 156.6 | 154.6 |
| Maximum range | km | 0.47 | 0.42 | 0.26 | 0.24 | 0.62 | 0.58 | 4.89 | 4.34 |
| Coverage efficiency | km2/site | 0.70 | 0.57 | 0.21 | 0.18 | 1.21 | 1.05 | 46.67 | 36.63 |
| Optimized parameters |  |  |  |  |  |  |  |  |  |
| Maximum Tx power per traffic Ch | dBm | 13 | 10 | 23 | 20 | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 2 | 2 | 10 | 0 | 10 | 0 | 17 | 0 |
| Rx antenna gain | dBi | 2 | 2 | 0 | 10 | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 124.7 | 126.4 | 130.1 | 131.9 | 145.2 | 147.1 | 170.6 | 158.6 |
| Maximum range | km | 0.692 | 0.784 | 0.36 | 0.40 | 0.85 | 0.95 | 13.46 | 6.46 |
| Coverage efficiency | km2/site | 1.51 | 1.93 | 0.40 | 0.49 | 2.28 | 2.81 | 352.97 | 81.41 |

Table 2. Link Budgets for Speech (Channel B)

| Frequency | 2500 MHz | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test environment |  | Indoor | Indoor | OIP | OIP | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | B | B | B | B | B | B | B | B |
| Mobile speed |  | 0.5 km/h | 0.5 km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | Speech | Speech | Speech | Speech | Speech | Speech | Speech | Speech |
| Note |  |  |  |  |  |  |  |  |  |
| Bit rate | kbps | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 | 17.8 |
| Average Tx power per traffic Ch | dBm | < 10 | < 4 | < 20 | < 14 | < 20 | < 14 | < 30 | < 24 |
| Maximum Tx power per traffic Ch | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Maximum total Tx power | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 2 | 0 | 10 | 0 | 10 | 0 | 13 | 0 |
| Tx EIRP per traffic Ch | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Total Tx EIRP | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Rx antenna gain | dBi | 0 | 2 | 0 | 10 | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Rx interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 | 42.5 |
| Required Eb/(No+Io) | dB | 8.6 | 7.9 | 6.9 | 6.9 | 6.9 | 6.9 | 7.5 | 7.9 |
| Rx sensitivity | dBm | -117.9 | -118.6 | -119.6 | -119.6 | -119.6 | -119.6 | -119.0 | -118.6 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 7.8 | 12.3 | 7.8 | 12.3 | 7.8 | 12.3 | 7.8 | 12.3 |
| Log-normal fade margin | dB | 15.4 | 15.4 | 26.3 | 26.3 | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 120.3 | 119.6 | 129.1 | 127.6 | 144.2 | 142.8 | 156.4 | 154.5 |
| Maximum range | km | 0.49 | 0.47 | 0.29 | 0.27 | 0.70 | 0.64 | 4.83 | 4.31 |
| Coverage efficiency | km2/site | 0.77 | 0.68 | 0.27 | 0.22 | 1.52 | 1.28 | 45.54 | 36.19 |
| Optimized parameters |  |  |  |  |  |  |  |  |  |
| Maximum Tx power per traffic Ch | dBm | 13 | 10 | 23 | 20 | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 2 | 2 | 10 | 0 | 10 | 0 | 17 | 0 |
| Rx antenna gain | dBi | 2 | 2 | 0 | 10 | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 125.3 | 127.6 | 132.1 | 133.6 | 147.2 | 148.8 | 170.4 | 158.5 |
| Maximum range | km | 0.725 | 0.860 | 0.40 | 0.44 | 0.96 | 1.04 | 13.30 | 6.42 |
| Coverage efficiency | km2/site | 1.65 | 2.32 | 0.50 | 0.60 | 2.87 | 3.42 | 344.43 | 80.42 |

Table 3. Link Budgets for LDD (Channel A)

| Frequency | 2500 MHz | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test environment |  | Indoor | Indoor | OIP | OIP | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | A | A | A | A | A | A | A | A |
| Mobile speed |  | 0.5 km/h | 0.5 km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | LDD2048 | LDD2048 | LDD384 | LDD384 | LDD384 | LDD384 | LDD144 | LDD144 |
| Note |  |  |  |  |  |  |  |  |  |
| Bit rate | kbps | 2273.3 | 2273.3 | 426.2 | 426.2 | 426.2 | 426.2 | 159.8 | 159.8 |
| Average Tx power per traffic Ch | dBm | < 10 | < 4 | < 20 | < 14 | < 20 | < 14 | < 30 | < 24 |
| Maximum Tx power per traffic Ch | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Maximum total Tx power | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 2 | 0 | 10 | 0 | 10 | 0 | 13 | 0 |
| Tx EIRP per traffic Ch | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Total Tx EIRP | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Rx antenna gain | dBi | 0 | 2 | 0 | 10 | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Rx interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 63.6 | 63.6 | 56.3 | 56.3 | 56.3 | 56.3 | 52.0 | 52.0 |
| Required Eb/(No+Io) | dB | 5.2 | 6.9 | 6.3 | 7.3 | 6.3 | 7.3 | 4.6 | 7.1 |
| Rx sensitivity | dB | -100.2 | -98.5 | -106.4 | -105.4 | -106.4 | -105.4 | -112.4 | -109.9 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 1.3 | 0.0 | 7.0 | 6.3 | 7.0 | 6.3 | 7.8 | 9.3 |
| Log-normal fade margin | dB | 15.4 | 15.4 | 26.3 | 26.3 | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 96.2 | 87.1 | 115.1 | 107.4 | 130.3 |  | 149.8 | 142.8 |
| Maximum range | km | 0.08 | 0.04 | 0.13 | 0.08 | 0.36 | 0.23 | 3.76 | 2.45 |
| Coverage efficiency | km2/site | 0.02 | 0.00 | 0.05 | 0.02 | 0.41 | 0.17 | 27.56 | 11.70 |
| Optimized parameters |  |  |  |  |  |  |  |  |  |
| Maximum Tx power per traffic Ch | dBm | 13 | 10 | 23 | 20 | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 2 | 2 | 10 | 0 | 10 | 0 | 17 | 0 |
| Rx antenna gain | dBi | 2 | 2 | 0 | 10 | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 101.2 | 95.1 | 118.1 | 113.4 | 133.3 | 128.5 | 163.8 | 146.8 |
| Maximum range | km | 0.11 | 0.07 | 0.18 | 0.14 | 0.43 | 0.32 | 8.86 | 3.13 |
| Coverage efficiency | km2/site | 0.04 | 0.02 | 0.10 | 0.06 | 0.57 | 0.33 | 153.09 | 19.09 |

Table 4. Link Budgets for LDD (Channel B)

| Frequency | 2500 MHz | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test environment |  | Indoor | Indoor | OIP | OIP | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | B | B | B | B | B | B | B | B |
| Mobile speed |  | 0.5 km/h | 0.5 km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | LDD2048 | LDD2048 | LDD384 | LDD384 | LDD384 | LDD384 | LDD144 | LDD144 |
| Note |  |  |  |  |  |  |  |  |  |
| Bit rate | kbps | 2273.3 | 2273.3 | 426.2 | 426.2 | 426.2 | 426.2 | 159.8 | 159.8 |
| Average Tx power per traffic Ch | dBm | < 10 | < 4 | < 20 | < 14 | < 20 | < 14 | < 30 | < 24 |
| Maximum Tx power per traffic Ch | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Maximum total Tx power | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 2 | 0 | 10 | 0 | 10 | 0 | 13 | 0 |
| Tx EIRP per traffic Ch | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Total Tx EIRP | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Rx antenna gain | dBi | 0 | 2 | 0 | 10 | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Rx interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 63.6 | 63.6 | 56.3 | 56.3 | 56.3 | 56.3 | 52.0 | 52.0 |
| Required Eb/(No+Io) | dB | 4.3 | 5.6 | 4.3 | 5.6 | 4.3 | 5.6 | 4.6 | 7.2 |
| Rx sensitivity | dB | -101.1 | -99.8 | -108.4 | -107.1 | -108.4 | -107.1 | -112.4 | -109.8 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 1.3 | 0.0 | 7.0 | 6.3 | 7.0 | 6.3 | 7.8 | 9.3 |
| Log-normal fade margin | dB | 15.4 | 15.4 | 26.3 | 26.3 | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 97.1 | 88.4 | 117.1 | 109.1 | 132.3 | 124.2 | 149.8 | 142.7 |
| Maximum range | km | 0.08 | 0.04 | 0.15 | 0.09 | 0.40 | 0.25 | 3.76 | 2.44 |
| Coverage efficiency | km2/site | 0.02 | 0.01 | 0.07 | 0.03 | 0.51 | 0.20 | 27.56 | 11.55 |
| Optimized parameters |  |  |  |  |  |  |  |  |  |
| Maximum Tx power per traffic Ch | dBm | 13 | 10 | 23 | 20 | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 2 | 2 | 10 | 0 | 10 | 0 | 17 | 0 |
| Rx antenna gain | dBi | 2 | 2 | 0 | 10 | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 102.1 | 96.4 | 120.1 | 115.1 | 135.3 | 130.2 | 163.8 | 146.7 |
| Maximum range | km | 0.12 | 0.08 | 0.20 | 0.15 | 0.48 | 0.36 | 8.86 | 3.11 |
| Coverage efficiency | km2/site | 0.05 | 0.02 | 0.13 | 0.07 | 0.72 | 0.40 | 153.09 | 18.86 |

Table 5. Link Budgets for UDD (Channel A)

| Frequency | 2500 MHz | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test environment |  | Indoor | Indoor | OIP | OIP | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | A | A | A | A | A | A | A | A |
| Mobile speed |  | 0.5 km/h | 0.5 km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | LDD2048 | LDD2048 | LDD384 | LDD384 | LDD384 | LDD384 | LDD144 | LDD144 |
| Note |  |  |  |  |  |  |  |  |  |
| Bit rate | kbps | 2985.7 | 2985.7 | 559.8 | 559.8 | 559.8 | 559.8 | 209.9 | 209.9 |
| Average Tx power per traffic Ch | dBm | < 10 | < 4 | < 20 | < 14 | < 20 | < 14 | < 30 | < 24 |
| Maximum Tx power per traffic Ch | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Maximum total Tx power | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 2 | 0 | 10 | 0 | 10 | 0 | 13 | 0 |
| Tx EIRP per traffic Ch | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Total Tx EIRP | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Rx antenna gain | dBi | 0 | 2 | 0 | 10 | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Rx interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 64.8 | 64.8 | 57.5 | 57.5 | 57.5 | 57.5 | 53.2 | 53.2 |
| Required Eb/(No+Io) | dB | 2.2 | 4.1 | 2.6 | 4.5 | 2.6 | 4.5 | 1.7 | 4.6 |
| Rx sensitivity | dB | -102.0 | -100.2 | -108.9 | -107.0 | -108.9 | -107.0 | -114.1 | -111.2 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 0.0 | -1.3 | 7.0 | 5.3 | 7.0 | 5.3 | 7.8 | 9.3 |
| Log-normal fade margin | dB | 15.4 | 15.4 | 26.3 | 26.3 | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 96.7 | 87.5 | 117.6 | 108.0 | 132.8 | 123.2 | 151.5 | 144.1 |
| Maximum range | km | 0.08 | 0.04 | 0.15 | 0.09 | 0.42 | 0.24 | 4.17 | 2.65 |
| Coverage efficiency | km2/site | 0.02 | 0.00 | 0.07 | 0.02 | 0.54 | 0.18 | 33.94 | 13.72 |
| Optimized parameters |  |  |  |  |  |  |  |  |  |
| Maximum Tx power per traffic Ch | dBm | 13 | 10 | 23 | 20 | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 2 | 2 | 10 | 0 | 10 | 0 | 17 | 0 |
| Rx antenna gain | dBi | 2 | 2 | 0 | 10 | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 101.7 | 95.5 | 120.6 | 114.0 | 135.8 | 129.2 | 165.5 | 148.1 |
| Maximum range | km | 0.12 | 0.07 | 0.21 | 0.14 | 0.49 | 0.34 | 9.84 | 3.39 |
| Coverage efficiency | km2/site | 0.04 | 0.02 | 0.13 | 0.06 | 0.76 | 0.36 | 188.53 | 22.39 |

Table 6. Link Budgets for UDD (Channel B)

| Frequency | 2500 MHz | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test environment |  | Indoor | Indoor | OIP | OIP | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | B | B | B | B | B | B | B | B |
| Mobile speed |  | 0.5 km/h | 0.5 km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | LDD2048 | LDD2048 | LDD384 | LDD384 | LDD384 | LDD384 | LDD144 | LDD144 |
| Note |  |  |  |  |  |  |  |  |  |
| Bit rate | kbps | 2985.7 | 2985.7 | 559.8 | 559.8 | 559.8 | 559.8 | 209.9 | 209.9 |
| Average Tx power per traffic Ch | dBm | < 10 | < 4 | < 20 | < 14 | < 20 | < 14 | < 30 | < 24 |
| Maximum Tx power per traffic Ch | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Maximum total Tx power | dBm | 10 | 4 | 20 | 14 | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 2 | 0 | 10 | 0 | 10 | 0 | 13 | 0 |
| Tx EIRP per traffic Ch | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Total Tx EIRP | dBm | 10 | 4 | 28 | 14 | 28 | 14 | 41 | 24 |
| Rx antenna gain | dBi | 0 | 2 | 0 | 10 | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Rx interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 64.8 | 64.8 | 57.5 | 57.5 | 57.5 | 57.5 | 53.2 | 53.2 |
| Required Eb/(No+Io) | dB | 2.3 | 3.8 | 1.7 | 4.0 | 1.7 | 4.0 | 1.7 | 4.7 |
| Rx sensitivity | dB | -101.9 | -100.5 | -109.8 | -107.5 | -109.8 | -107.5 | -114.1 | -111.1 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 0.0 | -1.3 | 7.0 | 5.3 | 7.0 | 5.3 | 7.8 | 9.3 |
| Log-normal fade margin | dB | 15.4 | 15.4 | 26.3 | 26.3 | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 96.6 | 87.8 | 118.5 | 108.5 | 133.7 | 123.7 | 151.5 | 144.0 |
| Maximum range | km | 0.08 | 0.04 | 0.16 | 0.09 | 0.44 | 0.25 | 4.17 | 2.64 |
| Coverage efficiency | km2/site | 0.02 | 0.01 | 0.08 | 0.02 | 0.60 | 0.19 | 33.94 | 13.55 |
| Optimized parameters |  |  |  |  |  |  |  |  |  |
| Maximum Tx power per traffic Ch | dBm | 13 | 10 | 23 | 20 | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 2 | 2 | 10 | 0 | 10 | 0 | 17 | 0 |
| Rx antenna gain | dBi | 2 | 2 | 0 | 10 | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 101.6 | 95.8 | 121.5 | 114.5 | 136.7 | 129.7 | 165.5 | 148.0 |
| Maximum range | km | 0.12 | 0.08 | 0.22 | 0.15 | 0.52 | 0.35 | 9.84 | 3.37 |
| Coverage efficiency | km2/site | 0.04 | 0.02 | 0.15 | 0.07 | 0.85 | 0.38 | 188.53 | 22.11 |

# Data Capacity

The data capacity was evaluated using system-level simulations based on the deployment models and parameter values detailed in M. [1] and for a carrier frequency of 2500 MHz. In the following, Table 7 through Table 9 contain the system parameters used for the performance evaluation. Table summarizes the results.

## Air-Interface Parameters

Table 7. OFDMA Parameters

| Parameters |  | Values |
| --- | --- | --- |
| System Channel Bandwidth (MHz) |  | 5 |
| Sampling Frequency (Fp, MHz) |  | 5.6 |
| FFT Size |  | 512 |
| Subcarrier Frequency Spacing (kHz) |  | 10.94 |
| Useful Symbol Time (Tb = 1/f, µs) |  | 91.4 |
| Guard Time (Tg =Tb/8, µs) |  | 11.4 |
| OFDMA Symbol Duration (Ts = Tb + Tg, µs) |  | 102.9 |
| Frame Duration (ms) |  | 5 |
| Number of OFDMA Symbols |  | 48 (including ~1.6 symbols for TTG/RTG) |
| DL PUSC | Null Sub-carriers | 92 |
|  | Pilot Sub-carriers | 60 |
|  | Data Sub-carriers | 360 |
|  | Subchannels | 15 |
| UL PUSC | Null Sub-carriers | 104 |
|  | Pilot Sub-carriers | 136 |
|  | Data Sub-carriers | 272 |
|  | Subchannels | 17 |

## Propagation and Channel Parameters

Table 8. Propagation and Channel Models

| Parameters | Indoor | OIP | Pedestrian | Vehicular |
| --- | --- | --- | --- | --- |
| Channel Impulse Response Model | ITU-Indoor A/B | ITU-Outdoor-to-Indoor & Pedestrian A/B |  | ITU-Vehicular A/B |
| Propagation Model | ITU-Indoor | ITU-Outdoor-to-Indoor & Pedestrian |  | ITU-Vehicular |
| Log-Normal Shadowing Standard Deviation | 12 dB | 10 dB |  |  |
| BS Shadowing Correlation | 1 between sectors, 0.5 between cells |  |  |  |
| Penetration Loss Mean | 0 | 12 | 0 |  |
| Penetration Loss Standard Deviation | 0 dB | 8 dB | 0 dB |  |

## Simulation Parameters

Table 9. System Parameters

| Parameters | Indoor | OIP | Pedestrian | Vehicular |
| --- | --- | --- | --- | --- |
| Number of Cells | 19 (3 floors) | 19 |  |  |
| Cell Configuration | Omni |  |  | 3-sector |
| Operating Frequency | 2500 MHz |  |  |  |
| Duplex | TDD |  |  |  |
| Channel Bandwidth | 5 MHz |  |  |  |
| Cell/Sector Radius | 20m | 150m | 250m | 1.5km |
| BS-BS distance | 35m | 260m | 430m | 2.6km |
| Antenna Pattern | Omni |  |  | 70° (-3 dB) with 20 dB front-to-back ratio |
| BS Height | N/A |  |  | 15m above rooftop |
| BS Antenna Gain | 2 dB | 10 dB |  | 13 dB |
| MS Antenna Gain | 0 dB |  |  |  |
| BS Maximum Transmit Power | 10 dBm | 20 dBm |  | 30 dBm |
| MS Maximum Transmit Power | 4 dBm | 14 dBm |  | 24 dBm |
| # of BS Tx/Rx Antenna | 1Tx, 2Rx |  |  |  |
| # of MS Tx/Rx Antenna | 1Tx, 2Rx |  |  |  |
| Receiver Type | MMSE |  |  |  |
| MS Noise Figure | 5 dB |  |  |  |
| BS Cable Loss | 2 dB |  |  |  |
| Frequency Reuse | 1 |  |  |  |
| Users/Sector | 10 |  |  |  |
| Traffic Type | Full Buffer |  |  |  |
| Channel Estimation | Ideal but there is additional noise modeled due to mobile speed; degradation amounts to ~1 dB for low mobility (3 km/h) and ~2 dB for high mobility (120 km/h) |  |  |  |
| Antenna Correlation | 0 at BS, 0.5 at MS |  |  |  |
| PHY Abstraction | MIC |  |  |  |
| Scheduler | Proprietary Proportional Fair |  |  |  |
| Link Adaptation | Adaptive for initial transmissions, non-adaptive for retransmissions, 2-frame delay feedback implemented |  |  |  |
| HARQ | Max 4 retransmissions, non-adaptive, synchronous |  |  |  |
| HARQ Operating PER | 30% |  |  |  |
| Power Control | No power control on downlink, slow power control (10 Hz) on uplink |  |  |  |
| Coding | CTC |  |  |  |
| Frame Overhead | 11 OFDM Symbols (7 DL, 3 UL, 1 TTG) |  |  |  |
| Data Symbols per Frame | 24 for downlink, 12 for uplink (after excluding downlink and uplink overhead) |  |  |  |
| DL/UL Partition | 24:12 |  |  |  |

## Simulation Methodology

The simulations employ a dynamic simulator that models the evolution of the channel of the subscriber and interferers in time, and employs a PHY abstraction to predict link layer performance. The data is collected over several trials (drops) for the MSs of the center cell BS of the 19-cell layout. A drop is defined as a simulation run for a given set of users over a specified number of TDD frames. For each drop the methodology is as follows:

- The system is modeled as a network of 19 cells. The cells are omni-cells for the Indoor, OIP and Pedestrian cases, and 3-sectored cells for the Vehicular case.

- MSs are first dropped randomly throughout the system with a minimum distance - based on the deployment scenario - of the MS from the BS. Each mobile corresponds to an active user session that runs for the duration of the drop.

- Cell/Sector assignment is based on the received power at an MS from all potential serving cells/sectors. The cell/sector with highest received power at the MS, taking into account slow fading characteristics (path loss, shadowing, and antenna gains) is chosen as the serving cell/sector. Each user is assigned to only one cell/sector.

- Mobile stations served by the center cell/sector are randomly selected one-at-a-time until the required number of active users have been selected.

- MSs are assigned channel models with the given probabilities.

- The signal and interference are computed for each MS by considering the propagation model and assigned channel models. Mean Instantaneous Capacity (MIC) is used for the link to system mapping.

- Packets are scheduled with a packet scheduler using the proportional fairness metric. Channel quality feedback delay, PDU errors and HARQ are modeled and packets are retransmitted as necessary. A non-adaptive, synchronous retransmission scheme is used for HARQ transmissions.

- Performance statistics are collected only for MSs in the center cell (of the middle floor for the Indoor deployment). Layer 1 and layer 2 overheads are accounted for in computing the performance metrics.

## Simulation Results

Table 10 summarizes the results for the Indoor, OIP, Pedestrian and Vehicular deployment cases.

The user throughput is defined with respect to the entire TDD frame, including both downlink and uplink sub-frames. Thus, it is calculated as the successfully delivered bits at the MAC Service Access Point (SAP) averaged over the entire simulation time, where the simulation time includes both downlink and uplink sub-frame times.

The downlink/uplink spectral efficiency is defined with respect to the downlink/uplink sub-frame of the TDD frame respectively. Thus, it is calculated as the successfully delivered bits at the MAC SAP averaged over all users and the net downlink/uplink resources.

Table 10. IP-OFDMA System Performance (5 MHz TDD, 2:1 DL/UL Ratio)

| Deployment | DL |  | UL |  |
| --- | --- | --- | --- | --- |
|  | 5th %ile user throughput (kbps) | Spectral Efficiency (bps/Hz/cell) | 5th %ile user throughput (kbps) | Spectral Efficiency (bps/Hz/cell) |
| Indoor Office | 216 | 1.47 | 95 | 0.92 |
| OIP | 136 | 1.37 | 89 | 0.88 |
| Pedestrian | 164 | 1.37 | 99 | 0.90 |
| Vehicular | 138 | 2.82 | 85 | 2.20 |

# Deployment Model Results Matrix

The results are summarized in the deployment model matrix of Table 11 through Table 14 using the M format. The numbers are based on the capacity simulation assumptions. The numbers within parentheses are based on the link budget, which are for low-loading scenarios and therefore are more optimistic.

Table 11. Deployment Model Result Matrix for Indoor Office

| Input assumptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Indoor office |  |  |
| Test service |  | Data |  |  |
| Base station antenna height (m) |  | N/A |  |  |
| Any other assumptions made by the proponent (e.g. antenna pattern, sectorization etc.) |  | Omni cells, 3 floors with 7 cells per floor |  |  |
| Deployment results |  |  |  |  |
| Total number of cell sites | Total number of RF channels | Number of voice channels per RF channel | Coverage efficiency (km2/site) | Spectrum efficiency (Mbps/MHz/cell) |
| 8 (min 1) per floor of area 10000 m2 | 1 | N/A | 0.001 (max 0.57) | 1.5 (DL) 0.9 (UL) |

Table 12. Deployment Model Result Matrix for OIP

| Input assumptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Outdoor-to-Indoor |  |  |
| Test service |  | Data |  |  |
| Base station antenna height (m) |  | N/A |  |  |
| Any other assumptions made by the proponent (e.g. antenna pattern, sectorization etc.) |  | Omni cells |  |  |
| Deployment results |  |  |  |  |
| Total number of cell sites | Total number of RF channels | Number of voice channels per RF channel | Coverage efficiency (km2/site) | Spectrum efficiency (Mbps/MHz/cell) |
| 566 (min 217) | 1 | N/A | 0.07 (max 0.18) | 1.4 (DL) 0.9 (UL) |

Table 13. Deployment Model Result Matrix for Pedestrian

| Input assumptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Pedestrian |  |  |
| Test service |  | Data |  |  |
| Base station antenna height (m) |  | N/A |  |  |
| Any other assumptions made by the proponent (e.g. antenna pattern, sectorization etc.) |  | Omni cells |  |  |
| Deployment results |  |  |  |  |
| Total number of cell sites | Total number of RF channels | Number of voice channels per RF channel | Coverage efficiency (km2/site) | Spectrum efficiency (Mbps/MHz/cell) |
| 128 (min 24) | 1 | N/A | 0.2 (max 0.6) | 1.4 (DL) 0.9 (UL) |

Table 14. Deployment Model Result Matrix for Vehicular

| Input assumptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Vehicular |  |  |
| Test service |  | Data |  |  |
| Base station antenna height (m) |  | 15m |  |  |
| Any other assumptions made by the proponent (e.g. antenna pattern, sectorization etc.) |  | 3-sector cells |  |  |
| Deployment results |  |  |  |  |
| Total number of cell sites | Total number of RF channels | Number of voice channels per RF channel | Coverage efficiency (km2/site) | Spectrum efficiency (Mbps/MHz/cell) |
| 35 (min 5) | 1 | N/A | 4.4 (max 36.6) | 2.8 (DL) 2.2 (UL) |

# References

- Recommendation ITU-R M.1225, “Guidelines for Evaluation of Radio Transmission Technologies for IMT-2000”

Appendix 3

ARIB (Association of Radio Industries and Businesses)

Questionnaire and comments in the evaluation process

Question from ARIB for clarification on the proposed document (Document  8F/1079(Rev.1))

Responses from IEEE are included below, marked ANS:

ARIB-1: Activity factor in Voice traffic capacity

Reference: 8F/1079(Rev.1), A3.1.1.1 in Section 3 and Section 2.3.1.1

Question: We understood that the voice activity factor assumed for voice capacity simulation was 40% in the proposal, which is different from one in M.1225 (50%). What is a rationale to use the different assumption? What are possible impacts on the voice capacity results if you use 50% for the voice activity factor?

ANS: We chose 40% as that seemed to be the more widely-used value. With voice activity factor (VAF) of 50% capacity the capacity will reduce but partly offset by the reduced overheads. With VAF 0.5, we estimate the numbers to roughly 60 and 67 Erlangs/MHz/cell for 5 and 10 MHz bandwidths respectively.

ARIB-2: Assumed path loss model and channel model in Voice traffic capacity

Reference: 8F/1079(Rev.1), A3.1.1.1 in Section 3

Question: Why did the simulation in proposal assume "Vehicular" for path loss model and "Pedestrian B" for channel impulse response model, which are for different test environments? Is it possible to consider capacity based on path loss model and channel impulse response model from the same test environment, such as "Vehicular", "Outdoor to indoor and pedestrian" and/or "Indoor office"?

ANS: The Vehicular path loss model is the more popular path loss model in industry today, in combination with the Pedestrian-B and Vehicular-A channel models. Hence the motivation for the particular choice of models used.

ARIB-3: Call blocking rate in Voice traffic capacity

Reference: 8F/1079(Rev.1), A3.1.1.1 in Section 3 and Section 2.3.1.3

Question: We understood that the call blocking rate assumed for voice capacity simulation was 2% in the proposal, which is different from one in M.1225 (1%). What is a rationale to use the different assumption? What are possible impacts on the voice capacity if you use 1% for the call blocking rate?

ANS: Actually, the results presented were rounded down and happened to correspond to the 1% blocking. For 2% blocking, the capacity would be 92 Erlangs/MHz/cell. The difference is marginal.

ARIB-4: Referenced Recommendation in A.3.2.1

Reference: 8F/1079(Rev.1), A3.2.1 in Section 3 (A.1.3.7.1/A.1.3.7.2)

Comment: This section is closely related to the item #1 "One-way end-to-end delay less than 40 ms." in "Voice and data performance requirements". Teherefore, referenced ITU Recommendattion need to be ITU-T G.174 and G.114 that referred from G.174. G.114 contains the guidance on the daley performance issue of IP over VoIP.

ANS: The M.1079 is end-to-end delay performance requirement whereas G.174 is delay contribution from specific components to the end-to-end delay.

Question: Isn't there any problem (such as the tap length calculation may not convergent due to echo characteristic varies in both side) when echo cancellers are installed on both MS and BS?

ANS: Unclear about the question but it seems to be applicable to all RTTs.

ARIB-5: Terminology and Assumption of "tolerable delay spread"

Reference: 8F/1079(Rev.1), A3.4.1.4 and A3.4.2.2.2 in Section 3 <Related attributes> A1.3.1.1, A1.3.1.2, and A1.3.1.3 in Section 2.1

Question: We have two questions on this issue. One is the verbiage of "delay spread". We believe you mean the "maximum excess delay" of delay profile because Vehicular B channel model has the path of 20us delay with -16dB power. Is our understanding correct? Second, even if our understanding is correct, the 20us delay occurs ISI (inter-symbol-interference) in the proposed system with CP=11.4us (See Table 2 in Section 1.3.3, A1.2.1.4 in Section 2.1, and Table 14 in Section 2.3.3.1), and the performance is significantly degraded in such a condition. Whilst the CP of Tg/4(=22.8us) is defined in Table 384b in the 802.16e standard document. What is your assumption of the items listed in the Reference mentioned above?

ANS: Yes, we’ve used ‘delay spread’ to mean excess delay. There will be ‘graceful degradation’ in the performance due to ISI as the excess delay goes beyond the CP length, without the need to resort to sophisticated equalizers. Note that the fading is still essentially flat for the specified sub-carrier spacing. The CP factor can be adjusted depending on the requirements and propagation environments. The maximum value as stated above is ~22.8 µsec. The CP factor of 11.84 µsec is the recommended value for typical Vehicular multi-path environment to trade off support for delay spread and power efficiency.

ARIB-6: Spectrum Bandwidth

Reference: 8F/1079(Rev.1), A1.2.5 in Section 2.1

Question: According to R03-WP8F-C-1079(REV.1) Document, each bandwidth for 5MH/10MHz system is about 4.7MHz / 9.4MHz.But there are no any detail information about how these number can be evaluated. Followings are our assumption of bandwidth calculation. Please confirm our interpretation is a way to verify the these values. 5MHz system (AMC or O-PUSC permutation) The number of used subcarrier is 433. Subcarrier BW = 10.94kHz Therefore Bandwidth is, 433 x 10.94kHz = 4.73MHz 10MHz system (AMC or O-PUSC permutation) The number of used subcarrier is 865. Subcarrier BW = 10.94kHz Therefore Bandwidth is, 865 x 10.94kHz = 9.46MHz.

ANS: We confirm your interpretation of the bandwidth calculation. In addition, we provide below the full list of bandwidth calculations for all permutation schemes and both channel bandwidths of 5 MHz and 10 MHz: - For 5 MHz: - for PUSC permutation: 4.6047 MHz - for FUSC permutation: 4.6703 MHz - for AMC permutation: 4.7359 MHz - For 10 MHz: - for PUSC permutation: 9.1984 MHz - for FUSC permutation: 9.3078 MHz - for AMC permutation: 9.4609 MHz

ARIB-7: Average Power

Reference: 8F/1079(Rev.1), A1.2.16.1.2 in Section 2.1

Question: Even though transmit power of each power class is specified as “peak transmit power” in A1.2.16, we assume that these number could be average power. Therefore A1.2.16.1.2 refers A1.2.16. Please verify our understanding.

ANS: The power-class dependent numbers shown in A1.2.16 refer to peak transmit power. By referring to A1.2.16 in the original (1079/rev1) response to A1.2.16.1.2, the close relationship of the average and peak transmit power is implied. In the following, please find the numbers with respect to the average transmit power; please note that the number referring to the PAPR (12 dB) has been used in the original responses to A1.2.11, A1.4.11, and A3.2.2.3.2 If we assume a PAPR of 12 dB (without considering any PAPR reduction schemes at the transmitter), the average transmit power in the active state for the different power classes of Section A1.2.16 is: - For 16QAM: 1. 6 <= Ptx,average < 9 2. 9 <= Ptx,average < 13 3. 13 <= Ptx,average < 18 4. 18 <= Ptx,average - For QPSK: 1. 8 <= Ptx,average < 11 2. 11 <= Ptx,average < 15 3. 15 <= Ptx,average < 18 4. 18 <= Ptx,average

ARIB-8: BS transmit power

Reference: 8F/1079(Rev.1), A1.2.16.2 in Section 2.1

Question: Your response specifies “See A1.2.16” but BS transmit power does not specified in this section. We believe BS transmit power will be determined in each region so it should be regional matter. Is our understanding correct?

ANS: Yes, your understanding is correct.

ARIB-9: Receiver linearity

Reference: 8F/1079(Rev.1), A1.4.11 in Section 2.1

Question: Are linear receivers required? Characterize the linearity requirements for the receivers for BS and MS (terrestrial only). We were concerned about proponents comments which may conflict with A3.6.7. To our concern we received the following comments from Intel Japan. Please verify IEEE is the same position with Intel Japan. Intel Japan comments: [There are no linear receivers required. However, the WiMAX devices are expected to have linear operation for providing good performance across all deployment scenarios, i.e., linear receivers are expected to be used at both BS and MS equipment]

ANS: We verify the response from Intel Japan: Although there is no stringent requirement from the RTT for linear receivers, it is expected that linear receive operation will be provided by IP-OFDMA device vendors; therefore, there is no conflict with A3.6.7. Further, we would like to stress that there are no explicit numbers stated in the contribution 1079/rev1 with respect to transmit or receive related requirements, e.g., out-of-band emissions, spurious emissions, etc., since those depend on regional regulatory requirements. Currently, the WiMAX Forum is specifying tests, e.g., spectral mask, spurious emission tests, etc., in the radio conformance testing procedures so that certified devices fully comply with the related regulatory requirements.

ARIB-10: Outage

Reference: 8F/1079(Rev.1), A3.1.1.2 in Section 3 and Section 2.3.3

Question: Please provide percentage of users in outage or CDF of the user throughput on the downlink and uplink respectively with full-buffer traffic in simulations. Please also verify that the performance of resource signaling mechanisms such as FCH, DL-MAP and UL-MAP was modeled in the simulations.

ARIB-11: Information Capacity

Reference: 8F/1079(Rev.1), A3.1.1.2 in Section 3 and Section 2.3.3

Question11-1: What frequency reuse is assumed for the simulation?

Question11-2: How many base station and mobile station transmitter and receiver antennas are used for this capacity? For a fair comparison with other technologies, only mandatory features of the standard should be used to meet the requirements. Does SIMO imply 1Tx 2 Rx? Simulation assumptions include 4 Rx options.

Question11-3: What’s the FER distribution among users?

Question11-4: How is the FCH, DL-MAP, UL-MAP reliability and overhead modeled? How is CQI feedback and ACK feedback reliability and overhead modeled? Overhead accounting seems to be clearly insufficient.

Question11-5: Please provide the following: - Average DL single antenna C/I for reuse 3. - Average UL pathloss distribution. - Interference over thermal distribution on UL. - Provide link level simulations. - Provide detailed description on system and link simulation interface.

Question11-6: The propagation model “COST 231 Suburban” used in 1079(Rev.1)-E is not consistent with well-known “COST 231 Hata”. Details should be provided. Why did you use it for simulation?

ARIB-12: Voice Traffic Capacity

Reference: 8F/1079(Rev.1), A.3.1.1.1 in Section 3 and Section 2.3.1

Question12-1: Is VoIP services only supported in reuse 3 system?

Question12-2: The voice traffic capacity has to takenn into account the frequency reuse. Does 90 Erlangs/MHz/cell taken into account of reuse 3? I.e., does this number imply 900 Erlangs or 2700 Erlangs over the 30 MHz bandwidth.

Question12-3: How is the effect of handoff latency modeled in this analysis? How is VoIP services supported by the hard handoff algorithm.

Question12-4: For the voice capacity using VOIP, What needs to be reported along with the number of Erlangs/MHz/cell (for a given reuse pattern) is also some metric to describe the latency or outage. Provide mean and 97% tail latency distribution.

Question12-5: How many base station and mobile station transmitter and receiver antennas are used for this capacity? For a fair comparison with other technologies, only mandatory features of the standard should be used to meet the requirements.

Question12-6: Is the bandwidth request contention based? How is the request queuing delay simulated? If not, provide overhead computation for pre-allocated UL-MAP for all VoIP users in the system.

Question12-7: Is the transmitter side packet drop captured in FER accounting?

Question12-8: How is the FCH, DL-MAP, UL-MAP reliability and overhead modeled? How is CQI feedback and ACK feedback reliability and overhead modeled?

Question12-9: Please provide the following: - Average DL single antenna C/I for reuse 3 - Average UL pathloss distribution - Interference over thermal distribution on UL. - Link level simulations. - Detailed description on system and link simulation interface.

______________