| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0208 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 2nd FG IPTV meeting: Busan, 16 – 20 October 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Nortel Networks (Canada) |  |  |  |
| Title: | Proposed changes to FG IPTV-OD-0018 |  |  |  |

This contribution proposes changes to FG IPTV-OD-0018, IPTV Middleware, Applications, and Content Platforms, in order to improve its appearance and accuracy.

Background

At the most recent FG IPTV meeting, i.e. 10-14 July 2006 in Geneva, a working document was initiated for IPTV Middleware, Applications, and Content Platforms.

Discussion

After reviewing this document it appears that it requires some improvement in order to enhance its appearance and accuracy. This contribution proposes changes which is hoped to improve both the structure and appearance of this document. In line with the idea of reusing ITU-T terms, provided they are applicable and suitable, a number of terms are proposed or are revised.  The addition of these terms also requires that clause 2 of the document be modified as well, and proposed text for clause 2 is also provided.

Reference

FG IPTV-OD-0018 (10-14 July 2006), IPTV Middleware, Applications, and Content Platforms

Proposals:

Proposal 1

It is proposed to replace FG IPTV-OD-0018, IPTV Middleware, Applications, and Content Platforms working document with the following.

CONTENTS

1	Scope	3

2	References	3

3	Definitions	3

4	Abbreviations	9

5	Conventions	10

IPTV Middleware, Applications, and Content Platforms

# Scope

This working document identifies and defines middleware platforms, including applications, content formats, and their uses, that facilitate effective and interoperable use of an IPTV system for presenting and interacting with IPTV services.

- References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[1] 	TBD

# Definitions

This working document uses or defines the following terms:

Note: In this clause [aaa] notation after title of a term indicates source of the definition for that term.

  - Application [Y.101]: A functional implementation realised as software running in one or spread over several interplaying hardware entities.

3.2	Application instance:

An occurrence of an application

  - Application manager [J.200]:

  - The entity that is responsible for managing the lifecycle of the applications. It manages applications running in both the Presentation engine and Execution engine if both are present.

  - Application Program Interface (API) [J.200]:

  - Consists of software libraries that provide uniform access to system services.

  - BroadcastTV (BTV):

- Same as the traditional CATV, and the channel switch and channel selection are implemented through screen menu or the remote controller. [0058]

  - Character[J.200]:

  - A specific "letter" or other identifiable symbol, e.g. "A".

  - Character encoding [J.200]:

  - Mapping between an integer input value, and the textual character that is represented by this mapping, e.g. in ASCII value 65 (decimal) is character "A", or shift-JIS for Japanese characters.

  - Character set:

  - See character encoding.

  - Content [J.200]:

  - A general term that refers to any of the following: application, application resource collection, or application resource.

3.8+	Content protection:

Act of preventing unauthorized use of any content.

  - Electronic Program Guide (EPG) [J.200]: A Program table that is presented electronically.

EPG provider [J.90]:

- The entity that collects, collates and assembles the elements of information that constitute the EPG database.

  - Electronic Service Guide (ESG):

- Description for available services. Through the information in the ESG, the user can select the services and items he/she is interested in and find stored items on the terminal. (ETSI, DVB-CBMS)

  - Execution Engine (EE) [J.200]: instructions and associated data and media content. An execution engine may be implemented with an operating system, computer language compilers, interpreters, and Application Interfaces (APIs), which a procedural application may use to present audiovisual content, interact with a user, or execute other tasks, which are not evident to the user. A common example of an execution engine is the JavaTV software environment, using the Java programming language and byte code interpreter, JavaTV APIs, and a Java Virtual Machine for program execution.

  - Extended Channel:

- Means to organise different operation content together, including live channel, TVOD, information service, EPG and so on. For each extended channel, there is a related channel number. [0062]

  - Globally Executable MHP:

- A terminal specification based on MHP that enables applications to interoperate across OCAP, MHP and other GEM based platforms. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 3.2.1)

  - Home Network (HN) [J.190]:

  - Short-range communications system designed for the residential environment, in which two or more devices exchange information under some sort of standard control.

  - Interactive Television (iTV): A service in which the user can send requests, within a browser environment, to the service provider in order to obtain additional information. This capability requires channels to have what is known as a 'return path'.

  - Interactive Program Guide (IPG)

- The IPG is a service facilitated by the middleware, which provides the consumer detailed information about the content available to them. (AITS, ATIS-0800002)

- Ability of the ITF(Terminal Function) to display and allow user selection of program, content, and service descriptions (AITS, ATIS-0800002)

  - IPTV Terminal Function (ITF):

- Functionality within the consumer network that is responsible for terminating the IP signal, and converting the content into a renderable format -- e.g., a STB. (AITS, ATIS-0800002)

  - Lifetime/Lifecycle of an application:

- Characterizes the time from which the application is loaded to the time the application is destroyed. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 3.2.1)

  - Metadata [J.98]:

  - Metadata is descriptive data associated with a content asset package or file. It may vary in depth from merely identifying the content package title or information to populate an EPG to providing a complete index of different scenes in a movie or providing business rules detailing how the content package may be displayed, copied, or sold. Separate uses for metadata have originated from the studios, distribution networks (Cable, Satellite), down to the CPE (STBs, PVRs).

  - Metadata format [H.222AMD1]: Identifies the

  - coding format of metadata.

  - Metadata service [H.222AMD1]:

  - Coherent set of metadata of the same format delivered to a receiver for a specific purpose.

  - Middleware: A system software facilitating interaction between a subscriber and the media delivery entity.

3.25+	Multimedia [J.148]: The combination of multiple forms of media such as audio, video, text, graphics, fax, and telephony in the communication of information.

  - Multimedia application [T.174]:

  - An application which involves the presentation of multimedia information to the user.

  - Multimedia service [Q.1702]:

  - A service in which the interchanged information consists of more than one type, such as text, graphics, sound, image and video.

  - Multimedia representation [T.171]:

  - The property of handling several types of representation media.

  - Navigation:

- The process of selection and movement among analogue and digital services offered on the cable network. The service information tables defined in this protocol assist in the navigation process by providing physical service locations, channel names and numbers for user reference. Those tables supporting electronic program guides also assist the navigation process. (ITU-T, J.94 Amd. 1 (00), B.3.2.3)

  - Presentation:

- The operation of rendering the content of a document in a form perceptible to a human being. (ITU-T, T.411 (93), 3.158)

  - Presentation engine [J.200]:

  - A subsystem in a receiver that evaluates and presents declarative applications consisting of content, such as audio, video, graphics, and text primarily based on presentation rules defined in the presentation engine. A presentation engine also responds to formatting information, or "markup", associated with the content, to user inputs, and to script statements, which control presentation behaviour and initiate other processes in response to user input and other events. A common example of a presentation engine is an HTML browser, capable of displaying text and graphic content formatted in HTML, with interactive behaviour programmed in ECMA script.

  - Service [Y.101]:

  - A structure set of capabilities intended to support applications.

  - Set-top box (STB): A device that contains demodulator, de-multiplexer, decoder, other functionalities and interfaces related to signal reception and presentation of the distributed programme at the subscriber's site.

  - Stream [J.200]:

  - A unidirectional continuous flow of content.

  - Telecommunication service [F.700]:

  - Set of telecommunication capabilities that works in a complementary and cooperative way in order to let users perform applications.

  - Time-shifting (Digital Recording / Playback):

- technology allows an audio/video stream to be digitally recorded and replayed at a later time (for example, PVR).. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 1.2.2.2.2.5, pp. 3)

  - Time-shifted TV:

- Time-shifted TV service breaks the user’s conventional habit that the user can watch TV program only according to fix timetable so that the user can discretionarily control his time to watch TV. The implementation of Time-shifted TV is to store the TV program into the network system while broadcasting TV program, the TV program will be stored according to prearranged time and displayed on the user terminal. PVR service includes the following services: Networked Personal Video recorder(nPVR),Client Personal Video Recorder(cPVR), Time shift TV and TV of Yesterday. [0062]

  - User Device:

- Also known as Home Network End-Devices (HNED), Home Network Device (HND), Consumer Equipment (CE), terminal, and physical device. A piece of HW equipment running its SW and attached to a Home Network and being identified by a GUID – e.g., a MAC address. A single Device can be used by one or more Users. (AITS, ATIS-0800002)

  - User Interface:

- Also known as UI, a user interface is the sensory and behavioural aspects of a program that are presented to a user. The term is generally used to denote the menuing and navigational constructs of a program. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 3.2.1)

  - Video-on-Demand (VoD):

  - A service in which the subscriber can view video content whenever desired.  The operating assumption is that the content is stored on the provider’s VoD server. Subscriber accesses the movie from a library directory which may include search engine that accesses movie description and rating.  Subscribers typically have the ability to pause, play, rewind, fast forward the content, or even stop viewing it and return to it at a later time when using this service.

- Abbreviations

API	Application Programming Interface

AV	Audio Video

DVB	Digital Video Broadcasting

EPG	Electronic Program Guide

ESG	Electronic Service Guide

ETSI	European Telecommunications Standards Institute

GEM	Globally Executable MHP

GIF	Graphics Interchange Format

GUI	Graphical User Interface

HN	Home Network

HTML	Hyper Text Mark-up Language

HTTP	Hyper Text Transport Protocol

I/O	Input / Output

ID	IDentifier

IETF	Internet Engineering Task Force

IHDN	In Home Digital Network

IP	Internet Protocol

IPR	Intellectual Property Rights

ISDN	Integrated Services Digital Network

ISO	International Organization for Standardization

ITU	International Telecommunication Union

ITV	Interactive Television

JDK	Java Development Kit

JFIF	JPEG File Interchange Format

JPEG	Joint Picture Expert Group

LMDS	Local Multipoint Distribution System

MAC	Media Access Control

MHP	Multimedia Home Platform

MIME	multipurpose internet mail extensions

MMDS	Multipoint Microwave Distribution System

MPEG	Moving Picture Experts Group

NTSC	National Television Systems Committee

NVOD	Near-Video-On-Demand

OCAP	OpenCable Application Platform

OS	Operating System

OSD	On Screen Display

PFR	Portable Font Resource

PNG	Portable Network Graphics

PSI	Program Specific Information

PSIP	Program and System Information Protocol

RAM	Random Access Memory

ROM	Read Only Memory

SI	Service Information

TCP	Transmission Control Protocol

TS	Transport Stream

UDP	User Datagram Protocol

UI	User Interface

URL	Uniform Resource Locator

UTF8	Universal Transformation Format 8

VM	Virtual Machine

WAN	Wide Area Network

XML	eXtensible Markup Language

# Conventions

TBD

Proposal 2

It is proposed that the following references be added in clause 2 of the working document.

[H.222AM1]	ITU-T Recommendation H.222.0 Amd 1(2002), Information technology – Generic coding of moving pictures and associated audio information: Systems Amendment 1: Carriage of metadata over ITU-T Rec. H.222.0 | ISO/IEC 13818-1 streams

[Y.101]	ITU-T Recommendation Y.101 (2000), Global Information Infrastructure terminology: Terms and definitions

[J.90]	ITU-T Recommendation J.90 (2000), Electronic programme guides for delivery by digital cable television and similar methods - Reference operating scenario and requirements

[J.200]	ITU-T Recommendation J.200 (2001), Worldwide common core  Application environment for digital interactive television services

[J.190]	ITU-T Recommendation J.190 (2002), Architecture of MediaHomeNet that supports cable-based services

[Q.1702]	ITU-T Recommendation Q.1702 (2002), Long-term vision of network aspects for systems beyond IMT-2000

[T.171]	ITU-T Recommendation T.171 (1996), Protocols for interactive audiovisual services: Coded representation of multimedia and hypermedia objects

[F.700]	ITU-T Recommendation T.700 (2000), Framework Recommendation for multimedia services

[T.174]	ITU-T Recommendation T.174 (1996), Application programming interface (API) for MHEG-1

[J.98]	ITU-T Recommendation J.98 (2003), Metadata requirements for video-on-demand in cable networks

[J.148]	ITU-T Recommendation J.148 (2003), Requirements for an objective perceptual multimedia quality model

___________________