| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0653 |  |
|  |  |  | English only |  |
| WG(s): 5 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ETRI, Republic of Korea |  |  |  |
| Title: | Considerations on personal IPTV broadcast service scenario with WG5 |  |  |  |

IPTV service is multimedia broadcast service that delivers the contents through IP network. IPTV service can make it more interesting by providing a way for user to broadcast the contents they own personally. Personal IPTV broadcast scenario makes IPTV customer a content provider also. To make this scenario possible, service provider can be responsible for distributing personal broadcast content, or just relaying session between content providing customer and receiving customer. By doing so, it will dramatically increase the number of contents available.

At the last meeting, FG already agreed on personal broadcast scenario. This contribution proposes some detailed considerations to make the proposed scenario feasible.

- Considerations to support the suggested scenario

For personal IPTV broadcast scenario, customer’s terminal device can play a role as content source or head-end. Some portion of both capabilities is needed to support the proposed scenario.

- Terminal device should support A/V capturing capability.

- Customer can generate a content source with their terminal device. Terminal device can be a mobile device with embedded camera, a PC with capability of capturing A/V being locally played out, or any other devices that support A/V input interfaces

- Terminal device should support A/V encoding capability

- For terminal device with low processing power, at least low-bandwidth encoding profile (e.g., MPEG2 simple profile, H.264 AVC Baseline profile) is required. For PC-like terminal device, a better-quality encoding profile can be applied.

- Terminal device should have streaming protocol for real-time transmission of content (e.g., RTP/RTCP).

Proposal

We hope that some considerations proposed in this contribution are concerned by relevant WG5 to support the proposed scenario.

_______________