| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0956 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | Proposal of additional text for clause 8.2 of IPTV architecture document |  |  |  |

This contribution proposes additional text for clause 8.2 Architectural Differences in IPTV architecture document (FG IPTV-DOC-0115).

# Introduction

The IPTV Architecture document that was developed at the 5th FG-IPTV meeting (23-31 July, Geneva, Switzerland) as FG-IPTV-doc-0115 defines an NGN-based IPTV architecture (IMS-based and non-IMS-based) and a non-NGN-based IPTV architecture. Although clause 8.2 of the document is a placeholder for describing the differences between these three architectures, there is no text so far. The target date for the completion of this document is December 2007, so it is necessary to add appropriate text immediately. This contribution proposes additional text for clause 8.2.

# Discussion

Although three figures showing different options of the IPTV architecture are given in clause 8.2, there is no descriptive text so far. Some text indicating the differences between the three approaches should be added to the clause.

- Proposal

This contribution proposes adding the following text to clause 8.2 of the IPTV architecture document.

8.2.1 Differences between NGN-based and non-NGN-based IPTV architectures

The NGN-based IPTV architecture is based on the NGN architecture defined in Y.2012 (formerly Y.FRA) [1] and can use the components and functions of the NGN. The non-NGN-based IPTV architecture does not necessarily require these components and functions and uses conventional and/or legacy technology networks for the delivery of IPTV services. The main differences are described below.

- The NGN-based IPTV architecture can use Network Attachment Control Functions (NACF) defined in ITU-T draft recommendation Y.NACF [2] to provide functions such as authentication and IP configuration.

- The NGN-based IPTV architecture can use Resource & Admission Control Functions (RACF) defined in ITU-T recommendation Y.2111 (formerly Y.RACF) [3] to provide resource and admission control functions.

- The NGN-based IPTV architecture can use Service Control Functions defined in ITU-T recommendation Y.2012 (formerly Y.FRA) [1] to provide service control functions.

8.2.2 Differences between NGN-non-IMS-based and NGN-IMS-based IPTV architectures

The NGN-IMS-based IPTV architecture uses Core IMS and associated functions such as Service User Profile Function defined in ITU-T recommendation Y.2012 (formerly Y.IFN) [4] to provide service control functions. The NGN-non-IMS-based IPTV architecture uses Service Control Functions other than Core IMS to provide service control functions.

# References

[1] ITU-T Y.2012	ITU-T Recommendation Y.2012 (2006), NGN Functional Requirements and Architecture.

[2] ITU-T Y.NACF 	ITU-T Draft Recommendation Y.NACF, Functional Architecture and Requirements for NACF in Next Generation Network.

[3] ITU-T Y.2111 	ITU-T Recommendation Y.2111 (2006), Resource and Admission Control Functions in Next Generation Networks.

[4] ITU-T Y.2021 	ITU-T Recommendation Y.2021 (2006), IMS for Next Generation Networks.

____________