| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1032 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11–18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | Proposal on improvement for performance monitoring in DOC-154 of WG2 |  |  |  |

# 1. Discussion

This contribution proposes some improvement for working document outputted in 6th FG meeting of WG6.

1) Performance monitoring system is an additional stacking system and it is necessary to pinpoint  that it should not increase the existing operation system load or affect the normal operation of the existing system.

2) The Figure ‎6-1 shows the IPTV Domains, in which the content flow is  from right side to left side. But Figure ‎6-2 used the opposite direction.

3)  In 6th section, it is mentioned that a whole performance monitoring system is recommended to include a performance monitoring management platform and also description about the management platform is presented. But apart from those, there is no text about the management platform elsewhere. So it should be defined in the future that how the platform communicates all the points and collects the parameters.

4) The 7th section list all the monitoring parameters in accordance with the hierarchical relationship. But there is only a hierarchical structure for television service shown in Figure 7-1 and other parameters, such as Service Line-Up parameters and VOD parameters, were not presented. So a hierarchical monitoring parameter model should be very useful when an IPTV operator considers all the parameters.

# 2. Proposal

1) It is proposed to add the below text at the end of 1st section.

The performance monitoring system should not increase the existing service operation system load or affect the normal operation of the existing system.

2) It is proposed to redraw the Figure ‎6-2 using the same data flow direction as that in  Figure ‎6-1.  Because the figure is not  revisable, it is proposed the editor completes redrawing at drafting stage.

3) It is proposed to add some editor’s notes after Figure ‎0-1 about the management platform like the below text:

“The interfaces between performance monitoring management platform and all the points should be farther defined in future to specify that how the platform communicates with all the points, config the monitoring tasks and collect the parameters.”

4) it is proposed to add a hierarchical monitoring parameter reference model shown as below before the table 7-1 in 7th section.

IP Layer/Network Parameters

Physical Layer Parameters

Service Line-Up parameters

Channel Attribute parameters

Transport Stream parameters

IPTV monitoring parameters model

VOD Parameters

OtherParameters

Contributor List

| Name | Company | Tel. | Email |
| --- | --- | --- | --- |
| Siqing Luo | China Telecom | +86 21 58755452 | luosq@sttri.com.cn |
| Jianting Meng | China Telecom | +86 21 58755057 | mengjt@sttri.com.cn |

____________